Supplementary Materials & Methods

The Digital Imaging and Communications in Medicine (DICOM) CECT images were scanned by 640-slice CT scanner (Aquilion One, Toshiba Medical Systems), dual-source CT scanner (SOMATOM Force) and the SOMATOM Definition Flash scanner (both by Siemens AG, Forchheim, Germany). An automatic tube current modulation technique was used and the tube voltage was set to 120 kVp. Prior to scanning, patients were instructed to hold their breath to avoid motion artefacts. The first series was a non-enhanced CT study of the chest (helical scan type, 100 kV and automatic mAs, rotation time of 0.6 s, slice thickness and spacing of 1.375:1 per 5 mm, scan field of view (SFOV) of 50 cm, and matrix of 512*512); the scanning area ranged from the thoracic inlet to the level of the diaphragm. A total of 50 to 120 mL (1 mL/kg weight) of contrast agent (iohexol, 300 mg/mL, iodine) was injected at a rate of 3.0 mL/s with a pump syringe.

The imaging data of 134 patients were retrospectively collected from the picture archiving and communication systems (PACS) of Shandong Provincial Hospital Affiliated to Shandong First Medical University, Qilu hospital of Shandong University, Shandong Cancer Hospital and Shandong Provincial Qianfoshan Hospital.

Parameters of Linear Discriminant Analysis in combined radiomics model:

solver: svd

shrink age: None

Supplementary Results

The calculation formula of the RS:

0.9118 * Non_enhanced_original_shape_Maximum3DDiameter + 0.7778 * Enhanced_squareroot_firstorder_90Percentile + 0.3946 * Non_enhanced_log-sigma-3-0-mm-3D_glszm_SmallAreaLowGrayLevelEmphasis + -0.144 * Non_enhanced_lbp-3D-m2_firstorder_RootMeanSquared + 1.117 * Non_enhanced_log-sigma-3-0-mm-3D_gldm_DependenceEntropy + 1.0408 * Enhanced_log-sigma-1-0-mm-3D_glrlm_ShortRunLowGrayLevelEmphasis + -0.0343 * Non_enhanced_wavelet-HL_glszm_ZoneEntropy + -0.8436 * Non_enhanced_wavelet-LH_glszm_ZoneEntropy + 0.3036 * Non_enhanced_log-sigma-2-0-mm-3D_glszm_GrayLevelNonUniformityNormalized + -0.2562 * Enhanced_wavelet-HH_glszm_GrayLevelNonUniformityNormalized + -1.699  * Non_enhanced_wavelet-HL_glcm_MCC + 1.1953 * Non_enhanced_log-sigma-4-0-mm-3D_firstorder_Range + -2.2662 * Non_enhanced_wavelet-HL_glcm_Imc1 + 1.2617 * Enhanced_log-sigma-1-0-mm-3D_firstorder_Skewness + 0.1562 * Non_enhanced_log-sigma-5-0-mm-3D_firstorder_Maximum + -0.3325 * Non_enhanced_lbp-3D-k_firstorder_Minimum + -0.7048 * Non_enhanced_exponential_glszm_SizeZoneNonUniformity + -0.1317 * Enhanced_lbp-3D-k_firstorder_Minimum + -1.2695 * Enhanced_exponential_glszm_SmallAreaEmphasis + -1.075 * Non_enhanced_wavelet-HH_glrlm_RunVariance + 0.4167 * Enhanced_exponential_glszm_SizeZoneNonUniformity

Supplementary Figures and Tables

Table S1 Performances of different machine learning classifiers in the validation set

|  | Logistic regression | Decision tree | Random Forest | SVM | Xgboost | LinearSVC |
| --- | --- | --- | --- | --- | --- | --- |
| ACC | 0.659 | 0.610 | 0.512 | 0.707 | 0.707 | 0.634 |
| AUC | 0.785 | 0.638 | 0.589 | 0.722 | 0.708 | 0.676 |
| F1-score | 0.72 | 0.636 | 0.583 | 0.76 | 0.769 | 0.681 |

AUC: Area under the curve; ACC: Accuracy; SVM: Support vector machines;

Xgboost: eXtreme gradient boosting; LinearSVC: Linear support vector classification;

TP: True positive; FP: False positive; TN: True negative; FN: False negative;

Accuracy = (TP + TN) / (TP + FP + TN + FN);

Precision= TP / (TP + FP);

Recall=TP/(TP+FN);

F1-Score = 2 × Precision × Recall / (Precision + Recall).

Table S2 Characteristics of patients with TC in each cohort

| Variables | Training cohort (n = 93) |  | Internal validation cohort (n = 41) |  |
| --- | --- | --- | --- | --- |
|  | n | % | n | % |
| Gender |  |  |  |  |
| Male | 62 | 66.7 | 25 | 61 |
| Female | 31 | 33.3 | 16 | 39 |
| Age (years), median (interquartile range) | 58(50-56) |  | 56(49-63) |  |
| BMI, median (interquartile range) | 23.4(21.5-26.6) |  | 23.4(22.1-27.7) |  |
| CK5/6 |  |  |  |  |
| Positive | 89 | 95.7 | 40 | 97.6 |
| Negative | 4 | 4.3 | 1 | 2.4 |
| Ki-67 |  |  |  |  |
| >60% | 9 | 9.7 | 5 | 12.2 |
| ≤60% | 84 | 90.3 | 36 | 87.8 |
| CD5 |  |  |  |  |
| Positive | 63 | 67.7 | 28 | 68.3 |
| Negative | 30 | 32.3 | 13 | 31.7 |
| CD117 |  |  |  |  |
| Positive | 82 | 88.2 | 40 | 97.6 |
| Negative | 11 | 11.8 | 1 | 2.4 |
| Depth of invasion |  |  |  |  |
| T1 | 3 | 3.2 | 0 | 0 |
| T2 | 9 | 9.7 | 3 | 7.3 |
| T3 | 15 | 16.1 | 2 | 4.9 |
| T4 | 66 | 71 | 36 | 87.8 |
| Lymph node metastasis |  |  |  |  |
| N0 | 42 | 45.1 | 15 | 36.6 |
| N1 | 13 | 14 | 14 | 34.1 |
| N2 | 38 | 40.9 | 12 | 29.3 |
| Distant metastasis |  |  |  |  |
| M(−) | 56 | 60.2 | 27 | 65.9 |
| M(+) | 37 | 39.8 | 14 | 34.1 |
| TNM stage |  |  |  |  |
| I | 1 | 1.1 | 0 | 0 |
| II | 8 | 8.6 | 3 | 7.3 |
| III | 24 | 25.8 | 10 | 24.4 |
| IV | 60 | 64.5 | 28 | 68.3 |
| LDH |  |  |  |  |
| Elevated | 25 | 26.9 | 8 | 19.5 |
| Normal | 68 | 73.1 | 33 | 80.5 |
| CEA |  |  |  |  |
| Elevated | 8 | 8.6 | 10 | 24.4 |
| Normal | 85 | 91.4 | 31 | 75.6 |
| NSE |  |  |  |  |
| Elevated | 47 | 50.5 | 19 | 46.3 |
| Normal | 46 | 49.5 | 22 | 53.7 |
| NE |  |  |  |  |
| Elevated | 22 | 23.7 | 8 | 19.5 |
| Normal | 71 | 76.3 | 33 | 80.5 |
| Lym |  |  |  |  |
| Normal | 53 | 57 | 25 | 61 |
| Declined | 40 | 43 | 16 | 39 |
| NLR, median (interquartile range) | 4(2.6-6.5) |  | 3.6(2.4-4.5) |  |
| Clinical Manifestation |  |  |  |  |
| Y | 67 | 72 | 28 | 68.3 |
| N | 26 | 28 | 13 | 31.7 |

LDH, lactate dehydrogenase; CEA, carcinoembryonic antigen; NSE, neuron-specific enolase; NE, neutrophils; Lym, lymphocytes; NLR, neutrophil-to-lymphocyte ratio

Table S3 IDI assessment of the combined radiomics model

|  | Results | 95% CI | P value |  |
| --- | --- | --- | --- | --- |
| Training |  |  |  |  |
| IDI | 27% | 17.3%-36.9% | ＜0.001 |  |
| Validation |  |  |  |  |
| IDI | 0.4% | -0.9%-8.9% | 0.108 |  |

IDI: integrated discrimination improvement

Table S4 ROC curve comparison between each model

|  | AUC | Z score* | P value |  |
| --- | --- | --- | --- | --- |
| Combined radiomics model vs. AJCC 8th TNM stage model |  |  |  |  |
| Training set | 0.937 vs. 0.799 | 3.086 | 0.002 |  |
| Validation set | 0.899 vs. 0.754 | 1.634 | 0.102 |  |
| Combined radiomics model vs. Clinical model |  |  |  |  |
| Training set | 0.937 vs. 0.799 | 3.58 | ＜0.001 |  |
| Validation set | 0.899 vs. 0.879 | 0.633 | 0.572 |  |
| Combined radiomics model vs. Radiomics model |  |  |  |  |
| Training set | 0.937 vs. 0.911 | 1.345 | 0.179 |  |
| Validation set | 0.899 vs. 0.785 | 1.783 | 0.075 |  |
| Radiomics model vs. Clinical model |  |  |  |
| Training set | 0.911 vs. 0.799 | 2.196 | 0.028 |  |
| Validation set | 0.785 vs. 0.879 | -1.783 | 0.075 |  |

* by Delong test

Fig. S1 Univariable and multivariable logistic analyses for predicting PD in the training set

* Statistically significant results from logistic regression analysis

Fig. S2 Confusion matrices for (1) Radiomics model, (2) AJCC 8th TNM stage model, (3) Combined radiomics model and (4) Clinical model in the training and validation sets. The figures in the grid express the values of the probabilities derived from the calculation of the number of people in each set