Supplementary materials

Appendix S1. The detailed CT parameters

Our hospital uses the following multidetector CT scanners: GE Lightspeed 64 (Ge Healthcare Chicago, Il, USA), Siemens Sensation 16 (Siemens Healthineers, Erlangen, Germany) and Siemens Definition 64 (Siemens Healthineers). The parameters were as follows: tube voltage 120 kVp, tube current 150–200 mA, Pixel size, 0.4-1.0mm, slice thickness 5 mm, interval thickness 5 mm, and nonionic contrast media iomeperol at a rate of 3.0 ml/s with a dose of 1.3 ml/kg. A 100-Hu threshold of the abdominal aorta at the celiac artery was taken as the baseline, then the arterial phase was examined 15-30 s after the unenhanced phase.

Appendix S2. The details about the extracted features

To obtain more high-throughput features, Wavelet filter, Laplacian of Gaussian filters (LoG), Square, Logarithm, Square-Root, Gradient Exponential and Local Binary Pattern 2D/3D (LBP) ﬁlters were used to transform the normalized CT images. Then, the ﬁrst-order features and texture features, including the gray-level size zone matrix (GLSZM), graylevel co-occurrence matrix (GLCM), neighborhood graytone difference matrix (NGTDM), gray-level dependence matrix (GLDM) and gray-level run length matrix (GLRLM), were extracted from the transformed images. 1834 features were extracted for each type of ROI, and each patient had a total of 3 types of ROI, totaling 5502 features.

Appendix S3. The inter- and intra-observer and correlation coefficient (ICCs) analysis

To assess the reproducibility of these features, a set of 30 CT images was randomly selected and evaluated. Interobserver repeatability was determined by comparing the IAT, PAT 3-mm, and PAT 5-mm ROI delineations performed by Radiologist 1 (L.X.) and Radiologist 2 (G.Y.) within the same timeframe. Additionally, to evaluate intraobserver repeatability, Radiologist 1 conducted a second delineation 1 month later. Throughout the evaluation process, both radiologists remained unaware of the pathological findings. The intraclass correlation coefficient (ICC), where an ICC value exceeding 0.75 indicated a high level of consistency, was used to measure the extent of agreement between the assessments.

Table S1. Multivariable Cox regression analysis for association of variables with Recurrence-free survival.

| Variables | RM-IAT+IPAT 5mm (X1+X2+X3+X4) HR (95% CI) | p- value | RM-IAT (X1 +X3) HR (95% CI) | p-value | RM-IPAT 5mm (X2 +X4) HR (95% CI) | p-value |
| --- | --- | --- | --- | --- | --- | --- |
| X1 | 1.63(0.99-2.7) | 0.056 | 2.1(1.7-2.6) | < 0.001 |  |  |
| X2 | 1.54(0.66-3.60) | 0.322 | - | - | 2.4(1.65-3.4) | < 0.001 |
| X3 | 0.97(0.53-1.80) | 0.919 | 1.5(1.3-1.7) | < 0.001 |  |  |
| X4 | 0.76(0.51-1.10) | 0.183 | - | - | 1.1(0.81-1.5) | 0.588 |

X1: exponential_gldm_DependenceNonUniformity from IAT region; X2: lbp_3D_k_glrlm_LongRunHighGrayLevelEmphasis from IPAT region; X3: lbp_3D_m1_glrlm_LongRunLowGrayLevelEmphasis from IAT region; X4: original_shape_Sphericity from IPAT region.

RM-IAT: mulivariable Cox regression analysis for only variables derived from IAT region;

RM-IPAT: mulivariable Cox regression analysis for only variables derived from IPAT region.

Supplementary figures

Fig. S1 Correlation matrix of clinico-pathological variables and radiomic features included in the final model. No selected radiomic features demonstrate significant correlation with the clinico-pathological variables. Abbreviations: X1: exponential_gldm_DependenceNonUniformity from IAT region; X2: lbp_3D_k_glrlm_LongRunHighGrayLevelEmphasis from IPAT region; X3: lbp_3D_m1_glrlm_LongRunLowGrayLevelEmphasis from IAT region; X4: original_shape_Sphericity from IPAT region.

Fig. S2 Calibration plots of the nomogram for predicting RFS rate in the training set (A) and in the validation set (B). 1-year RFS rate by red calibration curve, 2-year RFS rate by blue calibration curve, 3-year RFS rate by green calibration curve.