Supplementary information for:

Environmental zoning based on the distribution of potentially toxic elements, risk contamination indices, and bioindicators in three areas affected by gold mining in Antioquia, Colombia

Julián E. López a*, Juan F. Marín a, Juan F. Saldarriaga b

a Faculty of Architecture and Engineering, Environmental Engineering Program, Colegio Mayor de Antioquia, Carrera 78 # 65 – 46, 050034, Medellín, Colombia.

b Department of Civil and Environmental Engineering, Universidad de los Andes, Carrera 1Este #19A-40, 111711, Bogotá, Colombia.

*Corresponding author:

Julián E. López

E-mail: julian.lopez@colmayor.edu.co

Fig. S1 Study areas. Colombia (Green). Antioquia (Gray). Buriticá (Red). YalÍ (Yellow). Puerto Berrío (Blue).

Supplementary description S1

Buriticá: municipality located on the eastern margin of the western mountain range. The gold mineralization is found within the so-called Buriticá Tonalite associated with the Sabanalarga Batholith of reported Mesozoic age. This unit forms N-S elongated bodies across the Cauca River. The structural control is N-S associated with the Cauca and Tonusco faults.

Puerto Berrio: located in the lower part of the eastern slope of the central mountain range of Colombia. Three lithological units can be distinguished: Neis feldspathic, the Segovia batholith and the East Segovia sedimentite of Mesozoic age. Structurally, the gold mining area is affected by the Otú, Palestina, Cimitarra and El Bagre faults, locally in the Minas del Vapor sector the faults have a predominant NW trend; the mineralization of vetiform orogenic gold type.

Yalí: located on the eastern slope of the central mountain range. The gold mineralization is associated with the geological unit known as Batolito Antioqueño with variable composition of Tonalite, Granodiorite and Diorite, all Mesozoic age. This geological body has structural controls to the east by the Palestina fault system and to the west by the Romeral fault system. Gold mineralization is in the form of veinlets.

Supplementary description S2

Four certified synthetic solutions for trace elements ((Millipore Sigma) were used for ICP-OES calibration. Calibration curve was accepted at R2 > 0.9996. The LOD (mg L-1) for Pb, As, Cr, and Cd were 0.089, 0.028, 0.041, and 0.007, respectively. The LOQ (mg L-1) for Pb, As, Cr, and Cd were 0.015, 0.09, 0.013, and 0.003, respectively.  Certificated reference materials were included in each digestion/extraction batch for quality assurance. For mining tailings samples the recoveries of metals calculated relative to the certified concentration ranged between 90 and 103%. Duplicate samples were included to evaluate reproducibility. The coefficients of variation for each set of duplicate reference samples ranges from 0.8 to 9.8% (average 5.3%). Certified synthetic solutions for ICP-OES and blank were included in each batch for quality assurance. All reagents were of analytical grade, glassware was soaked in acid (3% HNO3) bath overnight prior use.

Table S1. Background values in the soil.

|  | As | Cd | Cr | Pb |
| --- | --- | --- | --- | --- |
| Background level |  |  |  |  |
| Soils | 6.0 | 0.35 | 70 | 35 |
| Soil forming rocks | 2.0 | 0.15 | 20.0 | 18.0 |

(Sparks et al. 2013; Gallego and Olivero-Verbel 2021).

Table S2. Soil clean-up standards (SCSs) for residential and industrial land use (Provoost et al. 2006).

|  | Potential toxic elements |  |  |  |
| --- | --- | --- | --- | --- |
| Standards | As | Cd | Cr | Pb |
| SCSs for residential land use (mg/kg) | 34 | 15 | 10175 | 419 |
| SCSs for industrial land use (mg/kg) | 173 | 254 | 14298 | 1150 |

Table S3. Calculated values for Pollution Index (PI) and Pollution Load Index (PLI) associated with concentrations of As, Cd, Pb, and Cr for the three municipalities under study.

| Area | Sampling points | PI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | As | Cd | Pb | Cr |
| Yalí | 1 | 2,0 | 4,0 | 0,5 | 0,3 |
| Yalí | 2 | 1,7 | 2,9 | 0,6 | 0,3 |
| Yalí | 3 | 1,8 | 4,0 | 0,5 | 0,3 |
| Yalí | 4 | 1,7 | 2,9 | 0,6 | 0,3 |
| Yalí | 5 | 2,0 | 8,9 | 0,6 | 0,3 |
| Yalí | 6 | 4,6 | 13,2 | 3,1 | 1,6 |
| Yalí | 7 | 5,9 | 11,7 | 2,6 | 1,3 |
| Yalí | 8 | 5,4 | 12,3 | 2,9 | 1,4 |
| Yalí | 9 | 5,0 | 8,8 | 3,4 | 1,7 |
| Yalí | 10 | 2,0 | 3,1 | 0,6 | 0,3 |
| Yalí | 11 | 1,9 | 0,1 | 0,4 | 0,2 |
| Yalí | 12 | 1,8 | 5,7 | 0,6 | 0,3 |
| Yalí | 13 | 1,6 | 0,1 | 0,3 | 0,2 |
| Yalí | 14 | 2,2 | 5,7 | 0,5 | 0,3 |
| Yalí | 15 | 1,7 | 5,7 | 0,6 | 0,3 |
| Yalí | 16 | 1,4 | 0,1 | 0,3 | 0,1 |
| Yalí | 17 | 2,0 | 2,9 | 0,5 | 0,3 |
| Yalí | 18 | 2,2 | 3,7 | 0,5 | 0,2 |
| Yalí | 19 | 1,5 | 0,3 | 0,2 | 0,1 |
| Yalí | 20 | 1,7 | 0,5 | 0,2 | 0,1 |
| Yalí | 21 | 1,7 | 0,1 | 0,3 | 0,1 |
| Puerto Berrío | 1 | 148,7 | 186,3 | 8,0 | 4,0 |
| Puerto Berrío | 2 | 76,5 | 58,9 | 7,7 | 3,8 |
| Puerto Berrío | 3 | 50,3 | 125,1 | 7,0 | 3,5 |
| Puerto Berrío | 4 | 18,7 | 159,4 | 7,9 | 4,0 |
| Puerto Berrío | 5 | 83,0 | 67,7 | 8,6 | 4,3 |
| Puerto Berrío | 6 | 39,0 | 53,4 | 5,1 | 2,5 |
| Puerto Berrío | 7 | 25,8 | 68,3 | 6,0 | 3,0 |
| Puerto Berrío | 8 | 31,2 | 128,9 | 6,8 | 3,4 |
| Puerto Berrío | 9 | 6,0 | 48,9 | 3,7 | 1,9 |
| Puerto Berrío | 10 | 1,0 | 3,1 | 2,8 | 1,4 |
| Puerto Berrío | 11 | 16,4 | 6,6 | 8,5 | 4,3 |
| Puerto Berrío | 12 | 16,4 | 39,7 | 3,1 | 1,6 |
| Puerto Berrío | 13 | 1,0 | 27,7 | 2,2 | 1,1 |
| Puerto Berrío | 14 | 2,3 | 1,4 | 2,8 | 1,4 |
| Puerto Berrío | 15 | 0,3 | 0,3 | 2,0 | 1,0 |
| Puerto Berrío | 16 | 1,8 | 4,6 | 2,6 | 1,3 |
| Puerto Berrío | 17 | 7,7 | 1,7 | 1,9 | 0,9 |
| Puerto Berrío | 18 | 7,7 | 0,0 | 2,2 | 1,1 |
| Puerto Berrío | 19 | 17,6 | 14,9 | 2,9 | 1,5 |
| Puerto Berrío | 20 | 50,5 | 157,7 | 4,5 | 2,2 |
| Puerto Berrío | 21 | 3,1 | 0,1 | 2,9 | 1,4 |
| Puerto Berrío | 22 | 1,7 | 5,1 | 2,5 | 1,3 |
| Puerto Berrío | 23 | 0,6 | 0,6 | 2,0 | 1,0 |
| Buriticá | 1 | 5,8 | 40,6 | 2,1 | 1,0 |
| Buriticá | 2 | 4,0 | 15,4 | 0,6 | 0,3 |
| Buriticá | 3 | 8,3 | 29,7 | 2,5 | 1,3 |
| Buriticá | 4 | 6,6 | 45,4 | 2,0 | 1,0 |
| Buriticá | 5 | 4,8 | 9,1 | 1,0 | 0,5 |
| Buriticá | 6 | 7,4 | 52,0 | 2,3 | 1,2 |
| Buriticá | 7 | 1,8 | 1,4 | 0,3 | 0,1 |
| Buriticá | 8 | 3,1 | 12,9 | 1,2 | 0,6 |
| Buriticá | 9 | 5,5 | 27,4 | 2,3 | 1,1 |
| Buriticá | 10 | 3,8 | 17,4 | 1,4 | 0,7 |
| Buriticá | 11 | 1,3 | 9,7 | 0,2 | 0,1 |
| Buriticá | 12 | 6,8 | 23,4 | 2,9 | 1,5 |
| Buriticá | 13 | 3,6 | 12,6 | 0,6 | 0,3 |
| Buriticá | 14 | 3,3 | 11,1 | 0,9 | 0,5 |
| Buriticá | 15 | 1,0 | 7,1 | 0,4 | 0,2 |
| Buriticá | 16 | 1,8 | 15,7 | 0,6 | 0,3 |
| Buriticá | 17 | 5,6 | 42,6 | 2,2 | 1,1 |
| Buriticá | 18 | 1,1 | 10,0 | 0,4 | 0,2 |
| Buriticá | 19 | 3,8 | 9,1 | 1,0 | 0,5 |

Table S4. Results for biomonitoring.

| Area | Sampling points | Germination percentage (%G) | Radicle Length (mm) |
| --- | --- | --- | --- |
| Yalí | 1 | 60 | 1,4 |
| Yalí | 2 | 70 | 2,1 |
| Yalí | 3 | 70 | 1,8 |
| Yalí | 4 | 60 | 1,5 |
| Yalí | 5 | 70 | 2,1 |
| Yalí | 6 | 60 | 1,8 |
| Yalí | 7 | 60 | 1,5 |
| Yalí | 8 | 50 | 1,3 |
| Yalí | 9 | 60 | 1,9 |
| Yalí | 10 | 60 | 2,1 |
| Yalí | 11 | 80 | 3,5 |
| Yalí | 12 | 80 | 2,1 |
| Yalí | 13 | 80 | 2,8 |
| Yalí | 14 | 80 | 3,2 |
| Yalí | 15 | 70 | 2,5 |
| Yalí | 16 | 80 | 3,1 |
| Yalí | 17 | 70 | 2,7 |
| Yalí | 18 | 60 | 1,3 |
| Yalí | 19 | 70 | 1,6 |
| Yalí | 20 | 80 | 2,9 |
| Yalí | 21 | 80 | 3,1 |
| Puerto Berrío | 1 | 30 | 0,8 |
| Puerto Berrío | 2 | 50 | 1,6 |
| Puerto Berrío | 3 | 50 | 1,9 |
| Puerto Berrío | 4 | 30 | 1,2 |
| Puerto Berrío | 5 | 50 | 2,1 |
| Puerto Berrío | 6 | 60 | 2,4 |
| Puerto Berrío | 7 | 60 | 1,9 |
| Puerto Berrío | 8 | 30 | 1,3 |
| Puerto Berrío | 9 | 50 | 2,1 |
| Puerto Berrío | 10 | 70 | 2,5 |
| Puerto Berrío | 11 | 70 | 2,8 |
| Puerto Berrío | 12 | 70 | 2,3 |
| Puerto Berrío | 13 | 50 | 1,9 |
| Puerto Berrío | 14 | 70 | 2,2 |
| Puerto Berrío | 15 | 70 | 2,4 |
| Puerto Berrío | 16 | 70 | 2,5 |
| Puerto Berrío | 17 | 70 | 2,0 |
| Puerto Berrío | 18 | 70 | 2,1 |
| Puerto Berrío | 19 | 50 | 1,6 |
| Puerto Berrío | 20 | 50 | 1,8 |
| Puerto Berrío | 21 | 70 | 2,9 |
| Puerto Berrío | 22 | 70 | 2,5 |
| Puerto Berrío | 23 | 70 | 2,8 |
| Buriticá | 1 | 50 | 1,1 |
| Buriticá | 2 | 60 | 2,4 |
| Buriticá | 3 | 40 | 2,5 |
| Buriticá | 4 | 50 | 1,2 |
| Buriticá | 5 | 60 | 2,8 |
| Buriticá | 6 | 40 | 2,3 |
| Buriticá | 7 | 60 | 2,3 |
| Buriticá | 8 | 60 | 2,4 |
| Buriticá | 9 | 50 | 1,3 |
| Buriticá | 10 | 70 | 2,5 |
| Buriticá | 11 | 60 | 2,4 |
| Buriticá | 12 | 60 | 2,1 |
| Buriticá | 13 | 60 | 2,3 |
| Buriticá | 14 | 60 | 2,5 |
| Buriticá | 15 | 80 | 2,6 |
| Buriticá | 16 | 70 | 2,2 |
| Buriticá | 17 | 40 | 2,1 |
| Buriticá | 18 | 70 | 2,6 |
| Buriticá | 19 | 70 | 2,3 |

References

Gallego JL, Olivero-Verbel J (2021) Cytogenetic toxicity from pesticide and trace element mixtures in soils used for conventional and organic crops of Allium cepa L. Environ Pollut 276:116558. https://doi.org/10.1016/j.envpol.2021.116558

Provoost J, Cornelis C, Swartjes F (2006) Comparison of soil clean-up standards for trace elements between countries: Why do they differ? J Soils Sediments 6:173–181. https://doi.org/10.1065/jss2006.07.169

Sparks DL, (2013) Environmental Soil Chemistry, 2nd ed, Journal of Chemical Information and Modeling. Academic Press, San Diego, California. https://doi.org/10.1017/CBO9781107415324.004