Supplementary Note 1

Differential Dynamic Microscopy

The principles of differential dynamic microscopy (DDM) and its application to measuring bacterial swimming behavior have been already described in detail1-4. Thus, we only briefly summarize the main points here.

The method exploits the rate of changes in pixel intensity in an image being proportional to the speed of motion of the object to measure it. The analysis is carried out in spatial Fourier space because the structure of the information is more easily exploitable in this format. We can write the pixel intensity of a movie frame featuring N moving bacteria in a static background Ibg as:

Ix,t=Ibgx+i=0NIpzit,x-xit+ϵx,t#1.1

Where xit is the 2D position and zit the position relative to the focal plane of bacterium i, ϵx,t is the camera shot noise, and Ip is the pixelated point spread function of the bacterium, that depends on the optics of the microscope and the camera and is centered around the position of the bacterium xit. DDM first computes the spatial Fourier transform of this intensity:

Iq,t=dx e-q⋅x Ix,t =Ibgq+ϵq,t+i=0Ne-q⋅xit Ipzit,q#1.2

And then focusses on the differential intensity correlation function (DCIF):

Dq,dt=Iq,t+dt-Iq,t2 #1.3

Under the assumption that Ipzit,q evolves much more slowly than e-q⋅xit, which has been verified for the low magnification optics we use and in the range of wave numbers q we consider2,3. In the case of a dilute system where the motion of different bacteria can be seen as independent from each other, we can write the DCIF as1,2:

Dq,dt=2NIpq2Sq1-fq,dt+ϵq2#1.4

With the structure factor Sq=e-q⋅xi-xj i,j that quantifies the spatial structure of the cell suspension, Ipq averaging over visible cells at different z, and the intermediate scattering function (ISF):

fq,dt=e-q⋅Δxdt= ∫dx e-q⋅x pxdt #1.5

Which is the Fourier transform of the probability of displacements pxdt of the bacteria during a time step dt, and thus fully characterizes the dynamics of the system. The functional form of fq,dt is known for various types of motion, including swimming cells and diffusing non-motile cells.

Since our sample contains both motile and non-motile cells, we fit the experimentally determined DCIF with the following model2:

Dq,dt=Aq 1-fpsq,dt+Bq#1.6

With the intermediate scattering function accounting for only a fraction ϕM of cells being motile:

fpsq,dt=ϕMfMq,dt+1-ϕMfBq,dt#1.7

The Brownian ISF accounts for the Brownian motion of the cells (primarily the non-motile ones):

fBq,dt=exp-D0q2dt#1.8

With D0 their diffusion coefficient. This function decays from 1 to 0 in a typical time dt∼1/D0q2.

The ISF of the motile cells has the form:

fMq,dt=gqv0dt σvv0#1.9

With v0 the average swimming speed of the population and σv its variance. In the limit σv=0, the function gx is a cardinal sine (sinxx). In Wilson et al (2011)2, it was computed analytically in the case of Schultz distributed velocities, a model that proved proficient:

gx | y=Z+1ZxsinZtan-1xZ+1  1+xZ+12Z2#1.10

With Z=y2-1y2. In all cases, the function decays from 1 to 0 on a typical time dt∼1/qv0.

In practice, the DCIF Dq,dt, which is computed over the accessible range of q given our pixel size and the width of our camera field of view, displays two distinct increases as a function of dt (Extended Data Fig. 2a). These are respectively due to the motile cells (corresponding to the 1/qv0 decay timescale of fps) and the non-motile ones (1/D0q2 timescale). The relative amplitudes of the two increases indicate the fraction of cells that are swimming (Extended Data Fig. 2a). Since the time scales have different dependences in q, we independently fit Dq,dt as a function of dt by the model (Eqs. 1.6-1.10) for different q, and verify the consistency of the fit parameters as a function of q (Extended Data Fig. 2b). We thus are able to validate the model and in particular the motile and Brownian nature of the motion of the two subpopulations. For very small and very large q, the fits fail, because the correlation function does not fully decorrelate over the duration of the experiment (small q) or because the signal over noise ratio is too small (large q). We thus reject the resulting very noisy values for the fitted parameters. We then use the average value of the parameters over the valid q range as the measured values for the given experiment, while the standard deviation measures the accuracy of our estimation of said parameters.

Dark Field Flicker Microscopy

Dark field flicker microscopy (DFFM) is a recently developed optical microscopy method, which exploits the specificity of dark field illumination to measure the rotation speed of the cell body and the flagellum5,6. Under dark field illumination, the objective collects only the light that is scattered by the microscopic objects that are in focus (Extended Data Fig. 5b). In the regime of Mie scattering in which bacteria lie, the directions in which an anisotropic object (like the cell body or the flagellum) scatters incident light depends on the angle between the object main axis and the incident light. The rotation of this axis thus modifies the main scattering directions and therefore the amount of light collected by the objective: the image of the object flickers at a frequency that is equal to the rotation frequency of the object. This flickering can thus be exploited to measure said frequency.

In practice, a bacterial suspension prepared as for DDM (OD600 ~ 0.1-0.2) is observed at 10x magnification (NA=0.3) under dark field illumination, and a 512 x 512 px2 (1 px = 0.7 µm) field of view is recorded at 800 frames/s during 104 frames with an EoSens 4CPX CMOS camera, far from the sample surfaces. The movie is divided in independent 8x8 px2 submovies, in which there is on average no more than one cell at a time. The temporal power spectrum of the average intensity of the submovies is computed as

Sω=Ikω2k#1.11

With Ikω the Fourier transform of the average intensity of submovie k:

Ikω=∫dt e-iωtIx,tx∈Vk#1.12

The power spectrum is then corrected for the effects of the cells Brownian motion by computing:

Eω=ω2Sω#1.13

For swimming E. coli, the corrected power spectrum Eω displays three characteristic peaks above a flat background (Extended Data Fig. 5 c). The first one, at a frequency of about 1 Hz corresponds to cells moving in and out of the submovie boxes. The second one, at about 20-40 Hz corresponds to the rotation rate of the cell body Ωb, while the third, at about 200 Hz, corresponds to the rotation rate of the flagellum. The location of the maxima of the peaks is estimated via local Gaussian fits and used as a measure of the average rotation speeds within the sample.

Supplementary Note 2

Detailed description of RFT model

The model extends on classical force balance analysis of mono-flagellated propulsion 7,8, and accounts for our measurements of swimming speeds as well as cell body and flagellar rotation frequencies. A schematic of the model for flagellar rotation is provided in Extended Data Fig. 5a. We assume that the N flagella of the cell bundle together tightly, and that the N motors are aligned along the axis of the bundle. Hence, the bundle is described as a helix of increased thickness rf=r N12. This assumption is based on previous experiments and simulations showing that the fluid flow generated by two helices is equivalent to the fluid flow generated by one helix with a thicker radius 9, and that the thrust generated by a bundle of helices doesn’t depend much on the bundle configuration 10. We also account for the increased length of the flagella at higher induction of flhDC.

Since no net torque or force are applied on the system, the friction forces and torques applying on the flagellar bundle Ff,Γf and the cell body Fb, Γb equilibrate in the low Reynolds number fluid:

Ff+Fb=0Γf+Γb=0 #2.1

The friction forces and torques on the flagellar bundle, modelled as a helix, relate to its rotation speed Ωf and the free-swimming speed U of the cell via

FfΓf=μTTf-μTRf-μTRfμRRfUΩf #2.2

Where μXf are the friction coefficient for translation (X=TT), rotation (X=RR) and rotation-translation coupling resulting from the flagellum being chiral (X=TR). We model the cell body as an (achiral) rod that wobbles. The forces and torques exerted on it relate to body rotation speed Ωb and swimming speed U via

FbΓb=μT,b UμR,b Ωb#2.3

Where μT,b and μR,b are translational and rotational effective friction coefficients. The expressions of the coefficients as a function of the bundle and body geometric parameters are determined below (Eqs. 2.12-2.17 for the flagellum, and 2.27-2.28 and 2.30 for the cell body).

We assume that the motors all rotate at the same speed Ωm, which we assume constant based on our observations (i.e., presumably at the maximum rotation speed):

Ωf-Ωb=Ωm=ΩMax#2.4

The set of equations (2.1-2.4) solves readily and provides relations between the swimming speed and the motor rotation speed:

U=ZU Ωm#2.5

The proportionality coefficient ZU depends on the number of flagella N and their length and reads:

ZU=μTRfμt,∥+μTTf11+μR,redfμR,∥#2.6

With the reduced flagellar rotational friction coefficient μR,redf=μRRf-μTRf2μT,∥+μTTf#2.7

Additionally, we obtain the following expressions for the swimming speed and the cell body rotation speed relative to the flagellar rotation speed:

UΩf=μTRfμT,b+μTTf#2.8

ΩbΩf=μR,redfμR,b#2.9

These ratios are plotted in Extended Data Fig. 5d, e for our experimental measurements compared with the resistive force theory (RFT) predictions, which account for both the length and number of flagella increasing (see below). The agreement is overall excellent. Consequently, the RFT predicts very well the swimming speed for the different strains (Extended Data Fig. 5f).

The torque generated by the motors relate to flagellar and cell body torques as:

Γf=Γb=NΓm#2.10

Which, combined with Eqs. (2.1-2.3), yields:

Γm=1N μR,redf1+μR,redfμR,b Ωm#2.11

The predicted torque generated by each motor thus decreases as a function of N (Extended Data Fig. 5g), and never exceeds 700 pN.nm, far from the ~2500 pN.nm maximal torque that the motor can generate, which might explain why we observe a constant, presumably maximal, motor rotation speed for all strains.

Our RFT model overestimates by about 10% the experimental value of UΩf and thus of U for the strain with the largest number of flagella, and thus do not capture the reduction of speed there. This discrepancy is not surprising since our model becomes less realistic with increasing flagellar number, and a number of factors, including hydrodynamic effects and tangling or steric effects within the bundle could increase drag (μTTf) and/or reduce thrust (μTRf) compared to RFT prediction.

Expression of friction coefficients

Flagellum

The friction coefficients of the flagellum modeled as a helix are 8,11-13:

μTTf= Kn lsinψ tanψ +γktanψ#2.12

μTRf = Kn l Rsinψ  1-γk#2.13

μRRf = Kn l R2sinψ  1tanψ +γktanψ#2.14

With the coefficients

Kn=4πηlncλrf+0.5#2.15

γk=0.7#2.16

tanψ=2πRλ#2.17

where the helix parameters are as indicated on Extended Data Fig. 5a, and the medium viscosity is η. The length of the helix  l varied between 5 and 8.5 µm, according to measured values in the different strains (Fig. 2), which were least-square fitted as l = 5.3983 + 3.6564 log10N to quantify the dependence of flagellar length on flagellar number. We fixed the helix wavelength λ=2.3 µm, radius R = 0.2 µm, and flagellar filament thickness r =0.02 µm according to previous measurements 14,15. The bundle thickness is  rf=r N12 to account for the increased cross-section of the bundle. The coefficient c in Eq. (2.15) takes various values through the literature 8,13, from c=2 in Gray and Hancock’s work11 to c=0.18 in Lighthill’s work12. We chose c=0.25, which is close to Lighthill’s prediction accounting for some of the hydrodynamic coupling throughout the helix 12.

Cell body

We model the cell body as a rod of length L=2.5 µm, diameter d=0.75 µm, aspect ratio p = L/d, consistently with the typical values for our strain. The rod is inclined with an angle θ relative to the axis of propulsion (uz) and it rotates about this axis at frequency φ=Ωb, which we can measure with DFFM. The cell body also rotate at a frequency α about its own axis (ur). Since this rotation does not affect the scattering of light by the cell body, α is not detectable by DFFM. The pulsation vector accounting for these two rotations is:

ω=sinθφ uz+α ur#2.18

Projecting this vector on the axes parallel (ur) and perpendicular (uθ) to the rod, we can compute the components of the torque acting on the rod:

Γr=ζ∥Rω⋅ur=ζ∥Rsinθcosθφ+α#2.19

Γθ=ζ⊥Rω⋅uθ=ζ⊥R-sinθ2φ#2.20

The rotational friction coefficients about both axes read 16,17,:

ζ⊥R=π3ηL3lnp-0.662+0.917p-0.050p2+o1p2#2.21

ζ∥R=3.841π4ηLd21+1.119×10-4+0.6884p+0.2019p2+o1p2#2.22

The torques perpendicular and parallel to the axis of propulsion of the cell are then:

Γz=Γr ur⋅uz+Γθ uθ⋅uz=ζ⊥Rsinθ3φ+ζ∥Rsinθcosθ2φ+αcosθ#2.23

Γφ=-ζ⊥R-ζ∥Rsinθ2cosθφ+ζ∥Rsinθα#2.24

While the torque along z-axis is balanced by the flagellar torque, the perpendicular torque must be zero (Γφ=0), which yields, assuming sinθ≠0:

ζ∥Rα=ζ⊥R-ζ∥Rsinθcosθφ#2.25

Substituting in Eq. (2.23), we obtain:

Γz=ζ⊥Rsinθφ#2.26

Hence the effective rotational friction coefficient is:

μR,b=ζ⊥Rsinθ#2.27

Furthermore, we assume that the rotation of the cell body is about a fixed anchor point, which constrains α=φ, and gives the value of the angle θ from Eq. (2.25):

sin2θ=2ζ∥Rζ⊥R-ζ∥R#2.28

For our parameter values, we obtain θ=21°, which is consistent with our microscopic observations of cell swimming at high magnification.

Similarly, the friction force resulting from translation at speed U along the z-axis is given by:

Fz=F⋅uz=uz⋅ζ∥Turur+ζ⊥Tuθuθ⋅U uz=ζ∥T+ζ⊥T-ζ∥Tsinθ2 U#2.29

Hence the effective translation friction coefficient of the cell body is:

μT,b=ζ∥T+ζ⊥T-ζ∥Tsinθ2#2.30

with the translational friction coefficients in the direction parallel and perpendicular to the rod 17:

ζ∥T=2πηLlnp-0.207+0.980p-0.133p2+o1p2#2.31

ζ⊥T=4πηLlnp+0.839+0.185p+0.233p2+o1p2#2.32

The friction force also has a component perpendicular to the axis of motion, in direction uφ:

Fφ=F⋅uφ=uφ⋅ζ∥Turur+ζ⊥Tuθuθ⋅U uz =ζ⊥T-ζ∥Tsinθcosθ U#2.33

Which however averages out over one period of cell body rotation, since uφ rotates. Moreover, in our case, it is negligible compared to the z-component because ζ⊥T is only 20% larger than ζ∥T, and sinθcosθ≃0.33.

Supplementary references

1.	Cerbino, R. & Trappe, V. Differential Dynamic Microscopy: Probing Wave Vector Dependent Dynamics with a Microscope. Phys. Rev. Lett. 100, 188102 (2008).

2.	Wilson, L.G. et al. Differential Dynamic Microscopy of Bacterial Motility. Phys. Rev. Lett. 106, 018101 (2011).

3.	Martinez, Vincent A. et al. Differential Dynamic Microscopy: A High-Throughput Method for Characterizing the Motility of Microorganisms. Biophys. J. 103, 1637-1647 (2012).

4.	Cerbino, R. & Cicuta, P. Perspective: Differential dynamic microscopy extracts multi-scale activity in complex fluids and biological systems. J. Chem. Phys. 147, 110901 (2017).

5.	Martinez, V.A. et al. Flagellated bacterial motility in polymer solutions. Proc. Natl. Acad. Sci. USA 111, 17771-17776 (2014).

6.	Cerbino, R., Piotti, D., Buscaglia, M. & Giavazzi, F. Dark field differential dynamic microscopy enables accurate characterization of the roto-translational dynamics of bacteria and colloidal clusters. J. Phys. Condens. Matter 30, 025901 (2018).

7.	Purcell, E.M. The efficiency of propulsion by a rotating flagellum. Proc. Natl. Acad. Sci. USA 94, 11307-11311 (1997).

8.	Chattopadhyay, S., Moldovan, R., Yeung, C. & Wu, X.L. Swimming efficiency of bacterium Escherichia coli. Proc. Natl. Acad. Sci. USA 103, 13712-13717 (2006).

9.	Kim, M.J. et al. Particle image velocimetry experiments on a macro-scale model for bacterial flagellar bundling. Exp. Fluids 37, 782-788 (2004).

10.	Danis, U. et al. Thrust and Hydrodynamic Efficiency of the Bundled Flagella. Micromachines 10, 449 (2019).

11.	Gray, J. & Hancock, G.J. The Propulsion of Sea-Urchin Spermatozoa. J.Exp. Biol. 32, 802-814 (1955).

12.	Lighthill, J. Flagellar Hydrodynamics. SIAM Review 18, 161-230 (1976).

13.	Johnson, R.E. & Brokaw, C.J. Flagellar hydrodynamics. A comparison between resistive-force theory and slender-body theory. Biophys. J. 25, 113-127 (1979).

14.	Morgan, D.G., Owen, C., Melanson, L.A. & DeRosier, D.J. Structure of Bacterial Flagellar Filaments at 11 Å Resolution: Packing of the α-Helices. J. Mol. Biol. 249, 88-110 (1995).

15.	Turner, L., Ryu, W.S. & Berg, H.C. Real-Time Imaging of Fluorescent Flagellar Filaments. J. Bacteriol. 182, 2793-2801 (2000).

16.	Tirado, M.M. & de la Torre, J.G. Rotational dynamics of rigid, symmetric top macromolecules. Application to circular cylinders. J. Chem. Phys. 73, 1986-1993 (1980).

17.	Tirado, M.M., Martínez, C.L. & de la Torre, J.G. Comparison of theories for the translational and rotational diffusion coefficients of rod‐like macromolecules. Application to short DNA fragments. J. Chem. Phys. 81, 2047-2052 (1984).