Supplementary Information for:

Faricimab maintains substance integrity and sterility after compounding and storage in two different polypropylene syringe types

Alexander Taschauer1,2, Arthur Sedivy3, Daniel Egger4, Reinhard Angermann4, Haider Sami5, Claudia Wunder1,2, Sebastian M. Waldstein4*

1 Pharmacy of the University Hospital St. Pölten, Dunant-Platz 1, 3100, St. Pölten, Austria

2 Karl Landsteiner University of Health Sciences, Dr. Karl-Dorrek-Strasse 30, 3500, Krems, Austria

3 Protein Technologies, Vienna Biocenter Core Facilities GmbH, Dr. Bohr-Gasse 3, 1030 Vienna, Austria

4 Department of Ophthalmology, Landesklinikum Mistelbach-Gänserndorf, Liechtensteinstrasse 67, Mistelbach, Austria

5 University of Vienna, Faculty of Life Sciences, Department of Pharmaceutical Sciences, Laboratory of Macromolecular Cancer Therapeutics (MMCT), Josef-Holaubek-Platz 2, 1090 Vienna, Austria

* Corresponding authors

Email:

sebastian.waldstein@mistelbach.lknoe.at (S. Waldstein)

Supplementary Methods

Reagents

Both VEGF (sequence: Ala 27 – Arg 191; Cat.Nr.: VE5-H4210) and Ang-2 (equipped with a C-terminal polyhistidine tag; sequence: Tyr 19 – Phe 496; Cat.Nr.: AN2-H52H4) were purchased from AcroBiosystems. All further compounds used for reagent and buffer preparation were obtained from VWR Chemicals and Merck.

Sample preparation

Compounding was conducted avoiding the introduction of air bubbles. In case of BD Microfine®, the distribution of faricimab from the vial into the syringes was done with a 1mL BD Plastipak® syringe as reservoir. The formulation was drawn up through the cone of the 1mL syringe into BD Microfine® syringes. The compounding process into ZeroResidual® syringes was conducted with a Luer-Lock bubble adapter provided by the manufacturer. ZeroResidual® syringes were closed with a 30G Luer-Lock cannula from SJJ solutions. Both BD Microfine® and ZeroResidual® syringes were equipped with a plunger protection to avoid unintended ejection of the content before sealing in sterile bags.

Tertiary structure analysis

Measurements were conducted with a 1mg/mL dilution of faricimab in 20mM potassium phosphate buffer (pH 5.5; 0.2µm filtered). The second derivative of the obtained spectrum was calculated with the LabSolutions UV-Vis software with a smoothing level of X20 and scaling factor 1. Spectra were compared to day 0 samples based on the characteristic wavelength of the absorption minima of phenylalanine (253nm, 258nm), tyrosine (275nm, 284nm) and tryptophan (292nm).

Size exclusion chromatography (SEC)

The OmniSEC system (Malvern) was equipped with detectors for refractive index, right-angle light scattering, low-angle light scattering, UV-Vis absorbance and intrinsic viscosity. The Superdex 200 10/300 column had a column volume of 24mL and a void volume of 8mL. The column oven was set to 35°C. The duration of each run was 55 minutes. The mobile phase (PBS) was composed of 136.9mM NaCl, 2.7mM KCl, 8.1mM Na2HPO4 and 1.5mM KH2PO4 dissolved in MilliQ water and set to a pH of 7.3. Bovine serum albumin was used as calibrant and for monitoring variations within the system. Samples were diluted to 1mg/mL with 20mM potassium phosphate buffer (pH 5.5; 0.2µm filtered), filtered through 0.45µm Ultrafree®-MC-HV Durapore® PVDF centrifugation filters and kept in the autosampler at 6°C until injection. For each measurement run, 100µL (100µg faricimab) were injected.

Nano Differential Scanning Fluorimetry (nanoDSF)

Samples were diluted to 1mg/mL with 20mM potassium phosphate buffer (pH 5.5; 0.2µm filtered) and filtered through 0.45µm Ultrafree®-MC-HV Durapore® PVDF centrifugation filters. Unfolding and aggregation profiles were measured between 20°C and 95°C and a temperature ramp of 1°C per minute.

Turbidity analysis

Samples were diluted to 1mg/mL in 20mM potassium phosphate buffer (pH 5.5; 0.2µm filtered). Absorption was measured at 280nm and 350nm.

Hydrodynamic diameter of antibody-derived nanoparticles

Faricimab was diluted to 3mg/mL with 20mM potassium phosphate buffer (pH 5.5; 0.2µm filtered) by slow pipetting avoiding introduction of air bubbles. For each dilution 6 consecutive measurements were conducted.

Specific binding of VEGF and Ang-2

Both antigens (VEGF and Ang-2) were reconstituted in sterile MilliQ water to 0.6mg/mL and filtered through 0.45µm Ultrafree®-MC-HV Durapore® PVDF centrifugation filters. For the binding experiment, the stock solutions were further diluted in HBS-T running buffer (50mM HEPES, 150mM NaCl, 0.005% Tween-20, pH 7.5) to 200nM for VEGF and 100nM for Ang-2. Faricimab samples were diluted in HBS-T buffer to 25nM. The flow rate was set to 7.5µL/min. The 4PCP Creoptix chip was pre-conditioned with borate buffer (pH 9.0; 0.1M sodium tetraborate decahydrate; 1M NaCl) and 0.2X PBS. The anti-human IgG (Fc) antibody was immobilized by EDC/NHS chemistry using an antibody capture kit (Cytiva BR100839) reaching immobilization levels of around 4000pg/mm2. After faricimab injection, a buffer-startup was conducted to wash away any loosely bound faricimab. Before antigen injection a blank measurement was done. The surface mass at the end of the blank measurement was used as reference level to quantify the binding shifts in the sensorgram after antigen injection. Chip regeneration (decoupling of faricimab together with bound antigens from the chip) after every experiment round was done with 3M MgCl2 (pH 2.1).

Three experiments were conducted with faricimab stored in BD Microfine® and ZeroResidual® syringes, whereas in each experiment round the samples for channel 2, 3 and 4 were rotated to exclude negative interference with the results because of channel dependent variations. As 3 replicates of day 0 samples were always included in the measurement cycles together with faricimab samples in BD Microfine® and ZeroResidual® in total 6 replicates of day 0 and 3 replicates of compounded faricimab samples were analyzed.

Bacterial endotoxins

Quantification of bacterial endotoxins was done with an Endosafe® PTS system (Charles River) equipped with limulus amebocyte lysate (LAL) cartridges (Charles River; Cat.No.: PTS2005F; measurement range: 0.05 Endotoxin Units (EU)/mL to 5.0EU/mL) as per Ph.Eur. For sample preparation, only endotoxin-free Eppendorf tubes and pipette tips were used. Only measurements showing a spike recovery between 50% to 200% were accepted.

Sterility

All culture media used for sterility test were prepared in accordance to the manufacturer´s instructions. Samples in tryptic soy broth were incubated between 20°C and 25°C for 14 days and samples in fluid thioglycolate medium between 30°C and 35°C for 14 days. At the end of the incubation period, the verification of suitability was confirmed by bacteriostasis and fungistasis tests.

Supplementary Figures

Supp. Figure 1: Overlay of nanoDSF measurements of faricimab. (a) 350nm/330nm graph as a function of temperature. The vertical dashed lines show the automatically detected onset- and the second inflection point. (b) The first derivative of the 350nm/330nm graph shows two peak maxima derived by the unfolding of the CH2 domain (first transition; 67°C) and unfolding process of the Fab/CH3 domain (second peak; maximum; 73°C). (c) Light scattering graph as a function of temperature to detect aggregation processes. The vertical dashed lines show the automatically detected onset- (73°C) and the inflection point (75°C).

Supp. Figure 2: Representative sensorgram of a GCI measurement circle to evaluate binding efficiency of faricimab to VEGF and Ang-2. The sensorgram of the control channel (channel 1) is depicted as a blue line, whereas the sensorgrams of channel 2, 3 and 4 are depicted as a red, green and yellow line, respectively. VEGF and Ang-2 injection into channel 1 resulted in bulk signal, which went back to the baseline level. Faricimab was immobilized in channel 2 to 4. In those channels specific binding was existent for both antigens.