Appendix E1.  The main parameters for chest CT imaging

All the patients underwent non-enhanced chest CT examinations for detecting pneumonia in the supine position during end-inspiration. The CT scans were performed with a 64-section multi-detector CT scanner (uCT780, United imaging or Somatom Definition Flash, Siemens Healthineers, or Light Speed VCT, GE Healthcare, or Acuilion, Toshiba Healthcare). The detailed imaging parameters for different scanners were demonstrated as follows.

1. For adults:

tube voltage: 120 kVp, automatic tube current: (120-440) mAs, thickness: (5-7) mm, slice interval: 5 mm, rotation speed: 0.5-1.0 s, helical pitch 1.0875:1 or 1.375:1. Lung window images at 0.625 to 1 mm thickness were reconstructed. Iterative reconstruction technique was implemented.

2. For children:

tube voltage: 100 kVp, automatic tube current: (20-100) mAs, thickness: (3-5) mm, slice interval: (1-3) mm, rotation speed: 0.6 s, helical pitch 0·969:1. Lung window images at 0.625 to 1 mm thickness were reconstructed. Iterative reconstruction technique was implemented. The informed consents for CT examination were obtained from all patients.

Appendix E2. Image segmentation and radiomics feature extraction

Three-dimensional (3D) segmentation of the entire volume of interest (VOI) of the pneumonia lesions were performed manually and independently by two experienced radiologists (radiologist 1 [HR] and radiologist 2 [RC], with 5 and 6 years of experience in thoracic imaging, respectively) via a free and widely used open-source software package (itk-SNAP, version 3.8.0, www.itksnap.org). The outline of the lesions was delineated along the border on thick-section images with lung window and excluded the intralesional vessels, bronchi, necrosis, and cavitation. Both of them were blinded to the results of laboratory tests. VOIs with a volume less than 125 mm3 were excluded.

The interobserver and intraobserver reproducibility evaluation of radiomics feature extraction was performed using intraclass correlation coefficients (ICC). Totally 15 VOIs from each group in the training cohort were randomly chosen. The intra-observer ICC was calculated by comparing two segmentations of radiologist 1 (repeated 7-day interval). The inter-observer ICC was calculated by comparing segmentation of radiologist 1 (first time) and radiologist 2. An ICC of 0.81 to 1.00 showed almost perfect agreement, 0.61 to 0.80 as substantial agreement, and 0.41 to 0.60 as moderate agreement.

Radiomics features were extracted from VOIs by using pyradiomics 3.0.0 version (http://www.radiomics.io/pyradiomics.html). Images were preprocessed and processed using the proposed default setting. Six classes of radiomics features were extracted: 18 first order statistics features, 14 shape-based features (3D), 22 gray level cooccurrence matrix (GLCM) features, 16 gray level run length matrix (GLRLM) features, 16 gray level size zone matrix (GLSZM) features, and 14 gray level dependence matrix (GLDM) features. The radiomics feature details were shown in the pyradiomics documentation (https://pyradiomics.readthedocs.io/en/latest /features. html). In addition, two image filters of wavelet and Laplacian of Gaussian were applied to the original image, respectively. Finally, 14 different image types were used for extracting radiomics features.

Appendix E3. Detailed information about the COVID-19 Reporting and Data System (CO-RADS) classification

The CO-RADS included 6 levels of suspicion for pulmonary involvement of COVID-19 besides CO-RADS 0, not interpretable (scan technically insufficient for assigning a score) as follows: CO-RADS 1, very low (normal or non-infectious); CO-RADS 2, low (typical for other infection but not COVID-19); CO-RADS 3, equivocal/unsure (features compatible with COVID-19, but also other diseases); CO-RADS 4, high (suspicious for COVID-19); CO-RADS 5, very high (typical for COVID-19); CO-RADS 6, proven (RT-PCR positive for SARS-CoV-2).

1. CO-RADS 0

CO-RADS 0 is chosen when scans that are incomplete or of insufficient quality, for example because of severe artifacts due to coughing or breathing.

2. CO-RADS 1

CO-RADS 1 implies a very low level of suspicion for pulmonary involvement by COVID-19 based on either a normal CT or CT findings of unequivocal non-infectious etiology. This was modelled on Lung-RADS, where cases with no nodules or with nodules with definitely benign features are reported together. Using the definition, mild or severe emphysema, perifissural nodules, lung tumors, or fibrosis are classified as CO-RADS 1.

3. CO-RADS 2

CO-RADS 2 implies a low level of suspicion for pulmonary involvement by COVID-19 based on CT findings in the lungs that are typical of infectious etiology that are considered not compatible with COVID-19. Examples are bronchitis, infectious bronchiolitis, bronchopneumonia, lobar pneumonia, and pulmonary abscess. Features include tree-in-bud sign, a centrilobular nodular pattern, lobar or segmental consolidation, and lung cavitation.

4. CO-RADS 3

CO-RADS 3 implies equivocal findings for pulmonary involvement of COVID-19 based on CT features that can also be found in other viral pneumonias or non-infectious etiologies. Findings include perihilar ground-glass, homogenous extensive ground glass with or without sparing of some secondary pulmonary lobules, or ground glass together with smooth interlobular septal thickening with or without pleural effusion in absence of other typical CT findings. CO-RADS 3 also includes small ground glass opacities that are not centrilobular (otherwise CO-RADS 2) or not located close to the visceral pleura (otherwise CO-RADS 4). In addition, it contains patterns of consolidation compatible with organizing pneumonia without other typical findings of COVID-19.

5. CO-RADS 4

CO-RADS 4 implies a high level of suspicion for pulmonary involvement by COVID-19 based on CT findings that are typical for COVID-19 but showing some overlap with other (viral) pneumonias. Findings are similar to CO-RADS 5 but are not located in contact with the visceral pleura or are located strictly unilaterally, are in a predominant peribronchovascular distribution, or are superimposed on severe diffuse pre-existing pulmonary abnormalities.

6. CO-RADS 5

CO-RADS 5 implies a very high level of suspicion for pulmonary involvement by COVID-19 based on typical CT findings. Mandatory features are ground-glass opacities, with or without consolidations, in lung regions close to visceral pleural surfaces, including the fissures, and a multifocal bilateral distribution. The vicinity to the minor or major fissure is also typical. Subpleural sparing is allowed to be present. CO-RADS 5 requires the presence of at least one confirmatory pattern, which aligns with the temporal evolution of the disease. The crazy paving pattern, which has been described to appear later in the course of the disease, shows visible intralobular lines. As the disease progresses, increasing consolidations occur within the ground-glass areas. Finally, opacities occur that resemble organizing pneumonia, such as reverse halo signs or ground glass with extensive subpleural consolidations and air bronchograms. Subpleural curvilinear bands or bands of ground glass with or without consolidation in a tethered, arching pattern with small connections to the pleura are also considered typical. Thickened vessels within lung abnormalities are typical and frequently found in all other confirmatory patterns. CO-RADS 5 is largely identical to the “typical appearance” of the RSNA consensus statement .

7. CO-RADS 6

CO-RADS 6 was introduced to indicate proven COVID-19 as signified by a positive RT-PCR test for virus-specific nucleic acid.

Appendix E4. Detailed results of selected features after applying mRMR algorithm

mRMR

Due to the redundancy of the features, we used mRMR method to filter the features, the final featureset is as following:

[1] "wavelet_HHL_glcm_JointEntropy"

[2] "wavelet_HHL_glcm_ClusterProminence"

[3] "wavelet_HLH_glcm_InverseVariance"

[4] "wavelet_HHL_glrlm_LongRunLowGrayLevelEmphasis"

[5] "wavelet_HHL_glszm_SizeZoneNonUniformity"

[6] "wavelet_HLL_gldm_LargeDependenceLowGrayLevelEmphasis"

[7] "wavelet_HHL_glszm_SmallAreaHighGrayLevelEmphasis"

[8] "wavelet_HHL_glcm_ClusterTendency"

[9] "wavelet_HHL_glcm_JointEnergy"

[10] "wavelet_LHL_glcm_ClusterProminence"

[11] "wavelet_HHL_gldm_LargeDependenceLowGrayLevelEmphasis"

[12] "wavelet_LLH_glcm_Imc1"

[13] "wavelet_HLL_gldm_SmallDependenceHighGrayLevelEmphasis"

[14] "wavelet_LHL_gldm_LargeDependenceLowGrayLevelEmphasis"

[15] "wavelet_HHL_gldm_DependenceEntropy"

[16] "wavelet_HLL_glcm_Contrast"

[17] "wavelet_HHL_gldm_SmallDependenceHighGrayLevelEmphasis"

[18] "wavelet_HHL_glszm_GrayLevelNonUniformityNormalized"

[19] "wavelet_HLH_glcm_Idmn"

[20] "wavelet_HLL_glcm_ClusterProminence"

[21] "wavelet_HHL_firstorder_90Percentile"

[22] "wavelet_HLL_glrlm_LongRunLowGrayLevelEmphasis"

[23] "wavelet_LLH_glcm_InverseVariance"

[24] "wavelet_HHL_glcm_MaximumProbability"

[25] "wavelet_HHL_firstorder_Maximum"

[26] "wavelet_HLL_glszm_SizeZoneNonUniformity"

[27] "wavelet_LHL_gldm_SmallDependenceHighGrayLevelEmphasis"

[28] "wavelet_HHL_firstorder_RootMeanSquared"

[29] "wavelet_LHL_glrlm_LongRunLowGrayLevelEmphasis"

[30] "wavelet_HHL_gldm_LowGrayLevelEmphasis

Appendix E5. Developed radiomics signature

Radscore=0.79wavelet_HHL_glcm_JointEntropy+-0.913wavelet_HHL_glcm_ ClusterProminence+4.913wavelet_HLH_glcm_InverseVariance+0.962wavelet_HHL_glszm_SizeZoneNonUniformity+-2.483wavelet_LLH_glcm_Imc1+-0.663wavelet_HHL_gldm_LargeDependenceLowGrayLevelEmphasis+0.093wavelet_LHH_glcm_Idmn+1.258wavelet_LHL_gldm_LargeDependenceLowGrayLevelEmphasis+0.953wavelet_HHL_glcm_ClusterTendency+-1.066wavelet_HHH_gldm_LargeDependenceHighGrayLevelEmphasis+0.35wavelet_LHL_glcm_ClusterProminence+-0.173wavelet_LHH_gldm_LargeDependenceHighGrayLevelEmphasis+2.623wavelet_HLL_glrlm_LongRunLowGrayLevelEmphasis+-0.583wavelet_HLH_glcm_Idmn+-2.054wavelet_LLH_glcm_InverseVariance+-1.078wavelet_HLL_gldm_LargeDependenceLowGrayLevelEmphasis+-1.3wavelet_LLH_glrlm_RunLengthNonUniformity+0.198wavelet_LHL_glrlm_LongRunLowGrayLevelEmphasis+0.044wavelet_HLH_gldm_LargeDependenceHighGrayLevelEmphasis+0.42wavelet_HHL_glszm_LowGrayLevelZoneEmphasis+-0.055wavelet_LHH_glcm_Idn+0.84wavelet_HHL_glcm_MaximumProbability+0.06*wavelet_HLL_gldm_SmallDependenceHighGrayLevelEmphasis + 3.984