| eTable 1. Participant demographics of the seven individuals who completed the mapping session. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Age (years) | TSI (years) | LOI | AIS | Height (cm) | Weight (kg) | V̇O2peak (mL/kg/min) | Classification a | Peak W |
| ESCS P1 | 24 | 3 | C6 | B | 179 | 81.7 | 16.0 | Excellent | 42 |
| ESCS P2 | 59 | 5 | T4 | A | 177 | 68.9 | 21.3 | Good | 55 |
| ESCS P3 | 38 | 10 | C6 | A | 172 | 83.4 | 14.2 | Good | 70 |
| TSCS P1 | 40 | 19 | T4 | A | 178 | 72.6 | 31.2 | Excellent | 162 |
| TSCS P2 | 54 | 6 | T4 | A | 173 | 80.5 | 13.9 | Fair | 66 |
| TSCS P3 | 22 | 1 | C7 | A | 182 | 66.1 | 15.0 | Good | 59 |
| TSCS P4 | 35 | 6 | C6 | B | 195 | 97.5 | 15.4 | Excellent | 35 |

a Classifications of peak oxygen uptake (V̇O2peak) are based on reference fitness values for the SCI population, described by Simmons et al.1 AIS, American Spinal Injury Association Impairment Scale Grade; LOI, level of injury; TSI, time since injury; V̇O2peak, peak oxygen uptake; W, watts.

| eTable 2. Specific spinal cord stimulation parameters identified as optimizing cardiovascular control (CV-SCS) for all seven participants who completed the mapping session. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Electrode locations | Time since implant | Frequency | Pulse width | Current |
| ESCS P1 | T10 – T12 | 7 months | 30 Hz | 500 μs | 6.1 mA |
| ESCS P2 | T11 – L1 | 3 years | 300 Hz | 380 μs | 3.6 mA |
| ESCS P3 | T11 – T12 | 5 years | 35 Hz | 300 μs | 6 V |
| TSCS P1 | T11 – L1 | - | 30 Hz burst + 10 kHz carrier | 1.0 ms | 39 mA |
| TSCS P2 | T11 – L1 | - | 30 Hz burst + 10 kHz carrier | 1.0 ms | 78 mA |
| TSCS P3 | T11 – L1 | - | 30 Hz burst + 10 kHz carrier | 1.0 ms | 39 mA |
| TSCS P4 | T11 – L1 | - | 30 Hz burst + 10 kHz carrier | 1.0 ms | 65 mA |

| eTable 3. Cardiovascular outcomes at baseline (without SCS) and with cardiovascular-optimised spinal cord stimulation (CV-SCS) during the mapping session. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean SBP (mmHg) |  | Mean HR (bpm) |  | Peak SV (mL) |  | Peak dP/dtmax (mmHg/s) |  | Mean TPR (mmHg/s/mL) |  |
|  | Baseline | CV-SCS | Baseline | CV-SCS | Baseline | CV-SCS | Baseline | CV-SCS | Baseline | CV-SCS |
| ESCS P1 | 123 | 134 | 59 | 65 | 106 | 99 | 942 | 1104 | 0.95 | 1.19 |
| ESCS P2 | 118 | 143 | 62 | 71 | 74 | 73 | 652 | 746 | 1.23 | 1.67 |
| ESCS P3 | 70 | 113 | 87 | 65 | 79 | 137 | 423 | 720 | 0.52 | 0.61 |
| TSCS P1 | 108 | 129 | 73 | 64 | 98 | 116 | 955 | 1208 | 0.73 | 0.93 |
| TSCS P2 | 117 | 146 | 98 | 83 | 68 | 97 | 961 | 1228 | 0.82 | 0.85 |
| TSCS P3 | 124 | 146 | 65 | 57 | 149 | 138 | 1120 | 1825 | 0.58 | 0.84 |
| TSCS P4 | 77 | 112 | 71 | 64 | 118 | 145 | 827 | 1233 | 0.58 | 0.51 |

dP/dtmax, left ventricular cardiac contractility; HR, heart rate; SBP, systolic blood pressure; SV, stroke volume; TPR, total peripheral resistance.

References

- Simmons OL, Kressler J, Nash MS. Reference fitness values in the untrained spinal cord injury population. Arch Phys Med Rehabil. 2014;95(12):2272-2278.