# Alterations in gut microbiota composition, plasma lipids, and brain activity, suggest inter-connected pathways influencing malnutrition-associated cognitive and neurodevelopmental changes.

# Portlock, T.1#, Shama, T.2#, Kakon, S.H.2#, Hartjen, B.3,4#, Pook, C.1#, Wilson, B.C.1#, Ho, D.1, Shennon, I.1, Engelstad, A.M.3,5, Di Lorenzo, R.3,4R, Greaves, G.3, Rahman, N.3,  Kelsey, C.3,4, Gluckman, P.D.1,4, O’Sullivan, J.M.1,4,5,6 ⱡ, Haque, R.2,3, Forrester, T.7, Nelson, C.A.3,4,5 ⱡ, on behalf of the M4EFaD consortium.

# Supplementary materials: Mass Spectrometry methods

## Solvents and chemicals

Methanol and propanol (Optima LC-MS grade) were obtained from Thermo Fisher Scientific (Auckland, Aotearoa New Zealand). Type 1 water was generated from a Merck Millipore unit (Auckland, Aotearoa New Zealand). AnalaR reagents included glacial acetic acid [GAA], also from Merck Millipore, and butanol, chloroform and ammonium hydroxide from Sigma-Aldrich Pty Ltd (Sydney, Australia). The SPLASH Lipidomix solution containing isotopically labelled internal standards was acquired from Avanti Polar Lipids, Inc (Alabama, USA).

## Extraction

Lipids were extracted according to method of Liu et al. (2016). Plasma samples were thawed and maintained on ice. A 10 µL volume of each sample was placed in an amber glass autosampler vial and 300 µL of a mixture of Type 1 water, butanol, methanol, chloroform and SPLASH Lipidomix in a ratio of 4:15:15:20:1 was added. The sample was immediately vortexed and then sonicated for 15 minutes at room temperature. The vials were centrifuged at 4,500 RCF for 10 minutes at 4°C to pellet protein and other precipitate, the supernatant was transferred to a new 1.8 mL amber autosampler vial with a low-volume insert and capped for negative ionisation LC-MS. Samples were diluted five times with 75% IPA in a new autosampler vial with low-volume insert for positive ionisation LC-MS.

## Instrument and data acquisition parameters

Extracted lipids were resolved using a liquid chromatography [LC] system comprised of an Accela 1250 pump (Thermo Fisher Scientific, Austin, Texas USA), a TriPlus autosampler (Thermo Fisher Scientific, Waltham, MA, USA) and a HotDog 5090 column oven (ProLab GmbH, Reinach, Switzerland) set to 45 °C. The mass spectrometer was a Q-Exactive (Thermo Fisher Scientific, Germany) equipped with a heated electrospray ionisation [HESI] source.

The three mobile phases (A/B/C) were water, MeOH and IPA, all containing 0.05% (v/v) GAA and 5mM NH4. Gradients for the positive and negative ionisation methods are shown in Table 1, below.

| Table 1: Details of ternary LC gradients for the positive (left) and negative (right) ionisation methods, respectively. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| minutes | A | B | C |  | minutes | A | B | C |
| 0 | 30 | 70 | 0 |  | 0 | 50 | 50 | 0 |
| 0.1 | 30 | 70 | 0 |  | 0.1 | 50 | 50 | 0 |
| 5 | 5 | 95 | 0 |  | 5 | 10 | 90 | 0 |
| 14.65 | 5 | 25 | 70 |  | 12 | 5 | 90 | 5 |
| 14.75 | 5 | 5 | 90 |  | 14.75 | 5 | 5 | 90 |
| 15.5 | 5 | 5 | 90 |  | 15.5 | 5 | 5 | 90 |
| 16 | 5 | 95 | 0 |  | 16 | 5 | 95 | 0 |
| 16.5 | 30 | 70 | 0 |  | 16.5 | 50 | 50 | 0 |
| 20 | 30 | 70 | 0 |  | 20 | 50 | 50 | 0 |

Quantitative and qualitative mass spectral data was acquired using Data Dependent Analysis [DDA]. For positive ionisation the full scan range was m/z 200-1,000 between zero and eight minutes, and m/z 500-1,250 from eight minutes until the end of the run. For negative ionisation the m/z range was 135-1,000 from zero to eight minutes. For both methods the HESI source was operated with a spray voltage of 3,500 Volts, capillary temperature of 375 °C, sheath gas flow 40, aux gas 8, probe heater temperature of 200 °C, S-Lens RF level of 55 and a max spray current of 100 µA. AGC target was 1e6, maximum ion trapping time was 100 ms, and Orbitrap resolution was 17,500. Four precursors were selected for DDA MS/MS scans every cycle with AGC 1e5, max IT time of 75 ms, isolation window of 4 m/z and an offset of 0.7 m/z with normalised collision energies of 20 and 30. Minimum AGC target was 2e4.

## Data processing and analysis

Data was processed using MS-DIAL v4.92 (Tsugawa et al. 2015, Tsugawa et al. 2020). The results were manually filtered to correct retention time drift and remove false positives and noise features. An in-house Python script (Python v3.9.2) was used to blank subtract peak areas and normalise to the closest internal standard batchwise. Batch effects were removed using Principle Component Analysis to correct for the vectors of the quality control samples.

Supplementary Material references

Liu, Z., S. Rochfort, and B. G. Cocks. 2016. Optimization of a single phase method for lipid extraction from milk. Journal of Chromatography A 1458:145-149.

Tsugawa, H., T. Cajka, T. Kind, Y. Ma, B. Higgins, K. Ikeda, M. Kanazawa, J. VanderGheynst, O. Fiehn, and M. Arita. 2015. MS-DIAL: data-independent MS/MS deconvolution for comprehensive metabolome analysis. Nature methods 12:523-526.

Tsugawa, H., K. Ikeda, M. Takahashi, A. Satoh, Y. Mori, H. Uchino, N. Okahashi, Y. Yamada, I. Tada, P. Bonini, Y. Higashi, Y. Okazaki, Z. Zhou, Z.-J. Zhu, J. Koelmel, T. Cajka, O. Fiehn, K. Saito, M. Arita, and M. Arita. 2020. A lipidome atlas in MS-DIAL 4. Nature Biotechnology 38:1159-1163.