Table S1

PSC characteristics, assumed values of factors affecting the risk of groundwater contamination as a result of potential impact from industrial and infrastructure facilities and crops, and qualitative assessment of unwanted event probability (PA).

| Industrial and infrastructure facilities |  |  | PAi qualitative assessment |  |  |  |  |  |  |  |  | PAi quantitative assessment necessity2) | AEi | Ri |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSC1) | Type | Characteristics | Adopted on the basis of Tables 2 and 4 |  |  |  |  |  |  | Calculated |  |  |  |  |
|  |  |  | LU | Rr | IMr | Tr | Nr | ELr | Hr | Iin | PAi |  |  |  |
| 1 | gas station | modern, well-maintained, fully compliant with environmental requirements | 3 | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | 0.4 | 2.9 | 0.2 |  | 0 | 0 |
| 2 | gas station | older generation, worn out, safety features potentially not fully operational | 3 | 0.7 | 0.4 | 1.0 | 0.1 | 0.7 | 0.7 | 11.9 | 0.6 | + | Tables 10 and S3 |  |  |
| 3 | a livestock farm | modern chicken farm, fully compliant with environmental requirements | 2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | 0.7 | 3.2 | 0.2 |  | 0 | 0 |
| 4 | a livestock farm | carelessly managed porker farm, obsolete pens | 2 | 1.0 | 1.0 | 1.0 | 0.1 | 0.7 | 0.7 | 16.4 | 0.8 | + | Tables 10 and S3 |  |  |
| 5 | dry cleaner | modern, well-maintained, fully compliant with environmental requirements | 2 | 0.1 | 0.1 | 0.4 | 0.1 | 0.4 | 0.1 | 3.8 | 0.2 |  | 0 | 0 |
| 6 | electroplating plant | a small old plant | 4 | 0.4 | 0.4 | 0.7 | 0.1 | 0.7 | 0.7 | 9.2 | 0.4 | 3) | Tables 11 and S4 |  |  |
| 7 | juice manufacturing plant | modern plant | 2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 2 | 0.2 |  | 0 | 0 |
| Landfills |  |  | PAi qualitative assessment |  |  |  |  |  |  |  |  | PAi quantitative assessment necessity2) | AEi | Ri |
| No | Type | Characteristics | Adopted on the basis of Tables 2 and 5 |  |  |  |  |  |  | Calculated |  |  |  |  |
|  |  |  | LU | Tr | Ir | Cr |  | Cmpr |  | Il | PAi |  |  |  |
| 8 | municipal waste landfill | old landfill without ground insulation or cover; waste was compacted multiple times during deposition | 4 | 0.5 | 1.0 | 1.0 |  | 0.5 |  | 13.0 | 0.6 | + | Tables 10 and S3 |  |  |
| 9 | hazardous waste landfill | insulated from the ground (compacted clay, thickness ≥1 m and k ≤ 1·10-9 m/s); drainage present; landfill without cover, partly solidified | 3 | 1.0 | 0.7 | 0.7 |  | 0.8 |  | 13.9 | 0.6 | + | Tables 10 and S3 |  |  |
| Agricultural crops |  |  | Surface area and soil type |  |  |  |  |  |  |  |  | PAi | AEi | Ri |
| I | mixed vegetables (carrot, cucumber, onion, pumpkin) |  | 3 ha of light soil4) |  |  |  |  |  |  |  |  | Table S2 | Tables 11&S4 | Table 12 |
| II | bean |  | 3 ha of medium soil |  |  |  |  |  |  |  |  |  |  |  |
| III | pea |  | 20 ha of medium soil |  |  |  |  |  |  |  |  |  |  |  |

- number acc. to Fig. 2; 2) PA quantitative assessment: “+” necessary, “” not necessary; 3) adverse effect (AE) assessment required; 4) soil category acc. Table S5.

Table S2

Characteristics of crops, fertilization applied, calculation of effective nitrogen loads from individual sources and a sample assessment of unwanted event probability (PA) in an agricultural area

| Scenario | Crop number; characteristics | Fertilization | Parameter | Nitrogen load (kgy-1) (kgha-1y-1) | Calculations |
| --- | --- | --- | --- | --- | --- |
| A, B, C | I mixed vegetables (carrot, cucumber, onion, pumpkin); 3 ha of light soil | - dung and liquid manure from shallow litter bedding, 10 beef cattle over one year old - mineral fertilizer “X”: 600 kg of product with a nitrogen content of 14% | Nas | 44.3 | 59 kgNha-1 (1) 0.75 (2) |
|  |  |  | Nan | 225.7 75.2 | 10 pcs. 7.0 t manure (3) 2.7 kgNt-1 (3) 0.40 (2) + 10 pcs. 6.9 m3 liquid manure (3) 2.9 kgNm-3 (3) 0.75 (2) |
|  |  |  | Nam | 84 28.0 | 600 kg 0.14 (6) |
|  |  |  | Na | 147.5 | 44.3 + 75.2 + 28.0 |
|  |  |  | Nd (4) | 180 |  |
|  |  |  | FN | -32.5 | PA = 0.1 (very low) |
| A, C | II bean; 3 ha of medium soil | - dung and liquid manure from shallow litter bedding, 40 porkers - mineral fertilizer “Y”: 800 kg of product with a nitrogen content of 28% | Nas | 46.5 | 62 kgNha-1 (1) 0.75 (2) |
|  |  |  | Nan | 246.3 81.2 | 40 pcs. 1.5 t dung (3) 4.4 kgNt-1 (3) 0.40 (2) + 40 pcs. 1.0 m3 liquid manure (3) 4.6 kgNm-3 (3) 0.75 (2) |
|  |  |  | Nam | 224.0 74.7 | 800 kg 0.28 (6) |
|  |  |  | Na | 202.4 | 46.5 + 81.2 + 74.7 |
|  |  |  | Nd (5) | 60 |  |
|  |  |  | FN | 142.4 | PA = 0.9 (7) |
| B | III pea; 20 ha of medium soil | - dung and liquid manure from shallow litter bedding, 200 pigs - mineral fertilizer “Z”: 5,000 kg of product with a nitrogen content of 34% | Nas | 46.5 | 62 kgNha-1 (1) 0.75 (2) |
|  |  |  | Nan | 2,318.0 115.9 | 200 pcs. 3.7 t dung (3) 4.0 kgNt-1 (3) 0.40 (2) + 200 pcs. 1.8 m3 liquid manure (3) 4.2 kgNm-3 (3) 0.75 (2) |
|  |  |  | Nam | 1,700.0 85.0 | 5,000 kg 0.34 (6) |
|  |  |  | Na | 247.4 | 46.5 + 115.9 + 85.0 |
|  |  |  | Nd (5) | 30 |  |
|  |  |  | FN | 217.4 | PA = 0.9 (7) |

(1) according to Table S5; (2) according to Table S6; (3) according to Table S7; (4) for light soil according to Table S8; (5) for medium soil according to Table S8; (6) nitrogen content of the mineral fertilizer applied; (7) adverse effect (AE) assessment required.

Table S3

Quantitative assessment of the probability of an unwanted event (PA)1).

| PSC and failure initiating accident2) | Pai | Event that conditions the initiating accident | Pcj | PaimaxPcj | PABZ | Fail-safe protection | PSFi | PSF | PAi | AEi | Ri |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. Gas station |  |  |  |  |  |  |  |  |  |  |  |
| damage to a gas pump | 0.01 | vehicle impact | 0.05 | 0.0005 | 0.0005 | protective bollards; speed limit | 0.0001 | 0.001 | 0.0000005 (5E-7) | 0 | 0 |
|  |  | pump mechanism failure | 0.002 |  |  | fuel flow shut-off valve | 0.0001 |  |  |  |  |
| leak from fuel tank | 0.01 | ruptured tank wall seams | 0.002 | 0.00002 |  | periodic tightness check | 0.001 |  |  |  |  |
|  |  | tank wall corrosion | 0.001 |  |  | double-wall tank; corrosion resistant material | 0.0001 |  |  |  |  |
|  |  |  |  |  |  | piezometer – monitoring | 0.0001 |  |  |  |  |
| 4. Livestock farm |  |  |  |  |  |  |  |  |  |  |  |
| leaking insulation of the slurry and dung storage area | 0.25 | mechanical damage, wear | 0.5 | 0.125 | 0.125 | periodic tightness check | 0.01 | 0.01 | 0.00125 | 0 | 0 |
|  |  | chemical corrosion | 0.25 |  |  |  |  |  |  |  |  |
| 8. Municipal waste landfill |  |  |  |  |  |  |  |  |  |  |  |
| no insulation from the ground and no cover | 1.0 | rainfall infiltration into waste generating leachate | 1.0 | 1.0 | 1.0 | none |  |  | 1.0 | Tables 11 and S4 |  |  |
| 9. Hazardous waste landfill |  |  |  |  |  |  |  |  |  |  |  |
| ground insulation leaking | 0.1 | soil suffosion as a result of prolonged precipitation | 0.01 | 0.001 | 0.001 | correct insulation design and execution | 0.05 | 0.05 | 0.00005 (5E-5) | 0 | 0 |
|  |  | natural mass ground movements | 0.001 |  |  |  |  |  |  |  |  |
| landfill drainage damage | 0.05 | soil suffosion as a result of prolonged precipitation | 0.01 | 0.0005 |  | correct drainage design and execution | 0.05 |  |  |  |  |
|  |  | natural mass ground movements | 0.001 |  |  |  |  |  |  |  |  |

1) probability expressed in dimensionless form, 2) PSC number according to Tables 8 and S1; “” not applicable.

Table S4

Assessment of the adverse effect (AE) of the possible events detected at the stage of assessing the probability of an unwanted event

| Scenario; well discharge | PSC1) and threat form | PSC area | Contaminant and its concentration in PSC (cs) | Transport time to aquifer (tv)2) | Baseline concentration (c0)3) | Groundwater flux beneath PSC (Qf) | Aquifer recharge from PSC area (Qi) | Maximal concen-tration (cmax) | Concen-tration in well (cw) | Threshold concentration in drinking water (ctr)4) | AEi |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m3d-1 |  | m2 | mgL-1 | y | mgL-1 | m3d-1 | m3d-1 | mgL-1 | mgL-1 | mgL-1 |  |
| A 1,000 | 8.1 leachate | 40,000 | sulfate 1,000 5) | 4.1 | 50 | 14.0 | 21.4 | 624.0 | 70.3 | 250 | 0 |
|  | 6 chromium plating bath |  | chromium 20,000 6) | 26.3 7)8) | 0.001 | 1.4 | 0.001 8) | 0.001 | 0.001 | 0.05 | 0 |
|  | crop II topsoil leachate, nitrogen load surplus 9) | 30,000 | nitrate 485 10) | 6.15 | 1 | 12.1 | 10.7 | 227.8 | 6.2 | 50 | 0 |
| B 1,000 | 8.2 leachate | 200,000 | sulfate 2,000 | 4.1 | 50 | 31.3 | 106.8 | 1,558.1 | 258.4 | 250 | 1 |
|  | crop III topsoil leachate, nitrogen load surplus 9) | 200,000 | nitrate 741 10) | 6.15 | 1 | 31.3 | 71.2 | 515.0 | 53.7 | 50 | 1 |
| C 100 | 8.1 leachate | 40,000 | sulfate 1,000 | 4.1 | 50 | 14.0 | 21.4 | 624.0 | 253.0 | 250 | 1 |
|  | crop II topsoil leachate, nitrogen load surplus 9) | 30,000 | nitrate 485 10) | 6.15 | 1 | 12.1 | 10.7 | 227.8 | 52.7 | 50 | 1 |

1) number according to Tables 8 and S1;
2) tv calculated according to Eq. (19) and (23),

3) based on Edmunds and Shand (2008),

4) based on WHO (2017),

5) based on Twardowska (2004), Pietrucin (2013),

6) based on Najdeker (1997),

7) chromium transport time accounting for sorption, i.e., tv Cr = tv·Rf, retardation factor Rf = 12 adopted for silty and loamy sand based on Witczak et al. (2013),

8) assumed volume of leakage through cracks in chromium plating bathtubs Ie = 0.001 m3d-1,

9) FN calculated in Table S2,

10) cNO3 calculated according to Eq. (20).

Table S5

Average mineral nitrogen stock in soil to a depth of 0.6 m (Ns) in spring, depending on soil category (based on Jadczyszyn, 2006; Jadczyszyn et al., 2012; CM, 2020)

| Soil category | Ns (kgha-1) | Soil type |
| --- | --- | --- |
| very light | 49 | sand, sandy soil, rocky soil, skeletal soil |
| light | 59 | silty sand, sandy silt, rendzina, sandy alluvial soil, muck soil |
| medium | 62 | sandy loam, loamy silt, slightly loamy soil, silt soil, loess, alluvial soil |
| heavy | 66 | silty loam, clayey silt, silty clay, loamy soil, clay soil, muck bog soil |

Table S6

Correction coefficient which determines the effectiveness of soil nitrogen () relative to manure from European livestock breeds () and relative to nitrogen from mineral fertilizers (based on CM, 2020, averaged)

| Nitrogen source | Factor |  |
| --- | --- | --- |
| mineral nitrogen from soil |  | 0.75 |
| dung |  | 0.40 |
| slurry/mixed feces |  | 0.60 |
| liquid manure |  | 0.75 |

Table S7

Approximate annual amount of excreted manure per farm animal and approximate concentration of nitrogen contained in manure, depending on the farming system (based on Maćkowiak and Żebrowski, 2000; Walczak et al., 2012; CM, 2020)

| species/group of animals (European breeds) | animal husbandry system |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | deep litter bedding |  | shallow litter bedding |  |  |  | no litter bedding |  |
|  | dung mixed with straw |  | dung mixed with straw |  | slurry |  | slurry or dung |  |
|  | t | kgNt-1 | t | kgNt-1 | m3 | kgNm-3 | m3 (t) | kgNm-3 (kgNt-1) |
| dairy cow (on average) | 24.0 | 3.1 | 14.8 | 3.3 | 7.6 | 3.2 | 23.0 | 4.0 |
| beef cattle | 15.0 | 3.0 | 7.0 | 2.7 | 6.9 | 2.9 | 14.2 | 3.2 |
| pig | 5.0 | 3.9 | 3.7 | 4.0 | 1.8 | 4.2 | 4.6 | 4.3 |
| porker | 2.0 | 4.2 | 1.5 | 4.4 | 1.0 | 4.6 | 1.9 | 4.6 |

Table S8

Approximate maximum effective nitrogen load (Nd) in medium soil1) for a given crop2) (based on CM, 2020)

| Crop | Maximum effective nitrogen load1) (kgha-1y-1) |  | Crop | Maximum effective nitrogen load1) (kgha-1y-1) |
| --- | --- | --- | --- | --- |
| corn (maize) | 240 |  | sorghum | 200 |
| wheat spring/winter | 160/200 |  | cabbage, cauliflower | 300350 |
| barley, triticale, millet | 140 |  | carrot, cucumber, onion, pumpkin | 200 |
| oat, winter rye | 120 |  | tomato, zucchini, asparagus | 150180 |
| malting barley, spring rye | 80 |  | bean | 60 |
| potato early/late | 90/180 |  | pea, soybean | 30 |

1) in light soil (Table S5) the maximum amount of effective nitrogen from all sources is reduced by about 10%, in very light soil by 20%, and in heavy soil it is increased by 10%,

2) the nitrogen load stated in the table is the sum of approximate nitrogen loads from all sources (mineral fertilizer, manure, and soil nitrogen) determined for optimal yields under Central European climate conditions, controlled soil reaction, balanced application of nitrogen, phosphate, and potassium (N-P-K) fertilizers and pesticides.

References

CM Council of Ministers (2020) The action program aimed at reducing water pollution by nitrates from agricultural sources and to prevent further pollution. Regulation of the Council of Ministers of the Republic of Poland, Dziennik Ustaw 2020, 243 (in Polish)

Edmunds WM, Shand P eds. (2008) Natural Groundwater Quality. Blackwell Publ., Malden, Massachusetts

Maćkowiak C, Żebrowski J (2000) Chemical composition of farmyard manure in Poland. Nawozy i Nawożenie 2(4):119–130. Institute of Soil Science and Plant Cultivation (IUNG), Puławy 
(in Polish)

Jadczyszyn T (2006) Selected aspects of agrochemical soil testing. In: Studia i Raporty, Institute of Soil Science and Plant Cultivation (IUNG), Puławy (in Polish)

Jadczyszyn T, Kowalczyk J, Lipiński W (2012) Mineral fertilization on arable land and permanent grassland. Instrukcja Rozpowszechniania 184:1–24, Institute of Soil Science and Plant Cultivation (IUNG), Puławy (in Polish)

Najdeker E (1997) Trivalent chromium plating baths, 1. Literature overview and primary investigations. Materiały Elektroniczne, 25 (2), 2634 (in Polish)

Pietrucin D (2013) Monitoring of the aquatic environment of an industrial area with multiple sources of pollution. Bulletin of Geography, Physical Geography Series, 6:43–58. https://doi.org/10.2478/bgeo-2013-0003

Twardowska I (2004) Assessment of pollution potential from solid waste. in: Twardowska I, Allen HE, Kettrup AF, Lacy WJ (Eds.) Solid Waste: Assessment, Monitoring and Remediation. Waste Management Series, 4, Elsevier, Amsterdam

Walczak J, Krawczyk W, Szewczyk A, Mazur D, Pająk T, Radecki P (2012) Estimation of the production amount and the unit nitrogen content of natural fertilizers generated in various livestock husbandry systems in Poland. National Research Institute of Animal Production, Kraków-Balice (in Polish, unpublished)

WHO (2017) Guidelines for drinking-water quality: Fourth edition incorporating the first Addendum. World Health Organization, Geneva, Licence: CC BY-NC-SA 3.0 IGO. https://www.who.int/publications/i/item/9789241549950

Witczak S, Kania J, Kmiecik E (2013) Catalog of selected physical and chemical indicators of groundwater pollution and methods of their determination. Environmental Monitoring Library, Publ. by Inspection of Environmental Protection, Warszawa, (in Polish).