Appendix 1

The parameters of four motor test process using built-in algorithms:

- Sit-to-stand process

| - Parameters | - Significance | - Unit |
| --- | --- | --- |
| - SiSt-Average Duration | - Average duration of the stand-up process | - sec |
| - SiSt-Trunk Sagittal Peak Velocity | - Peak angular velocity of the trunk sagittal plane during the stand-up process | - degree/sec |
| - SiSt-Trunk Sagittal RoM | - the angular range of the trunk sagittal rotation during the stand-up process | - degree |

- Walking process

| Parameters | - Unit |
| --- | --- |
| - Step Length | - cm |
| - Stride Velocity | - m/s |
| - Stride Length | - cm |
| - Gait Cycle | - sec |
| - Cadence | - step/min |
| - Double Support | - % |
| - Swing phase | - % |
| - Shank RoM | - degree |
| - Peak Shank Angular Velocity | - degree/sec |
| - Stride Length Asymmetry | - % |
| - Swing Asymmetry | - % |
| - Shank RoM Asymmetry* | - % |
| - Shank Symbolic Symmetry Index | - % |
| - phase coordination index | - % |
| - Trunk Coronal Peak Velocity | - degree/sec |
| - Trunk Coronal RoM | - degree |
| - Trunk Sagittal Peak Velocity | - degree/sec |
| - Trunk Sagittal RoM | - degree |
| - Trunk Transverse Peak Velocity | - degree/sec |
| - Trunk Transverse RoM | - degree/sec |
| - Arm Peak Velocity | - degree/sec |
| - Arm RoM | - degree |
| - Arm Symbolic Symmetry Index | - % |

- *Shank RoM Asymmetry: left and right shank swing range relative deviation.

- The coefficients of variation (CV) for stride length, stride velocity, gait cycle, cadence, double support phase and swing phase (CV=SD/mean×100, where "mean" is the mean value of a gait parameter during walking for each subject, and SD is the standard deviation of the corresponding variable during walking for each subject)

- Turning process

| - Parameters | - Unit |
| --- | --- |
| - Turning - Average Duration | - sec |
| - Turning - Average Steps | - number |
| - Turning - Peak Velocity | - degree/sec |
| - Turning - Average Angular Velocity | - degree/sec |

- Stand-to-sit process

| - Abbreviation | - Significance | - Unit |
| --- | --- | --- |
| - StSi-Average Duration | - Average duration of the sitting process | - sec |
| - StSi-Trunk Sagittal Peak Velocity | - Peak angular velocity of trunk sagittal plane during the sitting process | - degree/sec |
| - StSi-Trunk Sagittal RoM | - the angular range of trunk sagittal plane rotation during the sitting process | - degree |