Supporting Information

Experimental data and modeling of viscosity in the quinary system NaCl+KCl+CaCl2+MgCl2+H2O

Sheng Wang1·Mengjie Luo1*·Yuzhu Sun1·Congying Wang1·Xingfu Song1*

*Corresponding authors.

E-mail addresses: mengjie_luo@ecust.edu.cn (M. Luo), xfsong@ecust.edu.cn (X. Song).

1 National Engineering Research Center for Integrated Utilization of Salt Lake Resources, Joint International Laboratory for Potassium and Lithium Strategic Resources, East China University of Science and Technology, Shanghai 200237, China

Table S1 Density, experimental viscosity and calculated viscosity of binary solutions at 288.15K

| m/(mol·kg-1) | ρ/(g·cm-3) | η/(mPa·s) | ηcal/(mPa·s) |
| --- | --- | --- | --- |
| MgCl2 |  |  |  |
| 0.0525 | 1.0022 | 1.1626 | 1.1347 |
| 0.2504 | 1.0193 | 1.2553 | 1.2492 |
| 0.4146 | 1.0313 | 1.3433 | 1.3527 |
| 0.8724 | 1.0648 | 1.5995 | 1.6322 |
| 1.3546 | 1.0980 | 1.9324 | 1.9370 |
| 1.8857 | 1.1325 | 2.3929 | 2.3529 |
| 2.5055 | 1.1682 | 3.0720 | 3.0386 |
| 3.1532 | 1.2041 | 4.0756 | 4.0824 |
| 3.2979 | 1.2116 | 4.3310 | 4.3719 |
| 3.5500 | 1.2249 | 4.8521 | 4.9347 |
| 3.8143 | 1.2416 | 5.6731 | 5.6170 |
| 4.7005 | 1.2847 | 9.0769 | 8.9395 |
| 5.0300 | 1.3006 | 10.7080 | 10.8373 |
| 5.6168 | 1.3289 | 15.8895 | 15.8687 |
| AAD(%) |  |  | 1.40 |
| KCl |  |  |  |
| 0.1001 | 1.0038 | 1.1380 | 1.1364 |
| 0.2048 | 1.0088 | 1.1332 | 1.1334 |
| 0.4103 | 1.0182 | 1.1292 | 1.1283 |
| 0.5500 | 1.0243 | 1.1253 | 1.1252 |
| 0.6500 | 1.0291 | 1.1225 | 1.1233 |
| 0.8500 | 1.0376 | 1.1204 | 1.1199 |
| 1.1044 | 1.0490 | 1.1140 | 1.1163 |
| 1.3497 | 1.0591 | 1.1148 | 1.1139 |
| 1.7945 | 1.0774 | 1.1109 | 1.1117 |
| 2.9266 | 1.1191 | 1.1200 | 1.1185 |
| 3.5386 | 1.1406 | 1.1300 | 1.1297 |
| 4.1848 | 1.1630 | 1.1465 | 1.1472 |
| AAD(%) |  |  | 0.08 |
| NaCl |  |  |  |
| 0.3480 | 1.0133 | 1.1738 | 1.1719 |
| 0.4989 | 1.0198 | 1.1846 | 1.1842 |
| 0.6131 | 1.0242 | 1.1915 | 1.1939 |
| 1.2679 | 1.0494 | 1.2575 | 1.2579 |
| 1.9147 | 1.0735 | 1.3375 | 1.3369 |
| 2.5844 | 1.0964 | 1.4355 | 1.4362 |
| 3.0878 | 1.1142 | 1.5277 | 1.5227 |
| 3.6034 | 1.1296 | 1.6160 | 1.6221 |
| 4.5666 | 1.1607 | 1.8395 | 1.8369 |
| 5.2310 | 1.1792 | 2.0065 | 2.0072 |
| AAD(%) |  |  | 0.14 |
| CaCl2 |  |  |  |
| 0.0500 | 1.0028 | 1.1577 | 1.1608 |
| 0.1006 | 1.0075 | 1.1744 | 1.1760 |
| 0.2008 | 1.0164 | 1.2051 | 1.2051 |
| 0.2985 | 1.0248 | 1.2369 | 1.2340 |
| 0.4199 | 1.0357 | 1.2720 | 1.2715 |
| 0.8850 | 1.0748 | 1.4363 | 1.4358 |
| 1.3792 | 1.1147 | 1.6491 | 1.6497 |
| 1.9461 | 1.1573 | 1.9508 | 1.9514 |
| 2.5100 | 1.1967 | 2.3323 | 2.3295 |
| 3.1988 | 1.2423 | 2.9429 | 2.9552 |
| 3.4020 | 1.2556 | 3.1917 | 3.1899 |
| 3.9411 | 1.2894 | 3.9956 | 3.9725 |
| 4.4077 | 1.3166 | 4.8822 | 4.9040 |
| 4.8200 | 1.3423 | 6.0049 | 5.9982 |
| AAD(%) |  |  | 0.18 |

Table S2 Density, experimental viscosity and calculated viscosity of binary solutions at 293.15K

| m/(mol·kg-1) | ρ/(g·cm-3) | η/(mPa·s) | ηcal/(mPa·s) |
| --- | --- | --- | --- |
| MgCl2 |  |  |  |
| 0.0525 | 1.0017 | 1.0239 | 0.9970 |
| 0.2504 | 1.0182 | 1.0992 | 1.0964 |
| 0.4146 | 1.0301 | 1.1799 | 1.1872 |
| 0.8724 | 1.0634 | 1.4029 | 1.4318 |
| 1.3546 | 1.0967 | 1.6914 | 1.6964 |
| 1.8857 | 1.1308 | 2.0942 | 2.0562 |
| 2.5055 | 1.1672 | 2.6758 | 2.6479 |
| 3.1532 | 1.2025 | 3.5344 | 3.5426 |
| 3.2979 | 1.2096 | 3.7546 | 3.7890 |
| 3.5500 | 1.2237 | 4.1908 | 4.2655 |
| 3.8143 | 1.2402 | 4.8917 | 4.8390 |
| 4.7005 | 1.2835 | 7.6979 | 7.5787 |
| 5.0300 | 1.2985 | 9.0086 | 9.1216 |
| 5.6168 | 1.3270 | 13.2101 | 13.1918 |
| AAD(%) |  |  | 1.42 |
| KCl |  |  |  |
| 0.1001 | 1.0030 | 1.0032 | 1.0016 |
| 0.2048 | 1.0079 | 0.9999 | 0.9998 |
| 0.4103 | 1.0172 | 0.9970 | 0.9972 |
| 0.5500 | 1.0231 | 0.9953 | 0.9960 |
| 0.6500 | 1.0279 | 0.9965 | 0.9953 |
| 0.8500 | 1.0365 | 0.9936 | 0.9942 |
| 1.1044 | 1.0476 | 0.9931 | 0.9937 |
| 1.3497 | 1.0577 | 0.9936 | 0.9940 |
| 1.7945 | 1.0759 | 0.9965 | 0.9964 |
| 2.9266 | 1.1175 | 1.0096 | 1.0125 |
| 3.5386 | 1.1389 | 1.0333 | 1.0272 |
| 4.1848 | 1.1612 | 1.0443 | 1.0471 |
| AAD(%) |  |  | 0.14 |
| NaCl |  |  |  |
| 0.3480 | 1.0122 | 1.0304 | 1.0331 |
| 0.4989 | 1.0184 | 1.0433 | 1.0448 |
| 0.6131 | 1.0232 | 1.0547 | 1.0541 |
| 1.2679 | 1.0482 | 1.1165 | 1.1144 |
| 1.9147 | 1.0715 | 1.1892 | 1.1869 |
| 2.5844 | 1.0952 | 1.2764 | 1.2761 |
| 3.0878 | 1.1121 | 1.3551 | 1.3527 |
| 3.6034 | 1.1271 | 1.4333 | 1.4397 |
| 4.5666 | 1.1585 | 1.6257 | 1.6256 |
| 5.2310 | 1.1776 | 1.7732 | 1.7716 |
| AAD(%) |  |  | 0.16 |
| CaCl2 |  |  |  |
| 0.0500 | 1.0022 | 1.0181 | 1.0214 |
| 0.1006 | 1.0065 | 1.0331 | 1.0352 |
| 0.2008 | 1.0153 | 1.0628 | 1.0623 |
| 0.2985 | 1.0238 | 1.0891 | 1.0896 |
| 0.4199 | 1.0347 | 1.1259 | 1.1249 |
| 0.8850 | 1.0735 | 1.2861 | 1.2783 |
| 1.3792 | 1.1127 | 1.4687 | 1.4738 |
| 1.9461 | 1.1557 | 1.7411 | 1.7444 |
| 2.5100 | 1.1951 | 2.0786 | 2.0784 |
| 3.1988 | 1.2401 | 2.6204 | 2.6244 |
| 3.4020 | 1.2537 | 2.8378 | 2.8280 |
| 3.9411 | 1.2862 | 3.5051 | 3.5049 |
| 4.4077 | 1.3141 | 4.3015 | 4.3083 |
| 4.8200 | 1.3396 | 5.2540 | 5.2508 |
| AAD(%) |  |  | 0.18 |

Table S3 Density, experimental viscosity and calculated viscosity of binary solutions at 298.15K

| m/(mol·kg-1) | ρ/(g·cm-3) | η/(mPa·s) | ηcal/(mPa·s) |
| --- | --- | --- | --- |
| MgCl2 |  |  |  |
| 0.0525 | 1.0004 | 0.9120 | 0.8921 |
| 0.2504 | 1.0172 | 0.9828 | 0.9776 |
| 0.4146 | 1.0289 | 1.0469 | 1.0549 |
| 0.8724 | 1.0623 | 1.2457 | 1.2676 |
| 1.3546 | 1.0956 | 1.4969 | 1.5032 |
| 1.8857 | 1.1294 | 1.8541 | 1.8219 |
| 2.5055 | 1.1655 | 2.3560 | 2.3353 |
| 3.1532 | 1.2013 | 3.1052 | 3.1006 |
| 3.2979 | 1.2076 | 3.2713 | 3.3106 |
| 3.5500 | 1.2222 | 3.6616 | 3.7166 |
| 3.8143 | 1.2392 | 4.2417 | 4.2048 |
| 4.7005 | 1.2820 | 6.6258 | 6.5237 |
| 5.0300 | 1.2971 | 7.7167 | 7.8121 |
| 5.6168 | 1.3251 | 11.1642 | 11.1489 |
| AAD(%) |  |  | 1.05 |
| KCl |  |  |  |
| 0.1001 | 1.0017 | 0.8937 | 0.8928 |
| 0.2048 | 1.0067 | 0.8931 | 0.8923 |
| 0.4103 | 1.0161 | 0.8918 | 0.8917 |
| 0.5500 | 1.0219 | 0.8915 | 0.8916 |
| 0.6500 | 1.0266 | 0.8903 | 0.8916 |
| 0.8500 | 1.0354 | 0.8936 | 0.8919 |
| 1.1044 | 1.0460 | 0.8921 | 0.8930 |
| 1.3497 | 1.0562 | 0.8930 | 0.8946 |
| 1.7945 | 1.0742 | 0.8997 | 0.8991 |
| 2.9266 | 1.1156 | 0.9184 | 0.9193 |
| 3.5386 | 1.1368 | 0.9382 | 0.9353 |
| 4.1848 | 1.1603 | 0.9548 | 0.9564 |
| AAD(%) |  |  | 0.12 |
| NaCl |  |  |  |
| 0.3480 | 1.0111 | 0.9165 | 0.9193 |
| 0.4989 | 1.0172 | 0.9287 | 0.9308 |
| 0.6131 | 1.0217 | 0.9421 | 0.9399 |
| 1.2679 | 1.0467 | 1.0008 | 0.9978 |
| 1.9147 | 1.0698 | 1.0663 | 1.0655 |
| 2.5844 | 1.0937 | 1.1459 | 1.1468 |
| 3.0878 | 1.1110 | 1.2152 | 1.2153 |
| 3.6034 | 1.1254 | 1.2901 | 1.2923 |
| 4.5666 | 1.1564 | 1.4551 | 1.4541 |
| 5.2310 | 1.1746 | 1.5797 | 1.5795 |
| AAD(%) |  |  | 0.15 |
| CaCl2 |  |  |  |
| 0.0500 | 1.0007 | 0.9061 | 0.9079 |
| 0.1006 | 1.0054 | 0.9179 | 0.9204 |
| 0.2008 | 1.0142 | 0.9446 | 0.9453 |
| 0.2985 | 1.0227 | 0.9726 | 0.9702 |
| 0.4199 | 1.0334 | 1.0037 | 1.0024 |
| 0.8850 | 1.0722 | 1.1416 | 1.1406 |
| 1.3792 | 1.1112 | 1.3146 | 1.3159 |
| 1.9461 | 1.1537 | 1.5613 | 1.5599 |
| 2.5100 | 1.1931 | 1.8626 | 1.8636 |
| 3.1988 | 1.2377 | 2.3432 | 2.3595 |
| 3.4020 | 1.2518 | 2.5554 | 2.5426 |
| 3.9411 | 1.2836 | 3.1575 | 3.1419 |
| 4.4077 | 1.3116 | 3.8168 | 3.8358 |
| 4.8200 | 1.3369 | 4.6375 | 4.6311 |
| AAD(%) |  |  | 0.25 |

Table S4 Density, experimental viscosity and calculated viscosity of binary solutions at 303.15K

| m/(mol·kg-1) | ρ/(g·cm-3) | η/(mPa·s) | ηcal/(mPa·s) |
| --- | --- | --- | --- |
| MgCl2 |  |  |  |
| 0.0525 | 0.9999 | 0.8165 | 0.8026 |
| 0.2504 | 1.0158 | 0.8805 | 0.8759 |
| 0.4146 | 1.0277 | 0.9357 | 0.9421 |
| 0.8724 | 1.0607 | 1.1125 | 1.1289 |
| 1.3546 | 1.0939 | 1.3375 | 1.3407 |
| 1.8857 | 1.1281 | 1.6497 | 1.6259 |
| 2.5055 | 1.1638 | 2.0891 | 2.0770 |
| 3.1532 | 1.1995 | 2.7464 | 2.7409 |
| 3.2979 | 1.2059 | 2.8910 | 2.9225 |
| 3.5500 | 1.2202 | 3.2353 | 3.2735 |
| 3.8143 | 1.2371 | 3.7256 | 3.6953 |
| 4.7005 | 1.2817 | 5.7496 | 5.6841 |
| 5.0300 | 1.2955 | 6.7084 | 6.7709 |
| 5.6168 | 1.3238 | 9.5325 | 9.5223 |
| AAD(%) |  |  | 1.17 |
| KCl |  |  |  |
| 0.1001 | 1.0006 | 0.8039 | 0.8019 |
| 0.2048 | 1.0055 | 0.8052 | 0.8025 |
| 0.4103 | 1.0146 | 0.8039 | 0.8038 |
| 0.5500 | 1.0205 | 0.8036 | 0.8048 |
| 0.6500 | 1.0249 | 0.8034 | 0.8055 |
| 0.8500 | 1.0340 | 0.8062 | 0.8072 |
| 1.1044 | 1.0442 | 0.8117 | 0.8098 |
| 1.3497 | 1.0545 | 0.8105 | 0.8127 |
| 1.7945 | 1.0723 | 0.8196 | 0.8193 |
| 2.9266 | 1.1139 | 0.8436 | 0.8429 |
| 3.5386 | 1.1348 | 0.8619 | 0.8600 |
| 4.1848 | 1.1572 | 0.8797 | 0.8813 |
| AAD(%) |  |  | 0.18 |
| NaCl |  |  |  |
| 0.3480 | 1.0103 | 0.8194 | 0.8239 |
| 0.4989 | 1.0155 | 0.8334 | 0.8346 |
| 0.6131 | 1.0203 | 0.8449 | 0.8431 |
| 1.2679 | 1.0450 | 0.8998 | 0.8966 |
| 1.9147 | 1.0680 | 0.9599 | 0.9584 |
| 2.5844 | 1.0910 | 1.0336 | 1.0316 |
| 3.0878 | 1.1079 | 1.0911 | 1.0929 |
| 3.6034 | 1.1227 | 1.1564 | 1.1613 |
| 4.5666 | 1.1539 | 1.3060 | 1.3040 |
| 5.2310 | 1.1726 | 1.4145 | 1.4140 |
| AAD(%) |  |  | 0.24 |
| CaCl2 |  |  |  |
| 0.0500 | 0.9994 | 0.8123 | 0.8147 |
| 0.1006 | 1.0040 | 0.8242 | 0.8263 |
| 0.2008 | 1.0130 | 0.8491 | 0.8489 |
| 0.2985 | 1.0212 | 0.8729 | 0.8715 |
| 0.4199 | 1.0319 | 0.9021 | 0.9007 |
| 0.8850 | 1.0703 | 1.0272 | 1.0260 |
| 1.3792 | 1.1094 | 1.1843 | 1.1853 |
| 1.9461 | 1.1515 | 1.4070 | 1.4067 |
| 2.5100 | 1.1913 | 1.6787 | 1.6803 |
| 3.1988 | 1.2357 | 2.1104 | 2.1227 |
| 3.4020 | 1.2487 | 2.2995 | 2.2852 |
| 3.9411 | 1.2811 | 2.8203 | 2.8149 |
| 4.4077 | 1.3087 | 3.4157 | 3.4264 |
| 4.8200 | 1.3341 | 4.1305 | 4.1265 |
| AAD(%) |  |  | 0.21 |

Table S5 Density, experimental viscosity and calculated viscosity of binary solutions at 308.15K

| m/(mol·kg-1) | ρ/(g·cm-3) | η/(mPa·s) | ηcal/(mPa·s) |
| --- | --- | --- | --- |
| MgCl2 |  |  |  |
| 0.0525 | 0.9979 | 0.7400 | 0.7267 |
| 0.2504 | 1.0146 | 0.7975 | 0.7922 |
| 0.4146 | 1.0259 | 0.8444 | 0.8508 |
| 0.8724 | 1.0589 | 0.9997 | 1.0163 |
| 1.3546 | 1.0916 | 1.2010 | 1.2042 |
| 1.8857 | 1.1259 | 1.4765 | 1.4550 |
| 2.5055 | 1.1616 | 1.8667 | 1.8482 |
| 3.1532 | 1.1973 | 2.4205 | 2.4257 |
| 3.2979 | 1.2041 | 2.5541 | 2.5840 |
| 3.5500 | 1.2187 | 2.8616 | 2.8901 |
| 3.8143 | 1.2352 | 3.2824 | 3.2582 |
| 4.7005 | 1.2795 | 5.0518 | 4.9822 |
| 5.0300 | 1.2938 | 5.8433 | 5.9090 |
| 5.6168 | 1.3219 | 8.2153 | 8.2047 |
| AAD(%) |  |  | 0.95 |
| KCl |  |  |  |
| 0.1001 | 0.9989 | 0.7243 | 0.7242 |
| 0.2048 | 1.0036 | 0.7274 | 0.7254 |
| 0.4103 | 1.0129 | 0.7281 | 0.7277 |
| 0.5500 | 1.0193 | 0.7279 | 0.7293 |
| 0.6500 | 1.0236 | 0.7326 | 0.7306 |
| 0.8500 | 1.0318 | 0.7314 | 0.7332 |
| 1.1044 | 1.0427 | 0.7364 | 0.7369 |
| 1.3497 | 1.0530 | 0.7395 | 0.7409 |
| 1.7945 | 1.0707 | 0.7491 | 0.7489 |
| 2.9266 | 1.1121 | 0.7758 | 0.7755 |
| 3.5386 | 1.1332 | 0.7956 | 0.7933 |
| 4.1848 | 1.1553 | 0.8132 | 0.8149 |
| AAD(%) |  |  | 0.16 |
| NaCl |  |  |  |
| 0.3480 | 1.0089 | 0.7390 | 0.7442 |
| 0.4989 | 1.0148 | 0.7530 | 0.7544 |
| 0.6131 | 1.0187 | 0.7654 | 0.7624 |
| 1.2679 | 1.0427 | 0.8159 | 0.8126 |
| 1.9147 | 1.0656 | 0.8715 | 0.8696 |
| 2.5844 | 1.0898 | 0.9352 | 0.9362 |
| 3.0878 | 1.1058 | 0.9917 | 0.9914 |
| 3.6034 | 1.1206 | 1.0481 | 1.0524 |
| 4.5666 | 1.1508 | 1.1806 | 1.1787 |
| 5.2310 | 1.1703 | 1.2754 | 1.2751 |
| AAD(%) |  |  | 0.26 |
| CaCl2 |  |  |  |
| 0.0500 | 0.9980 | 0.7319 | 0.7353 |
| 0.1006 | 1.0024 | 0.7429 | 0.7460 |
| 0.2008 | 1.0111 | 0.7671 | 0.7668 |
| 0.2985 | 1.0194 | 0.7884 | 0.7875 |
| 0.4199 | 1.0301 | 0.8172 | 0.8143 |
| 0.8850 | 1.0682 | 0.9320 | 0.9297 |
| 1.3792 | 1.1070 | 1.0757 | 1.0763 |
| 1.9461 | 1.1492 | 1.2763 | 1.2792 |
| 2.5100 | 1.1881 | 1.5270 | 1.5281 |
| 3.1988 | 1.2327 | 1.9126 | 1.9260 |
| 3.4020 | 1.2468 | 2.0920 | 2.0709 |
| 3.9411 | 1.2790 | 2.5384 | 2.5400 |
| 4.4077 | 1.3062 | 3.0697 | 3.0774 |
| 4.8200 | 1.3310 | 3.6930 | 3.6894 |
| AAD(%) |  |  | 0.29 |

Table S6 AAD values of the extended Jones-Dole model and the GF model from experimental viscosity data of KCl+H2O and NaCl+H2O

| Reference | T(K) | m(mol/kg) | NP | AAD(%) |
| --- | --- | --- | --- | --- |
| KCl-H2O |  |  |  |  |
| Carman | 298.15 | 0.51-4.59 | 6 | 0.34 |
| Out and Los | 288.15 | 0.04-1.00 | 7 | 0.27 |
|  | 298.15 | 0.04-1.00 | 7 | 0.30 |
|  | 308.15 | 0.04-1.00 | 7 | 1.01 |
| Gonqalves and Kestin | 293.15 | 0.33-4.55 | 9 | 0.74 |
|  | 298.15 | 0.33-4.55 | 9 | 0.71 |
|  | 303.15 | 0.33-4.55 | 9 | 0.95 |
| Hu et al. | 298.15 | 0.10-4.20 | 15 | 0.57 |
| NaCl-H2O |  |  |  |  |
| Carman | 298.15 | 0.51-4.59 | 6 | 0.46 |
| Out and Los | 288.15 | 0.10-1.20 | 6 | 0.22 |
|  | 298.15 | 0.10-1.20 | 6 | 0.16 |
|  | 308.15 | 0.10-1.20 | 6 | 0.22 |
| Gonqalves and Kestin | 298.15 | 0.01-5.82 | 14 | 0.35 |
|  | 303.15 | 0.01-5.82 | 14 | 0.23 |
| Motin | 308.15 | 0.06-1.00 | 5 | 0.83 |
| Liu et al. | 293.15 | 0.21-4.81 | 14 | 1.29 |
|  | 298.15 | 0.21-4.81 | 14 | 0.19 |
|  | 303.15 | 0.21-4.81 | 14 | 0.63 |
| Hu et al. | 298.15 | 0.10-0.60 | 6 | 0.11 |

a NP: number of data points of viscosity.

Table S7 AAD values of the extended Jones-Dole model and the GF model from experimental viscosity data of MgCl2+H2O and CaCl2+H2O

| Reference | T(K) | m(mol/kg) | NP | AAD(%) |
| --- | --- | --- | --- | --- |
| MgCl2-H2O |  |  |  |  |
| Königsberger et al. | 298.15 | 1.01-4.04 | 4 | 1.91 |
| Isono | 288.15 | 0.05-5.00 | 8 | 1.53 |
|  | 293.15 | 0.05-5.00 | 8 | 1.56 |
|  | 298.15 | 0.05-5.00 | 8 | 1.44 |
|  | 303.15 | 0.05-5.00 | 8 | 1.38 |
|  | 308.15 | 0.05-5.00 | 8 | 1.72 |
| Pang and Stokes | 298.15 | 0.10-5.67 | 17 | 1.19 |
| CaCl2-H2O |  |  |  |  |
| Gonqalves and Kestin | 293.15 | 0.27-5.10 | 13 | 1.00 |
|  | 298.15 | 0.27-5.10 | 13 | 0.97 |
|  | 303.15 | 0.27-5.10 | 13 | 1.00 |
| Isono | 288.15 | 0.05-6.00 | 8 | 2.53 |
|  | 293.15 | 0.05-6.00 | 8 | 2.54 |
|  | 298.15 | 0.05-6.00 | 8 | 2.23 |
|  | 303.15 | 0.05-6.00 | 8 | 2.44 |
|  | 308.15 | 0.05-6.00 | 8 | 2.62 |

a NP: number of data points of viscosity.