Identifying and testing adaptive management options to increase river catchment system resilience using a Bayesian Network model

Supplementary Material

Kerr Adams1,2, Marc J. Metzger2, Rachel Helliwell1, Nicola Melville3, Christopher (Kit) A. J. Macleod1, Jim Pritchard3, Katie Edwards4, and Miriam Glendell1

1The James Hutton Institute, Craigiebuckler, Aberdeen, Scotland, UK

2University of Edinburgh, Institute of Geography, School of Geoscience, Edinburgh, Scotland, UK

3Scottish Environment Protection Agency, Angus Smith Building Eurocentral, Holytown, Scotland, UK

4Scottish Water, Buchanan Gate Business Park, Stepps, Scotland, UK

## A: Bayesian Network model description, equations, discretisation, parameterisation and justifications

Table S1: Bayesian Network variable log

| Variable Name | Identifier | Type | Equation | States & Discretisation if applicable. |  |  |  |  |  |  |  | Supporting Information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Simulation | i | Deterministic | -- | Green Road (GR) |  |  |  |  |  |  |  | Deterministic input variable for model simulation under a range of plausible simulations. State identifiers are taken from (O’Neill et al., 2017). Current simulation considers current catchment conditions. |
|  |  |  |  | Business As Usual (BAU) |  |  |  |  |  |  |  |  |
|  |  |  |  | Fossil Fuelled Development (FFD) |  |  |  |  |  |  |  |  |
|  |  |  |  | Current |  |  |  |  |  |  |  |  |
| Precipitation Change | j | Deterministic | -- | Extreme Low Rainfall (ExLR) |  |  |  |  |  |  |  | Deterministic input variable for executing annual precipitation change, and precipitation change for extreme low and high rainfall change. |
|  |  |  |  | Annual |  |  |  |  |  |  |  |  |
|  |  |  |  | Extreme High Rainfall (ExHIR) |  |  |  |  |  |  |  |  |
| Management Scenario | m | Deterministic | -- | No Management |  |  |  |  |  |  |  | Deterministic input node for simulating management scenarios. No management scenario ensures no changes are made to the model under simulations i. Each of the remaining management scenarios make changes to child variables based on the parameters detailed in Table S3. |
|  |  |  |  | Standard |  |  |  |  |  |  |  |  |
|  |  |  |  | Nature Based |  |  |  |  |  |  |  |  |
|  |  |  |  | BAT |  |  |  |  |  |  |  |  |
|  |  |  |  | Resource Centre |  |  |  |  |  |  |  |  |
| Climate Precipitation Choice | CPC | Deterministic | -- | Green Road & ExLR |  |  |  |  |  |  |  | Deterministic node that combines Precipitation Anomaly with the Simulation node to enable the selection of precipitation anomaly scenarios under the different diverse future pathway scenarios. |
|  |  |  |  | Green Road & Annual |  |  |  |  |  |  |  |  |
|  |  |  |  | Green Road & ExHIR |  |  |  |  |  |  |  |  |
|  |  |  |  | BAU & ExLR |  |  |  |  |  |  |  |  |
|  |  |  |  | BAU & Annual |  |  |  |  |  |  |  |  |
|  |  |  |  | BAU & ExHIR |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD & ExLR |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD & Annual |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD & ExHIR |  |  |  |  |  |  |  |  |
|  |  |  |  | Current & ExLR |  |  |  |  |  |  |  |  |
|  |  |  |  | Current & Annual |  |  |  |  |  |  |  |  |
|  |  |  |  | Current & ExHIR |  |  |  |  |  |  |  |  |
| Precipitation Change Anomaly (%) | PA | Equation | PA=βij | State |  | From |  |  | To |  |  | Equation variable that selects the precipitation change anomaly distribution β for each future simulation i and precipitation change simulation j. Values for β are derived from the UK Climate Projection User Interface product Anomalies for probabilistic projections (25km) over UK, 1961-2100. Anomalies take the coupled RCP distributions for i and j. Annual temporal averages are used for Annual state to represent the incremental predicted change. To represent shocks to the system, Q95 values for seasonal summer and winter averages are taken for ExLR and ExHIR as these represent the most extreme possible changes. The data is selected for the 1981-2010 baseline period, in grid cell 337500.00, 712500.00, during the time slice 2040-2069 (2050’s) using all sampling methods. |
|  |  |  |  | ExLR |  | 0 |  |  | 0.75 |  |  |  |
|  |  |  |  | Annual |  | 0.75 |  |  | 1.1 |  |  |  |
|  |  |  |  | ExHIR |  | 1.1 |  |  | 2.5 |  |  |  |
| Population Equivalent Change | PC | Deterministic | -- | GR |  |  |  |  |  |  |  | Deterministic node that sets acquires a population equivalent change values i. Values for derived from the Scottish Water Population Growth Model. The Growth Model provides Real and Raw estimations of Population Equivalents (PE) to the year 2030. For the Green Road scenario (GR), the lower Real PE estimate for 2030 remains consistent for 2050 to reflect as shared-socio-economic pathway (SSP) narrative which suggests population growth will stagnate in urban areas and migration to more rural areas will increase. For the Business As Usual (BAU) the Real PE trend for 2030 is extrapolated to 2050. For the Fossil Fuelled Development scenario (FFD), the Raw PE value for 2030 is extrapolated to 2050 as RAW PE provides an upper estimate of population growth, particularly in urban areas, which is reflected in the SSP narrative for FFD. Narratives are derived from Pedde, et al., (2021). |
|  |  |  |  | BAU |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD |  |  |  |  |  |  |  |  |
|  |  |  |  | Current |  |  |  |  |  |  |  |  |
| Land Cover Change | LCC | Deterministic | -- | GR |  |  |  |  |  |  |  | Deterministic variable that sets land cover change values for i. Current and project future value are derived from UKCEH land cover vector maps (Morton et al., 2020). Extrapolations of historical land cover change, interpretations from the SSP narratives (Pedde, at al., 2021) and catchment specific knowledge provided by stakeholders were used to create projections for different land cover areas. |
|  |  |  |  | BAU |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD |  |  |  |  |  |  |  |  |
|  |  |  |  | Current |  |  |  |  |  |  |  |  |
| Wastewater sub-model |  |  |  |  |  |  |  |  |  |  |  |  |
| Dry Weather Flow (Ml/d) | DWf | Equation | DWfik=βk+PCik ×γk | State |  |  | From |  |  |  | To | Stakeholders highlighted the Dry Weather Flow, DWf, at wastewater treatment works (WwTWs), k, in the catchment as an important variable that represents the wastewater system. Stakeholders highlighted that changes in PCik under simulations i could influence DWf at works k. The distribution β represents a truncated distribution of the current DWf at WwTW, k, derived from effluent flow summary statistics provided in the Scottish Water Strategic Study. We simulated effluent flows using the summary statistics to generate 365 data outputs, then calculated a Q80 value of the outputs, which was highlighted by stakeholders as the values used to derive asset dry weather flow values. We use the Q80 values as the mean and the standard deviations of the Q20 values to derive βk. PCik is multiplied by γkwhich is the 1 PE value of 200 litres per day wastewater sewage flow contribution (Mara, 2006) which is converted to Ml/d and added to βk. The 200 l per day standard was confirmed by participants during model development focus groups. Resilient states threshold c is the DWf licence condition for k. Anything three times greater than the licence condition value is set as the threshold value for high risk (H) u. Thresholds for states low (L) and moderate (M) risk, b1 uniformised between c and u Ml/d. |
|  |  |  |  | Resilient |  |  | 0 |  |  |  | c |  |
|  |  |  |  | L |  |  | c |  |  |  | b1 |  |
|  |  |  |  | M |  |  | b1 |  |  |  | u |  |
|  |  |  |  | H |  |  | u |  |  |  | inf |  |
| Daily Effluent Flow (Ml/d) | Ef | Equation | Efik=βk×PAij+PCik×γk×m | ‘’ |  |  |  |  |  |  |  | Stakeholders highlighted that the flow of effluent discharge Ef at WwTWs k as a significant wastewater system variable. Changes in PAij and PCik under different simulations i were identified as potentially influencing Efik. To measure potential impacts on Efikwe derived the distribution βk to represent the current Ef distribution for k, derived from the Scottish Water Strategic Study. We multiply current Ef distributions with the % anomaly change in PAij which is assumed to lead to a change in run-off and infiltration which currently influence Ef. PCik is multiplied by the 1 PE value of 200 litres per day waste sewage flow γk and added to βk to represent the influence of changes in PCik on Ef. Different management scenario m influence Efik. Parameter values for m can be found in Table S3. Discretisation of states is based on the >3 DWf (3DWf) licence condition at setting for storm overflow detailed in the SEPA Supporting Guidance (WAT-SG-13) document which is a standard threshold set for calculating the Flow to Full Treatment (FFT) limit for WwTWs. The FFT for values for each WwTW k are described as anything three times greater than DWF leads to the risk of the sewer overflow. The resilient threshold c is therefore set as three times the DWf at treatment works k. The high-risk threshold u is set at six times the DWF. Thresholds for states low (L) and moderate (M) risk, b1 are uniformised between c and u Ml/d. |
| Daily Influent Flow (Ml/d) | If | Equation | Ifik=Efik×γk | ‘’ |  |  |  |  |  |  |  | The change in influent flow If was also highlighted as being influence by PAij and PCik by stakeholders. We use an equation variable representing the change in influent flow If based on the change in Ef using the value γk to represent the difference between If and Ef. The value γk is used due to the limited If data available in the catchment. The only WwTW in the catchment with If data available was Cupar, where a reduction in flow volume after the treatment process was evident in the annual flow returns data from 2015 – 2019 provided by Scottish Water when comparing influent and effluent flows. We calculated the difference between influent and effluent flows using annual flow returns data to derive γk which is applied to each WwTWs k. The If variable is discretised using the same methods as Ef. |
| Spill Event | SP | Equation | SEis=IFeq(Ifik) | State |  | From |  |  | To |  |  | Stakeholders highlight that risk of spill events SE under different simulations i could occur due to changes in Ifik in waterbody sub catchment s. Spills SPis occur if the variable Ifik exceeds its c resilience threshold. We use statement equations IFeq to index the prior distributions of parent variable. Ifikbased in their discretised state thresholds. Each prior state discrete threshold for Ifik resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For SPis the sum of Ifik of prior Ifik values is as follows: IF(Ifik≥u,3,IF(Ifik≥b,2,IF(Ifik≥c,1,0). We set the resilience threshold c for SPis as a value for one, as anything greater than the value of one would mean at least one treatment works k is likely to spill. The upper value u set as the maximum possible index value of all variables (3 times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Wastewater Phosphorus Load (kg/d) | P | Equation | Pik=(βkm+PCik×γk)×Efik | State |  | From |  |  | To |  |  | Equation variable representing the change in Reactive Phosphorus (P) based on the change in PCi and Ef. The current concentration of P is represented using the distribution β for each of the different WwtW k. Current effluent P concentration (mg/l) are taken from the Scottish Water Eden Water Quality Strategic Study. The inverse of PAij is applied to represent the change in P concentration due to the change in dilution. Lower flows lead to higher concentrations of P, meaning when PAij decreases the P concentration increases. PCik is multiplied by the calculated P concentration (mg/l) per PE γ, based on the current PE for WwTW k. The P concentration is multiplied by Ef to provide the daily effluent P load (kg/d). The distribution βk is also influenced by different management scenarios m. The values for βkm can be found in Table S3. The variable Pik is discretised using the current mean P load for each k as the resilient threshold, which is calculated by multiplying the current P concentration by the current Ef. Anything greater than the current P load is seen as an increased risk, as higher loads demonstrate poor outcomes for both the environment and wastewater system. The high risk (H) value u is calculated as 3 times the c. The values for L and M risk are then uniformised between c and u (kg/d). |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b |  |  |  |
|  |  |  |  | M |  | b |  |  | u |  |  |  |
|  |  |  |  | H |  | u |  |  | inf |  |  |  |
| Bio Resource (m3/d) | BR | Equation | BRik=Ifik×γk×1000 | ‘’ |  |  |  |  |  |  |  | Volumes of Bio resource BR (m3/d) was identified as a significant wastewater system variable. Changes in BR due to the influence was simulations i is measured based on a change in If. Su et al. (2019) highlight that an increase in Ifikcan lead to an increase in bio resource concentrations and accumulations. The relationship between If and BR volume is derived by analysing the relationship between flows and sludge volumes at WwTW in the catchment to create an average BRk volume (m3/d) per Ifk (Ml/d) to provide a (m3/l/d) value ο for each k which is represented by γk. The γk value is multiplied by Ifik, then multiplied by 1000 to convert the value to (m3/d). The BRik variable is discretised by setting the resilient threshold c as the current volume of BRk. The high-risk threshold is set as three times the current c value. Thresholds for states low (L) and moderate (M) risk, b1 are uniformised between c and u m3/d. |
| Total Phosphorus Load (kg/d) | TPL | Equation | TPLis=Pik+IF(Ifik≥c, 1.05, 0) | ‘’ |  |  |  |  |  |  |  | Equation variable representing the relationship between overflow spills and Pik loads. As the concentration of P is higher for untreated spill events, the Pik load in the event the If exceeds the 3DWf threshold is added to the effluent P load to generate the Total Phosphorus Load (TPL) of WwTWs k in water body sub-catchment s. The Scottish Water Eden Water Quality Strategic Study sets a suitable concentration of 1.05 mg/l of P for spill events, which is multiplied by the spill volume. The discretisation of the TPLis sets the resilient c value as the current TPL for each water body sub catchment s which is derived from the Scottish Water Eden Water Quality Strategic Study, the high-risk u value is set as three times the c value. The values for L and M risk are then uniformised between c and u (kg/d). |
| Energy Demand Wastewater | EDW | Equation | EDWiks=IFeq(BRik) | State |  | From |  |  | To |  |  | As energy demands within the wastewater system EDW were highlighted by stakeholders as a manufactured capital resource we measured the potential change in energy demand EDW under the influence of differing simulation i at wastewater treatment works k in waterbody sub catchment s. As the Bio Resource is the final variable described by stakeholders in the wastewater system, which is a measure of both flows and accumulations of bio resource materials that require treatment and transportation, we assume any change in bio-resource volumes (m3/day) under simulation i at wastewater treatment works k BRik leads to a change in EDWik. We measure ΔEDWik using IF statement equations IFeq to index the prior distributions of parent variable BRik based on their discretised state thresholds. Each prior state discrete threshold for BRik, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For EDWik the IFeq of prior BRik values is as follows: IF(BRik≥b2,3,IF(BRik≥b1,2,IF(BRik≥c,1,0). For the indexed variable ΔEDWik the resilient threshold c is set at 25% of the total variables included in the equation. The 25% value is selected as it ensures that for a capital resource to be resilient, the majority of parent variables (at least 75%) must fall within a resilient index threshold value. The upper value u set as the maximum possible index value of all variables (three times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Chemical Demand Wastewater | CDW | Equation | CDWis=IFeq(TPLis) | “ |  |  |  |  |  |  |  | As chemical demands within the wastewater system CDW were highlighted by stakeholders as a manufactured capital resource we measured the potential change in chemical demand CDW under the influence of differing simulation i in waterbody sub catchment s. As there was no data available to measure the current chemical demands at wastewater treatment works we assume that a change in total P loads at under different simulation i in in waterbody sub catchment s TPLis leads to a change in CDWis. We measure CDWis using IF statement equations IFeq to index the prior distributions of parent variableTPLis based on their discretised state thresholds. Each prior state discrete threshold for TPLis , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For EDWik the IFeq of prior TPLis values is as follows: IF(TPLis ≥b2,3,IF(TPLis ≥b1,2,IF(TPLis ≥c,1,0). For the indexed variable ΔCDWis we applied the same discretisation method as EDWik. |
| Asset Compliance and Capability Wastewater | ACW | Equation | ACWis=IFeq(DWFik, SPis) | “ |  |  |  |  |  |  |  | The compliance and capability of manufactured assets in the wastewater system ACW was seen as an important manufactured capital resource due to the role of assets in providing wastewater services and meeting licence and environmental standards. Stakeholders highlighted that the influence of future simulations i could influence ACW in waterbody sub catchments s in the future. We measure the current and future ability of ACWis, using variables DWFik and SPis to determine if assets exceed their DWF licence condition and if they are capable of handling extreme flow events. We calculate ACWis using IF statement equations IFeq to index the prior distributions of parent variable DWFik and SPis based on their discretised state thresholds. Each prior state discrete threshold for both DWFik and SPis , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For ACWis the sum of IFeq of prior DWFik and SPisvalues, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable ACWis we applied the same discretisation method as EDWik. |
| Land Management sub-model |  |  |  |  |  |  |  |  |  |  |  |  |
| Land Cover (Ha) | LC | Equation | LCivs=Aivs | See Supporting Information |  |  |  |  |  |  |  | Equation variable the represents land cover area A (Ha) for each simulation i were applied to land cover categories v in waterbody sub-catchments s. Changes in Land Cover was highlighted by stakeholders as a factor for future change that would influence future water and chemical demands which would influence water availability and quality. UKCEH land cover vector maps 1990, 2007 and 2019 (Morton et al., 2020) were used to analyse historic land cover change in the catchment. Extrapolations of land cover change, interpretations from both the SSP narratives and catchment specific knowledge provided by stakeholders were used to create projections for different land cover areas A for simulations i. Current conditions for land cover categories v are represented using the 2019 UKCEH land cover vector map data for the catchment. Land cover percentage proportions of v in sub-catchment water bodes s are taken from the Scottish Water Strategic Study to derive the area A of land cover categories v in each sub-catchment s. The BAU narrative continues catchment land cover change trends from the previous 30 years. BAU trends include increased in arable, urban, woodland and semi-natural land covers, while improved grasslands and coniferous land cover area decreases. The GR narrative assumes a more intensive move from improved grassland land cover to woodland and semi-natural grasslands. For GR assumes an increase in arable land due to the nature of the catchment being prime agricultural land. Urban land cover is reduced in the GR narrative as populations move to more rural areas of the catchment. The FFD narrative assumes a more intensive move from to arable and improved grassland land cover. In the FFD narrative to woodland and semi-natural grassland cover decline. Urban land cover increases in the FFD narrative due to population increases in urban areas of the catchment. States and their discretisation were set based on the land cover value for the different simulations i, for different land cover types v in waterbody sub catchments s. |
| Septic Tanks | ST | Equation | STis=Tis | See Supporting Information |  |  |  |  |  |  |  | The number of septic tanks T for simulation i in waterbody sub-catchments s. Stakeholders identified that septic tanks influence water quality in the catchment. The current number if septic tanks T in water-body sub-catchment s was taken from the Eden Water Quality Strategic Study. The BAU narrative assumes that the number of septic tanks in the catchment will remain the same. A 20% increase in septic tanks is assumed for the GR narrative as more people move to rural areas. For the FFD narrative a 20% decrease in septic tank numbers T is assumed due to population intensification in urban areas. States and their discretisation were set based on the number of septic tanks for the different simulations i, in waterbody sub catchments s. |
| Land Cover Phosphorus Applications(kg/d) | PD | Equation | PDivs=LCivs×γvs×m | State |  | From |  |  | To |  |  | The application demand of P, PD, to agricultural land (kg/d) under varying simulations i for land cover type v in water body sub-catchment s was highlighted as an important land management system component. The demand for P is influenced by the proportion of different land cover categories LCivs and their associated P demands per Ha, which is represented as a coefficient γvs for different land covers v in water body sub catchments s. Current P (kg/d) loadings taken from a ADAS UK Ltd model of rural diffuse pollution provided by SEPA. P loadings per Ha of each land cover type v in water body sub-catchment s are calculated to quantify coefficients for γvs. Different management scenario m influence PDivs. Parameter values for m can be found in Table S3. The discretisation of PDivs sets the resilient c value as the current P loadings for each land cover type in each water body sub catchment s which, the high-risk u value is set as two times the c value. Thresholds for states low (L) and moderate (M) risk, b1 are uniformised between c and u (kg/d). |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | u |  |  |  |
|  |  |  |  | H |  | u |  |  | inf |  |  |  |
| Septic Tank Phosphorus (kg d−1) | STP | Equation | STPis=STis×γis ×m |  |  |  |  |  |  |  |  | The volume of P from Septic Tanks, STP, (kg/d) under varying simulations i was highlighted as an important land management system variable. Current P (kg/d) loadings from septic tanks are taken from a ADAS UK Ltd model of rural diffuse pollution provided by SEPA. P loadings per ST in water body sub-catchment s are calculated to quantify coefficients for γvs. STPis is calculated by multiplying STis by γis. Different management scenario m influence STPis. Parameter values for m can be found in Table S3. The discretisation of STPis sets the resilient c value as the current P loadings for STPis, the high-risk u value is set as two times the c value. Thresholds for states low (L) and moderate (M) risk, b1 are uniformised between c and u (kg/d). |
| Diffuse Phosphorus (kg/d) | DP | Equation | DPivs=(PDivs+STPis)×PAij | ‘’ |  |  |  |  |  |  |  | Stakeholders highlighted that the applications of sources PDivs (kg/d) are a diffuse source of surface water quality issues in the catchment. The volume of diffuse PDivs sources are likely to be influenced by changes in PAij, with increases in high intensity rainfall likely to increase the proportion of PDivs to surface waters (Heathwaite et al., 2005) which is represented in the equation. The discretisation of PDivssets the resilient c value as the current diffuse P loadings for in each water body sub catchment s which are provided in the Scottish Water Eden Water Quality Strategic Study, the high-risk u value is set as three times the c value. Thresholds for states low (L) and moderate (M) risk, b1 are uniformised between c and u (kg/d). |
| Irrigation Demand (ML/year) | ID | Equation | IDis=βs×∆LCivs×1+1-PAij×m | ‘’ |  |  |  |  |  |  |  | Stakeholders highlighted irrigation demand, ID, in the catchment could be influenced by changes in land cover and climate. The variable focuses on surface water abstractions as stakeholder’s highlighted increases in ID could impact future surface water flows. Current ID in each waterbody sub catchment, s, is represented as a truncated normal distribution βs, where mean and standard deviation values are quantified by analysing annual irrigation abstraction licence return data (Ml/year) from 2008-2019 provided by SEPA. To quantify the potential change in IDisthe equation variable multiplies the current ID βs with the % change in LCivs,, ∆LCivs, to represent the change in irrigation demand from the change in arable cover which is the main source of irrigation demand the catchment. βs, is also multiplied by the inverse of PAij to represent the potential change in IDis due to a reduction of PAijincreasing demand and increases in PAij increasing demand. Different management scenario m influence IDis. Parameter values for m can be found in Table S3. The discretisation of IDis sets the resilient c value as 50% of the sum of all licence volumes (ML/year) in each waterbody sub catchment s. The high-risk value u is set as the total licence volume (ML/day) in each waterbody sub catchment s. The values for L and M risk, b1, are then uniformised between c and u (ML/day). |
| Groundwater Nitrate (mg/L) | GN | Equation | GNis=βs+∆LCivs×γvs×PAij | ‘’ |  |  |  |  |  |  |  | Stakeholders highlighted that groundwater nitrate concentrations (mg/l), GN, was an important variable in the catchment system due to its influences drinking water quality, particularly as the catchment falls within a Nitrate Vulnerable Zone (NVZ). The influence of land cover change and precipitation change were identified as important variables that could influence drinking water groundwater sources in the future (Stuart et al., 2011). Current GN concertation in water bodies sub catchments s which include drinking water boreholes are represented as a truncated normal distribution βs, where mean and standard deviation values are quantified by analysing groundwater nitrate samples (mg/l) from 2008-2019 provided by SEPA. To quantify the potential change in GNs the equation variable adds the a change in GN concentration to βs by multiplying the sum of change in ∆LCivs (Ha) with N loadings per Ha of each land cover type v in water body sub-catchment s calculated to quantify coefficients for γvs. βs, is also multiplied by PAij to represent the potential change in GNs due to an increase of PAijincreasing Nitrate leaching rates to groundwater, particularly during higher intensity rainfall events. The discretisation of GNis sets the resilient c value as 50% of the mean threshold values indicative of risks to the quality of water being abstracted, or intended to be abstracted, for human consumption (mg/l) (Scottish Government, 2015) . The high-risk value u is set as the mean threshold value of 37.5 mg/l. The values for L and M risk, b1, are then uniformised between c and u (mg/l). |
| Energy Demand Land Management | EDLM | Equation | EDLMis=IFeq(IDis) | State |  | From |  |  | To |  |  | As energy demands within the land management system EDLM were highlighted by stakeholders as a manufactured capital resource we measured the potential change in energy demand EDLM under the influence of differing simulation i in waterbody sub catchment s. Stakeholder identified the irrigation activities as a key source of energy use in the land management system. As there was no data available to measure current energy use from irrigation abstraction, we use the variable IDis as a measure of the potential direction of change in EDLMis. We calculate EDLMisusing IF statement equations IFeq to index the prior distributions of parent variable IDis based on their discretised state thresholds. Each prior state discrete threshold for IDis, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For ∆EDLMisthe IFeq of prior IDis values is as follows: IF(IDis≥b2,3,IF(IDis≥b1,2,IF(IDis≥c,1,0). For the indexed variable EDLMis the resilient threshold c is set at 25% of the total variables included in the equation. The 25% value is selected as it ensures that for a capital resource to be resilient, the majority of parent variables (at least 75%) must fall within a resilient index threshold value. The upper value u set as the maximum possible index value of all variables (3 times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Chemical Demand Land Management | CDLM | Equation | CDLMis=IFeq(PDivs) | ‘’ |  |  |  |  |  |  |  | As chemical demands within the land management system CDLM were highlighted by stakeholders as a manufactured capital resource we measured the potential change in chemical demand CDLM under the influence of differing simulation i in waterbody sub catchment s. Stakeholder identified the phosphorus applications as a key source of chemical use in the land management system. As there was no data available to measure current chemical use from P applications for land cover types v, we use the variable PDivs for land cover types arable and pasture as a measure of the potential direction of change in CDLMis. We calculate EDLMisusing IF statement equations IFeq to index the prior distributions of parent variable PDivs based on their discretised state thresholds. Each prior state discrete threshold for PDivs resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For CDLMisthe sum of vsiIFeqof prior PDivs values for land cover types arable and pasture is as follows: IF(PDivs≥b2,3,IF(PDivs≥b1,2,IF(PDivs≥c,1,0). For the indexed variable CDLMis the resilient threshold c is set at 25% of the total variables included in the equation. The 25% value is selected as it ensures that for a capital resource to be resilient, the majority of parent variables (at least 75%) must fall within a resilient index threshold value. The upper value u set as the maximum possible index value of all variables (3 times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. |
| Water Resource sub-model |  |  |  |  |  |  |  |  |  |  |  |  |
| Public Commercial Demand | PCD | Deterministic | -- | GR |  |  |  |  |  |  |  | Deterministic variable used to enable varying values for public commercial water resource demand. Stakeholders highlight that the demand for water resource by commercial business could change in the catchment under the varying future pathway simulations i. The water utility business, Scottish Water will have to account for potential changes in commercial demand. |
|  |  |  |  | BAU |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD |  |  |  |  |  |  |  |  |
|  |  |  |  | Current |  |  |  |  |  |  |  |  |
| Leakage | L | Deterministic | -- | GR |  |  |  |  |  |  |  | Deterministic variable used to enable varying values for leakage in from assets as part of the water resource delivery system. Stakeholders highlight that leakage rates could change in the future due to aging assets and the influence of high intensity rainfall under the varying future pathway simulations i. The water utility business, Scottish Water will have to account for potential changes in leakage rates. |
|  |  |  |  | BAU |  |  |  |  |  |  |  |  |
|  |  |  |  | FFD |  |  |  |  |  |  |  |  |
|  |  |  |  | Current |  |  |  |  |  |  |  |  |
| Water Resource Abstraction Demand (Ml/d) | WA | Equation | WAit=(βt+PCit×γ)×PCDit×Lit ×m | State |  | From |  |  |  | To |  | Stakeholders highlighted that the demand for drinking water and water resource services WA is likely to change in the future due to changes in population served in the catchment PC, the demand by commercial business PCD and leakage from assets L. The current demand for water resources (ML/day) βt is derived by analysing annual abstraction data for all Scottish Water boreholes in the catchment from 2014-2018 provided by SEPA and Scottish Water. The change in PC under simulation i for the entire catchment t is multiplied by the coefficient γ which represents the normal consumption rate of 165 l/d per person per day identified by Scottish Water to represent the influence of PC on WA for the entire catchment t. The βt distribution is also multiplied by a % change in PCDit and Lit. As there is limited data to represent changes in PCDit and Lit, therefore % change values for different simulations i are used to represent a direction of change associated with interpretations of SSP narratives. A % reduction in demand and leakage is assumed for the GR simulation due to the associated increased efficient use of water described in the narrative. For the BAU and FFD narrative a % increase values are assumed, with a greater % increase value for the FFD simulation. Different management scenario m influence WAit. Parameter values for m can be found in Table S3. The resilient threshold value c is set at 75% of the nominal borehole capacity (Ml/day) of all boreholes in the catchment and the high-risk threshold values u is set at 95% of the nominal borehole capacity. The values for L and M risk, b1, are then uniformised between c and u (ML/day). |
|  |  |  |  | Resilient |  | 0 |  |  |  | c |  |  |
|  |  |  |  | L |  | c |  |  |  | b1 |  |  |
|  |  |  |  | M |  | b1 |  |  |  | u |  |  |
|  |  |  |  | H |  | u |  |  |  | inf |  |  |
| Water Resource Supply Capacity (Ml/day) | SC | Equation | SCit=βa× ACit×IF(GNis≥u,0, 1) | State |  | From |  |  | To |  |  | Stakeholder identified that the supply capacity (Ml/day) of water resources was a significant component of the catchment system, which could be influenced by groundwater nitrate concentrations GNis (mg/) and changes in the age and condition of assets AC under simulation i. The current borehole capacity βa (ML/day) is derived by analysing abstraction rate data for all Scottish Water boreholes a in the catchment from 2012-2019 provided by Scottish Water. The βa is multiplied by ACit to account for the potential impact of future asset conditions. As there was limited data on the conditions of water resource assets in the catchment, a % change value was assigned for ACit. All simulations GR, BAU and FFD assumed a % reduction in capacity due to changes in ACit based on the assumption that as assets age their efficiency decreases. The FFD had a greater % decrease than BAU and BAU has a greater % decrease than GR. Stakeholder highlighted that future GNis concentrations in groundwater were a potential risk as high concentrations would lead to safe drinking water standards being exceeded. An IF statement is used to represent that IF GNis exceeds its high-risk value u of 37.5 mg/l then a zero value should be returned as safe drinking water standard would be exceeded. The resilient threshold value c is set at 75% of the nominal borehole capacity (Ml/day) of all boreholes in the catchment and the high-risk threshold values u is set at 95% of the nominal borehole capacity. The values for L and M risk, b1, are then uniformised between c and u (ML/day). |
|  |  |  |  | H |  | -10 |  |  | u |  |  |  |
|  |  |  |  | M |  | u |  |  | b1 |  |  |  |
|  |  |  |  | L |  | b1 |  |  | c |  |  |  |
|  |  |  |  | Resilient |  | c |  |  | inf |  |  |  |
| Resilience of Eden Supply (Ml/day) | RS | Equation | RSit=SCit+βooc- WAit | “ |  |  |  |  |  |  |  | Stakeholders highlight that calculation of future supply and demand would provide the best measure of the resilience of the water resource system supply (Ml/day) RS in simulations i across catchment t. The supply volume is measured by calculating the difference between the supply capacity SCit and the abstraction demand WAit. Stakeholders identified that supply capacity in t is supplemented by a supply source out with the catchment. The volume of water supplied from out of the catchment RSOC is represented by βooc. A truncated distribution for βooc was derived using demand for water resources (ML/day) analysing annual abstraction data for the RSOC source from 2014-2018 provided by SEPA and Scottish Water. We use the demand data as a proxy for the supply capacity and is added to SCit. The resilient threshold value c is set at as a positive value where SCit> WAit (Ml/day) and the high risk threshold value u is set at zero where a negative value would suggest WAit > SCit which would lead to a deficit in supply volumes. |
| Resilience of Outside of Catchment Supply (Ml/day) | RSOC | Equation | RSOCit=SCit- WAit | “ |  |  |  |  |  |  |  | Stakeholders highlighted that the abstracted water in the catchment is supplied to populations outside of the catchment boundary RSOC. The supply volume is measured by calculating the difference between the supply capacity SCit and the abstraction demand WAit. The outside of catchment supply is not supplemented by any other source and is dependent on the supply from within the Eden catchment. We include RSOCit as a variable to measure if there is enough supply capacity SCit (ML/day) to supply populations both inside and outside of the catchment. The resilient threshold value c is set at as a positive value where SCit > WAit (Ml/day) and the high risk threshold value u is set at zero where a negative value would suggest WAit > SCit which would lead to a deficit in supply volumes. |
| Energy Demand Water Resources | EDWR | Equation | EDWRit=IFeq(WAit) | State |  | From |  |  | To |  |  | As energy demands within the water resource system EDWR were highlighted by stakeholders as a manufactured capital resource we measured the potential change in energy demand EDWR under the influence of differing simulation i across the catchment t. Stakeholder identified the abstraction of drinking water as a key source of energy use in the water resource system. As there was no data available to measure current energy use from drinking water abstraction, we use the variable WAit as a measure of the potential direction of change in EDWRit. We calculate EDWRit using IF statement equations IFeq to index the prior distributions of parent variable WAit based on its discretised state thresholds. Each prior state discrete threshold for WAit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For EDWRit the IFeq of prior WAit values is as follows: IF(WAit≥b2,3,IF(WAit≥b1,2,IF(WAit≥c,1,0). For the indexed variable EDWRit the resilient threshold c is set at 25% of the total variables included in the equation. The 25% value is selected as it ensures that for a capital resource to be resilient, the majority of parent variables (at least 75%) must fall within a resilient index threshold value. The upper value u set as the maximum possible index value of all variables (3 times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Chemical Demand Water Resources | CDWR | Equation | CDWRit=IFeq(WAit) | ‘’ |  |  |  |  |  |  |  | As chemical demands within the water resource system EDWR were highlighted by stakeholders as a manufactured capital resource we measured the potential change in chemical demand CDWR under the influence of differing simulation i across the catchment t. Stakeholder identified the treatment of abstracted drinking water as a key source of chemical use in the water resource system. As there was no data available to measure current chemical use from drinking water abstraction, we use the variable WAit as a measure of the potential direction of change in CDWRit. We calculate CDWRit using IF statement equations IFeq to index the prior distributions of parent variable WAit based on its discretised state thresholds. Each prior state discrete threshold for WAit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For CDWRit the IFeq of prior WAit values is as follows: IF(WAit≥b2,3,IF(WAit≥b1,2,IF(WAit≥c,1,0). For the indexed variable ΔCDWRit we applied the same discretisation method as EDWRit. |
| Asset Capability Water Resources | ACWR | Equation | ACWRit=IFeq(RSit, RSOCit) | ‘’ |  |  |  |  |  |  |  | The capability of manufactured assets ACWR to supply water resource to populations both within and out with the catchment t was identified by stakeholders as a key manufactured capital of the water resource system. Stakeholders highlighted that the influence of future simulations i could influence ACWRit in the future. As there was no specific data available to measure current asset capability to supply water resource, we use the variables RSit and RSOCit as a measure of the current and potential direction of change in ACWRit. We calculate ACWRit using IF statement equations IFeq to index the prior distributions of parent variable RSit and RSOCit based on their discretised state thresholds. Each prior state discrete threshold for both RSit and RSOCit , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For ACWRit the sum of IFeq of prior RSit and RSOCit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable ACWRit we applied the same discretisation method as EDWRit. |
| Customer Complaints | CuC | Equation | CuCit=IFeq(RSit,RSOCit) |  |  |  |  |  |  |  |  | The potential complaints of customers CuC due to interruptions of water resource both within and out with the catchment t was identified by stakeholders as a key social and intellectual capital risk associated with the water resource system. Stakeholders highlighted that the influence of future simulations i could influenceCuCit in the future. As there was no specific data available to measure current CuCit, we use the variables RSit and RSOCit as a measure of the current and potential direction of change inCuCit, assuming that if there is no interruption to supply, there will be no complaints and if there is an interruption to supply then there is the potential for complaints. We calculate CuCit using IF statement equations IFeq to index the prior distributions of parent variable RSit and RSOCit based on their discretised state thresholds. Each prior state discrete threshold for both RSit and RSOCit , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. ForCuCit the sum of IFeq of prior RSit and RSOCitvalues, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable CuCit we applied the same discretisation method as EDWRit. |
| Food Production sub-model |  |  |  |  |  |  |  |  |  |  |  |  |
| Crop Cover (Ha) | CC | Equation | CCizt=Aizt | See supporting information. |  |  |  |  |  |  |  | Stakeholders highlighted that food production was an important component of the catchment, as it is predominantly covered in arable land. Stakeholders highlighted The area A of different crop types z grown in the catchment CC could vary under different simulations i across the catchment t. The current Aizt by calculating the average current % proportion of crop cover z in the catchment based upon analysis of the UKCEH Land Cover® Plus: Crops © 2016-2020 UKCEH. © RSAC. © Crown Copyright 2007, Licence number 100017572. We then convert the % proportion to a Ha proportion using the LCivs area for arable land cover category for each simulation i. States and their discretisation were set based on the land cover value for the different simulations i, for different crop cover types z |
| Crop Yields (t/Ha) | CY | Equation | CYijzt=βijzt | State |  | From |  |  | To |  |  | Crop yields CY (t/ha) could for crop type z differ under future change simulations i and precipitation change j across the catchment t. To measure current CY for crop types z we analysed crop yield data from 2010-2019 available from the Scottish Agriculture Tables from the Economic Report 2020 to produce a truncated normal distribution βijzt. For future simulation types i and precipitation change j we applied a range of crop yield values taken from The Farm Management Handbook 2020/21 produced by SAC Consulting. Lower yield range values are applied to extreme precipitation change simulations from the Farm Management Handbook. For all simulations, an increase in yield (t/ha) is applied for each simulation based on UK SSP narratives, with FFD simulation applying the highest yield increase and GR having the lowest yield increase. The resilient threshold value c is set at as the current mean yield (t/ha) for crop types z. The high-risk threshold u is set at 50% of the c value (t/ha). The values for L and M risk, b1, are then uniformised between c and u (t/ha). |
|  |  |  |  | H |  | 0 |  |  | u |  |  |  |
|  |  |  |  | M |  | u |  |  | b1 |  |  |  |
|  |  |  |  | L |  | b1 |  |  | c |  |  |  |
|  |  |  |  | Resilient |  | c |  |  | inf |  |  |  |
| Crop Price (£/t) | CP | Equation | CPizt=βizt | State |  | From |  |  | To |  |  | Crop Prices CP (£/t) for crop type z could differ under future change simulations i across the catchment t. To measure current CP for crop types z we analysed crop price data from 2010-2019 available from Scottish Agriculture Tables from the Economic Report 2020 to produce a truncated normal distribution βizt. Future CPiztvalue assumptions βizt follow an increase in price based on SPP narratives, where FFD assumes the greatest increase (15%), BAU (10%) and GR (5%) to give an estimation of direction of change. The resilient threshold value c is set at as the current mean price (£/t) for crop types z. The high-risk threshold u is set at 50% of the c value (£/t). The values for L and M risk are then uniformised between c and u (£/t). |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b |  |  |  |
|  |  |  |  | M |  | b |  |  | u |  |  |  |
|  |  |  |  | H |  | u |  |  | inf |  |  |  |
| Fertiliser Costs (£/ha) | FCost | Equation | FCostizt=βizt | “ |  |  |  |  |  |  |  | Fertiliser Costs FCost (£/ha) for crop type z could differ under future change simulations i across the catchment t. To measure current FCost for crop types z we analysed literature parameters of Fertiliser costs 2016-2020 available from The Farm Management Handbooks produced by SAC Consulting to produce a truncated normal distribution βizt. Future FCostizt βizt value assumptions are based on SPP narratives and the input from stakeholders regarding concerns regarding future P supplies. The GR simulation assumes a decrease on costs (10%), BAU assumes an increase in costs (10%) and FFD assumes a higher increase (50%).The resilient threshold value c is set at as the current mean cost (£/ha) for crop types z. The high-risk threshold u is set as double the c value (£/ha). The values for L and M risk, b1, are then uniformised between c and u (£/ha). |
| Total Crop Margin (£M) | TCM | Equation | TCMijt=z(Aizt × CYijzt×CPizt)-(FCostizt × Aizt) | ‘’ |  |  |  |  |  |  |  | The total crop margin TCM (£M) was identified as a key representation of food production in the catchment. To calculate the sum of TCM under simulation i for precipitation simulation j for all crop types z for the entire catchment t by multiplying the area A (ha) of each crop type z with the total yield CY (t/Ha) for each crop type z, to give the total tonnage of each crop type z which is then multiplied by the price CP (£/t) of each crop type z to give the total output (£M). The cost of fertiliser FCost (£/ha) for each crop type z multiplied by A for crop type z which is subtracted from the total output (£M). We acknowledge that there are other variable and fixed costs which influence the margins of z however, there was limited data available to consider these costs. We therefore only consider the variables we can measure to provide a strategic consideration of how margins, based on the variables included, will differ between current and future simulations. To measure TCMijt the equation calculates the sum of income minus costs for all crop types z. The resilient threshold value c is set at as the current mean cost (£/ha) for crop types z. The high-risk threshold u is set at 50% of the c value (£/ha). The values for L and M risk are then uniformised between c and u (£/ha). |
| Capital Resources sub-model |  |  |  |  |  |  |  |  |  |  |  |  |
| Surface Water Flows (Ml/day) | SWF | Equation | SWFis=βs×PAij-IDisγ | State |  | From |  |  | To |  |  | Stakeholders identified surface water flows SWF (ML/day) as a significant natural capital resource in the catchment, which could be influence by future climatic change PAij and demands for irrigation IDis. Current flows (ML/day) for each waterbody sub catchment s are represented by a customised distribution βs which was derived by analysing river discharge data 2010-2019 for available waterbodies s provided by SEPA. The distribution of βs is multiplied by the potential anomaly change in PAij. Simulated changes in IDis - which is converted from Ml/yr to ML/day using γ – is subtracted from SWFis. As there can be high risk of low flows (H1) and high flows (H2), two high risk threshold values u1, u2 are derived by taking the Q5 and Q95 (ML/day) values from the analysed river discharge data for waterbodies s. The resilient threshold value c set as the median value of the river discharge data. The value for L2 is uniformised between c and u2 (ML/day). |
|  |  |  |  | H1 |  | 0 |  |  | u1 |  |  |  |
|  |  |  |  | M1 |  | u1 |  |  | b1 |  |  |  |
|  |  |  |  | L1 |  | b1 |  |  | c |  |  |  |
|  |  |  |  | Resilient |  | c |  |  | b2 |  |  |  |
|  |  |  |  | L2 |  | b2 |  |  | b3 |  |  |  |
|  |  |  |  | M2 |  | b3 |  |  | u2 |  |  |  |
|  |  |  |  | H2 |  | u2 |  |  | inf |  |  |  |
| Surface Water Quality (mg/l) | SWQ | Equation | EQ1. SWQis=TPLis+DPivsSWFis×γ EQ2. SWQis=βs×∆TPLis×∆DPivs | State |  | From |  |  | To |  |  | The quality of surface water SWQ was a current issue in the catchment, particularly in regard to P concentrations (mg/l) due to the influence of effluent loads TPLis (kg d−1) and diffuse loads DPivs (kg d−1), was highlighted by stakeholders. Surface water flows SWFis (ML/day) were also identified as influencing P concentrations, with higher flows diluting concentrations and low flows having the opposite effect on P concentrations. To simulate both current and future influence of TPLis, DPivs and SWFison SWQis . As SWFis data wasn’t available for all waterbodies s, EQ1 is used where SWFis data was available. The unit conversion coefficient γ is applied. Where SWFis wasn’t available EQ2 is used which derives a customised distribution βs current P concentration using random water quality sampling data 2010-2019 provided by SEPA at the end of waterbodies s where flow data was not available. The influence of future TPLis and DPivs on SWQis is estimated by multiplying βs by the % change in ∆TPLis and ∆DPivs as a measure of the direction of change in P concentrations. For both EQ1 and EQ2 the resilient c threshold value is set as the good/moderate Water Framework Directive (WFD) status threshold (mg/l) determined by SEPA for each waterbody s. The high-risk value threshold u is set as the poor/bad WFD status threshold. The L and M risk threshold value b1 is set as the moderate/poor WFD status threshold. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | u |  |  |  |
|  |  |  |  | H |  | u |  |  | inf |  |  |  |
| Soil Erosion | SE | Equation | SEis=βs × PAij | ‘’ |  |  |  |  |  |  |  | Stakeholders highlighted that soil in the catchment is important for supporting and protecting the natural environment and food production. Currently, soil is at risk to the impacts of intense and prolonged rainfall events. The potential future change in extreme rainfall events was seen as an influencing factor on future soil erosion SEis. To measure the current risk of soil erosion we analysed maps of the risk of soil erosion by water produced by Lilly (2018) using ArcGIS pro for each waterbody sub-catchment s to produce point raster data for mineral soil risk class. The soil risk maps give each raster point one of nine of the risk classification values which are divided evenly between low, moderate and high. We assigned each risk classification with a value of between zero and eight. Values of risk classification were analysed in each waterbody sub catchment s to produce a truncated distribution βs of current risk of soil erosion by water. To estimate the potential impact of changes in rainfall on SEis, we multiply βs by PAij. The resilient c threshold value is set at the lowest risk classification value and the high-risk threshold values u is set using the first high risk classification value. The M risk value b1 is set using the first moderate risk classification value. |
| Surface Water Flow | SWFC | Equation | SWFCit=sIFeq(SWFis) | State |  | From |  |  | To |  |  | As surface water flow SWFis was highlighted by stakeholders as a key natural capital resource we measured the overall state of SWF in the catchment t under simulations i. We calculate the overall capital resource state values using IF statement equations to index the prior distributions of parent variables SWFis based in their discretised state thresholds. Each prior state discrete threshold, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u1, u2, b1, b2, b3. For SWFCt the sum IFeq of prior SWFis values are as follows: IF(SWFis≤u1,3,IF(SWFis≤b1,2, IF(SWFis≤c,1,IF(SWFis≥u2,3,IF(SWFis≥b3,2,IF(SWFis≥b2,1,0). For the indexed capital resource variable SWFCt the resilient threshold c is set at 25% of the total variables included in the equation. The 25% value is selected as it ensures that for a capital resource to be resilient, the majority of parent variables (at least 75%) must fall within a resilient index threshold value. The upper value u set as the maximum possible index value of all variables (3 times the number of parent variables). Threshold values for high and moderate risk, b1 and b2 are uniformised values between c and u. The discretisation method is consistent for all capital resources and their capitals. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Surface Water Quality | SWQC | Equation | SWQCit=IFeq(SWQis) | State |  | From |  |  | To |  |  | As surface water quality SWQis was highlighted by stakeholders as a key natural capital resource we measured the overall state of SWQ in the catchment t under simulations i. We calculate the overall capital resource state values using IF statement equations index the prior distributions of parent variables SWQis based in their discretised state thresholds. Each prior state discrete threshold for SWQis, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For SWQCit the sum IFeq of prior SWQis values is as follows: IF(SWQis≥b2,3,IF(SWQis≥b1,2,IF(SWQis≥c,1,0). We apply the same discretisation methods described for all capital resources as described for the variable SWFC. |
|  |  |  |  | Resilient |  | 0 |  |  | c |  |  |  |
|  |  |  |  | L |  | c |  |  | b1 |  |  |  |
|  |  |  |  | M |  | b1 |  |  | b2 |  |  |  |
|  |  |  |  | H |  | b2 |  |  | u |  |  |  |
| Flood Risk | FR | Equation | FRis=IFeq(SWFis) | “ |  |  |  |  |  |  |  | As stakeholders highlighted the risk of flooding as a potential impacts due to the influence of future change on SWFis, we measure flood risk under differing simulations i in waterbody sub catchment s values FRis using IF statement equations to index the prior distributions of parent variables SWFis based on their discretised state thresholds. Each prior state discrete threshold, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of, u2, b2, b3 only as lower flows would not influence flood risk. For FRis the sum IFeq of prior SWFis values is as follows: IF(SWFis≥u2,3,IF(SWFis≥b3,2,IF(SWFis≥b2,1,0). We apply the same discretisation methods described for all capital resources as described for the variable SWFC. |
| Soil | S | Equation | Sit=sIFeq(SEis) | “ |  |  |  |  |  |  |  | As soil was highlighted as a key natural capital resource, we use the same equation as SWQCit provide a measure of the state of soils under the influence future simulations i for the overall conditions in the catchment t. We apply the same discretisation methods described for all capital resources as described for the variable SWFCit. |
| Air Quality | AQ | Equation | AQit=stIFeq(EDWisEDLMisEDWRit) | “ |  |  |  |  |  |  |  | Air quality AQ was identified as a key natural capital resource, summarised by stakeholders as a reflection of the amount of emissions produced in the catchment t. Stakeholders highlighted that the influence of future simulations i could influence AQit in the future. As there was no specific data available to measure current AQit, we use the variables EDWis,EDLMis and EDWRit as to measure of the potential direction of change in AQit as stakeholder identified the relationship between energy demand and emissions. We calculate AQit using IF statement equations IFeqto index the prior distributions of parent variable EDWis,EDLMis and EDWRit based on their discretised state thresholds. Each prior state discrete threshold for EDWis,EDLMis and EDWRit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For AQit the sum of IFeq of prior EDWis,EDLMis and EDWRit values, which we will denote as α is as follows: : IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable AQit we applied the same discretisation method as SWFC. |
| Groundwater Quality | GWQ | Equation | GWQit=sIFeq(GNis) | “ |  |  |  |  |  |  |  | Air quality GWQ was identified as a key natural capital resource, summarised by stakeholders as a reflection of the amount of emissions produced. Stakeholders highlighted that the influence of future simulations i could influence GWQ in the catchment t GWQitin the future. As stakeholders specifically highlight GNis as the specific compound of interest when discussing GWQ. The proportions of the catchment are also designated as a Nitrate Vulnerable Zone (NVZ). To measure of the current conditions and potential direction of change we use GNis as an indicator of GWQit. We calculate GWQit using IF statement equations IFeqto index the prior distributions of parent variable GNis based on their discretised state thresholds. Each prior state discrete threshold for GNis, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For GWQit apply a IFeq of prior GNis values, as follows: IF(GNis≥b2,3,IF(GNis≥b1,2,IF(GNis≥c,1,0). For the indexed variable GWQit we applied the same discretisation method as SWFC. |
| Energy Demand Change SW | SWED | Equation | SWEDit=stIFeq(EDWis, EDWRit ) | “ |  |  |  |  |  |  |  | The change in energy demand, specifically for Scottish Water assets, was identified as a key manufactured capital resource SWED. Stakeholders required an overall understanding of the change in energy demand under simulations i for all Scottish Water related systems in the catchment t, SWEDit. Differentiating the energy demand across different sectors in the catchment would allow for the extent of the influence on each sector to be measured. We used the variables EDWisand EDWRit to measure of the potential direction of change in SWEDit We calculate SWEDitusing IF statement equations IFeqto index the prior distributions of parent variable EDWis and EDWRit based on their discretised state thresholds. Each prior state discrete threshold for EDWis and EDWRit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. ForSWEDit the sum of IFeq of prior EDWis and EDWRit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable SWEDit we applied the same discretisation method as SWFC. |
| Chemical Demand Change SW | SWCD | Equation | SWCDit=stIFeq(CDWis, CDWRit) | “ |  |  |  |  |  |  |  | The change in chemical demand, specifically for Scottish Water related systems, was identified as a key manufactured capital resource SWCD. Stakeholders required an overall understanding of the change in chemical demand under simulations i for all Scottish Water related systems in the catchment t, SWCDit. Differentiating the chemical demand across different sectors in the catchment would allow for the extent of the influence on each sector to be measured. We used the variables CDWisand CDWRit to measure of the potential direction of change in SWCDit We calculate SWCDitusing IF statement equations IFeqto index the prior distributions of parent variable CDWis and CDWRit based on their discretised state thresholds. Each prior state discrete threshold for CDWis and CDWRit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. ForSWCDit the sum of IFeq of prior CDWis and CDWRit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable SWCDit we applied the same discretisation method as SWFC. |
| Asset Compliance & Capability | ACC | Equation | ACCit=stIFeq(ACWis, ACWRit) | “ |  |  |  |  |  |  |  | Stakeholders required an overall understanding of the change in asset compliance and capability under simulations i for related Scottish Water related systems in the catchment t, ACCit. We used the variables ACWisand ACWRit to measure of the potential direction of change in ACCit We calculate ACCitusing IF statement equations IFeqto index the prior distributions of parent variableACWisand ACWRit based on their discretised state thresholds. Each prior state discrete threshold for CDWis and CDWRit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For ACCit the sum of IFeq of prior ACWisand ACWRit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable ACCit we applied the same discretisation method as SWFC. |
| Energy Demand Change LM | LMED | Equation | LMEDit=sIFeq(EDLMis) | “ |  |  |  |  |  |  |  | The change in energy demand, specifically for the land management system, was identified as a key manufactured capital resource LMED. Stakeholders required an overall understanding of the change in energy demand under simulations i for all land management systems in the catchment t, SWEDit. Differentiating the energy demand across different sectors in the catchment would allow for the extent of the influence on each sector to be measured. We used the variable EDLMis to measure of the potential direction of change in LMEDit We calculate LMEDitusing IF statement equations IFeqto index the prior distributions of parent variable EDLMis based on their discretised state thresholds. Each prior state discrete threshold for EDLMis, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For LMEDit the sum of IFeq of prior EDLMis values, which we will denote as α is as follows: IF(EDLMis≥b2,3,IF(EDLMis≥b1,2,IF(EDLMis≥c,1,0). For the indexed variable LMEDit we applied the same discretisation method as SWFC. |
| Chemical Demand Change LM | LMCD | Equation | LMCDit=sIFeq(CDLMis) | “ |  |  |  |  |  |  |  | The change in energy demand, specifically for the land management system, was identified as a key manufactured capital resource LMED. Stakeholders required an overall understanding of the change in energy demand under simulations i for all land management systems in the catchment t, SWEDit. Differentiating the energy demand across different sectors in the catchment would allow for the extent of the influence on each sector to be measured. We used the variable EDLMis to measure of the potential direction of change in LMEDit We calculate LMEDitusing IF statement equations IFeqto index the prior distributions of parent variable EDLMis based on their discretised state thresholds. Each prior state discrete threshold for EDLMis, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For LMEDit the sum of IFeq of prior EDLMis values, which is as follows: IF(EDLMis≥b2,3,IF(EDLMis≥b1,2,IF(EDLMis≥c,1,0). For the indexed variable LMEDit we applied the same discretisation method as SWFC. |
| Community Relationship | CRS | Equation | CRSit=stIFeq(SPis, FRis,SWQit,AQit,CuCit) | “ |  |  |  |  |  |  |  | The relationship with local communities CRS in the catchment t was identified as a key social capital resource by stakeholders. Stakeholders highlight that the influence of simulations i for could influence CRSit in the future. As there is no specific measure for community relationship, stakeholders highlighted that current and future conditions of variables SPis, FRis,SWQit,AQit and CuCit could influence CRSit. We calculate CRSit using IF statement equations IFeqto index the prior distributions of parent variables SPis, FRis,SWQit,AQit and CuCit based on their discretised state thresholds. Each prior state discrete threshold for SPis, FRis,SWQit,AQit and CuCit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. ForCRSit the sum of IFeq of prior CRSit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable CRSit we applied the same discretisation method as SWFC. |
| Water Treatment & Supply Costs | WTS | Equation | WTSit=IFeq(SWEDit, SWCDit,ACCit) | “ |  |  |  |  |  |  |  | Changes in costs associated with the treatment and supply of water resources WTS in the catchment t was identified as a key financial capital resource by stakeholders. Stakeholders highlight that the influence of simulations i for could influence WTSit in the future. Stakeholders identified that future conditions of variables SWEDit, SWCDit and ACCit could influence WTSit. We calculate WTSit using IF statement equations IFeqto index the prior distributions of parent variables SWEDit, SWCDit and ACCit based on their discretised state thresholds. Each prior state discrete threshold for SWEDit, SWCDit and ACCit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For WTSit the sum of IFeq of prior SWEDit, SWCDit and ACCit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable ACCit we applied the same discretisation method as SWFC. |
| Food Production | FPF | Equation | FPFit=IFeq(TCMijt,LMEDit, LMCDit,) | “ |  |  |  |  |  |  |  | Changes in income and costs associated with food production FPF in the catchment t was identified as a key financial capital resource by stakeholders. Stakeholders highlight that the influence of simulations i for could influence FPFit in the future. Stakeholders identified that current and future conditions of variables TCMijt,LMEDit and LMCDit could influence FPFit. We calculate FPFit using IF statement equations IFeqto index the prior distributions of parent variables TCMijt,LMEDit and LMCDit based on their discretised state thresholds. Each prior state discrete threshold for TCMijt,LMEDit and LMCDit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For FPFit the sum of IFeq of prior TCMijt,LMEDit and LMCDitvalues, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable FPFit we applied the same discretisation method as SWFC |
| Reputation | R | Equation | Rit=stIFeq(SPis, FRis,CuCit) | “ |  |  |  |  |  |  |  | The reputation of sectors R in in the catchment t was identified as a key intellectual capital resource by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence Rit in the future. As there was no specific measure of reputation, we used the variables identified by stakeholders that they believe influence reputation. Stakeholders identified SPis, FRis and CuCit could influence Rit. We calculate Rit using IF statement equations IFeqto index the prior distributions of parent variables SPis, FRis and CuCit based on their discretised state thresholds. Each prior state discrete threshold forSPis, FRis and CuCit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For Rit the sum of IFeq of prior SPis, FRis and CuCitvalues, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable Rit we applied the same discretisation method as SWFC |
| Capital Outputs |  |  |  |  |  |  |  |  |  |  |  |  |
| Natural Capital | NC | Equation | NCit=IFeq(SWQCit,SWFCit,AQit,Sit,GWQit) | “ |  |  |  |  |  |  |  | The overall measure of natural capital NC in in the catchment t was required by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence NCit in the future. As there was no specific measure of natural capital, we used the natural capital resource variables identified by stakeholders to measure both the current and future condition of natural capital NCit. Stakeholders identified SWQCit,SWFCit,AQit,Sit and GWQit as capital resources for NCit. We calculate NCit using IF statement equations IFeqto index the prior distributions of parent variables SWQCit,SWFCit,AQit,Sit and GWQit based on their discretised state thresholds. Each prior state discrete threshold for SWQCit,SWFCit,AQit,Sit and GWQit , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For NCit the sum of IFeq of prior SWQCit,SWFCit,AQit,Sit and GWQit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable NCit we applied the same discretisation method as SWFC |
| Social Capital | SC | Equation | SCit=IFeq(CRSit) | “ |  |  |  |  |  |  |  | The overall measure of social capital SC in in the catchment t was required by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence SCit in the future. As there was no specific measure of social capital, we used the social capital resource variable CRSit identified by stakeholders to measure both the current and future condition of social capital SCit. We calculate SCit using IF statement equations IFeqto index the prior distributions of parent variable CRSit based on their discretised state thresholds. Each prior state discrete threshold for CRSit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For SCit the IFeq of prior CRSit values is as follows: IF(CRSit≥b2,3,IF(CRSit≥b1,2,IF(CRSit≥c,1,0). The overall discretised output for SCit will be equal to CRSit, however, we retain both variables to ensure model structure continuity. For the indexed variable SCit we applied the same discretisation method as SWFC. |
| Manufactured Capital | MC | Equation | MCit=IFeq(SWEDit, SWCDit,LMEDit,LMCDit, ACCit) | “ |  |  |  |  |  |  |  | The overall measure of manufactured capital MC in in the catchment t was required by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence MCit in the future. As there was no specific measure of manufactured capital, we used the manufactured capital resource variables identified by stakeholders to measure both the current and future condition of manufactured capital MCit. Stakeholders identified SWEDit, SWCDit,LMEDit,LMCDit and ACCit as manufactured capital resources for MCit. We calculate MCit using IF statement equations IFeqto index the prior distributions of parent variables SWEDit, SWCDit,LMEDit,LMCDit and ACCit based on their discretised state thresholds. Each prior state discrete threshold for SWEDit, SWCDit,LMEDit,LMCDit and ACCit , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For MCit the sum of IFeq of prior SWEDit, SWCDit,LMEDit,LMCDit and ACCit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b1,2,IF(α≥c,1,0). For the indexed variable MCit we applied the same discretisation method as SWFC. |
| Financial Capital | FC | Equation | FCit=IFeq(WTSit, FPFit) | “ |  |  |  |  |  |  |  | The overall measure financial capital FC in in the catchment t was required by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence FCit in the future. As there was no specific measure of financial capital, we used the financial capital resource variables identified by stakeholders to measure both the current and future condition of financial capital FCit. Stakeholders identified WTSit and FPFit as financial capital resources for FCit. We calculate FCit using IF statement equations IFeqto index the prior distributions of parent variables WTSit and FPFit based on their discretised state thresholds. Each prior state discrete threshold for WTSit and FPFit , resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For FCit the sum of IFeq of prior WTSit and FPFit values, which we will denote as α is as follows: IF(α≥b2,3,IF(α≥b2,2,IF(α≥c,1,0). For the indexed variable FCit we applied the same discretisation method as SWFC. |
| Intellectual Capital | IC | Equation | ICit=IFeq(Rit) | “ |  |  |  |  |  |  |  | The overall measure of intellectual capital SC in in the catchment t was required by stakeholders. Stakeholders highlighted that the influence of simulations i for could influence ICit in the future. As there was no specific measure of intellectual capital, we used the intellectual capital resource variable Rit identified by stakeholders to measure both the current and future condition of intellectual capital ICit. We calculate ICit using IF statement equations IFeqto index the prior distributions of parent variable Rit based on their discretised state thresholds. Each prior state discrete threshold for Rit, resilient to high-risk, was assigned a value of zero, one, two or three based on the values of c, u, and b. For ICit the IFeq of prior Rit values is as follows: IF(Rit≥b2,3,IF(Rit≥b1,2,IF(Rit≥c,1,0). The overall discretised output for ICit will be equal to Rit, however, we retain both variables to ensure model structure continuity. For the indexed variable ICit we applied the same discretisation method as SWFC. |
| Good Status | GS | Equation | GS=IF(SWQism) | State | From |  |  | To |  |  |  | SWQ variables are discretised using threshold values set by the Water Framework Directive (WFD) status threshold (mg/l) values determined by SEPA for each waterbody sub-catchment s, across future change scenarios i and management scenarios m Using the example of SWQ in sub-catchment 6200 using the IF statement equation can be explained as follows: IF the reactive phosphorus concentration in waterbody sub-catchment 6200 is >= 1046 µg L−1, then compute the index value 3 (bad status), >= 191 µg L−1, 2 (poor status), >=78, 1 (moderate status), else 0 (good status). A sum of all indexed IF statement equation values for 10,000 samples is generated for GS. The sum must equal zero for good status to be achieved. The maximum value is 15 (3x the five parent variables). Moderate, poor and bad threshold values are uniformised between 1 (c) and 15 (u). |
|  |  |  |  | Good | 0 |  |  | c |  |  |  |  |
|  |  |  |  | Moderate | c |  |  | b1 |  |  |  |  |
|  |  |  |  | Poor | b1 |  |  | b2 |  |  |  |  |
|  |  |  |  | Bad | b2 |  |  | u |  |  |  |  |

## B: Bayesian Network parameter values

Table S2: Bayesian Network parameter values their sources and discretisation thresholds

| Variable | Parameter Values |  |  |  |  |  |  |  |  |  |  |  |  |  | Parameter Source | Discretisation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precipitation Change Anomaly (%) | i | j | k | s/a | v | z | β |  |  | γ | PC/A/T |  |  |  | UK Climate Projection User Interface product Anomalies for probabilistic projections (25km) over UK, 1961-2100 for β. | c | b1 | b2 | u |
|  |  |  |  |  |  |  | M | SD | Trunc |  | GR | BAU | FFD | Current |  |  |  |  |  |
|  | GR | ExLP | -- | -- | -- | -- | 0.74 | 0.1 | -- | -- | -- | -- | -- | -- |  | States are discretised based on j values and are available in Table D1. |  |  |  |
|  | GR | BAU | -- | -- | -- | -- | 1.05 | 0.5 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | ExHP | -- | -- | -- | -- | 1.36 | 0.12 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | BAU | ExLP | -- | -- | -- | -- | 0.72 | 0.09 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | BAU | BAU | -- | -- | -- | -- | 1.05 | 0.05 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | BAU | ExHP | -- | -- | -- | -- | 1.4 | 0.19 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | ExLP | -- | -- | -- | -- | 0.67 | 0.13 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | ExHP | -- | -- | -- | -- | 1.49 | 0.15 | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
| Dry Weather Flow (Ml/day) | -- | -- | Cupar | 6200 | -- | -- | 1.39 | 0.4 | 0 | 0.002 | 1408 | 2816 | 5554 | 0 | Scottish Water Eden Water Quality Strategic Study. We simulated effluent flows using effluent flow summary statistics to generate Q80 value of the outputs to derive β. See Table D1 for γ source. PC Values for GR, BAU and FFD are derived from Scottish Water Growth Model outputs. | 3.2 | 6.4 | -- | 9.6 |
|  | -- | -- | Springfield | 6200 | -- | -- | 0.89 | 0.13 | 0 | 0.002 | 6 | 12 | 668 | 0 |  | 1.17 | 2.34 | -- | 3.51 |
|  | -- | -- | Freuchie | 6200 | -- | -- | 0.26 | 0.07 | 0 | 0.002 | -176 | -352 | 442 | 0 |  | 0.28 | 0.56 | -- | 0.84 |
|  | -- | -- | Dairsie | 6200 | -- | -- | 0.09 | 0.01 | 0 | 0.002 | 37 | 74 | 178 | 0 |  | 0.1 | 0.2 | -- | 0.3 |
|  | -- | -- | Strathmiglo | 6201 | -- | -- | 0.56 | 0.08 | 0 | 0.002 | -153 | -306 | 367 | 0 |  | 0.28 | 0.56 | -- | 0.84 |
|  | -- | -- | Bowhouse | 6201 | -- | -- | 1.53 | 0.24 | 0 | 0.002 | -178 | -356 | 1116 | 0 |  | 1.94 | 3.89 | -- | 5.83 |
|  | -- | -- | Ceres | 6202 | -- | -- | 0.25 | 0.03 | 0 | 0.002 | -170 | -340 | -408 | 0 |  | 0.27 | 0.54 | -- | 0.81 |
|  | -- | -- | Pitscottie | 6202 | -- | -- | 0.03 | 0.005 | 0 | 0.002 | -15 | -30 | -54 | 0 |  | 0.04 | 0.08 | -- | 0.12 |
|  | -- | -- | Letham | 6206 | -- | -- | 0.06 | 0.009 | 0 | 0.002 | -27 | -54 | -65 | 0 |  | 0.07 | 0.14 | -- | 0.21 |
| Daily Effluent Flow (Ml/day) | -- | -- | Cupar | 6200 | -- | -- | 2.27 | 1.14 | 0 | 0.002 | 1408 | 2816 | 5554 | 0 | Scottish Water Eden Water Quality Strategic Study. We used survey samples where available and SAGIS outputs in absent survey statistics for β. See Table D1 for γ source | 9.6 | 14.4 | -- | 19.2 |
|  | -- | -- | Springfield | 6200 | -- | -- | 1.14 | 0.29 | 0 | 0.002 | 6 | 12 | 668 | 0 |  | 3.51 | 5.27 | -- | 7.02 |
|  | -- | -- | Freuchie | 6200 | -- | -- | 0.66 | 0.18 | 0 | 0.002 | -176 | -352 | 442 | 0 |  | 0.84 | 1.26 | -- | 1.68 |
|  | -- | -- | Dairsie | 6200 | -- | -- | 0.13 | 0.07 | 0 | 0.002 | 37 | 74 | 178 | 0 |  | 0.3 | 0.45 | -- | 0.6 |
|  | -- | -- | Strathmiglo | 6201 | -- | -- | 0.81 | 0.42 | 0 | 0.002 | -153 | -306 | 367 | 0 |  | 0.84 | 1.26 | -- | 1.68 |
|  | -- | -- | Bowhouse | 6201 | -- | -- | 2.16 | 1.02 | 0 | 0.002 | -178 | -356 | 1116 | 0 |  | 5.83 | 8.73 | -- | 11.64 |
|  | -- | -- | Ceres | 6202 | -- | -- | 0.35 | 0.175 | 0 | 0.002 | -170 | -340 | -408 | 0 |  | 0.81 | 1.22 | -- | 1.62 |
|  | -- | -- | Pitscottie | 6202 | -- | -- | 0.05 | 0.025 | 0 | 0.002 | -15 | -30 | -54 | 0 |  | 0.12 | 0.18 | -- | 0.24 |
|  | -- | -- | Letham | 6206 | -- | -- | 0.117 | 0.04 | 0 | 0.002 | -27 | -54 | -65 | 0 |  | 0.21 | 0.32 | -- | 0.42 |
| Daily Influent Flow (Ml/day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -- | Daily Influent Flow variables follow the same discretisation as Daily Effluent Flow. |  |  |  |
| Spill Event | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1 | 4.67 | 8.33 | 12 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Phosphorus Load (kg/d) | -- | -- | Cupar | 6200 | -- | -- | 2.6 | 1.6 | 0 | 0.0002 | 1408 | 2816 | 5554 | 0 | Values for the current effluent P concentration (mg/l) β are taken from the Scottish Water Eden Water Quality Strategic Study. See Table D1 for γ source | 5.4 | 10.8 | -- | 16.2 |
|  | -- | -- | Springfield | 6200 | -- | -- | 3.7 | 1.6 | 0 | 0.0004 | 6 | 12 | 668 | 0 |  | 5.96 | 11.92 | -- | 17.88 |
|  | -- | -- | Freuchie | 6200 | -- | -- | 4.42 | 2.59 | 0 | 0.0032 | -176 | -352 | 442 | 0 |  | 2.46 | 4.92 | -- | 7.38 |
|  | -- | -- | Dairsie | 6200 | -- | -- | 2.7 | 1.8 | 0 | 0.0002 | 37 | 74 | 178 | 0 |  | 0.34 | 0.68 | -- | 1.02 |
|  | -- | -- | Strathmiglo | 6201 | -- | -- | 2.1 | 1.3 | 0 | 0.0014 | -153 | -306 | 367 | 0 |  | 1.54 | 3.08 | -- | 4.62 |
|  | -- | -- | Bowhouse | 6201 | -- | -- | 2.8 | 1.4 | 0 | 0.0003 | -178 | -356 | 1116 | 0 |  | 5.94 | 11.88 | -- | 17.82 |
|  | -- | -- | Ceres | 6202 | -- | -- | 2.5 | 1.5 | 0 | 0.0016 | -170 | -340 | -408 | 0 |  | 0.8 | 1.6 | -- | 2.4 |
|  | -- | -- | Pitscottie | 6202 | -- | -- | 4.2 | 2.5 | 0 | 0.0191 | -15 | -30 | -54 | 0 |  | 0.19 | 0.38 | -- | 0.57 |
|  | -- | -- | Letham | 6206 | -- | -- | 2.3 | 1.4 | 0 | 0.0002 | -27 | -54 | -65 | 0 |  | 0.24 | 0.48 | -- | 0.72 |
| Bio Resource (m3/day) | -- | -- | Cupar | 6200 | -- | -- | -- | -- | -- | 0.003 | -- | -- | -- | -- | See Table D1 for γ source | 6.7 | 13.4 | -- | 20.1 |
|  | -- | -- | Springfield | 6200 | -- | -- | -- | -- | -- | 0.0091 | -- | -- | -- | -- |  | 10.33 | 20.66 | -- | 30.99 |
|  | -- | -- | Freuchie | 6200 | -- | -- | -- | -- | -- | 0.0014 | -- | -- | -- | -- |  | 0.94 | 1.88 | -- | 2.82 |
|  | -- | -- | Dairsie | 6200 | -- | -- | -- | -- | -- | 0.0053 | -- | -- | -- | -- |  | 0.8 | 1.38 | -- | 2.07 |
|  | -- | -- | Strathmiglo | 6201 | -- | -- | -- | -- | -- | 0.0004 | -- | -- | -- | -- |  | 0.43 | 0.86 | -- | 1.29 |
|  | -- | -- | Bowhouse | 6201 | -- | -- | -- | -- | -- | 0.0009 | -- | -- | -- | -- |  | 1.98 | 3.96 | -- | 5.94 |
|  | -- | -- | Ceres | 6202 | -- | -- | -- | -- | -- | 0.0026 | -- | -- | -- | -- |  | 0.95 | 1.9 | -- | 2.85 |
|  | -- | -- | Pitscottie | 6202 | -- | -- | -- | -- | -- | 0.011 | -- | -- | -- | -- |  | 0.58 | 1.16 | -- | 1.74 |
|  | -- | -- | Letham | 6206 | -- | -- | -- | -- | -- | 0.0032 | -- | -- | -- | -- |  | 0.4 | 0.8 | -- | 1.2 |
| Total Phosphorus Load (kg d−1) | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 21.5 | 43 | -- | 64.5 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 7.8 | 15.6 | -- | 23.4 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 1.04 | 2.08 | -- | 3.12 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.24 | 0.48 | -- | 0.72 |
| Energy Demand Wastewater | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1 | 4.62 | 8.33 | 12 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Chemical Demand Wastewater | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Asset Compliance and Capability Wastewater |  |  |  | 6200 |  |  |  |  |  |  |  |  |  |  |  | 1.25 | 5.83 | 10.42 | 15 |
|  |  |  |  | 6201 |  |  |  |  |  |  |  |  |  |  |  | 0.75 | 3.5 | 6.25 | 9 |
|  |  |  |  | 6202 |  |  |  |  |  |  |  |  |  |  |  | 0.75 | 3.5 | 6.25 | 9 |
|  |  |  |  | 6205 |  |  |  |  |  |  |  |  |  |  |  | 0.5 | 2.33 | 4.17 | 6 |
| Land Cover (Ha) | -- | -- | -- | 6200 | Arable | -- | -- | -- | -- | -- | 3589 | 3729 | 3833 | 3485 | UKCEH land cover vector maps (Morton et al., 2019) used to determine A. | States are discretised based on A values and are available in Table D1 |  |  |  |
|  | -- | -- | -- | 6200 | Pasture | -- | -- | -- | -- | -- | 442 | 531 | 885 | 885 |  |  |  |  |  |
|  | -- | -- | -- | 6200 | Urban | -- | -- | -- | -- | -- | 546 | 642 | 728 | 520 |  |  |  |  |  |
|  | -- | -- | -- | 6201 | Arable | -- | -- | -- | -- | -- | 2834 | 2944 | 3081 | 2751 |  |  |  |  |  |
|  | -- | -- | -- | 6201 | Pasture | -- | -- | -- | -- | -- | 551 | 937 | 1191 | 1103 |  |  |  |  |  |
|  | -- | -- | -- | 6201 | Urban | -- | -- | -- | -- | -- | 174 | 132 | 270 | 193 |  |  |  |  |  |
|  | -- | -- | -- | 6202 | Arable | -- | -- | -- | -- | -- | 2573 | 2673 | 2773 | 2499 |  |  |  |  |  |
|  | -- | -- | -- | 6202 | Pasture | -- | -- | -- | -- | -- | 1061 | 1388 | 1715 | 1633 |  |  |  |  |  |
|  | -- | -- | -- | 6205 | Arable | -- | -- | -- | -- | -- | 1007 | 1027 | 1057 | 997 |  |  |  |  |  |
|  | -- | -- | -- | 6205 | Pasture | -- | -- | -- | -- | -- | 94 | 140 | 168 | 187 |  |  |  |  |  |
|  | -- | -- | -- | 6205 | Urban | -- | -- | -- | -- | -- | 116 | 119 | 124 | 111 |  |  |  |  |  |
|  | -- | -- | -- | 6206 | Arable | -- | -- | -- | -- | -- | 2433 | 2528 | 2669 | 2362 |  |  |  |  |  |
|  | -- | -- | -- | 6206 | Pasture | -- | -- | -- | -- | -- | 1025 | 1340 | 1703 | 1576 |  |  |  |  |  |
|  | -- | -- | -- | 6206 | Urban | -- | -- | -- | -- | -- | -- | -- | -- | -- |  |  |  |  |  |
| Septic Tanks (No of) | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | 136 | 113 | 90 | 113 | Scottish Water Eden Water Quality Strategic Study provided the current no of T. See SMTable1 for GR, BAU and FFD values. | States are discretised based on j A values and are available in Table D1. |  |  |  |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | 73 | 61 | 49 | 61 |  |  |  |  |  |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | 56 | 47 | 38 | 47 |  |  |  |  |  |
|  | -- | -- | -- | 6205 | -- | -- | -- | -- | -- | -- | 24 | 14 | 8 | 14 |  |  |  |  |  |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | 74 | 62 | 50 | 62 |  |  |  |  |  |
| Phosphorus Applications (kg d−1) | -- | -- | -- | 6200 | Arable | -- | -- | -- | -- | 0.0037 | -- | -- | -- | -- | Values for γ taken from the ADAS UK Ltd model of rural diffuse pollution provided by SEPA. | 12.77 | 19.16 | -- | 25.54 |
|  | -- | -- | -- | 6200 | Pasture | -- | -- | -- | -- | 0.0064 | -- | -- | -- | -- |  | 5.7 | 8.55 | -- | 11.4 |
|  | -- | -- | -- | 6200 | Urban | -- | -- | -- | -- | 0.00057 | -- | -- | -- | -- |  | 0.3 | 0.45 | -- | 0.6 |
|  | -- | -- | -- | 6200 | Septic Tank | -- | -- | -- | -- | 0.073 | -- | -- | -- | -- |  | 8.3 | 12.45 | -- | 16.6 |
|  | -- | -- | -- | 6201 | Arable | -- | -- | -- | -- | 0.00042 | -- | -- | -- | -- |  | 1.5 | 2.25 | -- | 3 |
|  | -- | -- | -- | 6201 | Pasture | -- | -- | -- | -- | 0.0008 | -- | -- | -- | -- |  | 0.83 | 1.25 | -- | 1.66 |
|  | -- | -- | -- | 6201 | Septic Tank | -- | -- | -- | -- | 0.015 | -- | -- | -- | -- |  | 0.93 | 1.4 | -- | 1.86 |
|  | -- | -- | -- | 6202 | Arable | -- | -- | -- | -- | 0.0011 | -- | -- | -- | -- |  | 2.8 | 4.2 | -- | 5.6 |
|  | -- | -- | -- | 6202 | Pasture | -- | -- | -- | -- | 0.0014 | -- | -- | -- | -- |  | 2.23 | 3.35 | -- | 4.46 |
|  | -- | -- | -- | 6202 | Septic Tank | -- | -- | -- | -- | 0.018 | -- | -- | -- | -- |  | 0.83 | 1.25 | -- | 1.66 |
|  | -- | -- | -- | 6205 | Arable | -- | -- | -- | -- | 0.0005 | -- | -- | -- | -- |  | 0.52 | 0.78 | -- | 1.04 |
|  | -- | -- | -- | 6205 | Pasture | -- | -- | -- | -- | 0.00017 | -- | -- | -- | -- |  | 0.03 | 0.045 | -- | 0.06 |
|  | -- | -- | -- | 6205 | Urban | -- | -- | -- | -- | 0.00007 | -- | -- | -- | -- |  | 0.01 | 0.015 | -- | 0.02 |
|  | -- | -- | -- | 6205 | Septic Tank | -- | -- | -- | -- | 0.015 | -- | -- | -- | -- |  | 0.21 | 0.32 | -- | 0.42 |
|  | -- | -- | -- | 6206 | Arable | -- | -- | -- | -- | 0.000015 | -- | -- | -- | -- |  | 0.034 | 0.056 | -- | 0.078 |
|  | -- | -- | -- | 6206 | Pasture | -- | -- | -- | -- | 0.000011 | -- | -- | -- | -- |  | 0.017 | 0.026 | -- | 0.034 |
|  | -- | -- | -- | 6206 | Septic Tank | -- | -- | -- | -- | 0.0008 | -- | -- | -- | -- |  | 0.05 | 0.08 | -- | 0.1 |
| Total Diffuse Phosphorus (kg d−1) | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 27 | 36 | -- | 54 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 2.9 | 4.35 | -- | 5.8 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 5.84 | 8.76 | -- | 11.68 |
|  | -- | -- | -- | 6205 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.77 | 1.16 | -- | 1.54 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.1 | 0.15 | -- | 0.2 |
| Groundwater Nitrate (mg/l) | -- -- -- | -- -- -- | -- -- -- | 6201 | Arable | -- | 3.41 | 0.29 | -- | 0.001 | -- | -- | -- | -- | Values for β and γ derived using groundwater nitrate samples (mg/l) from 2008-2019 provided by SEPA. | 18.75 | 28.13 | -- | 37.5 |
|  |  |  |  |  | Urban |  |  |  |  | 0.0001 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Septic Tank |  |  |  |  | 0.000059 |  |  |  |  |  |  |  |  |  |
|  | -- | -- | -- | 6201 | Arable | -- | 4.07 | 0.2 | -- | 0.0013 | -- | -- | -- | -- |  | 18.75 | 28.13 | -- | 37.5 |
|  |  |  |  |  | Urban |  |  |  |  | 0.00013 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Septic Tank |  |  |  |  | 0.00007 |  |  |  |  |  |  |  |  |  |
| Irrigation Demand (ML/year) | -- | -- | -- | 6200 | -- | -- | 93 | 82 | 0 | -- | -- | -- | -- | -- | Values for β derived using annual irrigation abstraction licence return data (Ml/year) from 2008-2019 provided by SEPA. | 625 | 937.5 | -- | 1250 |
|  | -- | -- | -- | 6201 | -- | -- | 44 | 57 | 0 | -- | -- | -- | -- | -- |  | 542 | 813 | -- | 1084 |
|  | -- | -- | -- | 6202 | -- | -- | 55 | 75 | 0 | -- | -- | -- | -- | -- |  | 193.5 | 290.3 | -- | 387 |
|  | -- | -- | -- | 6206 | -- | -- | 21 | 16 | 0 | -- | -- | -- | -- | -- |  | 96.5 | 144.8 | -- | 193 |
| Energy Demand Land Management | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Chemical Demand Land Management | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6202 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6205 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
|  | -- | -- | -- | 6206 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
| Public Commercial Demand | GR | -- | -- | -- | -- | -- | -- | -- | -- | 0.95 | -- | -- | -- | -- | Values γ are used for variable PCDit in the equation for variable WA (see Table D1). | As the variable is deterministic in the model there is no discretisation. |  |  |  |
|  | BAU | -- | -- | -- | -- | -- | -- | -- | -- | 1.05 | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | -- | -- | -- | -- | 1.1 | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | -- | -- | -- | -- | 1 | -- | -- | -- | -- |  |  |  |  |  |
| Leakage | GR | -- | -- | -- | -- | -- | -- | -- | -- | 0.95 | -- | -- | -- | -- | Values γ are used for variable Lit in the equation for variable WA (see Table D1). | As the variable is deterministic in the model there is no discretisation. |  |  |  |
|  | BAU | -- | -- | -- | -- | -- | -- | -- | -- | 1 | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | -- | -- | -- | -- | 1.05 | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | -- | -- | -- | -- | 1 | -- | -- | -- | -- |  |  |  |  |  |
| Water Resource Abstraction Demand (Ml/day) | -- | -- | -- | -- | -- | -- | 7.71 | 0.58 | -- | 0.000165 | 743 | 1,486 | 6,339 | 0 | Values for β derived using annual abstraction data for all Scottish Water boreholes in the catchment from 2014-2018 provided by SEPA and Scottish Water. The coefficient γ which represents the normal consumption rate of 165 l/d per person per day identified by Scottish Water | 7.5 | 8.5 | -- | 9.5 |
| Asset Conditions | GR | -- | -- | -- | -- | -- | -- | -- | -- | 0.98 | -- | -- | -- | -- | Values γ are used for variable ACit in the equation for variable SC (see Table D1). | As the variable is deterministic in the model there is no discretisation. |  |  |  |
|  | BAU | -- | -- | -- | -- | -- | -- | -- | -- | 0.98 | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | -- | -- | -- | -- | 0.95 | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | -- | -- | -- | -- | 1 | -- | -- | -- | -- |  |  |  |  |  |
| Water Resource Supply Capacity | -- | -- | -- | a1 | -- | -- | Custom |  |  | -- | -- | -- | -- | -- | Custom values for β derived using annual abstraction rate data for all Scottish Water boreholes a in the catchment from 2012-2019 provided by Scottish Water | 5 | 3 | -- | 1 |
|  | -- | -- | -- | a2 | -- | -- | Custom |  |  | -- | -- | -- | -- | -- |  | 5 | 3 | -- | 1 |
| Resilience of Eden Supply (Ml/day) | -- | -- | -- | -- | -- | -- | 0.97 | 0.12 | 0 | -- | -- | -- | -- | -- | The β values represent βooc used in the RS variable equation are derived from the 2014-2018 abstraction data provided by SEPA and Scottish Water (see Table D1 1). | 0.5 | 0.25 | -- | 0 |
| Resilience of Out of Catchment Supply (Ml/day) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.5 | 0.25 | -- | 0 |
| Energy Demand Water Resources | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Chemical Demand Water Resources | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Asset Capability Water Resources | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
| Customer Complaints Water Resources | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.5 | 2.33 | 4.17 | 6 |
| Crop Cover (Ha) | -- | -- | -- | -- | -- | Wheat | -- | -- | -- | -- | 5595 | 5758 | 5866 | 5432 | Values for A we use the UKCEH Land Cover® Plus: Crops © 2016-2020 maps and SSP narratives. | States are discretised based on current A values for z. |  |  |  |
|  | -- | -- | -- | -- | -- | Barley | -- | -- | -- | -- | 6700 | 6896 | 7026 | 6505 |  |  |  |  |  |
|  | -- | -- | -- | -- | -- | Potato | -- | -- | -- | -- | 1233 | 1269 | 1293 | 1197 |  |  |  |  |  |
|  | -- | -- | -- | -- | -- | Oilseed | -- | -- | -- | -- | 412 | 423 | 431 | 399 |  |  |  |  |  |
| Crop Yield (t/Ha) | GR | BAU | -- | -- | -- | Wheat | 8.62 | 0.94 | 0 | -- | -- | -- | -- | -- | For β values we use a combination of crop yield data from 2010-2019 available from the Scottish Agriculture Tables from the Economic Report 2020 and crop yield values taken from The Farm Management Handbook 2020/21 produced by SAC Consulting. | 8.1 | 6.1 | -- | 4.1 |
|  | BAU | BAU | -- | -- | -- | Wheat | 9.03 | 0.94 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | BAU | -- | -- | -- | Wheat | 10 | 0.94 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | BAU | -- | -- | -- | Wheat | 8.21 | 0.94 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | All | ExLR/ExHIR | -- | -- | -- | Wheat | 6 | 0.94 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | BAU | -- | -- | -- | Barley | 6.33 | 0.48 | 0 | -- | -- | -- | -- | -- |  | 6.03 | 4.52 | -- | 3.02 |
|  | BAU | BAU | -- | -- | -- | Barley | 6.63 | 0.48 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | BAU | -- | -- | -- | Barley | 7.5 | 0.48 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | BAU | -- | -- | -- | Barley | 6.03 | 0.48 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | All | ExLR/ExHIR | -- | -- | -- | Barley | 4 | 0.48 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | BAU | -- | -- | -- | Potato | 36.31 | 1.9 | 0 | -- | -- | -- | -- | -- |  | 42.2 | 31.65 | -- | 21.1 |
|  | BAU | BAU | -- | -- | -- | Potato | 38 | 1.9 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | BAU | -- | -- | -- | Potato | 39.78 | 1.9 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | BAU | -- | -- | -- | Potato | 34.8 | 1.9 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | All | ExLR/ExHIR | -- | -- | -- | Potato | 30 | 1.9 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | BAU | -- | -- | -- | Oilseed | 3.91 | 0.44 | 0 | -- | -- | -- | -- | -- |  | 3.72 | 2.79 | -- | 1.86 |
|  | BAU | BAU | -- | -- | -- | Oilseed | 4.1 | 0.44 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | BAU | -- | -- | -- | Oilseed | 5 | 0.44 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | BAU | -- | -- | -- | Oilseed | 3.72 | 0.44 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | All | ExLR/ExHIR | -- | -- | -- | Oilseed | 3 | 0.44 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
| Crop Price (£/t) | GR | -- | -- | -- | -- | Wheat | 152.99 | 23.14 | 0 | -- | -- | -- | -- | -- | For β values we use crop price data from 2010-2019 available from Scottish Agriculture Tables from the Economic Report 2020 | 145.7 | 109.2 | -- | 72.9 |
|  | BAU | -- | -- | -- | -- | Wheat | 160.27 | 23.14 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Wheat | 167.56 | 23.14 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Wheat | 145.7 | 23.14 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Barley | 146.27 | 28.4 | 0 | -- | -- | -- | -- | -- |  | 139.3 | 104.5 | -- | 69.7 |
|  | BAU | -- | -- | -- | -- | Barley | 153.23 | 28.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Barley | 160.12 | 28.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Barley | 139.3 | 28.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Potato | 174.1 | 23.5 | 0 | -- | -- | -- | -- | -- |  | 165.8 | 124.4 | -- | 82.9 |
|  | BAU | -- | -- | -- | -- | Potato | 182.38 | 23.5 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Potato | 190.67 | 23.5 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Potato | 165.8 | 23.5 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Oilseed | 322.82 | 40.4 | 0 | -- | -- | -- | -- | -- |  | 308.4 | 231.3 | -- | 154.2 |
|  | BAU | -- | -- | -- | -- | Oilseed | 339.24 | 40.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Oilseed | 354.66 | 40.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Oilseed | 308.4 | 40.4 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
| Fertiliser Cost (£/ha) | GR | -- | -- | -- | -- | Wheat | 161.1 | 23.75 | 0 | -- | -- | -- | -- | -- | For β values we use literature parameters of Fertiliser costs 2016-2020 available from The Farm Management Handbooks produced by SAC Consulting. | 179.4 | 269.1 | -- | 358.8 |
|  | BAU | -- | -- | -- | -- | Wheat | 188.37 | 23.75 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Wheat | 296 | 23.75 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Wheat | 179.4 | 23.75 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Barley | 150.21 | 21.71 | 0 | -- | -- | -- | -- | -- |  | 166.9 | 250.4 | -- | 333.8 |
|  | BAU | -- | -- | -- | -- | Barley | 175.25 | 21.71 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Barley | 250.35 | 21.71 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Barley | 166.9 | 21.71 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Potato | 177.84 | 21.27 | 0 | -- | -- | -- | -- | -- |  | 197.6 | 296.4 | -- | 395.2 |
|  | BAU | -- | -- | -- | -- | Potato | 207.5 | 21.27 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Potato | 296.4 | 21.27 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Potato | 197.6 | 21.27 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | GR | -- | -- | -- | -- | Oilseed | 117.36 | 18.8 | 0 | -- | -- | -- | -- | -- |  | 130.4 | 195.6 | -- | 260.8 |
|  | BAU | -- | -- | -- | -- | Oilseed | 136.92 | 18.8 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | FFD | -- | -- | -- | -- | Oilseed | 195.6 | 18.8 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
|  | Current | -- | -- | -- | -- | Oilseed | 130.4 | 18.8 | 0 | -- | -- | -- | -- | -- |  |  |  |  |  |
| Total Crop Margin (£M) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 17 | 12.75 |  | 8.5 |
| Surface Water Flows (Ml/day) | -- | -- | -- | 6200 | -- | -- | Custom |  |  | -- | 365 | -- | -- | -- | Custom values for β derived from river discharge data 2010-2019 provided by SEPA. |  |  |  |  |
|  | -- | -- | -- | 6201 | -- | -- | Custom |  |  | -- | 365 | -- | -- | -- |  |  |  |  |  |
| Surface Water Quality (mg/l) | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | 1000 | -- | -- | -- | -- | Custom values for β derived from water quality sampling data 2010-2019 provided by SEPA. | 78 | 191 | -- | 1046 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | 1000 | -- | -- | -- | -- |  | 67 | 170 | -- | 996 |
|  | -- | -- | -- | 6202 | -- | -- | Custom |  |  | -- | -- | -- | -- | -- |  | 75 | 186 | -- | 1034 |
|  | -- | -- | -- | 6205 | -- | -- | Custom |  |  | -- | -- | -- | -- | -- |  | 72 | 197 | -- | 1015 |
|  | -- | -- | -- | 6206 | -- | -- | Custom |  |  | -- | -- | -- | -- | -- |  | 71 | 178 | -- | 1015 |
| Soil Erosion | -- | -- | -- | 6200 | -- | -- | 3.1 | 1.1 | 0 | -- | -- | -- | -- | -- | Values for β derived from maps of the risk of soil erosion by water produced by Lilly & Baggaley (2018). | 1 | 3 | -- | 5 |
|  | -- | -- | -- | 6201 | -- | -- | 3.6 | 1.14 | 0 | -- | -- | -- | -- | -- |  | 1 | 3 | -- | 5 |
|  | -- | -- | -- | 6202 | -- | -- | 3.62 | 1.1 | 0 | -- | -- | -- | -- | -- |  | 1 | 3 | -- | 5 |
|  | -- | -- | -- | 6205 | -- | -- | 3.7 | 0.96 | 0 | -- | -- | -- | -- | -- |  | 1 | 3 | -- | 5 |
|  | -- | -- | -- | 6206 | -- | -- | 3.93 | 1.45 | 0 | -- | -- | -- | -- | -- |  | 1 | 3 | -- | 5 |
| Surface Water Flow (Capital Resource) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.5 | 2.33 | 4.17 | 6 |
| Surface Water Quality (Capital Resource) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Flood Risk | -- | -- | -- | 6200 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.25 | 1.17 | 2.08 | 3 |
|  | -- | -- | -- | 6201 | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- |  | 0.25 | 1.17 | 2.08 | 3 |
| Soil (Capital Resource) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Air Quality | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Groundwater Quality | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.5 | 2.33 | 4.17 | 6 |
| Energy Demand Change SW | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Chemical Demand Change SW | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Asset Compliance & Capability (Capital Resource) | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Energy Demand Change LM | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1 | 4.67 | 8.33 | 12 |
| Chemical Demand Change LM | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Community Relationship | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.75 | 8.17 | 15.58 | 21 |
| Water Treatment & Supply Costs | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.75 | 3.5 | 6.25 | 9 |
| Food Production | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.75 | 3.5 | 6.25 | 9 |
| Reputation | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Natural Capital | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Social Capital | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.25 | 1.17 | 2.08 | 3 |
| Manufactured Capital | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1.25 | 5.83 | 10.42 | 15 |
| Financial Capital | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 1 | 2.67 | 4.33 | 6 |
| Intellectual Capital | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | -- | 0.25 | 1.17 | 2.08 | 3 |

## C: Management scenario parametrisation, justification and data sources

Table S3: Management scenario parametrisation, justification and data sources

|  | Management Scenario |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | No Management |  |  |  | Standard Scenarios |  |  |  | Nature Based |  |  |  | Best Available Technology |  |  |  | Resource Centre |  |  |  |  |
| Model Variable | β |  |  | m | β |  |  | m | β |  |  | m | β |  |  | m | β |  |  | m | Value Justification and Data Source |
|  | M | SD | T |  | M | SD | T |  | M | SD | T |  | M | SD | T |  | M | SD | T |  |  |
| Wetland Effluent Flows (Ml/day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Springfield | -- | -- | -- | 1 | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 1 | -- | -- | -- | 1 | Stakeholders identified the use of Constructed Wetlands as a management option for reducing Wastewater Effluent P concentrations. Wetland treatment options would include effluent flow reductions (Ml/day) (Almuktar et al., 2018) by 50%, which is a value highlighted by the project team with similar values identified by (Cooper et al., 2020). At sites and management scenarios where wetlands are considered, a value of 0.5 is applied to effluent flow, else a value of one maintains current flow conditions. |
| Freuchie | -- | -- | -- | 1 | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 1 | -- | -- | -- | 1 |  |
| Dairsie | -- | -- | -- | 1 | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 1 | -- | -- | -- | 0.5 |  |
| Strathmiglo | -- | -- | -- | 1 | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 0.5 | -- | -- | -- | 0.5 |  |
| Bowhouse | -- | -- | -- | 1 | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 1 | -- | -- | -- | 1 |  |
| Wastewater Phosphorus Treatment Concentrations (mg/l) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cupar | 2.6 | 1.6 | 0 | -- | 0.5 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- | Stakeholders identified different Phosphorus treatment options which could be applied at different sites across the catchment. Under a standard scenario, stakeholders identified ferric dosing as a treatment option. The β value for the standard scenario is based on ferric dosing concentrations of P take from the Eden Water Quality Strategic Study. An ultra-low P Nereda treatment technology was also identified, which would change β value to a mean P concentration of 0.2 mg/l which is also taken from the Eden Water Quality Strategic Study. The ultra-low P treatment option is applied to varying sites across nature based, best available technology and resource centre management scenarios. Wetland treatment options were also highlighted as reducing P loads from wastewater treatment works. A 50% reduction in P concentrations, was identified by stakeholders. With the 50% effluent flow reduction and overall, 50% P reduction is consistent with literature findings from previous studies (Cooper et al., 2020, Land et al., 2016). |
| Springfield | 3.7 | 1.6 | 0 | -- | 0.5 | 0.1 | 0 | -- | 1.85 | 0.8 | 0 | -- | 0.2 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- |  |
| Freuchie | 4.42 | 2.59 | 0 | -- | 0.5 | 0.1 | 0 | -- | 2.21 | 1.3 | 0 | -- | 0.2 | 0.1 | 0 | -- | 2.21 | 1.3 | 0 | -- |  |
| Dairsie | 2.7 | 1.8 | 0 | -- | 0.5 | 0.1 | 0 | -- | 1.62 | 1.08 | 0 | -- | 0.2 | 0.1 | 0 | -- | 1.62 | 0.9 | 0 | -- |  |
| Strathmiglo | 2.1 | 1.3 | 0 | -- | 0.5 | 0.1 | 0 | -- | 1.05 | 0.65 | 0 | -- | 1.05 | 0.65 | 0 | -- | 1.05 | 0.65 | 0 | -- |  |
| Bowhouse | 2.8 | 1.4 | 0 | -- | 0.5 | 0.1 | 0 | -- | 1.4 | 0.7 | 0 | -- | 0.2 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- |  |
| Ceres | 2.5 | 1.5 | 0 | -- | 0.5 | 0.1 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.2 | 0.1 | 0 | -- | 0.2 | 0.1 | 0 | -- |  |
| Pitscottie | 4.2 | 2.5 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.5 | 0.2 | 0 | -- |  |
| Letham | 2.3 | 1.4 | 0 | -- | 0.5 | 0.1 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.5 | 0.2 | 0 | -- | 0.5 | 0.2 | 0 | -- |  |
| GBR, RSUDS, Cover Crops and Minimum Tillage, and Septic Tanks Treatment - Diffuse Phosphorus Load (kg d−1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Arable | -- | -- | -- | 1 | -- | -- | -- | 0.8 | -- | -- | -- | 0.2 | -- | -- | -- | 0.2 | -- | -- | -- | 0.1 | Stakeholders identified that 100% General Binding Rules (GBR) compliance as the standard management option for reducing diffuse P sources from Arable fields. Stakeholders highlighted that 100% GBR compliance would yield an 20% reduction in diffuse P which is supported by reductions for 2m buffers identified by Balana et al. (2012b).For nature based, best available technology and resource centre scenarios, stakeholders identified a combination of Rural Sustainable Drainage Systems (RSuDs) and Magic Margins options for reducing Diffuse P from arable sources. We use an upper estimate of 80% P reductions based on findings for RSuDs and buffer strip methods (Stutter et al., 2021b, Environment Agency, 2016). For the resource centre scenario, the reduction in P was increased to 90% to reflect an increase in P application efficiency associated with the use of Struvite for fertiliser application and the use of cover crops and minimum tillage in the catchment, |
| Pasture | -- | -- | -- | 1 | -- | -- | -- | 0.8 | -- | -- | -- | 0.8 | -- | -- | -- | 0.8 | -- | -- | -- | 0.8 | Stakeholders identified that 100% General Binding Rules (GBR) compliance as the standard management option for reducing diffuse P sources from Pasture fields. Stakeholders highlighted that 100% GBR compliance would yield an 20% reduction in diffuse P, which we maintain across all management scenarios. |
| Septic Tanks | -- | -- | -- | 1 | -- | -- | -- | 0.5 | -- | -- | -- | 0.5 | -- | -- | -- | 0.5 | -- | -- | -- | 0.5 | Stakeholders highlighted that a 50% P reduction was possible via a septic tank investigation and associated improvements, as highlighted in the Eden Water Quality Strategic Study. The improvements are considered across all management scenarios. |
| Agriculture Water Efficiency - Irrigation Demand (Ml/year) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6200 | -- | -- | -- | 1 | -- | -- | -- | 0.6 | -- | -- | -- | 0 | -- | -- | -- | 0 | -- | -- | -- | 0 | Stakeholders identified that in the standard scenario irrigation efficiency improvements could be achieved by coupling soil moisture sensors and irrigation scheduling. We apply a 40% reduction in irrigation demand based on findings from (Adams and Kovacs, 2019). This option is applied across all scenarios in waterbody sub catchments 6202 and 6206. For nature based, best available technology and resource centre scenarios in waterbody sub-catchments 6200 and 6201 stakeholders identified that lagoons would allow for the storage of water for irrigation, which would eliminate the need for surface water abstractions, therefore a value of zero is applied. |
| 6201 | -- | -- | -- | 1 | -- | -- | -- | 0.6 | -- | -- | -- | 0 | -- | -- | -- | 0 | -- | -- | -- | 0 |  |
| 6202 | -- | -- | -- | 1 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 |  |
| 6206 | -- | -- | -- | 1 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 | -- | -- | -- | 0.6 |  |
| Water Resource Water Efficiency Campaigns - Abstraction Demand (Ml/day) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Resource | -- | -- | -- | 1 | -- | -- | -- | 0.85 | -- | -- | -- | 0.85 | -- | -- | -- | 0.85 | -- | -- | -- | 0.85 | Stakeholders identified that a water efficiency campaign would reduce abstraction demands (ML/day). We assume a water efficiency campaign would achieve a 15% reduction in abstraction demand across all scenarios based on the findings of Manouseli et al. (2019). |

Standard Scenario 1 applies only diffuse arable and pasture reduction values and maintain ‘No Management Scenario’ values for Wastewater Phosphorus Treatment Concentrations (mg/l). Standard Scenario 2 applies only Wastewater Phosphorus Treatment Concentrations (mg/l) under the ‘Standard Scenario’ heading. Standard Scenario 3 includes both diffuse and wastewater reduction values, plus septic tank and water efficiency savings.

M represents the mean value; SD represents the standard deviation and Trunc represents the truncated values for the distribution which is typically zero to prevent values being non-negative.

Surface water flows variable is discretised using c, u1, u2, b1, b2 and b3 values (see SMTable1). For sub catchment 6200, the values are as follows: u1 = 98.5, b1 = 172.75, c = 247, b2 = 679.5, b3 = 895.75, u2 = 1112. For sub catchment 6201, the values are as follows: u1 = 10.54, b1 = 19.87, c = 29.2, b2 = 63.64, b3 = 98.67, u2 = 133.41.

M represents the mean value; SD represents the standard deviation and T represents the truncated values for the distribution which is typically zero to prevent values being non-negative.