### Supplementary Information

Limits of detection and quantitation for soil samples were calculated by Eq. 1 and Eq. 2, respectively where LoD is the limit of detection, SDblank is the blank standard deviation, b is the slope of analyte calibration line and LoQ is the limit of quantitation (Table 13).

Table 13 LoD and LoQ from FAAS and ICP OES

| Analytical methods | Elements | LoD (mg.L−1) | LoQ (mg.L−1) |
| --- | --- | --- | --- |
| FAAS | Cd | 0.0011 | 0.0037 |
|  | Cu | 0.0546 | 0.1818 |
|  | Fe | 0.1537 | 0.5125 |
|  | Pb | 0.0819 | 0.2729 |
| ICP OES | Al | 0.0425 | 0.1415 |
|  | Ba | 0.0008 | 0.0028 |
|  | Ca | 0.4458 | 1.4859 |
|  | Cd | 0.0008 | 0.0026 |
|  | Co | 0.0026 | 0.0087 |
|  | Cu | 0.0012 | 0.0040 |
|  | Cr | 0.0199 | 0.0663 |
|  | Fe | 0.1330 | 0.4435 |
|  | K | 0.0725 | 0.2416 |
|  | Mg | 0.0252 | 0.0841 |
|  | Mn | 0.7427 | 2.4757 |
|  | Mo | 0.0044 | 0.0146 |
|  | Na | 0.0839 | 0.2798 |
|  | Ni | 0.0057 | 0.0191 |
|  | P | 0.0144 | 0.0479 |
|  | Pb | 0.0115 | 0.0385 |
|  | Zn | 0.0409 | 0.1363 |