Table 1. Reactor operation plan based on “10th Basic Plan on Electricity Demand and Supply (2022~2036)” with consistent lifespans

| Type | Unit name | Power capacity (MWe) | Operation start (yr) | Operation end (yr) | Storage at the end of 2022 (ton) | Initial fuel loading (ton) | Annual fuel loading (ton) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PWR | Kori1 | 587 | 1978 | 2017 | 209 | 52 | 17 |
|  | Kori2 | 650 | 1983 | 2043 | 322 | 52 | 17 |
|  | Kori3 | 950 | 1984 | 2044 | 866 | 68 | 19 |
|  | Kori4 | 950 | 1985 | 2045 | 847 | 68 | 19 |
|  | Shin-Kori1 | 1,000 | 2010 | 2070 | 379 | 76 | 20 |
|  | Shin-Kori2 | 1,000 | 2011 | 2071 | 405 | 76 | 20 |
|  | Saewool1 | 1,400 | 2016 | 2076 | 127 | 104 | 29 |
|  | Saewool2 | 1,400 | 2017 | 2077 | 86 | 104 | 29 |
|  | Saewool3 | 1,400 | 2024 | 2084 | 0 | 104 | 29 |
|  | Saewool4 | 1,400 | 2025 | 2085 | 0 | 104 | 29 |
|  | Hanbit1 | 950 | 1985 | 2045 | 788 | 68 | 19 |
|  | Hanbit2 | 950 | 1986 | 2046 | 668 | 68 | 19 |
|  | Hanbit3 | 1,000 | 1994 | 2054 | 420 | 76 | 20 |
|  | Hanbit4 | 1,000 | 1995 | 2055 | 384 | 76 | 20 |
|  | Hanbit5 | 1,000 | 2001 | 2061 | 366 | 76 | 20 |
|  | Hanbit6 | 1,000 | 2002 | 2062 | 395 | 76 | 20 |
|  | Hanul1 | 950 | 1987 | 2047 | 398 | 68 | 19 |
|  | Hanul2 | 950 | 1988 | 2048 | 379 | 68 | 19 |
|  | Hanul3 | 1,000 | 1997 | 2057 | 491 | 76 | 20 |
|  | Hanul4 | 1,000 | 1998 | 2058 | 498 | 76 | 20 |
|  | Hanul5 | 1,000 | 2003 | 2063 | 485 | 76 | 20 |
|  | Hanul6 | 1,000 | 2004 | 2064 | 518 | 76 | 20 |
|  | Shin-Hanul1 | 1,400 | 2022 | 2082 | 15 | 104 | 29 |
|  | Shin-Hanul2 | 1,400 | 2023 | 2083 | 0 | 104 | 29 |
|  | Shin-Hanul3 | 1,400 | 2032 | 2092 | 0 | 104 | 29 |
|  | Shin-Hanul4 | 1,400 | 2033 | 2093 | 0 | 104 | 29 |
|  | Shin-Wolsung1 | 1,000 | 2012 | 2072 | 201 | 76 | 20 |
|  | Shin-Wolsung2 | 1,000 | 2014 | 2074 | 140 | 76 | 20 |
| PHWR | Wolsung1 | 679 | 1982 | 2018 | 622 | 87 | 103 |
|  | Wolsung2 | 700 | 1996 | 2036 | 736 | 87 | 103 |
|  | Wolsung3 | 700 | 1997 | 2037 | 779 | 87 | 103 |
|  | Wolsung4 | 700 | 1999 | 2039 | 779 | 87 | 103 |
|  | Dry storage | - | - | - | 6,528 | - | - |

Table 2. Reactor design parameters and compositions of fresh fuel and spent fuel

|  |  | PWR | SFR | PHWR |
| --- | --- | --- | --- | --- |
| Power capacity (GWe) |  | 1 | 1 | 1 |
| Avg. burn-up (GWd/tU) |  | 44* | 138 | 6* |
| Thermal efficiency (%) |  | 34 | 39 | 34 |
| Capacity factor (%) |  | 85 | 85 | 85 |
| Residence time (yr) |  | 3.1* | 3.4 | 0.7* |
| Construction time (yr) |  | 5 | 5 | 5 |
| Fresh fuel composition |  | U-235 4.5% | TRU 38.87% | U-235 0.711% |
| Spent fuel composition (%) | U | 92.92 | 55.66 | 98.74 |
|  | TRU | 1.36 | 30.53 | 0.41 |
|  | FP | 5.72 | 13.81 | 0.85 |

* Averaged value

Table 3. Volume and element mass of wastes generated from pyro-processing

| Class | PWR spent fuel |  | SFR spent fuel |  |
| --- | --- | --- | --- | --- |
|  | Volume | Element mass | Volume | Element mass |
| HLW | - | 13.3 kgFP/tHM | - | 62.4 kgFP/tHM |
| LILW | 2.3 m3/tHM | - | 2.6 m3/tHM | - |
| Cs/Sr | - | 5.3 kgCsSr/tHM | - | 16.7 kgCsSr/tHM |

Table 4. Unit costs for the modules of reactors and fuel cycle facilities from AFC-CBR 2017

|  | Module | Units | Low | Mode | High | Mean | PDF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PWR (overnight capital) | R1 | $/kWe | 2500 | 4400 | 6300 | 4400 | Triangular |
| PWR (O&M, fixed component) | R1 | $/kWe-yr | 60 | 73 | 87 | 73 | Triangular |
| PWR (O&M, variable component) | R1 | $/kWh | 0.0008 | 0.0018 | 0.0027 | 0.0018 | Triangular |
| SFR (overnight capital) | R2 | $/kWe | 2400 | 4100 | 7600 | 4700 | Triangular |
| SFR (O&M, fixed component) | R2 | $/kWe-yr | 65 | 76 | 92 | 78 | Triangular |
| SFR (O&M, variable component) | R2 | $/kWh | 0.0011 | 0.0022 | 0.0029 | 0.0021 | Triangular |
| PHWR (overnight capital) | R5 | $/kWe | 2400 | 4200 | 6100 | 4230 | Triangular |
| PHWR (O&M, fixed component) | R5 | $/kWe-yr | 60 | 72 | 87 | 73 | Triangular |
| PHWR (O&M, variable component) | R5 | $/kWh | 0.0008 | 0.002 | 0.0027 | 0.0018 | Triangular |
| NU Mining & Milling | A1 | $/kgU | 34 | 86 | 296 | 139 | Triangular |
| Conversion | B | $/kgU | 6.5 | 13 | 19 | 13 | Uniform |
| Enrichment | C1 | $/SWU | 97 | 125 | 154 | 125 | Uniform |
| PWR Fuel Fabrication | D1-1 | $/kgHM | 230 | 400 | 575 | 402 | Triangular |
| PHWR Fuel Fabrication | D1-7 | $/kgHM | 125 | 218 | 327 | 223 | Triangular |
| Metal Fuel Fabrication | D2 | $/kgHM | 1,000 | 1,400 | 1,800 | 1,400 | Triangular |
| Recycled Product Storage | E3 | $/kgHM | 712 | 950 | 1300 | 987 | Triangular |
| Managed Decay Storage | E4 | $/kgCs-Sr | 11,400 | 25,650 | 39,900 | 25,650 | Triangular |
| Echem Separation | F2 | $/kgHM | 1,000 | 1,200 | 1,400 | 1,200 | Triangular |
| HLW Conditioning & Packaging | G1 | $/kgFP | 13,700 | 17,214 | 20,660 | 17,191 | Triangular |
| SF Conditioning & Packaging | G2 | $/kgHM | 67.5 | 135 | 175 | 126 | Triangular |
| LLW-GTCC Conditioning & Packaging | G4 | $/m3 | 10,800 | 12,770 | 17,100 | 13,560 | Triangular |
| Consolidated Interim Storage | I | $/kgHM | 223 | 501 | 644 | 456 | Triangular |
| DU Storage & Disposition | K1 | $/kgHM | 8.8 | 20.6 | 54.5 | 28 | Triangular |
| RU Storage & Disposition | K3 | $/kgU | 81.8 | 98.1 | 164 | 115 | Triangular |
| HLW Deep Geologic Repository | L1 | $/kgHM | 1500 | 6000 | 7500 | 5000 | Triangular |
| SF Deep Geologic Repository | L1 | $/kgHM | 289 | 600 | 873 | 587 | Triangular |
| GTCC Intermediate Depth Disposal | L2 | $/m3 | 2300 | 3800 | 5320 | 3807 | Uniform |

Table 5. The capacity size and scaling factor of reference facility for the unit cost in AFC-CBR

| Fuel cycle module | Reference facility | Scaling factor | Capacity size |
| --- | --- | --- | --- |
| DU Disposition | DU Disposition | 0.6 | 6,630 ton/y |
| Pyro-processing | Pyro-processing | 0.9 | 70 ton/y |
| Metal Fuel Fabrication |  |  |  |
| RU Disposition |  |  |  |
| HLW Conditioning and Packaging |  |  |  |
| Recycled Product Storage | Recycled Product Storage | 0.41 | 50 ton(TRU)/y |

Table 6. Cost breakdown of the total costs for OT and Pyro-SFR cycles

|  | Future value |  | Present value |  |
| --- | --- | --- | --- | --- |
|  | OT | Pyro-SFR | OT | Pyro-SFR |
| Reactor capital [B$] | 189.5 | 307.5 | 135.6 | 183 |
| Reactor O&M [B$] | 119.3 | 193.2 | 85.3 | 115 |
| FE fuel cycle [B$] | 100.9 | 107.4 | 72 | 74.7 |
| BE fuel cycle [B$] | 78.9 | 148 | 40.4 | 76.5 |
| Total [B$] | 488.6 | 756.1 | 333.2 | 449.2 |
| Total electricity generation [TWh] | 9,936 | 15,744 | 7,102 | 9,438 |

Table 7. Cost breakdown of LCOEs of total costs for OT and pyro-SFR cycles

|  | OT | Pyro-SFR |
| --- | --- | --- |
| Reactor capital [mills/kWh] | 19.09 | 19.39 |
| Reactor O&M [mills/kWh] | 12.01 | 12.18 |
| FE fuel cycle [mills/kWh] | 10.13 | 7.91 |
| BE fuel cycle [mills/kWh] | 5.69 | 8.11 |
| Total [mills/kWh] | 46.92 | 47.59 |

Table 8. Statistical values of the cost distribution for OT and pyro-SFR cycles

|  | Total cost |  | Fuel cycle cost |  |
| --- | --- | --- | --- | --- |
|  | Mean | STD | Mean | STD |
| OT [mills/kWh] | 46.87 | 3.81 | 15.78 | 1.87 |
| Pyro-SFR [mills/kWh] | 47.55 | 3.09 | 15.98 | 1.39 |