# Appendix

# The allocation formula

The goal is to find a formula for a fixed subsidy sum so that new subsidized zero-carbon power plants can supply as much electricity as the number of new subsidized electric vehicles consume.

The input values for such a formula are:

MAX$ 		Maximum subsidy amount available to the government

EV$ 		Total subsidy for new electric vehicles (EVs)

PP$ 		Total subsidy amount for new zero-carbon power plants

PP 		Number of subsidized power plants with 1 MW of output each

EV 		Number of new EVs

PP1$ 		Subsidy per PP of 1 megawatt (MW) of output

EV1$ 		Subsidy per EV

EF 		Efficiency factor of the power plant (for wind turbines about 30%)

Based on this, the following equations are obtained:

MAX$ = PP$ + EV$                                                        (9)

PP$ = MAX$ - EV$                                                       (10)

EV$ = EV · EV1$                                            (11)

EV=EV$EV1$                                                (12)

PP$ = PP · PP1$                                            (13)

PP=PP$PP1$                                                  (14)

The electricity consumption per year of the new EV is (in megawats: MWh)

EV∙kmyear∙MWhkm                                              (15)

The electricity production per year by the new zero-carbon power plant is (each PP at 1 MWh)

PP∙24 hours∙365 days∙EF                                           (16)

Setting (15) and (16) equal:

EV∙kmyear∙MWhkm= PP∙24 hours∙365 days∙EF                         (17)

By inserting the EV from (12) and the PP from (14):

EV$EV1$∙kmyear∙MWhkm= PP$PP1$∙24 hours∙365 days∙EF                        (18)

Replace PP$ from (10):

EV$EV1$∙kmyear∙MWhkm= MAX$-EV$PP1$∙24 hours∙365 days∙EF               (19)

EV$EV1$∙kmyear∙MWhkm= MAX$∙24 ∙365∙EFPP1$-EV$∙24 ∙365∙EFPP1$                     (20)

EV$·1EV1$∙kmyear∙MWhkm+24 ∙365∙EFPP1$= MAX$∙24 ∙365∙EFPP1$                  (21)

EV$ =8760∙EFPP1$1EV1$∙kmyear∙MWhkm + 8760∙EFPP1$ · MAX$                     (22)

Sample Calculation:

MAX$ 	= $2,500,000,000

EV1$  	= $6,000

PP1$   	= $1,500,000

km/year 	= 13,000 km

MWh/km 	= 0.0002 MWh/km = 20 kWh/100 km

EF		= 30% = .30

Inserting in (14):

EV$ =8760∙.30150000016000∙13000km∙0.0002MWhkm + 8760∙.301500000 · $2.500.000.000                (23)

EV$  = 0.8017 ·  $2,500,000,000 = $2,004,270,897                             (24)

PP$  = (1–0.8017) ·  $2,500,000,000 = $495,729,103                          (25)

EV=$2,004,270,897$6000   = 334,045 new EVs                                 (26)

PP=$495,729,103$1,500,000   = 330 New 1 MW Power Plants                         (27)

Test through insertion in (17):

334,045∙13,000 kmyear∙0.0002 MWhkm= 330∙24 hours∙365 days∙.30            (28)

868,517 MWh  867,240 MWh                                        (29)