Procedure for the analysis of the volume of tissue activated

All patients treated with DBS, per protocol, were evaluated with structural MRI scans before surgery (including T1-weighted, T2-weighted and FLAIR sequences), and with a computed tomography (CT) scan to localize the electrodes after the surgery.

We processed all neuroimaging data using the MATLAB® toolbox Lead-DBS v2.6 [1]. For each subject, T1-weighted images was co-registered to head CT using ANTs [2], and to T2-weighted and FLAIR images using SPM12 [3]. Normalization to MNI_ICBM_2009b_NLIN_ASYM [4] space was carried out using ANTs, which was followed by non-linear deformation of the region of interest (ROI) based on the subcortical refinition mask computed as defined by Schönecker et al. to account for brainshift [5].  Electrodes’ position was determined on the post-surgical CT images and reconstructed using PaCER automatic algorithm, followed (if needed) by manual adjustment on the 3 space axes (based on the CT hyperdensity of the leads). Each individual VTA was obtained by entering the actual stimulation parameters. To visualize each VTA, we used Fastfield [6] as an estimation algorithm, setting the E-field threshold to 0.20 V/mm and brain conductivity to 0.1 S/mm.

After calculating the VTAs of the three stimulations setting (in the case of combined STN-SNr stimulation, VTA was calculated simulating each VTA separately), their volumetric data were extracted using FMRIB Software Library [7]. Initially the VTAs were linearly co-registered using FLIRT to the MNI_ICBM_2009b_NLIN_ASYM atlas, then they were binarized using FSLmaths. The same process was applied to the STN map and the SN map extracted from the DISTAL atlas [8]. Last, the volumes of the STN, SN, VTAs and the VTAs superposition on both SN and STN (depending on the stimulation target) were calculated using FSLstats.

- www.leads-dbs.org

- Tustison NJ, Cook PA, Klein A, et al. Large-scale evaluation of ANTs and FreeSurfer cortical thickness measurements. Neuroimage 2014; 99: 166-179.

- www.fil.ion.ucl.ac.uk/spm/software/spm12/

- www.bic.mni.mcgill.ca/ServicesAtlases/ICBM152NLin2009/

- Schönecker T, Kupsch A, Kühn AA, Schneider GH, Hoffmann KT. Automated Optimization of Subcortical Cerebral MR Imaging−Atlas Coregistration for Improved Postoperative Electrode Localization in Deep Brain Stimulation. Am J Neuroradiol 2009; 30: 1914–1921.

- Baniasadi M, Proverbio D, Gonçalves J, Hertel F, Husch A. FastField: An open-source toolbox for efficient approximation of deep brain stimulation electric fields. Neuroimage 2020; 223: 117330.

- fsl.fmrib.ox.ac.uk/fsl/fslwiki/FSL/

- Ewert S, Plettig P, Li N, et al. Toward defining deep brain stimulation targets in MNI space: A subcortical atlas based on multimodal MRI, histology and structural connectivity. Neuroimage 2018; 170: 271–282.