1. MRI Protocol

Contrast-enhanced brain MRI was performed with 1.5T (567 patients, Ingenia; Philips Healthcare, Best, the Netherlands), 18-channel cross cranial coil. Position: supine position. T1WI, T2WI, T2WI-FLAIR and DWI of the head were followed by enhanced scanning. Scanning parameters: (1) T1WI TR/TE = 528/15 ms, slice thickness = 6 mm, number of excitation = 1. (2) T2WI TR/TE  = 4397/110 ms, slice thickness = 6 mm, number of excitation = 1. (3) T2 FLAIR: TR/TE = 6000/120 ms, TI = 2000 ms, flip angle =100◦, slice thickness = 6 mm without slice gap, matrix size = 240×144, FOV = 230 mm×183 mm. (4) Diffusion scan was acquired using a single-shot, spin–echo planar imaging sequence with TR/TE = 2879/94 ms; slice thickness = 6 mm without slice gap; FOV: 230 mm× 230 mm; matrix: 152 ×106; and two diffusion weighting (b) values (0, 800s/mm2). The three-dimensional contrast-enhanced T1-weighted imaging (3D CET1WI) was obtained after intravenous injection of contrast material (Gadoterate meglumine, 0.1 mmol/kg; Gadodiamide, 0.1 mmol/kg), followed by a 15 ml saline flush with an injection velocity of 1.5 ml/s.

The other patients were performed with 3.0T (256 patients, MAGNETOM Prisma, Siemens, Erlangen, Germany), using a 64-channel head-neck coil. Position: supine position. T1WI, T2WI, T2WI-FLAIR and DWI of the head were followed by enhanced scanning. Scanning parameters: (1) T1WI TR/TE = 2000/20 ms, slice thickness = 6 mm, number of excitation = 1. (2) T2WI TR/TE =  3880/96 ms, slice thickness = 6 mm, number of excitation = 1. (3) T2 FLAIR: TR/TE = 8000/95 ms, TI = 2370 ms, flip angle = 150◦, slice thickness = 6 mm without slice gap, matrix size = 320 ×(320×70%), FOV = 230 mm×(230×84.4%) mm. (4) Diffusion scan was acquired using a single-shot, spin–echo planar imaging sequence with TR/TE = 2000/61 ms; slice thickness = 6 mm without slice gap; FOV: 256 mm× 256 mm; matrix: 128 × 128; and two diffusion weighting (b) values (0, 1000s/mm2). The three-dimensional contrast-enhanced T1-weighted imaging (3D CET1WI) was obtained after intravenous injection of contrast material (Gadoterate meglumine, 0.1 mmol/kg; Gadodiamide, 0.1 mmol/kg), followed by a 15 ml saline flush with an injection velocity of 1.5 ml/s.

2. CT Protocol

A standard contrast-enhanced chest CT scan was performed on all patients with a 64-multi-detector CT scanner (Brilliance CT, Philips Healthcare). The specific parameters were as follows: slice thickness, 1.0 mm; tube voltage, 120 kVp; tube current, 80 - 160 mAs; matrix, 512 × 512; and detector collimation, 0.625 mm. At the time of the examination, the patient held his/her breath. After the unenhanced scan, patients were injected intravenously with 1.2 ml/kg iodide contrast agent (Ioversol; 320 mg/ml iodine, HENGRUI Medicine) at a rate of 2.5 ml/s by an automatic pump injector. All images were reconstructed with a sharp kernel (b40).