Conceptualizing radical solutions to global warming via worldwide pipeline transport of green fuel ammonia produced from abundant renewable energy in equatorial waters

Hiroshi Kobayashi*

Professor Emeritus of Tokyo Denki University, 5 Senju-Asahi-cho, Adachi-ku, Tokyo, Japan

Supplementary Information

Supplementary Note A: Kobayashi, H. Carbon-free energy supply system using floating offshore photovoltaics and hydrogen energy carriers: towards the achievement of the Paris agreement goal. J. Jpn Soc. Energy Resour. 43, 274–284 (2022).

Permission to share this article was obtained from the Japan Society of Energy and Resources.

The English translated version of the pdf file of the aforementioned paper (Reference 4) can be downloaded from here.

Supplementary Note B: Global primary energy demand

According to Japan’s Energy White Paper 2023 [6] released by the Ministry of Economy, Trade, and Industry, the global primary energy demand continues to increase (average annual rate: 2.4%) based on economic and population growth (Supplementary Fig. 1). Since 2000, the socioeconomic growth in emerging countries in the Asia–Pacific region has been remarkable, and developed countries, especially those in the Organization for Economic Co-operation and Development, have experienced a decrease in the share of global energy demand from 71% (1965) to 39% (2021). In 2019, the share of fossil fuels, such as petroleum and liquefied natural gas, increased by an average of 2.5% annually, accounting for 82% of the total, whereas that of non-fossil fuels remained constant at 18%. In the future, GHG emissions will continue to increase; thus, the potential consequences for the human habitability of Earth could be catastrophic.

Supplementary Fig. S1 Changes in the world’s primary energy demand in gigatons of oil equivalent (Gtoe) (Source: Energy White Paper 2023 [6] with revisions).

Supplementary Fig. S3 Properties of carbon steel pipes used for high-pressure piping.

(a) Design and operating pressures according to the Schedule (Sch). Relationship between the design pressure (Pd) and Sch number is shown, representing the wall thickness of the pressure pipe when Sat = 121 N/mm2, used for STS 480, with the highest Sat at −40 ℃ < T < 60 ℃. Pd indicates the pressure that can withstand instantaneous pressurization, and Po is the operational pressure that can withstand long-term operation. Po can be expressed as follows: Po = Pd/SF = 12.9 MPa, where SF is the safety factor, which is generally set to 1.5 for piping. For carbon steel pipes used for high-pressure piping, the Sch ranges from 2 to 160. Dotted lines in the figure indicate the Sch values that exceed the standard, along with the corresponding Pd and Po values. (b) Outer diameter (OD), inner diameter (ID), and wall thickness (WT) of Sch 160. In JIS G 3455:2020 [17], the nominal diameter is defined from 15 to 650 for Sch 160. Solid lines: OD, ID, and WT of a nominal diameter; dotted lines: OD, ID, and WT when nominal diameter of Sch 160 = 1,000 (maximum nominal diameter of thin pipes with Sch = 10S or 20S). The ID for a nominal diameter of 1,000 is ~1.5 times larger than that for 650; the amount of LNH3 transport can be increased by a factor of 5. If the pressure resistance strength of JIS G 3455 is extended to that of Sch 200, LNH3 transport can be increased by ~6.5 times. This will allow the number of pipelines required to be reduced without compromising the supply. The service life of pipelines is 40–80 years [16]; pipelines of different diameters and pressures will become part of the network.

Supplementary Fig. S4 Properties of LNH3. (a) Vapor pressure line of NH3, (b) dynamic viscosity σ, and (c) density ρ.

Supplementary Table S1. Comparison of hydrogen energy carrier (HEC) characteristics with those of existing energy sources†

| HEC | Volumetric energy density (Wh/L) |  | Mass energy density (Wh/kg) |  | Storage conditions |  | Long-term storage | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High-pressure hydrogen gas (H2) | △ | 1,200 | ◎ | 32,900 | △ | 70 MPa | △ | △ | Highly flammable, explosive |
| Liquid hydrogen (LH2) | 〇 | 2,330 | ◎ | 32,900 | ✖ | －253 ℃ | ✖ | △ | Highly flammable, explosive |
| Fuel liquefied ammonia (LNH3) | 〇 | 3,960 | 〇 | 5,810 kg | ◎ | Room temperature, 0.86 MPa | ◎ | △ | Acute toxicity |
|  |  |  |  |  |  |  |  | 〇 | Many accomplishments |
| Methylcyclohexane (MCH) | △ | 1,550 | △ | 2,010 | ◎ | Room temperature | 〇 | 〇 | Flammability comparable to gasoline |
| (ref.) Liquefied natural gas | ◎ | 6,977 | ◎ | 15,167 | ✖ | －162 ℃ | △ | ✖ | CO2 emissions |
| (ref.) Crude oil | ◎ | 10,628 | ◎ | 12,541 | ◎ | Room temperature | ◎ | ✖✖ | High CO2 emissions |
| (ref.) Coal | ◎ | 9,972 | 〇 | 7,150 | ◎ | Room temperature | ◎ | ✖✖ | High CO2 emissions |
| (ref.) Li-ion secondary batter | ✖ | \520 | ✖ | 201 | ◎ | Room temperature | ✖ | ✖ | Lithium is rare and expensive |
| (ref.) Uranium fuel | ◉ | 7.6×109 | ◉ | 4.2×108 | 〇 | Cooled in fuel pool | ✖ Cooling required | ✖✖✖ | Final disposal method not defined |

Note: ◉, ◎, 〇, △, ✖, ✖✖, and ✖✖✖ represent better to worse from left to right, respectively.

†The energy density of LNH3 is lower than that of fossil fuels, but much higher than that of lithium-ion batteries; the energy density of uranium is > 10,000 times higher than that of fossil fuels.

Supplementary Table S2. Continental modeling

| Item | Value | Unit | Remarks |
| --- | --- | --- | --- |
| Area of modeled continent: Amc | 1.27E+08 | km2 | 85.2% of the total land area |
| Population of modeled continent: Pmc | 6.44E+09 | persons | 99% of the world’s population |
| Average population density: Pad | 50.9 | persons/km2 | = Pmc/Amc |
| East–west distance of modeled continent: Lew | 31,675 | km | = Amc/Lns |
| Latitudinal distance of modeled continent: Lns | 4,000 | km |  |

Supplementary Table S3. Resources for submarine HVDC cables between FOPV plants and landing stations

| Item | Copper | Aluminum | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Average submarine cable length: Lsm | 300 | 300 | km | An average cable length of 300 km is assumed based on Fig. 2. |
| Number of submarine cables: Nsm | 56,048 | 56,048 | cables | See Table 1, FOPV number satisfying 2TPESw8. |
| Maximum transmission voltage: Vsm | 500 | 500 | kV | See Table 1, maximum output voltage of FOPV8. |
| Maximum transmission current: Ism | 6.14 | 6.14 | kA | See Table 1, maximum output current of FOPV8. |
| Power transmission loss: Msm | 5% | 5% |  |  |
| Conductor diameter: Dsm | 56.7 | 73.3 | mm | Copper is next to silver, and aluminum is next to copper. |
| Cable conductor weight over Lsm: Wsm | 1.40E+04 | 6.87E+03 | tons | Copper and aluminum have specific gravities of 8.96 and 2.71 g/cm3, respectively. |
| Total conductor weight: Wtc | 7.85E+08 | 3.85E+08 | tons | = Nsm･Wsm. |
| Conductor mineral reserves [31]: Rcd | 5.6E+09 | 1.6E+10 | tons | Estimated copper resources, including undiscovered [31]. |
| Resource usage rate (Wtc/Rcd): Ucr | 14% | 2.3% |  | Aluminum is the best conductor for submarine cables. |

Supplementary Table S4. Resources for LNH3 pipelines and HVDC grids in intra-continental energy transport

| Item | HVDC power grid | LNH3 pipeline network | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Average east–west spacing: Sew | 15.4 | 15.4 | km | = Lew/PLw, see Table 1 |
| Average latitudinal spacing: Sns | 300 | 300 | km | Standard unit length of pipeline transport |
| Total energy transport length: Ltt | 8.62E+06 | 8.62E+06 | km | = Lew/Sew･Lns+Lns/Sns･Lew |
| Transmission voltage: Vtx | 500 | － | kV | The maximum HVDC voltage used is 500 kV |
| Transmission current: Itx | 23.2 | － | kA | Vtx･Itx･PLw･(hours/year) = 208 TWh/year |
| LNH3 flow rate: Qw | － | 47,879 | tons/day | Qw･EDNH3･(hours/year) = 208 TWh/year, see Supplementary Table 1 |
| Transmission media | Aluminum | Steel |  |  |
| Transmission unit length: Ltu | 1,000 | 300 | km |  |
| Energy loss over Ltu: Mtu | 5% | 0.114% |  | = Ep/(Vtx･Itx･θ), see Table 1 |
| Transmission material diameter: Dtm | 261 | ID: 532 | mm | Pipe OD: 660 mm |
| Material weight over Ltu: Wmu | 2.90E+05 | 2.84E+05 | tons | Specific gravity: Al = 2.72, Fe = 7.87 g/cm3 |
| Total material weight: Wtm | 2.50E+09 | 8.56E+09 | tons | = Wmu･Ltt/Ltu |
| Material reserves [31]: Rmt | 1.6E+10 | 2.3E+11 | tons | Suppose that aluminum reserves are 1/4 of bauxite reserves |
| Resource usage rate (Wtm/Rmt): Umr | 16% | 3.7% |  | LNH3 pipeline can transport carbon-free energy worldwide |
| Total energy loss over Lns: Mel | 20% | 1.52% |  | = Mtu･Lns/Ltn |
| Energy storage | Nowhere | Anywhere |  | HVDC grids can only store energy in LNH3 before converting it into electricity |