Table 1

specification of chemicals

| Chemical | CAS no. | Molecular weight g. mol-1 | Source | Purification method | Mass fraction purity (supplier) | Structure |
| --- | --- | --- | --- | --- | --- | --- |
| Sorbic acid | 110-44-1 | 112.13 | Loba Chemie Pvt. Ltd. India | Vacuum dried | 0.99 |  |
| Propylene glycol | 57-55-6 | 76.09 | Loba Chemie Pvt. Ltd. India | Vacuum dried | 0.99 |  |
| Hexylene glycol | 107-41-5 | 118.17 | Loba Chemie Pvt. Ltd. India | Vacuum dried | 0.99 |  |

As per the supplier.

Table 2

Values of densities (ρ), apparent molar volume (V∅),  of glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1 MPa.

| amA (mol.kg-1) | ρ×10-3/(kg.m-3) |  |  |  |  | V∅×106/ (m3 mole-1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |
| 0.00 mol.kg-1 PG + Water |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.998210 | 0.997040 | 0.995650 | 0.994030 |  |  |  |  |  |
| 0.09957 | 0.998468 | 0.997302 | 0.995917 | 0.994304 |  | 73.607 | 73.679 | 73.743 | 73.806 |
| 0.19901 | 0.998711 | 0.997550 | 0.996170 | 0.994564 |  | 73.663 | 73.728 | 73.791 | 73.853 |
| 0.30106 | 0.998948 | 0.997794 | 0.996419 | 0.994817 |  | 73.712 | 73.767 | 73.829 | 73.901 |
| 0.39926 | 0.999166 | 0.998015 | 0.996645 | 0.995050 |  | 73.753 | 73.811 | 73.873 | 73.942 |
| 0.50059 | 0.999373 | 0.998230 | 0.996869 | 0.995280 |  | 73.809 | 73.859 | 73.912 | 73.980 |
| 0.01 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.998860 | 0.997670 | 0.996290 | 0.994670 |  |  |  |  |  |
| 0.09994 | 0.999110 | 0.997923 | 0.996547 | 0.994927 |  | 73.651 | 73.728 | 73.800 | 73.879 |
| 0.20011 | 0.999347 | 0.998163 | 0.996789 | 0.995170 |  | 73.702 | 73.777 | 73.859 | 73.935 |
| 0.29996 | 0.999569 | 0.998387 | 0.997018 | 0.995398 |  | 73.755 | 73.832 | 73.907 | 73.991 |
| 0.39998 | 0.999779 | 0.998600 | 0.997235 | 0.995614 |  | 73.806 | 73.880 | 73.954 | 74.042 |
| 0.49994 | 0.999973 | 0.998800 | 0.997435 | 0.995815 |  | 73.863 | 73.929 | 74.010 | 74.098 |
| 0.03 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.999540 | 0.998340 | 0.996970 | 0.995330 |  |  |  |  |  |
| 0.10004 | 0.999781 | 0.998583 | 0.997214 | 0.995578 |  | 73.699 | 73.776 | 73.849 | 73.926 |
| 0.19997 | 1.000008 | 0.998812 | 0.997444 | 0.995812 |  | 73.750 | 73.827 | 73.901 | 73.978 |
| 0.29996 | 1.000222 | 0.999028 | 0.997661 | 0.996032 |  | 73.800 | 73.876 | 73.952 | 74.032 |
| 0.40012 | 1.000423 | 0.999232 | 0.997866 | 0.996239 |  | 73.851 | 73.926 | 74.002 | 74.086 |
| 0.50013 | 1.000611 | 0.999422 | 0.998060 | 0.996435 |  | 73.902 | 73.975 | 74.048 | 74.136 |
| 0.05 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1.000290 | 0.999110 | 0.997780 | 0.996120 |  |  |  |  |  |
| 0.09991 | 1.000520 | 0.999341 | 0.998014 | 0.996359 |  | 73.750 | 73.820 | 73.889 | 73.958 |
| 0.20112 | 1.000739 | 0.999562 | 0.998237 | 0.996589 |  | 73.804 | 73.871 | 73.943 | 74.001 |
| 0.29999 | 1.000941 | 0.999766 | 0.998443 | 0.996799 |  | 73.851 | 73.920 | 73.990 | 74.055 |
| 0.39995 | 1.001132 | 0.999958 | 0.998640 | 0.996999 |  | 73.902 | 73.971 | 74.036 | 74.107 |
| 0.50010 | 1.001311 | 1.000138 | 0.998825 | 0.997188 |  | 73.952 | 74.022 | 74.083 | 74.155 |
| 0.00 mol.kg-1 Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.998250 | 0.997080 | 0.995675 | 0.994050 |  |  |  |  |  |
| 0.09993 | 0.998445 | 0.997285 | 0.995918 | 0.994298 |  | 116.401 | 116.430 | 116.460 | 116.492 |
| 0.19989 | 0.998629 | 0.997479 | 0.996124 | 0.994525 |  | 116.430 | 116.462 | 116.491 | 116.521 |
| 0.29993 | 0.998804 | 0.997663 | 0.996321 | 0.994741 |  | 116.461 | 116.492 | 116.521 | 116.550 |
| 0.39895 | 0.998969 | 0.997836 | 0.996505 | 0.994943 |  | 116.486 | 116.521 | 116.551 | 116.582 |
| 0.49938 | 0.999129 | 0.998006 | 0.996683 | 0.995138 |  | 116.509 | 116.544 | 116.581 | 116.612 |
| 0.01 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.998860 | 0.997670 | 0.996290 | 0.994670 |  |  |  |  |  |
| 0.09895 | 0.999046 | 0.997866 | 0.996499 | 0.994899 |  | 116.423 | 116.452 | 116.484 | 116.512 |
| 0.19989 | 0.999222 | 0.998052 | 0.996697 | 0.995116 |  | 116.450 | 116.482 | 116.512 | 116.543 |
| 0.30018 | 0.999387 | 0.998228 | 0.996885 | 0.995322 |  | 116.483 | 116.511 | 116.543 | 116.573 |
| 0.39998 | 0.999544 | 0.998394 | 0.997063 | 0.995518 |  | 116.511 | 116.542 | 116.573 | 116.603 |
| 0.49899 | 0.999691 | 0.998552 | 0.997231 | 0.995703 |  | 116.542 | 116.572 | 116.604 | 116.634 |
| 0.03 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 0.999790 | 0.998490 | 0.997120 | 0.995480 |  |  |  |  |  |
| 0.10011 | 0.999963 | 0.998674 | 0.997316 | 0.995693 |  | 116.451 | 116.480 | 116.510 | 116.540 |
| 0.19997 | 1.000125 | 0.998848 | 0.997500 | 0.995894 |  | 116.482 | 116.510 | 116.540 | 116.570 |
| 0.29995 | 1.000278 | 0.999012 | 0.997676 | 0.996085 |  | 116.511 | 116.543 | 116.569 | 116.602 |
| 0.39983 | 1.000419 | 0.999165 | 0.997839 | 0.996262 |  | 116.548 | 116.577 | 116.606 | 116.641 |
| 0.49991 | 1.000552 | 0.999311 | 0.997995 | 0.996433 |  | 116.581 | 116.606 | 116.635 | 116.671 |
| 0.05 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1.000750 | 0.999420 | 0.997950 | 0.996380 |  |  |  |  |  |
| 0.09994 | 1.000909 | 0.999591 | 0.998134 | 0.996579 |  | 116.474 | 116.506 | 116.539 | 116.572 |
| 0.20014 | 1.001059 | 0.999752 | 0.998309 | 0.996768 |  | 116.505 | 116.539 | 116.572 | 116.601 |
| 0.29996 | 1.001197 | 0.999902 | 0.998472 | 0.996944 |  | 116.541 | 116.574 | 116.606 | 116.640 |
| 0.39992 | 1.001328 | 1.000044 | 0.998626 | 0.997112 |  | 116.571 | 116.604 | 116.636 | 116.670 |
| 0.49999 | 1.001449 | 1.000178 | 0.998771 | 0.997271 |  | 116.604 | 116.632 | 116.668 | 116.700 |

amA is the molality of (PG/HG) in the aqueous solution sorbic acid. Standard uncertainties u are  uT=0.01K, u(m)=1%, u(ρ)=0.5( kg.m-3) , u ( V∅ )=± (0.05-0.07) × 10-6(m3.mol-1)

Table 3

| amB (mol.kg-1) |  | V∅°×106/( m3 mole-1) |  |  |  | Sv*×106/ (m3 Kg mole-2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  |
| Propylene glycol (PG) |  |  |  |  |  |  |  |  |
| 0.00 | 73.56(±0.004) | 73.64(±0.003) | 73.70(±0.003) | 73.77(±0.004) | 0.49(±0.013) | 0.44(±0.009) | 0.42(±0.010) | 0.44(±0.012) |
| 0.01 | 73.60(±0.002) | 73.68(±0.002) | 73.75(±0.002) | 73.83(±0.006) | 0.53(±0.006) | 0.53(±0.006) | 0.51(±0.007) | 0.54(±0.004) |
| 0.03 | 73.65(±0.000) | 73.73(±0.000) | 73.80(±0.002) | 73.87(±0.001) | 0.51(±0.001) | 0.50(±0.001) | 0.50(±0.007) | 0.53(±0.004) |
| 0.05 | 73.70(±0.001) | 73.77(±0.001) | 73.84(±0.003) | 73.91(±0.004) | 0.50(±0.002) | 0.50(±0.002) | 0.48(±0.008) | 0.50(±0.010) |
| Hexylene glycol (HG) |  |  |  |  |  |  |  |  |
| 0.00 | 116.38(±0.003) | 116.40(±0.003) | 116.43(±0.000) | 116.46(±0.001) | 0.29(±0.009) | 0.30(±0.010) | 0.30(±0.001) | 0.27(±0.004) |
| 0.01 | 116.39(±0.001) | 116.42(±0.001) | 116.45(±0.001) | 116.48(±0.000) | 0.30(±0.004) | 0.30(±0.001) | 0.30(±0.003) | 0.30(±0.001) |
| 0.03 | 116.42(±0.002) | 116.45(±0.001) | 116.48(±0.002) | 116.50(±0.003) | 0.33(±0.007) | 0.32(±0.005) | 0.32(±0.007) | 0.33(±0.009) |
| 0.05 | 116.44(±0.002) | 116.48(±0.002) | 116.51(±0.001) | 116.54(±0.003) | 0.33(±0.005) | 0.32(±0.007) | 0.32(±0.004) | 0.33(±0.010) |

Partial molar volumes, (V∅°) and experimental slopes, (Sv*) of glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1 MPa.

amB is the molality of aqueous sorbic acid solution. Standard uncertainties u are  uT=0.001K, um1%, uρ=0.5 kg.m-3, u  V∅ =± 0.01 × 106m3 . mol-1 , uSv*= ± 0.03 × 106m3.kg. mol-2

Table 4

The partial molar volumes of transfer (∆V∅°) for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| amB (mol.kg-1) |  | ∆V∅°×106/( m3 mole-1) |  |  |  | ∆V∅°×106/( m3 mole-1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  | T=298.15K | T=303.15K | T=308.15K | T=293.15K |
|  | Propylene glycol (PG) |  |  |  |  | Hexylene glycol (HG) |  |  |  |
| 0.01 | 0.036 | 0.040 | 0.048 | 0.060 |  | 0.016 | 0.019 | 0.023 | 0.022 |
| 0.03 | 0.088 | 0.091 | 0.098 | 0.108 |  | 0.041 | 0.040 | 0.047 | 0.044 |
| 0.05 | 0.140 | 0.133 | 0.140 | 0.140 |  | 0.066 | 0.072 | 0.078 | 0.078 |

amB is the molality of aqueous sorbic acid solution. Standard uncertainties u are  uT=0.001K, um1%, uρ=0.5 kg.m-3.

Table 5

The empirical parameters values in Equation (4) for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| amB ( mol.kg-1) | a×106 /( m3 mole-1) | b×106 /( m3 mole-1) | c×106 /( m3 mole-1) | R2 | ARD |
| --- | --- | --- | --- | --- | --- |
| Propylene glycol (PG) |  |  |  |  |  |
| 0.01 | 73.677 | 0.016 | 0.000 | 0.999 | 0.00057 |
| 0.03 | 73.727 | 0.015 | 0.000 | 0.999 | 0.00030 |
| 0.05 | 73.773 | 0.014 | 0.000 | 0.999 | 0.00010 |
| Hexylene glycol (HG) |  |  |  |  |  |
| 0.01 | 116.422 | 0.006 | 0.000 | 0.999 | 0.00031 |
| 0.03 | 116.448 | 0.006 | 0.000 | 0.999 | 0.00020 |
| 0.05 | 116.475 | 0.007 | 0.000 | 0.999 | 0.00018 |

amB is the molality of aqueous sorbic acid solution. Standard uncertainties u are  uT=0.001K, um1%, uρ=0.5 kg.m-3,uU=1.0(m.s-1).

Table 6

Partial molar expansibilities, (E∅°) for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| E∅°×106/( m3 mole-1 .K-1) |  |  |  |  | (∂E∅°/∂T)P× 106/( m3.mol-1.K-2) |
| --- | --- | --- | --- | --- | --- |
| amB (mol.kg-1) | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  |
| Propylene glycol (PG) |  |  |  |  |  |
| 0.01 | 0.0162 | 0.0155 | 0.0149 | 0.0143 | -0.00012 |
| 0.03 | 0.0159 | 0.0153 | 0.0147 | 0.0141 | -0.00012 |
| 0.05 | 0.0149 | 0.0141 | 0.0133 | 0.0125 | -0.00014 |
| Hexylene glycol (HG) |  |  |  |  |  |
| 0.01 | 0.0061 | 0.0061 | 0.0060 | 0.0059 | -0.00001 |
| 0.03 | 0.0066 | 0.0600 | 0.0054 | 0.0048 | -0.00006 |
| 0.05 | 0.0070 | 0.0066 | 0.0062 | 0.0058 | -0.00004 |

amB is the molality of aqueous sorbic acid solution. Standard uncertainties u are  uT=0.001K, um1%,  uρ=0.5 kg.m-3,uU=1.0(m.s-1).

Table 7

The values of the speed of sound, denoted as U, and the apparent molar isentropic compression, represented as (K∅,S) for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| amA ( mol.kg-1) | U/ (m. s-1) |  |  |  |  | K∅,S×106/ m3 mole-1.GPa-1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |
| 0.00 mol.kg-1 PG + Water |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1481.3 | 1495.5 | 1509.0 | 1519.2 |  |  |  |  |  |
| 0.09957 | 1486.2 | 1500.3 | 1513.5 | 1523.6 |  | -45.129 | -44.276 | -43.486 | -42.903 |
| 0.19901 | 1490.5 | 1504.4 | 1516.6 | 1526.5 |  | -45.370 | -44.512 | -43.719 | -43.133 |
| 0.30106 | 1493.9 | 1507.8 | 1520.1 | 1529.5 |  | -45.458 | -44.599 | -43.805 | -43.219 |
| 0.39926 | 1496.9 | 1510.6 | 1523.0 | 1532.7 |  | -45.505 | -44.646 | -43.851 | -43.265 |
| 0.50059 | 1499.5 | 1513.5 | 1525.2 | 1534.4 |  | -45.538 | -44.678 | -43.883 | -43.297 |
| 0.01 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1482.8 | 1497.0 | 1510.9 | 1520.7 |  |  |  |  |  |
| 0.09994 | 1487.7 | 1501.8 | 1515.0 | 1525.1 |  | -45.040 | -44.189 | -43.378 | -42.819 |
| 0.20011 | 1492.0 | 1506.2 | 1518.1 | 1528.0 |  | -45.279 | -44.424 | -43.610 | -43.048 |
| 0.29996 | 1495.8 | 1509.3 | 1521.5 | 1531.0 |  | -45.365 | -44.508 | -43.693 | -43.131 |
| 0.39998 | 1498.9 | 1512.1 | 1524.5 | 1534.2 |  | -45.412 | -44.555 | -43.739 | -43.177 |
| 0.49994 | 1501.7 | 1515.0 | 1526.7 | 1536.2 |  | -45.443 | -44.586 | -43.770 | -43.207 |
| 0.03 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0 | 1484.6 | 1498.8 | 1512.7 | 1522.5 |  |  |  |  |  |
| 0.00000 | 1489.5 | 1503.6 | 1516.8 | 1526.9 |  | -44.932 | -44.083 | -43.275 | -42.719 |
| 0.10004 | 1493.8 | 1507.9 | 1519.9 | 1529.8 |  | -45.168 | -44.316 | -43.504 | -42.946 |
| 0.19997 | 1497.6 | 1511.1 | 1523.3 | 1532.8 |  | -45.254 | -44.400 | -43.587 | -43.028 |
| 0.29996 | 1500.7 | 1514.3 | 1526.3 | 1536.0 |  | -45.301 | -44.446 | -43.633 | -43.073 |
| 0.40012 | 1503.8 | 1517.4 | 1528.9 | 1538.5 |  | -45.332 | -44.477 | -43.663 | -43.103 |
| 0.05 mol.kg-1 Sorbic acid + Water + PG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1486.7 | 1501.0 | 1514.8 | 1524.6 |  |  |  |  |  |
| 0.09991 | 1491.6 | 1505.7 | 1518.7 | 1528.7 |  | -44.804 | -43.953 | -43.155 | -42.601 |
| 0.20112 | 1495.9 | 1509.8 | 1522.0 | 1531.9 |  | -45.042 | -44.187 | -43.385 | -42.829 |
| 0.29999 | 1499.7 | 1513.2 | 1525.4 | 1534.9 |  | -45.125 | -44.269 | -43.465 | -42.909 |
| 0.39995 | 1502.8 | 1516.4 | 1528.4 | 1538.1 |  | -45.171 | -44.314 | -43.510 | -42.953 |
| 0.50010 | 1505.9 | 1519.5 | 1531.0 | 1540.6 |  | -45.201 | -44.344 | -43.540 | -42.983 |
| 0.00 mol.kg-1 HG + Water |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1482.1 | 1495.5 | 1508.6 | 1519.8 |  |  |  |  |  |
| 0.09993 | 1490.3 | 1504.7 | 1515.5 | 1525.6 |  | -45.081 | -44.277 | -43.511 | -42.870 |
| 0.19989 | 1500.8 | 1512.4 | 1523.7 | 1533.4 |  | -45.319 | -44.510 | -43.742 | -43.100 |
| 0.29993 | 1508.5 | 1520.8 | 1531.3 | 1539.7 |  | -45.403 | -44.594 | -43.824 | -43.182 |
| 0.39895 | 1517.4 | 1528.5 | 1537.6 | 1545.8 |  | -45.448 | -44.639 | -43.869 | -43.227 |
| 0.49938 | 1523.6 | 1535.7 | 1543.4 | 1551.5 |  | -45.478 | -44.669 | -43.899 | -43.257 |
| 0.01 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1484.1 | 1497.5 | 1510.6 | 1521.8 |  |  |  |  |  |
| 0.09895 | 1492.3 | 1506.7 | 1517.5 | 1527.6 |  | -44.956 | -44.155 | -43.390 | -42.753 |
| 0.19989 | 1502.8 | 1514.4 | 1525.7 | 1535.4 |  | -45.196 | -44.391 | -43.624 | -42.985 |
| 0.30018 | 1510.5 | 1522.8 | 1533.3 | 1541.7 |  | -45.280 | -44.474 | -43.706 | -43.067 |
| 0.39998 | 1519.4 | 1530.5 | 1539.6 | 1547.8 |  | -45.325 | -44.518 | -43.751 | -43.112 |
| 0.49899 | 1525.6 | 1537.7 | 1545.4 | 1553.5 |  | -45.353 | -44.547 | -43.780 | -43.141 |
| 0.03 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1486.3 | 1499.7 | 1512.8 | 1524.0 |  |  |  |  |  |
| 0.10011 | 1494.5 | 1508.9 | 1519.7 | 1529.8 |  | -44.828 | -44.031 | -43.270 | -42.634 |
| 0.19997 | 1505.0 | 1516.6 | 1527.9 | 1537.6 |  | -45.062 | -44.260 | -43.497 | -42.860 |
| 0.29995 | 1512.7 | 1525.0 | 1535.5 | 1543.9 |  | -45.144 | -44.342 | -43.578 | -42.941 |
| 0.39983 | 1521.6 | 1532.7 | 1541.8 | 1550.0 |  | -45.188 | -44.386 | -43.622 | -42.985 |
| 0.49991 | 1527.8 | 1539.9 | 1547.6 | 1555.7 |  | -45.216 | -44.414 | -43.650 | -43.014 |
| 0.05 mol.kg-1 Sorbic acid + Water + HG |  |  |  |  |  |  |  |  |  |
| 0.00000 | 1488.6 | 1502.0 | 1515.1 | 1526.3 |  |  |  |  |  |
| 0.09994 | 1496.8 | 1511.2 | 1522.0 | 1532.5 |  | -44.689 | -43.895 | -43.137 | -42.506 |
| 0.20014 | 1507.3 | 1518.9 | 1530.2 | 1540.0 |  | -44.922 | -44.124 | -43.364 | -42.731 |
| 0.29996 | 1515.5 | 1527.3 | 1537.8 | 1547.2 |  | -45.003 | -44.205 | -43.444 | -42.810 |
| 0.39992 | 1524.5 | 1535.5 | 1545.1 | 1553.3 |  | -45.047 | -44.248 | -43.487 | -42.853 |
| 0.49999 | 1530.5 | 1542.2 | 1550.9 | 1560.0 |  | -45.074 | -44.276 | -43.515 | -42.882 |

amA is the molality of (PG/HG) in the aqueous solution sorbic acid. Standard uncertainties u are  uT=0.001K, um1%,  uρ=0.5 kg.m-3,uU=1.0m.s-1, uK∅,S= ±0.25 ×106(m3 mole-1.GPa-1)

Table 8

The partial molar isentropic compression (K∅,S°) for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| amB (mol.kg-1) | K∅,S°×106/ (m3 mole-1.GPa-1) |  |  |  |  | SK*×106/ (m3 Kg mole-2GPa-1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K | T=293.15K | T=298.15K | T=303.15K | T=308.15K |
| Propylene glycol (PG) |  |  |  |  |  |  |  |  |
| 0.00 | -45.12(±0.08) | -44.26(±0.08) | -43.47(±0.08) | -42.89(±0.08) | -0.95(±0.24) | -0.94(±0.23) | -0.93(±0.23) | -0.92(±0.23) |
| 0.01 | -45.03(±0.08) | -44.17(±0.08) | -43.36(±0.08) | -42.81(±0.07) | -0.93(±0.23) | -0.92(±0.23) | -0.90(±0.23) | -0.90(±0.22) |
| 0.03 | -44.92(±0.08) | -44.07(±0.08) | -43.26(±0.08) | -42.70(±0.07) | -0.93(±0.23) | -0.91(±0.23) | -0.90(±0.23) | -0.89(±0.22) |
| 0.05 | -44.79(±0.08) | -43.94(±0.08) | -43.14(±0.08) | -42.59(±0.07) | -0.95(±0.24) | -0.94(±0.23) | -0.93(±0.23) | -0.92(±0.23) |
| Hexylene glycol (HG) |  |  |  |  |  |  |  |  |
| 0.00 | -45.07(±0.08) | -44.26(±0.08) | -43.50(±0.08) | -42.86(±0.08) | -0.92(±0.24) | -0.91(±0.23) | -0.91(±0.23) | -0.90(±0.23) |
| 0.01 | -44.94(±0.08) | -44.14(±0.08) | -43.38(±0.08) | -42.74(±0.07) | -0.93(±0.24) | -0.91(±0.23) | -0.91(±0.23) | -0.91(±0.23) |
| 0.03 | -44.82(±0.08) | -44.02(±0.08) | -43.26(±0.08) | -42.62(±0.07) | -0.90(±0.23) | -0.89(±0.23) | -0.89(±0.23) | -0.88(±0.22) |
| 0.05 | -44.68(±0.08) | -43.88(±0.08) | -43.13(±0.08) | -42.49(±0.08) | -0.89(±0.23) | -0.88(±0.23) | -0.88(±0.23) | -0.88(±0.22) |

amB is the molality of aqueous sorbic acid solution Standard uncertainties u are  uT=0.001K, um1%,  uρ=0.5 kg.m-3,uc=1.0m.s-1, uK∅,S°= ±0.01 ×106(m3 mole-1.GPa-1), u (SK*)×106 (m3 Kg mole-2GPa-1)

Table 9

Partial molar isentropic compression transfer, ∆K∅,S° , for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| amB (mol.kg-1) | ∆K∅,S°×106/ m3 mole-1.GPa-1s |  | ∆K∅,S°×106/ m3 mole-1.GPa-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |  | T=293.15K | T=298.15K | T=303.15K | T=308.15K |
|  | Propylene glycol (PG) |  |  |  |  | Hexylene glycol (HG) |  |  |  |  |
| 0.01 | 0.089 | 0.087 | 0.107 | 0.083 |  | 0.124 | 0.121 | 0.119 | 0.116 |
| 0.03 | 0.198 | 0.192 | -0.469 | 0.183 |  | 0.252 | 0.246 | 0.400 | 0.234 |
| 0.05 | 0.324 | 0.321 | 0.330 | 0.301 |  | 0.390 | 0.380 | 0.372 | 0.362 |

amB is the molality of aqueous sorbic acid solution. Standard uncertainties u are  uT=0.001K, um1%,  uρ=0.5 kg.m-3

Table 10

Pair (VAB, KAB) and triplet (VABB, KABB), for glycols (PG and HG) in aqueous Sorbic acid solutions at different temperatures and the experimental pressure = 0.1

| T/K | VAB×106 /m3mole-2.Kg | VABB×106 /m3mole-1.Kg2 | KAB×106 /m3mole-2.Kg. GPa-1 | KABB×106 /m3mole-1.Kg2. GPa-1 |
| --- | --- | --- | --- | --- |
| Propylene glycol (PG) |  |  |  |  |
| 293.15 | 1.609 | -2.745 | 3.962 | -10.529 |
| 298.15 | 1.924 | -8.433 | 3.683 | -6.936 |
| 303.15 | 2.302 | -12.266 | -14.388 | 227.046 |
| 308.15 | 2.787 | -18.762 | 3.456 | -6.477 |
| Hexylene glycol (HG) |  |  |  |  |
| 293.15 | 0.670 | -0.848 | 6.689 | -55.131 |
| 298.15 | 0.769 | -1.048 | 7.496 | -79.714 |
| 303.15 | 0.939 | -1.896 | 11.582 | -133.254 |
| 308.15 | 0.880 | -2.490 | 6.126 | -50.119 |

T/K is the temperatures.