Supplementary information

SI. The chemical and physical analysis of sediment and seawater

SII. Statistical analysis

Table S1. Physical and chemical properties of sediment and seawater with standard deviation for replicate sediment samples (n=3).

|  | Physical and chemical properties | Content |
| --- | --- | --- |
| Sediment | TN (mg∙g-1 DW) | 0.98±0.05 |
|  | TOC (mg∙g-1 DW) | 6.81±0.05 |
|  | pH | 8.86±0.06 |
|  | NO3-(mg∙g-1 DW) | 0.126±0.08 |
|  | NO2-(mg∙g-1 DW) | 0.074±0.003 |
|  | NH3/NH4+(mg∙g-1 DW) | 0.089±0.012 |
|  | NH4+(mg∙g-1 DW) | 0.096±0.061 |
|  | Ca (mg∙g-1 DW) | 34.59±0.03 |
|  | Fe (mg∙g-1 DW) | 1.73±0.08 |
|  | Al (mg∙g-1 DW) | 61.39±0.06 |
|  | Mn (mg∙g-1 DW) | 0.61±0.01 |
|  | Quartz (%) | 26 |
|  | Potash feldspar (%) | 4 |
|  | Illite (%) | 47 |
|  | Chlorite (%) | 3 |
|  | Calcite (%) | 2 |
|  | Dolomite (%) | 10 |
|  | Plagioclase (%) | 8 |
| Seawater | Na+ (g∙L-1) | 9.46 |
|  | K+ (g∙L-1) | 0.34 |
|  | Ca2+ (g∙L-1) | 0.38 |
|  | Mg2+ (g∙L-1) | 1.02 |
|  | SO42- (g∙L-1) | 2.69 |
|  | Cl- (g∙L-1) | 17.84 |
|  | HCO3- (g∙L-1) | 0.16 |
|  | NH4+ (μg∙L-1) | 0.13±0.07 |
|  | NO3- (mg∙L-1) | 0.98±0.06 |

SII. Statistical analysis

Table S2. The total amount of adsorbed nitrate, the amount of adsorbed nitrate onto unit sediment, the total amount of desorbed nitrate and the amount of desorbed nitrate and ammonium from unit sediment in solution, with different salinities and sediment concentrations (Sc).

| Treatment |  | Total adsorbed nitrate (mg) | Unit adsorbed nitrate (mg∙g-1) | Total desorbed nitrate (mg) | Unit desorbed nitrate (mg∙g-1) | Total adsorbed ammonium (mg) | Unit adsorbed ammonium (mg∙g-1) | Total desorbed ammonium (mg) | Unit desorbed ammonium (mg∙g-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Salinity | 0 | -0.0458a | -1.8311a | 0.0042a | 0.1670a | -0.0113a | -0.4393a | 0.0036a | 0.1433a |
|  | 5 | -0.0398a | -1.5918a | 0.0101ab | 0.4044ab | -0.0112a | -0.4460a | 0.0040bc | 0.1610bc |
|  | 10 | -0.0218b | -0.8720b | 0.0124ab | 0.4959ab | -0.0126b | -0.5040b | 0.0041b | 0.1631b |
|  | 15 | -0.0102b | -0.4080b | 0.0143b | 0.5704b | -0.0130c | -0.5187c | 0.0042d | 0.1684d |
|  | 20 | -0.0156c | -0.6220c | 0.0110b | 0.4390b | -0.0154bc | -0.6173bc | 0.0043bc | 0.1733bc |
|  | 25 | -0.0116c | -0.4652c | 0.0108c | 0.4319c | -0.0161c | -0.6440c | 0.0051bc | 0.2034bc |
|  | 30 | -0.0094d | -0.3740d | 0.0086d | 0.3422d | -0.0180c | -0.7187c | 0.0062c | 0.2480c |
| Sc (g·L-1) | 0.5 | -0.0458a | -1.8311a | 0.0042a | 0.1670a | -0.0043a | -0.1738a | 0.0023a | 0.0930a |
|  | 1 | -0.0434ab | -0.8685b | 0.0032a | 0.0634b | -0.0054ab | -0.1086b | 0.0029a | 0.0577b |
|  | 3 | -0.0426a | -0.2843c | 0.0038b | 0.0251c | -0.0049abc | -0.0324c | 0.0046a | 0.0306c |
|  | 5 | -0.0403a | -0.1613d | 0.0039b | 0.0154d | -0.0052abcd | -0.0209d | 0.0043a | 0.0173cd |
|  | 10 | -0.0424ab | -0.0848e | 0.0033b | 0.0067de | -0.0050acd | -0.0100e | 0.0047a | 0.0094de |
|  | 20 | -0.0388b | -0.0388e | 0.0034c | 0.0034e | -0.0062cd | -0.0062ef | 0.0062a | 0.0062e |
|  | 40 | -0.0384b | -0.0192e | 0.0031c | 0.0016e | -0.0074d | -0.0037f | 0.0090a | 0.0045e |

Different letters in columns indicate significant differences (P<0.05; t-test) within salinity or sediment concentration to amount of adsorbed nitrate and ammonium, unit adsorbed nitrate and ammonium, desorbed nitrate and ammonium and unit desorbed nitrate and ammonium.

Table S3. Analysis of variance for the total amount of adsorbed nitrate and ammonium, the amount of adsorbed nitrate and ammonium onto unit sediment, the total amount of desorbed nitrate and ammonium and the amount of desorbed nitrate and ammonium from unit sediment as affected by sediment concentration (Sc) and salinity and the interactions between each other. And analysis of variance for the amount of desorbed nitrate and ammonium from unit sediment as affected by salt ion and sediment particle size.

| Treatment | Total adsorbed nitrate (mg) | Unit adsorbed nitrate (mg∙g-1) | Total desorbed nitrate (mg) |  | Unit desorbed nitrate (mg∙g-1) |  | Total adsorbed ammonium (mg) |  | Unit adsorbed ammonium (mg∙g-1) |  | Total desorbed ammonium (mg) |  | Unit desorbed ammonium (mg∙g-1) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sc | NS | ** |  | ** |  | ** |  | * |  | ** |  | NS |  | ** |  |
| Salinity | ** | ** |  | ** |  | ** |  | ** |  | ** |  | ** |  | ** |  |
| Sc*Salinity | NS | ** |  | ** |  | ** |  | * |  | ** |  | ** |  | ** |  |
| Salt ion | / | / |  | / |  | NS |  | / |  | / |  | / |  | * |  |
| Sediment particle size | / | / |  | / |  | NS |  | / |  | / |  | / |  | NS |  |

NS not significant (P≥0.05); * P<0.05; ** P<0.01.