Supplementary Information for:

Lake toxin concentrations increase when phosphorus is reduced

Charlotte Schampera 1, *, Julian Merder2, Thomas Niemeyer 1, Julia Pries 1, Ferdi L. Hellweger 1, **

Affiliation:

1 Water Quality Engineering,

Technical University of Berlin, Germany

Sekr. KF 4

Straße des 17. Juni 135

10623 Berlin / Germany

2 Department of Global Ecology,

Carnegie Institution for Science

260 Panama St

94305 Stanford, CA / USA

*Corresponding author

Charlotte Schampera

Water Quality Engineering

Technical University of Berlin

Sekr. KF 4

Straße des 17. Juni 135

10623 Berlin / Germany

email: schampera@tu-berlin.de

**Corresponding author

Ferdi L. Hellweger

Water Quality Engineering

Technical University of Berlin

Sekr. KF 4

Straße des 17. Juni 135

10623 Berlin / Germany

phone: +49 (0)30 314-25847

email: ferdi.hellweger@tu-berlin.de

Material and Methods

Literature review on laboratory studies

Studies investigating the effect of varying nitrogen concentration or irradiance on MC quota were catalogued by Hellweger et al.1. Abbreviations in figure 1 of the main paper are consistent with that study and indicate the letter of authors’ initial and year (e.g.WO75 is Watanabe and Oishi, 1975)2. The two-fold change (i.e. logarithm to the base of two - log2 of ratio) of lowest to highest value (above zero) was calculated. In cases of time-series data, only the last data point was considered. In cases of other altered conditions (e.g. varying phosphorus concentrations, temperatures, etc.) or various strains investigated, the average of all measurements was used (i.e. each point corresponds to one study and are potentially the average of several individual experiments). For light conditions, environmentally relevant ranges of irradiances were included (i.e. if several high light intensities were measured, irradiances below 200 µmol photons/m2/s were used).

Literature review on field data

A strategic literature review on long-term data of cyanotoxin in lakes with phosphorus reduction3 was performed using the web-based search platforms “GoogleScholar”, “Web of Science” and “Google Search”. The combination of “Lake name” and “cyanotoxin/cyanobacteria/microcystin” was applied. Furthermore, references of 3 were audited. Lakes were grouped in the categories “no data”, “single data points” and “long-term data” (i.e. > 3 data points in one blooming season).

Analysis National Lakes Assessment

The NLA survey from the Environmental Protection Agency (EPA) is conducted in the United States of America to strategically sample lakes, reservoirs and ponds for common limnological data including measurements from 6918 lakes in the years 2007, 2012 and 2017. For methodology of sampling and analyses refer to the Field and Lab Manual (Manuals Used in the National Aquatic Resource Surveys | US EPA). Data were extracted from the website (Data from the National Aquatic Resource Surveys | US EPA) and processed with Phyton, R.Studio and Excel. Data were analyzed using two different approaches:

- Regression analysis

The dataset was filtered for lakes with detectable concentrations of MCs within two sampling events. For all parameters, the change among sampling years was calculated for each lake using a two-fold change (log2). Hereby, data show proportional change of a parameter rather than absolute change. For instance, in 2007 a microcystin (MC) concentration of 0.3 and in 2012 of 0.66 was measured in Cedar Creek Reservoir 9-A. The fold change is 1.14 (i.e. log2(0.660.3)), and the parameter has more than doubled. Correlation analyses were performed for changes in TP and changes in the parameters chlorophyll a (Chla), dissolved inorganic nitrogen (DIN), turbidity, MC per biomass (MC/Chla) and MC concentration. Spearman rank correlation tests were performed in R.Studio (R4.3.2) with the dplyr package4.

- Generalized Additive Models (GAMs)

GAMs were fitted to each response variable using smoothing splines to allow for non-linear main effects and non-linear interactions between log TN and log TP, resulting in models of the following form:

g(E(Y)) = f1(logTN)+f2(logTP)+f3(logTN,logTP) 

where E(⋅) denotes the expected value, g(.) the log link function, f1(.) the main effect term of log TN, f1(.) the main effect term of log TP, and f3(.) the non-linear interaction. We used thin plate splines5 for DIN, turbidity, the MC/Chla ratio and MC using the “R” package “mgcv”6. For Chla, we exchanged the thin-place splines with monotonic splines from the “scam” package 7 to reduce the fit’s wiggliness, which did not affect the general trend observed. We tested if each model could be simplified by dropping the interaction term or by exchanging a smoothing function with a linear term, using the Akaike Information Criterion (AIC)8. For all models, non-linear main effects and non-linear interactions resulted in the best model with the lowest AIC. The only exception was the MC to Chla ratio model, where a model excluding the interaction (f3(.)) produced the lowest AIC. Predictions were generated on a grid of log TN and log TP values to visualize and capture the joint effects of these predictors on each response variable.

Reference

1.	Hellweger, F. L. et al. Models predict planned phosphorus load reduction will make Lake Erie more toxic. Science (1979) 376, 1001–1005 (2022).

2.	Watanabe, M. F. & Oishi, S. Effects of Environmental Factors on Toxicity of a Cyanobacterium (Microcystis Aeruginosa) under Culture Conditions. APPLIED AND ENVIRONMENTAL MICROBIOLOGY vol. 49 https://journals.asm.org/journal/aem (1985).

3.	Schindler, D. W., Carpenter, S. R., Chapra, S. C., Hecky, R. E. & Orihel, D. M. Reducing phosphorus to curb lake eutrophication is a success. Environ Sci Technol 50, 8923–8929 (2016).

4.	Wickham, H., François, R., Henry, L., Müller, K. & Vaughan, D. dplyr: A Grammar of Data Manipulation. Preprint at https://dplyr.tidyverse.org (2023).

5.	Wood, S. N. Thin Plate Regression Splines. J. R. Statist. Soc. B vol. 65 (2003).

6.	Wood, S. N. Generalized Additive Models: An Introduction with R. (CRC Press, A Chapman & Hall Book, 2017).

7.	Pya Arnqvist, Natalya. _scam: Shape Constrained Additive Models_. R package. Preprint at https://CRAN.R-project.org/package=scam (2023).

8.	Akaike, H. A new look at the statistical model identification. IEEE Transactions on Automatic Control 19, 716–723 (1974).