MRI image acquisition and processing

The specific machines included Simens Magnetom Skyra 3.0T MRI (Siemens, Germany), GE Medical Systems 3.0T MRI (GE, USA) and Philips Medical Systems 3.0T MRI (Philips Netherlands) imaging system], a 16-channel phased array body coil, and a high-pressure syringe. Scanning sequence and parameters: ①T1-weighted imaging (T1WI) : repetition time (TR) 3.70 ~ 4.50 ms, echo time (TE) 1.20 ~ 1.90 ms, layer thickness 3.50 ~ 5.00 mm, field of view, FOV) 260 mm×320 mm ~ 480 mm×480 mm; ②T2-weighted imaging (T2WI) : TR 4 000 ~ 6 000ms, TE 70 ~ 100 ms, layer thickness 7.50 ~ 9.00 mm, layer thickness 8.45 ~ 9.60 mm, FOV 260 mm×320 mm ~ 512 mm×512 mm. ③T1WI multi-phase enhanced scanning: Simens Magnetom Skyra 3.0T MRI (Siemen, Germany), GE Medical Systems 3.0T MRI (GE, USA) and Philips Medical Systems 3.0T MRI (Philips, Netherlands) imaging systems respectively used 3D VIBE T1WI, BH Ax LAVA+C and mDIXON-W-dyn sequence, TR 2.90 ~ 4.20ms, TE 1.50 ~ 1.96ms, layer thickness 3.50 ~ 5.00mm. FOV 260 mm x 320 mm ~ 512 mm x 512 mm. A high-pressure syringe was used to inject the contrast agent disodium gadolinate (Bayer Healthcare Co., LTD., Germany) into the cubitus vein mass with a dose of 0.025 mmol/kg and a flow rate of 1 ml/s. AP, PP, HBP images were acquired at 25 ~ 30s, 55 ~ 60s and 20 min after injection of the contrast agent, respectively. All the original loaded MRI images of patients in the pre‑contrast phase (Pre), AP, PP and HBP were in DICOM format.