Appendix

The ground motion prediction equations compared to previous models.

MF13 model

(Morikawa and Fukushima 2013)

In case of interplate events

preMF13 = 0.5507 Mwj-0.004716 Rij -log10(Rij+ 0.006875 100.5 Mwj) + 0.5418 + Gd + AI + ηj+ξij

In case of intraslab events

preMF13 = 0.5507 Mwj-0.005273 Rij -log10(Rij+ 0.006875 100.5 Mwj) + 0.9338 + Gd + AI + ηj+ξij

In case of crustal events

preMF13 = 0.5507 Mwj-0.004531 Rij -log10(Rij+ 0.006875 100.5 Mwj) + 0.4631 + Gd + AI + ηj+ξij

AI = 0.00007602 Xvfij (Hj-30)

Gd = 0.0663 log10(D14ij/250)

where Xvf is the distance from the volcanic front to observation sites (km). If the focal depth is <3 km, H=30.

H120 model

(Hu et al. 2020)

preH20 = 1.073 Mwj-0.00509 Rij-logeRij+5.00+ 0.00609 Hj-15 δh+Sk+F+ ηj+ξij

where δh is dummy variable for a focal depth, which is 1 in H > 15 or 0 in H ≦ 15, F is 0.164 in case of interplate events, 0.344 in case of intraslab, 0 in other case.

HZ23 model

(Hu et al. 2023)

preHZ23 = 1.101 Mwj -0.00564 Rij -loge(Rij+ 0.0055 * exp(1.080*Mwj)) + 0.01412 (Hj-15) δh+ZF+Fres+Ck + ηj+ξij

ZF is 0 in the case of interplate events, 2.607 -0.528 logeRij in the case of intraslab, 0.251 in the case of crustal events.

In the case of interplate events,

Fres = 12.8616+3.9797 Mwj-20.4891 loge(Mwj) -0.0887 loge(Hj)+0.2237 loge(Rij)

In the case of intraslab events,

Fres = 9.2943+3.9797 Mwj-20.3401 loge(Mwj) -0.7248 loge(Hj)+0.0603 loge(Rij)

In the case of crustal events,

Fres = 11.0858+3.8848 Mwj-18.4392 loge(Mwj) -0.0698 loge(Hj)+0.0022 loge(Rij)

The appendix Tables A1 and A2.

Table A1: List of seismic events.

Table A2: List of S-net stations.