Extended Data Figures for

Synthesis of Paracrystalline Diamond

Hu Tang1, Xiaohong Yuan1, Yong Cheng2, Hongzhan Fei3, Fuyang Liu1, Tao Liang1, Zhidan Zeng1, Takayuki Ishii3, Ming-Sheng Wang2, Tomoo Katsura3, Howard Sheng4,*, Huiyang Gou1,5,*

1Center for High Pressure Science and Technology Advanced Research, Beijing 100094, China

2Department of Materials Science and Engineering, College of Materials, Xiamen University, Xiamen, Fujian 361005, China

3Bayerisches Geoinstitut, Universität Bayreuth, Bayreuth D95440, Germany

4Department of Physics and Astronomy, George Mason University, Fairfax 22030, USA

5College of Environmental and Chemical Engineering, Yanshan University, Qinhuangdao 066004, Hebei, China

*Corresponding authors: Howard Sheng (hsheng@gmu.edu), Huiyang Gou (huiyang.gou@hpstar.ac.cn)

Table of content

Extended Data Fig. 1 XRD patterns of samples from different precursors

Extended Data Fig. 2 Raman spectra and EELS data of recovered samples

Extended Data Fig. 3 HRTEM images

Extended Data Fig. 4 Comparisons between a-D, p-D and NPD

Extended Data Fig. 5 Short-range ordering up to second-nearest-neighbors

Extended Data Fig. 6 Two atomic-shell structures of CD and HD

Extended Data Fig. 7 Mechanical properties of recovered samples

Extended Data Fig. 8 Thermostability of p-D vs. NPD

Extended Data Fig. 9 Structural evolution of fcc-C60 under room-temperature compression

Extended Data Fig. 10 Structural evolution of fcc-C60 heated to 1600K at 30 GPa

Extended Data Fig. 11 Enthalpy and volume during the heating treatment of C60 at 30 GPa

Extended Data Fig. 12 Hidden orientational ordering of sp3 bonds in compressed C60

Extended Data Fig. 13 Classical MD simulation of nucleation and growth of p-D from a-D

Extended Data Fig. 1 XRD patterns of samples synthesized from different precursors. C60 (top), type-1 glassy carbon (middle), and carbon onion (bottom) at 30 GPa and 1400-1800 K, respectively. At 1800 K, the appearance of weak Bragg diffraction peaks indicates nano-crystallization in the non-crystalline diamond formed from C60. At 1600 K, fully sp3-bonded p-D was identified. By contrast, in cases of using type-1 glassy carbon and carbon onion as the starting material, crystallization precedes the complete conversion of sp2 to sp3 carbon at 30 GPa and 1400-1600 K, preempting the formation of fully sp3-bonded paracrystalline diamond. The experiments demonstrate the importance of the structure of the starting material in the formation pathway of p-D.

Extended Data Fig. 2 Raman spectra and EELS in the low-loss region of the recovered samples.

a, Visible Raman (532 nm) spectrum of the sample recovered at 30 GPa and 1400 K, with background removed. The broadened Raman peak can be fit with a D peak at 1378 cm-1 and a G peak at 1556 cm-1. The peak intensity ratio of ID/IG is 0.66 and the full width at half maximum (FWHM) of the G peak is 175 cm-1. Similar to our work, the presence of sp2 bonds results in a wide Raman peak between 1400 cm-1 and 1700 cm-1 in the visible Raman spectra of diamond-like carbon (DLC) films27.

b, Ultraviolet (UV) Raman spectra of the samples recovered from 1400 and 1500 K, respectively. The strong peak at 1110 cm-1 in the UV Raman spectrum of the sample recovered from 30 GPa and 1400 K is attributed to the T peak caused by sp3 bonds27. Fitting the Raman profile yields a peak intensity ratio of IT/IG 0.91, which is higher than that of DLC films with 88% sp3 bonds, indicating a higher concentration of sp3 bonds in our sample27. On the other hand, for the sample recovered from 1500 K, no obvious peaks are discernible in the UV Raman spectrum, indicating the absence of sp2 bonds in the as-synthesized disordered carbon8.

c, EELS in the low-loss region of diamond crystal, single-crystal graphite, and samples synthesized from C60 under 30 GPa and 1400-1500 K, respectively. Based on plasmon peak energy derived from low-loss EELS, the densities of the recovered samples at 1400-1600 K were calculated to be 3.11, 3.20, and 3.25 g/cm3, respectively. Due to the presence of ~5.2% sp2 carbon, the sample recovered from 1400 K has a lower density of 3.11 g/cm3.

Extended Data Fig. 3 HRTEM images of samples recovered at 30 GPa and different temperatures. a-c, Typical HRTEM images of samples synthesized at 30 GPa and 1500, 1600, and 1800 K, respectively. a-c and h-j, The inverse FFT images and FFT patterns corresponding to the areas marked by the white boxes (7.0 × 7.0 nm2) in (a), (b), and (c), respectively. The cyan and yellow circles in (e and f) indicate cubic- and hexagonal-like MRO clusters, respectively. In subfigure g, the lattice fringes marked with cyan and yellow solid lines indicate that nanosized cubic and hexagonal diamond crystallites are precipitated from the non-crystalline matrix at 1800 K.

Extended Data Fig. 4 Structure factor S(Q), structural models, and simulated HRTEM images of simulated a-D, p-D with different fractions of paracrystals, and NPD with different grain sizes. a, S(Q) of simulated a-D and p-D with different fractions of paracrystals. With increasing , the intensity of the first peak increases, and the position of the peak shifts to high-Q, which is consistent with the experimental observation that at higher annealing temperatures, the intensity of the first peak increases and the position moves to the right (from 2.89Å-1 at 1200 K to 2.95 Å-1 at 1600 K, Figure 3b in the main text). b, S(Q) of simulated NPDs with different grain sizes, juxtaposed with that of the 1800 K sample from our experiment. For the simulated NPD samples, the peak intensity decreases and the peak width broadens as the grain size is refined. Bragg peaks are clearly discernable for the samples with a grain size of 1.2 nm. Experimentally, the protruding diffraction peaks in the 1800 K sample corresponds exactly to the Bragg peaks characteristic of NPD, indicative of crystallization. c, In ultrafine NPD, the less-distorted “core” regions of the nanocrystals (blue spheres) are clearly seen. For NPD with d = 1.56 nm,  is estimated to be 94%, meaning that 6% atoms are located at the interfaces and cannot be properly assigned to any of the nanocrystalline grains.

Extended Data Fig. 5 Short-range ordering of a-D, a-Si, p-D, and NPD. a, Distributions of tetrahedrality qt for a-D and a-Si, both of which were obtained from the WWW bond-switching approach and further relaxed with ab initio MD. The tetrahedral order parameter qt is defined as qt=1-38i=13j=i+14cosθij+1/32, where θij is the angle formed by two vectors pointing from the center atom to two of its neighboring atoms. The summation runs over all the combinations of the four nearest neighbors. The narrow distribution of a-D clearly indicates that a-D has strong tetrahedral SRO than a-Si. b, Bond-angle distribution within the first atomic shell for a-D and a-Si. Compared with a-Si, a-D has a narrower bond-angle distribution. c, Distributions of the 2nd-nearest-neighbor order parameter s of a-D and a-Si. The order parameter s measures the similarity of the local atomic environment (16 atoms) of a central atom with respect to perfect CD or HD lattice arrangements. The higher the s value, the less distortion of the local cluster. Compared with a-Si, the distribution function of s for a-D shifts to high s values with a centroid of 0.42, suggesting much enhanced SRO within the first two atomic shells in a-D. d, Distributions s of a-D, p-D, and NPD with a grain size of 1.92 nm. The inset atomic clusters illustrate the cluster configurations (red spheres) corresponding to their respective degree of similarity with the perfect CD lattice arrangement (green spheres). Paracrystalline diamond has a medium s value (0.51) between that of a-D and NPD, revealing crystalline ordering in p-D. It is worth noting that, for NPD with d = 1.56 nm (for which the atomic configuration can be found in Extended Data Fig. 4), the distribution peak at s = 0.95 corresponds to less-distorted interior atoms in the nano-grains. Such atoms are absent in paracrystalline diamond, marking the distinction between p-D and NPD.

Extended Data Fig. 6 Two atomic-shell structures (side and top views) of CD and HD (16 nearest neighbors) atomic packing. The 12 second nearest neighbors in CD and HD are arranged in the FCC and HCP packing modes, respectively.

Extended Data Fig. 7 Mechanical properties of recovered samples. a, Vickers hardness Hv of the samples recovered at 30 GPa and different temperatures, measured at a load of 4.9 N. Inset: an optical image of the indentation of p-D recovered from 1600 K. b, Loading/unloading displacement curves of p-D recovered from 1600 K during indentation measurement. Inset: A table of nanoindentation hardness HN and elastic modulus E obtained from different test points. c, Overview of shear modulus G versus Vickers hardness Hv for typical superhard ceramics42. The shear modulus G (558 GPa) was derived through a formula G = E/[2(1+μ)], where the elastic modulus E (1194 GPa) was experimentally measured (shown in b) and μ is the Poisson's ratio (0.07) of diamond. Note that both the hardness and the shear modulus of the p-D recovered from 1600 K are higher than those of natural diamond.

Extended Data Fig. 8 Thermostability of p-D. a, Both thermogravimetry (TG) and DSC curves of p-D were measured in air at a heating rate of 5 K/min. b, Thermostability of p-D, compared with other superhard carbons43-45. Superhard (50-80 GPa) DLC films have a wide range of onset oxidation temperature (673-873 K), depending on the fraction of sp3 bonds. The thermostability of p-D is superior to that of NPD (d = ~13 nm)64.

Extended Data Fig. 9 Structural evolution of C60 under room-temperature compression. a, Structural model of C60 under 30 GPa and room temperature (300 K), obtained by classic and ab initio MD. Under high pressure (30 GPa), the inter-molecule distance decreases to 0.86 or 0.87 nm from 1.0 nm. During compression to 30 GPa at room temperature, the buckyballs are deformed and inter-buckyball bonding occurs by formation four-fold (sp3) bonds (15%). In this and the following figures, the atoms with sp3 bonding are colored in blue. The fraction of sp3 atoms was approximated by the number of 4-fold coordinated atoms within a cut off distance (in this work, rcut=1.85 Å). We shall point out that, the sp3 fraction calculated this way is slightly underestimated in comparison to that calculated quantum-mechanically, e.g., with the maximally localized Wannier function (MLWF)9. b, In situ Raman spectra of C60 under compression (up to ~32 GPa), with an excitation wavelength of 532 nm. With increasing pressure, the Raman peaks of the Ag mode corresponding to the breathing of totally symmetric vibration and the Hg mode shift rightward and are gradually broadened with decreased intensity, suggesting the polymerization and deformation of buckyballs, consistent with our theoretical results.

Extended Data Fig. 10 MD simulation of fcc-C60 heated to different temperatures at 30 GPa. a, Classical MD simulation at a heating rate of 1010 K/s. The fraction of sp3 bonds increases with increasing temperature. There is a percolation transition of sp3 bonds at 500-800 K. Such a percolation transition corresponds to the polymerization of C60 molecules. It should be pointed out that the fractions of sp3 bonding were determined based on the criterion of 4-fold bonding within 1.85 Å, which may be slightly underestimated by a few percent. b, Ab initio MD simulation of fcc-C60 (720 atoms) at a heating rate of 2 × 1014 K/s. Similar to classic MD simulation, the fraction of sp3 bonds increases with increasing temperature in ab initio MD. The simulation results validate what was observed in classical MD. Note the persistence of parallel sp3 bonds at 900 - 1500 K.

Extended Data Fig. 11 Enthalpy and volume as a function of temperature during the heating treatment of C60 at 30 GPa. The decrease of enthalpy with increasing temperature (500-1250 K) indicates that the sp2-sp3 transition (25% to 70% as shown in Extended Data Fig. 10a) mainly occurs between 500 K and 1250 K, accompanied by the densification of the material due to the formation of sp3 bonding. The uprises of the enthalpy at temperatures below 500 K and above 1250 K are mainly caused by the thermal effect.

Extended Data Fig. 12 Hidden orientational ordering of sp3 bonds in compressed C60. Top: MD configuration of a-D obtained from C60 after 30 GPa and 1600 K without long-time isothermal annealing (Extended Data Fig. 10a). Middle: 3D bond-orientation distribution function projected onto the unit-sphere surface. Bottom: Two-dimensional stereographic projection of the bond-orientation function showing the pole figure of preferred bond orientations. The lines serve as a guide to the eye. Non-uniform intensity distributions suggest that the sp3 bonds in the as-obtained a-D have preferred orientations, inherited from the initial parallel sp3 bonds at the early stage of polymerization.

Extended Data Fig. 13 Classical MD simulation of the structural evolution of a-D during isothermal annealing at 1600 K and 30 GPa, 6400 atoms, up to 15 ns. Only the core atoms of the paracrystals are shown. The paracrystals marked by the turquoise and gold colors have CD and HD-like structures, respectively. The fraction of paracrystals increases with the annealing time (a-o, at a time interval of 1 ns per image). The last image (p) shows a 3D configuration paracrystalline diamond with ϕ=20% obtained from brute-force classical MD. To further increase the volume fraction of p-D, we resorted to adiabatic-bias molecular dynamics and the final results are given in Figure 3 of the main text. See Supplementary Video for the fluctuations of the paracrystals.