Table 1 Descriptive characteristics of the Swarna watershed and its sub-watersheds.

| Watershed | Stream Order (u) | Number of Streams (Nu) | Stream Length (Km) (Lu) | Mean stream Length (km) (Lsm) | Stream Length Ratio (SLR) | Bifurcation Ratio (Rb) |
| --- | --- | --- | --- | --- | --- | --- |
| SRW | 1 | 1856 | 1039.09 | 0.56 |  | 4.24 |
|  | 2 | 438 | 363.15 | 0.83 | 0.35 | 4.13 |
|  | 3 | 106 | 171.98 | 1.62 | 0.47 | 5.05 |
|  | 4 | 21 | 112.68 | 5.37 | 0.66 | 3.5 |
|  | 5 | 6 | 77.57 | 12.93 | 0.69 | 6 |
|  | 6 | 1 | 60.81 | 60.81 | 0.78 |  |
|  | Total | 2428 | 1825.28 | 82.12 |  | 4.58 (mean) |
| SW1 | 1 | 260 | 115.11 | 0.44 |  | 4.33 |
|  | 2 | 60 | 36.65 | 0.61 | 0.32 | 4.62 |
|  | 3 | 13 | 20.8 | 1.6 | 0.57 | 4.33 |
|  | 4 | 3 | 9.75 | 3.25 | 0.47 | 3 |
|  | 5 | 1 | 8.26 | 8.26 | 0.85 |  |
|  | Total | 337 | 190.57 | 14.16 |  | 4.07 (mean) |
| SW2 | 1 | 61 | 27.81 | 0.46 |  | 4.69 |
|  | 2 | 13 | 6.31 | 0.49 | 0.23 | 3.25 |
|  | 3 | 4 | 2.23 | 0.56 | 0.35 | 2 |
|  | 4 | 2 | 8.72 | 4.36 | 3.91 | 2 |
|  | 5 | 1 | 2.35 | 2.35 | 0.27 |  |
|  | Total | 81 | 47.43 | 8.21 |  | 3 (mean) |
| SW3 | 1 | 114 | 63.08 | 0.55 |  | 4.22 |
|  | 2 | 27 | 30.73 | 1.14 | 0.49 | 3.38 |
|  | 3 | 8 | 8.84 | 1.11 | 0.29 | 2.67 |
|  | 4 | 3 | 5.62 | 1.87 | 0.64 | 3 |
|  | 5 | 1 | 5.76 | 5.76 | 1.02 |  |
|  | Total | 153 | 114.03 | 10.43 |  | 3.32 (mean) |
| SW4 | 1 | 157 | 78.12 | 0.5 |  | 3.65 |
|  | 2 | 43 | 29 | 0.67 | 0.37 | 5.38 |
|  | 3 | 8 | 14.83 | 1.85 | 0.51 | 4 |
|  | 4 | 2 | 7.87 | 3.93 | 0.53 | 2 |
|  | 5 | 1 | 7.26 | 7.26 | 0.92 |  |
|  | Total | 211 | 137.08 | 14.22 |  | 3.76 |
| SW5 | 1 | 172 | 109.32 | 0.64 |  | 4.53 |
|  | 2 | 38 | 33.17 | 0.87 | 0.3 | 3.8 |
|  | 3 | 10 | 11.77 | 1.18 | 0.35 | 3.33 |
|  | 4 | 3 | 11.35 | 3.78 | 0.96 | 3 |
|  | 5 | 1 | 15.79 | 15.79 | 1.39 |  |
|  | Total | 224 | 181.4 | 22.26 |  | 3.66 (mean) |
| SW6 | 1 | 376 | 221.07 | 0.59 |  | 4.48 |
|  | 2 | 84 | 89.72 | 1.07 | 0.41 | 5.25 |
|  | 3 | 16 | 27.11 | 1.69 | 0.3 | 5.33 |
|  | 4 | 3 | 30.92 | 10.31 | 1.14 | 3 |
|  | 5 | 1 | 36.92 | 36.92 | 1.19 |  |
|  | Total | 480 | 405.73 | 50.57 |  | 4.51 (mean) |

Note: SRW= Swarna River watershed, SW= Sub-watershed

Table 2 Morphometric characteristics of Swarna watershed and its sub-watersheds

| Sl No | Parameters | SRW | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Watershed Area (A) (km2) | 789.11 | 43.3 | 12.7 | 38 | 46.96 | 85.6 | 204.6 |
| 2 | Watershed Perimeter (P)(km) | 208 | 30 | 21 | 33 | 40 | 58 | 119 |
| 3 | Watershed Length (Lb)(km) | 60.43 | 7.78 | 5.49 | 8.9 | 11.27 | 18.19 | 37.59 |
| 4 | Watershed Relief (m) | 1208 | 1098 | 1110 | 892 | 1012 | 340 | 860 |
| 5 | Drainage density (Dd) (m/km2) | 2.31 | 4.4 | 3.71 | 3 | 2.92 | 2.12 | 1.98 |
| 6 | Stream frequency (Fs) (No./km2) | 3.08 | 7.78 | 6.33 | 4.03 | 4.49 | 2.61 | 2.35 |
| 7 | Form factor (Ff) | 0.22 | 0.72 | 0.42 | 0.48 | 0.37 | 0.26 | 0.14 |
| 8 | Circularity Ratio (Rc) | 0.23 | 0.60 | 0.36 | 0.44 | 0.37 | 0.32 | 0.18 |
| 9 | Elongation Ratio (Re) | 0.52 | 0.95 | 0.73 | 0.78 | 0.69 | 0.57 | 0.43 |
| 10 | Length of Overland flow (Lo) | 0.22 | 0.11 | 0.13 | 0.16 | 0.17 | 0.24 | 0.25 |
| 11 | Time of concentration (hr) (Tc) | 5.14 | 0.58 | 0.40 | 0.72 | 0.89 | 2.08 | 3.42 |
| 12 | Relief Ratio (Rh) | 0.02 | 0.141 | 0.202 | 0.100 | 0.090 | 0.019 | 0.023 |
| 13 | Ruggedness number (Rn) | 2.79 | 4.83 | 4.12 | 2.68 | 2.96 | 0.72 | 1.7 |

Table 3 Ranking and prioritization of morphometric parameters for sub-watersheds of Swarna watershed.

| Sl No | Parameters | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Watershed Area (A) (km2) | 3 | 1 | 2 | 4 | 5 | 6 |
| 2 | Watershed Perimeter (P)(km) | 2 | 1 | 3 | 4 | 5 | 6 |
| 3 | Watershed Length (Lb)(km) | 2 | 1 | 3 | 4 | 5 | 6 |
| 4 | Watershed Relief (m) | 2 | 1 | 4 | 3 | 6 | 5 |
| 5 | Drainage density (Dd) (m/km2) | 1 | 2 | 3 | 4 | 5 | 6 |
| 6 | Stream frequency (Fs ) (No./km2) | 1 | 2 | 4 | 3 | 6 | 5 |
| 7 | Form factor (Ff) | 1 | 3 | 2 | 4 | 5 | 6 |
| 8 | Circularity Ratio (Rc) | 1 | 4 | 2 | 3 | 5 | 6 |
| 9 | Elongation Ratio (Re) | 1 | 3 | 2 | 4 | 5 | 6 |
| 10 | Length of Overland flow (Lo) | 1 | 2 | 3 | 4 | 5 | 6 |
| 11 | Time of concentration (Tc) | 2 | 1 | 3 | 4 | 5 | 6 |
| 12 | Relief Ratio (Rh) | 2 | 1 | 3 | 4 | 6 | 5 |
| 13 | Ruggedness number (Rn) | 1 | 2 | 4 | 3 | 6 | 5 |
| Compound value |  | 1.54 | 1.85 | 2.92 | 3.69 | 5.31 | 5.69 |
| Priority |  | 1 | 2 | 3 | 4 | 5 | 6 |

Table 4 Classification of Land Use and Land Cover areas (km2) in Sub-watersheds within the Swarna watershed (Landsat 8-14/04/2022).

| Sl No | LULC classification | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Water Bodies | 0.63 | 0.89 | 0.02 | 0.01 | 0.19 | 1.77 |
| 2 | Evergreen Broadleaf forest | 11.57 | 4.33 | 2.89 | 0.86 | 1.21 | 14.60 |
| 3 | Deciduous Broadleaf Forest | 13.01 | 3.18 | 7.88 | 2.65 | 7.31 | 69.55 |
| 4 | Agricultural Land | 10.57 | 2.26 | 14.35 | 18.73 | 23.84 | 35.44 |
| 5 | Builtup Land | 0.73 | 0.18 | 0.46 | 0.19 | 4.11 | 30.55 |
| 6 | Barren Land | 2.11 | 0.87 | 3.41 | 6.23 | 14.09 | 24.01 |
| 7 | Mixed Forest | 2.94 | 0.65 | 5.41 | 13.51 | 17.71 | 8.90 |
| 8 | Grass Land | 1.38 | 0.34 | 2.94 | 4.19 | 11.67 | 12.15 |
| 9 | Shrub Land | 0.36 | 0.08 | 0.64 | 0.58 | 5.48 | 7.63 |
| Total Area (km2) |  | 43.30 | 12.7 | 38 | 46.96 | 85.6 | 204.6 |

Table 5 Prioritization of Land use land cover in each sub-watershed of the Swarna watershed

| Sl. No | Classification of Landuse Landcover | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Water Bodies | 0.03 | 0.15 | 0.00 | 0.00 | 0.00 | 0.02 |
| 2 | Evergreen Broadleaf forest | 5.34 | 6.77 | 1.53 | 0.36 | 0.28 | 1.43 |
| 3 | Deciduous Broadleaf Forest | 2.67 | 2.21 | 1.84 | 0.50 | 0.76 | 3.02 |
| 4 | Agricultural Land | 2.71 | 1.97 | 4.20 | 4.43 | 3.09 | 1.92 |
| 5 | Builtup Land | 0.08 | 0.06 | 0.05 | 0.02 | 0.21 | 0.66 |
| 6 | Barren Land | 0.32 | 0.45 | 0.60 | 0.89 | 1.10 | 0.78 |
| 7 | Mixed Forest | 0.91 | 0.68 | 1.90 | 3.84 | 2.76 | 0.58 |
| 8 | Grass Land | 0.50 | 0.41 | 1.20 | 1.39 | 2.12 | 0.92 |
| 9 | Shrub Land | 0.15 | 0.11 | 0.30 | 0.22 | 1.14 | 0.66 |
| Compound value |  | 12.71 | 12.83 | 11.62 | 11.64 | 11.47 | 10.01 |
| Priority |  | 2 | 1 | 4 | 3 | 5 | 6 |

Table 6 Flash flood risk levels across sub-watersheds within the Swarna watershed

| Sl. No | Parameters | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Morphometric Parameters | 1 | 2 | 3 | 4 | 5 | 6 |
| 2 | Bifurcation ratio v/s Drainage density | 1 | 2 | 4 | 3 | 5 | 6 |
| 3 | Bifurcation ratio v/s Stream frequency | 3 | 1 | 2 | 4 | 5 | 6 |
| 4 | Land use/Land cover analysis | 2 | 1 | 4 | 3 | 5 | 6 |
| Compound value |  | 1.75 | 1.5 | 3.25 | 3.5 | 5 | 6 |
| Priority |  | 2 | 1 | 3 | 4 | 5 | 6 |
| Flood risk levels |  | High |  | Moderate |  | Low |  |