Surface dispersal, emission source and human health risk assessment of heavy metal(loid)s in an active gas field, Southern Iran

Nafise Khodabakhshloo*, Bhabananda Biswas2, 3, Farid Moore1, Jianhua Du3, 4, Ayanka Wijayawardena3,4, Morteza Arabzadeh5,

- Department of Earth Sciences, College of Science, Shiraz University, 71454 Shiraz, Iran

- Future Industries Institute, STEM unit of University of South Australia, Mawson Lakes SA 5095, Australia.

- Cooperative Research Centre for Contamination Assessment and Remediation of the Environment (CRC CARE), ATC building, The University of Newcastle, Callaghan, NSW 2308, Australia

- Global Centre for Environmental Remediation (GCER), The University of Newcastle, Australia

- Department of health, safety and environment, Fajr e jam gas refinery, Bushehr, Iran

# Supplementary information

Supplementary Table 1. Applied indices of heavy metal pollution in the current study

| Pollution index | Formula | Classification | Description of variables |
| --- | --- | --- | --- |
| Geoaccumulation (Igeo) | Igeo= log2 Ci1.5Bi | non-pollution (Igeo < 0), no pollution (0<Igeo < 1), moderate pollution (1<Igeo <2), moderate to severe pollution (2<Igeo < 3), severe pollution (3<Igeo < 4) severe to extreme pollution (4<Igeo < 5), and extreme pollution (Igeo > 5) | Ci :measured concentration of (Mohammadi et al.) of i (mg/kg) Bi: geochemical background concentration of (Mohammadi et al.) of i (mg/kg) Constant (1.5): used to normlize possible lithogenic variations in background matrix (G. MÜLLER, 1969; Zhuo et al., 2020) |
| Contamination factor (Cf) | Cfi= CiCn | (Cf <8) low contamination; (8 ≤ Cf <16) moderate contamination; (16 ≤ Cf≥32) considerable contamination; (32 ≥ Cf) very high contamination | Ci the concentration of the corresponding element in sample of interest Cnis the concentration of heavy metal i in reference soil (Hakanson, 1980; Hu and Cheng, 2016; Keshav Krishna and Rama Mohan, 2016) |
| Pollution Load Index (PLI) | PLI=nCF1×CF2×….×CFn | (PLI = 0), unpolluted (0 < PLI ≤ 1), unpolluted to moderately polluted (1 < PLI ≤ 2), moderately polluted (2 <PLI ≤ 3), moderately to highly polluted (3 < PLI ≤ 4), highly polluted (4 < PLI ≤ 5), or very highly polluted (PLI > 5). | PLI: overall pollution status for a sample Cf: Contamination factor (Kowalska et al., 2018; Kumar et al., 2019b) |
| Ecological risk factor | Eri= Tri×Cf | Eri < 40, low risk; 40 ≤ Eri < 80, moderate risk; 80 ≤ Eri < 160, considerable risk; 160 ≤ Eri < 320, high risk; Eri ≥ 320, very high risk | Eri: potential ecological risk factor of single metal Tri is the biological toxic factor of metals i; Tri is the toxic response factor (Cd: 30, As: 10, Pb: 5, Cu: 5, Zn:1, Cr:2). Cf: Contamination factor (Adimalla and Wang, 2018; Hakanson, 1980) |
| Potential ecological risks (PERI) | RI= i=1nEri | PERI < 150 Low risk, 150 < PERI < 300 Moderate risk, 300 < PERI < 600, Considerable risk, PERI > 600 High risk | PERI is the Eri of eight metals (Kowalska et al., 2018) |

Supplementary Table 2. Definition and parameters for health risk assessment of heavy metal in studied soils with RFD and SF

| Factor | Definition | unit | Value |  | Reference |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Adult | Child |  |  |  |  |
|  |  |  | (>21 y) | (5-11 y) |  |  |  |  |
| AT | Average time: | d |  |  | U.S. EPA, 2011 |  |  |  |
|  | for carcinogens |  | 365×70 | 365×70 |  |  |  |  |
|  | for non-carcinogens |  | 365×ED | 365×ED |  |  |  |  |
| AF | Soil-adherence factor | mg (cm2·d)-1 | 0.07 | 0.2 | U.S. EPA, 2004 |  |  |  |
| BW | Average body weight | kg | 80 | 15 | U.S. EPA, 2011 |  |  |  |
| CF | Conversion factor | kg·mg-1 | 10-6 | 10-6 | U.S. EPA, 2004 |  |  |  |
| ABSd | Dermal absorption factor |  | 0.001 | 0.001 | U.S. EPA, 2011 |  |  |  |
| FE | Dermal exposure ratio |  | 0.61 | 0.61 | U.S. EPA, 2004 |  |  |  |
| EF | Exposure frequency | d·y-1 | 350 | 350 | U.S. EPA, 2004 |  |  |  |
| ED | Exposure duration | y | 26 | 6 | U.S. EPA, 2011 |  |  |  |
| SA | Exposed skin surface area | cm2 | 6032 | 2373 | U.S. EPA, 2011 |  |  |  |
| PEF | particulate emission factor | M3·kg | 1.36×109 | 1.36×109 | U.S. EPA, 2004 |  |  |  |
| Cs | Studied soils concentration | mg·kg-1 |  |  | This study |  |  |  |
| InhR | Inhalation rate | m3·d-1 | 20 | 12 | U.S. EPA, 2011 |  |  |  |
| IngR | Ingestion rate | mg·d-1 | 100 | 200 | U.S. EPA, 2011 |  |  |  |
| Factor | Pb | Cd | As | Cr | Ni | Co | Cu | Zn |
| RfD ing. | 3.50E-03 | 1.00E-03 | 3.00E-04 | 3.00E-03 | 2.00E-02 | 2.00E-02 | 3.70E-02 | 3.00E-01 |
| RfD inh. | 5.25E-03 | 1.70E-05 | 1.23E-04 | 2.86E-05 | 2.06E-02 | 5.71E-06 | 4.02E-02 | 3.00E-01 |
| RfD derm | 5.25E-04 | 1.00E-05 | 1.23E-04 | 6.00E-05 | 5.40E-03 | 1.60E-02 | 1.20E-02 | 6.00E-02 |
| CSF ing. | 1.50E+00 | 6.10E+00 | 1.50E+00 | 5.00E-01 | 9.10E-01 |  |  |  |
| CSF inh. | 1.50E+01 | 6.30E+00 | 1.51E+01 | 4.20E+01 | 8.40E-01 | 9.80E+00 |  |  |
| CSF derm | 1.50E+00 | 6.10E+00 | 3.66E+00 |  |  |  |  |  |

Supplementary Fig 1. Soil texture of studied area (classified according USDA)

* Corresponding author

E-mail addresses: Nafise_kh@shirazu.ac.ir