Supplementary:

Determination of STV Dimension in SCART

In order to facilitate to determination of the right size of STV, the dimensions of STV and its GTV relationship was extensive explored doing many tests on hypothetical spherical tumors.

1)  Method of finding the optimal dimension of STV

We used 15Gy x 3 as the SCART prescription dose to STV and 3Gy x3 as the tissue dose constraints on a hypothetical 10cm spherical tumor. The diameter of the circular STV in axial view was incrementally increased by 1mm starting from 1cm. The new STV with each increment was re-planned as the target in VMAT treatment planning, where the maximum MLC opening was limited to the new STV. The largest STV that maintained sufficient coverage of the prescription dose without violating tissue dose constraints was identified as the optimal STV dimension.  In this case, it was determined that the diameter of the optimal STV is around 2.1 cm in axial view, 21% of the 10cm sphere tumor.

2) STV/GTV independent with dimension of GTV

We conducted the same tests on 9 hypothetical spherical tumors ranging from 4cm to 20cm in diameter. The results revealed that the dimension of STV was similarly proportional to GTV (around 20~22%, as listed in Table 2) in these tumors.  The proportion of STV/GTV was pretty much constant and not really related to the size of GTV.

| Tumor Diameter (cm) | Tumor Vol (cc) | STV Diameter (cm) | STV vol (cc) | STVs DimensionGTV's Dimension | STV Vol % |
| --- | --- | --- | --- | --- | --- |
| 4 | 32.8 | 0.9 | 0.9 | 22.5% | 2.7% |
| 6 | 112.1 | 1.2 | 2.8 | 20.0% | 2.5% |
| 8 | 267 | 1.8 | 8.3 | 22.5% | 3.1% |
| 10 | 522.6 | 2.1 | 15 | 21.0% | 2.9% |
| 12 | 904 | 2.5 | 22.5 | 20.8% | 2.5% |
| 14 | 1435 | 2.9 | 28.3 | 20.7% | 2.0% |
| 16 | 2143 | 3.4 | 47.5 | 21.3% | 2.2% |
| 18 | 3053 | 3.9 | 81.8 | 21.7% | 2.7% |
| 20 | 4189 | 4.2 | 127.5 | 21.0% | 3.0% |

Table 2.  The appropriate STV dimension and volume were listed in the nine hypothetical spherical tumors ranging from 4cm to 20cm in diameter. It is found that, regardless the tumor’s dimension, there is a consistent proportionality between the dimension of the STV and GTV.

3) STV/GTV depends on the Tissue constrain dose / SCART prescription dose

We explored the interplay between Stereotactic Treatment Volume (STV) and Gross Tumor Volume (GTV), specifically focusing on the relationship with Tissue Dose Constraints and SCART Prescription Dose.

We conducted tests using a 10cm spherical tumor with four distinct prescription doses ranging from 15Gy~24Gyx1~3 and two tissue dose constraints (3~5Gyx1~3). We observed a clear correlation listed in Table 3 in supplementary material. The proportionality between STV and GTV appears to be significantly influenced by the prescription dose and tissue dose constraints.  It is also found that STV/GTV proportionality is approximately same as Tissue Constraint Dose/SCART Prescription Dose proportionality.

As a rule of thumb for VMAT-based SCART planning, in axial view, we used Eq. (1):

STV dimension = GTV dimension * Tissue Dose Constraints / Prescription Dose (1)

| Escalation level | Prescription Dose | Tissue Dose Constrains | Tissue ConstrainPrescription Dose | STV's DimensionGTV's Dimension | % of GTV Vol |
| --- | --- | --- | --- | --- | --- |
| -1 | 15 Gy x1 | 3 Gy x 1 | 20% | 21% | 3% |
| -1 | 15 Gy x1 | 5 Gy x 1 | 33% | 36% | 10.6% |
| 1 | 15 Gy x3 | 5 Gy x 3 | 33% | 36% | 10.6% |
| 2 | 18 Gy x3 | 5 Gy x 3 | 28% | 27% | 6.3% |
| 3 | 21 Gy x3 | 5 Gy x 3 | 24% | 24% | 4.5% |
| 4 | 24 Gy x3 | 5 Gy x 3 | 21% | 21% | 3.3% |

Table 3.:  The list of STV and GTV proportion with the escalated prescription doses (15Gy ~ 24Gy) and two tissue dose constrains (3Gy and 5Gy) . It is found that STV/GTV dimension is similar to the Tissue Constrain Dose/SCART Prescription Dose.