Table 1. Accuracy estimates of INAGEOID Version 2.0 (unit: cm) (BIG, 2022).

| Island | Standard deviation |
| --- | --- |
| Java | 11.81 |
| Bali | 21.67 |
| Sumatera | 28.58 |
| Sulawesi | 25.10 |
| Kalimantan Barat | 6.43 |
| Kalimantan Timur | 5.88 |

Table 2. Difference statistics between the EGM08-derived gravity anomalies of degree 1800 and the gravity effects computed by the 2D-FFT technique and the corresponding equivalent topography (unit: mgal).

| FFT | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- |
| 30 | 1.00 | 18.27 | -19.19 | 2.36 | 2.56 |
| 40 | 1.16 | 20.58 | -17.34 | 2.45 | 2.71 |
| 50 | 1.28 | 22.68 | -15.76 | 2.53 | 2.84 |

Table 3. Difference statistics between the airborne gravity anomalies and GGM-derived gravity anomalies at different degrees (unit: mgal).

| GGM | Degree | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- | --- |
| EGM08 | 360 | -1.91 | 145.35 | -117.57 | 30.47 | 30.53 |
|  | 720 | -2.64 | 98.70 | -101.58 | 20.72 | 20.89 |
|  | 1080 | -2.77 | 90.72 | -97.08 | 18.61 | 18.82 |
|  | 1440 | -2.87 | 90.63 | -92.65 | 18.30 | 18.53 |
|  | 1800 | -2.84 | 90.30 | -93.68 | 18.22 | 18.44 |
|  | 2160 | -2.79 | 91.87 | -94.59 | 18.26 | 18.47 |
| EIGEN -6C4 | 360 | -2.79 | 91.87 | -94.59 | 18.26 | 18.47 |
|  | 720 | -1.62 | 143.30 | -116.83 | 29.98 | 30.02 |
|  | 1080 | -2.34 | 100.03 | -100.31 | 20.04 | 20.17 |
|  | 1440 | -2.47 | 92.23 | -95.90 | 17.87 | 18.04 |
|  | 1800 | -2.57 | 89.85 | -91.09 | 17.56 | 17.74 |
|  | 2160 | -2.54 | 89.61 | -92.02 | 17.46 | 17.65 |
| GECO | 360 | -1.67 | 141.23 | -113.67 | 29.63 | 29.67 |
|  | 720 | -2.39 | 95.62 | -97.67 | 19.64 | 19.78 |
|  | 1080 | -2.53 | 87.64 | -93.15 | 17.37 | 17.55 |
|  | 1440 | -2.63 | 83.86 | -88.72 | 17.03 | 17.23 |
|  | 1800 | -2.59 | 84.77 | -89.75 | 16.94 | 17.13 |
|  | 2160 | -2.55 | 86.26 | -90.66 | 16.99 | 17.18 |
| XGM 2019e | 360 | -1.64 | 129.65 | -117.58 | 28.38 | 28.43 |
|  | 720 | -2.40 | 92.43 | -100.77 | 16.40 | 16.58 |
|  | 1080 | -2.51 | 89.86 | -97.05 | 13.25 | 13.49 |
|  | 1440 | -2.59 | 86.67 | -93.70 | 12.68 | 12.94 |
|  | 1800 | -2.55 | 87.75 | -94.70 | 12.66 | 12.92 |
|  | 2160 | -2.46 | 90.39 | -95.46 | 12.74 | 12.98 |
| SGG- UGM-2 | 360 | -1.79 | 144.73 | -115.39 | 29.75 | 29.80 |
|  | 720 | -2.62 | 99.07 | -99.88 | 19.22 | 19.39 |
|  | 1080 | -2.75 | 95.79 | -96.67 | 16.85 | 17.07 |
|  | 1440 | -2.85 | 88.38 | -92.57 | 16.46 | 16.71 |
|  | 1800 | -2.82 | 90.54 | -94.64 | 16.34 | 16.58 |
|  | 2160 | -2.77 | 92.28 | -96.49 | 16.36 | 16.59 |

Table 4. Difference statistics between the geometric and modeled geoid heights for the three GPS/leveling datasets (D1, D2 and D3).

| Model | Dataset | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- | --- |
| EGM08 (1440) | D1 | 0.773 | 1.030 | 0.533 | 0.186 | 0.793 |
|  | D2 | 0.460 | 1.312 | -0.720 | 0.420 | 0.619 |
|  | D3 | 0.429 | 0.541 | 0.346 | 0.055 | 0.433 |
| EGM08 (1440+a) | D1 | 0.587 | 0.678 | 0.507 | 0.067 | 0.590 |
|  | D2 | 0.479 | 1.171 | 0.167 | 0.229 | 0.530 |
|  | D3 | 0.333 | 0.392 | 0.253 | 0.043 | 0.336 |
| EGM08 (1440+a+t) | D1 | 0.562 | 0.637 | 0.477 | 0.061 | 0.565 |
|  | D2 | 0.430 | 1.091 | 0.164 | 0.222 | 0.482 |
|  | D3 | 0.312 | 0.382 | 0.240 | 0.040 | 0.315 |
| EGM08 (1800) | D1 | 0.775 | 1.017 | 0.565 | 0.175 | 0.792 |
|  | D2 | 0.464 | 1.332 | -0.664 | 0.413 | 0.618 |
|  | D3 | 0.438 | 0.552 | 0.353 | 0.056 | 0.441 |
| EGM08 (1800+a) | D1 | 0.586 | 0.671 | 0.519 | 0.059 | 0.589 |
|  | D2 | 0.487 | 1.165 | 0.185 | 0.231 | 0.538 |
|  | D3 | 0.339 | 0.404 | 0.259 | 0.041 | 0.341 |
| EGM08 (1800+a+t) | D1 | 0.562 | 0.630 | 0.503 | 0.052 | 0.564 |
|  | D2 | 0.438 | 1.085 | 0.181 | 0.225 | 0.491 |
|  | D3 | 0.318 | 0.360 | 0.246 | 0.037 | 0.320 |
| EGM08 (2160) | D1 | 0.766 | 1.016 | 0.553 | 0.177 | 0.784 |
|  | D2 | 0.466 | 1.354 | -0.649 | 0.416 | 0.621 |
|  | D3 | 0.440 | 0.552 | 0.338 | 0.062 | 0.444 |
| EGM08 (2160+a) | D1 | 0.564 | 0.649 | 0.477 | 0.062 | 0.567 |
|  | D2 | 0.483 | 1.156 | 0.157 | 0.242 | 0.539 |
|  | D3 | 0.342 | 0.405 | 0.252 | 0.044 | 0.345 |
| EGM08 (2160+a+t) | D1 | 0.540 | 0.626 | 0.468 | 0.058 | 0.543 |
|  | D2 | 0.435 | 1.076 | 0.154 | 0.238 | 0.495 |
|  | D3 | 0.321 | 0.368 | 0.238 | 0.041 | 0.323 |

Table 5. Difference statistics of the 9 points with the same position in the first and second GPS/leveling datasets (unit: m).

| Dataset 1 |  | Dataset 2 |  | Modeled geoid heights | Geometric geoid height difference between Datasets 1 and 2 | Difference between geometric and modeled geoid heights for Dataset 1 | Difference between geometric and modeled geoid heights for Dataset 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Point name | Geometric geoid height | Point name | Geometric geoid height |  |  |  |  |
| G21 | 57.423 | G722 | 57.068 | 56.794 | 0.355 | 0.629 | 0.274 |
| G30 | 56.893 | G723 | 56.563 | 56.263 | 0.330 | 0.630 | 0.300 |
| G46 | 56.844 | G724 | 56.570 | 56.246 | 0.274 | 0.598 | 0.324 |
| G41 | 57.288 | G725 | 57.017 | 56.700 | 0.271 | 0.588 | 0.317 |
| G22 | 56.980 | G727 | 56.708 | 56.454 | 0.272 | 0.526 | 0.254 |
| G33 | 56.987 | G728 | 56.719 | 56.478 | 0.268 | 0.509 | 0.241 |
| G47 | 57.051 | G730 | 56.896 | 56.548 | 0.155 | 0.503 | 0.348 |
| G26 | 56.800 | G731 | 56.420 | 56.239 | 0.380 | 0.561 | 0.181 |
| G28 | 56.950 | G732 | 56.687 | 56.440 | 0.263 | 0.510 | 0.247 |
|  |  |  |  | Mean | 0.285 | 0.561 | 0.276 |
|  |  |  |  | Standard deviation | 0.065 | 0.052 | 0.051 |

Table 6. Difference statistics between the geometric and modeled geoid heights for the five routes of the second GPS/leveling dataset (unit: m).

| Geoid model | Routes | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- | --- |
| EGM08 (1440+a+t) | R1 | 0.272 | 0.368 | 0.164 | 0.063 | 0.279 |
|  | R2 | 0.277 | 0.309 | 0.225 | 0.030 | 0.279 |
|  | R3 | 0.390 | 0.473 | 0.299 | 0.049 | 0.393 |
|  | R4 | 0.672 | 0.753 | 0.569 | 0.058 | 0.674 |
|  | R5 | 1.051 | 1.091 | 1.011 | 0.057 | 1.052 |
| EGM08 (1800+a+t) | R1 | 0.276 | 0.375 | 0.181 | 0.054 | 0.281 |
|  | R2 | 0.268 | 0.331 | 0.198 | 0.047 | 0.271 |
|  | R3 | 0.406 | 0.456 | 0.374 | 0.025 | 0.406 |
|  | R4 | 0.690 | 0.738 | 0.633 | 0.033 | 0.691 |
|  | R5 | 1.064 | 1.085 | 1.043 | 0.030 | 1.064 |
| EGM08 (2160+a+t) | R1 | 0.256 | 0.371 | 0.154 | 0.056 | 0.262 |
|  | R2 | 0.285 | 0.397 | 0.202 | 0.067 | 0.292 |
|  | R3 | 0.397 | 0.480 | 0.399 | 0.047 | 0.400 |
|  | R4 | 0.703 | 0.751 | 0.584 | 0.047 | 0.704 |
|  | R5 | 1.067 | 1.076 | 1.057 | 0.013 | 1.067 |

Table 7. MSL geopotential estimation using the points of the first GPS\leveling dataset and the geoid model of EGM08(1800+a+t).

| Model | Point name | Geometric geoid height (m) | Gravimetric geoid height (m) | Geopotential of mean sea levelWG (m2s-2) |
| --- | --- | --- | --- | --- |
| EGM08 (1800+a+t) | G21 | 57.423 | 56.794 | 62636853.848 |
|  | G30 | 56.893 | 56.263 | 62636853.840 |
|  | G46 | 56.844 | 56.246 | 62636854.147 |
|  | G41 | 57.288 | 56.700 | 62636854.251 |
|  | G22 | 56.980 | 56.454 | 62636854.857 |
|  | G33 | 56.987 | 56.478 | 62636855.017 |
|  | G47 | 57.051 | 56.548 | 62636855.076 |
|  | G26 | 56.800 | 56.239 | 62636854.510 |
|  | G28 | 56.950 | 56.440 | 62636855.013 |
|  |  | Mean |  | 62636854.507 |
|  |  | Standard deviation |  | 0.505 |

Table 8. MSL geopotential estimation for the three GPS\leveling datasets (D1, D2 and D3).

| Dataset (Route) |  | Number of Point | Geopotential of mean sea levelWG (m2s-2) | Correction to geometric geoid height WG-U0γ0 (m) |
| --- | --- | --- | --- | --- |
| D1 |  | 9 | 62636854.507±0.505 | -0.562±0.052 |
| D2 | R1 | 17 | 62636857.299±0.528 | -0.276±0.054 |
|  | R2 | 6 | 62636857.380±0.460 | -0.268±0.047 |
|  | R3 | 8 | 62636856.035±0.244 | -0.405±0.025 |
|  | R4 | 11 | 62636853.252±0.320 | -0.690±0.033 |
|  | R5 | 2 | 62636849.594±0.296 | -1.064±0.030 |
| D3 |  | 11 | 62636856.890±0.360 | -0.318±0.037 |

Table 9. Difference statistics between the corrected geometric and modeled geoid heights (unit: m).

| Geoid Model | Dataset (route) |  | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EGM08 (1800+a+t) | D1 |  | 0.000 | 0.068 | -0.059 | 0.052 | 0.049 |
|  | D2 | R1 | 0.000 | 0.099 | -0.095 | 0.054 | 0.052 |
|  |  | R2 | 0.000 | 0.063 | -0.070 | 0.047 | 0.043 |
|  |  | R3 | 0.001 | 0.051 | -0.031 | 0.025 | 0.023 |
|  |  | R4 | 0.000 | 0.048 | -0.057 | 0.033 | 0.031 |
|  |  | R5 | 0.000 | 0.021 | -0.021 | 0.030 | 0.021 |
|  | D3 |  | 0.000 | 0.042 | -0.072 | 0.037 | 0.035 |
|  | All |  | 0.000 | 0.099 | -0.095 | 0.042 | 0.041 |

Table 10. Difference statistics between the corrected geometric and modeled geoid heights (unit: m).

| Geoid model | GPS/ leveling |  | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EIGEN-6C4 (1800+a+t) | D1 |  | 0.847 | 0.948 | 0.753 | 0.062 | 0.849 |
|  | D2 | R1 | 0.880 | 1.065 | 0.745 | 0.087 | 0.884 |
|  |  | R2 | 1.000 | 1.095 | 0.921 | 0.059 | 1.001 |
|  |  | R3 | 0.915 | 1.048 | 0.749 | 0.106 | 0.920 |
|  |  | R4 | 0.949 | 1.082 | 0.835 | 0.065 | 0.964 |
|  |  | R5 | 0.954 | 0.959 | 0.950 | 0.006 | 0.955 |
|  | D3 |  | 0.984 | 1.044 | 0.938 | 0.039 | 0.983 |
|  | All |  | 0.923 | 1.095 | 0.745 | 0.086 | 0.927 |
| GECO (1800+a+t) | D1 |  | 0.847 | 0.945 | 0.768 | 0.058 | 0.849 |
|  | D2 | R1 | 0.882 | 1.088 | 0.739 | 0.088 | 0.886 |
|  |  | R2 | 0.974 | 1.014 | 0.902 | 0.044 | 0.975 |
|  |  | R3 | 0.906 | 1.022 | 0.739 | 0.107 | 0.911 |
|  |  | R4 | 0.955 | 1.069 | 0.881 | 0.053 | 0.957 |
|  |  | R5 | 0.978 | 0.985 | 0.971 | 0.010 | 0.978 |
|  | D3 |  | 0.992 | 1.049 | 0.944 | 0.034 | 0.993 |
|  | All |  | 0.923 | 1.088 | 0.739 | 0.084 | 0.927 |
| XGM2019e (1800+a+t) | D1 |  | 0.864 | 0.961 | 0.756 | 0.058 | 0.866 |
|  | D2 | R1 | 0.881 | 1.026 | 0.734 | 0.088 | 0.885 |
|  |  | R2 | 0.932 | 0.996 | 0.856 | 0.058 | 0.933 |
|  |  | R3 | 0.932 | 1.029 | 0.723 | 0.106 | 0.937 |
|  |  | R4 | 0.940 | 1.043 | 0.769 | 0.090 | 0.944 |
|  |  | R5 | 0.992 | 1.007 | 0.977 | 0.021 | 0.992 |
|  | D3 |  | 1.002 | 1.094 | 0.890 | 0.056 | 1.003 |
|  | All |  | 0.924 | 1.094 | 0.723 | 0.089 | 0.928 |
| SGG-UGM-2 (1800+a+t) | D1 |  | 0.860 | 0.931 | 0.808 | 0.046 | 0.861 |
|  | D2 | R1 | 0.889 | 1.051 | 0.775 | 0.072 | 0.892 |
|  |  | R2 | 1.018 | 1.099 | 0.942 | 0.055 | 1.020 |
|  |  | R3 | 0.924 | 1.029 | 0.783 | 0.079 | 0.927 |
|  |  | R4 | 0.965 | 1.149 | 0.901 | 0.069 | 0.967 |
|  |  | R5 | 0.870 | 0.897 | 0.843 | 0.038 | 0.877 |
|  | D3 |  | 0.999 | 1.057 | 0.939 | 0.036 | 0.999 |
|  | All |  | 0.933 | 1.149 | 0.775 | 0.081 | 0.936 |

Table 11. Difference statistics between the GGM-derived gravity anomalies of degree 1800 (unit: mgal).

| vs. EGM08 | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- |
| EIGEN-6C4 | 0.17 | 27.01 | -21.96 | 4.99 | 5.00 |
| GECO | 0.15 | 20.37 | -23.10 | 4.18 | 4.18 |
| XGM2019e | 0.14 | 98.45 | -85.08 | 9.08 | 9.09 |
| SGG-UGM-2 | 0.20 | 66.54 | -74.49 | 6.24 | 6.24 |

Table 12. Difference statistics between the GGM-derived geoid heights of degree 1800 (unit: m).

| vs. EGM08 | Mean | Maximum | Minimum | Standard deviation | RMS |
| --- | --- | --- | --- | --- | --- |
| EIGEN-6C4 | -0.935 | -0.115 | -1.536 | 0.176 | 0.952 |
| GECO | -0.936 | -0.257 | -1.805 | 0.163 | 0.951 |
| XGM2019e | -0.935 | 0.546 | -2.466 | 0.204 | 0.957 |
| SGG-UGM-2 | -0.940 | 0.173 | -1.894 | 0.184 | 0.958 |

Table 13. Datasets used in the development of the GGMs (A: altimetry, S: satellite, G: ground data, and T: topography), obtained from the ICGEM (modified from ICGEM).

| GGM | Year | Degree | Data |
| --- | --- | --- | --- |
| EGM08 | 2008 | 2190 | A, G, S(Grace) |
| EIGEN-6C4 | 2014 | 2190 | A, G, S(Goce), S(Grace), S(Lageos) |
| GECO | 2015 | 2190 | EGM2008, S(Goce) |
| XGM2019e | 2019 | 5540 | A, G, S(GOCO06s), T |
| SGG-UGM-2 | 2020 | 2190 | A, EGM2008, S(Goce), S(Grace) |