Supporting Information for

Contribution of nitrogen sources to streams in mixed-use watershed varies seasonally in a temperate region

Zuhong Lina, Junchi Liub, Yong Xiaoa,*, Chaojie Yua, Jinlan Zhanga, Tingting Zhanga,#

a Department of Environmental Science and Engineering, Research Centre for Resource and Environment, Beijing University of Chemical Technology, Beijing 100029, People’s Republic of China

b Key Laboratory of Vertebrate Evolution and Human Origins, Institute of Vertebrate Paleontology and Paleoanthropology, Chinese Academy of Sciences, Beijing 100044, People’s Republic of China

* Corresponding authors.

E-mail addresses: * xiaoyong@mail.buct.edu.cn (Xiao Y);

# zhangtt@mail.buct.edu.cn (Zhang T).

The geographic information of the study area

The total length of the river in Beiyun riverine watershed (39°46′-40°23′N, 116°05′-116°56′E) is about 89.4 km with a drainage area of 6166 km2, which covers part of Beijing, Tianjin, and Hebei province. The Beiyun River originates from the Yan Mountain. The Beiyun River has eight tributaries, including Lin Gou River, Qing River, Liangshui River, and other rivers. The water flow of the watershed is about 913 million m3 year-1, of which about 551 million m3 year-1 originates from domestic sewage and industrial wastewater (Liu et al., 2019). T The watershed is located at a temperate continental monsoon climate zone with hot, rainy summers and cold, dry winters. The annual average temperature is 13°C, and the average yearly rainfall is 643 mm. Due to the variation in precipitation, the seasons in this watershed can be classified as a wet season (April-October) and a dry season (November-March). Most of the annual rainfall occurs from June to September.

The detail of meta-analysis

The calculation of weighted response ratios: All literature search for meta-analysis was conducted using the database in Web of Science. The following keywords were employed: isotope, nitrate, source, and river. The search included all available publication until January 15, 2020, but was restricted to accepted, peer-reviewed publications in English, with original data. The retrieval flow chart is shown in Fig. S1. Exactly 25 river watersheds globally, which have a similar land-use environment, or climatic conditions, were accepted to explore the influence of the different sources about the δ15N-NO3 in this research. Detailed information on each watershed is shown in Table S3. We set the dry season as the experimental group and wet season was the control group since the Beiyun riverine watershed lacked rainfall throughout the year. Detailed criteria for selecting the appropriate observations to construct our database are as follows:

(1) The study must at least have used dual nitrate isotopes to trace the source of surface rivers.

(2) All of the studies must have been conducted for at least two months and grouped into the evident wet season and dry season.

(3) The study watersheds accorded at least one of the two conditions: Climate is the temperate continental monsoon climate (CMC) and land-use type, which is a complex urban and rural environment (CURE).

The procedures of meta-analysis followed those stipulated by Hedges et al. (1999) and Li et al. (2019). The natural logarithm transformation of response ratio (RR) was employed as a metric of the effect size in this study, and the formula as follows:

ln(RR)=ln(XtXc)                                                                                                     (1)

Where —Xt and —Xc denoted the mean value of the δ15N-NO3 under the dry season and wet season, respectively. A value of zero for ln (RR) showed that the CURE or CMC would not change a specific source of nitrate; the positive value indicated that a ratio of nitrate from a particular source would increase in the condition of CURE or CMC, and the negative value, on the contrary. Since the weighing function of different studies should be treated as an emotional level, the weight of ln (RR) (w) for each observation was calculated using the following formula: (Wu et al., 2011):

Wn=NcNtNc+Nt                                                                                                       (2)

Where Nt and Nc denoted the sample size of the δ15N-NO3 at each end source during the dry season and wet season, respectively. The linear bootstrap model was used to calculate the 95% confidence interval (CI). If the 95% CI of a variable overlapped with zero, this meant that the change of nitrate in a specific source was not significant in the condition CURE or CMC, or else there was a significant change.

Publication bias: To test for has publication bias in the included literature reports, the contour-enhanced funnel chart of four sources was used to evaluate weighted response ratios against standard errors by the Egger test (Egger et al., 1997). Notably, if the funnel chart showed a good symmetry, this proved that there is no publication bias in the study. The publication bias was analyzed using package metafor in R.

Table S1 List of sampling sites in Beiyun river watersheds.

| Grades | River name | Sampling sites |
| --- | --- | --- |
| Main stream | Wenyu river | M1, M2 |
|  | Beiyun river | M3, M4, M5, M6, M7, M8 |
| Tributary | Nansha river | T1 |
|  | Qing river | T2, T3 |
|  | Xiaozhong river | T4 |
|  | Nanchang river | T5, T7 |
|  | Tucheng river | T6 |
|  | Ba river | T8 |
|  | Lingou river | T9 |
|  | Nanhucheng river | T10 |
|  | Liangshui river | T11, T12 |
|  | Kunyu river | T13 |
|  | Ganggou river | T14 |
|  | Fenggang river | T15 |
|  | Feng river | T16, T17 |
|  | Yongdingxin river | T18 |

Table S2 Specific δ15N-NO3 and δ18O-NO3 values of potential nitrate sources

| Sources | n | δ15N-NO3 |  | δ18O-NO3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Mean | SD | Mean | SD |
| Precipitationa | 31 | 4.23 | 4.34 | 54 | 13.2 |
| Fertilizerb | 7 | -0.11 | 0.68 | 2.2 | 2.34 |
| Soil nitrogenc | 10 | 1.79 | 1.56 | 1.7 | 0.5 |
| Manure&Sewaged | 16 | 14.3 | 1.9 | 6.7 | 2.5 |

a. Data obtained from Zhang et al. (2018) and Ding et al. (2014).

b. Data obtained from Liu et al. (2018), Ding et al. (2014) and Lu et al. (2015).

c. Data obtained from Liu et al. (2019).

d. Data obtained from Xian et al. (2016).

Table S3 Details of the sampling sites in the referenced literatures.

| ID | Watershed | Climate type | Land-use type | Reference | AD.ck | AD.cn | AD.tr | AD.tn | NF.ck | NF.cn | NF.tr | NF.tn | SON.ck | SON.cn | SON.tr | SON.tn | M&S.ck | M&S.cn | M&S.tr | M&S.tn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ‰ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Han river watershed, Korea | Monsoon climate of medium latitude | Construction land, agricultural land | Lee et al. (2008) | NA | NA | NA | NA | NA | NA | NA | NA | 4.84 | 3 | NA | NA | 5.38 | 3 | 8.17 | 18 |
| 2 | Illinois river watershed, USA | Monsoon climate of medium latitude | Construction land, agricultural land, mining lease | Panno et al. (2008) | NA | NA | NA | NA | 3.9 | 1 | 4.35 | 2 | 5.03 | 3 | 5.13 | 4 | 9.35 | 11 | 8.45 | 11 |
| 3 | Taichung River watershed in Taiwan, China | Subtropical monsoon climate | Construction land, agricultural land | Peng et al. (2012) | NA | NA | NA | NA | 3.2 | 10 | 3.8 | 11 | 4.21 | 10 | 4.68 | 12 | 6.36 | 11 | 5.96 | 9 |
| 4 | Six rivers in Korea | Monsoon climate of medium latitude | Construction land, agricultural land | Shin e al. (2013) | NA | NA | NA | NA | NA | NA | NA | NA | 4.96 | 4 | NA | NA | 8.2 | 16 | 10.62 | 16 |
| 5 | Liaohe River watershed, China | Monsoon climate of medium latitude | Construction land, agricultural land | Yue et al. (2013) | NA | NA | NA | NA | 1.83 | 3 | NA | NA | 3.4 | 2 | NA | NA | 10.41 | 20 | 10.28 | 12 |
| 6 | Chaobai River, China | Temperate continental monsoon climate | Construction land, agricultural land | Li et al. (2014a) | 1.04 | 2 | 7.25 | 2 | 1.16 | 4 | 1.02 | 3 | 5.59 | 2 | NA | NA | 9.96 | 37 | 9.75 | 18 |
| 7 | Jialing River, China | Subtropical monsoon climate | Agricultural land | Li et al. (2014b) | NA | NA | NA | NA | 3.95 | 13 | 3.73 | 5 | 4.31 | 10 | 4.83 | 9 | 6.12 | 19 | 7.76 | 20 |
| 8 | The Yellow River watershed, China | Warm temperate subhumid climate | Construction land, agricultural land | Zhang et al. (2014) | NA | NA | 1.3 | 1 | 2.5 | 1 | 0.77 | 3 | 2.5 | 1 | 3.5 | 1 | 15.55 | 4 | 13.17 | 7 |
| 9 | Northwestern Taihu watershed, China | Subtropical monsoon climate | Construction land, agricultural land | Ding et al. (2015) | NA | NA | NA | NA | 4.67 | 4 | 5.02 | 2 | 5.23 | 15 | 5.26 | 7 | 6.34 | 26 | 7.15 | 31 |
| 10 | Sanjiang Plain of Northeast China | Monsoon climate of medium latitude | Construction land, agricultural land | Lu et al. (2015) | NA | NA | NA | NA | 3.12 | 6 | 4.9 | 1 | 3.52 | 5 | 5.21 | 2 | 8.31 | 5 | 8.98 | 9 |
| 11 | Western Taihu watershed, China | Subtropical monsoon climate | Construction land, agricultural land | Zeng & Wu. (2015) | NA | NA | NA | NA | 4.5 | 5 | 4.66 | 1 | 4.71 | 6 | 5.11 | 2 | 9.29 | 21 | 10.61 | 24 |
| 12 | Chongqing pan xi river watershed, China | Subtropical monsoon climate | Construction land, agricultural land, forest land | Zhang et al. (2015) | NA | NA | NA | NA | NA | NA | 4.29 | 2 | NA | NA | 4.77 | 3 | 8.65 | 5 | 10.89 | 6 |
| 13 | Feng River, west China | Temperate continental monsoon climate | Construction land, agricultural land, forest land | Zhi et al. (2016) | NA | NA | NA | NA | 2.07 | 16 | 4.26 | 13 | 3.45 | 7 | 4.46 | 14 | 5.93 | 3 | 7.92 | 25 |
| 14 | Haicheng river watershed, China | Monsoon climate of medium latitude | Construction land, agricultural land, forest land | Bu et al. (2017) | NA | NA | NA | NA | 1.55 | 9 | 3.19 | 5 | 4.65 | 5 | 4.22 | 5 | 8.24 | 11 | 7.6 | 10 |
| 15 | The complex river network in the west of Taihu lake, China | Subtropical monsoon climate | Construction land, agricultural land | Yi et al. (2017) | NA | NA | NA | NA | 4.32 | 7 | 4.07 | 11 | 4.82 | 11 | 4.55 | 16 | 8.38 | 25 | 7.81 | 29 |
| 16 | Beiyun river in Beijing, China | Temperate continental monsoon climate | Construction land, agricultural land | Liu et al. (2018) | NA | NA | NA | NA | 2.92 | 1 | NA | NA | NA | NA | NA | NA | 21.09 | 15 | 17.33 | 15 |
| 17 | Jinyun and Jinyang watershed, China | Temperate continental monsoon climate | Construction land, agricultural land | Yang et al. (2018) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 8.29 | 10 | 8.37 | 11 |
| 18 | Zhejiang Hexi reservoir watershed, China | Subtropical monsoon climate | Construction land, agricultural land | Zhang et al. (2018) | NA | NA | NA | NA | 2.65 | 22 | 2.7 | 32 | 3.42 | 20 | 4.56 | 33 | 8.23 | 42 | 6.76 | 40 |
| 19 | Taihu lake northwest plain watershed, China | Subtropical monsoon climate | Construction land, agricultural land | Ding et al. (2019) | NA | NA | NA | NA | 4.68 | 2 | 4.5 | 12 | 5.64 | 15 | 5.04 | 34 | 6.99 | 59 | 6.91 | 60 |
| 20 | Zhejiang yongan river watershed, China | Subtropical monsoon climate | Construction land, agricultural land, forest land | Hu et al. (2019) | NA | NA | NA | NA | 2.65 | 43 | 3.05 | 14 | 4.5 | 37 | 4.43 | 13 | 8.63 | 81 | 10.96 | 61 |
| 21 | Xijiang River, China | Subtropical monsoon climate | Construction land, agricultural land, forest land | Li et al. (2019a) | NA | NA | NA | NA | 4.39 | 1 | NA | NA | 5.61 | 12 | NA | NA | 7.76 | 51 | 8.25 | 35 |
| 22 | Huai River watershed, Eastern China | Temperate continental monsoon climate | Agricultural land | Ma et al. (2019) | NA | NA | NA | NA | NA | NA | NA | NA | 5.1 | 1 | NA | NA | 12.35 | 14 | 12.67 | 14 |
| 23 | Suburbs around Beijing, China | Temperate continental monsoon climate | Agricultural land | Peters et al. (2019) | 1.03 | 5 | 4.13 | 1 | 1.4 | 7 | 1.6 | 7 | 6.03 | 2 | 4.74 | 4 | 14.21 | 14 | 9.95 | 21 |
| 24 | Chongqing and Hubei province border area, China | Subtropical monsoon climate | Construction land, agricultural land | Zhao et al. (2019) | NA | NA | NA | NA | 4.47 | 6 | 4.91 | 2 | 5.09 | 13 | 5.47 | 8 | 6.22 | 17 | 6.93 | 23 |
| 25 | Bow and Oldman Rivers, Canada | Temperate continental climate | Construction land, agricultural land, forest land | Kruk et al. (2020) | NA | NA | NA | NA | 2.05 | 6 | 1.81 | 6 | 3.09 | 3 | 2.82 | 3 | 10.06 | 21 | 9.57 | 11 |
| 26 | The Beiyun riverine watershed, China | Temperate continental monsoon climate | Construction land, agricultural land | This study | NA | NA | NA | NA | 2.34 | 3 | NA | NA | 5.53 | 2 | 6 | 1 | 16.01 | 53 | 14.42 | 51 |

AD: atmospheric deposition.  NF: nitrogen fertilizer.  SON: soil nitrogen.  M&S: manure & sewage.

ck: mean value of control groups.  cn: the number of control groups in study.  tr: mean value of experimental groups.  tn: the number of experimental groups in study.

Table S4 Regular indices for the analyzed river water samples from Beiyun river watersheds.

| Site |  | NO3-N | NO2-N | NH4-N | pH | DO | EC | δ15N-NO3 | δ18O-NO3 | Cl- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit |  | mg/L | mg/L | mg/L | - | mg/L | uS/cm | ‰ | ‰ | mg/L |
| Wet season |  |  |  |  |  |  |  |  |  |  |
| Main stream | M1 | 4.8±1.5 | 0.2±0.1 | 1±0.3 | 8.4±0.4 | 10.9±3.5 | 699±104 | 14.5±0.4 | 1.6±1.1 | 89.0 |
|  | M2 | 5.3±1.3 | 0.2±0.1 | 0.7±0.6 | 8.3±0.5 | 14.6±5.3 | 3492±2868 | 18.3±6.8 | 1.2±0.7 | 111.5 |
|  | M3 | 7.9±0.9 | 0.4±0.2 | 1.2±1.1 | 8.2±0.2 | 9.5±0.1 | 4010.5±3349.5 | 14±0.9 | 2.3±0.9 | 140.6 |
|  | M4 | 8.5±0.1 | 0.2±0 | 0.3±0.2 | 8.2±0.3 | 8.8±0.8 | 1200±970.1 | 12.2±0.5 | 4.1±0.6 | 167.2 |
|  | M5 | 8.1±0.9 | 0.2±0 | 0.4±0 | 8.1±0.3 | 9.5±1.7 | 2106.5±1483.5 | 13.3±0.2 | 3.1±2.2 | 168.2 |
|  | M6 | 5.6±1 | 0.1±0 | 0.3±0.1 | 8.3±0.4 | 8.5±0.3 | 2504.5±1715.5 | 15.1±0.9 | 2.8±0.4 | 188.0 |
|  | M7 | 5.1±1.6 | 0.1±0 | 0.8±0.4 | 8.4±0.2 | 8.8±1.3 | 1045.5±314.5 | 8.4±0.5 | -2.2±0.5 | 194.2 |
|  | M8 | 1.2±0 | 0.1±0 | 0.8±0.4 | 8.3±0 | 6.6±0 | 1891±0 | 9±3.2 | 2.2±1.2 | 19.7 |
| Tributary | T1 | 4.9±0.5 | 0.3±0.1 | 0.3±0.2 | 8.7±0 | 8.8±0 | 1370±0 | 12.7±5.5 | -2±0.8 | 100.2 |
|  | T2 | 7±2.7 | 0.2±0.1 | 0.9±0.5 | 8±0.2 | 7.7±0.8 | 2465±1565 | 20.8±1.3 | 3.7±1.6 | 111.0 |
|  | T3 | 6.5±5 | 0±0 | 0.5±0.2 | 8±0.2 | 9±1.5 | 1948±1212 | 14±1.1 | 1.5±1.6 | 126.1 |
|  | T4 | 11±1.7 | 0.1±0.1 | 1±0.6 | 8.5±0.1 | 10.7±2 | 1038.4±551.7 | 15.3±1.4 | -1.5±3 | 181.3 |
|  | T5 | 1±0.1 | 0±0 | 0.5±0.2 | 8.2±0.2 | 10±0.2 | 1574.5±735.5 | 15.9±1.5 | -1.7±3.4 | 19.7 |
|  | T6 | 8±2.3 | 0±0 | 0.3±0.2 | 8.5±0.1 | 9.1±1.7 | 1398.5±688.5 | 19.5±6.5 | 2.6±0.3 | 90.3 |
|  | T7 | 1.1±0.1 | 0±0 | 0.4±0.2 | 8.1±0.2 | 8.3±0.6 | 1306±473 | 17.5±4.3 | 1.1±0.2 | 18.4 |
|  | T8 | 6.5±1.1 | 0.1±0.1 | 0.7±0.6 | 8.3±0 | 9.2±1.3 | 1352±418 | 17.7±0.9 | -0.3±1.5 | 192.6 |
|  | T9 | 6.8±1.4 | 0.3±0.1 | 1.2±0.5 | 8.4±0 | 9.6±0.7 | 1301.5±701.5 | 14.5±1.7 | -0.5±1.4 | 120.3 |
|  | T10 | 4.3±0.8 | 0.1±0 | 0.5±0.1 | 8.2±0 | 7.8±0.3 | 1078±248 | 16.3±0.1 | -0.9±0.9 | 133.2 |
|  | T11 | 15.9±0.8 | 0.1±0 | 0.5±0.1 | 8.3±0 | 8±0.1 | 1076±156 | 19±0.1 | -1.8±2.8 | 158.2 |
|  | T12 | 10.4±1.7 | 0.1±0.1 | 0.8±0.4 | 8.3±0.1 | 8.6±0.8 | 1374.5±425.5 | 19.6±3.2 | 0±1.1 | 206.9 |
|  | T13 | 5.5±0.9 | 0.1±0 | 0.2±0.1 | 8.5±0.1 | 11.6±2 | 1506±524 | 14.5±2.2 | 2.6±1.9 | 139.1 |
|  | T14 | 7.1±1.9 | 0.2±0.1 | 0.8±0.4 | 8.1±0.2 | 7.9±0.4 | 2295±1019 | 9.2±8.6 | -1.9±3.7 | 197.4 |
|  | T15 | 7.7±1.4 | 0.2±0.1 | 0.4±0 | 8±0.3 | 8.7±0.6 | 1290.5±403.5 | 19.1±1.1 | -3.6±5 | 167.2 |
|  | T16 | 7.1±0.6 | 0.1±0.1 | 0.9±0.5 | 8.3±0.2 | 9±0.6 | 1404±477 | 19.4±0.9 | 1.8±0.5 | 182.8 |
|  | T17 | 6.4±2.5 | 0.3±0 | 0.4±0.4 | 8.2±0.3 | 8.4±1.2 | 1456.5±431.5 | 8.9±8.3 | 1.7±2.1 | 244.3 |
|  | T18 | 3.9±0.3 | 0.1±0.1 | 0.3±0.2 | 8.3±0.2 | 7.9±0.5 | 784.5±394.6 | 13.4±5 | -1.4±3.3 | 187.8 |
| Dry season |  |  |  |  |  |  |  |  |  |  |
| Main stream | M1 | 8±1.2 | 0.1±0 | 0.9±0.2 | 8.2±0 | 9±0.1 | 1089±0 | 7.6±14.1 | -2.2±7.4 | 93.4 |
|  | M2 | 8.5±0.3 | 0.2±0.1 | 0.5±0.4 | 8.1±0 | 10±1.1 | 1082±0 | 13.9±7.2 | -1.8±-4.2 | 92.3 |
|  | M3 | 9.2±0.3 | 0.2±0 | 1.1±1 | 8.2±0 | 10.8±1.2 | 1803.5±137.5 | 16.3±14.4 | -4.1±-1.6 | 157.6 |
|  | M4 | 9.9±0 | 0.2±0 | 0.2±0.2 | 8.3±0.1 | 10.8±1.5 | 1669.5±63.5 | 16±14.6 | -5.1±2.5 | 145.9 |
|  | M5 | 9.6±0.2 | 0.2±0 | 0.7±0.3 | 8.2±0 | 11±1.9 | 1885.5±131.5 | 16.5±15.1 | -3.9±-1.9 | 158.9 |
|  | M6 | 8.1±0.6 | 0.1±0 | 0.6±0.4 | 8±0 | 11.7±1 | 1888±0 | 14.5±13.3 | -13.3±3 | 180.2 |
|  | M7 | 6.8±1.8 | 0.1±0 | 0.8±0.4 | 8.1±0 | 10.2±0.2 | 1964±0 | 15.2±16 | -2.2±2.7 | 177.4 |
|  | M8 | 0.8±0.1 | 0±0 | 0.7±0.3 | 8.3±0 | 9.5±0.5 | 1193±0 | 16.2±6 | -9.7±9.8 | 12.4 |
| Tributary | T1 | 6±0.5 | 0.1±0 | 0.7±0.6 | 8.1±0 | 11.5±0 | 954±0 | 14.8±14.2 | -8.1±7.1 | 80.2 |
|  | T2 | 9.7±0.7 | 0.1±0 | 0.7±0.3 | 8.2±0 | 10.2±0.7 | 1312±0 | 10.3±12.8 | -11.9±6.1 | 126.3 |
|  | T3 | 12.3±1.4 | 0.1±0 | 0.7±0.1 | 8.4±0.2 | 11.7±2 | 1804±147 | 15.1±10.6 | -4±-0.1 | 131.9 |
|  | T4 | 11.7±1.2 | 0.1±0 | 0.2±0.2 | 8±0 | 10±1.4 | 817±0 | 10.4±13.7 | -3±0.5 | 183.6 |
|  | T5 | 1.1±0 | 0±0 | 0.6±0.1 | 8.4±0.1 | 9.5±1.1 | 1307.5±119.5 | 6.6±10.3 | -6.3±1.4 | - |
|  | T6 | 10.5±0.3 | 0±0 | 0.7±0.3 | 8.1±0.1 | 9±0.2 | 1809±89 | 13.6±10.8 | -1.1±5.7 | 88.4 |
|  | T7 | 1.2±0 | 0±0 | 0.4±0.1 | 8.4±0 | 9.6±1 | 1306±153 | 10.3±7.5 | -9.9±0.8 | 7.6 |
|  | T8 | 8.2±0.3 | 0±0 | 0.4±0.3 | 8±0 | 10.4±0.7 | 1864±0 | 15.9±24.7 | -0.5±0.4 | 145.2 |
|  | T9 | 13.7±1.7 | 0.5±0.1 | 0.5±0.2 | 8.1±0 | 10.2±0.7 | 1746±0 | 17±15 | -1.7±4.9 | 72.3 |
|  | T10 | 12.1±0.2 | 0.2±0.2 | 0.6±0.2 | 8.3±0 | 10±0.5 | 1810±92 | 14.3±15.8 | -3±4.6 | 40.6 |
|  | T11 | 11.1±5.6 | 0.1±0 | 0.7±0.3 | 8.2±0 | 9.5±0.1 | 1987±0 | 14±15.3 | -6.6±2.4 | 114.6 |
|  | T12 | 11.2±1.3 | 0.1±0.1 | 0.7±0.3 | 8±0 | 11±0 | 1972±0 | 17.1±19.3 | -6.3±2.7 | 170.2 |
|  | T13 | 4.6±0.8 | 0.1±0 | 0.2±0.1 | 8.1±0 | 10±1.1 | 1973.5±43.5 | 16.7±14.5 | -6.1±3.9 | 157.0 |
|  | T14 | 9±1.9 | 0.3±0 | 0.7±0.3 | 8.5±0 | 12.6±2.9 | 1685.5±166.5 | 17.9±14.8 | -7.7±9.9 | 162.6 |
|  | T15 | 10±0 | 0.2±0.1 | 0.3±0.1 | 8.3±0.1 | 11.2±1.4 | 1668±63 | 16.7±14.7 | -9.3±8 | 162.5 |
|  | T16 | 8±0.5 | 0.2±0.1 | 0.9±0.5 | 8.1±0 | 11.5±0.4 | 1549±0 | 17.6±19 | -5.5±3 | 183.1 |
|  | T17 | 8.4±1.4 | 0.3±0 | 0.8±0.8 | 8±0 | 10.5±0.5 | 2021±0 | 14.9±16.4 | -6.9±-0.9 | 233.1 |
|  | T18 | 7.3±2.3 | 0.1±0.1 | 0.3±0.2 | 0±0 | 9.6±0.5 | 1694±0 | 15.4±16.6 | -5.2±2.6 | 129.5 |

Fig. S1. Flow diagram of study selection.

Fig. S2. Contour-enhanced funnel chart of four sources (atmospheric deposition, NO3/NH4-fertilizer, soil nitrogen, and manure & sewage). The solid dots are valid samples. Only the atmospheric deposition showed significant asymmetry because the sample size was too small.

Fig. S3. Accumulation diagram of inorganic nitrogen. (a) June of 2019. (b) September of 2019. (c) November of 2019. (d) January of 2020.

Fig. S4. The relationship between the value of δ15N-NO3 and δ18O-NO3. (a) June of 2019. (b) September of 2019. (c) November of 2019. (d) January of 2020.

Fig. S5. The relationship between NO3 concentration and δ15N-NO3 value in all months.

Fig. S6. Distribution of NO3 / Cl- in main stream and tributary in different seasons.

Fig. S7. Cluster dendrogram based on the NO3-N, NO2-N, NH4-N, TN, δ15N-NO3 δ18O-NO3, and land-use type in the (a) wet season and (b) dry season. Clustering heat map in the (c) wet season and (d) dry season (The lower distance of the matrix along with the higher similarity of the site).

Fig. S8. A comparative map of world climate Köppen-Geiger classification (Beck et al., 2018; Kottek et al., 2006) and world population distribution in 2018 (datahub.io).

Reference

Beck H., Zimmermann N., Mcvicar T., Vergopolan N., Berg A., Wood E., 2018. Present and future Köppen-Geiger climate classification maps at 1-km resolution. Scientific Data. 5(1), 180214.

Bu H., Song X., Zhang Y., Meng W., 2017. Sources and fate of nitrate in the Haicheng River basin in Northeast China using stable isotopes of nitrate. Ecological Engineering. 98, 105-113.

Ding J., Xi B., Gao R., He L., Liu H., Dai X., Yu Y., 2014. Identifying diffused nitrate sources in a stream in an agricultural field using a dual isotopic approach. Science of the Total Environment. 484, 10-18.

Ding J., Xi B., Xu Q., Meng H., Shen Y., Cheng H., 2019. Isotopic evidence of nitrate sources and its transformations in a human-impacted watershed. Environmental Science-Processes & Impacts. 21(3), 575-583.

Ding J., Xi B., Xu Q., Su J., Huo S., Liu H., Yu Y., Zhang Y., 2015. Assessment of the sources and transformations of nitrogen in a plain river network region using a stable isotope approach. Journal of Environmental Sciences. 30, 198-206.

Egger M., Smith G., Schneider M., Minder C., 1997. Bias in meta-analysis detected by a simple, graphical test. The BMJ. 315(7109), 629-634.

Hedges L., Gurevitch J., Curtis P., 1999. The meta-analysis of response ratios in experimental ecology. Ecology. 80(4), 1150-1156.

Hu M., Liu Y., Zhang Y., Dahlgren R., Chen D., 2019. Coupling stable isotopes and water chemistry to assess the role of hydrological and biogeochemical processes on riverine nitrogen sources. Water Research. 150, 418-430.

Kottek M., Grieser J., Beck C., Rudolf B., Rubel F., 2006. World Map of the Köppen-Geiger climate classification updated. Meteorologische Zeitschrift. 15(3), 259-263.

Kruk M., Mayer B., Nightingale M., Laceby J., 2020. Tracing nitrate sources with a combined isotope approach (delta(15)NNO3, delta(18)ONO3 and delta(11)B) in a large mixed-use watershed in southern Alberta, Canada. Science of the Total Environment. 703, 135043.

Lee K., Bong Y., Lee D., Kim Y., Kim K., 2008. Tracing the sources of nitrate in the Han River watershed in Korea, using delta15N-NO3- and delta18O-NO3- values. Science of the Total Environment. 395(2-3), 117-124.

Li C., Jiang Y., Guo X., Cao Y., Ji H., 2014a. Multi-isotope ((15)N, (18)O and (13)C) indicators of sources and fate of nitrate in the upper stream of Chaobai River, Beijing, China. Environmental Science-Processes & Impacts. 16(11), 2644-2655.

Li C., Li S., Yue F., Liu J., Zhong J., Yan Z., Zhang R., Wang Z., Xu S., 2019. Identification of sources and transformations of nitrate in the Xijiang River using nitrate isotopes and Bayesian model. Science of the Total Environment. 646, 801-810.

Li X., Liu C., Liu X., Yu J., Liu X., 2014b. Sources and Processes Affecting Nitrate in a Dam-Controlled Subtropical River, Southwest China. Aquatic Geochemistry. 20(5), 483-500.

Li Z., Cui J., Mi Z., Tian D., Wang J., Ma Z., Wang B., Chen H., Niu S., 2019b. Responses of soil enzymatic activities to transgenic Bacillus thuringiensis (Bt) crops - A global meta-analysis. Science of the Total Environment. 651(2), 1830-1838.

Liu J., Shen Z., Yan T., Yang Y., 2018b. Source identification and impact of landscape pattern on riverine nitrogen pollution in a typical urbanized watershed, Beijing, China. Science of the Total Environment. 628-629, 1296-1307.

Liu Y., Wang X., Wen Q., Zhu N., 2019. Identifying sources and variations of organic matter in an urban river in Beijing, China using stable isotope analysis. Ecological Indicators. 102, 783-790.

Lu L., Cheng H., Pu X., Liu X., Cheng Q., 2015. Nitrate behaviors and source apportionment in an aquatic system from a watershed with intensive agricultural activities. Environmental Science-Processes & Impacts. 17(1), 131-144.

Ma P., Liu S., Yu Q., Li X., Han X., 2019. Sources and transformations of anthropogenic nitrogen in the highly disturbed Huai River Basin, Eastern China. Environmental Science and Pollution Research. 26(11), 11153-11169.

Panno S., Kelly W., Hackley K., Hwang H., Martinsek A., 2008. Sources and fate of nitrate in the Illinois River Basin, Illinois. Journal of Hydrology. 359(1-2), 174-188.

Peng T., Lin H., Wang C., Liu T., Kao S., 2012. Pollution and variation of stream nitrate in a protected high-mountain watershed of Central Taiwan: evidence from nitrate concentration and nitrogen and oxygen isotope compositions. Environmental Monitoring and Assessment. 184(8), 4985-4998.

Peters M., Guo Q., Strauss H., Wei R., Li S., Yue F., 2019. Contamination patterns in river water from rural Beijing: A hydrochemical and multiple stable isotope study. Science of the Total Environment. 654, 226-236.

Shin W., Ryu J., Lee K., Chung G., 2013. Seasonal and spatial variations in water chemistry and nitrate sources in six major Korean rivers. Environmental Earth Sciences. 68(8), 2271-2279.

Wu Z., Dijkstra P., Koch G., PeÑuelas J., Hungate B., 2011. Responses of terrestrial ecosystems to temperature and precipitation change: a meta-analysis of experimental manipulation. Global Change Biology. 17(2), 927-942.

Xian C., Ouyang Z., Li Y., Xiao Y., Ren Y., 2016. Variation in nitrate isotopic signatures in sewage for source apportionment with urbanization: a case study in Beijing, China. Environmental Science and Pollution Research. 23(22), 22871-22881.

Yang B., Wang H., Jiang Y., Dong F., He X., Lai X., 2018. Combing N-15 and O-18 to identify the distribution and the potential sources of nitrate in human-impacted watersheds, Shandong, China. Rsc Advances. 8(41), 23199-23205.

Yi Q., Chen Q., Hu L., Shi W., 2017. Tracking Nitrogen Sources, Transformation, and Transport at a Basin Scale with Complex Plain River Networks. Environmental Science & Technology. 51(10), 5396-5403.

Yue F., Li S., Liu C., Zhao Z., Jian H., 2013. Using dual isotopes to evaluate sources and transformation of nitrogen in the Liao River, northeast China. Applied Geochemistry. 36, 1-9.

Zeng H., Wu J., 2015. Tracing the Nitrate Sources of the Yili River in the Taihu Lake Watershed: A Dual Isotope Approach. Water. 7(1), 188-201.

Zhang M., Zhi Y., Shi J., Wu L., 2018a. Apportionment and uncertainty analysis of nitrate sources based on the dual isotope approach and a Bayesian isotope mixing model at the watershed scale. Science of the Total Environment. 639, 1175-1187.

Zhang Q., Wang X., Sun F., Sun J., Liu J., Ouyang Z., 2015. Assessment of temporal and spatial differences of source apportionment of nitrate in an urban river in China, using delta(15)N and delta(18)O values and an isotope mixing model. Environmental Science and Pollution Research. 22(24), 20226-20233.

Zhang Y., Li F., Zhang Q., Li J., Liu Q., 2014. Tracing nitrate pollution sources and transformation in surface- and ground-waters using environmental isotopes. Science of the Total Environment. 490, 213-222.

Zhang Y., Liu X., Fangmeier A., Goulding K., Zhang F., 2008. Nitrogen inputs and isotopes in precipitation in the North China Plain. Atmospheric Environment. 42(7), 1436-1448.

Zhao Y., Zheng B., Jia H., Chen Z., 2019. Determination sources of nitrates into the Three Gorges Reservoir using nitrogen and oxygen isotopes. Science of the Total Environment. 687, 128-136.

Zhi J., Ding A., Zhang S., 2016. Nitrate sources and nitrogen biogeochemical processes in the Feng River in West China inferred from the nitrogen and oxygen dual isotope measurements of nitrate. Desalination and Water Treatment. 57(18), 8243-8251.