Appendix

| Table A-1. Brain tissue material library: conversion of HU values to densities and elemental compositions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUL | HUU | Density | Elemental composition (wt%) |  |  |  |  |  |  |  |  |
|  |  | g/cm3 | H | C | N | O | Na | P | S | Cl | K |
| min | 5 | 1.030a | 10.80 | 4.10 | 1.10 | 83.20 | 0.30 | 0.00 | 0.10 | 0.40 | 0.00 |
| 5 | 10 | 1.030b | 10.70 | 9.10 | 1.60 | 77.60 | 0.20 | 0.30 | 0.10 | 0.20 | 0.20 |
| 10 | 12 | 1.031 | 10.70 | 9.10 | 1.60 | 77.60 | 0.20 | 0.30 | 0.10 | 0.20 | 0.20 |
| 12 | 15 | 1.032 | 10.70 | 9.19 | 1.64 | 77.40 | 0.20 | 0.30 | 0.12 | 0.22 | 0.22 |
| 15 | 17 | 1.033 | 10.70 | 9.23 | 1.67 | 77.30 | 0.20 | 0.30 | 0.13 | 0.23 | 0.23 |
| 17 | 20 | 1.034 | 10.70 | 9.28 | 1.69 | 77.20 | 0.20 | 0.30 | 0.14 | 0.24 | 0.24 |
| 20 | 22 | 1.035 | 10.70 | 9.32 | 1.71 | 77.10 | 0.20 | 0.30 | 0.16 | 0.26 | 0.26 |
| 22 | 25 | 1.036 | 10.70 | 9.37 | 1.73 | 77.00 | 0.20 | 0.30 | 0.17 | 0.27 | 0.27 |
| 25 | 27 | 1.037 | 10.70 | 9.41 | 1.76 | 76.90 | 0.20 | 0.30 | 0.18 | 0.28 | 0.28 |
| 27 | 30 | 1.038 | 10.70 | 9.46 | 1.78 | 76.80 | 0.20 | 0.30 | 0.19 | 0.29 | 0.29 |
| 30 | 35 | 1.039c | 10.70 | 9.50 | 1.80 | 76.70 | 0.2 | 0.30 | 0.20 | 0.30 | 0.30 |
| 35 | 40 | 1.040d | 10.70 | 14.50 | 2.20 | 71.20 | 0.20 | 0.40 | 0.20 | 0.30 | 0.30 |
| 40 | 45 | 1.041 | 10.67 | 16.13 | 2.30 | 69.50 | 0.20 | 0.40 | 0.20 | 0.30 | 0.30 |
| 50 | max | 1.043e | 10.60 | 19.40 | 2.50 | 66.10 | 0.20 | 0.40 | 0.20 | 0.30 | 0.30 |
| HUL : the lower limit of the image HU value; HUu: the upper limit of the image HU value; alymph data from ICRU-46; bbrain(whole)-infant data from ICRU-46; cgray matter data from physical properties of tissues: a comprehensive reference book; dbrain(whole)-adult data from ICRU-46; ewhite matter data from physical properties of tissues: a comprehensive reference book. |  |  |  |  |  |  |  |  |  |  |  |

| Table A-2. Brain tumor material library: conversion of HU values to densities and elemental compositions (expand from table A-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUL | HUU | Density | Elemental composition (wt%) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | g/cm3 | H | C | N | O | Na | Mg | P | S | Cl | K | Ca | F |  |
| 50 | 55 | 1.043a | 10.60 | 19.40 | 2.50 | 66.10 | 0.20 | 0.00 | 0.40 | 0.20 | 0.30 | 0.30 | 0.00 | 0.00 |  |
| 55 | 60 | 1.045 | 10.57 | 19.71 | 2.50 | 65.67 | 0.20 | 0.00 | 0.44 | 0.20 | 0.30 | 0.30 | 0.11 | 0.00 |  |
| 60 | 65 | 1.05 | 10.49 | 20.47 | 2.52 | 64.60 | 0.19 | 0.01 | 0.55 | 0.20 | 0.29 | 0.29 | 0.38 | 0.01 |  |
| 65 | 70 | 1.06 | 10.34 | 22.01 | 2.54 | 62.45 | 0.19 | 0.01 | 0.77 | 0.20 | 0.29 | 0.28 | 0.92 | 0.01 |  |
| 70 | 75 | 1.07 | 10.19 | 23.54 | 2.56 | 60.31 | 0.18 | 0.02 | 0.99 | 0.20 | 0.28 | 0.26 | 1.46 | 0.02 |  |
| 75 | 80 | 1.08 | 10.03 | 25.07 | 2.58 | 58.16 | 0.17 | 0.03 | 1.21 | 0.20 | 0.27 | 0.25 | 2.00 | 0.03 |  |
| 80 | 85 | 1.09 | 9.88 | 26.60 | 2.60 | 56.01 | 0.17 | 0.03 | 1.43 | 0.20 | 0.27 | 0.23 | 2.54 | 0.03 |  |
| 85 | 90 | 1.10 | 9.73 | 28.14 | 2.62 | 53.87 | 0.16 | 0.04 | 1.65 | 0.20 | 0.26 | 0.22 | 3.08 | 0.04 |  |
| 90 | 100 | 1.14 | 9.11 | 34.27 | 2.71 | 45.28 | 0.13 | 0.07 | 2.52 | 0.20 | 0.23 | 0.16 | 5.24 | 0.07 |  |
| 100 | 130 | 1.18b | 8.50 | 40.40 | 2.80 | 36.70 | 0.10 | 0.10 | 3.40 | 0.20 | 0.20 | 0.10 | 7.40 | 0.10 |  |
| 130 | 170 | 1.24 | 7.95 | 36.83 | 3.08 | 38.43 | 0.10 | 0.10 | 4.08 | 0.23 | 0.18 | 0.10 | 7.40 | 0.10 |  |
| 170 | 210 | 1.3 | 7.40 | 33.25 | 3.35 | 40.15 | 0.10 | 0.10 | 4.75 | 0.25 | 0.15 | 0.10 | 8.88 | 0.07 |  |
| 210 | 250 | 1.36 | 6.85 | 29.68 | 3.63 | 41.88 | 0.10 | 0.10 | 5.43 | 0.28 | 0.13 | 0.10 | 10.35 | 0.05 |  |
| 250 | 330 | 1.42c | 6.30 | 26.10 | 3.90 | 43.60 | 0.10 | 0.10 | 6.10 | 0.30 | 0.10 | 0.10 | 11.83 | 0.02 |  |
| 330 | 410 | 1.46 | 6.03 | 25.07 | 3.92 | 43.58 | 0.10 | 0.12 | 6.52 | 0.30 | 0.08 | 0.1 | 13.3 | 0.0 |  |
| 410 | 490 | 1.5 | 5.75 | 24.04 | 3.94 | 43.56 | 0.10 | 0.14 | 6.94 | 0.30 | 0.06 | 0.08 | 14.21 | 0.00 |  |
| 490 | 570 | 1.54 | 5.48 | 23.01 | 3.96 | 43.54 | 0.10 | 0.16 | 7.36 | 0.30 | 0.04 | 0.06 | 15.11 | 0.00 |  |
| 570 | 650 | 1.58 | 5.21 | 21.97 | 3.98 | 43.52 | 0.10 | 0.18 | 7.78 | 0.30 | 0.02 | 0.04 | 16.02 | 0.00 |  |
| 650 | 730 | 1.61d | 5.00 | 21.20 | 4.00 | 43.50 | 0.10 | 0.20 | 8.10 | 0.30 | 0.00 | 0.02 | 16.92 | 0.00 |  |
| HUL : the lower limit of the image HU value; HUu: the upper limit of the image HU value; a white matter data from physical properties of tissues: a comprehensive reference book; bskeleton-spongiosa data; cskeleton-vertebral column data; dskeleton-cranium data from ICRU-46. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table A-3. Skeleton tissue material library: conversion of HU values to densities and elemental compositions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUL | HUU | Density | Elemental composition (wt%) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | g/cm3 | H | C | N | O | Na | Mg | P | S | Cl | K | Ca | F |  |
| min | 40 | 1.03a | 10.50 | 41.40 | 3.40 | 43.90 | 0.00 | 0.00 | 0.10 | 0.20 | 0.20 | 0.20 | 0.00 | 0.10 |  |
| 40 | 50 | 1.08 | 9.83 | 41.07 | 3.20 | 41.50 | 0.03 | 0.03 | 1.20 | 0.20 | 0.20 | 0.17 | 2.47 | 0.10 |  |
| 50 | 90 | 1.13 | 9.17 | 40.73 | 3.00 | 39.10 | 0.07 | 0.07 | 2.30 | 0.20 | 0.20 | 0.13 | 4.93 | 0.10 |  |
| 90 | 130 | 1.18b | 8.50 | 40.40 | 2.80 | 36.70 | 0.10 | 0.10 | 3.40 | 0.20 | 0.20 | 0.10 | 7.40 | 0.10 |  |
| 130 | 170 | 1.24 | 7.95 | 36.83 | 3.08 | 38.43 | 0.10 | 0.10 | 4.08 | 0.23 | 0.18 | 0.10 | 8.88 | 0.07 |  |
| 170 | 210 | 1.300 | 7.40 | 33.25 | 3.35 | 40.15 | 0.10 | 0.10 | 4.75 | 0.25 | 0.15 | 0.10 | 10.35 | 0.05 |  |
| 210 | 250 | 1.36 | 6.85 | 29.68 | 3.63 | 41.88 | 0.10 | 0.10 | 5.43 | 0.28 | 0.13 | 0.10 | 11.83 | 0.02 |  |
| 250 | 330 | 1.42c | 6.30 | 26.10 | 3.90 | 43.60 | 0.10 | 0.10 | 6.10 | 0.30 | 0.10 | 0.10 | 13.30 | 0.00 |  |
| 330 | 410 | 1.46 | 6.03 | 25.07 | 3.92 | 43.58 | 0.10 | 0.12 | 6.52 | 0.30 | 0.08 | 0.08 | 14.21 | 0.00 |  |
| 410 | 490 | 1.50 | 5.75 | 24.04 | 3.94 | 43.56 | 0.10 | 0.14 | 6.94 | 0.30 | 0.06 | 0.06 | 15.11 | 0.00 |  |
| 490 | 570 | 1.54 | 5.48 | 23.01 | 3.96 | 43.54 | 0.10 | 0.16 | 7.36 | 0.30 | 0.04 | 0.04 | 16.02 | 0.00 |  |
| 570 | 650 | 1.58 | 5.21 | 21.97 | 3.98 | 43.52 | 0.10 | 0.18 | 7.78 | 0.30 | 0.02 | 0.02 | 16.92 | 0.00 |  |
| 650 | 730 | 1.61d | 5.00 | 21.20 | 4.00 | 43.50 | 0.10 | 0.20 | 8.10 | 0.30 | 0.00 | 0.00 | 17.60 | 0.00 |  |
| 730 | 800 | 1.65 | 4.77 | 20.46 | 4.06 | 43.50 | 0.10 | 0.20 | 8.39 | 0.30 | 0.00 | 0.00 | 18.23 | 0.00 |  |
| 800 | 900 | 1.68e | 4.60 | 19.90 | 4.10 | 43.50 | 0.10 | 0.20 | 8.60 | 0.30 | 0.00 | 0.00 | 18.70 | 0.00 |  |
| 900 | 1000 | 1.74 | 4.30 | 18.80 | 4.13 | 43.50 | 0.10 | 0.20 | 9.03 | 0.30 | 0.00 | 0.00 | 19.65 | 0.00 |  |
| 1000 | 1100 | 1.80 | 4.00 | 17.70 | 4.15 | 43.50 | 0.10 | 0.20 | 9.45 | 0.30 | 0.00 | 0.00 | 20.60 | 0.00 |  |
| 1100 | 1200 | 1.86 | 3.70 | 16.60 | 4.18 | 43.50 | 0.10 | 0.20 | 9.88 | 0.30 | 0.00 | 0.00 | 21.55 | 0.00 |  |
| 1200 | max | 1.92f | 3.40 | 15.50 | 4.20 | 43.50 | 0.10 | 0.20 | 10.30 | 0.30 | 0.00 | 0.00 | 22.50 | 0.00 |  |
| HUL : the lower limit of the image HU value; HUu: the upper limit of the image HU value; aSkeleton-red marrow (40% fat, 40% water and 20% protein) data ; bSkeleton-spongiosa (33% cortical bone+67% marrow) data; CSkeleton-vertebral column (C4) data from; dSkeleton-cranium data; eSkeleton-mandible data; fSkeleton-cortical bone data from ICRU-46. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Table A-4. HNC material library: conversion of HU values to densities and elemental compositions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HUL | HUU | Density | Elemental composition (wt%) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | g/cm3 | H | C | N | O | Na | Mg | P | S | Cl | K | Ca | F |
| min | -80 | 0.92a | 11.80 | 77.30 | 0.00 | 10.90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| -80 | -60 | 0.93b | 11.60 | 68.10 | 0.20 | 19.80 | 0.10 | 0.00 | 0.00 | 0.10 | 0.10 | 0.00 | 0.00 | 0.00 |
| -60 | -40 | 0.95c | 11.40 | 59.80 | 0.70 | 27.80 | 0.10 | 0.00 | 0.00 | 0.10 | 0.10 | 0.00 | 0.00 | 0.00 |
| -40 | -20 | 0.97d | 11.20 | 51.70 | 1.30 | 35.50 | 0.10 | 0.00 | 0.00 | 0.10 | 0.10 | 0.00 | 0.00 | 0.00 |
| -20 | 0 | 0.98e | 10.90 | 71.90 | 3.00 | 13.80 | 0.10 | 0.00 | 0.00 | 0.20 | 0.10 | 0.00 | 0.00 | 0.00 |
| 0 | 20 | 1.00f | 10.10 | 11.10 | 2.60 | 76.20 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 20 | 40 | 1.02g | 10.60 | 31.50 | 2.40 | 54.70 | 0.10 | 0.00 | 0.20 | 0.20 | 0.10 | 0.00 | 0.00 | 0.00 |
| 40 | 60 | 1.03h | 10.50 | 25.60 | 2.70 | 60.20 | 0.10 | 0.00 | 0.20 | 0.30 | 0.20 | 0.20 | 0.00 | 0.00 |
| 60 | 65 | 1.04 | 10.35 | 19.95 | 3.05 | 65.60 | 0.10 | 0.00 | 0.20 | 0.30 | 0.15 | 0.20 | 0.00 | 0.00 |
| 65 | 70 | 1.05i | 10.20 | 14.30 | 3.40 | 71.00 | 0.10 | 0.00 | 0.20 | 0.30 | 0.10 | 0.30 | 0.00 | 0.00 |
| 70 | 75 | 1.08 | 9.81 | 20.32 | 3.26 | 63.08 | 0.10 | 0.02 | 0.94 | 0.28 | 0.12 | 0.40 | 0.00 | 0.00 |
| 75 | 80 | 1.11 | 9.42 | 26.35 | 3.12 | 55.17 | 0.10 | 0.05 | 1.68 | 0.25 | 0.15 | 0.33 | 1.71 | 0.02 |
| 80 | 85 | 1.13 | 9.15 | 30.36 | 3.03 | 49.89 | 0.10 | 0.06 | 2.17 | 0.24 | 0.16 | 0.26 | 3.42 | 0.05 |
| 85 | 90 | 1.15 | 8.89 | 34.38 | 2.94 | 44.62 | 0.10 | 0.08 | 2.66 | 0.22 | 0.18 | 0.22 | 4.55 | 0.06 |
| 90 | 130 | 1.18j | 8.50 | 40.40 | 2.80 | 36.70 | 0.10 | 0.10 | 3.40 | 0.20 | 0.20 | 0.17 | 5.69 | 0.08 |
| 130 | 170 | 1.24 | 7.95 | 36.83 | 3.08 | 38.43 | 0.10 | 0.10 | 4.08 | 0.23 | 0.18 | 0.10 | 7.40 | 0.10 |
| 170 | 210 | 1.30 | 7.40 | 33.25 | 3.35 | 40.15 | 0.10 | 0.10 | 4.75 | 0.25 | 0.15 | 0.10 | 8.88 | 0.07 |
| 210 | 250 | 1.36 | 6.85 | 29.68 | 3.63 | 41.88 | 0.10 | 0.10 | 5.43 | 0.28 | 0.13 | 0.10 | 10.35 | 0.05 |
| 250 | 330 | 1.42k | 6.30 | 26.10 | 3.90 | 43.60 | 0.10 | 0.10 | 6.10 | 0.30 | 0.10 | 0.10 | 11.83 | 0.02 |
| 330 | 410 | 1.46 | 6.03 | 25.07 | 3.92 | 43.58 | 0.10 | 0.12 | 6.52 | 0.30 | 0.08 | 0.10 | 13.30 | 0.00 |
| 410 | 490 | 1.50 | 5.75 | 24.04 | 3.94 | 43.56 | 0.10 | 0.14 | 6.94 | 0.30 | 0.06 | 0.08 | 14.21 | 0.00 |
| 490 | 570 | 1.54 | 5.48 | 23.01 | 3.96 | 43.54 | 0.10 | 0.16 | 7.36 | 0.30 | 0.04 | 0.06 | 15.11 | 0.00 |
| 570 | 650 | 1.58 | 5.21 | 21.97 | 3.98 | 43.52 | 0.10 | 0.18 | 7.78 | 0.30 | 0.02 | 0.04 | 16.02 | 0.00 |
| 650 | 730 | 1.61l | 5.00 | 21.20 | 4.00 | 43.50 | 0.10 | 0.20 | 8.10 | 0.30 | 0.00 | 0.02 | 16.92 | 0.00 |
| 730 | 800 | 1.65 | 4.77 | 20.46 | 4.06 | 43.50 | 0.10 | 0.20 | 8.39 | 0.30 | 0.00 | 0.00 | 17.6 | 0.00 |
| 800 | 900 | 1.68m | 4.60 | 19.90 | 4.10 | 43.50 | 0.10 | 0.20 | 8.60 | 0.30 | 0.00 | 0.00 | 18.23 | 0.00 |
| 900 | 1000 | 1.74 | 4.30 | 18.80 | 4.13 | 43.50 | 0.10 | 0.20 | 9.03 | 0.30 | 0.00 | 0.00 | 18.70 | 0.00 |
| 1000 | 1100 | 1.80 | 4.00 | 17.70 | 4.15 | 43.50 | 0.10 | 0.20 | 9.45 | 0.30 | 0.00 | 0.00 | 19.65 | 0.00 |
| 1100 | 1200 | 1.86 | 3.70 | 16.60 | 4.18 | 43.50 | 0.10 | 0.20 | 9.88 | 0.30 | 0.00 | 0.00 | 20.60 | 0.00 |
| 1200 | max | 1.92n | 3.40 | 15.50 | 4.20 | 43.50 | 0.10 | 0.20 | 10.30 | 0.30 | 0.00 | 0.00 | 21.55 | 0.00 |
| HUL : the lower limit of the image HU value; HUu: the upper limit of the image HU value; alipid data; badipose-adult#3 data; cadipose-adult#2 data; dadipose-adult#1 data; elipoma data from ICRU-46; faverage soft tissue data from ICRU-33; gaverage soft tissue(female) data; haverage soft tissue(male) data from ICRU-44; imuscle data; jskeleton-spongiosa data; kskeleton-vertebral column data; lskeleton-cranium data; mskeleton-mandible data; nskeleton-cortical bone data from ICRU-46. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |