# Appendix 1: Calculation of conversion factor in PCXMC

Siemens ArtisQ Ceiling DynaCT

Parameters in PCXMC:

Avagerage tube voltage: 90 kV

Anode angle: 11.5 degrees

Tube-to-detector distance: 105 cm

Field of view: 30x38 cm (diameter 48 cm)

Area: abdomen, kidney

|  | DAP (mGycm2) | Effective dose (mSv)* |  |
| --- | --- | --- | --- |
| 0 degrees | 250 | 0.023383 |  |
| 90 degrees | 250 | 0.050701 |  |
| 180 degrees | 250 | 0.016885 |  |
| 270 degrees | 250 | 0.082371 |  |
| Total | 1000 | 0.17334 |  |

*ICRP103

This results in a conversion factor of 0.17 mSv Gy-1 cm-2