Supplementary Information

Table of Supplementary Tables

Supplementary Table 1. Capital cost inputs for renewable energy and storage by year	2

Supplementary Table 2. Main parameters for IRA incentives11	3

Supplementary Table 3. Main parameters for carbon capture	3

Supplementary Table 4. Sensitivities of fuel cost to hydrogen cost for all pathways	4

Supplementary Table 5. Main parameter for Power and Biomass to Liquids pathway.	4

Supplementary Table 6. Main parameters for e-fuel production costs for Biomass to Liquids pathway.	5

Supplementary Table 7. Main parameters for e-fuel production costs for Power to Liquids pathways.	6

Supplementary Table 8. US 2030 target horizons: Comparison of least-cost vs. BtL and PBtL pathways	7

Supplementary Table 9. GHG emissions from BtL and PBtL pathways	8

Supplementary Table 10. Assumptions and results for land, water and electricity use for all pathways	9

Table of Supplementary Figures

Supplementary Figure 1. Levelized cost of crude fuel production pathways for the year 2027	10

Supplementary Figure 2. Sensitivities of fuel cost in USD/gal to hydrogen cost for all pathways.	11

Supplementary Figure 3. Sensitivity for PBtL pathway to electrolyzer costs	12

Supplementary Figure 4. Hydrogen output based on wind and solar profiles	13

Notation used throughout the supplementary information:

BtL: Biomass to Liquids

PBtL: Power and Biomass to Liquids

PtL-CC: Power to Liquids using Carbon Capture from point sources

PtL-DAC: Power to Liquids using Carbon Capture from Direct Air Capture

Supplementary Table 1. Capital cost inputs for renewable energy and storage by year

| CAPEX | Solar PV | Wind | H2 Storage | US-PEM - Bloomberg | CC | DAC |
| --- | --- | --- | --- | --- | --- | --- |
|  | ATB, NREL 60 | ATB, NREL 60 | BNEF, 61 | BNEF, 62 | Supplementary Table 4 | Supplementary Table 4 |
| Year | 2022 M USD/MW | 2022 M USD/MW | 2022 USD/kg | 2022 M USD/MW | 2022 USD/t | 2022 USD/t |
| 2023 | 1.17 | 1.42 | 650 | 1.37 | 55 | 400 |
| 2027 | 0.97 | 1.20 | 650 | 0.80 | 55 | 300 |
| 2030 | 0.82 | 1.03 | 578 | 0.52 | 55 | 200 |
| 2035 | 0.78 | 0.98 | 578 | 0.23 | 55 | 200 |
| 2036 | 0.78 | 0.98 | 578 | 0.23 | 55 | 200 |
| 2040 | 0.74 | 0.93 | 578 | 0.16 | 55 | 150 |
| 2045 | 0.71 | 0.88 | 578 | 0.14 | 55 | 150 |
| 2050 | 0.67 | 0.83 | 578 | 0.13 | 55 | 120 |

Supplementary Table 2. Main parameters for IRA incentives11

|  | 2022$ | Term (years) | Present Value | Levelized Value (PxIRA ) | Unit |
| --- | --- | --- | --- | --- | --- |
| Renewable Energy Production Tax Credit (45Y): | 25 | 10 | $176 | 15.1 | USD/MWh |
| Clean Hydrogen Production Tax Credit (45V): | 3 | 10 | $21 | 1.8 | USD/kg |
| Credit for Carbon Capture (45Q): | 60 | 12 | $477 | 40.9 | USD/ton |
| Credit for Direct Air Capture (45Q): | 130 | 12 | $1,033 | 88.6 | USD/ton |

Supplementary Table 3. Main parameters for carbon capture (Costcapture)

|  | IEA45 | Zhou et al.7 | BNEF-low21 | BNEF-high21 | Used* |
| --- | --- | --- | --- | --- | --- |
|  | (2019 USD/ton) | (2020 USD/ton) | (2023 USD/ton) | (2023 USD/ton) | (2022 USD/ton) |
| CC (bioethanol) | 25-35 |  |  |  |  |
| CC (cement) | 60-120 |  |  |  |  |
| CC (power plant) | 50-100 |  |  |  |  |
| CC (point source) |  | 40 |  |  |  |
| CC 2023 (biogenic US) |  |  | 55 |  | 55 |
| CC 2050 (biogenic US) |  |  | 55 |  | 55 |
| DAC |  | 100-700 |  |  |  |
| DAC 2023 |  |  | 287 | 532 | 400 |
| DAC 2030 |  |  | 157 | 235 | 200 |
| DAC 2040 |  |  | 138 | 153 | 150 |
| DAC 2050 |  |  | 100 | 131 | 120 |

* Values used in this study were set within the range of the cited references, either by calculating a mean or selecting a representative value based on discussions with industry experts.

**Implicit values

Supplementary Table 4. Sensitivities of fuel cost (in $/gal) to hydrogen cost for all pathways with biomass at 85 USD/t

| H2 ($/kg) | BtL | PBtL | PtL-CC | PtL-DAC |
| --- | --- | --- | --- | --- |
| 0 | 4.4 | 1.9 | 1.1 | 3.3 |
| 0.5 | 4.4 | 2.8 | 2.5 | 4.7 |
| 1 | 4.4 | 2.9 | 2.6 | 4.8 |
| 1.5 | 4.4 | 3.4 | 3.4 | 5.5 |
| 2 | 4.4 | 3.9 | 4.1 | 6.3 |
| 2.5 | 4.4 | 4.4 | 4.9 | 7.1 |
| 3 | 4.4 | 4.9 | 5.6 | 7.8 |
| 3.5 | 4.4 | 5.4 | 6.4 | 8.6 |
| 4 | 4.4 | 5.9 | 7.2 | 9.3 |
| 4.5 | 4.4 | 6.4 | 7.9 | 10.1 |

Supplementary Table 5. Main parameters for e-fuel production costs for Power and Biomass to Liquids pathway.

| Technology | PBtL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference | Ostadi et al. 201944 | Dossow et al., 202143 | Albrecht, 201723 | Nielsen et al., 202245 | Isaacs, 202119 | Hillestad et al., 201821 | Habermeyer et al., 202320 | Used* |
| Carbon efficiency | 91% | 97% | 98% | 91% | 61% | 91% | 92% | 91% |
| Biomass eff (gal fuel/ton)** |  |  |  | 186 |  |  |  | 184 |
| Fischer-Tropsch energy efficiency |  | 49.9%-53% | 51% | 65-75% |  |  | 45% |  |
| FT efficiency (gal fuel/kgH2) |  | 1.17 |  |  |  | 1.1-0.9 | 0.993 | 1.00 |
| FT CAPEX (USD/kWoutput) |  |  |  |  |  | 3,918 | 1097 |  |
| CAPEX (USD/(galfuel/yr)) |  |  | 2.2 |  |  | 18.0 | 5.0 | 8.5 |
| FT output (%jet) |  |  |  |  |  |  |  |  |
| OPEX |  |  |  |  |  | 4.5% | 6.6% | 6.0% |
| LCA (g/MJ) |  |  |  |  | 4-187 |  | <32.9 |  |
| $/lt |  |  | 2.2 |  | 1.64-1.89 | 1.7 | 1.3 |  |
| $/gal |  |  | 8.13 |  | 6.2-7.1 | 6.43 | 4.99 |  |

Supplementary Table 6. Main parameters for e-fuel production costs for Biomass to Liquids pathway.

| Technology | BtL |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference | S&T Consultants, 201839 | IRENA, 2016 | Dossow et al., 202143 | Albrecht, 201723 | Nielsen et al., 202245 | Isaacs, 202119 | Hillestad et al., 201821 | Cheng et al., 202316 | Used* |
| Carbon efficiency | 33.20% |  | 41% | 24.9% | 38% | 23% | 37.8 |  | 40% |
| Biomass eff (gal fuel/ton) ** | 56 |  | 73 |  | 78 |  |  | 72 | 73 |
| Fischer-Tropsch energy efficiency | 38.50% |  | 50% | 36.3% | 53% |  |  | 49.5% | 50% |
| FT efficiency (gal fuel/kgH2) |  |  |  |  |  |  |  |  |  |
| FT CAPEX (USD/kWoutput) | 3500-5500 | 3000-5000 |  |  |  |  | 7145 | 3826 |  |
| CAPEX (USD/(galfuel/yr)) | 16-25 | 13-23 |  | 4.3 |  |  | 32.8 | 17.6 | 20.0 |
| FT output (%jet) | 38% |  |  |  |  |  |  | 82% |  |
| OPEX |  |  |  |  |  |  |  |  | 6.0% |
| LCA (g/MJ) |  |  |  |  |  | 26-27 |  | 11 |  |
| $/lt |  |  |  | 2.34 |  | 0.77-0.82 | 2.5 | 1.46 |  |
| $/gal |  |  |  | 8.85 |  | 2.9-3.1 | 9.45 | 5.5 |  |

Supplementary Table 7. Main parameters for e-fuel production costs for Power to Liquids pathways.

| Technology | PtL (CC/DAC) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference | Zhou et al., 20227 | Bergero et al., 20229 | Schmidt et al.,201854 | Albrecht, 201723 | BNEF, 202321 | Isaacs, 202119 | de Klerk, 201127 | Cheng et al., 202316 | Brynolf et al., 201733 | Hannula (IEA), 202342 | Used* |
| Carbon efficiency |  |  |  | 98% |  | 96% |  |  |  |  |  |
| Carbon efficiency ratio (kgCO2/kg H2) |  | 7.1 |  |  |  |  |  |  |  |  |  |
| Carbon efficiency (kgCO2/galfuel) |  | 9.8 |  |  |  |  |  |  |  |  | 10.8 |
| Fischer-Tropsch energy efficiency | 73% | 80% | 75% | 82.6% |  |  |  | 68% | 63%-83% | 65%-85% | 73% |
| FT efficiency (gal fuel/kgH2) | 0.66 | 0.72 |  |  | 0.75 |  |  | 0.61 |  |  | 0.66 |
| FT CAPEX (USD/kWoutput) | 450 |  |  |  | 1,469 |  |  | 1004 | 360-840 | 1200 | 1004 |
| CAPEX (USD/(galfuel/yr)) | 2.1 |  |  | 1.95 | 6.7 |  | 2-to-4 | 4.6 | 1.7-3.9 | 5.5 | 5.5 |
| FT output (%jet) | 40% |  |  |  | 50% |  | 35%-64% | 82% |  | 75% |  |
| OPEX |  |  |  |  | 6% |  |  |  |  | 3.0% | 6.0% |
| LCA (g/MJ) |  |  |  |  |  | 2-380 |  |  |  |  |  |
| $/lt | 2.30 |  |  | 2.74 | 3.4-4.2 | 2.89-3.39 |  | 0.78 | 2.25 | 1.75 |  |
| $/gal | 8.7 |  |  | 10.36 | 13-16 | 10.9-12.8 |  | 2.94 | 8.51 | 6.6 |  |

Supplementary Table 8. US 2030 target horizons: Comparison of least-cost vs. BtL and PBtL pathways

| Near-term horizon | Least-cost pathway | BtL Pathway | PBtL Pathway only | Units |
| --- | --- | --- | --- | --- |
| Cost below* | 2.77 | 4.10 | 2.68 | USD/gal |
| Potential production** | 6,009 | 6,016 | 6,000 | Mgal/y |
| RE capacity used | 116 | - | 104 | GW |
| Electricity used | 407 | - | 378 | TWh/y |
| Wind share | 73% | - | 76% |  |
| H2 used | 6,415 | - | 6,000 | Mkg/y |
| Average %RE county | 2.4% | 0.0% | 3.4% |  |
| Max %RE county | 24% | - | 24% |  |
| Biomass use | 32.3 | 82.8 | 32.68 | MMT/y |
| Land use for biomass*** | 2.99 | 7.67 | 3.03 | Mhe |
| Land use for H2**** | 0.22 | - | 0.18 | Mhe |
| Total land use | 3.20 | 7.67 | 3.21 | Mhe |
| Water use for biomass***** | 32,541 | 83,590 | 32,973 | Mm3/y |
| Water use for H2****** | 57.74 | - | 54.00 | Mm3/y |
| Total water use | 32,599 | 83,590 | 33,027 | Mm3/y |
| Emissions******* | 220,795 | 227,397 | 14,999 | MkgCO2e |
| Medium-term horizon******* | Least-cost pathway | BtL Pathway | PBtL Pathway only | Units |
| Cost below* | 2.44 | 4.61 | 2.38 | USD/gal |
| Potential production** | 45,019 | 45,022 | 45,020 | Mgal/y |
| RE capacity used | 1,030 | - | 974 | GW |
| Electricity used | 2,957 | - | 1945 | TWh/y |
| Wind share | 61% | - | 62% |  |
| H2 used | 45,554 | - | 28,621 | Mkg/y |
| Average %RE county | 4.2% | 0.0% | 4.6% |  |
| Max %RE county | 76% | - | 31% |  |
| Biomass use | 293.8 | 595.1 | 245.8 | MMT/y |
| Land use for biomass*** | 27.20 | 55.10 | 22.76 | Mhe |
| Land use for H2**** | 2.29 | - | 1.45 | Mhe |
| Total land use | 29.49 | 55.10 | 24.21 | Mhe |
| Water use for biomass***** | 296,431 | 600,439 | 248,039 | Mm3/y |
| Water use for H2****** | 409.98 | - | 257.59 | Mm3/y |
| Total water use | 296,841 | 600,439 | 248,297 | Mm3/y |
| Emissions******* | 635,446 | 1,626,597 | 112,550 | MkgCO2e |

*All figures in 2022 US dollars for projects coming online in 2027.

**Potential e-fuel production required to meet 2030 and 2050 U.S. sustainable aviation fuel (SAF) goals, assuming that e-fuels constitute 50% and 75% of the total jet fuel share, respectively.

***Switchgrass yield of 10.8 .52

****19 m2/MWh for solar and 0.4 m2/MWh for wind.53

*****1009 m3/ton.51

******9lt/kgH2 stoichimetrical.

*******CO2e emissions are estimated as 37.8 kgCO2e/gal for BtL, 2.5 kgCO2e/gal for PBtL and 263.2 kgCO2e/gal for PtL using carbon capture from ethanol sources.

*******Near-term and medium-term scenarios in this study are based on biomass data from the 2023 Billion-Ton Report13 and CAPEX cost projections for the years 2027 and 2035 as shown in Supplementary Table 4.

Potential emissions reductions

Using corn stover for biomass, the BtL process results in emissions of approximately 5.6 gCO2/MJ. Highlighting a prospective pathway that encompasses electric transport, tractors, and green ammonia suggests the potential to slash these emissions considerably. This pathway could result in emissions reductions of around 66%, bringing BtL process emissions down to roughly 1.2 gCO2/MJ.

Compared to BtL, PBtL biomass output increases by a factor of 2.5 and carbon efficiency increases to 91%, which brings down PBtL emissions by a factor of 14, lowering them to 0.4 g CO2/MJ. This suggests that, compared to jet fuel emissions of 85 g/MJ, an emission reduction of over 99.5% is achievable, and 99.9% for forest residue. Data was obtained from the GREET model and its accuracy was ensured by cross-referencing the GREET model aviation module.37

Supplementary Table 9. GHG emissions from BtL and PBtL pathways

| Present Scenario | Unit | Corn Stover | Forest Residue | Source |
| --- | --- | --- | --- | --- |
| Phosphoric acid (P2O5) | gCO2e/MJ | 0.4 | - | GREET |
| Potassium oxide | gCO2e/MJ | 0.7 | - | GREET |
| Nitrogen | gCO2e/MJ | 1.1 | - | GREET |
| Tractors diesel | gCO2e/MJ | 2.1 | - | GREET |
| Collection and storage | gCO2e/MJ | 0.1 | - | GREET |
| Farming/collection | gCO2e/MJ | 4.5 | 0.1 | GREET |
| Transport | gCO2e/MJ | 1.1 | 3.4 | GREET |
| BtL emissions | gCO2e/MJ | 5.6 | 3.5 | GREET |
| PBtL efficiency | gCO2e/MJ | -5.2 | -3.2 |  |
| PBtL emissions | gCO2e/MJ | 0.4 | 0.2 |  |
| Future Scenario |  |  |  |  |
| BtL Emissions | gCO2e/MJ | 5.6 | 3.5 | GREET |
| Electric transport | gCO2e/MJ | -1.1 | (3.4) | Assumptions |
| Electric tractors | gCO2e/MJ | -2.1 | - |  |
| Green ammonia | gCO2e/MJ | -1.1 | - |  |
| BtL emissions future | gCO2e/MJ | 1.2 | 0.1 |  |
| PBtL efficiency | gCO2e/MJ | (1.15) | (0.07) |  |
| PBtL emissions future | gCO2e/MJ | 0.09 | 0.00 |  |

Supplementary Table 10. Assumptions and results for land, water and electricity use for all pathways

| Assumptions |  |  | Value |  | Unit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Switchgrass yield upland |  |  | 8.752 |  | ton/ha |  |  |  |  |  |
| Switchgrass yield lowland |  |  | 12.952 |  | ton/ha |  |  |  |  |  |
| Switchgrass avg yield |  |  | 10.852 |  | ton/ha |  |  |  |  |  |
| Electricity use per kg of H2 |  |  | 0.063 |  | MWh/kgH2 |  |  |  |  |  |
| Land use for wind |  |  | 0.453 |  | m2/MWh |  |  |  |  |  |
| Land use for solar |  |  | 1953 |  | m2/MWh |  |  |  |  |  |
| Land use for H2 (Wind) |  |  | 0.03 |  | m2/kgH2 |  |  |  |  |  |
| Land use for H2 (Solar) |  |  | 1.20 |  | m2/kgH2 |  |  |  |  |  |
| US RE potential |  |  | 46340050 |  | TWh/y |  |  |  |  |  |
| Water per H2 (lt/kg) |  |  | 9 |  | l/kg |  |  |  |  |  |
| Water Use | Low |  |  | High |  | Used |  |  |
| gal/year-acre | 20,00051 |  |  | 617,00051 |  | 61,000 |  |  |
| lt/ton biomass | 330,99651 |  |  | 10,211,22551 |  | 1,009,538 |  |  |
| Land Use |  | Unit |  |  |  |  | BtL | PBtL |  | PtL-CC/DAC (Wind) |  | PtL-CC/DAC (Solar) |
| Biomass use |  | gal fuel/ton biomass |  |  |  |  | 73 | 184 |  | 0 |  | 0 |
| Land use for biomass |  | m2/gal |  |  |  |  | 12.8 | 5.0 |  | 0 |  | 0 |
| Hydrogen use |  | gal fuel/kg H2 |  |  |  |  | 0 | 1.00 |  | 0.66 |  | 0.66 |
| Land for H2 production |  | m2/gal |  |  |  |  | 0 | 0.03 |  | 0.04 |  | 1.82 |
| Total land use |  | m2/gal |  |  |  |  | 12.8 | 5.1 |  | 0.04 |  | 1.8 |

| Water Use |  |  |  | BtL |  | PBtL | PtL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Water for biomass (lt/gal) |  |  |  | 13,902 |  | 5,499 | - |
| Water per H2 (lt/gal) |  |  |  |  |  | 9 | 14 |
| Total (lt/gal-fuel) |  |  |  | 13,902 |  | 5,508 | 14 |
| Electricity Use for Carbon Capture63 | CC |  | DAC |  | Units |  |  |  |
| Thermal energy |  |  | 2 |  | MWh/tCO2 |  |  |  |
| Electricity | 0.35 |  | 0.5 |  | MWh/tCO2 |  |  |  |
| HP thermal (COP=3) |  |  | 0.67 |  | MWh/tCO2 |  |  |  |
| Total electricity | 0.35 |  | 1.17 |  | MWh/tCO2 |  |  |  |
| Total electricity | 0.004 |  | 0.013 |  | MWh/gal fuel |  |  |  |

| Electricity Use | Unit | BtL | PBtL | PtL-CC | PtL-DAC |
| --- | --- | --- | --- | --- | --- |
| Electricity use for H2 | MWh/gal | - | 0.063 | 0.096 | 0.096 |
| Electricity use for CC/DAC | MWh/gal |  |  | 0.004 | 0.013 |
| Total electricity | MWh/gal | - | 0.063 | 0.100 | 0.108 |
| 35 bn gallon electricity needs | TWh/y | - | 2,756 | 4,355 | 4,741 |
| % of US RE potential |  | 0.0% | 0.6% | 0.9% | 1.0% |

Supplementary Figure 1. Levelized cost of crude fuel production pathways for the year 2027 in the Minnesota 10th percentile location. The waterfall chart illustrates the cost buildup for four production pathways: Biomass-to-Liquid (BtL), Power-and-Biomass-to-Liquid (PBtL), Power-to-Liquid with Carbon Capture (PtL-CC), and Power-to-Liquid with Direct Air Capture (PtL-DAC). Columns represent estimated production costs, including Inflation Reduction Act (IRA) incentives on all studied pathways. We assume that tax credits accrued throughout the supply chain will pass on to final products [12]. The Aviation Fuel Production Tax Credits (40B and 45Z) of 1.75 USD/gal are not considered due to their short timeframe (2023-2027) but their extension could impact these figures significantly. We do not consider potential additional revenues generated from the sale of by-products in either of these pathways, instead focusing on crude fuel production costs to avoid arbitrarities. The resulting e-kerosene can constitute 40% - 80% of the total e-fuel output from this process, with the remainder of the output being heavier or lighter hydrocarbons (e.g., waxes, diesel, and propane).

Supplementary Figure 2.. Sensitivities of fuel cost in USD/gal to hydrogen cost for all pathways with biomass at 85 USD/ton. Below a hydrogen price of 2.7 USD/kg, PBtL becomes economic vs BtL; between 1.5-2.7 USD/kg, PBtL produces at the least cost; and below 2 USD/kg, PBtL becomes competitive with fossil jet fuel (at 4 USD/gal). IRA Production Tax Credit incentives for renewable energy, hydrogen, and carbon capture shown in Supplementary Table 3 are considered.

Supplementary Figure 3.. Sensitivity for PBtL pathway calculations to electrolyzer costs at three representative locations: top site, 10th-percentile site, and 50th-percentile site. Percentiles are weighted by production quantities per county.

Supplementary Figure 4. Hydrogen output based on wind and solar profiles for a 10th-percentile location in Cottonwood County, Minnesota. The HOOPS model relies on off-grid solar, wind, and storage technologies to ensure a continuous and reliable H2 supply while adhering to an hourly target of ± 5% H2 output levels.

# References

- Friedlingstein, P. et al. Global carbon budget 2019. Earth Syst. Sci. Data 11, 1783–1838 (2019).

- U.S. Environmental Protection Agency. Inventory of U.S. Greenhouse Gas Emissions and Sinks: 1990-2020 – Main Text. https://www.epa.gov/ghgemissions/inventory-us-greenhouse-gas-emissions-and-sinks-1990-2020  (2022).

- Davis, S. J. et al. Net-zero emissions energy systems. Science (80-. ). 360, (2018).

- International Energy Agency. Energy Technology Perspectives 2020. Energy Technology Perspectives 2020 https://iea.blob.core.windows.net/assets/7f8aed40-89af-4348-be19-c8a67df0b9ea/Energy_Technology_Perspectives_2020_PDF.pdf (2020) doi:10.1787/ab43a9a5-en.

- Sustainable Aviation Fuel: Agencies Should Track Progress Toward Ambitious Federal Goals [Reissued with Revisions May 17, 2023] | U.S. GAO. https://www.gao.gov/products/gao-23-105300.

- SAF Grand Challenge Roadmap Flight Plan for Sustainable Aviation Fuel.

- Zhou, Y., Searle, S. & Pavlenko, N. Current and future cost of e-kerosene in the United States and Europe. (2022).

- Rutherford, D. Sustainable aviation fuels: What does real leadership look like? International Council on Clean Transportation.

- Bergero, C. et al. Pathways to net-zero emissions from aviation. Nat. Sustain. 2023 64 6, 404–414 (2023).

- International Energy Agency. Chapter 4. Trends to Watch. Is the biofuel industry approaching a feedstock crunch? in Renewables 2022 158 (2022).

- Rep. Yarmuth, J. A. [D-K.-3]. Text - H.R.5376 - 117th Congress (2021-2022): Inflation Reduction Act of 2022. (2022).

- U.S. Department of Energy Office of Energy Efficiency & Renewable Energy. Alternative Aviation Fuels: Overview of Challenges, Opportunities, and Next Steps. https://www.energy.gov/sites/prod/files/2017/03/f34/alternative_aviation_fuels_report.pdf (2017).

- U.S. Department of Energy. (2023). 2023 Billion-Ton Report: Assessment of U.S. Renewable Carbon Resources. Office of Energy Efficiency & Renewable Energy. https://www.energy.gov/eere/bioenergy/2023-billion-ton-report-assessment-us-renewable-carbon-resources

- O'Malley, J. (2024). Assessing the role of biomass-based diesel in U.S. rail decarbonization strategy. International Council on Clean Transportation. https://theicct.org/wp-content/uploads/2024/03/ID-108-%E2%80%93-Rail-biodiesel_final2.pdf

- Langholtz, M.H., Stokes, B.J. and Eaton, L.M. (2016). 2016 Billion-Ton Report: Advancing Domestic Resources for a Thriving Bioeconomy. [online] doi:https://doi.org/10.2172/1271651.

- Cheng, F., Luo, H., Jenkins, J. D. & Larson, E. D. Impacts of the Inflation Reduction Act on the Economics of Clean Hydrogen and Synthetic Liquid Fuels. Environ. Sci. Technol. (2023) doi:10.1021/ACS.EST.3C03063/ASSET/IMAGES/LARGE/ES3C03063_0004.JPEG.

- Brown, Craig. “Biofuel Production and Greenhouse Gas Reduction Potential“, Oct 2023  https://www.nrel.gov/docs/fy24osti/85351.pdf

- “The Role of E-Fuels in Decarbonising Transport – Analysis.” IEA, https://www.iea.org/reports/the-role-of-e-fuels-in-decarbonising-transport . Accessed 26 Feb. 2024 .

- Isaacs, Stewart A., et al. “Environmental and Economic Performance of Hybrid Power-to-Liquid and Biomass-to-Liquid Fuel Production in the United States.” Environmental Science & Technology, vol. 55, no. 12, June 2021, pp. 8247–57. DOI.org (Crossref), https://doi.org/10.1021/acs.est.0c07674.

- Habermeyer, Felix, et al. “Sustainable Aviation Fuel from Forestry Residue and Hydrogen – a Techno-Economic and Environmental Analysis for an Immediate Deployment of the PBtL Process in Europe.” Sustainable Energy & Fuels, vol. 7, no. 17, Aug. 2023, pp. 4229–46. pubs.rsc.org, https://doi.org/10.1039/D3SE00358B.

- BloombergNEF, E-Fuel Touted as Ticket to Clean Aviation Needs a Tailwind, May 8, 2023

- Hillestad, M., et al. “Improving Carbon Efficiency and Profitability of the Biomass to Liquid Process with Hydrogen from Renewable Power.” Fuel, vol. 234, Dec. 2018, pp. 1431–51. ScienceDirect, https://doi.org/10.1016/j.fuel.2018.08.004.

- Albrecht, Friedemann G., et al. “A Standardized Methodology for the Techno-Economic Evaluation of Alternative Fuels – A Case Study.” Fuel, vol. 194, Apr. 2017, pp. 511–26. ScienceDirect, https://doi.org/10.1016/j.fuel.2016.12.003 .

- Jenkins, Jesse D.; Farbes, Jamil; Jones, Ryan; and Mayfield, Erin N. (2022), "REPEAT Project Section-by-Section Summary of Energy and Climate Policies in the 117th Congress," https://docs.google.com/spreadsheets/d/1X2PORZp5JzP2yWbdUSbXphElIGPEOlJNI-T12gz7n1s/edit#gid=1108881515

- Joseph Rand, Rose Strauss, Will Gorman, Joachim Seel, Julie Mulvaney Kemp, Seongeun Jeong, Dana Robson, Ryan Wiser, Queued Up: Characteristics of Power Plants Seeking Transmission Interconnection https://emp.lbl.gov/queues

- Firm Power generation - IEA-PVPS. https://iea-pvps.org/key-topics/firm-power-generation/.

- de Klerk, A. Transport Fuel: Biomass-, Coal-, Gas- and Waste-to-Liquids Processes. Futur. Energy Improv. Sustain. Clean Options Our Planet 199–226 (2020) doi:10.1016/B978-0-08-102886-5.00010-4.

- Tan, E. C. D. et al. Comparative techno-economic analysis and process design for indirect liquefaction pathways to distillate-range fuels via biomass-derived oxygenated intermediates upgrading. Biofuels, Bioprod. Biorefining 11, 41–66 (2017).

- Zhang, H., Wang, L., Van herle, J., Maréchal, F. & Desideri, U. Techno-economic evaluation of biomass-to-fuels with solid-oxide electrolyzer. Appl. Energy 270, 115113 (2020).

- de Klerk, A. Fischer-Tropsch Refining. Fischer-Tropsch Refining (Wiley-VCH, 2011). doi:10.1002/9783527635603.

- Swanson, R.M., Platon, A., Satrio, J.A. and Brown, R.C. (2010). Techno-economic analysis of biomass-to-liquids production based on gasification. Fuel, [online] 89, pp.S11–S19. doi:https://doi.org/10.1016/j.fuel.2010.07.027.

- He, Z. et al. Synthesis of liquid fuel via direct hydrogenation of CO2. Proc. Natl. Acad. Sci. U. S. A. 116, 12654–12659 (2019).

- Brynolf, S., Taljegard, M., Grahn, M. & Hansson, J. Electrofuels for the transport sector: A review of production costs. Renew. Sustain. Energy Rev. 81, 1887–1905 (2018).

- Chan, Y. H., Syed Abdul Rahman, S. N. F., Lahuri, H. M. & Khalid, A. Recent progress on CO-rich syngas production via CO2 gasification of various wastes: A critical review on efficiency, challenges and outlook. Environ. Pollut. 278, 116843 (2021).

- Rezaei, E. & Dzuryk, S. Techno-economic comparison of reverse water gas shift reaction to steam and dry methane reforming reactions for syngas production. Chem. Eng. Res. Des. 144, 354–369 (2019).

- U.S. Gulf Coast Kerosene-Type Jet Fuel Spot Price FOB (Dollars per Gallon). https://www.eia.gov/dnav/pet/hist/eer_epjk_pf4_rgc_dpgD.htm.

- Argonne GREET Model. https://greet.anl.gov/.

- Zang, G., Sun, P., Elgowainy, A., Bafana, A. & Wang, M. Life Cycle Analysis of Electrofuels: Fischer-Tropsch Fuel Production from Hydrogen and Corn Ethanol Byproduct CO2. Environ. Sci. Technol. 2–11 (2021) doi:10.1021/acs.est.0c05893.

- Hydrogen Production: Biomass Gasification | Department of Energy. https://www.energy.gov/eere/fuelcells/hydrogen-production-biomass-gasification.

- S&T Consultants, Description and Data Collection on Biofuel Technologies, 2018 https://ens.dk/sites/ens.dk/files/Analyser/renewable_fuels_technology_catalogue_revised_draft_report.docx

- RECCE: Resource to Climate Comparison Evaluator. https://recce.aiatools.org/. Accessed 26 Feb. 2024.

- Hannula, I. Hydrogen enhancement potential of synthetic biofuels manufacture in the European context: A techno-economic assessment. Energy 104, 199–212 (2016).

- Dossow, M., Dieterich, V., Hanel, A., Spliethoff, H. & Fendt, S. Improving carbon efficiency for an advanced Biomass-to-Liquid process using hydrogen and oxygen from electrolysis. Renew. Sustain. Energy Rev. 152, 111670 (2021).

- Ostadi, M., Rytter, E. & Hillestad, M. Boosting carbon efficiency of the biomass to liquid process with hydrogen from power: The effect of H2/CO ratio to the Fischer-Tropsch reactors on the production and power consumption. Biomass and Bioenergy 127, 105282 (2019).

- Nielsen, Anders S., et al. “Enhancing the Efficiency of Power- and Biomass-to-Liquid Fuel Processes Using Fuel-Assisted Solid Oxide Electrolysis Cells.” Fuel, vol. 321, Aug. 2022, p. 123987. ScienceDirect, https://doi.org/10.1016/j.fuel.2022.123987.

- BNEF, Electrolysis Primer_ An Old Technology Adapts for New Uses _ Full Report _ BloombergN

- Is carbon capture too expensive? – Analysis - IEA. https://www.iea.org/commentaries/is-carbon-capture-too-expensive.

- Simple Levelized Cost of Energy (LCOE) Calculator Documentation | Energy Analysis | NREL. https://www.nrel.gov/analysis/tech-lcoe-documentation.html.

- EPA, Facility Level Information on GreenHouse gases Tool (FLIGHT) https://ghgdata.epa.gov/ghgp/main.do?site_preference=normal

- DOE, 2022, Renewable Energy Resource Assessment Information for the United States https://www.energy.gov/sites/default/files/2022-03/Renewable%20Energy%20Resource%20Assessment%20Information%20for%20the%20United%20States.pdf

- DOE, Billion Ton Report, Chapter 8 https://www.energy.gov/sites/default/files/2017/02/f34/2016_billion_ton_report_volume_2_chapter_8.pdf

- Flowers, 2010, Using Remote Sensing to Assess the In-Season Nitrogen Status of Perennial Ryegrass for Seed Production https://acsess.onlinelibrary.wiley.com/doi/10.2134/agronj2010.0089

- Hannah Ritchie (2022) - “How does the land use of different electricity sources compare?” Published online at OurWorldInData.org. Retrieved from: 'https://ourworldindata.org/land-use-per-energy-source' [Online Resource]

- Schmidt, Patrick, et al. “Power‐to‐Liquids as Renewable Fuel Option for Aviation: A Review.” Chemie Ingenieur Technik, vol. 90, no. 1–2, Jan. 2018, pp. 127–40. DOI.org (Crossref), https://doi.org/10.1002/cite.201700129.

- “Incentives for Clean Hydrogen Production in the Inflation Reduction Act.” Resources for the Future, https://www.rff.org/publications/reports/incentives-for-clean-hydrogen-production-in-the-inflation-reduction-act/. Accessed 26 Feb. 2024.

- Innovation Outlook Advanced Liquid Biofuels. 1 Oct. 2016, https://www.irena.org/publications/2016/Oct/Innovation-Outlook-Advanced-Liquid-Biofuels  .

- Low Carbon Fuel Standard https://ww2.arb.ca.gov/es/our-work/programs/low-carbon-fuel-standard

- FY 2023-23, New Sustainable Aviation Fuel Purchase Credit Enacted. https://tax.illinois.gov/research/news/fy-2023-23--new-sustainable-aviation-fuel-purchase-credit-enacte.html . Accessed 26 Feb. 2024.

- Bossel, Ulf. “Does a Hydrogen Economy Make Sense?” Proceedings of the IEEE, vol. 94, no. 10, Oct. 2006, pp. 1826–37. IEEE Xplore, https://doi.org/10.1109/JPROC.2006.883715 .

- Index | Electricity | 2022 | ATB | NREL. https://atb.nrel.gov/electricity/2022/index . Accessed 26 Feb. 2024.

- Bloomberg New Energy Finance, Hydrogen: The Economics of Storage.

- Bloomberg New Energy Finance, Electrolysis System Capex by 2050 - Updated Forecast

- Viebahn, P., Scholz, A. & Zelt, O. The Potential Role of Direct Air Capture in the German Energy Research Program—Results of a Multi-Dimensional Analysis. Energies 12, 3443 (2019). https://doi.org/10.3390/en12183443

- IEA Greenhouse Gas R&D Programme. (2011). Potential for Improvement in Gasification Combined Cycle Power Generation with CO2 Capture. https://ieaghg.org/docs/General_Docs/Reports/2011-02.pdf