Supplementary Materials for

Laser Interferometry for High-Speed Railway Health Inspection using Telecom Fiber along the Line

Guan Wang,1,2,3,† Dongqi Song,1,2,3,‡ Zhongwang Pang,1,2,3 Fangmin Wang,1,2,3 Hongfei Dai,1,2,3 Wenlin Li,1,2,3 Bo Wang,1,2,3,*

1, Department of Precision Instrument, Tsinghua University, No.1 Qinghua Garden, Chengfu Road, Haidian District, Beijing, P.R.China, 100084.

2, State Key Laboratory of Precision Space-time Information Sensing Technology, No.1 Qinghua Garden, Chengfu Road, Haidian District, Beijing, P.R.China, 100084.

3, Key Laboratory of Photonic Control Technology (Tsinghua University), Ministry of Education, Beijing, P.R.China, 100084.

*corresponding author: bo.wang@tsinghua.edu.cn,

†, ‡: These authors contributed equally to this work.

Carriage passage frequency analysis

After analyzing the time-frequency curve and obtain the overall passage speed of the High-Speed train, we focus on vibrations occurring at each typical section to reveal the relation between vibration spectrum and HSR health condition.

We first notice that after the train leaves the sensing rail (out of Dujiakan Bridge), vibrations excited by carriage passage in the next 2 km railway will still be detected by laser interferometer (shown as vibrations during 329 ~ 400 s). The detailed waveform and frequency spectrum are shown as Fig. S1a and S1b. Since high frequency vibrations attenuate rapidly, only fundamental frequency component of carriage passage reaches the sensing fiber cable and appear as a time-attenuating curve. The waveform is clear enough to interpret even in time domain: the loading and unloading of a 25-m carriage induce vibration with period of s (frequency Hz), and it is modulated by 5-second envelope, which is shown as red curve in Fig. S1a and corresponds to the 8-carriage train with total length of 200 m (s, Hz). One step further, we can find that the period of this red envelope is not constant. It increases from 329 s to 347 s, and

Figure. S1. The carriage passage vibrations caused by trains out of sensing rail line. (a) The vibration waveform of 8-carriage High-Speed train. Vibration envelope of the whole train passing is shown as red curve. (b) The frequency spectrum of the 8-carriage train vibration. (c) The vibration waveform of 16-carriage High-Speed train. (d) The frequency spectrum of the 16-carriage train vibration.

decreases after 347 s. The reason is the speed of train changes during the time, which can also be observed in Fig. 3d in main text. At around 347 s, the train passes by the conjunction point, thus it should slow down before 347 s and speed up afterwards.

The precise parameters can be determined in frequency domain. In Fig. S1b, there are two main peaks at  Hz and  Hz. Due to the speed changing in this process, the frequency of train modulation envelope (red curve) will change accordingly, which makes it a non-ideal amplitude modulation case. These two frequency components correspond to carrier frequency and upper sideband frequency. That’s to say, the precise carriage frequency is  Hz, and the train modulation frequency is  Hz, which are both in line with the estimated value. What’s more, we can also inverse the average speed of train from precise carriage frequency , and obtain  m/s. According to the real speed curve of train in Fig. 3d in main text, this deduction is also reasonable.

The conclusion can be further proved in Fig. S1c and S1d, when a 16-carriage train is detected. In frequency domain, two main peaks are at  Hz and  Hz. It corresponds to the carriage frequency of  Hz, which is in line with that of 8-carriage case, and train modulation frequency turns into  Hz, which is half of 8 carriage case due to the longer train body. The frequency analysis of carriage passage is the basis of establishing the relationship between the vibration spectrum and health condition of HSR system. It gives a simple description of the train loading-unloading process, which is a solid demonstration of the analysis.