Substitution and carbon storage impacts of harvested wood products - Effects of increased cascading with different market responses

Janni Kunttu1*, Elias Hurmekoski2, Tanja Myllyviita3, Antti Kilpeläinen4,5

1European Forest Institute, Yliopistokatu 6, 80100, Joensuu, Finland

2Department of Forest Sciences, University of Helsinki, Latokartanonkaari 7, 00790, Helsinki, Finland

3Finnish Environment Institute, Latokartanonkaari 11, 00790 Helsinki, Finland

4School of Forest Sciences, University of Eastern Finland, Yliopistokatu 7, 80100, Joensuu, Finland

5Finnish Environment Institute, Yliopistokatu 6 B, 80100 Joensuu, Finland

*Corresponding author email: janni.kunttu@efi.int

Appendix B

Table 1. General assumptions for GHG reductions and product-specific substitution. Table originally presented in the supplementary material of Kunttu et al. 2020 (article IV).

| Product | Assumption | Source |  |
| --- | --- | --- | --- |
| All | Emissions from energy production will decrease by 80% by 2050 (compared to 2015 levels) | Haller et al. 2012 |  |
| Wood-concrete hybrid | 1 t of wood-concrete hybrid substitutes for 0.17 t of concrete |  |  |
| Pyrolysis oil | 1 t of wood-based pyrolysis oil substitutes for 0.37 t of heavy fuel oil | Karvonen et al. 2018 |  |
| Platform chemical (adipic acid) | 1 t of wood-based adipic acid substitutes for 1 t of fossil alternative | Aryapratama and Janssen 2017 |  |

Table 3. Indicative displacement factors (DFs) in 2016 and 2050. Table originally presented in the supplementary material of Kunttu et al. 2020 (article IV).

|  |  |  |  |  | Share of emission from energy (of total emissions) |  |  |  |  | Carbon storage (share of carbon) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intermediate product | End use |  | DF |  | Wood product |  |  | Substituted product |  | Wood product | Substituted product |
| Sawnwood | Furniture | 2016 | 0.91 |  | 0.682 |  |  | 0.752 |  | 0.45 | 0 |
|  |  | 2050 | 0.12 |  |  |  |  |  |  |  |  |
|  | Construction | 2016 | 1.13 |  | 0.682 |  |  | 0.782 |  |  | 0 |
|  |  | 2050 | 0.821 |  |  |  |  |  |  |  |  |
|  | Packaging (pallets) | 2016 | 1.14 |  | 0.622 |  |  | 0.755 |  | 0.45 | 0 |
|  |  | 2050 | -0 |  |  |  |  |  |  |  |  |
|  | Other | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
| Panels | Construction | 2016 | 1.13 |  | 0.682 |  |  | 0.782 |  | 0.45 | 0 |
|  |  | 2050 | 0.74 |  |  |  |  |  |  |  |  |
|  | Other | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
| Mechanical pulp | Newsprint | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
|  | Graphic paper (WCUC. WCC) | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
|  | Packaging (carton boards) | 2016 | 1.41 |  | 0.622 |  |  | 0.754 |  | 0.45 | 0 |
|  |  | 2050 | 0.3 |  |  |  |  |  |  |  |  |
| Semi-chemical pulp | Packaging (carton boards. sack paper) | 2016 | 1.41 |  | 0.622 |  |  | 0.754 |  | 0.45 | 0 |
|  |  | 2050 | 0.3 |  |  |  |  |  |  |  |  |
| Dissolving and kraft pulp | Graphic papers (WCC. WCF. WCUC. WFUC) | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
|  | Hygienic papers (tissue. toilet. etc.) | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
|  | Packaging (carton boards. sack paper) | 2016 | 1.41 |  | 0.622 |  |  | 0.754 |  |  | 0 |
|  |  | 2050 | 0.3 |  |  |  |  |  |  |  |  |
|  | Textiles (viscose) | 2016 | 45 | 0.882 |  |  |  | 0.882 |  |  | 0 |
|  |  | 2050 | 1.2 |  |  |  |  |  |  |  |  |
|  | Other | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
| Energy | CHP | 2016 | 0.76 | 0.902 |  |  |  | 0.902 |  |  | 0 |
|  |  | 2050 | 0.14 |  |  |  |  |  |  |  |  |
|  | Mill energy | 2016 | 0 |  |  |  |  |  |  |  | 0 |
|  |  | 2050 | 0 |  |  |  |  |  |  |  |  |
| Diesel | Transport fuel | 2016 | 0.637 |  |  | 0.952 |  | 0.952 |  | 0.45 | 0 |
|  |  | 2050 | 0.16 |  |  |  |  |  |  |  |  |
| Ethanol | Transport fuel | 2016 | 0.72 |  |  | 0.52 |  | 0.382 |  | 0.35 | 0 |
|  |  | 2050 | 0.52 |  |  |  |  |  |  |  |  |
| Ethylene | Packaging (PE, PET) | 2016 | 1.31 |  |  | 0.622 |  | 0.754 |  | 0.45 | 0 |
|  |  | 2050 | 0.82 |  |  |  |  |  |  |  |  |
| Pyrolysis oil | Replacing heavy oil | 2016 | 0.338 | 0.952 |  |  |  | 0.952 | 0.45 |  | 0 |
|  |  | 2050 | 0.08 |  |  |  |  |  |  |  |  |
| Wood-plastic composites* | Construction | 2016 | 7.38 | 9.2 |  | 0.739.11 |  | 0.76 2 | 0.21 |  | 0 |
|  |  | 2050 | 3.2 |  |  |  |  |  |  |  |  |
|  | Plastic components for cars/furniture | 2016 | 7.38 | 9.2 |  | 0.719.11 |  | 0.754 |  | 0.25 | 0 |
|  |  | 2050 | 3.2 |  |  |  |  |  |  |  |  |
| Wood-concrete hybrids | Construction | 2016 | 1.18 |  |  | 0.24*Expert estimate based on private company information |  | 0.30 |  | 0.13 | 0 |
|  |  | 2050 | 0.9 |  |  |  |  |  |  |  |  |
| Some platform chemicals* | Adipic acid | 2016 | 4710 |  |  | 0.9510 |  | 0.9510 |  | 0.49 | 0 |
|  |  | 2050 | 11.5 |  |  |  |  |  |  |  |  |

*Chemicals and mixed material composites are assumed to be based on by-products originating from primary production, thus emissions from the begin of the value chain are excluded in those and the DF impact is higher

1 Knauf et al. 2105

2Ecoinvent

3Vares et al. 2017.

4 Material Economics 2018

5 Rüter et al. 2016

6Soimakallio et al. 2016

7Bright et al. 2010

8Karvonen et al. 2018

9 Judl et al. 2016

10Aryapratama and Janssen 2017

11Ovaskainen 2012

Table 3. Assumed recycling targets for raw materials. Recycling rate indicates the recyclable share of the total production. Table originally presented in the supplementary material of Kunttu et al. 2020 (article IV).

| Raw material | Recycled raw material in 2050 | Assumption |
| --- | --- | --- |
| Glass | 88% | Assumed to grow linearly from 2018 target presented in article of The Finnish Packaging Association (2018) |
| Plastics | 68% | Assumed to grow linearly from 2018 target presented in article of The Finnish Packaging Association (2018) |
| Steel | 100% | Assumed to be 100% (European Steel Association, 2013) |
| Glass wool | 100% | European Insulation Manufacturers Association (2018) |
| Rock wool | 100% | European Insulation Manufacturers Association (2018) |
| Polystyrene insulation | 68% | Assumed to be in the same level with plastics |
| Gypsum board | 100% | New technologies assumed to be adopted by 2020, which enable 100% recycling rate (MTV Uutiset, 2015) |

References

Aryapratama, R., Janssen, M. 2017. Prospective life cycle assessment of bio-based adipic acid production from forest residues. Journal of Cleaner Production.164; 434-443.

Bright, R. M., Strømman. A. H., Hawkins, T. R. 2010. Environmental Assessment of Wood‐Based Biofuel Production and Consumption Scenarios in Norway. Journal of Industrial Ecology. 14: 422-439. doi:10.1111/j.1530-9290.2010.00249.x

European Insulation Manufacturers Association. (2018). Insulation: Infinitely Recyclable. Retrieved January 20, 2020, from https://www.eurima.org/sustainable-construction/recycling.html

European Steel Association. (2013). A Steel Roadmap for A Low Carbon Europe. Eurofer. Brussels. Retrieved from http://www.nocarbonnation.net/docs/roadmaps/2013-Steel_Roadmap.pdf

(IPCC) Pingoud, K., Skog, K.E., Martino, D.L., Tonosaki, M., Xiaoquan, Z., Ford-Robertson, J., 2006. IPCC Guidelines for National Greenhouse Gas Inventories: Volume 4 Agriculture, Forestry and Other Land Use.

Material Economics 2018. The Circular Economy a Powerful Force for Climate Mitigation Transformative innovation for prosperous and low-carbon industry

https://www.sitra.fi/julkaisut/circular-economy-powerful-force-climate-mitigation/

(MTV Uutiset) Mäki-Petäjä, P. (2015, February 25). Kipsilevyille uusi kierrätysmenetelmä - keräysjärjestelmä puuttuu [A new recycling method for gypsum board - The waste collection system is missing]. Retrieved January 20, 2020, from https://www.mtvuutiset.fi/artikkeli/kipsilevyille-uusi-kierratysmenetelma-keraysjarjestelma-puuttuu/4819450#gs.t196bj

Frischknecht, R., Jungbluth, N., Althaus, H.J., Doka, G., Dones, R., Heck, T., Hellweg, S., Hischier,. R., Nemecek, T., Rebitzer,G. et al.. 2005. The Ecoinvent Database: Overview and Methodological Framework (7 pp). Int. J. Life Cycle Assess. 10. 3–9.

Judl, J., Manninen. K., Dahlbo, H., Väntsi, O. 2016 Wood-polymer composites: a step towards

circular economy? SUM 2016 – 3rd Symposium on Urban Mining and Circular Economy

24th May

2016 Available at: http://arvifinalreport.fi/content/plastic-recycling

Karvonen, J., Kunttu, J., Suominen, T., Kangas, J., Leskinen, P., Judl, J. 2018.  Integrating fast pyrolysis reactor with combined heat and power plant improves environmental and energy efficiency in bio-oil production. Journal of Cleaner Production 183: 143-152.

Kunttu J. Wood utilization scenarios and their sustainability impacts in Finland. Sciece FS of F, editor. Diss. For. Finnnish Society of Forest Sciece; 2020.

Knauf, M., Köhl, V., Mues, K,. Olschofsky, Frühwald, A.  2015. Modeling the CO 2-effects

of forest management and wood usage on a regional basis. Carbon Balance and Management

10(1): 13.

Ovaskainen, M. (2012). Carbon Footprint Assessment of UPM Formi. VTT Research Report VTT-R-04983-12. Retrieved from https://www.upmformi.com/siteassets/documents/formi/upm-formi-lca-handout-en.pdf

Rüter, S., Werner, F., Forsell, N., Prins. C., Via, E., Levet,. A. L. 2016. ClimWood2030. Climate benefits of material substitution by forest biomass and harvested wood products: Perspective 2030 - Final

Report. p. 142. Braunschweig: Johann Heinrich von Thünen-Institut.

Soimakallio. S., Saikku, L., Valsta, L., Pingoud, K. 2016. Climate Change Mitigation Challenge

746 for Wood Utilization. The Case of Finland. Environmental Science & Technology 50(10):

747 5127–5134.

The Finnish Packaging Association. (2018). Kierrätystavoitteet kasvavat [The recycling targets are growing]. Retrieved January 20, 2020, from https://www.pakkaus.com/kierratystavoitteet-kasvavat/. Originally published in Pakkaus-lehti 2018/2.

Vares, S. T., Häkkinen, S., Vainio, T. 2017. Rakentamisen hiilivarasto.

759 http://www.ym.fi/download/noname/%7B2859F537-ECD2-479D-A62B-

760 F13AD75403F2%7D/136827