2.2.1 PM2.5, BC and Elements

Table S1. The reference, measurement concentrations and Detection limitation of  standard samples

| elements（chemical compound/(ng/cm2) | reference concentration/(ng/cm2) | measurement concentration/(ng/cm2) | Detection limitation/(ng/cm2) |
| --- | --- | --- | --- |
| M(MgF2) | 19624 | 18466 | 25.5 |
| Al | 23100 | 23953 | 21.9 |
| Si(SiO) | 9557 | 9676 | 19.9 |
| S(CuS) | 12700 | 11809 | 24.2 |
| Cl(KCl) | 8464 | 8023 | 23.1 |
| K(KCl) | 9336 | 9415 | 17.8 |
| Ca(CaF2) | 9240 | 9335 | 10.6 |
| Ti | 45500 | 44855 | 7 |
| Mn | 24300 | 23931 | 5.2 |
| Fe | 19100 | 19269 | 5 |
| Cu(CuS) | 38600 | 32596 | 11.1 |
| Zn(ZnTe) | 15826 | 14529 | 10.5 |
| Br(CsBr) | 18098 | 20931 | 11 |
| Cr | 18400 | 16883 | 57.7 |
| Pb | 50600 | 45382 | 113 |

3.2 Common elements

Figure S1. The concentration temporal trends of Cr, Cu, Pb, Ni and soil elements.

3.2 Element In

Only Element In were considered among the samples from 2017.1 to 2018.6, because some samples had been analyzed by PIXE, and some representative membranes, e.g. the samples collected from pollution days, were re-analyzed by NAA. There was no good time continuity. In addition, NAA is significantly sensitive for element In, therefore, this element was only discussed in the part.

Table S2 Parameters of NAA

| NAA(short-irradiation) |  |
| --- | --- |
| Analysis equipment | CMRR*,CAEP* |
| Neutron Flux | 7.8×1013 n.cm-2·s-1 |
| Comparator, F | Fe, Zr |
| Monitor | Mn |
| Detector | HPGe* |
| Certified reference materials | NIST SRM 2703 (sediment,5 mg) |
| Analysis software | ADVNAA(Ni, et al., 2000) |
| Measured elements | Ba, Ti, I, Br, Mn, Mg, Cu, In, Na, V, K, Cl, Al, Ca and S |

Note:

More details of parameters can refer to Yang, et al.(2019)

CMRR*:China Mianyang Research Reactor

CAEP*:China Academy of Engineering Physics

HPGe*: High Purity Germanium gamma detector, the energy resolution to 60Co(1332.5 keV) of 1.75 keV, the relative efficiency of 48%.

3.3 Source apportionment

Table.S3. The statistics of elements in PM2.5 in 2007 and 2016 to 2017

|  | 2007 |  |  |  |  | 2016-2017 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Species | Max | Min | Mean | S/N |  | Max | Min | Mean | S.D. | S/N |
| Mg | 1749 | 101 | 373 | 2.3 |  | 579 | 28.2 | 222 | 115 | 5.3 |
| Al | 5144 | 137 | 1083 | 9.2 |  | 1581 | 71.1 | 594 | 338 | 9.3 |
| Si | 12480 | 254 | 2236 | 10 |  | 4086 | 171 | 1449 | 898 | 10 |
| S | 15243 | 77.2 | 3912 | 9.8 |  | 9856 | 330 | 2425 | 2267 | 10 |
| Cl | 2753 | 100 | 757 | 8.7 |  | 5361 | 32.7 | 1475 | 1074 | 6.1 |
| K | 4067 | 30.6 | 1304 | 9.8 |  | 1470 | 158 | 570 | 280 | 9.8 |
| Ca | 4389 | 158 | 1311 | 10 |  | 2491 | 188 | 833 | 470 | 10 |
| Ti | 378 | 3.45 | 62.6 | 2.9 |  | 105 | 2.79 | 42.1 | 23.9 | 3.7 |
| Cr | 51.3 | 2.04 | 11.1 | 0.1 |  | 34.5 | 1.56 | 11.7 | 7.46 | 0.2 |
| Mn | 202 | 2.28 | 52.5 | 2.6 |  | 63.2 | 2.73 | 25.0 | 12.7 | 1.9 |
| Fe | 3062 | 70.4 | 849 | 10 |  | 1324 | 181 | 588 | 261 | 10 |
| Cu | 176 | 5.28 | 25.3 | 0.6 |  | 88.2 | 3.92 | 20.2 | 15.0 | 0.7 |
| Zn | 945 | 6.06 | 206 | 6.6 |  | 781 | 0.93 | 238 | 195 | 7.1 |
| Br | 86.0 | 14.4 | 28.1 | 0.4 |  | 116 | 21.8 | 57.7 | 15.7 | 0.8 |
| Pb | 436 | 27.9 | 104 | 0.4 |  | 308 | 31.4 | 99.5 | 46.4 | 0.4 |
| BC | 12451 | 417 | 5244 | 6.3 |  | 12324 | 504 | 4219 | 2379 | 7.4 |

*Note: unit: ng/m3

(a)                                              (b)

Figure S2. Correlation between measured valued and PMF predicted: (a)2007 (b)2016-2017

Reference

Ni B F, et al., 2000. Automation and Computerrization of NAA. Journal of  Radioanalytical Nuclear Chemistry. 244: 665~668

Yang J. K.,  Liu G. P., Yao Y.G., Xiao, C. J., Lin, Q. C., Tang, C. J., Wang, P. S., Jin, X. C., Ni, B. F., Cui, D. Q., 2019. Elemental analysis of PM2.5 using PIXE and NAA in Xinzhen, Beijing[J]. Journal of Radioanalytical and Nuclear Chemistry, 323(11).