Appendix No. 1: Infiltration Analysis

Field infiltration rate data calculation

The rate of water moving into the soil in each experimental area is the same as:

I = Q / t ............................... i

I = infiltration rate

Q = volume of water enters into the soil

A = an experimental area exposed in filed which is constant as 1

Soil unit 1 – Infiltration rate measured at Gungu-West (Augie fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 13.0 | 2.0 | 2.0 |
| 3 | 13.0 | 2.0 | 0.0 |
| 5 | 12.1 | 2.9 | 0.9 |
| 10 | 11.9 | 3.1 | 0.2 |
| 20 | 11.5 | 3.5 | 0.4 |
| 30 | 11.0 | 4.0 | 0.5 |
| 45 | 10.0 | 5.0 | 1.0 |
| 60 | 9.8 | 5.2 | 0.2 |
| 90 | 8.5 | 6.5 | 1.3 |
| 120 | 6.4 | 8.6 | 2.1 |
| t = 120 |  |  | Q = 8.6 cm I = Q / t = 8.6 / 120 = 0.072 cm/min Or 0.072 x 60 = 4.3 cm/hr |

Soil unit 2 – Infiltration rate measured at Gyadaye-East (Argungu fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 12.2 | 2.8 | 2.8 |
| 3 | 11.6 | 3.4 | 0.6 |
| 5 | 11.4 | 3.6 | 0.2 |
| 10 | 10.1 | 4.9 | 1.3 |
| 20 | 9.4 | 5.6 | 0.7 |
| 30 | 9.1 | 5.9 | 0.3 |
| 45 | 8.4 | 6.6 | 0.7 |
| 60 | 8.0 | 7.0 | 0.4 |
| 90 | 7.7 | 7.3 | 0.3 |
| 120 | 5.5 | 9.5 | 2.2 |
| t = 120 |  |  | Q = 9.5 cm I = Q/t = 9.5 / 120 = 0.079 cm/min Or 0.079 x 60 = 4.8 cm/hr |

Soil unit 3: Matanfada –north (Argungu fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 14.1 | 0.9 | 0.9 |
| 3 | 11.9 | 3.1 | 2.2 |
| 5 | 11.0 | 4.0 | 0.9 |
| 10 | 10.6 | 4.4 | 0.4 |
| 20 | 9.5 | 5.5 | 1.1 |
| 30 | 9.0 | 6.0 | 0.5 |
| 45 | 8.8 | 6.2 | 0.2 |
| 60 | 8.5 | 6.5 | 0.3 |
| 90 | 7.9 | 7.1 | 0.6 |
| 120 | 7.4 | 7.6 | 0.5 |
| t = 120 |  |  | Q = 7.6 cm I = Q/t = 7.6 / 120 = 0.063 cm/min Or 0.063 x 60 = 3.8 cm/hr |

Soil unit 4: Kuruma north-west (Argungu fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 13.6 | 1.4 | 1.4 |
| 3 | 12.5 | 2.5 | 1.1 |
| 5 | 12.0 | 3.0 | 0.5 |
| 10 | 11.1 | 3.9 | 0.9 |
| 20 | 10.8 | 4.2 | 0.3 |
| 30 | 10.1 | 4.9 | 0.7 |
| 45 | 10.0 | 5.0 | 0.1 |
| 60 | 9.7 | 5.3 | 0.3 |
| 90 | 8.3 | 6.7 | 1.4 |
| 120 | 7.2 | 7.8 | 1.1 |
| t = 120 |  |  | Q = 7.8 cm I = Q/t = 78 / 120 = 0.065 cm/min Or 0.065 x 60 = 3.9 cm/hr |

Soil unit 5: Gyadaye – South (Argungu fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 14.2 | 0.8 | 0.8 |
| 3 | 11.1 | 3.9 | 3.1 |
| 5 | 10.8 | 4.2 | 0.3 |
| 10 | 9.5 | 5.5 | 1.3 |
| 20 | 9.0 | 6.0 | 0.5 |
| 30 | 8.4 | 6.6 | 0.6 |
| 45 | 8.0 | 7.0 | 0.4 |
| 60 | 7.6 | 7.4 | 0.4 |
| 90 | 7.1 | 7.9 | 0.5 |
| 120 | 6.8 | 8.2 | 0.3 |
| t = 120 |  |  | Q = 8.2 cm I = Q/t = 8.2 / 120 = 0.07 cm Or 0.068 x 60 = 4.1 cm/hr |

Soil unit 6: Zauro – North (Birnin Kebbi fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15.0 | 0.0 | 0.0 |
| 1 | 13.2 | 1.8 | 1.8 |
| 3 | 13.0 | 2.0 | 0.2 |
| 5 | 12.3 | 2.7 | 0.7 |
| 10 | 11.6 | 3.4 | 0.7 |
| 20 | 11.1 | 3.9 | 0.5 |
| 30 | 10.0 | 5.0 | 1.1 |
| 45 | 9.8 | 5.2 | 0.2 |
| 60 | 8.6 | 6.4 | 1.2 |
| 90 | 7.1 | 7.9 | 1.5 |
| 120 | 6.2 | 8.8 | 0.9 |
| t = 120 |  |  | Q = 8.8 cm I = Q/t = 8.8 / 120 = 0.073 cm/min Or 0.073 x 60 = 4.4 cm/hr |

Soil unit 7: Zauro – Central (Birnin Kebbi fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 12.1 | 2.9 | 2.9 |
| 3 | 10.2 | 4.8 | 1.9 |
| 5 | 9.6 | 5.4 | 0.6 |
| 10 | 8.4 | 6.6 | 1.2 |
| 20 | 8 | 7.0 | 0.4 |
| 30 | 7.5 | 7.5 | 0.5 |
| 45 | 6.4 | 8.6 | 1.1 |
| 60 | 5.2 | 9.8 | 1.2 |
| 90 | 3.1 | 11.9 | 2.1 |
| 120 | 1.1 | 13.9 | 2.0 |
| t = 120 |  |  | Q = 13.9 cm I = Q/t = 13.9 / 120 = 0.116 cm Or 0.116 x 60 = 6.9 cm/hr |

Soil unit 8: Gungu – South-south (Augie fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 8.6 | 6.4 | 6.4 |
| 3 | 8.1 | 6.9 | 0.5 |
| 5 | 8.0 | 7.0 | 0.1 |
| 10 | 7.5 | 7.5 | 0.5 |
| 20 | 6.2 | 8.8 | 1.3 |
| 30 | 3.9 | 11.1 | 2.3 |
| 45 | 3.1 | 11.9 | 0.8 |
| 60 | 2.6 | 12.4 | 0.5 a = 12.4 |
| 90 | *6.6 | 8.4 | 8.4 |
| 120 | 3.4 | 11.6 | 3.2 b = 11.6 |
| t = 120 |  |  | Q = 12.4 + 11.6 = 24.0 cm I = Q/t = 24.0 / 120 = 0.20 cm/min Or 0.20 x 60 = 12.0 cm/hr |

Soil unit 9: Zauro – East (Birnin Kebbi fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 13.6 | 1.4 | 1.4 |
| 3 | 11.5 | 3.5 | 2.1 |
| 5 | 9.8 | 5.2 | 1.7 |
| 10 | 7.6 | 7.4 | 2.2 |
| 20 | 5.2 | 9.8 | 2.4 |
| 30 | 4.5 | 10.5 | 0.7 |
| 45 | 3.5 | 11.5 | 1.0 |
| 60 | 2.9 | 12.1 | 0.6 |
| 90 | 2.7 | 12.3 | 0.2 |
| 120 | 2.5 | 12.5 | 0.2 |
| t = 120 |  |  | Q = 12.5 cm I = Q/t = 12.5 / 120 = 0.104 cm/min Or 0.104 x 60 = 6.25 cm/hr |

Soil unit 10: Zauro – South (Birnin Kebbi fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 14.8 | 0.2 | 0.2 |
| 3 | 14 | 1.0 | 0.8 |
| 5 | 13.9 | 1.1 | 0.1 |
| 10 | 13.8 | 1.2 | 0.1 |
| 20 | 13.5 | 2.5 | 0.3 |
| 30 | 13.2 | 2.7 | 0.3 |
| 45 | 12.4 | 2.6 | 0.8 |
| 60 | 11.8 | 4.2 | 0.6 |
| 90 | 10.9 | 4.1 | 0.9 |
| 120 | 10.2 | 4.8 | 0.7 |
| t = 120 |  |  | Q = 4.8 cm I = Q/t = 4.8 / 120 = 0.04 cm/min Or 0.04 x 60 = 2.4 cm/hr |

Soil unit 11: Augie – East (Augie fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 14.7 | 0.3 | 0.3 |
| 3 | 14.1 | 0.9 | 0.6 |
| 5 | 13.9 | 1.1 | 0.2 |
| 10 | 13.2 | 1.8 | 0.7 |
| 20 | 12 | 3.0 | 1.2 |
| 30 | 11 | 4.0 | 1.0 |
| 45 | 9.5 | 5.5 | 1.5 |
| 60 | 8.5 | 6.5 | 1.0 |
| 90 | 5.5 | 9.5 | 3.0 |
| 120 | 3.5 | 11.5 | 2.0 |
| t = 120 |  |  | Q = 11.5 cm I = Q/t = 11.5 / 120 = 0.096 cm/min Or 0.096 x 60 = 5.75 cm/hr |

Soil unit 12: Zauro – West (Birnin Kebbi fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 14.5 | 0.5 | 0.5 |
| 3 | 14 | 1.0 | 0.5 |
| 5 | 13.6 | 1.4 | 0.4 |
| 10 | 13.4 | 1.6 | 0.2 |
| 20 | 13 | 2.0 | 0.4 |
| 30 | 12.1 | 2.9 | 0.9 |
| 45 | 11.5 | 3.5 | 0.6 |
| 60 | 10.7 | 4.3 | 0.8 |
| 90 | 9.8 | 5.2 | 0.9 |
| 120 | 6.7 | 8.3 | 3.1 |
| t = 120 |  |  | Q = 8.3 cm I = Q/t = 8.3 / 120 = 0.069 cm/min Or 0.069 x 60 = 4.15 cm/hr |

Soil unit 13: Natsini–Gyadaye South (Argungu fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 8.1 | 6.9 | 6.9 |
| 3 | 4.1 | 10.9 | 4.0 |
| 5 | 2.4 | 12.6 | 1.7 |
| 10 | 1.5 | 13.5 | 0.9 |
| 20 | 0.5 | 14.5 | 1.0 a = 14.5 |
| 30 | *13.8 | 1.2 | 1.2 |
| 45 | 12.9 | 2.1 | 0.9 |
| 60 | 12.7 | 2.3 | 0.2 |
| 90 | 12.2 | 2.8 | 0.5 |
| 120 | 11.8 | 3.2 | 0.4 b = 3.2 |
| t = 120 |  |  | Q = 14.5 + 3.2 = 17.7 cm I = Q/t = 17.7 / 120 = 0.1475 cm/min Or 0.1475 x 60 = 8.85 cm/hr |

Soil unit 14: Natsini – North (Augie fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 12.4 | 2.6 | 2.6 |
| 3 | 11.2 | 3.8 | 1.2 |
| 5 | 9.8 | 5.2 | 1.4 |
| 10 | 8.2 | 6.8 | 1.6 |
| 20 | 6.2 | 8.8 | 2.0 |
| 30 | 5.4 | 9.6 | 0.8 |
| 45 | 3.3 | 11.7 | 2.1 |
| 60 | 2.1 | 12.9 | 1.2 |
| 90 | 1.4 | 13.6 | 0.7 |
| 120 | 0.2 | 14.8 | 1.2 |
| t = 120 |  |  | Q = 14.8 cm I = Q/t = 14.8 / 120 = 0.123 cm/min Or 0.123 x 60 = 7.39 cm/hr |

Soil unit 15: Natsini – North-west (Augie fadama site)

| Time (minute) | Depth (cm) | Cumulative (cm) | Infiltration (Q) (cm) |
| --- | --- | --- | --- |
| 0 | 15 | 0.0 | 0.0 |
| 1 | 14 | 1.0 | 1.0 |
| 3 | 12.1 | 2.9 | 1.9 |
| 5 | 11.8 | 3.2 | 0.3 |
| 10 | 9.2 | 5.8 | 2.6 |
| 20 | 8.5 | 6.5 | 0.7 |
| 30 | 7.5 | 7.5 | 1.0 |
| 45 | 6.6 | 8.4 | 0.9 |
| 60 | 5.5 | 9.5 | 1.1 |
| 90 | 3.5 | 11.5 | 2.0 |
| 120 | 1.1 | 13.9 | 2.4 |
| t = 120 |  |  | Q = 13.9 cm I = Q/t = 13.9 / 120 = 0.116 cm/min Or 0.116 x 60 = 6.95 cm/hr |