# Appendices

Appendix 1. Water sampling locations, description and sample types

| TYPE | COD | Name | Catchment | Long. (East) | Lat. (South) | Elevation (m.a.s.l.) |
| --- | --- | --- | --- | --- | --- | --- |
| Stream natural conditions (5) | STN_01 | Kikafu undisturbed | Kikafu | 37.209447 | -3.191871 | 1502 |
|  | STN_02 | Weruweru undisturbed | Weruweru | 37.257663 | -3.194194 | 1592 |
|  | STN_03 | Karanga undisturbed | Karanga | 37.320205 | -3.208268 | 1570 |
|  | STN_04 | Mrusunga | Rau | 37.413409 | -3.261944 | 1492 |
|  | STN_05 | Himo undisturbed | Himo | 37.508484 | -3.236656 | 1886 |
| Stream (14) | STR_01 | Kikafu @ Kalali | Kikafu | 37.216461 | -3.219271 | 1331 |
|  | STR_02 | Kikafu @ Mnyama | Kikafu | 37.216884 | -3.269159 | 1066 |
|  | STR_03 | Kikafu @ old bridge | Kikafu | 37.219362 | -3.316996 | 949 |
|  | STR_04 | Weruweru @ Maoa | Weruweru | 37.242956 | -3.223363 | 1290 |
|  | STR_05 | Weruweru @ Nshara | Weruweru | 37.230437 | -3.267135 | 1126 |
|  | STR_06 | Weruweru @ Highway | Weruweru | 37.259004 | -3.324541 | 965 |
|  | STR_07 | Karanga @ Uri | Karanga | 37.315728 | -3.238802 | 1336 |
|  | STR_08 | Karanga @ HOM2 | Karanga | 37.310271 | -3.268604 | 1169 |
|  | STR_09 | Karanga @ Kibosho | Karanga | 37.311446 | -3.300613 | 1023 |
|  | STR_10 | Karanga @ Highway | Karanga | 37.317601 | -3.345787 | 901 |
|  | STR_11 | Rau @ staff gauge | Rau | 37.353587 | -3.330409 | 858 |
|  | STR_12 | Monjo | Himo | 37.507439 | -3.28786 | 1409 |
|  | STR_13 | Whona | Himo | 37.528771 | -3.332597 | 1050 |
|  | STR_14 | Himo @ Highway | Himo | 37.544889 | -3.3905 | 852 |
| Spring (9) | SPR_01 | Mwanga | Kikafu | 37.232309 | -3.191864 | 1557 |
|  | SPR_02 | Lawii | Kikafu | 37.226306 | -3.216732 | 1315 |
|  | SPR_03 | Shirimatunda | Karanga | 37.291345 | -3.323589 | 978 |
|  | SPR_04 | Karanga | Karanga | 37.315056 | -3.35165 | 890 |
|  | SPR_05 | Kisimeni | Karanga | 37.342209 | -3.239073 | 1429 |
|  | SPR_06 | Sabasaba | Rau | 37.352526 | -3.32652 | 899 |
|  | SPR_07 | Mkongo | Miwaleni | 37.497152 | -3.343121 | 1233 |
|  | SPR_08 | Kitsuru | Himo | 37.495382 | -3.262702 | 1721 |
|  | SPR_09 | Miwaleni | Miwaleni | 37.447843 | -3.432325 | 713 |
| Canal (7) | CAN_01 | Upstream Kikafu | Kikafu | 37.21604 | -3.183583 | 1724 |
|  | CAN_02 | Downsteam Kikafu | Kikafu | 37.217908 | -3.273895 | 1138 |
|  | CAN_03 | MIddle Karanga | Karanga | 37.299677 | -3.237194 | 1345 |
|  | CAN_04 | Downsteam Karanga | Karanga | 37.317912 | -3.308162 | 1021 |
|  | CAN_05 | Upstream Himo | Himo | 37.49893 | -3.280482 | 1535 |
|  | CAN_06 | Downsteam Himo | Himo | 37.545868 | -3.369611 | 928 |
|  | CAN_07 | Upstream Karanga | Karanga | 37.308899 | -3.210233 | 1608 |
| Domestic water (8) | DOM_01 | Nkweseko | Kikafu | 37.240922 | -3.184747 | 1682 |
|  | DOM_02 | Nshara | Kikafu | 37.227683 | -3.267132 | 1169 |
|  | DOM_03 | Uri | Karanga | 37.308899 | -3.210233 | 1608 |
|  | DOM_04 | Mawenzi S.S. | Karanga | 37.324438 | -3.343454 | 917 |
|  | DOM_05 | Mnini | Rau | 37.356537 | -3.285721 | 1150 |
|  | DOM_06 | Komalyangue | Himo | 37.493132 | -3.246714 | 1875 |
|  | DOM_07 | Rauya | Himo | 37.528407 | -3.323469 | 1159 |
|  | DOM_08 | Holili Vijijini | East | 37.635826 | -3.36055 | 958 |
| Lake (1) | LAK_01 | Lake Chala | East | 37.687382 | -3.310555 | 953 |
| Groundwater (4) | GWA_01 | Kilimanjaro Plantation | Karanga | 37.314219 | -3.296918 | 1064 |
|  | GWA_02 | Kaloleni Moshi | Rau | 37.344894 | -3.37354 | 788 |
|  | GWA_03 | Sido Moshi | Rau | 37.338869 | -3.362123 | 812 |
|  | GWA_04 | Nganga Mfumoni | Rau | 37.35196 | -3.337747 | 845 |
| Rain (3) | RAI_01 | Nkweseko (2) | Kikafu | 37.241151 | -3.18446 | 1681 |
|  | RAI_02 | Nkweseko | Kikafu | 37.241151 | -3.18446 | 1681 |
|  | RAI_03 | Kidia | Rau | 37.4130427 | -3.2979887 | 1248 |

Appendix 2. Physical, chemical and biological analysis of water samples. The abbreviations of the parameters mean Electrical Conductivity (EC), Total Dissolved Solids (TDS), Total Hardness (TH), Total Alkalinity (ALK), Sodium Adsorption Ratio (SAR)

| TYPE | COD | Color | EC (μS/cm) | pH | TDS (mg/l) | Turbidity (NTU) | TH (mg CaCO3/l) | E. coli (CFU/100ml) | F- (mg/l) | Cl- (mg/l) | NO3- (mg/l) | NO3-N (mg/l) | SO4- (mg/l) | ALK (mg CaCO3/l) | HCO3- (mg/l) | Ca2+ (mg/l) | Mg2+ (mg/l) | Na+ (mg/l) | K+ (mg/l) | SAR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream natural condition (5) | STN_01 | No color | 57.1 | 7.75 | 31 | 1.17 | 14.4 | 27 | 0.06 | 0.62 | 0.63 | 0.19 | 0.51 | 70.3 | 85.77 | 5.10 | 2.06 | 2.71 | 1.35 | 0.26 |
|  | STN_02 | Milky | 59.9 | 7.65 | 33 | 7.84 | 4.2 | 255 | 0.01 | 0.43 | 0.69 | 0.23 | 0.40 | 76 | 92.72 | 3.39 | 1.06 | 5.58 | 2.82 | 0.68 |
|  | STN_03 | No color | 57.7 | 7.85 | 31 | 1.74 | 1.1 | 100 | 0.10 | 1.04 | 4.11 | 0.96 | 2.02 | 38 | 46.36 | 1.35 | 0.69 | 6.82 | 3.13 | 1.19 |
|  | STN_04 | No color | 109.4 | 8.04 | 60 | 4.69 | 29.1 | 200 | 0.06 | 0.62 | 0.56 | 0.18 | 0.21 | 57.35 | 69.97 | 8.53 | 6.49 | 3.04 | 1.16 | 0.19 |
|  | STN_05 | No color | 86.7 | 7.61 | 47 | 1.91 | 18.4 | 520 | 0.15 | 0.83 | 0.42 | 0.08 | 0.50 | 51.8 | 63.20 | 7.93 | 4.74 | 3.00 | 1.65 | 0.21 |
| Stream (14) | STR_01 | No color | 47.4 | 7.57 | 26 | 2.40 | 9.4 | 11 | 0.14 | 0.91 | 0.92 | 0.35 | 0.73 | 101.75 | 124.14 | 3.59 | 1.50 | 3.02 | 1.48 | 0.34 |
|  | STR_02 | Milky | 76.9 | 7.76 | 41 | 5.32 | 19.2 | 3 | 0.01 | 1.58 | 1.59 | 0.46 | 0.97 | 64.75 | 79.00 | 5.04 | 2.14 | 5.87 | 3.22 | 0.55 |
|  | STR_03 | Milky | 90.7 | 8.49 | 49 | 5.87 | 23.4 | 2 | 0.13 | 3.28 | 1.33 | 0.31 | 0.70 | 42.55 | 51.91 | 5.93 | 3.05 | 6.27 | 3.26 | 0.52 |
|  | STR_04 | Milky | 58.7 | 7.67 | 32 | 9.74 | 4.2 | 1500 | 0.12 | 0.48 | 0.72 | 0.17 | 0.53 | 32 | 39.04 | 3.41 | 1.08 | 5.62 | 2.75 | 0.68 |
|  | STR_05 | No color | 59.3 | 7.64 | 32 | 6.90 | 4.6 | 700 | 0.01 | 0.86 | 1.11 | 0.19 | 0.66 | 26 | 31.72 | 3.45 | 1.12 | 5.58 | 2.75 | 0.67 |
|  | STR_06 | Milky | 148.3 | 7.59 | 80 | 5.30 | 14.7 | 175 | 0.11 | 0.98 | 0.98 | 0.48 | 0.55 | 34 | 41.48 | 8.30 | 3.47 | 14.19 | 7.58 | 1.04 |
|  | STR_07 | No color | 56.0 | 7.49 | 31 | 2.83 | 0.9 | 370 | 0.09 | 2.45 | 4.95 | 0.68 | 3.47 | 30 | 36.60 | 1.34 | 0.74 | 6.58 | 2.84 | 1.13 |
|  | STR_08 | No color | 79.4 | 7.19 | 43 | 4.15 | 5 | 0 | 0.04 | 2.71 | 2.17 | 0.73 | 0.69 | 42 | 51.24 | 3.50 | 1.40 | 7.21 | 5.41 | 0.82 |
|  | STR_09 | Milky | 105.0 | 7.53 | 57 | 5.72 | 9.2 | 0 | 0.04 | 5.08 | 6.22 | 1.14 | 0.80 | 56 | 68.32 | 4.45 | 2.04 | 10.58 | 5.58 | 1.04 |
|  | STR_10 | Light Brown | 166.9 | 7.51 | 90 | 6.73 | 10 | 285 | 0.01 | 3.52 | 4.10 | 1.58 | 0.48 | 84 | 102.48 | 5.57 | 2.68 | 19.51 | 8.97 | 1.70 |
|  | STR_11 | Milky | 148.6 | 7.76 | 80 | 10.70 | 24.3 | 180 | 0.07 | 5.13 | 3.11 | 0.68 | 0.66 | 74 | 90.28 | 11.45 | 6.35 | 8.05 | 3.69 | 0.47 |
|  | STR_12 | No color | 102.0 | 7.89 | 56 | 2.97 | 19.1 | 290 | 0.02 | 1.70 | 0.65 | 0.10 | 0.94 | 74 | 90.28 | 9.31 | 6.29 | 3.18 | 1.38 | 0.20 |
|  | STR_13 | No color | 99.6 | 8.34 | 54 | 5.21 | 19.5 | 890 | 0.02 | 1.02 | 0.53 | 0.10 | 1.00 | 61.05 | 74.48 | 8.11 | 5.33 | 3.98 | 1.60 | 0.27 |
|  | STR_14 | Light Brown | 126.6 | 8.14 | 68 | 7.67 | 23 | 0 | 0.01 | 1.81 | 0.07 | 0.00 | 1.28 | 81.4 | 99.31 | 8.71 | 6.73 | 4.73 | 3.75 | 0.29 |
| Spring (9) | SPR_01 | No color | 50.2 | 5.72 | 29 | 2.78 | 10.4 | 47 | 0.01 | 2.56 | 2.20 | 0.50 | 3.87 | 68.45 | 83.51 | 2.37 | 0.80 | 3.29 | 2.40 | 0.47 |
|  | SPR_02 | No color | 63.7 | 5.72 | 34 | 1.25 | 11.8 | 2 | 0.12 | 3.25 | 2.59 | 0.60 | 0.87 | 48.1 | 58.68 | 4.08 | 1.55 | 4.78 | 2.14 | 0.51 |
|  | SPR_03 | No color | 123.4 | 6.93 | 67 | 1.89 | 6.1 | 705 | 0.12 | 1.06 | 1.97 | 0.71 | 0.74 | 70 | 85.40 | 3.86 | 1.70 | 15.57 | 6.33 | 1.66 |
|  | SPR_04 | No color | 182.7 | 6.86 | 98 | 1.52 | 10.5 | 0 | 0.10 | 4.49 | 7.75 | 1.59 | 1.96 | 79.55 | 97.05 | 5.80 | 2.66 | 22.65 | 9.33 | 1.95 |
|  | SPR_05 | No color | 45.0 | 7.55 | 24 | 2.03 | 4.6 | 100 | 0.04 | 0.61 | 0.63 | 0.29 | 0.62 | 30 | 36.60 | 2.95 | 0.80 | 3.58 | 2.17 | 0.48 |
|  | SPR_06 | No color | 159.1 | 6.21 | 86 | 1.02 | 24.1 | 89 | 0.02 | 6.56 | 12.88 | 3.18 | 0.90 | 68.45 | 83.51 | 11.69 | 5.13 | 11.57 | 5.41 | 0.71 |
|  | SPR_07 | No color | 401.0 | 7.69 | 218 | 2.07 | 92.9 | 800 | 0.01 | 3.19 | 3.88 | 0.21 | 0.56 | 229.4 | 279.87 | 22.78 | 29.51 | 9.48 | 3.39 | 0.31 |
|  | SPR_08 | No color | 83.6 | 6.72 | 46 | 0.36 | 15.5 | 190 | 0.04 | 0.75 | 1.15 | 0.33 | 0.26 | 42.55 | 51.91 | 7.70 | 4.73 | 2.79 | 1.67 | 0.20 |
|  | SPR_09 | No color | 321.0 | 7.66 | 173 | 1.48 | 64.3 | 1600 | 0.04 | 6.45 | 2.78 | 0.84 | 1.01 | 177.6 | 216.67 | 12.23 | 12.70 | 13.26 | 3.92 | 0.63 |
| Canal (7) | CAN_01 | Light Brown | 41.4 | 7.69 | 23 | 27.40 | 5 | 5 | 0.07 | 0.39 | 0.04 | 0.00 | 0.61 | 103.6 | 126.39 | 1.81 | 0.68 | 4.11 | 2.17 | 0.66 |
|  | CAN_02 | Milky | 57.9 | 7.42 | 31 | 63.20 | 12.4 | 12 | 0.17 | 0.93 | 0.07 | 0.00 | 0.53 | 33.3 | 40.63 | 4.51 | 1.65 | 3.29 | 3.53 | 0.34 |
|  | CAN_03 | Light Brown | 67.5 | 7.81 | 37 | 60.30 | 3.7 | 0 | 0.03 | 3.57 | 4.52 | 1.03 | 2.03 | 30 | 36.60 | 2.75 | 0.95 | 6.61 | 3.58 | 0.88 |
|  | CAN_04 | No color | 106.0 | 7.26 | 57 | 23.80 | 8.8 | 0 | 0.05 | 4.41 | 6.70 | 1.51 | 0.63 | 50 | 61.00 | 5.47 | 2.03 | 10.19 | 5.56 | 0.94 |
|  | CAN_05 | No color | 80.6 | 7.70 | 44 | 6.34 | 15.8 | 180 | 0.02 | 0.76 | 0.11 | 0.00 | 0.47 | 62.9 | 76.74 | 7.33 | 4.76 | 2.81 | 1.47 | 0.20 |
|  | CAN_06 | Light Brown | 104.0 | 7.78 | 56 | 18.20 | 19.9 | 340 | 0.01 | 1.15 | 0.30 | 0.07 | 0.75 | 59.2 | 72.22 | 7.41 | 4.42 | 3.60 | 1.51 | 0.26 |
|  | CAN_07 | No color | 38.3 | 7.76 | 21 | 1.92 | 0 | 25 | 0.19 | 1.88 | 0.45 | 0.05 | 1.78 | 38 | 46.36 | 0.60 | 0.25 | 5.41 | 0.98 | 1.48 |
| Domestic water (8) | DOM_01 | No color | 17.7 | 6.89 | 10 | 1.28 | 0 | 111 | 0.20 | 0.52 | 0.45 | 0.18 | 0.40 | 70.3 | 85.77 | - | - | - | - | - |
|  | DOM_02 | No color | 32.1 | 7.21 | 17 | 1.70 | 0 | 10 | 0.10 | 0.67 | 1.58 | 0.47 | 0.45 | 51.8 | 63.20 | - | - | - | - | - |
|  | DOM_03 | No color | 38.3 | 7.76 | 21 | 1.92 | 0 | 25 | 0.19 | 1.88 | 0.45 | 0.05 | 1.78 | 38 | 46.36 | - | - | - | - | - |
|  | DOM_04 | No color | 90.7 | 8.06 | 49 | 1.16 | 21.2 | 59 | 0.05 | 0.84 | 1.74 | 0.39 | 0.94 | 46.25 | 56.43 | - | - | - | - | - |
|  | DOM_05 | No color | 128.4 | 7.36 | 70 | 6.37 | 24.5 | 328 | 0.02 | 1.12 | 1.77 | 0.45 | 0.63 | 68.45 | 83.51 | - | - | - | - | - |
|  | DOM_06 | No color | 90.2 | 7.89 | 49 | 2.51 | 20 | 220 | 0.02 | 0.67 | 0.69 | 0.13 | 0.24 | 46.25 | 56.43 | - | - | - | - | - |
|  | DOM_07 | No color | 64.6 | 7.18 | 35 | 0.38 | 11 | 950 | 0.04 | 1.23 | 1.29 | 0.27 | 0.80 | 33.3 | 40.63 | - | - | - | - | - |
|  | DOM_08 | No color | 82.1 | 7.41 | 45 | 0.87 | 18.2 | 500 | 0.08 | 0.76 | 0.82 | 0.17 | 0.51 | 62.9 | 76.74 | - | - | - | - | - |
| Lake (1) | LAK_01 | No color | 367.0 | 8.70 | 199 | 1.87 | 69.1 | 25 | 0.10 | 10.28 | 0.13 | 0.00 | 1.87 | 201.65 | 246.01 | 10.60 | 20.13 | 16.55 | 7.93 | 0.69 |
| Groundwater (4) | GWA_01 | No color | 96.3 | 6.83 | 52 | 0.37 | 15 | - | 0.01 | 3.65 | 9.40 | 1.91 | 0.86 | 46 | 56.12 | 7.09 | 2.24 | 7.08 | 3.98 | 0.59 |
|  | GWA_02 | Light Brown | 282.0 | 7.31 | 153 | 155.00 | 13.2 | - | 0.01 | 23.68 | 0.03 | 0.00 | 1.14 | 114.7 | 139.93 | 6.27 | 3.10 | 29.48 | 8.99 | 2.41 |
|  | GWA_03 | Light Brown | 165.2 | 7.34 | 88 | 62.40 | 11.9 | - | 0.04 | 9.66 | 0.02 | 0.00 | 0.42 | 90.65 | 110.59 | 7.84 | 2.62 | 15.07 | 6.51 | 1.19 |
|  | GWA_04 | No color | 139.7 | 6.54 | 75 | 6.80 | 15.9 | - | 0.08 | 5.86 | 11.46 | 1.66 | 1.62 | 74 | 90.28 | 5.94 | 2.74 | 12.84 | 5.40 | 1.09 |
| Rain (3) | RAI_01 | No color | 25.8 | 7.02 | 14 | - | - | - | 0.16 | 1.65 | 1.14 | 0.32 | 1.84 | 11.1 | 13.54 | 2.37 | 0.22 | 0.72 | 0.42 | 0.12 |
|  | RAI_02 | No color | 21.8 | 6.97 | 12 | - | - | - | 0.07 | 1.21 | 0.92 | 0.23 | 2.08 | 10 | 12.20 | 1.64 | 0.15 | 0.43 | 0.25 | 0.09 |
|  | RAI_03 | No color | 35.4 | 7.28 | 20 | - | - | - | 0.10 | 1.64 | 1.27 | 0.27 | 3.28 | 40.7 | 49.65 | 3.73 | 0.42 | 0.72 | 0.51 | 0.09 |

Appendix 3. Values and classification of the water quality indices for each water sample. DWQI=drinking water quality index, IWQI=irrigation water quality index, KI=Kelley’s Index, SSP=Soluble Sodium Percentage, PI=Permeability Index, RSBC=Residual Sodium Bicarbonate and MR=Magnesium Ratio

| COD | DWQI |  | IWQI |  | KI |  | SSP (%) |  | PI (%) |  | RSBC (meq/l) |  | MR (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STN_01 | 968 | Unsuitable | 8 | No restriction | 0.28 | Suitable | 26.47 | Good | 240.65 | Good | 1.15 | Safe | 39.95 | Suitable |
| STN_02 | 9112 | Unsuitable | 9 | No restriction | 0.95 | Suitable | 55.13 | Permissible | 295.42 | Good | 1.35 | Safe | 33.92 | Suitable |
| STN_03 | 3576 | Unsuitable | 13 | No restriction | 2.39 | Unsuitable | 75.19 | Doubtful | 277.45 | Good | 0.69 | Safe | 45.66 | Suitable |
| STN_04 | 7149 | Unsuitable | 12 | No restriction | 0.14 | Suitable | 14.43 | Excellent | 109.52 | Good | 0.71 | Safe | 55.63 | Unsuitable |
| STN_05 | 18575 | Unsuitable | 8 | No restriction | 0.17 | Suitable | 18.01 | Excellent | 125.21 | Good | 0.64 | Safe | 49.64 | Suitable |
| STR_01 | 396 | Unsuitable | 7 | No restriction | 0.43 | Suitable | 35.86 | Good | 358.82 | Good | 1.86 | Safe | 40.82 | Suitable |
| STR_02 | 112 | Poor | 10 | No restriction | 0.60 | Suitable | 44.13 | Permissible | 204.04 | Good | 1.04 | Safe | 41.14 | Suitable |
| STR_03 | 79 | Good | 15 | No restriction | 0.50 | Suitable | 39.44 | Good | 145.78 | Good | 0.55 | Safe | 45.89 | Suitable |
| STR_04 | 53577 | Unsuitable | 9 | No restriction | 0.95 | Suitable | 54.92 | Permissible | 207.61 | Good | 0.47 | Safe | 34.24 | Suitable |
| STR_05 | 25004 | Unsuitable | 9 | No restriction | 0.92 | Suitable | 54.24 | Permissible | 190.12 | Good | 0.35 | Safe | 34.86 | Suitable |
| STR_06 | 6255 | Unsuitable | 13 | No restriction | 0.88 | Suitable | 53.69 | Permissible | 109.48 | Good | 0.27 | Safe | 40.81 | Suitable |
| STR_07 | 13218 | Unsuitable | 10 | No restriction | 2.25 | Unsuitable | 73.78 | Doubtful | 256.44 | Good | 0.53 | Safe | 47.47 | Suitable |
| STR_08 | 3 | Excellent | 8 | No restriction | 1.08 | Unsuitable | 60.89 | Doubtful | 203.72 | Good | 0.66 | Safe | 39.74 | Suitable |
| STR_09 | 5 | Excellent | 13 | No restriction | 1.18 | Unsuitable | 60.75 | Doubtful | 178.69 | Good | 0.90 | Safe | 43.00 | Suitable |
| STR_10 | 10184 | Unsuitable | 17 | No restriction | 1.70 | Unsuitable | 68.38 | Doubtful | 159.19 | Good | 1.40 | Safe | 44.25 | Suitable |
| STR_11 | 6436 | Unsuitable | 13 | No restriction | 0.32 | Suitable | 28.88 | Good | 108.46 | Good | 0.91 | Safe | 47.76 | Suitable |
| STR_12 | 10362 | Unsuitable | 10 | No restriction | 0.14 | Suitable | 15.01 | Excellent | 120.86 | Good | 1.01 | Safe | 52.71 | Unsuitable |
| STR_13 | 31792 | Unsuitable | 13 | No restriction | 0.21 | Suitable | 20.25 | Good | 124.74 | Good | 0.79 | Safe | 52.00 | Unsuitable |
| STR_14 | 7 | Excellent | 13 | No restriction | 0.21 | Suitable | 23.40 | Good | 123.66 | Good | 1.18 | Safe | 56.02 | Unsuitable |
| SPR_01 | 1684 | Unsuitable | 13 | No restriction | 0.78 | Suitable | 52.70 | Permissible | 401.89 | Good | 1.25 | Safe | 35.65 | Suitable |
| SPR_02 | 77 | Good | 13 | No restriction | 0.63 | Suitable | 44.22 | Permissible | 220.35 | Good | 0.76 | Safe | 38.57 | Suitable |
| SPR_03 | 25181 | Unsuitable | 11 | No restriction | 2.04 | Unsuitable | 71.62 | Doubtful | 184.25 | Good | 1.21 | Safe | 42.06 | Suitable |
| SPR_04 | 4 | Excellent | 16 | No restriction | 1.94 | Unsuitable | 70.65 | Doubtful | 150.07 | Good | 1.29 | Safe | 43.05 | Suitable |
| SPR_05 | 3574 | Unsuitable | 7 | No restriction | 0.73 | Suitable | 49.81 | Permissible | 252.35 | Good | 0.45 | Safe | 30.82 | Suitable |
| SPR_06 | 3185 | Unsuitable | 19 | No restriction | 0.50 | Suitable | 38.95 | Good | 110.63 | Good | 0.78 | Safe | 41.97 | Suitable |
| SPR_07 | 28578 | Unsuitable | 21 | No restriction | 0.12 | Suitable | 12.28 | Excellent | 64.21 | Marginal | 3.45 | Safe | 68.11 | Unsuitable |
| SPR_08 | 6788 | Unsuitable | 6 | No restriction | 0.16 | Suitable | 17.50 | Excellent | 117.16 | Good | 0.47 | Safe | 50.29 | Unsuitable |
| SPR_09 | 57148 | Unsuitable | 18 | No restriction | 0.35 | Suitable | 29.03 | Good | 110.28 | Good | 2.94 | Safe | 63.12 | Unsuitable |
| CAN_01 | 189 | Poor | 8 | No restriction | 1.23 | Unsuitable | 61.65 | Doubtful | 498.12 | Good | 1.98 | Safe | 38.23 | Suitable |
| CAN_02 | 449 | Unsuitable | 6 | No restriction | 0.40 | Suitable | 39.28 | Good | 190.20 | Good | 0.44 | Safe | 37.70 | Suitable |
| CAN_03 | 21 | Excellent | 12 | No restriction | 1.33 | Unsuitable | 63.76 | Doubtful | 211.07 | Good | 0.46 | Safe | 36.25 | Suitable |
| CAN_04 | 10 | Excellent | 11 | No restriction | 1.01 | Unsuitable | 57.08 | Permissible | 163.37 | Good | 0.73 | Safe | 37.93 | Suitable |
| CAN_05 | 6433 | Unsuitable | 8 | No restriction | 0.16 | Suitable | 17.42 | Excellent | 140.97 | Good | 0.88 | Safe | 51.74 | Unsuitable |
| CAN_06 | 12151 | Unsuitable | 10 | No restriction | 0.21 | Suitable | 21.03 | Good | 139.01 | Good | 0.80 | Safe | 49.60 | Suitable |
| CAN_07 | 897 | Unsuitable | 11 | No restriction | 4.66 | Unsuitable | 83.75 | Unsuitable | 386.43 | Good | 0.73 | Safe | 40.92 | Suitable |
| DOM_01 | 3968 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_02 | 362 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_03 | 901 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_04 | 2116 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_05 | 11727 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_06 | 7867 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_07 | 33931 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| DOM_08 | 17862 | Unsuitable | - | - | - | - | - | - | - | - | - | - | - | - |
| LAK_01 | 902 | Unsuitable | 27 | No restriction | 0.33 | Suitable | 29.68 | Good | 93.88 | Good | 3.50 | Safe | 75.80 | Unsuitable |
| GWA_01 | - | - | 10 | No restriction | 0.57 | Suitable | 43.24 | Permissible | 149.84 | Good | 0.57 | Safe | 34.23 | Suitable |
| GWA_02 | - | - | 19 | No restriction | 2.26 | Unsuitable | 72.69 | Doubtful | 151.11 | Good | 1.98 | Safe | 44.93 | Suitable |
| GWA_03 | - | - | 12 | No restriction | 1.08 | Unsuitable | 57.54 | Permissible | 158.64 | Good | 1.42 | Safe | 35.50 | Suitable |
| GWA_04 | - | - | 14 | No restriction | 1.07 | Unsuitable | 57.14 | Permissible | 164.21 | Good | 1.18 | Safe | 43.23 | Suitable |
| RAI_01 | - | - | 2 | No restriction | 0.23 | Suitable | 23.53 | Good | 298.88 | Good | 0.10 | Safe | 13.53 | Suitable |
| RAI_02 | - | - | 2 | No restriction | 0.20 | Suitable | 21.08 | Good | 413.88 | Good | 0.12 | Safe | 12.78 | Suitable |
| RAI_03 | - | - | 4 | No restriction | 0.14 | Suitable | 16.68 | Excellent | 369.26 | Good | 0.63 | Safe | 15.79 | Suitable |

Appendix 4. Bloxplots per water type for the irrigation water suitability indicators. The abbreviations of the indicators mean: Kelley’s Index (KI), Soluble Sodium Percentage (SSP), Permeability Index (PI), Residual Sodium Bicarbonate (RSBC) and Magnesium Ratio (MR)