Programmable Light-Induced Carbene Generation for On Demand Chemical Synthesis: Introducing DigiChemTree

Abhilash Rana,1 Ruchi Chauhan,1 Amirreza Mottafegh,2 Dong Pyo Kim,2 and Ajay K. Singh1*

1Department of Organic Synthesis & Process Chemistry, CSIR-Indian Institute of Chemical Technology, Hyderabad-500007, India.

And

Academy of Scientific and Innovative Research (AcSIR), Ghaziabad-201002, Uttar Pradesh, India.

2Center for Intelligent Microprocess of Pharmaceutical Synthesis, Department of Chemical Engineering, Pohang University of Science and Technology (POSTECH), Pohang 37673, Republic of Korea

Abhilash Rana and Ruchi Chauhan equally contributed the work

E-mail: ajaysingh015@gmail.com. ajaysingh015@iict.res.in

Table of contents:

| S1. | General S1.1. Materials. S1.2 Analysis. | S3 |
| --- | --- | --- |
| S2. | Automated system overview | S4 |
| S3 | Case study-1: Carbene insertion (C-O bond formation) single objective multi-variant auto-optimization. | S20 |
| S4 | Case study-2: Carbene insertion (C-S bond formation) single objective multi-variant auto-optimization. | S48 |
| S5 | Case study-3: Carbene insertion (C-N bond formation) single objective multi-variant auto-optimization. | S70 |
| S6 | Case study-4: Carbene insertion (C-C bond formation, cyclopropanation) single objective multi-variant auto-optimization. | S93 |
| S7 | Case study-5: Carbene insertion (C-C bond formation, cyclopropenation) single objective multi-variant auto-optimization. | S115 |
| S8 | Case study-6: Carbene insertion (N-C-O bond formation, oxazole) single objective multi-variant auto-optimization. | S136 |
| S9 | Case study-7: Carbene insertion (C=C bond formation, fumarate) single objective multi-variant auto-optimization. | S152 |
| S10. | Case study-8: On demand chemical synthesis. | S168 |
| S11. | Spectra | S220 |
| S12. | Troubleshooting | S388 |
| S13. | Supporting references | S389 |

S1. General

S1.1. Materials: Most of the reagents and chemicals are bought from Sigma-Aldrich and used as such without any further purification. Common organic chemicals and salts were purchased from AVRA chemicals, India. Water used for the experiments was deionized water (18.2 mS conductivity). All work-up and purification procedures were carried out with reagent-grade solvents in air. Analytical thin-layer chromatography (TLC) was performed using analytical chromatography silica gel 60 F254 precoated plates (0.25 mm). The developed chromatogram was analysed by UV lamp (254 nm). PTFE (id = 500 μm) tubing, T-junction, high-purity PFA tubing was purchased from Upchurch IDEX HEALTH &SCIENCE. The photo-batch reactor bought from Lelesil Mumbai India was slightly modified for the continuous flow reaction. Visible light (Blue, Red, Green LED) reactor was bought from the Smartchemsynth Machine Pvt. Ltd, Hyderabad.

S1.2. Analysis: High-resolution mass spectra (HRMS) were obtained from a JMS-T100TD instrument (DART) and Thermo Fisher Scientific Exactive (APCI). Nuclear magnetic resonance (NMR) spectra were recorded on a Bruker 600, 500, 400 or 300 MHz in CDCl3 or DMSO-d6 solvent. Chemical shifts for 1H NMR are expressed in parts per million (ppm) relative to tetramethylsilane (δ 0.00 ppm). Chemical shifts for 13C NMR are expressed in ppm relative to CDCl3 (δ 77.0 ppm). Data are reported as follows: chemical shift, multiplicity (s = singlet, d = doublet, dd = doublet of doublets, t = triplet, q = quartet, quin = quintet, sext = sextet, m = multiplet), coupling constant (Hz). GC/MS analysis was conducted on a Shimadzu technology GCMS-QP2010 instrument equipped with a HP-5 column (30 m × 0.25 mm, Hewlett-Packard) and inbuilt MS 5975C VL MSD system with triple axis detector.

S2. Automated system overview

S2.1. Automated system components

Table S1. Outlines the hardware components utilized in the automated platform, with color coding representing distinct modules (green: operation module, blue: reaction module, yellow: analysis module, purple: fractional sample collection).

| Type | Manufacturer |  | Module | Function | Communication protocol | Details in section |
| --- | --- | --- | --- | --- | --- | --- |
| Syringe pump |  | HARVARD | Operational module | Solvent delivery | Serial, USB | 2.2.1 |
| HPLC pump |  | KNAUER | Operational module | Solvent delivery | LAN | 2.2.1 |
| Continuous Syringe |  | SMART CHEM SYNTH | Operational module | Solvent delivery | Serial, USB | 2.2.1 |
| Power supply |  | OWON | Reaction module | Reaction | Serial, USB | 2.2.2 |
| Blue LED |  | SMART CHEM SYNTH | Reaction module | Reaction | Serial, USB | 2.2.2 |
| In-line IR |  | METTLER TOLEDO | Analysis module | Sample analysis | Serial, USB | 2.2.3 |
| 3D printer |  | ENDER | Collection module | Fractional sample collection | Serial, USB | 2.2.4 |

S2.2. Automated system hardware module.

S2.2.1 Operational module

The HPLC pumps (figure S1a) and Harvard syringe pump (figure S1b) were directly interfaced to the main computer via an RS-232 interface or LAN. These pumps will help in introducing reactants and reagent solutions into the micro reactor with varied flow rate as the central computer's prescribed flow rate (fr). This control protocol also includes starting/stopping of the pump. The central computer and the HPLC pump communicated through serial communication using ASCII code to exchange information about flow rate, operational status, and duration. This facilitated the transmission of essential details from the main computer to the pumps, enabling them to commence their operations as required. The syringe pump (figure S1c) The parameters for the serial communication of the syringe pump with the computer are given as follows: Baud Rate ‐ 9600, Flow control – none. The basic control protocols include starting/stopping the pump and setting different parameters. Arduino (figure S1d) connected to syringe pump directly interfaced to the main computer via an RS-232 interface or LAN and helps in controlling flow rate. It has inbuilt proximity sensor and pulse will generate through Arduino program.

Figure S1. Picture of working syringe pump and HPLC pump (a) HPLC pump (Knauer) (b) syringe pump (Harvard) (c) Continuous syringe pump with Arduino (smartchemsynth) (d) Arduino.

S2.2.2 Reaction module

We employed a commercially accessible cylindrical blue LED (figure S2b) for the photochemical reaction, utilizing a programmable power supply manufacture by Owon (figure S2a) to regulate the lamp's power (measured in watts) by managing both current and voltage. The Blue LED produces light in the range of wavelength 450-495 nm and it can be operated up to 5 A current, and 14.5 v voltage corresponds to about 70 w wattage. Additionally, rotating fan was attached to the bottom of cylindrical blue LED light to maintain the room temperature throughout the reaction. The central computer and the power supply communicated through serial communication using ASCII code to exchange information about varied voltage, current and operational status.

Figure S2. Manual designed photo-flow reactor; (a) power supply (b-c) blue LED module with cooling fan (1-70W, 460 nm max); (d) 1mL PFA coiled reactor (1.2-meter length, OD = 1.58 mm and id = 1.0 mm).

Figure S3. 3D graph elucidating the correlation among voltage (V), current (Amp), and the emission of blue light (Wattage).

S2.2.3 Analysis module

In-line FTIR analysis

In-situ reaction analysis was accomplished by recording IR (functional group, shifting peak in product) spectra using a benchtop ReactIR 15 base unit with DS Micro Flow Cell (Mettler toledo). It works through sensing the amount of infrared radiation after passing through a sample, generating a 2D graph with absorption on the Y-axis and wavenumber on the X-axis, having optical range from 4000 to 650 cm-1. The system collected spectra with 16 scans at resolution 4 cm-1.

Figure S4. Actual photograph of ReactIR 15 flow cell setup.

Figure S5. Work flow of In-line FTIR measurement when running reaction optimization.

S2.2.4. Collection module.

We employed a commercially accessible 3D printer as an automated collector. The Python program on the central system interfaced with the 3D printer and give commands to the stepper motors on the X, Y, and Z axis of the printer for movement. This information is transmitted via USB serial communication. PFA tubing (od = 1/16, id = 1mm) is connected to the printer nozzle and move along x-aixs, and a test tube stand is placed on the printing platform which move along Y-axis to collect the fraction of reaction mixture output (see figure S6). This configuration enables precise control of the 3D printer, facilitating tasks such as sample collection, reactor stabilization, and washing in diverse scenarios. With this 3D printer setup, we can collect maximum 22 fraction of out coming reaction mixture in one run of experiment.

Figure S6. (a) Original photograph of dismantle 3D printer; (b) Assembled photograph of 3D printer as auto-fraction collector.

S2.3. Automated system software

Table S2. List of the software required for the automated system.

| Type | Version | Function |
| --- | --- | --- |
| Windows 10 | Professional | Control system |
| Python | 3.9, 32‐bit | Control of platform |
| Spyder web | 3.7 | run Bayesian Optimizer alogrithm |
| Arduino IDE | 1.8.19 | Control temperature |
| iC IR | 7.0 | Control benchtop In-line FTIR |

S2.4. Bayesian optimization

The machine learning algorithm utilized in this project is Bayesian Optimization (BO), implemented through the open-source package Bayesian optimization in an algorithm which uses Baye’s theorem to find the maximum of objective.  is a robust machine learning technique, especially suited for optimizing chemical reactions during the initial phases of process development its efficiency lies in the ability to navigate extensive reaction spaces effectively, forecasting optimal reaction conditions with high yields by assessing only a limited number of experiments. The selected initial points are the variable space (reaction conditions), the objectives to attain (e.g., maximize yield and/or conversion, minimize costs), the total number of experiments to perform, the information of communication port (e.g., pump = COM 7, printer = COM 9) and the overall experimental cost (indication how many cycles of experiment to be performed in each run). The initial points are chosen through Latin hypercube sampling for Euclidean and integral variables, while for discrete numeric values, selection is done uniformly at random. The variable space encompasses the range within which the objective is intended to be optimized. It accommodates both discrete and continuous variables. In Bayesian optimization incorporation of multiple variables of the same type (in a single line, as in String varA, varB, varC) is feasible, leaving the determination of the most effective among them to the model. Each time the BO calls the objective function, the platform is run, and the parameter settings are sent to the platform. The model then waits until the result is sent back before updating and deciding on the next experiment.

S2.4.1. Step-by-step instructions for setting up the experiments and operating the automated system.

Step 1

Background collection:  This marks the beginning of our new experiment. We'll start by selecting the wavenumber range and then proceed to click on "collect background." It's crucial to ensure that our solvent is running through the In-line IR. Upon clicking "collect background," a new dialogue box will appear, where we'll select 128 scans and proceed to collect the background.

Figure S7. Home page of the iC IR 7.0 containing step-by-step instructions on how to take solvent background before starting experiment.

Step 2

After background collection create new experiment by clicking on tab this dialogue box will appear.

Type your experiment name and select the folder to save your experiment at your desired location then select duration (1h-2day) and scan of sample interval (15sec-) for one run.

Figure S8. User Settings page, where the name of the experiments, folder and the location they are saved to are defined.

Step 3

Upon creating the new experiment, you'll encounter this dialogue box. Firstly, (a) pin all the samples by clicking on "smart pin," followed by clicking on "pin 6 samples." Secondly, (b) export all the pinned spectra by clicking on "spectra" and then exporting all the files in CSV format only, which will be in an Excel file. Thirdly, (c) after clicking on "export," a new dialogue box will appear showing all the CSV files exported from the In-line IR. Create a text document in that folder and save it with a AB.py extension to save the Python code.

Figure S9. User Settings page, where in first step (a) pin all the sample, in second step (b) export spectrum file in CSV format and in third step (c) you can see all the spectrum file in excel format.

Step 4

Open spyder web app in central system where we will write our python code or open the previously saved python code. In this python code we have to define path in mypath and then we write the communication port for syringe pump, power supply and auto collector. Then we will give reaction condition variable such as flow rate (residence time), voltage, current and select no of cycles (1-22) in one run.

Figure S10. Reaction parameter settings page, where you will define the file path and communication port with the instruments attached to central system.

Figure S11. Reaction parameter settings page, where you will define the boundary of flow rate, voltage, current and communication port with the instruments attached to central system.

Figure S12. Example layout on excel file automatically generated from iC IR and bayesian optimizer.

S2.4.2. General Procedure for optimization with automated system.

List of starting material (2a-2g): Previously, reported protocol has been used to synthesized the diazo-compound.1

Figure S13. Starting material diazo ester reported earlier.

| Starting material | Substrate | product | Probable molecule | Reported molecule |
| --- | --- | --- | --- | --- |
| (2) 1971 | (3) 1,159,909 | (4) | 1971 1159909 = 2,286,180,639 | 5675 |
| (2) 1971 | (5) 31,582 | (6) | 1971 31582 = 62,248,122 | 2590 |
| (2) 1971 | (7) 666,981 | (8) | 1971 666981 = 1,314,619,551 | 2410 |
| (2) 1971 | (9) 64,464 | (10) | 1971 64464 = 127,058,544 | 954 |
| (2) 1971 | (11) 46,269 | (12) | 1971 46269 = 91,196,199 | 396 |
| (2) 1971 | (13) 5,467,743 | (14) | 1971 5646743 = 11,129,730,453 | 385 |
| (2) 1971 | (15) 133666 | (16) | 1971 133666 = 263,455,686 | 113 |
| Total no of probable molecule = 1.5×1010 |  |  |  |  |
| Total no of known molecule = 12523 |  |  |  |  |

Table S3. Literature study of the probable molecules of product and their corresponding reported known molecule.

* Data extracted from Reaxys on 07 march 2024

% known molecule = 1252315000000000 100

= ~ 0.00000083%

S3 Case study-1 carbene insertion (C-O bond formation) single objective multi-variant auto-optimization.

S3.1. Background collection

The collection of solvent background or an air background before automated experiment stand out as most important step in obtaining finest quality infrared data from a fully operational React IR 15. Each sample measurement will use the background to 'ratio out' all infrared absorbing materials in the optical path and the source solvent peaks. It will give infrared fingerprint end-result in form of absorbance versus wavenumber of only the reaction mixture components. For background collection set the scan to128 scans at 4 cm-1 resolutions.

Sample preparation: We prepared 0.5 M stock solutions of compound 2a, reagent 3a, and product 4a in ethyl acetate, with each solution loaded into separate syringes. Our analysis utilized an In-line FTIR system. The experimental procedure began by introducing ethyl acetate solvent using a syringe pump for 10 min, during which data was recorded. Subsequently, the stock solution of compound 2a was introduced into the In-line FTIR for another 10 minutes. This process was repeated for the reaction product 4a and reagent 3a, each pumped for 10 minutes. After collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure S14. In-line IR background analysis of EtOAc, 0.5 M stock solution of 2a, 3a, and 4a in EtOAc.

The distinctive keto peak associated with benzoic acid (3a) in the initial substrate undergoes a noticeable shift in the resulting product (4a), aligning within the range of 1756 to 1775 cm-1. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

Micro reactor system: As shown in Figure. S2d, we had utilized pump to deliver the solution at varied flow rates. Stock solution starting material and reagent were introduced through a T-mixer with the same flow rate to maintain a stoichiometric ratio. Subsequently, they passed through a PFA tubing (od 1/16, id 1mm, l = 1.3 m, vol = 1 mL) for the carbene insertion reaction. The cylindrical reactor was enveloped by cylindrical blue light LED for irradiation. To enhance light absorption, the entire reactor was covered with aluminum foil.

S3.2. General procedure for the auto-optimized synthesis of compound 3a.

In our approach we are employing a python-coded based Bayesian optimization strategy for further refinement. Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in EtOAc) and reagent 3a (0.2 M in EtOAc), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 50-60 experiments in a 24 h time fame, and the results were tabulated in the optimization table (Table S4).

S3.2.1. Python code for optimizing condition of carbene insertion reaction (C-O bond formation).

Table S4. Auto-optimization table of carbene insertion reaction into O-H bonds of carboxylic acid.

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Space time yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.066608 | 10.882021 | 24.45 | 0.088501 | 22.30199 | 0.013547726 |
| 2 | 0.029862 | 14.306068 | 65.808 | 0.200355 | 50.48887 | 0.013750297 |
| 3 | 0.065704 | 12.421805 | 40.39 | 0.171524 | 43.22355 | 0.025900525 |
| 4 | 0.099795 | 10.154336 | 16.488 | 0.227276 | 57.27289 | 0.052125911 |
| 5 | 0.075772 | 13.720454 | 57.344 | 0.256559 | 64.65212 | 0.044677424 |
| 6 | 0.041725 | 10 | 13.469 | 0.019331 | 4.871431 | 0.001853775 |
| 7 | 0.046836 | 11.089352 | 24.981 | 0.097945 | 24.68185 | 0.010542825 |
| 8 | 0.095982 | 10 | 13.83 | 0.026577 | 6.697226 | 0.005862467 |
| 9 | 0.098594 | 14.4 | 66.616 | 0.381375 | 96.10539 | 0.086416489 |
| 10 | 0.027592 | 13.123195 | 49.103 | 0.092042 | 23.19437 | 0.005836805 |
| 11 | 0.094381 | 12.699958 | 43.77 | 0.071 | 17.89184 | 0.015400604 |
| 12 | 0.1 | 13.164905 | 49.799 | 0.270311 | 68.11758 | 0.062123235 |
| 13 | 0.025 | 12.628105 | 42.957 | 0.135531 | 34.15342 | 0.007786979 |
| 14 | 0.040461 | 13.437396 | 53.558 | 0.200978 | 50.64587 | 0.01868879 |
| 15 | 0.076298 | 12.251856 | 38.395 | 0.12583 | 31.70879 | 0.022064265 |
| 16 | 0.090613 | 12.933006 | 46.788 | 0.166475 | 41.95121 | 0.034668386 |
| 17 | 0.028235 | 10 | 15.458 | 0.039305 | 9.904745 | 0.002550535 |
| 18 | 0.061374 | 14.4 | 67.262 | 0.330339 | 83.24446 | 0.046594855 |
| 19 | 0.102368 | 14.08762 | 62.363 | 0.08926 | 22.49326 | 0.077462861 |
| 20 | 0.104702 | 14.27705 | 65.017 | 0.054675 | 13.77794 | 0.08534457 |
| 21 | 0.119409 | 14.2009 | 63.843 | 0.111543 | 28.10851 | 0.074518828 |
| 22 | 0.066445 | 12.17059 | 37.334 | 0.114956 | 28.96858 | 0.017554331 |
| 23 | 0.125 | 11.8592 | 33.843 | 0.025144 | 6.336214 | 0.007223285 |
| 24 | 0.066 | 13.39582 | 52.746 | 0.099361 | 25.03868 | 0.015071283 |
| 25 | 0.056156 | 14.49334 | 68.059 | 0.238695 | 60.15044 | 0.032096196 |
| 26 | 0.125 | 12.5 | 41.184 | 0.196633 | 49.55094 | 0.056488073 |
| 27 | 0.066 | 14.16656 | 63.265 | 0.200791 | 50.59875 | 0.035006859 |
| 28 | 0.069702 | 14.5 | 68.049 | 0.26917 | 67.83005 | 0.04311836 |
| 29 | 0.094102 | 14.5 | 67.904 | 0.379021 | 95.51218 | 0.081969534 |
| 30 | 0.124355 | 13.5 | 53.919 | 0.097555 | 24.58357 | 0.027880665 |
| 31 | 0.125 | 14.27383 | 64.7 | 0.267493 | 67.40745 | 0.076844498 |
| 32 | 0.066 | 12.9345 | 46.537 | 0.106264 | 26.77822 | 0.016118345 |
| 33 | 0.082366 | 12 | 35.361 | 0.025258 | 6.364942 | 0.004781204 |
| 34 | 0.124833 | 14.13302 | 62.821 | 0.208424 | 52.52224 | 0.065533216 |
| 35 | 0.115972 | 14.5 | 68.02 | 0.351063 | 88.46685 | 0.093568259 |
| 36 | 0.036977 | 12 | 35.328 | 0.153633 | 38.71507 | 0.013055891 |
| 37 | 0.066536 | 14 | 60.984 | 0.276992 | 69.80117 | 0.042355933 |
| 38 | 0.115403 | 14.5 | 68.081 | 0.291776 | 73.5267 | 0.077385039 |
| 39 | 0.087432 | 14.00118 | 60.736 | 0.199524 | 50.27946 | 0.046119953 |
| 40 | 0.106282 | 13.49823 | 53.844 | 0.233256 | 58.77983 | 0.05697481 |
| 41 | 0.123375 | 14.15004 | 62.812 | 0.293364 | 73.92687 | 0.083181036 |
| 42 | 0.072075 | 14.492 | 67.649 | 0.272292 | 68.61679 | 0.048416336 |
| 43 | 0.067177 | 14.00765 | 60.664 | 0.209733 | 52.8521 | 0.032380065 |
| 44 | 0.110513 | 12.11331 | 36.53 | 0.10991 | 27.697 | 0.027915211 |
| 45 | 0.120569 | 14.34578 | 66.173 | 0.279088 | 70.32936 | 0.077333411 |
| 46 | 0.089661 | 14.12967 | 55.222 | 0.258843 | 65.22768 | 0.053337216 |
| 47 | 0.099916 | 13.9295 | 60.131 | 0.341064 | 85.94713 | 0.078317941 |
| 48 | 0.125 | 14.5 | 68.339 | 0.285321 | 71.90006 | 0.087711599 |
| 49 | 0.027135 | 13.5 | 54.216 | 0.127316 | 32.08326 | 0.007939683 |
| 50 | 0.096834 | 13.9295 | 60.131 | 0.337922 | 85.15536 | 0.075202916 |
| 51 | 0.039347 | 13.56446 | 54.622 | 0.263359 | 66.3657 | 0.023814975 |
| 52 | 0.078418 | 14.5 | 67.875 | 0.315884 | 79.60184 | 0.056929023 |
| 53 | 0.099858 | 14.5 | 69.733 | 0.396378 | 99.88607 | 0.090966741 |
| 54 | 0.11909 | 14.1 | 62.209 | 0.247817 | 62.44916 | 0.067826082 |
| 55 | 0.082793 | 14.2 | 63.659 | 0.298597 | 75.24557 | 0.056815836 |
| 56 | 0.088865 | 14.5 | 68.005 | 0.339088 | 85.44918 | 0.07231565 |
| 57 | 0.068636 | 14.41542 | 66.722 | 0.282448 | 71.17607 | 0.044553396 |
| 58 | 0.075903 | 14 | 60.802 | 0.192995 | 48.63418 | 0.033666296 |
| 59 | 0.084141 | 14.2 | 63.801 | 0.211859 | 53.38785 | 0.040968016 |
| 60 | 0.10001 | 11.43321 | 29.232 | 0.099703 | 25.12486 | 0.022916168 |
| 61 | 0.099987 | 13.437396 | 53.422 | 0.25504 | 64.26933 | 0.058606013 |
| 62 | 0.095871 | 13.537396 | 55.717 | 0.260815 | 65.72462 | 0.057465893 |

Reaction condition: compound 2a (0.1 M in EtOAc); reagent 3a (0.2 M in EtOAc); 1 mL reactor volume.

Graph: We have plotted 3D graph of optimization table S4, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S15. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated C-O bond formation reaction. Compound 2a (0.1 M in EtOAc); reagent 3a (0.2 M in EtOAc); 1 mL reactor volume.

Figure S16. Reproducibility test for the AI based system to auto-optimization for the photo activated O-H reaction. Compound 2a (0.1 M in EtOAc); reagent 3a (0.2 M in EtOAc); 1 mL reactor volume.

Figure S17. AI based system navigated multi-numerical variable complexity and identify the optimal conditions for the photo activated O-H reaction. Compound 2a (0.1 M in EtOAc); reagent 3a (0.2 M in EtOAc); 1 mL reactor volume.

S3.3. Procedure for the synthesis, of compound 4a for longer time.

Figure S18. Schematic presentation of continuous flow for the synthesis, of compound 4a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in ethyl acetate (100 mL) charged in one syringe, another syringe charged with the reagent 3a (2.56 g, 0.02 mol) dissolved in ethyl acetate (100 mL). These two syringes connected via pump and out-put is further connected with the T1-mixer. Both syringes were running with the 50 L/min. flow rate each to maintain the stoichiometry and then passed through PFA tubular reactor (id = 1000 µm, l = 1.3 m, vol. = 1 mL) under blue light (70 W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. Then out coming first one hour of the product mixture 4a was discarded and next 5 h of the product mixture [30 mL; 2a (0.32 g)] was collected in HPLC bottle. The EtOAc reaction mixture washed it with 10% aq. NaOH and separated through the separating funnel. The organic EtOAc phase was concentrated under reduced pressure to give the product 4a (0.45 g in 5 h, 99.88%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 8.12 – 8.10 (m, 2H), 7.60 – 7.51 (m, 3H), 7.47 – 7.38 (m, 4H), 6.15 (s, 1H), 3.74 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 168.97, 165.71, 135.36, 133.65, 132.55, 129.99, 129.14, 129.03, 128.55, 74.14, 52.82. IR (max): 3023.51, 1754.46, 1724.80, 1255.08, 815.32 cm-1. HRMS (ESI): m/z calcd for C16H14ClO4 [M+H]+ 305.7335, found 305.7332.

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 70W = 70 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=70 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.6 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.6 ×10246.02214× 1023

Mole’s photons incident  =4.38 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.45 g.

ofMole’s of product formed=0.45304

Mole’s of product formed =1.5 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=1.5 ×10-34.38 ×100

Quantum efficiency ∅:=0.34 %

Productivity per h:=0.455= 0.09 g/h

Space time yield :=productivityvolume of reactor

=0.091=0.09 g mL-1h-1

Space time yield in previous reported batch process2

Productivity = 33 mg in 12 h

Productivity per h:=0.03312= 0.00275 g/h

Space time yield :=productivityvolume of reactor

=0.00284=0.0007 g mL-1h-1

Table S5: Comparative table for the carbene insertion into O-H bonds of carboxylic acid.

| Entry | Compound 2a | Reagent 3a | Product 4a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 99.8%, 10 min. (our study) Blue light, overnight, 91% 2 Guanidine (5 mol%), Rh(OAC)4 (2.5 mol%), 3 h, 89% 3 |

S3.5. General procedure for scope of substrate for carbene insertion into the O-H bonds of carboxylic acid for synthesis of compound 4b4p.

Figure S19. Scope of substrate for the photo activated O-H reaction. Compound 2 (0.1 M in EtOAc); reagent 3 (0.2 M in EtOAc); 1 mL reactor volume.

Example 2. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2,4,5-trifluorobenzoate (4b):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3b (2,4,5-trifluorobenzoic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4b (0.48 g in 5 h, 90%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.90 – 7.83 (m, 1H), 7.51 – 7.49 (m, 2H), 7.42 – 7.39 (m, 2H), 7.07 – 7.01 (m, 1H), 6.13 (s, 1H), 3.76 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 168.57, 161.79, 159.44, 157.34, 154.87, 152.81, 147.69, 145.64, 135.69, 131.89, 129.31, 129.04, 120.50, 120.33, 107.62, 107.46, 107.40, 107.23, 74.78, 53.10. 19F NMR (376 MHz, CDCl3) δ -107.92, -107.95, -107.96, -107.99, -123.52, -123.55, -123.59, -123.61, -140.79, -140.85, -140.90.IR (max): 3021.92, 2925.33, 1746.25, 1623.42, 1518.64, 1216.49, 824.87 cm-1. HRMS (ESI): m/z calcd for C16H11ClF3O4 [M+H]+ 359.0292, found 359.0298.

Example 3. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzo[d][1,3]dioxole-5-carboxylate (4c):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3c (benzo[d][1,3]dioxole-5-carboxylic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4c (0.45 g in 5 h, 86%) as a colorless oil 1H NMR (400 MHz, CDCl3) δ 7.74 – 7.72 (m, 1H), 7.52 – 7.49 (m, 3H), 7.41 – 7.39 (m, 2H), 6.85 (d, J = 8.2 Hz, 1H), 6.10 (s, 1H), 6.05 (s, 2H), 3.75 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 169.13, 165.13, 152.34, 147.95, 135.44, 132.66, 131.09, 129.21, 129.08, 126.16, 123.01, 109.85, 108.24, 102.06, 74.15, 52.90. IR (max): 2955.71, 2920.24, 1754.04, 1718.03, 1256.96, 1036.39, 831.60 cm-1. HRMS (ESI): m/z calcd for C17H14ClO6 [M+H]+ 349.0473, found 349.0475.

Example 4. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-isobutylbenzoate (4d):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3d (4-isobutylbenzoic acid).  The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (10% ethylacetate/hexane); to give compound 4d (0.46 g in 5 h, 86%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 8.04 – 8.01 (m, 2H), 7.53 – 7.51 (m, 2H), 7.39 – 7.37 (m, 2H), 7.22 (d, J = 8.3 Hz, 2H), 6.14 (s, 1H), 3.73 (s, 3H), 2.52 (d, J = 7.2 Hz, 2H), 1.91 – 1.83 (m, 1H), 0.89 (d, J = 6.6 Hz, 6H). 13C NMR (101 MHz, CDCl3) δ 169.02, 165.73, 148.11, 135.25, 132.68, 129.90, 129.27, 129.07, 129.00, 126.57, 73.97, 52.71, 45.42, 30.12, 22.29. IR (max): 3022.14, 2960.22, 1753.90, 1721.69, 1492.80, 1215.16, 1016.94 cm-1. HRMS (ESI): m/z calcd for C20H22ClO4 [M+H]+ 361.1201, found 361.1209.

Example 5. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-phenoxybenzoate (4e):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3e (4-phenoxybenzoic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.6 (10% ethylacetate/hexane); to give compound 4e (0.49 g in 5 h, 83%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 8.11 – 8.08 (m, 2H), 7.54 – 7.52 (m, 2H), 7.42 – 7.38 (m, 4H), 7.22 – 7.18 (m, 1H), 7.09 – 7.00 (m, 4H), 6.15 (s, 1H), 3.76 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 169.06, 165.19, 162.51, 155.48, 135.36, 132.65, 132.22, 130.13, 129.15, 129.03, 124.72, 123.25, 120.23, 117.40, 74.05, 52.83. IR (max): 3022.03, 1754, 1720.35, 1491.57, 1215.73, 865.28 cm-1. HRMS (ESI): m/z calcd for C22H18ClO5 [M+H]+ 397.0837, found 397.0847.

Example 6. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzofuran-2-carboxylate (4f):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3f (benzofuran-2-carboxylic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4f (0.45 g in 5 h, 87%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.67 – 7.65 (m, 2H), 7.57 – 7.51 (m, 3H), 7.45 – 7.37 (m, 3H), 7.30 – 7.25 (m, 1H), 6.19 (s, 1H), 3.75 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 168.53, 158.52, 156.04, 144.36, 135.54, 132.01, 129.17, 129.11, 128.12, 126.81, 123.99, 123.02, 115.41, 74.21, 52.94. IR (max): 2956.04, 2854.56, 1730.43, 1566.86, 1172.31, 888.28 cm-1. HRMS (ESI): m/z calcd for C18H14ClO5 [M+H]+ 345.0524, found 345.0531.

Example 7. Methyl 2-(4-chlorophenyl)-2-(2-(2,4,5-trifluorophenyl)acetoxy)acetate (4g):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3g (2-(2,4,5-trifluorophenyl)acetic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (10% ethylacetate/hexane); to give compound 4g (0.48 g in 5 h, 87%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.39 – 7.34 (m, 4H), 7.22 – 7.15 (m, 1H), 6.97 – 6.90 (m, 1H), 5.94 (s, 1H), 3.77 (d, J = 3.8 Hz, 2H), 3.71 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 169.27, 168.63, 157.40, 157.32, 154.95, 154.88, 150.95, 150.82, 150.69, 148.46, 148.32, 148.20, 148.00, 147.87, 145.60, 145.57, 145.47, 145.44, 135.54, 131.94, 129.17, 128.99, 119.33, 119.28, 119.13, 119.09, 117.10, 117.05, 117.00, 116.92, 116.87, 116.82, 105.83, 105.62, 105.55, 105.34, 74.36, 52.89, 33.32. 19F NMR (471 MHz, CDCl3) δ -118.27, -118.30, -134.23, -134.27, -142.50, -142.54, -142.57. IR (max): 3024.90, 2958.73, 1749.06, 1633.89, 1215.42, 979.95, 850.85. HRMS (ESI): m/z calcd for C17H13ClF3O4 [M+H]+ 373.0449, found 373.0456.

Example 8. Methyl 2-(4-chlorophenyl)-2-(2-(4-isobutylphenyl)acetoxy)acetate (4h):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3h (2-(4-isobutylphenyl)acetic acid).  The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (10% ethylacetate/hexane); to give compound 4h (0.49 g in 5 h, 88%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.37 – 7.32 (m, 4H), 7.20 (d, J = 8.1 Hz, 2H), 7.10 (d, J = 8.1 Hz, 2H), 5.91 (s, 1H), 3.74 (d, J = 3.8 Hz, 2H), 3.69 (s, 3H), 2.45 (d, J = 7.2 Hz, 2H), 1.89 – 1.79 (m, 1H), 0.89 (d, J = 6.6 Hz, 6H). 13C NMR (101 MHz, CDCl3) δ 171.00, 168.85, 140.75, 135.27, 132.28, 130.43, 73.92, 52.76, 45.09, 40.47, 30.23, 22.39. IR (max): 3020.64, 2957.88, 1748.49, 1492.75, 1215.51, 1016.03 cm-1. HRMS (ESI): m/z calcd for C21H24ClO4 [M+H]+ 375.8685, found 375.8690.

Example 9. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2-(4-isobutylphenyl)propanoate (4i):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3i (2-(4-isobutylphenyl)propanoic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4i (0.49 g in 5 h, 85%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.31 (d, J = 1.5 Hz, 2H), 7.27 – 7.19 (m, 4H), 7.13 – 7.06 (m, 2H), 5.89 (d, J = 3.3 Hz, 1H), 3.90 – 3.83 (m, 1H), 3.65 (d, J = 31.5 Hz, 3H), 2.44 (dd, J = 10.1, 7.2 Hz, 2H), 1.89 – 1.79 (m, 1H), 1.56 (dd, J = 7.2, 2.0 Hz, 3H), 0.89 (t, J = 6.8 Hz, 6H). 13C NMR (101 MHz, CDCl3) δ 173.89, 173.82, 168.94, 168.74, 140.78, 140.71, 137.02, 136.92, 135.18, 135.08, 132.44, 132.39, 129.40, 129.33, 128.98, 128.91, 128.82, 128.71, 127.46, 127.37, 73.90, 73.71, 52.72, 52.59, 45.10, 45.06, 44.96, 44.82, 30.24, 22.41, 18.49, 18.35. IR (max): 3020.99, 2956.85, 2872.11, 1744.81, 1599.53, 1215.33, 1016.16 cm-1. HRMS (ESI): m/z calcd for C22H26ClO4 [M+H]+ 389.1514, found 389.1520.

Example 10. Methyl 2-(2-([1,1'-biphenyl]-4-yl)acetoxy)-2-(4-chlorophenyl)acetate (4j):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3j (2-([1,1'-biphenyl]-4-yl)acetic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4j (0.52 g in 5 h, 87%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.58 – 7.55 (m, 4H), 7.45 – 7.32 (m, 9H), 5.94 (s, 1H), 3.87 – 3.77 (m, 2H), 3.70 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.82, 168.84, 140.74, 140.29, 135.36, 132.22, 132.18, 129.83, 129.07, 128.94, 128.81, 127.38, 127.09, 74.04, 52.80, 40.45. IR(max): 3024.43, 1744.86, 1489.15, 1215.88, 1013.88 cm-1. HRMS (ESI): m/z calcd for C23H20ClO4 [M+H]+ 395.1045, found 395.1055.

Example 11. Methyl 2-(2-(9H-fluoren-9-yl)acetoxy)-2-(4-chlorophenyl)acetate (4k):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3k (2-(9H-fluoren-9-yl)acetic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.45 (10% ethylacetate/hexane); to give compound 4k (0.55 g in 5 h, 91%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.71 (d, J = 7.5 Hz, 2H), 7.50 (dd, J = 10.7, 7.6 Hz, 2H), 7.43 – 7.29 (m, 8H), 6.04 (s, 1H), 4.49 (t, J = 6.9 Hz, 1H), 3.78 (s, 3H), 3.01 (dd, J = 16.5, 6.8 Hz, 1H), 2.90 (dd, J = 16.5, 7.1 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 171.66, 168.85, 145.93, 140.86, 140.81, 135.32, 132.15, 129.05, 129.01, 127.59, 127.30, 127.25, 124.46, 120.00, 74.07, 52.83, 43.37, 38.15. IR (max): 3019.04, 2922.32, 1741.47, 1489.45, 1216.03, 1144.72 cm-1. HRMS (ESI): m/z calcd for C24H20ClO4 [M+H]+ 407.1045, found 407.1053.

Example 12. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (3R)-3-methoxy-3-phenylpropanoate (4l):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3l ((R)-3-methoxy-3-phenylpropanoic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.6 (10% ethylacetate/hexane); to give compound 4l (0.44 g in 5 h, 82%) as a colorless oil.  1H NMR (400 MHz, CDCl3) δ 7.50 – 7.48 (m, 2H), 7.41 – 7.33 (m, 7H), 5.93 (s, 1H), 4.90 (s, 1H), 3.59 (s, 3H), 3.49 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.10, 168.34, 135.70, 135.47, 132.02, 129.14, 129.00, 128.96, 128.69, 127.46, 82.49, 74.15, 57.71, 52.76.  IR (max): 2929.68, 1750.89, 1598.38, 1217.54, 1164.43, 1016.87, 828.38 cm-1. HRMS (ESI): m/z calcd for C22H18ClO5 [M+H]+ 363.0994, found 363.0999.

Example 13. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-methylisoxazole-3-carboxylate (4m):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3m (5-methylisoxazole-3-carboxylic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.6 (10% ethylacetate/hexane); to give compound 4m (0.38 g in 5 h, 82%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.87 – 7.83 (m, 2H), 7.77 – 7.68 (m, 2H), 6.83 (d, J = 0.8 Hz, 1H), 6.50 (s, 1H), 4.12 (s, 3H), 2.86 (d, J = 0.7 Hz, 3H). 13C NMR (101 MHz, CDCl3) δ 171.80, 168.21, 159.34, 155.55, 135.69, 131.62, 129.25, 129.15, 128.83, 128.04, 111.21, 102.55, 74.70, 72.26, 53.07, 12.40. IR (max): 3021.10, 2957.90, 1746.23, 1598.19, 1207.20, 1006.59, 808.66 cm-1. HRMS (ESI): m/z calcd for C14H13ClNO5 [M+H]+ 310.0477, found 310.0478.

Example 14. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-acetylthiophene-2-carboxylate (4n):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3n (5-acetylthiophene-2-carboxylic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.6 (10% ethylacetate/hexane); to give compound 4n (0.47 g in 5 h, 90%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.84 (d, J = 4.0 Hz, 1H), 7.64 (d, J = 4.0 Hz, 1H), 7.49 – 7.46 (m, 2H), 7.40 – 7.37 (m, 2H), 6.09 (s, 1H), 3.74 (s, 3H), 2.58 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 190.79, 168.43, 160.69, 149.64, 138.16, 135.61, 134.49, 131.85, 131.76, 129.22, 128.99, 74.54, 53.00, 27.07. IR (max): 3021.89, 1724.77, 1673.48, 1524.47, 1214.90, 1017.28 cm-1. HRMS (ESI): m/z calcd for C16H14ClO5S [M+H]+ 353.0245, found 353.0250.

Example 15. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-chlorothiophene-2-carboxylate (4o):

The title compound was synthesised following the general procedure described in  section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3o (5-chlorothiophene-2-carboxylic acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4o (0.47 g in 5 h, 91%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.68 (d, J = 4.1 Hz, 1H), 7.48 – 7.46 (m, 2H), 7.40 – 7.37 (m, 2H), 6.95 (d, J = 5.0 Hz, 1H), 6.08 (s, 1H), 3.74 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 168.53, 160.11, 138.53, 135.47, 134.15, 132.03, 130.39, 129.13, 128.95, 127.54, 74.20, 52.87. IR (max): 3022.57, 1755.23, 1717.81, 1422.40, 1215.11, 1089.70 cm-1. HRMS (ESI): m/z calcd for C14H11Cl2O4S [M+H]+ 344.9750, found 344.9756.

Example 16. 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (1s,3s)-adamantane-1-carboxylate (4p):

The title compound was synthesised following the general procedure described in section 3.3, and involve corresponding reactant exchange with compound 2a and compound 3p (admantyl acid). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (10% ethylacetate/hexane); to give compound 4p (0.48 g in 5 h, 88%) as viscous oil. 1H NMR (400 MHz, CDCl3) δ 7.42 – 7.40 (m, 2H), 7.35 – 7.32 (m, 2H), 5.88 (s, 1H), 3.68 (s, 3H), 2.02 (s, 3H), 1.97 (s, 6H), 1.71 (s, 6H). 13C NMR (101 MHz, CDCl3) δ 176.70, 169.12, 135.10, 132.72, 129.00, 128.80, 73.24, 52.65, 40.75, 38.69, 36.46, 27.90. IR (max): 2907.80, 2853.75, 1754.93, 1730.89, 1215.77, 1072.97 cm-1. HRMS (ESI): m/z calcd for C20H24ClO4 [M+H]+ 363.1358, found 363.1364.

S4. Case study-2: Carbene insertion (C-S bond formation) single objective multi-variant auto-optimization.

S4.1 Background collection

Sample Preparation: We prepared 0.5 M stock solutions of compound 2a, reagent 5a, and product 6a in ethyl acetate, each loaded into separate syringes. For our analysis, we utilized an In-line FTIR system. The experimental protocol commenced with the initial introduction of ethyl acetate solvent for 10 minutes using a syringe pump, during which data was recorded. Subsequently, the stock solution of compound 2a was introduced into the In-line FTIR for another 10 minutes. This procedure was repeated for the reaction product 6a and reagent 5a, with each pumped for 10 minutes. After collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for enhanced outcomes.

Figure S20. In-line IR background analysis of EtOAc, 0.5 M stock solution of 2a, 5a, and 6a in EtOAc.

The distinctive shifting peak associated with product (6a) aligning within the range of 2070 to 2085 cm-1 was observed. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S4.2. General procedure for the auto-optimized synthesis of compound 6a.

In our approach we are employing a Python-coded based Bayesian optimization strategy for further refinement. Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in EtOAc) and reagent 5a (0.2 M in EtOAc), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 70-80 experiments and the results were tabulated in the optimization table (Table S6).

S4.2.1. Python code for optimizing condition of carbene insertion reaction (C-S bond formation).

Table S6. Auto-optimization table of carbene insertion reaction into S-H bonds of thiophenol.

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Spacetime yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.12187 | 14.0141 | 61.3 | 0.639877 | 62.1628401 | 0.066363879 |
| 2 | 0.15077 | 14.1614 | 63.583 | 0.725702 | 70.500557 | 0.093113272 |
| 3 | 0.14389 | 14.3314 | 65.894 | 0.771888 | 74.987459 | 0.094519922 |
| 4 | 0.17094 | 14.5 | 65.85 | 0.94132 | 91.4474572 | 0.136936568 |
| 5 | 0.1 | 12.3572 | 39.777 | 0.536517 | 52.1216116 | 0.045658532 |
| 6 | 0.16593 | 14.0518 | 62.246 | 0.886884 | 86.1591028 | 0.125236288 |
| 7 | 0.2 | 14.5 | 68.778 | 0.790801 | 76.8248211 | 0.134597087 |
| 8 | 0.1894 | 14.5 | 68.778 | 0.910954 | 88.4974578 | 0.146830026 |
| 9 | 0.17392 | 14.5 | 68.778 | 0.978956 | 95.1037234 | 0.144894251 |
| 10 | 0.2 | 14 | 60.96 | 0.672707 | 65.3522124 | 0.114497076 |
| 11 | 0.175 | 14.5 | 68.3 | 0.97634 | 94.849584 | 0.145404412 |
| 12 | 0.14684 | 14.5 | 68.213 | 0.909719 | 88.3774799 | 0.113681579 |
| 13 | 0.16295 | 14.5 | 68.169 | 0.953319 | 92.6131374 | 0.132199882 |
| 14 | 0.2 | 14 | 61.002 | 0.758361 | 73.6733365 | 0.129075686 |
| 15 | 0.15126 | 14.5 | 68.368 | 0.919708 | 89.3478923 | 0.118389317 |
| 16 | 0.13402 | 14.2839 | 66.944 | 0.797798 | 77.5045665 | 0.090991539 |
| 17 | 0.15638 | 14.0233 | 61.295 | 0.669423 | 65.0331781 | 0.089088222 |
| 18 | 0.12526 | 14.0342 | 61.455 | 0.745254 | 72.4000163 | 0.079442916 |
| 19 | 0.1 | 14.5 | 68.281 | 0.900546 | 87.4863403 | 0.076638034 |
| 20 | 0.11313 | 14.0352 | 61.319 | 0.61147 | 59.4031537 | 0.058869642 |
| 21 | 0.1 | 11.0324 | 25.032 | 0.206977 | 20.107424 | 0.017614103 |
| 22 | 0.16846 | 14.0379 | 61.305 | 0.851698 | 82.7408495 | 0.122101466 |
| 23 | 0.18174 | 12.3572 | 39.777 | 0.205725 | 19.9857946 | 0.031818232 |
| 24 | 0.18955 | 11.7409 | 32.482 | 0.466023 | 45.2732528 | 0.075174335 |
| 25 | 0.13922 | 14.1921 | 63.566 | 0.75473 | 73.3205917 | 0.089419389 |
| 26 | 0.14375 | 14.5 | 67.904 | 0.870399 | 84.5576163 | 0.106479178 |
| 27 | 0.18405 | 10.0002 | 15.42 | 0.103039 | 10.0100439 | 0.016138974 |
| 28 | 0.2 | 14 | 60.844 | 0.794354 | 77.1699884 | 0.13520182 |
| 29 | 0.18552 | 14.3822 | 66.263 | 0.894818 | 86.9298759 | 0.14127454 |
| 30 | 0.10265 | 14.104 | 62.297 | 0.769329 | 74.738857 | 0.067206227 |
| 31 | 0.16804 | 14.4279 | 66.956 | 0.89149 | 86.606567 | 0.127487499 |
| 32 | 0.17293 | 14.5 | 68.961 | 0.977279 | 94.9408061 | 0.143822675 |
| 33 | 0.1 | 14.4738 | 67.545 | 0.691974 | 67.2239651 | 0.058888193 |
| 34 | 0.12095 | 14.0335 | 61.184 | 0.622655 | 60.4897553 | 0.064090227 |
| 35 | 0.12869 | 14.3928 | 66.309 | 0.896598 | 87.1027996 | 0.098193111 |
| 36 | 0.1 | 13.466 | 54.12 | 0.584978 | 56.8295061 | 0.049782647 |
| 37 | 0.1653 | 14.5 | 67.41 | 0.931312 | 90.475199 | 0.131010621 |
| 38 | 0.1583 | 14.0035 | 60.003 | 0.89358 | 86.8096066 | 0.120379576 |
| 39 | 0.17399 | 14.0821 | 61.355 | 0.855364 | 83.0969945 | 0.126652484 |
| 40 | 0.1736 | 14.0805 | 61.186 | 0.830028 | 80.63565 | 0.122625536 |
| 41 | 0.19014 | 14.052 | 60.143 | 0.717789 | 69.7318435 | 0.116147199 |
| 42 | 0.17339 | 14.4756 | 68.784 | 1.008769 | 97.9786321 | 0.148819392 |
| 43 | 0.10246 | 14.0741 | 61.785 | 0.789538 | 76.6853989 | 0.068828949 |
| 44 | 0.1083 | 13.3572 | 52.066 | 0.597804 | 58.0628649 | 0.055084704 |
| 45 | 0.15031 | 13.3544 | 52 | 0.649264 | 63.0610165 | 0.083033424 |
| 46 | 0.16092 | 14.4505 | 67.288 | 0.928714 | 90.203136 | 0.127155681 |
| 47 | 0.18975 | 11.1796 | 26.503 | 0.190577 | 18.5101582 | 0.03076777 |
| 48 | 0.1005 | 10 | 15.48 | 0.061864 | 6.00866015 | 0.005289904 |
| 49 | 0.18795 | 11.6288 | 30.954 | 0.394747 | 38.3405627 | 0.063125513 |
| 50 | 0.16037 | 10.909 | 23.563 | 0.227011 | 22.0488806 | 0.030975176 |
| 51 | 0.19911 | 12.9452 | 46.563 | 0.687769 | 66.8008888 | 0.116514351 |
| 52 | 0.19707 | 13.7912 | 57.233 | 0.794465 | 77.1639433 | 0.133210677 |
| 53 | 0.10353 | 13.9953 | 60.109 | 0.823241 | 79.9588677 | 0.07251652 |
| 54 | 0.1882 | 13.6429 | 54.882 | 0.765239 | 74.3253118 | 0.122535087 |
| 55 | 0.11839 | 13.1852 | 49.839 | 0.751321 | 72.9734992 | 0.075680553 |
| 56 | 0.19909 | 14.499 | 68.131 | 0.903623 | 87.7661243 | 0.153066613 |
| 57 | 0.10173 | 10.0506 | 15.849 | 0.092939 | 9.02687938 | 0.008044347 |
| 58 | 0.11477 | 12.3255 | 39.218 | 0.464055 | 45.0722357 | 0.045314959 |
| 59 | 0.12176 | 12.3888 | 39.803 | 0.521891 | 50.6896686 | 0.054066493 |
| 60 | 0.19812 | 12.2467 | 38.122 | 0.419045 | 40.7005527 | 0.070637079 |
| 61 | 0.14555 | 12.194 | 36.286 | 0.272067 | 26.4250314 | 0.033692391 |
| 62 | 0.18235 | 12.5432 | 39.924 | 0.590353 | 57.3391722 | 0.091592791 |
| 63 | 0.18723 | 12.0402 | 34.157 | 0.234019 | 22.7295461 | 0.03727952 |
| 64 | 0.2 | 12.29 | 37.001 | 0.427683 | 41.5395351 | 0.072777266 |
| 65 | 0.12925 | 10.4935 | 19.748 | 0.193939 | 18.8366989 | 0.021327476 |
| 66 | 0.10105 | 10.525 | 20.027 | 0.360808 | 35.0441719 | 0.031021031 |
| 67 | 0.16517 | 14.4505 | 67.288 | 0.895807 | 87.0069803 | 0.12588946 |
| 68 | 0.14869 | 14.3822 | 66.263 | 0.842513 | 81.8306979 | 0.106586481 |
| 69 | 0.13311 | 13.3572 | 52.066 | 0.573379 | 55.6905397 | 0.064937597 |
| 70 | 0.10145 | 10.5483 | 20.261 | 0.195224 | 18.961507 | 0.016851129 |
| 71 | 0.17331 | 10.5219 | 20.043 | 0.288514 | 28.0224779 | 0.042543603 |
| 72 | 0.15263 | 10.5261 | 20.094 | 0.355223 | 34.5017181 | 0.046130136 |
| 73 | 0.1 | 13.466 | 54.12 | 0.692583 | 67.2684578 | 0.058927169 |
| 74 | 0.1 | 14.5 | 67.41 | 0.819158 | 79.5622985 | 0.069696573 |
| 75 | 0.19775 | 14.4028 | 66.182 | 0.858552 | 83.3885166 | 0.144453093 |
| 76 | 0.11677 | 10.5449 | 20.13 | 0.190198 | 18.4733471 | 0.018896483 |
| 77 | 0.12524 | 11.6656 | 31.411 | 0.290907 | 28.2549027 | 0.030998522 |
| 78 | 0.1297 | 14.4989 | 67.952 | 0.796329 | 77.3449879 | 0.08787721 |
| 79 | 0.17576 | 14.5 | 68.3 | 0.975133 | 94.7116708 | 0.145823544 |
| 80 | 0.14697 | 10.0002 | 15.42 | 0.118176 | 11.4780716 | 0.014777526 |
| 81 | 0.18865 | 11.7409 | 32.482 | 0.225615 | 21.9132914 | 0.036213336 |
| 82 | 0.11754 | 14.4997 | 67.304 | 0.77118 | 74.9023428 | 0.077123227 |
| 83 | 0.17818 | 10.0003 | 16.025 | 0.114992 | 11.1688195 | 0.017432928 |
| 84 | 0.11092 | 10 | 16.01 | 0.175406 | 17.0366456 | 0.016553813 |

Reaction condition: compound 2a (0.1 M in EtOAc); reagent 5a (0.2 M in EtOAc); 1 mL reactor volume.

Graph: We have plotted 3D graph of optimization table S6, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S21. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated S-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 5a (0.2 M in EtOAc); 1 mL reactor volume.

Figure S22. Reproducibility test for the AI based system to auto-optimization for the photo activated S-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 5a (0.2 M in EtOAc); 1 mL reactor volume.

Figure S23.  AI based system navigated multi-numerical variable complexity and identify the optimal condition for the phtoto activated S-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 5a (0.2 M in EtOAc); 1 mL reactor volume.

S4.3. General procedure of running AI-optimized condition for longer time for synthesis of compound (6a).

Figure S24. Schematic presentation of continuous flow for the synthesis, of compound 6a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in ethyl acetate (100 mL) charged in one syringe, another syringe charged with the reagent 5a (1.9 mL, 0.02 mol) dissolved in ethyl acetate (100 mL). These two syringes connected via pump and out-put is further connected with the T1-mixer and both syringes were running with the 173 L/min. flow rate to maintain the stoichiometry and then passed through PFA tubular reactor (id = 1000 µm, l = 1.3 m, vol. = 1 mL) under blue light (69W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. Then outcoming first one hour of the product mixture 6a was discarded and next 5 h of the product mixture [52 mL; 2a (0.546 g)] was collected in HPLC bottle. The EtOAc reaction mixture washed it with 10% aq. NaOH and separated through the separating funnel. The organic EtOAc phase was concentrated under reduced pressure to give the product 6a (0.74 g in 5 h, 97.98%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.37 – 7.33 (m, 4H), 7.29 – 7.25 (m, 5H), 4.86 (s, 1H), 3.67 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.55, 134.32, 133.16, 133.10, 129.98, 129.14, 128.89, 128.40, 55.73, 52.88. IR (max): 3021.35, 2952.59, 1737.18, 1487.9, 1214.19, 829.21 cm-1. HRMS (ESI): m/z calcd for C15H14ClO2S [M+H]+ 293.0398, found 293.0404. Verified the analytical data with those reported in the literature.4

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 69W = 69 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=69 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.6 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.6 ×10246.02214× 1023

Mole’s photons incident  =4.38 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.74 g.

ofMole’s of product formed=0.74292

Mole’s of product formed =2.5 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=2.5 ×10-34.38 ×100

Quantum efficiency ∅:=0.57 %

Productivity per h:=0.745= 0.148 g/h

Space time yield :=productivityvolume of reactor

=0.1481=0.148 g mL-1h-1

Space time yield in previous reported batch process 4

Productivity = 130 mg in 12 h

Productivity per h:=0.13012= 0.011 g/h

Space time yield :=productivityvolume of reactor

=0.0111=0.011 g mL-1 h-1

Figure S25. Scope of substrate for the photo activated S-H insertion reaction. Compound 2 (0.1 M in EtOAc); reagent 5 (0.2 M in EtOAc); 1 mL reactor volume.

S4.3.2. General procedure for scope of substrate for carbene insertion into the S−H bonds of organic aromatic thiol for synthesis of compound 6bh.

Example 18. Methyl 2-((4-bromophenyl)thio)-2-(4-chlorophenyl)acetate (6b).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2a and compound 5b (4-bromobenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6b (0.76 g in 5 h, 79%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.39 – 7.32 (m, 4H), 7.30 – 7.25 (m, 2H), 7.22 – 7.17 (m, 2H), 4.83 (s, 1H), 3.68 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 170.23, 137.70, 134.68, 132.24, 129.94, 128.98, 122.88, 55.62, 52.98. IR (max): 3020.98, 1736.81, 1483.63, 1214.01, 817.91, 667.71 cm-1. HRMS (ESI): m/z calcd. for C15H13BrClO2S [M+H]+ 370.9503, found 370.9509.

Example 19. Methyl 2-(4-bromophenyl)-2-((4-chlorophenyl)thio)acetate (6c).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2b and compound 5c (4-chlorobenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6c (0.75 g in 5 h, 78%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.42 (d, J = 8.4 Hz, 2H), 7.28 – 7.25 (m, 4H), 7.21 (d, J = 8.6 Hz, 2H), 4.81 (s, 1H), 3.66 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 170.15, 134.75, 134.59, 134.47, 131.91, 131.43, 130.26, 129.30, 122.66, 55.79, 52.96. IR (max): 3019.95, 2952.20, 1735.81, 1497.43, 1213.23, 817.44, 626.54 cm-1. HRMS (ESI): m/z calcd for C15H13BrClO2S [M+H]+ 370.9503, found 370.9510.

Example 20. Methyl 2-(4-bromophenyl)-2-((4-fluorophenyl)thio)acetate (6d).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2b and compound 5d (4-fluorobenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6d (0.74 g in 5 h, 81%) as a colorless oil. 1H NMR (500 MHz, CDCl3) δ 7.42–7.40 (m, 2H), 7.34 – 7.32 (m, 2H), 7.27– 7.24 (m, 2H), 6.96 – 6.92 (m, 2H), 4.77 (s, 1H), 3.64 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.25, 164.38, 161.90, 136.37, 136.29, 134.63, 131.81, 130.29, 127.85, 127.82, 122.52, 116.34, 116.13, 56.26, 52.83. 19F NMR (471 MHz, CDCl3) δ -111.71.IR (max): 3021.30, 1737.68, 1437.42, 1216.36, 828.38, 668.88 cm-1. HRMS (ESI): m/z calcd for C15H13BrFO2S [M+H]+ 354.9798, found 354.9802.

Example 21. Methyl 2-(4-bromophenyl)-2-(p-tolylthio)acetate (6e).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2b and compound 5e (4-methylbenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6e (0.77 g in 5 h, 85%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.42 (d, J = 8.5 Hz, 2H), 7.30 – 7.24 (m, 4H), 6.79 (d, J = 8.9 Hz, 2H), 4.70 (s, 1H), 3.77 (s, 3H), 3.66 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.57, 160.44, 136.48, 135.02, 131.72, 130.36, 123.05, 122.33, 114.63, 56.67, 55.36, 52.75. IR (max): 3020.59, 1735.43, 1437.72, 1214.76, 1010.62, 826.89 cm-1. HRMS (ESI): m/z calcd for C16H16BrO2S [M+H]+ 351.0049, found 351.0055.

Example 22. Methyl 2-(4-bromophenyl)-2-((3-methoxyphenyl)thio)acetate (6f).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2b and compound 5f (3-methoxybenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6f (0.74 g in 5 h, 79%) as a yellow oil. 1H NMR (500 MHz, CDCl3) δ 7.44 –7.42 (m, 2H), 7.31–7.30 (m, 2H), 7.18 – 7.14 (m, 1H), 6.93 – 6.91 (m, 1H), 6.86 – 6.85 (m, 1H), 6.80 – 6.78 (m, 1H), 4.87 (s, 1H), 3.71 (s, 3H), 3.67 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.39, 159.74, 134.82, 134.33, 131.82, 130.25, 129.89, 124.70, 122.47, 117.58, 114.29, 55.56, 55.26, 52.90. IR (max): 3019.75, 1737.02, 1481.61, 1215.45, 669.75 cm-1. HRMS (ESI): m/z calcd for C16H16BrO3S [M+H]+ 366.9998, found 366.9993.

Example 23. Methyl 2-(4-bromophenyl)-2-(naphthalen-1-ylthio)acetate (6g).

The title compound was synthesised following the general procedure described in  section 4.3, and involve corresponding reactant exchange with compound 2b and compound 5g (3-methoxybenzenethiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6g (0.77 g in 6 h, 77%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.83 (d, J = 1.4 Hz, 1H), 7.77 – 7.74 (m, 1H), 7.72 – 7.69 (m, 2H), 7.50 – 7.42 (m, 3H), 7.41 – 7.38 (m, 2H), 7.33 – 7.31 (m, 2H), 4.95 (s, 1H), 3.64 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 170.47, 134.80, 133.56, 132.77, 132.22, 131.88, 130.50, 130.29, 129.72, 128.75, 127.74, 127.66, 126.74, 126.69, 122.55, 55.73, 52.92. IR (max): 3053.68, 2951.64, 1738.16, 1489.43, 1212.16, 634.22 cm-1. HRMS (ESI): m/z calcd for C19H16BrO2S [M+H]+ 387.0049, found 387.0053.

Example 24. Methyl 2-(4-chlorophenyl)-2-((1-phenyl-1H-tetrazol-5-yl)thio)acetate (6h).

The title compound was synthesised following the general procedure described in section 4.3, and involve corresponding reactant exchange with compound 2a and compound 5h (1-phenyl-1H-tetrazole-5-thiol). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 6h (0.70 g in 6 h, 75%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.56 – 7.49 (m, 5H), 7.45 – 7.42 (m, 2H), 7.32 – 7.29 (m, 2H), 5.78 (s, 1H), 3.76 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 169.17, 152.44, 135.38, 133.34, 132.35, 130.39, 129.93, 129.92, 129.38, 123.77, 54.07, 53.70. IR (max): 3018.80, 2955.24, 1741.03, 1216.96, 1009.64, 832.24 cm-1. HRMS (ESI): m/z calcd for C16H14ClN4O2S [M+H]+ 361.0521, found 361.0526

Table S7. Comparative result for the carbene insertion into S-H bonds of thiols.

| Entry | Compound 2a | Reagent 5a | Product 6a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 98%, 5.7 min. (our study) [(CH3CN)4Cu]PF6 (5 mol%), 12-24 h, 89% 4 Fe(OTf)2 (15 mol%), 24-48 h, 35 % 5 |

S5. Case study-3: Carbene insertion (C-N bond formation) single objective multi-variant auto-optimization.

S5.1. Background collection

Sample Preparation: We prepare 0.5 M stock solution of compound 2a in ethyl acetate, 0.5 M stock solution of reagent 7a in ethyl acetate and 0.5 M stock solution of product 8a in ethyl acetate. The three of the solution taken in three different syringes.

Delivery system: The HPLC pumps and syringe pump were directly interfaced to the main computer via an RS-232 interface or LAN. These pumps will help in introducing reactants and reagent solutions into the microreactor with varied flow rate as the central computer's prescribed flow rate (fr). The central computer and the HPLC pump communicated through serial communication using ASCII code to exchange information about flow rate, operational status, and duration. This facilitated the transmission of essential details from the main computer to the pumps, enabling them to commence their operations as required. To conduct our analysis, we utilized an In-line FTIR system. The experimental protocol involved the initial introduction of the solvent, ethyl acetate, for a duration of 10 min. using a syringe pump, during which data was recorded. Subsequently, a previously prepared stock solution of compound 2a was similarly introduced into the In-line FTIR for 10 min. This process was repeated for the reaction product 8a and reagent 7a, each pumped for 10 min. Upon collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure S26. In-line IR background analysis of EtOAc, 0.5 M stock solution of 2a, 7a, and 8a in EtOAc.

The distinctive C-N stretching associated with product (8a) aligning within the range of 1295 to 1325 cm-1 was observed. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S5.2. General procedure for the auto-optimized synthesis of compound 8a.

Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in EtOAc) and reagent 7a (0.3 M in EtOAc), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 40-44 experiments and the results were tabulated in the optimization table (Table S8).

S5.2.1. Python code for optimizing condition of carbene insertion reaction (C-N bond formation).

Table S8. Auto-optimization table of carbene insertion reaction (C-N bond formation).

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Spacetime yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.118968 | 12.38799 | 40.459 | 1.196524 | 71.6165410 | 0.0702906 |
| 2 | 0.093580 | 10.18322 | 17.197 | 0.279182 | 16.7101331 | 0.012900 |
| 3 | 0.111095 | 13.78164 | 59.183 | 1.514104 | 90.6249622 | 0.0830608 |
| 4 | 0.05 | 11.07727 | 25.851 | 0.628177 | 37.5988264 | 0.0155095 |
| 5 | 0.066191 | 14.23180 | 65.638 | 1.482614 | 88.7401704 | 0.0484595 |
| 6 | 0.061313 | 10.04277 | 15.985 | 0.343314 | 20.5487147 | 0.0103943 |
| 7 | 0.115638 | 13.65416 | 57.251 | 1.346334 | 80.5832479 | 0.0768775 |
| 8 | 0.069548 | 12.82883 | 17.25 | 0.234918 | 14.0607889 | 0.0080677 |
| 9 | 0.099196 | 11.69255 | 69.716 | 1.506151 | 90.1489571 | 0.0737751 |
| 10 | 0.093306 | 10.18078 | 17.163 | 0.291460 | 17.4450195 | 0.0134288 |
| 11 | 0.092505 | 14.3295 | 68.113 | 1.537078 | 92.0000251 | 0.0702116 |
| 12 | 0.051560 | 14.48697 | 69.75 | 1.456641 | 87.1855572 | 0.0370862 |
| 13 | 0.111049 | 13.75428 | 58.757 | 1.311133 | 78.4763496 | 0.0718964 |
| 14 | 0.118626 | 10.00002 | 15.663 | 0.210988 | 12.6248993 | 0.0123556 |
| 15 | 0.050115 | 10.06854 | 16.238 | 0.178406 | 10.6783159 | 0.0044149 |
| 16 | 0.062578 | 13.06313 | 49.698 | 1.021805 | 61.158672 | 0.0315746 |
| 17 | 0.100030 | 12.22525 | 38.894 | 1.029414 | 61.6143808 | 0.0508472 |
| 18 | 0.05 | 10.10525 | 16.638 | 0.240613 | 14.4016513 | 0.0059406 |
| 19 | 0.111160 | 13.82081 | 60.249 | 1.373565 | 82.2131677 | 0.0753953 |
| 20 | 0.093994 | 11.74773 | 33.39 | 0.921292 | 55.1429018 | 0.0427608 |
| 21 | 0.122704 | 14.31194 | 67.567 | 1.225070 | 73.3251847 | 0.0742279 |
| 22 | 0.057572 | 10.62465 | 21.511 | 0.980855 | 58.7079763 | 0.0278848 |
| 23 | 0.086404 | 14 | 62.807 | 1.494519 | 89.4526871 | 0.0637649 |
| 24 | 0.148922 | 11 | 25.125 | 0.133867 | 8.01247153 | 0.0098441 |
| 25 | 0.052426 | 12 | 36.174 | 1.244392 | 74.4816629 | 0.0322148 |
| 26 | 0.15 | 11 | 25.18 | 0.156175 | 9.34772818 | 0.0115678 |
| 27 | 0.05 | 10.0673 | 15.564 | 0.303881 | 18.1884792 | 0.0075027 |
| 28 | 0.05 | 10 | 15.604 | 0.378768 | 22.6707418 | 0.0093516 |
| 29 | 0.15 | 13 | 48.67 | 1.093202 | 65.4323299 | 0.0809725 |
| 30 | 0.063750 | 11.6785 | 32.683 | 1.165559 | 69.7631768 | 0.0366912 |
| 31 | 0.052816 | 12.9356 | 47.678 | 1.242195 | 74.3501624 | 0.0323970 |
| 32 | 0.05 | 13.3951 | 53.794 | 1.265565 | 75.7489373 | 0.0312464 |
| 33 | 0.055842 | 11 | 29.217 | 0.797093 | 47.7090838 | 0.0219795 |
| 34 | 0.101936 | 14.0832 | 67.409 | 1.504863 | 90.0718120 | 0.0757479 |
| 35 | 0.05 | 13.0828 | 49.985 | 0.773444 | 46.2935985 | 0.0190961 |
| 36 | 0.132422 | 11 | 26.081 | 0.482431 | 28.8753526 | 0.0315458 |
| 37 | 0.05 | 10.085 | 16.437 | 0.105391 | 6.30806718 | 0.0026020 |
| 38 | 0.067590 | 12.315 | 40.073 | 1.017212 | 60.8840568 | 0.0339503 |
| 39 | 0.089018 | 14 | 63.507 | 1.528104 | 91.4628977 | 0.0671702 |
| 40 | 0.053891 | 11.315 | 28.469 | 0.783364 | 46.8873703 | 0.0208464 |
| 41 | 0.139535 | 13.38552 | 58.881 | 1.272373 | 76.1564519 | 0.0876686 |
| 42 | 0.1282496 | 11.937 | 35.17 | 0.748672 | 44.8109424 | 0.0474126 |
| 43 | 0.15 | 13.2041 | 51.367 | 0.640692 | 38.3479029 | 0.0474555 |
| 44 | 0.068291 | 14 | 62.656 | 1.513058 | 90.562340 | 0.0510233 |

Reaction condition: compound 2a (0.1 M in EtOAc); reagent 7a (0.3 M in EtOAc); 1 mL reactor volume.

Graph: We have plotted 3D graph of optimization table S8, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S27. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated N-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 7a (0.3 M in EtOAc); 1 mL reactor volume.

Figure S28. Reproducibility test for the AI based system to auto-optimization for the photo activated N-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 7a (0.3 M in EtOAc); 1 mL reactor volume.

Figure S29. AI based system navigated multi-numerical variable complexity and identify the optimal conditions for the photo activated N-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 7a (0.3 M in EtOAc); 1 mL reactor volume.

S5.3. General procedure of running AI optimized condition for longer time for synthesis of compound 8a.

Figure S30. Schematic presentation of continuous flow for the synthesis, of compound 8a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in ethyl acetate (100 mL) charged in one syringe, another syringe charged with the reagent 7a (2.9 mL, 0.03 mol) dissolved in ethyl acetate (100 mL). These two syringes connected via pump and out-put is further connected with the T1-mixer and both syringes were running with the 46 L/min. each flow rate to maintain the stoichiometry and then passed through PFA tubular reactor (id = 1000 µm, l = 1.3 m, vol. = 1 mL) under blue light (68.02 W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. First one hour of the product mixture was discarded and next 5 h of the product mixture [28 mL; 2a (0.294 g)] was collected in HPLC bottle The organic EtOAc phase was concentrated under reduced pressure and purified through the regular batch process protocols to give the product 8a (0.35 g in 5 h, 92 %) as a colourless oil. 1H NMR (400 MHz, CDCl3) δ 7.43 – 7.40 (m, 2H), 7.30 – 7.27 (m, 2H), 7.11 – 7.07 (m, 2H), 6.71 – 6.67 (m, 1H), 6.52 – 6.49 (m, 2H), 5.03 (s, 1H), 5.00 (s, 1H), 3.69 (s, 3H).   13C NMR (126 MHz, CDCl3) δ 171.86, 145.67, 136.29, 134.17, 129.33, 129.10, 128.67, 118.37, 113.50, 60.12, 52.98. IR (max): 3418.02, 3020.09, 1740.08, 1214.36, 688.27 cm-1.  HRMS (ESI): m/z calcd for C15H15ClNO2 [M+H]+ 276.0786, found 276.0782. Verified the analytical data with those reported in the literature.6

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 68W = 68 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=68 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.58 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.58 ×10246.02214× 1023

Mole’s photons incident  =4.28 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.35 g.

Mole’s of product formed=0.35275

Mole’s of product formed =1.2 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=1.2 ×10-34.28 ×100

Quantum efficiency ∅:=0.28 %

Productivity per h:=0.295= 0.058 g/h

Space time yield :=productivityvolume of reactor

=0.0581=0.058 g mL-1 h-1

Table S9. Comparative result for the carbene insertion into N-H bonds of amines.

| Entry | Compound 2a | Reagent 5a | Product 6a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 92%, 11 min. (our study) Blue light, chiral spiro phosphamides (2 mol%), 1 h 6 White light, 80 min., 83% 1 |

Figure S31. Scope of substrate for the photo activated N-H insertion reaction. Compound 2a (0.1 M in EtOAc); reagent 7a (0.3 M in EtOAc); 1 mL reactor volume.

S5.3.1. General procedure for scope of substrate for carbene insertion into the N−H bonds of organic aromatic and aliphatic amines for synthesis of compound 8bj.

Example 26. Methyl 2-(benzylamino)-2-(4-chlorophenyl)acetate (8b).

The title compound was synthesized following the general procedure described in section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7b (benzyl amine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8b (0.74 g in 5 h, 89%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.34 – 7.22 (m, 9H), 4.36 (s, 1H), 3.70 (d, J = 6.2 Hz, 2H), 3.66 (s, 3H), 2.32 (s, 1H). 13C NMR (101 MHz, CDCl3) δ 173.02, 139.28, 136.61, 133.96, 129.05, 128.89, 128.51, 128.31, 127.27, 63.64, 52.36, 51.30. IR (max): 3340.60, 2951.52, 1736.67, 1489.77, 1169.20, 769.20 cm-1. HRMS (ESI): m/z calcd for C16H17ClNO2 [M+H]+ 290.0948, found 290.0967.

Example 27. Methyl 2-(4-chlorophenyl)-2-(dibenzylamino)acetate (8c).

The title compound was synthesized following the general procedure described in  section 5.3 and involved corresponding reactant exchange with compounds 2a and 7c (dibenzylamine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8c (0.79 g in 5 h, 74%) as a colourless liquid. 1H NMR (400 MHz, CDCl3) δ 7.34 – 7.21 (m, 14H), 4.57 (s, 1H), 3.77 (s, 3H), 3.73 (d, J = 2.9 Hz, 4H). 13C NMR (101 MHz, CDCl3) δ 172.28, 139.32, 135.32, 133.87, 130.26, 128.88, 128.69, 128.48, 127.28, 65.31, 54.35, 51.67. IR (max): 3024.08, 1736.91, 1451.25, 1213.67, 688.73 cm-1. HRMS (ESI): m/z calcd for C23H23ClNO2 [M+H]+ 380.1412, found 380.1408.

Example 28. Methyl 2-((5-bromopyrimidin-2-yl)amino)-2-(4-chlorophenyl)acetate (8d).

The title compound was synthesized following the general procedure described in  section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7d (5-bromopyrimidin-2-amine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8d (0.83 g in 5 h, 83%) as a yellow liquid. 1H NMR (500 MHz, CDCl3) δ 8.21 (s, 2H), 7.40 – 7.38 (m, 2H), 7.33 – 7.32 (m, 2H), 6.61 (s, 1H), 5.56 (d, J = 6.8 Hz, 1H), 3.74 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 171.30, 159.18, 158.36, 135.38, 134.50, 129.11, 128.84, 107.86, 58.01, 52.93. IR(max): 3251.65, 3001.76, 2952.73, 1742.79, 1577.46, 1429.20, 1170.80 cm-1. HRMS (ESI): m/z calcd for C13H12BrClN3O2 [M+H]+ 355.9796, found 355.9792.

Example 29. Methyl 2-(tert-butylamino)-2-(4-chlorophenyl)acetate (8e).

The title compound was synthesized following the general procedure described in  section 4.8.2 and involved corresponding reactant exchange with compound 2a and compound 7e (tert-butyl amine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8e (0.56 g in 5 h, 81%) as a colorless oil. 1H NMR (500 MHz, CDCl3) δ 7.37 – 7.27 (m, 4H), 4.46 (s, 1H), 3.68 (s, 3H), 2.03 (s, 1H), 1.08 (s, 9H). 13C NMR (101 MHz, CDCl3) δ 175.11, 139.37, 133.50, 128.76, 59.08, 52.58, 51.36, 29.52. IR (max): 3341.84, 2928.86, 1736.93, 1488.43, 1167.16, 823.38 cm-1. HRMS (ESI): m/z calcd for C13H19ClNO2 [M+H]+ 256.1099, found 256.1103.

Example 30. Methyl 2-(4-chlorophenyl)-2-(2-(pyridin-4-yl)-1H-benzo[d]imidazol-1-yl)acetate (8f).

The title compound was synthesized following the general procedure described in  section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7f (2-(pyridin-4-yl)-1H-benzo[d]imidazole). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8f (0.92 g in 5 h, 84%) as an off white solid and mp: 166-168 °C. 1H NMR (400 MHz, CDCl3) δ 8.77 (d, J = 6.0 Hz, 2H), 7.88 (d, J = 8.1 Hz, 1H), 7.56 – 7.55 (m, 2H), 7.37 – 7.33 (m, 3H), 7.25 – 7.23 (m, 1H), 7.15 – 7.09 (m, 3H), 6.37 (s, 1H), 3.76 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 168.01, 151.50, 150.58, 143.19, 137.46, 135.06, 134.64, 131.44, 129.23, 128.53, 124.32, 123.55, 123.51, 120.76, 120.51, 112.40, 61.38, 53.44. IR (max): 2958.15, 1747.03, 1411.75,1212.68, 1004.48 cm-1. HRMS (ESI): m/z calcd for C21H17ClN3O2 [M+H]+ 378.1007, found 378.1009.

Example 31. Methyl 2-(4-chlorophenyl)-2-(indolin-1-yl)acetate (8g).

The title compound was synthesized following the general procedure described in  section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7g (indoline). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; Rf = 0.5 (30% ethylacetate/hexane); to give compound 8g (0.70 g in 5 h, 81%) as a white solid and mp: 105-107 °C. 1H NMR (400 MHz, CDCl3) δ 7.39 (s, 4H), 7.13 – 7.06 (m, 2H), 6.76 – 6.72 (m, 1H), 6.47 (d, J = 7.9 Hz, 1H), 5.29 (s, 1H), 3.78 (s, 3H), 3.66 (dd, J = 17.9, 8.6 Hz, 1H), 3.23 – 3.17 (m, 1H), 3.05 – 2.89 (m, 2H). 13C NMR (101 MHz, CDCl3) δ 171.30, 150.63, 134.33, 133.76, 130.25, 130.03, 128.92, 127.24, 124.77, 118.64, 106.90, 63.21, 52.15, 49.90, 28.20. IR (max): 2954.36, 1714.79, 1213.16, 1013.78, 838.25 cm-1. HRMS (ESI): m/z calcd for C17H17ClNO2 [M+H]+ 302.0942, found 302.0970.

Example 32. Methyl 2-(4-chlorophenyl)-2-(methyl(tetrahydro-2H-pyran-4-yl)amino)acetate (8h).

The title compound was synthesized following the general procedure described in  section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7h (N-methyltetrahydro-2H-pyran-4-amine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8h (0.62 g in 5 h, 75%) as a colourless liquid. 1H NMR (400 MHz, CDCl3) δ 7.38 – 7.36 (m, 2H), 7.33 – 7.30 (m, 2H), 4.47 (s, 1H), 4.00 – 3.95 (m, 2H), 3.70 (s, 3H), 3.32 – 3.21 (m, 2H), 2.74 – 2.66 (m, 1H), 2.24 (s, 3H), 1.72 – 1.59 (m, 4H).13C NMR (101 MHz, CDCl3) 172.32, 135.51, 133.93, 129.92, 128.70, 68.63, 67.57, 67.36, 56.94, 51.93, 33.42, 29.69, 28.64. IR (max): 2952.50, 2846.49, 1738.35, 1489.25, 1206.43, 1011.50, 816.53 cm-1. HRMS (ESI): m/z calcd for C15H21ClNO3 [M+H]+ 298.1210, found 298.1230.

Example 33. Methyl 2-(4-chlorophenyl)-2-(diisopropylamino)acetate (8i).

The title compound was synthesized following the general procedure described in section 5.3 and involved corresponding reactant exchange with compounds 2a and 7i (diisopropylamine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8i (0.66 g in 5 h, 82%) as a white solid. 1H NMR (400 MHz, CDCl3) δ 7.29 – 7.22 (m, 4H), 4.73 (s, 1H), 3.73 (s, 3H), 3.30 – 3.20 (m, 2H), 1.10 (d, J = 6.6 Hz, 6H), 0.96 (d, J = 6.7 Hz, 6H). 13C NMR (126 MHz, CDCl3) δ 175.62, 138.16, 133.05, 129.78, 128.33, 61.04, 51.69, 45.95, 23.24, 21.55. IR (max): 2963.52, 2871.93, 1740.85, 1459.80, 1140.85, 1015.15, 821.93 cm-1. HRMS (ESI): m/z calcd for C15H23ClNO2 [M+H]+ 284.1412, found 284.1411.

Example 34. Methyl 2-(((3s,5s,7s)-adamantan-1-yl)amino)-2-(4-chlorophenyl)acetate (8j).

The title compound was synthesized following the general procedure described in section 5.3 and involved corresponding reactant exchange with compound 2a and compound 7j (adamantly amine). The crude mixture was concentrated under vacuum, and the product was purified by flash chromatography; to give compound 8j (0.76 g in 5 h, 85%) as a white solid. 1H NMR (400 MHz, CDCl3) δ 7.37 – 7.26 (m, 4H), 4.57 (s, 1H), 3.67 (s, 3H), 2.04 (d, J = 7.0 Hz, 4H), 1.64 – 1.55 (m, 12H). 13C NMR (101 MHz, CDCl3) δ 175.05, 139.49, 133.33, 128.68, 128.63, 56.90, 52.45, 51.31, 43.07, 36.51, 29.56. IR (max): 3328.25, 2904.43, 2848.81, 1736.79, 1487.26, 1163.15, 821.61 cm-1. HRMS (ESI): m/z calcd for C19H25ClNO2 [M+H]+ 334.1574, found 334.1592.

S6 Case study-4: Carbene insertion (C-C bond formation, cyclopropanation) single objective multi-variant auto-optimization.

5.6.1 Background collection

Sample Preparation: We prepare 0.5 M stock solution of compound 2a in DCE, 0.5 M stock solution of reagent 9a in DCE and 0.5 M stock solution of product 10a in DCE. The three of the solution taken in three different syringes. To conduct our analysis, we utilized an In-line FTIR system. The experimental protocol involved the initial introduction of the solvent, DCE, for a duration of 10 min. using a syringe pump, during which data was recorded. Subsequently, a previously prepared stock solution of compound 2a was similarly introduced into the In-line FTIR for 10 min. This process was repeated for the reaction product 10a and reagent 9a, each pumped for 10 min. Upon collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure S32. In-line IR background analysis of DCE, 0.5 M stock solution of 2a, 9a, and 10a in DCE.

The distinctive shifting peak associated with product (10a) aligning within the range of 1910 to 1950 cm-1 was observed. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S6.2. General procedure for the auto-optimized synthesis of compound 10a.

Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in DCE) and reagent 9a (0.2 M in DCE), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 5.0 m, volume = 4 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 50-60 experiments and the results were tabulated in the optimization table (Table S10).

S6.2.1. Python code for optimizing condition for C-C bond formation (cyclopropanation).

Table S10. Auto-optimization table of C-C bond formation (cyclopropanation).

| No. of experiments | Flow rate mL/min. | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Space time yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.05238 | 11.3274 | 27.907 | 0.221759 | 15.04351243 | 0.001690215 |
| 2 | 0.11695 | 12.0946 | 36.33 | 0.883698 | 59.94760912 | 0.015038322 |
| 3 | 0.0759 | 14.4634 | 67.166 | 0.965877 | 65.52240341 | 0.010667408 |
| 4 | 0.13854 | 14 | 60.298 | 1.178701 | 79.95979035 | 0.023761515 |
| 5 | 0.05488 | 12.406 | 40.357 | 0.586033 | 39.75484522 | 0.004679845 |
| 6 | 0.10281 | 14.484 | 67.22 | 1.280536 | 86.8679929 | 0.019156777 |
| 7 | 0.09679 | 14.0054 | 60.698 | 1.104894 | 74.9529292 | 0.015561319 |
| 8 | 0.05326 | 14.1323 | 62.576 | 0.860929 | 58.40302362 | 0.006672119 |
| 9 | 0.10344 | 14.0963 | 62.093 | 1.268002 | 86.01772127 | 0.019085509 |
| 10 | 0.06814 | 14.4914 | 67.943 | 1.050196 | 71.24236934 | 0.010412806 |
| 11 | 0.09225 | 14.4998 | 67.938 | 1.119739 | 75.95997262 | 0.015030675 |
| 12 | 0.05962 | 14.5 | 67.889 | 1.076398 | 73.01983999 | 0.009338135 |
| 13 | 0.1326 | 14.2594 | 64.379 | 1.344057 | 91.17708048 | 0.025933223 |
| 14 | 0.09069 | 10.0003 | 15.612 | 0.104695 | 7.102216975 | 0.001381595 |
| 15 | 0.08657 | 14.5 | 64.141 | 0.86797 | 58.88066544 | 0.010933707 |
| 16 | 0.09243 | 12.5787 | 42.061 | 0.566808 | 38.45067481 | 0.007623321 |
| 17 | 0.06643 | 14.5 | 58.947 | 0.863185 | 58.55606437 | 0.008343791 |
| 18 | 0.14817 | 14.0122 | 59.401 | 0.97105 | 65.87332531 | 0.020936167 |
| 19 | 0.05161 | 14.0002 | 60.993 | 0.980286 | 66.49986981 | 0.007361765 |
| 20 | 0.07201 | 14.0001 | 60.965 | 0.938318 | 63.65287767 | 0.009831916 |
| 21 | 0.14721 | 14.4998 | 60.856 | 1.198318 | 81.29055295 | 0.025668748 |
| 22 | 0.1287 | 13.7505 | 57.251 | 1.108318 | 75.18520381 | 0.02075574 |
| 23 | 0.14672 | 14.4002 | 62.576 | 1.206441 | 81.84159463 | 0.025756728 |
| 24 | 0.11723 | 14.5 | 68.44 | 1.244494 | 84.42300408 | 0.021228869 |
| 25 | 0.06946 | 12.406 | 40.357 | 0.32261 | 21.88496316 | 0.003260678 |
| 26 | 0.15 | 10 | 15.58 | 0.069309 | 4.701729369 | 0.001512781 |
| 27 | 0.05 | 10 | 15.58 | 0.294156 | 19.95472311 | 0.002140144 |
| 28 | 0.05058 | 14.1178 | 62.919 | 0.412608 | 27.99017662 | 0.003036769 |
| 29 | 0.128 | 14.5 | 68.353 | 1.37646 | 93.37520968 | 0.025637098 |
| 30 | 0.09281 | 14.5 | 68.251 | 1.167093 | 79.17233598 | 0.015761427 |
| 31 | 0.05643 | 14.5 | 68.179 | 0.679223 | 46.07659507 | 0.005577219 |
| 32 | 0.05563 | 10.002 | 15.602 | 0.132377 | 8.980086695 | 0.001071561 |
| 33 | 0.14537 | 10.0003 | 15.612 | 0.289655 | 19.64938782 | 0.006127046 |
| 34 | 0.14994 | 13.0001 | 47.886 | 0.798213 | 54.14854499 | 0.017415325 |
| 35 | 0.14993 | 10.0356 | 15.895 | 0.277093 | 18.79721676 | 0.006045182 |
| 36 | 0.15 | 14.5 | 67.947 | 0.979234 | 66.42850506 | 0.021373372 |
| 37 | 0.14897 | 10.7891 | 22.869 | 0.293017 | 19.87745653 | 0.006351655 |
| 38 | 0.14945 | 11.2435 | 27.424 | 0.330208 | 22.4003903 | 0.007180899 |
| 39 | 0.05023 | 10.7667 | 22.585 | 0.504851 | 34.24768463 | 0.00368996 |
| 40 | 0.09691 | 13.0001 | 47.886 | 0.921254 | 62.49530347 | 0.012991021 |
| 41 | 0.05134 | 12.3099 | 39.229 | 0.813469 | 55.18346951 | 0.006077041 |
| 42 | 0.14539 | 14.1487 | 62.167 | 1.265278 | 85.83293271 | 0.026767991 |
| 43 | 0.05 | 14.5 | 67.831 | 0.983382 | 66.70989382 | 0.007154636 |
| 44 | 0.05 | 13.1051 | 41.708 | 0.871355 | 59.1102944 | 0.006339579 |
| 45 | 0.05 | 14.5 | 67.802 | 1.223662 | 83.00981927 | 0.008902803 |
| 46 | 0.11106 | 14.5 | 67.744 | 1.27144 | 86.25094561 | 0.020547019 |
| 47 | 0.15 | 12.5787 | 42.061 | 0.927784 | 62.93828047 | 0.020250392 |
| 48 | 0.15 | 13.1051 | 48.708 | 0.957731 | 64.96980148 | 0.020904034 |
| 49 | 0.1251 | 14.5 | 69.301 | 1.406376 | 95.40462773 | 0.02560083 |
| 50 | 0.15 | 13.3803 | 52.251 | 1.129606 | 76.6293224 | 0.024655484 |
| 51 | 0.15 | 11.126 | 26.133 | 0.46734 | 31.70304294 | 0.010200454 |
| 52 | 0.15 | 14.5 | 67.947 | 1.313304 | 89.09088268 | 0.028664992 |
| 53 | 0.13 | 13.3538 | 52.209 | 1.04556 | 70.92787602 | 0.019778238 |
| 54 | 0.06674 | 14.4746 | 67.719 | 1.186773 | 80.50737233 | 0.011525218 |
| 55 | 0.15 | 12.6835 | 43.512 | 0.99783 | 67.69000587 | 0.021779259 |
| 56 | 0.15 | 13.3591 | 55.044 | 1.033796 | 70.12983905 | 0.022564276 |
| 57 | 0.14801 | 10.0012 | 15.5 | 0.037879 | 2.569605777 | 0.000815802 |
| 58 | 0.09802 | 12.9662 | 46.924 | 0.988208 | 67.03727621 | 0.014094782 |
| 59 | 0.14937 | 14.4998 | 60.391 | 1.096345 | 74.37298887 | 0.023829005 |
| 60 | 0.14409 | 14.1585 | 61.436 | 1.12544 | 76.34671257 | 0.023596711 |

Reaction condition: compound 2a (0.1 M in DCE); reagent 9a (0.2 M in DCE); 4 mL reactor volume.

Graph:

We have plotted 3D graph of optimization table S10, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S33. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated cyclopropanation reaction. Compound 2a (0.1 M in DCE); reagent 9a (0.2 M in DCE); 4 mL reactor volume.

Figure S34. Reproducibility test for the AI based system to auto-optimization for the photo activated cyclopropanation reaction. Compound 2a (0.1 M in DCE); reagent 9a (0.2 M in DCE); 4 mL reactor volume.

Figure S35. AI based system navigated multi-numerical variable complexity and identify the optimal condition for the photo activated cyclopropanation reaction. Compound 2a (0.1 M in DCE); reagent 9a (0.2 M in DCE); 4 mL reactor volume.

S6.3. General procedure for AI optimized carbene insertion into the double bonds of aromatic alkenes for longer run to synthesis compound 10a.

Figure S36. Schematic presentation of continuous flow for the synthesis, of compound 9a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in ethyl acetate (100 mL) charged in one syringe, another syringe charged with the reagent 9a (2.4 mL, 0.02 mol) dissolved in ethyl acetate (100 mL). These two syringes connected via pump and out-put is further connected with the T-mixer and both syringes were running with the 125 L/min. flow rate to maintain the stoichiometry and then passed through PFA tubular reactor (id = 1000 µm, l = 5 m, vol. = 4 mL) under blue light (69W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. First one hour of the product mixture was discarded and next 5 h of the product mixture [37.5 mL; 2a (0.393 g)] was collected in HPLC bottle. The organic EtOAc phase was concentrated under reduced pressure to give the product 10a (0.507 g in 5 h, 95%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.12 – 7.07 (m, 5H), 6.97 – 6.93 (m, 2H), 6.79 – 6.77 (m, 2H), 3.67 (s, 3H), 3.11 (dd, J = 9.3, 7.4 Hz, 1H), 2.14 (dd, J = 9.3, 5.0 Hz, 1H), 1.84 (dd, J = 7.3, 5.0 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 173.99, 135.99, 133.56, 133.33, 133.07, 131.63, 129.46, 128.15, 128.07, 128.02, 126.68, 52.80, 36.82, 33.31, 20.47. IR (max): 3024.29, 2953.82, 1716.75, 1494.38, 1213.56, 825.68 cm-1. HRMS (ESI): m/z calcd for C17H16ClO2 [M+H]+ 286.0833, found 286.0842. Verified the analytical data with those reported in the literature.7

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 69W = 69 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=69 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.6 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.6 ×10246.02214× 1023

Mole’s photons incident  =4.31 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.507 g.

ofMole’s of product formed=0.507285

Mole’s of product formed =1.7 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=1.7 ×10-34.31 ×100

Quantum efficiency ∅:=0.39 %

Productivity per h:=0.5075= 0.101 g/h

Space time yield :=productivityvolume of reactor

=0.1014=0.025 g mL-1 h-1

Space time yield in previous reported batch process8

Productivity = 100.4 mg in 12 h

Productivity per h:=0.10012= 0.008 g/h

Space time yield :=productivityvolume of reactor

=0.0083=0.0026 g mL-1 h-1

Figure S37. Scope of substrate for the photo activated cyclopropanation reaction. Compound 2a (0.1 M in DCE); reagent 9a (0.2 M in DCE); 4 mL reactor volume.

S6.3.1. General procedure for scope of substrate for synthesis of compound 10bi.

Example 36. Methyl 1-(4-chlorophenyl)-2-(4-fluorophenyl)cyclopropane-1-carboxylate (10b).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2a and compound 9b (1-fluoro-4-vinylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10b (0.37 g in 5 h, 65%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.13 –7.10 (m, 2H), 6.96 –6.93 (m, 2H), 6.81 – 6.71 (m, 4H), 3.66 (s, 3H), 3.09 (dd, J = 9.3, 7.3 Hz, 1H), 2.13 (dd, J = 9.4, 5.1 Hz, 1H), 1.78 (dd, J = 7.3, 5.1 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 173.83, 162.92, 160.48, 133.34, 133.28, 133.20, 131.75, 129.57, 129.49, 128.17, 115.07, 114.85, 52.81, 36.65, 32.50, 20.55. 19F NMR (471 MHz, CDCl3) δ -114.38 (s). IR (max): 3021.76, 1715.56, 1437.27, 1214.76, 836.50 cm-1. HRMS (ESI): m/z calcd for C17H15ClFO2 [M+H]+ 305.0739, found 305.0748.

Example 37. Methyl 1-(4-chlorophenyl)-2-(4-methoxyphenyl)cyclopropane-1-carboxylate (10c).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2a and compound 9c (1-methoxy-4-vinylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10c (0.375 g in 5 h, 63%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.11 – 7.08 (m, 2H), 6.97 – 6.94 (m, 2H), 6.70 – 6.68 (m, 2H), 6.64 – 6.61(m, 2H), 3.68 (s, 3H), 3.64 (s, 3H), 3.08 – 3.04 (m, 1H), 2.13 – 2.10 (m, 1H), 1.78 – 1.75 (m, 1H).13C NMR (101 MHz, CDCl3) δ 190.83, 173.95, 158.31, 133.69, 133.32, 132.89, 132.03, 131.69, 130.08, 129.07, 128.51, 127.99, 127.84, 114.37, 113.42, 55.22, 55.13, 52.63, 36.42, 32.81, 20.43. IR (max): 3020.53, 1714.16, 1437.97, 1214.91, 833.50 cm-1. HRMS (ESI): m/z calcd for C18H18ClO3 [M+H]+ 317.0939, found 317.0942.

Example 38.  Methyl 1-(4-bromophenyl)-2-phenylcyclopropane-1-carboxylate (10d).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2b and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10d (0.494 g in 5 h, 80%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.23 (d, J = 8.4 Hz, 2H), 7.08 – 7.05 (m, 3H), 6.88 (d, J = 8.4 Hz, 2H), 6.77 – 6.75 (m, 2H), 3.63 (s, 3H), 3.10 (t, J = 8.3 Hz, 1H), 2.12 (dd, J = 9.3, 5.0 Hz, 1H), 1.82 (dd, J = 7.2, 5.1 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 173.78, 135.88, 134.03, 133.64, 130.94, 128.08, 127.97, 126.63, 121.26, 52.73, 36.82, 33.21, 20.37. IR (max): 3021.33, 1715.74, 1491.81, 1214.63, 668.65 cm-1. HRMS (ESI): m/z calcd for C17H16BrO2 [M+H]+ 331.0328, found 331.0337.

Example 39. Methyl 1-(4-fluorophenyl)-2-phenylcyclopropane-1-carboxylate (10e).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2c and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10e (0.370 g in 6 h, 73%) as a colorless oil. 1H NMR (500 MHz, CDCl3) δ 7.05 – 7.01 (m, 3H), 6.97 – 6.94 (m, 2H), 6.78 – 6.73 (m, 4H), 3.59 (s, 3H), 3.10 (dd, J = 9.3, 7.3 Hz, 1H), 2.12 (dd, J = 9.3, 5.0 Hz, 1H), 1.82 (dd, J = 7.3, 5.0 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 173.92, 162.89, 160.45, 135.99, 133.49, 133.41, 130.66, 130.63, 127.99, 127.76, 126.41, 114.67, 114.46, 52.46, 36.54, 33.09, 20.36. 19F NMR (377 MHz, CDCl3) δ -114.84. IR (max): 3023.59, 2954.03, 1715.89, 1510.89, 1216.92 cm-1. HRMS (ESI): m/z calcd for C17H16FO2 [M+H]+ 271.1129, found 271.1139.

Example 40.  Methyl 1-(4-methoxyphenyl)-2-phenylcyclopropane-1-carboxylate (10f).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2d and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10f (0.403 g in 5 h, 76%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.04 – 7.00 (m, 3H), 6.92 – 6.90 (m, 2H), 6.76 – 6.73 (m, 2H), 6.62 (d, J = 8.6 Hz, 2H), 3.62 (s, 3H), 3.59 (s, 3H), 3.06 (t, J = 8.2 Hz, 1H), 2.11 – 2.07 (m, 1H), 1.81 – 1.71 (m, 1H). 13C NMR (101 MHz, CDCl3) δ 174.64, 158.56, 136.59, 133.03, 128.21, 127.82, 126.87, 126.37, 113.26, 55.07, 52.63, 36.79, 33.30, 20.81 IR (max): 3028.49, 2952.42, 1714.73, 1513.38, 1247.84, 934.88 cm-1. HRMS (ESI): m/z calcd for C18H19O3 [M+H]+ 283.1329, found 283.1335.

Example 41. Methyl 1-(4-nitrophenyl)-2-phenylcyclopropane-1-carboxylate (10g).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2e and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10g (0.373 g in 5 h, 67%) as a colorless oil. 1H NMR (500 MHz, CDCl3) δ 7.99 – 7.97 (m, 2H), 7.21– 7.18 (m, 2H), 7.09 – 7.07 (m, 3H), 6.80 – 6.7 (m, 2H), 3.68 (s, 3H), 3.21 (dd, J = 9.3, 7.4 Hz, 1H), 2.22 (dd, J = 9.3, 5.2 Hz, 1H), 1.96 (dd, J = 7.4, 5.2 Hz, 1H). 13C NMR (101 MHz, CDCl3) δ 172.97, 146.94, 142.76, 135.21, 132.89, 128.23, 128.01, 127.06, 122.98, 52.94, 37.04, 33.64, 20.06. IR (max): 3023.03, 2953.82, 1720.22, 1519.83, 1214.04 cm-1. HRMS (ESI): m/z calcd for C17H16NO4 [M+H]+ 298.1074, found 298.1082.

Example 42. Methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cyclopropane-1-carboxylate (10h).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2k and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10h (0.368 g in 5 h, 64%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.11 – 7.09 (m, 3H), 6.88 – 6.85 (m, 3H), 6.67 – 6.60 (m, 1H), 3.68 (s, 3H), 3.22 (dd, J = 9.3, 7.6 Hz, 1H), 2.10 (dd, J = 9.4, 5.3 Hz, 1H), 1.86 (dd, J = 7.5, 5.4 Hz, 1H). 13C NMR (126 MHz, CDCl3) δ 172.59, 158.68, 158.62, 156.71, 156.64, 150.53, 150.42, 150.32, 148.62, 148.53, 148.43, 148.32, 147.15, 147.13, 147.05, 147.03, 145.50, 145.21, 145.18, 145.11, 145.08, 135.62, 135.19, 129.30, 128.13, 127.94, 127.89, 127.17, 126.94, 120.23, 120.19, 120.08, 120.04, 119.60, 119.56, 119.51, 119.47, 119.43, 105.40, 105.24, 105.18, 105.02, 52.75, 52.10, 33.60, 33.10, 32.27, 19.72, 18.86. 19F NMR (471 MHz, CDCl3) δ -113.59, -134.16, -134.20, -143.40, -143.44, -143.47. IR (max): 3027.15, 2926.40, 1725.42, 1516.46, 1264.81, 885.38 cm-1. HRMS (ESI): m/z calcd for C17H14F3O2 [M+H]+ 307.0940, found 307.0949.

Example 43. Methyl 1,2-diphenylcyclopropane-1-carboxylate (10i).

The title compound was synthesised following the general procedure described in  section 6.3, and involve corresponding reactant exchange with compound 2l and compound 9a (styrene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.5 (5% ethylacetate/hexane); to give compound 10i (0.395 g in 5 h, 79%) as a colorless oil. 1H NMR (300 MHz, CDCl3) δ 7.12 – 7.09 (m, 3H), 7.05 – 7.00 (m, 5H), 6.78 – 6.74 (m, 2H), 4.19 – 4.06 (m, 2H), 3.09 (dd, J = 9.3, 7.3 Hz, 1H), 2.11 (dd, J = 9.3, 4.9 Hz, 1H), 1.86 (dd, J = 7.3, 4.9 Hz, 1H), 1.17 (t, J = 7.1 Hz, 3H). 13C NMR (101 MHz, CDCl3) δ 173.77, 136.59, 134.99, 132.05, 128.18, 127.80, 127.72, 127.02, 126.38, 61.33, 37.73, 33.04, 20.27, 14.30. IR (max): 3028.05, 2983.04, 1711.52, 1451.75, 1252.14, 1164.61 cm-1. HRMS (ESI): m/z calcd for C18H19O2 [M+H]+ 267.1380, found 267.1391.

Table S11. Comparative result for C-C bond formation (cyclopropanation).

| Entry | Compound 2a | Reagent 9a | Product 10a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 94.7%, 32 min. (our study) Bis(imino)pyridine iron complexes, 12 h, 88% 8 Borane-catalyzed, 18-24 h, 83% 9 Tris(pentafluorophenyl)borane-catalyzed, 69% 10 |
| 2 |  |  |  | 75%, 32 min. (our study) Rh2(S-TPPTTL)4, 13 h, 61% 11 Rh2(S-biTISP)2, 22 h, 86% 12 |
| 3 |  |  |  | 73%, 32 min. (our study) bismuth–rhodium complexes, 22 h, 90% (based on 1H NMR), 32% ee 13 |
| 4 |  |  |  | 76%, 32 min. (our study) Ag, 16 h, 95% 14. bis(imino)pyridine iron complex, 12 h, 83% 8 |

S7 Case study-5: Carbene insertion (C-C bond formation, cyclopropenation) single objective multi-variant auto-optimization.

S7.1. Background collection

Sample Preparation: We prepare 0.5 M stock solution of compound 2a in DCE, 0.5 M stock solution of reagent 11a in DCE and 0.5 M stock solution of product 12a in DCE. The three of the solution taken in three different syringes.

To conduct our analysis, we utilized an In-line FTIR system. The experimental protocol involved the initial introduction of the solvent, DCE, for a duration of 10 min. using a syringe pump, during which data was recorded. Subsequently, a previously prepared stock solution of compound 2a was similarly introduced into the In-line FTIR for 10 min. This process was repeated for the reaction product 12a and reagent 11a, each pumped for 10 min. Upon collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure S38. In-line IR background analysis of DCE, 0.5 M stock solution of 2a, 11a, and 12a in DCE.

The distinctive shifting peak associated with product (12a) aligning within the range of 1730 to 1770 cm-1 was observed. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S7.2. General procedure for the auto-optimized synthesis of compound 12a.

In our approach we are employing a Python-coded based Bayesian optimization strategy for further refinement. Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in DCE) and reagent 11a (0.2 M in DCE), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 5.1 m, volume = 4 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 50-60 experiments and the results were tabulated in the optimization table (Table S12).

S7.2.1. Python code for optimizing condition of C-C bond formation, (cyclopropenation)

Table S12. Auto-optimization table of C-C bond formation, (cyclopropenation)

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Space time yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.120701 | 14.1092 | 60.443 | 0.454837 | 73.79562 | 0.018972347 |
| 2 | 0.090227 | 14.0756 | 60.142 | 0.467826 | 75.90304 | 0.014587312 |
| 3 | 0.066047 | 10.6181 | 20.55 | 0.114833 | 18.63123 | 0.002621043 |
| 4 | 0.052175 | 14.1067 | 60.91 | 0.445654 | 72.30571 | 0.008035533 |
| 5 | 0.05922 | 10.4612 | 19.499 | 0.106311 | 17.24857 | 0.00217571 |
| 6 | 0.127527 | 13.1035 | 49.136 | 0.358988 | 58.24447 | 0.015821093 |
| 7 | 0.130375 | 10.6304 | 21.864 | 0.249116 | 40.41815 | 0.01122407 |
| 8 | 0.093723 | 14.4455 | 68.115 | 0.583323 | 94.642 | 0.018893381 |
| 9 | 0.069852 | 12.4586 | 49.071 | 0.41648 | 67.57234 | 0.010053734 |
| 10 | 0.15 | 14.5 | 68.251 | 0.48846 | 79.25083 | 0.025320639 |
| 11 | 0.149893 | 10.9074 | 23.886 | 0.260363 | 42.24293 | 0.01348699 |
| 12 | 0.099249 | 14.5 | 68.105 | 0.574865 | 93.26972 | 0.019717253 |
| 13 | 0.05 | 14.5 | 68.077 | 0.497185 | 80.66643 | 0.008590974 |
| 14 | 0.15 | 11.2061 | 26.928 | 0.254111 | 41.22857 | 0.013172528 |
| 15 | 0.11223 | 14.5 | 68.352 | 0.539139 | 87.47331 | 0.020910486 |
| 16 | 0.144445 | 14.5 | 68.164 | 0.452476 | 73.41256 | 0.022586684 |
| 17 | 0.064064 | 14.5 | 68.091 | 0.514453 | 83.4681 | 0.011389749 |
| 18 | 0.054786 | 10.9936 | 24.807 | 0.238132 | 38.63604 | 0.004508601 |
| 19 | 0.098561 | 13.1959 | 50.192 | 0.135787 | 22.03094 | 0.004625063 |
| 20 | 0.107359 | 10.7921 | 22.836 | 0.231832 | 37.61388 | 0.008601343 |
| 21 | 0.099419 | 13.6901 | 56.831 | 0.288356 | 46.78469 | 0.009907242 |
| 22 | 0.099154 | 14.4518 | 69.581 | 0.591693 | 96.00162 | 0.020275353 |
| 23 | 0.078101 | 10.4687 | 19.751 | 0.212029 | 34.40092 | 0.00572277 |
| 24 | 0.149026 | 13.6182 | 55.984 | 0.326608 | 52.99094 | 0.016820669 |
| 25 | 0.128138 | 12.0086 | 35.58 | 0.349058 | 56.63337 | 0.015457169 |
| 26 | 0.133455 | 13.0499 | 48.233 | 0.377999 | 61.32894 | 0.017433312 |
| 27 | 0.138758 | 11.0478 | 25.215 | 0.373725 | 60.6355 | 0.017921097 |
| 28 | 0.15 | 10.2395 | 17.684 | 0.307325 | 49.86234 | 0.015931019 |
| 29 | 0.09817 | 14.4834 | 68.2 | 0.559436 | 90.76642 | 0.018979449 |
| 30 | 0.091267 | 13.7496 | 57.585 | 0.510067 | 82.75648 | 0.016087754 |
| 31 | 0.126242 | 12.456 | 40.893 | 0.384914 | 62.45087 | 0.016792756 |
| 32 | 0.068906 | 12.2385 | 38.326 | 0.424445 | 68.86463 | 0.010107247 |
| 33 | 0.092488 | 14.5 | 68.324 | 0.56245 | 91.25543 | 0.017977269 |
| 34 | 0.112444 | 13.2561 | 51.079 | 0.403716 | 65.50143 | 0.015687966 |
| 35 | 0.075546 | 14.4222 | 67.192 | 0.506462 | 82.17159 | 0.013222475 |
| 36 | 0.088877 | 13.7497 | 57.553 | 0.351022 | 56.95202 | 0.010781474 |
| 37 | 0.092205 | 13.7496 | 57.553 | 0.373007 | 60.51901 | 0.01188573 |
| 38 | 0.097674 | 14.4875 | 67.973 | 0.565181 | 91.69853 | 0.019077477 |
| 39 | 0.125538 | 12.4198 | 40.238 | 0.361648 | 58.67605 | 0.015689738 |
| 40 | 0.125824 | 12.4595 | 40.787 | 0.387129 | 62.81025 | 0.016833468 |
| 41 | 0.12398 | 12.5461 | 42.079 | 0.403667 | 65.49348 | 0.017295347 |
| 42 | 0.139188 | 10.9791 | 24.766 | 0.253061 | 41.05821 | 0.012172546 |
| 43 | 0.15 | 12.1401 | 37.379 | 0.251172 | 40.75173 | 0.013020177 |
| 44 | 0.15 | 10.8394 | 23.41 | 0.227043 | 36.83689 | 0.011769385 |
| 45 | 0.133045 | 12.5478 | 42.296 | 0.293369 | 47.59803 | 0.01348861 |
| 46 | 0.15 | 13.8612 | 59.614 | 0.303369 | 49.2205 | 0.015725949 |
| 47 | 0.149999 | 11.0341 | 25.511 | 0.233369 | 37.86326 | 0.01209723 |
| 48 | 0.050002 | 11.4698 | 29.911 | 0.173369 | 28.12848 | 0.002995803 |
| 49 | 0.098806 | 14.4781 | 67.772 | 0.569669 | 92.4267 | 0.019451826 |
| 50 | 0.149282 | 10.0468 | 16.022 | 0.133369 | 21.63863 | 0.006880448 |
| 51 | 0.140899 | 11.6944 | 32.679 | 0.264725 | 42.95065 | 0.012890129 |
| 52 | 0.137499 | 14.2234 | 65.412 | 0.39556 | 64.17815 | 0.018796038 |
| 53 | 0.063767 | 10.58 | 21.028 | 0.221869 | 35.99742 | 0.004889304 |
| 54 | 0.138262 | 11.4412 | 29.286 | 0.208132 | 33.76865 | 0.009944801 |
| 55 | 0.076134 | 13.8555 | 59.2 | 0.413512 | 67.09079 | 0.010879806 |

Reaction condition: compound 2a (0.1 M in DCE); reagent 11a (0.2 M in DCE); 4 mL reactor volume.

Graph:

We have plotted 3D graph of optimization table S12, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S39. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated cyclopropenation reaction. Compound 2a (0.1 M in DCE); reagent 11a (0.2 M in DCE); 4 mL reactor volume.

Figure S40. Reproducibility test for the AI based system to auto-optimization for the photo activated cyclopropenation reaction. Compound 2a (0.1 M in DCE); reagent 11a (0.2 M in DCE); 4 mL reactor volume.

Figure S41. AI based system navigated multi-numerical variable complexity and identify the optimal condition for the photo activated cyclopropenation reaction. Compound 2a (0.1 M in DCE); reagent 11a (0.2 M in DCE); 4 mL reactor volume.

S7.3. General procedure for AI optimized carbene insertion into the triple bonds of aromatic alkynes for synthesis of compound 12a

Figure S42. Schematic presentation of continuous flow for the synthesis, of compound 12a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in ethyl acetate (100 mL) and connected with pump.  Another pump was connected with the reagent 11a (2.3 mL, 0.02 mol) dissolved in ethyl acetate (100 mL). These two pump out-put is further connected with the T1-mixer and pumps were running with the flow rate 50 L/min.each  to maintain the stoichiometry and then passed through PFA tubular reactor (id = 1000 µm, l = 5 m, vol. = 4 mL) under blue light (68.581 W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. First one hour of the product mixture was discarded and next 5 h of the product mixture [30 mL; 2a (0.315 g)] was collected in HPLC bottle. The organic EtOAc phase was concentrated under reduced pressure to give the product 12a (0.410 g in 5 h, 96%) as a yellow oil. 1H NMR (400 MHz, CDCl3) δ 7.59 – 7.57 (m, 2H), 7.42 – 7.39 (m, 3H), 7.34 – 7.31 (m, 2H), 7.25 – 7.22 (m, 2H), 7.17 (s, 1H), 3.70 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 174.65, 139.48, 132.32, 130.25, 129.94, 129.68, 129.01, 128.23, 125.11, 117.09, 99.86, 52.32, 32.98. IR (max): 3021.78, 1720.27, 1595.83, 1490.51, 1214.17, 830.32 cm-1. HRMS (ESI): m/z calcd for C17H14ClO2 [M+H]+ 285.0677, found 285.0686. Verified the analytical data with those reported in the literature.15

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 70W = 70 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=70 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.6 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.6 ×10246.02214× 1023

Mole’s photons incident  =4.31 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.410 g.

ofMole’s of product formed=0.410284

Mole’s of product formed =1.4 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=1.4 ×10-34.31 ×100

Quantum efficiency ∅:=0.32 %

Productivity per h:=0.4105= 0.082 g/h

Space time yield :=productivityvolume of reactor

=0.0824=0.02 g mL-1 h-1

Space time yield in previous reported batch process 15

Productivity = 67 mg in 16 h

Productivity per h:=0.06716= 0.0041 g/h

Space time yield :=productivityvolume of reactor

=0.00412=0.002 g mL-1 h-1

Figure S43. Scope of substrate for the photo activated cyclopropenation reaction. Compound 2a (0.1 M in DCE); reagent 11a (0.2 M in DCE); 4 mL reactor volume.

S.7.3.1. General procedure for scope of substrate for carbene insertion into the S−H bonds of organic aromatic thiol for synthesis of compound 12b-12h.

Example 45. Methyl 1-(4-chlorophenyl)-2-(3-fluorophenyl)cycloprop-2-ene-1-carboxylate (12b).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2a and compound 11b (1-ethynyl-3-fluorobenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12b (0.410 g in 5 h, 90%) as a yellow oil. 1H NMR (500 MHz, CDCl3) δ 7.42 – 7.35 (m, 2H), 7.32 – 7.23 (m, 6H), 7.12 – 7.08 (m, 1H), 3.71 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 174.26, 164.08, 161.62, 139.02, 132.52, 130.72, 130.64, 129.59, 129.28, 128.32, 127.11, 125.66, 125.64, 117.45, 117.24, 116.60, 116.38, 101.44, 52.40, 33.28. 19F NMR (471 MHz, CDCl3) δ -112.83 (s). IR (max): 3021.73, 1726.97, 1587.32, 1214.51, 1014.81 cm-1. HRMS (ESI): m/z calcd for C17H13ClFO2 [M+H]+ 303.0583, found 303.0597.

Example 46. Methyl 1-(4-bromophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12c).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2b and compound 11a (ethynylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12c (0.440 g in 5 h, 89%) as a colorless oil. 1H NMR (400 MHz, CDCl3) δ 7.59 – 7.57 (m, 2H), 7.42 – 7.39 (m, 3H), 7.33 (d, J = 12.0 Hz, 2H), 7.24 (d, J = 8.5 Hz, 2H), 7.17 (s, 1H), 3.70 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 174.57, 140.01, 131.18, 130.28, 130.07, 129.95, 129.03, 125.09, 120.45, 117.04, 99.78, 52.34, 33.06. IR (max): 3136.19, 2950.74, 1719.37, 1587.93, 1216.09, 629.14 cm-1. HRMS (ESI): m/z calcd for C17H14BrO2 [M+H]+ 329.0172, found 329.0180.

Example 47. Methyl 1-(4-fluorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12d).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2c and compound 11a (ethynylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12d (0.302 g in 5 h, 75%) as a yellow oil. 1H NMR (500 MHz, CDCl3) δ 7.64 – 7.62 (m, 2H), 7.47 – 7.38 (m, 5H), 7.22 (s, 1H), 7.01 – 6.98 (m, 2H), 3.73 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 174.96, 162.85, 160.42, 136.79, 130.22, 129.95, 129.90, 129.04, 125.30, 117.41, 115.07, 114.86, 100.23, 52.33, 32.95. 19F NMR (377 MHz, CDCl3) δ -116.21. IR (max): 3024.19, 2952.42, 1718.44, 1507.81, 1217.41, 1019.92, 837.24 cm-1. HRMS (ESI): m/z calcd for C17H14FO2 [M+H]+ 269.0972, found 269.0990.

Example 48. Methyl 1-(4-methoxyphenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12e).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2d and compound 11a (ethynylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12e (0.380 g in 5 h, 90%) as a yellow oil. 1H NMR (400 MHz, CDCl3) δ 7.60 – 7.58 (m, 2H), 7.40 – 7.30 (m, 5H), 7.18 (s, 1H), 6.81 (d, J = 8.7 Hz, 2H), 3.72 (s, 3H), 3.68 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 175.36, 158.32, 133.16, 130.02, 129.93, 129.40, 128.96, 125.59, 117.71, 113.62, 100.63, 55.25, 52.23. IR (max): 3021.06, 1718.02, 1606.03, 1511.72, 1214.64 cm-1. HRMS (ESI): m/z calcd for C18H17O3 [M+H]+ 281.1172, found 281.1185.

Example 49. Methyl 1-(4-nitrophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12f).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2e and compound 11a (ethynylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12f (0.390 g in 5 h, 88%) as a yellow oil. 1H NMR (400 MHz, CDCl3) δ 8.16 – 8.13 (m, 2H), 7.60 – 7.56 (m, 4H), 7.47 – 7.44 (m, 3H), 7.19 (s, 1H), 3.73 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 173.75, 148.59, 146.45, 130.63, 129.99, 129.13, 129.05, 124.45, 123.33, 116.18, 98.99, 52.46, 33.25. IR (max): 3022.46, 2925.68, 1722.41, 1520.33, 1215.42 cm-1. HRMS (ESI): m/z calcd for C17H14NO4 [M+H]+ 296.0917, found 296.0927.

Example 50. Methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12g).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2a and compound 11c (1-ethynyl-4-fluorobenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12g (0.385 g in 5 h, 86%) as a yellow oil. 1H NMR (400 MHz, CDCl3) δ 7.60 – 7.58 (m, 2H), 7.40 – 7.30 (m, 5H), 6.81 (d, J = 8.8 Hz, 2H), 3.72 (s, 3H), 3.68 (s, 3H). 13C NMR (101 MHz, CDCl3) δ 175.36, 158.32, 133.16, 130.02, 129.93, 129.40, 128.96, 125.59, 117.71, 113.62, 100.63, 55.25, 52.23. 19F NMR (376 MHz, CDCl3) δ -110.11, IR (max): 2954.04, 2844.40, 1723.50, 1602.75, 1247.68, 1029.28 cm-1. HRMS (ESI): m/z calcd for C18H16FO3 [M+H]+ 299.1078, found 299.1089.

Example 51. Methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cycloprop-2-ene-1-carboxylate (12h).

The title compound was synthesised following the general procedure described in  section 7.3, and involve corresponding reactant exchange with compound 2k and compound 11a (ethynylbenzene). The crude mixture was concentrated under vacuum and the product was purified by flash chromatography; Rf = 0.4 (5% ethylacetate/hexane); to give compound 12h (0.408 g in 5 h, 89%) as a yellow oil. 1H NMR (500 MHz, CDCl3) δ 7.66 – 7.65 (m, 2H), 7.48 – 7.42 (m, 3H), 7.22 (s, 1H), 7.03 – 6.98 (m, 1H), 6.93 – 6.88 (m, 1H), 3.71 (s, 3H). 13C NMR (176 MHz, CDCl3) δ 174.06, 157.56, 156.13, 149.99, 148.57, 147.43, 145.97, 130.64, 130.05, 129.14, 125.39, 125.29, 124.77, 118.69, 117.83, 117.75, 105.78, 105.62, 105.50, 98.98, 52.69, 29.83, 29.28. 19F NMR (471 MHz, CDCl3) δ -115.94, -115.96, -135.30, -135.34, -142,94, -142.98, -143.01. IR (max):  3071.13, 2956.10, 1724.39, 1512.59, 1247.95, 880.71 cm-1. HRMS (ESI): m/z calcd for C17H12F3O2 [M+H]+ 305.0784, found 305.0797.

Table S13. Comparative result for C-C bond formation, (cyclopropenation).

| Entry | Compound 2a | Reagent 11a | Product 12a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 95%, 40 min, (our study) Blue LED, 16 h, 80% 15 dirhodium tetraacetate, 10 h, 72% 16 |
| 2 |  |  |  | 89%, 40 min, (our study) Cu, 16h, 81% 17 dirhodium tetraacetate, 10h, 69% 16 |
| 3 |  |  |  | 75%, 40 min, (our study) tris(pentafluorophenyl)borate, 24h, 75% 18 Irradiation, 16 h, 56% 15 |
| 4 |  |  |  | 90%, 40 min, (our study) tris(pentafluorophenyl)borate, 24h, 50% 18 dirhodium tetraacetate, 10 h, 50% 19 |
| 5 |  |  |  | 88%, 40 min, (our study) dirhodium tetraacetate, 64% 20 |

S8. Case study-6: Carbene insertion (N-C-O bond formation, oxazole) single objective multi-variant auto-optimization.

S.8.1 Background collection

Sample Preparation: We prepare 0.5 M stock solution of compound 2a in DCE, 0.5 M stock solution of reagent 13a in DCE and 0.5 M stock solution of product 14a in DCE. The three of the solution taken in three different syringes.

To conduct our analysis, we utilized an In-line FTIR system. The experimental protocol involved the initial introduction of the solvent, DCE, for a duration of 10 min. using a syringe pump, during which data was recorded. Subsequently, a previously prepared stock solution of compound 2a was similarly introduced into the In-line FTIR for 10 min. This process was repeated for the reaction product 14a and reagent 13a, each pumped for 10 min. Upon collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure S44. In-line IR background analysis of DCE, 0.5 M stock solution of 2a, 13a, and 14a in DCE.

The distinctive shifting peak associated with product (14a) aligning within the range of 1730 to 1770 cm-1. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S8.2. General procedure for the auto-optimized synthesis of compound 14a.

In our approach we are employing a Python-coded based Bayesian optimization strategy for further refinement. Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in DCE) and reagent 13a (0.2 M in DCE), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 40-45 experiments and the results were tabulated in the optimization table (Table S14).

S8.2.1. Python code for optimized condition for N-C-O bond formation, (oxazole)

Table S14. Auto-optimization table of carbene insertion reaction (N-C-O bond formation, oxazole).

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Space time yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.060718 | 12.40142 | 41.056 | 1.155747 | 66.97147 | 0.027204 |
| 2 | 0.10674 | 10.77376 | 22.862 | 0.55897 | 32.39033 | 0.02313 |
| 3 | 0.107843 | 10.36182 | 18.909 | 0.267647 | 15.50919 | 0.011189 |
| 4 | 0.108263 | 10.79726 | 23.103 | 0.543014 | 31.46576 | 0.02279 |
| 5 | 0.099346 | 10.72808 | 22.398 | 0.541964 | 31.40494 | 0.020873 |
| 6 | 0.068879 | 10.66267 | 21.772 | 0.510561 | 29.58525 | 0.013633 |
| 7 | 0.060402 | 14.4994 | 69.755 | 1.570414 | 91.00002 | 0.036772 |
| 8 | 0.149805 | 14.34272 | 67.192 | 1.231955 | 71.38748 | 0.071544 |
| 9 | 0.053698 | 14.13864 | 64.004 | 1.410356 | 81.72518 | 0.029359 |
| 10 | 0.145992 | 13.77436 | 58.806 | 1.178274 | 68.27688 | 0.066685 |
| 11 | 0.129902 | 14.38545 | 67.638 | 1.292653 | 74.90471 | 0.065096 |
| 12 | 0.15 | 14.44019 | 68.469 | 1.252034 | 72.55097 | 0.072805 |
| 13 | 0.13959 | 12.22695 | 38.597 | 0.937301 | 54.31332 | 0.050721 |
| 14 | 0.147351 | 14.00744 | 62.093 | 1.237856 | 71.72945 | 0.07071 |
| 15 | 0.070144 | 14.49956 | 69.252 | 1.496397 | 86.71099 | 0.04069 |
| 16 | 0.15 | 12.64771 | 44.051 | 1.159221 | 67.17278 | 0.067408 |
| 17 | 0.141495 | 14.42064 | 16.392 | 0.123688 | 7.167289 | 0.006785 |
| 18 | 0.15 | 14.47181 | 68.795 | 1.170652 | 67.83517 | 0.068073 |
| 19 | 0.05 | 14.30231 | 66.266 | 1.542238 | 89.36728 | 0.029893 |
| 20 | 0.060666 | 14.03974 | 62.491 | 1.458827 | 84.53391 | 0.034309 |
| 21 | 0.146306 | 13.80666 | 59.163 | 1.196568 | 69.33694 | 0.067866 |
| 22 | 0.053558 | 13.26608 | 51.711 | 1.147437 | 66.48996 | 0.023824 |
| 23 | 0.101353 | 12.36236 | 40.263 | 0.970972 | 56.26446 | 0.03815 |
| 24 | 0.111683 | 10.08164 | 68.24 | 1.389288 | 80.50436 | 0.06015 |
| 25 | 0.050659 | 14.27468 | 66.303 | 1.43998 | 83.44181 | 0.028279 |
| 26 | 0.05 | 10.52857 | 20.456 | 0.350086 | 20.28625 | 0.006786 |
| 27 | 0.05 | 15 | 73.955 | 1.561132 | 90.46212 | 0.03026 |
| 28 | 0.15 | 13.76539 | 58.639 | 1.178978 | 68.31763 | 0.068557 |
| 29 | 0.063485 | 10.41661 | 19.403 | 0.406903 | 23.57859 | 0.010014 |
| 30 | 0.139424 | 10.02272 | 15.941 | 0.291171 | 16.87234 | 0.015738 |
| 31 | 0.062383 | 14.73195 | 70.464 | 1.57821 | 91.45174 | 0.038167 |
| 32 | 0.082665 | 14.57888 | 70.718 | 1.457362 | 84.44903 | 0.046703 |
| 33 | 0.064109 | 14.40021 | 67.925 | 1.564911 | 90.68111 | 0.038892 |
| 34 | 0.146834 | 10.72916 | 22.411 | 0.51166 | 29.64892 | 0.029125 |
| 35 | 0.05799 | 14.59723 | 70.971 | 1.55543 | 90.13171 | 0.034967 |
| 36 | 0.077781 | 14.85116 | 74.015 | 1.550575 | 89.85043 | 0.046754 |
| 37 | 0.149019 | 10.96034 | 24.693 | 0.691132 | 40.04866 | 0.039926 |
| 38 | 0.06201 | 14.99412 | 73.149 | 1.543011 | 89.4121 | 0.037092 |
| 39 | 0.09372 | 13.86823 | 60.073 | 1.510841 | 87.54798 | 0.054892 |
| 40 | 0.060974 | 14.27468 | 66.303 | 0.979244 | 56.74375 | 0.023147 |
| 41 | 0.064852 | 14.76182 | 73.203 | 1.320306 | 76.50714 | 0.033193 |
| 42 | 0.13607 | 13.16311 | 50.48 | 0.771156 | 44.68577 | 0.040678 |
| 43 | 0.053277 | 10.40976 | 19.38 | 0.261369 | 15.14544 | 0.005398 |
| 44 | 0.147724 | 10.71269 | 22.349 | 0.495395 | 28.70643 | 0.02837 |

Reaction condition: compound 2a (0.1 M in DCE); reagent 13a (0.2 M in DCE); 1 mL reactor volume.

Graph:

We have plotted 3D graph of optimization table S14, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S45. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated [3+2] cycloaddition reaction. Compound 2a (0.1 M in DCE); reagent 13a (0.2 M in DCE); 1 mL reactor volume.

Figure S46. Reproducibility test for the AI based system to auto-optimization for the photo activated [3+2] cycloaddition reaction.Compound 2a (0.1 M in DCE); reagent 13a (0.2 M in DCE); 1 mL reactor volume.

Figure S47. AI based system navigated multi-numerical variable complexity and identify the optimal condtitons for the photo activated [3+2] cycloaddition reaction. Compound 2a (0.1 M in DCE); reagent 13a (0.2 M in DCE); 1 mL reactor volume.

S.8.3. General procedure for AI optimized carbene insertion into the triple bonds of C≡N for synthesis of compound 14a.

Figure S48. Schematic presentation of continuous flow for the synthesis, of compound 14a.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in DCE (100 mL) and connected with pump.  Another pump was connected with the reagent 13a (1.09 mL, 0.026 mol) dissolved in DCE (100 mL). These two pump out-put is further connected with the T1-mixer and pumps were running with the flow rate 31 L/min. each to maintain the stoichiometry and then passed through PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) under blue light (73.464 W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. First one hour of the product mixture was discarded and next 5 h of the product mixture [18.6 mL; 2a (0.195 g)] was collected in HPLC bottle. The organic EtOAc phase was concentrated under reduced pressure to give the product 13a (0.190g in 5 h, 91%) as a colourless oil. 1H NMR (500 MHz, CDCl3) δ 7.70 – 7.67 (m, 2H), 7.32 – 7.30 (m, 2H), 4.00 (s, 3H), 2.38 (s, 3H). 13C NMR (126 MHz, CDCl3) δ 154.28, 151.75, 131.56, 130.07, 129.22, 128.56, 126.01, 113.27, 59.84, 14.11. IR (max): 3019.92, 1741.94, 1646.73, 1214.78, 1013.60,745.47 cm-1. HRMS (ESI): m/z calcd for C11H11ClNO2 [M+H]+ 224.0473, found 224.0451. Verified the analytical data with those reported in the literature.21

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 70W = 70 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=70 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.6 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.6 ×10246.02214× 1023

Mole’s photons incident  =4.31 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.190 g.

ofMole’s of product formed=0.190223

Mole’s of product formed =0.8 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=0.8×10-34.31 ×100

Quantum efficiency ∅:=0.18 %

Productivity per h:=0.1905= 0.038 g/h

Space time yield :=productivityvolume of reactor

=0.0381=0.038 g mL-1 h-1

Space time yield in previous reported batch process 21

Productivity = 39.8 mg in 24 h

Productivity per h:=0.3924= 0.0016 g/h

Space time yield :=productivityvolume of reactor

=0.00161=0.0016 g mL-1 h-1

Table S15. Comparative result for the carbene insertion (N-C-O bond formation, oxazole).

| Entry | Compound 2a | Reagent 13a | Product 14a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 91.4%, 16.6 min. (our study) Blue LED, RVC (+), Pt (−), 1.5 h, 88% 22 Et3SiCl, Blue LED, 8 h, 83% 23 Blue LED, 24 h, 89% 21 |

S9. Case study-7: Carbene insertion (C=C bond formation, fumarate) single objective multi-variant auto-optimization.

S9.1 Background collection

Sample Preparation: We prepare 0.5 M stock solution of compound 2a in DCE, 0.5 M stock solution of reagent 15 in DCE and 0.5 M stock solution of product 16a in DCE. The three of the solution taken in three different syringes.

To conduct our analysis, we utilized an In-line FTIR system. The experimental protocol involved the initial introduction of the solvent, DCE, for a duration of 10 min. using a syringe pump, during which data was recorded. Subsequently, a previously prepared stock solution of compound 2a was similarly introduced into the In-line FTIR for 10 min. This process was repeated for the reaction product 16a and reagent 15, each pumped for 10 min. Upon collecting all relevant data, we identified the signature peak present in the product. Using this peak as a reference, we employed a Bayesian optimizer to fine-tune and optimize the reaction conditions for improved results.

Figure. S49. In-line IR background analysis of DCE, 0.5 M stock solution of 1a, 15a, and 16a in DCE.

The distinctive shifting peak associated with product (16a) aligning within the range of 1715 to 1760 cm-1. This specific peak, falling within this range, is designated as the signature peak for our analysis. During the optimization of the reaction through Bayesian methods, this signature peak serves as a pivotal reference point. We utilize it to calculate the area under the curve in the In-line FT-IR spectra, providing a quantitative measure of the area in the forthcoming reaction mixture.

S8.2. General procedure for the auto-optimized synthesis of compound 16a.

Initial steps involved the preparation of stock solutions for compound 2a (0.1 M in DCE) and reagent 15 (0.1 M in DCE), each filled into separate syringes and connected to a syringe pump. The solutions were then directed through a PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) surrounded by a cylindrical-shaped blue LED light source. Once the solution setup was complete, input ranges for variable flow rates, voltages, and current were specified in the Python code designed for the reaction. The Bayesian optimizer systematically explored these varying reaction conditions, aiming to achieve maximum yield. The optimization process involved running 40-45 experiments and the results were tabulated in the optimization table (Table S16).

S8.2.1. Python code for optimized condition for C=C bond formation, fumarate

Table S16. Auto-optimization table of cross coupling reaction (C=C bond formation, fumarate).

| No. of experiments | Flow rate mL/min | Voltage (V) | Light intensity (W) | area-results | Yield (%) | Space time yield (g mL-1 h-1) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.114805 | 11.92934 | 34.102 | 0.470179 | 36.42071 | 0.033618 |
| 2 | 0.098673 | 12.30966 | 38.515 | 0.854069 | 66.1574 | 0.052485 |
| 3 | 0.103371 | 11.69926 | 31.712 | 0.830144 | 64.3041 | 0.053443 |
| 4 | 0.053233 | 14.41624 | 67.034 | 1.090795 | 84.49451 | 0.036163 |
| 5 | 0.15 | 12.46267 | 41.012 | 0.60424 | 46.80531 | 0.056447 |
| 6 | 0.136292 | 11.89984 | 34.436 | 0.627558 | 48.61151 | 0.053268 |
| 7 | 0.0808 | 13.69994 | 57.96 | 1.106513 | 85.71207 | 0.055682 |
| 8 | 0.070825 | 11.19236 | 26.491 | 0.542513 | 42.0238 | 0.02393 |
| 9 | 0.052078 | 10.03053 | 15.605 | 0.349642 | 27.08376 | 0.01134 |
| 10 | 0.109072 | 12.22021 | 37.699 | 0.508583 | 39.39557 | 0.034548 |
| 11 | 0.100732 | 12.0365 | 35.578 | 0.421314 | 32.63555 | 0.026431 |
| 12 | 0.114871 | 11.90975 | 34.56 | 0.456292 | 35.345 | 0.032643 |
| 13 | 0.148623 | 14.25763 | 65.083 | 1.000345 | 77.48811 | 0.092593 |
| 14 | 0.053555 | 10.00233 | 15.532 | 0.192328 | 14.89802 | 0.006415 |
| 15 | 0.05 | 12.69937 | 44.027 | 0.739746 | 57.30175 | 0.023035 |
| 16 | 0.129758 | 11.12454 | 26.086 | 0.44318 | 34.32936 | 0.035814 |
| 17 | 0.123543 | 12.8464 | 45.873 | 0.795025 | 61.58378 | 0.06117 |
| 18 | 0.142808 | 11.61169 | 31.302 | 0.302434 | 23.42699 | 0.026898 |
| 19 | 0.080479 | 11.63094 | 31.389 | 0.381501 | 29.5516 | 0.019121 |
| 20 | 0.081458 | 14.41624 | 67.034 | 1.148959 | 88.99998 | 0.058288 |
| 21 | 0.065678 | 12.35186 | 39.644 | 0.714388 | 55.33753 | 0.029221 |
| 22 | 0.068097 | 12.28934 | 39.054 | 0.431773 | 33.44574 | 0.018311 |
| 23 | 0.094075 | 10.32873 | 18.363 | 0.20385 | 15.79052 | 0.011943 |
| 24 | 0.054858 | 12.70035 | 44.104 | 0.90043 | 69.74862 | 0.030763 |
| 25 | 0.132587 | 14.37057 | 66.907 | 0.816223 | 63.22577 | 0.067399 |
| 26 | 0.15 | 10 | 15.53 | 0.124816 | 9.668397 | 0.01166 |
| 27 | 0.149923 | 11.35554 | 28.524 | 0.250264 | 19.38581 | 0.023367 |
| 28 | 0.15 | 10 | 16.018 | 0.193638 | 14.99947 | 0.018089 |
| 29 | 0.15 | 10 | 16.018 | 0.19419 | 15.04221 | 0.018141 |
| 30 | 0.081458 | 14.32353 | 66.382 | 1.09276 | 84.64676 | 0.055437 |
| 31 | 0.14972 | 10.04417 | 15.585 | 0.164063 | 12.70858 | 0.015298 |
| 32 | 0.124635 | 14.5 | 68.203 | 0.653962 | 50.6568 | 0.050762 |
| 33 | 0.050661 | 10.01346 | 15.52 | 0.189311 | 14.6643 | 0.005973 |
| 34 | 0.148887 | 14.32353 | 66.382 | 0.96474 | 74.73017 | 0.089456 |
| 35 | 0.059379 | 10 | 15.499 | 0.156635 | 12.13321 | 0.005792 |
| 36 | 0.050761 | 11.45305 | 29.297 | 0.368252 | 28.5253 | 0.011642 |
| 37 | 0.052621 | 12.00522 | 35.367 | 0.75496 | 58.48026 | 0.024741 |
| 38 | 0.090012 | 10.00458 | 15.458 | 0.35354 | 27.38574 | 0.019819 |
| 39 | 0.079081 | 14.4398 | 67.541 | 1.03432 | 80.11992 | 0.050941 |
| 40 | 0.129892 | 10.00877 | 15.502 | 0.417781 | 32.3619 | 0.033797 |
| 41 | 0.080521 | 14.47602 | 68.587 | 1.140182 | 88.32009 | 0.057177 |
| 42 | 0.078882 | 10.02039 | 14.682 | 0.251696 | 19.4967 | 0.012365 |
| 43 | 0.094613 | 14.48282 | 68.427 | 1.068802 | 82.79091 | 0.062978 |
| 44 | 0.094114 | 14.49842 | 68.595 | 1.089417 | 84.38781 | 0.063854 |

Reaction condition: compound 2a (0.1 M in DCE); reagent 15 (0.1 M in DCE); 1 mL reactor volume.

Graphs:

We have plotted 3D graph of optimization table S16, we have taken flow rate on X-axis, blue light intensity (watt) on Y-axis and product yield on Z axis.

Figure S50. AI based system to auto-optimize and navigate this complexity and identify the optimal conditions for the photo activated cross-coupling reaction. Compound 2a (0.1 M in DCE); reagent 15 (0.1 M in DCE); 1 mL reactor volume.

Figure S51. Reproducibility test for the AI based system to auto-optimization for the photo activated cross copling reaction. Compound 2a (0.1 M in DCE); reagent 15 (0.1 M in DCE); 1 mL reactor volume.

Figure S52. AI based system navigated multi-numerical variable complexity and identify the optimal condition for the photo activated cross coupling reaction. Compound 2a (0.1 M in DCE); reagent 15 (0.1 M in DCE); 1 mL reactor volume.

S.8.3. General procedure for AI optimized cross coupling reaction of ethyl diazoetherate and aryldiazoester for synthesis of compound 16a.

Figure S53. Schematic presentation of continuous flow for the synthesis, of compound 16.

To prepare the stock solution of compound 2a (2.20 g, 0.01 mol) was dissolved in DCE (100 mL) and connected with pump.  Another pump was connected with the reagent 15 (1.32 mL, 0.01 mol) dissolved in DCE (100 mL). These two pump out-put is further connected with the T1-mixer and pumps were running with the flow rate 40 L/min.each to maintain the stoichiometry and then passed through PFA tubular reactor (inner diameter = 1 mm, length = 1.3 m, volume = 1 mL) under blue light (67.034 W) exposure. The room temperature of the reactor was controlled by fan attached to the bottom of the photochemical reactor. The out-put of the tubular reactor was connected with spring based back pressure regulator (~3 bar) to maintain the evaporation. First one hour of the product mixture was discarded and next 5 h of the product mixture [24.3 mL; 2a (0.255 g)] was collected in HPLC bottle. The organic EtOAc phase was concentrated under reduced pressure to give the product 16a (0.29 g in 5 h, 89%) as a colourless oil. 1H NMR (400 MHz, CDCl3) δ 7.35– 7.32 (m, 2H), 7.21 – 7.16 (m, 2H), 7.03 (s, 1H), 4.06 (q, J = 7.1 Hz, 2H), 3.79 (s, 3H), 1.10 (t, J = 7.1 Hz, 3H). 13C NMR (126 MHz, CDCl3) δ 166.34, 164.93, 142.65, 134.63, 132.35, 130.36, 129.81, 128.06, 61.02, 52.97, 13.84. IR (max): 2985.96, 1719.69, 1439.07, 1242.01, 832.82 cm-1. HRMS (ESI): m/z calcd for C13H14ClO4 [M+H]+ 269.0575, found 269.0590. Verified the analytical data with those reported in the literature.24

Quantum efficiency () calculation:

The values of the number of incident photons (Nphotons) were calculated using the following

equations. Here,

Nphoton=Pλthc

Area of lamp (a) = 448 cm2 = 352 x 10-4 m2

Power of the light (p) = 67W = 67 𝐽 𝑠­1

λ = Wavelength of the light (420 nm) = 420 x 10-9 m

t = Duration of irradiation time, 5 h = 18000 s,

h = Planck’s constant = 6.626 x 10-34 J s

c = Velocity of light = 3 x 108 m s-1

No of photons incident

Nphoton=67 J s-1 ×420× 10-9 m ×18000 s6.626 × 10-34 J s× 3×108 m s-1

Nphoton= 2.5 ×1024

Mole’s photons incident =NphotonN

[N = Avogadro’s number (6.02214 × 1023)]

Mole’s photons incident=2.5 ×10246.02214× 1023

Mole’s photons incident  =4.1 mole

Moles of product formed is:

Mole’s of product formed =productivitymole. wt

The product formed is = 0.290 g.

ofMole’s of product formed=0.290268

Mole’s of product formed =1.1 ×10-3

Quantum efficiency ():

Quantum efficiency ∅:=Mole’s of product formedMole’s photons incident×100

Quantum efficiency ∅:=1.1×10-34.1 ×100

Quantum efficiency ∅:=0.27 %

Productivity per h:=0.2905= 0.058 g/h

Space time yield :=productivityvolume of reactor

=0.0581=0.058 g mL-1 h-1

Space time yield in previous reported batch process 24

Productivity = 41.5 mg in 12 h

Productivity per h:=0.04112= 0.0034 g/h

Space time yield :=productivityvolume of reactor

=0.00342=0.0017 g mL-1 h-1

Table S17. Comparative result for the carbene insertion (C=C bond formation, fumarate).

| Entry | Compound 2a | Reagent 15a | Product 16a | Comparative result |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | 89%, 12.3 min. (our study) Blue LED, 12 h, 93% 24 Artificial metalloenzymes, 22 h, 68% 25 |

S10. Case study-8: On demand chemical synthesis.

After individually optimizing the experimental conditions and validating our codes for carbene insertion reactions with varied nucleophiles photo chemically, we aimed to integrate all the python codes (AB1, AB2, AB3, AB4, AB5, AB6, AB7) with optimized conditions. We drew inspiration from the natural grafting technique observed in hibiscus plants, where the root remains the same, but a portion of one plant is placed into the branch of another, resulting in the growth of the same kind of flower with different colours. Similarly, in our "digichemtree" approach, we maintain the same starting material, 4-chloro phenyl diazoester (2a), but graft our tree with varied nucleophiles (3a, 5a, 7a, 9a, 11a, 13, 15) to obtain our respective carbene-inserted products photo chemically. The neural network remains unchanged from our individual experiments, but we've now integrated a total of 9 pumps to flow the starting material and nucleophiles. The outlets of these pumps are connected to an 8-port valve for mixing purposes. The resulting stock solution then passes through a 1 mL PFA tubular reactor with a blue LED and tunable power supply. For fraction collection, we've utilized our tailor-made 3D printer module. This integrated system employs 4 syringe pumps and 5 HPLC pumps. As a model reaction, we've conducted 7 chemical transformations with 7 different nucleophiles.

Figure S54. Actual picture of the setup.

S10.1. Python code for integrated optimized condition for carbene insertion reactions.

AB1.PY CODE (for benzoic acid)

AB2.py (for thiophenol)

AB3.py (for aniline)

AB4.py (for styrene)

AB5.py (for phenyl acetylene)

AB6.py (for acetonitrile)

AB7.py (for diazoetherate)

S10.2. General procedure of Integrated AI optimized carbene inserion reaction into diazo ester photochemically.

Figure S55. Schematic diagram of Integrated AI optimized carbene inserion reaction into diazo ester photochemically.

Our interest turned to integrating our protocol into a unified flow process akin to grafting techniques observed in hibiscus plants. In this process, reminiscent of the grafting of branches onto a common trunk, we maintain the same starting material and combine it with different substrates to yield respective products, each designated with a distinct colour. We coined this process "digichemtree," where we sequentially run seven different reagents, each mixed with the starting material to obtain a single product at a time.

Before commencing the experiment, we prepared 0.1 M stock solutions of compound 2a in both EtOAc and DCE, each stored in separate conical flasks covered with aluminum foil. Similarly, we prepared 0.2 M stock solutions of compounds 3a, 5a, and 7a in EtOAc, each loaded into syringes and connected to pumps. Additionally, 0.2 M stock solutions of compounds 9a, 11a, 13a, and 15a in DCE were prepared, also stored in separate conical flasks and connected to pumps. After ensuring all connections were secure, we checked communication ports with the master computer. Our integrated continuous flow setup, coupled with AI, was now prepared for operation. Upon executing the code AB1.py, pumps 1 and 9 were activated. Pump 1 dispensed the 0.2 M solution of 3a in EtOAc, while pump 9 in EtOAc dispensed the 0.1 M solution of 2a, each at an optimized flow rate of 50 μL/min. The resulting reaction mixture flowed through a 1 mL PFA tubular reactor, exposed to blue LED illumination (70 watts), with a residence time of 10 minutes under 3 bar pressure. Upon completion, the first drop of product emerged, and subsequent solution was collected in individual test tubes over a period of 6000 seconds using tailor made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis.

After the successful execution of the AB1.py experiment code, we proceeded to test the AB2.py code. This time, pumps 2 and 9 were activated. Pump 2 delivered the 0.2 M solution of 5a in EtOAc, while pump 9 dispensed the 0.1 M solution of 2a in EtOAc, each at an optimized flow rate of 86 μL/min. The resulting reaction mixture traversed through a 1 mL PFA tubular reactor, subjected to intense blue LED illumination (70 watts), with a residence time of 11 minutes under 3 bar pressure. As the reaction concluded, the initial drop of product appeared, and the ensuing solution was meticulously collected in individual test tubes over a span of 3300 seconds utilizing a tailor-made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis.

Energized by the successful outcomes of the preceding experiments, we were keen to explore our AB3.py code for a photochemical N-H insertion reaction. Initiating the AB3.py experiment code prompted pump 3 to dispense the 0.2 M solution of 7a in EtOAc, while pump 9 delivered the 0.1 M solution of 2a in EtOAc, each flowing at an optimized rate of 46 μL/min. The resultant reaction mixture journeyed through a 1 mL PFA tubular reactor, bathed in vibrant blue LED illumination (70 watts), with a residence time of 10.86 minutes under 3 bar pressure. Upon completion, the droplet of product emerged, and the successive solution was gathered into individual test tubes over a period of 6000 seconds via the tailor-made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis.

Having accomplished three successful experimental runs, we were tantalized by the prospect of investigating the feasibility of the remaining experiment codes. We started with clicking on experiment code AB4.py, now pump 4 and pump 8 is activated, pumping the 0.2 M solution of reagent 9a in DCE with pump 4 and 0.1 M solution of compound 2a in DCE each flowing at an optimized rate of 15 μL/min. The resultant reaction mixture journeyed through a 1 mL PFA tubular reactor, bathed in vibrant blue LED illumination (70 watts), with a residence time of 32.0 min under 3 bar pressure. Upon completion, the droplet of product emerged, and the successive solution was gathered into individual test tubes over a period of 15000 seconds via the tailor-made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis. Next we have run experiment code AB5.py This time, pumps 5 and 8 were activated. Pump 5 delivered the 0.2 M solution of 11a in DCE, while pump 8 dispensed the 0.1 M solution of 2a in DCE, each at an optimized flow rate of 12.5 μL/min. The resulting reaction mixture traversed through a 1 mL PFA tubular reactor, subjected to intense blue LED illumination (70 watts), with a residence time of 40 min under 3 bar pressure. As the reaction concluded, the initial drop of product appeared, and the ensuing solution was meticulously collected in individual test tubes over a span of 3300 seconds utilizing a tailor-made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis.

Then for [3+2] cycloaddtion reaction have done utilizing experiment code AB6.py, in which upon executing, pumps 6 and 8 were activated. Pump 6 dispensed the 0.2 M solution of 13a in DCE, while pump 8 dispensed the 0.1 M solution of 2a in DCE, each at an optimized flow rate of 31 μL/min. The resulting reaction mixture flowed through a 1 mL PFA tubular reactor, exposed to blue LED illumination (70 watts), with a residence time of 16.6 minutes under 3 bar pressure. Upon completion, the first drop of product emerged, and subsequent solution was collected in individual test tubes over a period of 8400 seconds using tailor made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis.

In last for cross coupling reaction experiment code AB7.py is subjected and pump 7 and pump 8 is activated, pumping the 0.2 M solution of reagent 15a in DCE with pump 8 and 0.1 M solution of compound 2a in DCE each flowing at an optimized rate of 40 μL/min. The resultant reaction mixture journeyed through a 1 mL PFA tubular reactor, bathed in vibrant blue LED illumination (70 watts), with a residence time of 12.5 minutes under 3 bar pressure. Upon completion, the droplet of product emerged, and the successive solution was gathered into individual test tubes over a period of 6600 seconds via the tailor-made auto fraction collector. Following the first test tube of product, the subsequent ones were reserved for external analysis

S11. Spectra

Fig. S56. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzoate (4a).

Fig. S57. 13C NMR (126 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzoate (4a).

.

Fig. S58. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzoate (4a).

Fig. S59. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2,4,5-trifluorobenzoate (4b).

Fig. S60. 13C NMR (126 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2,4,5-trifluorobenzoate (4b).

Fig. S61. 19F NMR (376 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2,4,5-trifluorobenzoate (4b).

Fig. S62. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2,4,5-trifluorobenzoate (4b).

Fig. S63. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzo[d][1,3]dioxole-5-carboxylate (4c).

Fig. S64. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzo[d][1,3]dioxole-5-carboxylate (4c).

.

Fig. S65. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzo[d][1,3]dioxole-5-carboxylate (4c).

Fig. S66. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-isobutylbenzoate (4d).

Fig. S67. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-isobutylbenzoate (4d).

Fig. S68. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-isobutylbenzoate (4d).

Fig. S69. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-phenoxybenzoate (4e).

Fig. S70. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-phenoxybenzoate (4e).

Fig. S71. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 4-phenoxybenzoate (4e).

Fig. S72. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzofuran-2-carboxylate (4f).

Fig. S73. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzofuran-2-carboxylate (4f).

Fig. S74. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl benzofuran-2-carboxylate (4f).

Fig. S75. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(2,4,5-trifluorophenyl)acetoxy)acetate (4g).

Fig. S76. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(2,4,5-trifluorophenyl)acetoxy)acetate (4g).

Fig. S77. 19F NMR (471 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(2,4,5-trifluorophenyl)acetoxy)acetate (4g).

Fig. S78. IR spectra of spectra of methyl 2-(4-chlorophenyl)-2-(2-(2,4,5-trifluorophenyl)acetoxy)acetate (4g).

Fig. S79. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(4-isobutylphenyl)acetoxy)acetate (4h).

Fig. S80. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(4-isobutylphenyl)acetoxy)acetate (4h).

Fig. S81. IR spectra of spectra of methyl 2-(4-chlorophenyl)-2-(2-(4-isobutylphenyl)acetoxy)acetate (4h).

Fig. S82. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2-(4-isobutylphenyl)propanoate (4i).

Fig. S83. 1H NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2-(4-isobutylphenyl)propanoate (4i).

Fig. S84. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 2-(4-isobutylphenyl)propanoate (4i).

Fig. S85. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(2-([1,1'-biphenyl]-4-yl)acetoxy)-2-(4-chlorophenyl)acetate (4j).

Fig. S86. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(2-([1,1'-biphenyl]-4-yl)acetoxy)-2-(4-chlorophenyl)acetate (4j).

Fig. S87. IR spectra of spectra of methyl 2-(2-([1,1'-biphenyl]-4-yl)acetoxy)-2-(4-chlorophenyl)acetate (4j).

Fig. S88. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(2-(9H-fluoren-9-yl)acetoxy)-2-(4-chlorophenyl)acetate (4k).

Fig. S89. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-(2-(9H-fluoren-9-yl)acetoxy)-2-(4-chlorophenyl)acetate (4k).

Fig. S90. IR spectra of spectra of methyl 2-(2-(9H-fluoren-9-yl)acetoxy)-2-(4-chlorophenyl)acetate (4k).

Fig. S91. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (3R)-3-methoxy-3-phenylpropanoate (4l).

Fig. S92. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (3R)-3-methoxy-3-phenylpropanoate (4l).

Fig. S93. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (3R)-3-methoxy-3-phenylpropanoate (4l).

Fig. S94. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-methylisoxazole-3-carboxylate (4m).

Fig. S95. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-methylisoxazole-3-carboxylate (4m).

Fig. S96. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-methylisoxazole-3-carboxylate (4m).

Fig. S97. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-acetylthiophene-2-carboxylate (4n).

Fig. S98. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-acetylthiophene-2-carboxylate (4n).

Fig. S99. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-acetylthiophene-2-carboxylate (4n).

Fig. S100. 1H NMR (500 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-chlorothiophene-2-carboxylate (4o).

Fig. S101. 13C NMR (126 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-chlorothiophene-2-carboxylate (4o).

Fig. S102. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl 5-chlorothiophene-2-carboxylate (4o).

Fig. S103. 1H NMR (400 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (1s,3s)-adamantane-1-carboxylate (4p).

Fig. S104. 13C NMR (101 MHz, CDCl3) spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (1s,3s)-adamantane-1-carboxylate (4p).

Fig. S105. IR spectra of spectra of 1-(4-chlorophenyl)-2-methoxy-2-oxoethyl (1s,3s)-adamantane-1-carboxylate (4p).

Fig. S106. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(phenylthio)acetate (6a).

Fig. S107. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(phenylthio)acetate (6a).

Fig. S108. IR spectra of spectra of methyl 2-(4-chlorophenyl)-2-(phenylthio)acetate (6a).

Fig. S109. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-((4-bromophenyl)thio)-2-(4-chlorophenyl)acetate (6b).

Fig. S110. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-((4-bromophenyl)thio)-2-(4-chlorophenyl)acetate (6b).

Fig. S111. IR spectra of spectra of methyl 2-((4-bromophenyl)thio)-2-(4-chlorophenyl)acetate (6b).

Fig. S112. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((4-chlorophenyl)thio)acetate (6c).

Fig. S113. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((4-chlorophenyl)thio)acetate (6c).

Fig. S114. IR spectra of spectra of methyl 2-(4-bromophenyl)-2-((4-chlorophenyl)thio)acetate (6c).

Fig. S115. 1H NMR (500 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((4-fluorophenyl)thio)acetate (6d).

Fig. S116. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((4-fluorophenyl)thio)acetate (6d).

Fig. S117. 19F NMR (471 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((4-fluorophenyl)thio)acetate (6d).

Fig. S118. IR spectra of spectra of methyl 2-(4-bromophenyl)-2-((4-fluorophenyl)thio)acetate (6d).

Fig. S119. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-(p-tolylthio)acetate (6e).

Fig. S120. 1H NMR (101 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-(p-tolylthio)acetate (6e).

Fig. S121. IR spectra of spectra of methyl 2-(4-bromophenyl)-2-(p-tolylthio)acetate (6e).

Fig. S122. 1H NMR (500 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((3-methoxyphenyl)thio)acetate (6f).

Fig. S123. 1H NMR (101 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-((3-methoxyphenyl)thio)acetate (6f).

Fig. S124. IR spectra of spectra of methyl 2-(4-bromophenyl)-2-((3-methoxyphenyl)thio)acetate (6f).

Fig. S125. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-(naphthalen-1-ylthio)acetate (6g).

Fig. S126. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-bromophenyl)-2-(naphthalen-1-ylthio)acetate (6g).

Fig. S127. IR spectra of spectra of methyl 2-(4-bromophenyl)-2-(naphthalen-1-ylthio)acetate (6g).

Fig. S128. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-((1-phenyl-1H-tetrazol-5-yl)thio)acetate (6h).

Fig. S129. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-((1-phenyl-1H-tetrazol-5-yl)thio)acetate (6h).

Fig. S130. IR spectra of spectra of methyl 2-(4-chlorophenyl)-2-((1-phenyl-1H-tetrazol-5-yl)thio)acetate (6h).

Fig. S131. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(phenylamino)acetate (8a).

Fig. S132. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(phenylamino)acetate (8a).

Fig. S133. IR spectra of methyl 2-(4-chlorophenyl)-2-(phenylamino)acetate (8a).

Fig. S134. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(benzylamino)-2-(4-chlorophenyl)acetate (8b).

Fig. S135. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(benzylamino)-2-(4-chlorophenyl)acetate (8b).

Fig. S136. IR spectra of methyl 2-(benzylamino)-2-(4-chlorophenyl)acetate (8b).

Fig. S137. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(dibenzylamino)acetate (8c).

Fig. S138. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(dibenzylamino)acetate (8c).

Fig. S139. IR spectra of methyl 2-(4-chlorophenyl)-2-(dibenzylamino)acetate (8c).

Fig. S140. 1H NMR (500 MHz, CDCl3) spectra of methyl 2-((5-bromopyrimidin-2-yl)amino)-2-(4-chlorophenyl)acetate (8d).

Fig. S141. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-((5-bromopyrimidin-2-yl)amino)-2-(4-chlorophenyl)acetate (8d).

Fig. S142. IR spectra of methyl 2-((5-bromopyrimidin-2-yl)amino)-2-(4-chlorophenyl)acetate (8d).

Fig. S143. 1H NMR (500 MHz, CDCl3) spectra of methyl 2-(tert-butylamino)-2-(4-chlorophenyl)acetate (8e).

Fig. S144. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(tert-butylamino)-2-(4-chlorophenyl)acetate (8e).

Fig. S145. IR spectra of methyl 2-(tert-butylamino)-2-(4-chlorophenyl)acetate (8e).

Fig. S146. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(pyridin-4-yl)-1H-benzo[d]imidazol-1-yl)acetate (8f).

Fig. S147. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(2-(pyridin-4-yl)-1H-benzo[d]imidazol-1-yl)acetate (8f).

Fig. S148. IR spectra of methyl 2-(4-chlorophenyl)-2-(2-(pyridin-4-yl)-1H-benzo[d]imidazol-1-yl)acetate (8f).

Fig. S149. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(indolin-1-yl)acetate (8g).

Fig. S150. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(indolin-1-yl)acetate (8g).

Fig. S151. IR spectra of methyl 2-(4-chlorophenyl)-2-(indolin-1-yl)acetate (8g).

Fig. S152. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(methyl(tetrahydro-2H-pyran-4-yl)amino)acetate (8h).

Fig. S153. 13C NMR (101 MHz, CDCl3) spectra of Methyl 2-(4-chlorophenyl)-2-(methyl(tetrahydro-2H-pyran-4-yl)amino)acetate (8h).

Fig. S154. IR spectra of Methyl 2-(4-chlorophenyl)-2-(methyl(tetrahydro-2H-pyran-4-yl)amino)acetate (8h).

Fig. S155. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(diisopropylamino)acetate (8i).

Fig. S156. 13C NMR (126 MHz, CDCl3) spectra of methyl 2-(4-chlorophenyl)-2-(diisopropylamino)acetate (8i).

Fig. S157. IR spectra of methyl 2-(4-chlorophenyl)-2-(diisopropylamino)acetate (8i).

Fig. S158. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(((3s,5s,7s)-adamantan-1-yl)amino)-2-(4-chlorophenyl)acetate (8j).

Fig. S159. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(((3s,5s,7s)-adamantan-1-yl)amino)-2-(4-chlorophenyl)acetate (8j).

Fig. S160. IR spectra of methyl 2-(((3s,5s,7s)-adamantan-1-yl)amino)-2-(4-chlorophenyl)acetate (8j).

Fig. S161. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-phenylcyclopropane-1-carboxylate (10a).

Fig. S162. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-phenylcyclopropane-1-carboxylate (10a).

Fig. S163. IR spectra of spectra of methyl 1-(4-chlorophenyl)-2-phenylcyclopropane-1-carboxylate (10a).

Fig. S164. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(4-fluorophenyl)cyclopropane-1-carboxylate (10b).

Fig. S165. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(4-fluorophenyl)cyclopropane-1-carboxylate (10b).

Fig. S166. 19F NMR (471 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(4-fluorophenyl)cyclopropane-1-carboxylate (10b).

Fig. S167. IR spectra of spectra of methyl 1-(4-chlorophenyl)-2-(4-fluorophenyl)cyclopropane-1-carboxylate (10b).

Fig. S168. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(4-methoxyphenyl)cyclopropane-1-carboxylate (10c).

Fig. S169. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(4-methoxyphenyl)cyclopropane-1-carboxylate (10c).

Fig. S170. IR spectra of spectra of methyl 1-(4-chlorophenyl)-2-(4-methoxyphenyl)cyclopropane-1-carboxylate (10c).

Fig. S171. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-bromophenyl)-2-phenylcyclopropane-1-carboxylate (10d).

Fig. S172. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-bromophenyl)-2-phenylcyclopropane-1-carboxylate (10d).

Fig. S173. IR spectra of spectra of methyl 1-(4-bromophenyl)-2-phenylcyclopropane-1-carboxylate (10d).

Fig. S174. 1H NMR (500 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcyclopropane-1-carboxylate (10e).

Fig. S175. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcyclopropane-1-carboxylate (10e).

Fig. S176. 19F NMR (377 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcyclopropane-1-carboxylate (10e).

Fig. S177. IR spectra of spectra of methyl 1-(4-fluorophenyl)-2-phenylcyclopropane-1-carboxylate (10e).

Fig. S178. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-methoxyphenyl)-2-phenylcyclopropane-1-carboxylate (10f).

Fig. S179. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-methoxyphenyl)-2-phenylcyclopropane-1-carboxylate (10f).

Fig. S180. IR spectra of spectra of methyl 1-(4-methoxyphenyl)-2-phenylcyclopropane-1-carboxylate (10f).

Fig. S181. 1H NMR (500 MHz, CDCl3) spectra of methyl 1-(4-nitrophenyl)-2-phenylcyclopropane-1-carboxylate (10g).

Fig. S182. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-nitrophenyl)-2-phenylcyclopropane-1-carboxylate (10g).

Fig. S183. IR spectra of spectra of methyl 1-(4-nitrophenyl)-2-phenylcyclopropane-1-carboxylate (10g).

Fig. S184. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cyclopropane-1-carboxylate (10h).

Fig. S185. 13C NMR (126 MHz, CDCl3) spectra of methyl methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cyclopropane-1-carboxylate (10h).

Fig. S186. 19F NMR (471 MHz, CDCl3) spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cyclopropane-1-carboxylate (10h).

Fig. S187. IR spectra of spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cyclopropane-1-carboxylate (10h).

Fig. S188. 1H NMR (300 MHz, CDCl3) spectra of ethyl 1,2-diphenylcyclopropane-1-carboxylate (10i).

Fig. S189. 13C NMR (101 MHz, CDCl3) spectra of ethyl 1,2-diphenylcyclopropane-1-carboxylate (10i).

Fig. S190. IR spectra of spectra of ethyl 1,2-diphenylcyclopropane-1-carboxylate (10i).

Fig. S191. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12a).

Fig. S192. 13C NMR (126 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12a).

Fig. S193. IR spectra of spectra of methyl 1-(4-chlorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12a).

Fig. S194. 1H NMR (500 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(3-fluorophenyl)cycloprop-2-ene-1-carboxylate (12b).

Fig. S195. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-chlorophenyl)-2-(3-fluorophenyl)cycloprop-2-ene-1-carboxylate (12b).

Fig. S196. IR spectra of spectra of methyl 1-(4-chlorophenyl)-2-(3-fluorophenyl)cycloprop-2-ene-1-carboxylate (12b).

Fig. S197. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-bromophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12c).

Fig. S198. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-bromophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12c).

Fig. S199. IR spectra of spectra of methyl 1-(4-bromophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12c).

Fig. S200. 1H NMR (500 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12d).

Fig. S201. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12d).

Fig. S202. 19F NMR (377 MHz, CDCl3) spectra of methyl 1-(4-fluorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (3c).

Fig. S203. IR spectra of spectra of methyl 1-(4-fluorophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (3c).

Fig. S204. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-methoxyphenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12e).

Fig. S205. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12e).

Fig. S206. IR spectra of spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12e).

Fig. S207. 1H NMR (400 MHz, CDCl3) spectra of methyl 1-(4-nitrophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12f).

Fig. S208. 13C NMR (101 MHz, CDCl3) spectra of methyl 1-(4-nitrophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12f).

Fig. S209. IR spectra of spectra of methyl 1-(4-nitrophenyl)-2-phenylcycloprop-2-ene-1-carboxylate (12f).

Fig. S210. 1H NMR (400 MHz, CDCl3) spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12g).

Fig. S211. 13C NMR (101 MHz, CDCl3) spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12g).

Fig. S212. 19F NMR (376 MHz, CDCl3) spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12g).

Fig. S213. IR spectra of spectra of methyl 2-(4-fluorophenyl)-1-(4-methoxyphenyl)cycloprop-2-ene-1-carboxylate (12g).

Fig. S214. 1H NMR (500 MHz, CDCl3) spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cycloprop-2-ene-1-carboxylate (12h).

Fig. S215. 13C NMR (176 MHz, CDCl3) spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cycloprop-2-ene-1-carboxylate (12h).

Fig. S216. 19F NMR (471 MHz, CDCl3) spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cycloprop-2-ene-1-carboxylate (12h).

Fig. S217. IR spectra of spectra of methyl 2-phenyl-1-(2,4,5-trifluorophenyl)cycloprop-2-ene-1-carboxylate (3h).

Fig. S218. 1H NMR (500 MHz, CDCl3) spectra of 4-(4-chlorophenyl)-5-methoxy-2-methyloxazole (14a).

Fig. S219. 13C NMR (101 MHz, CDCl3) spectra of 4-(4-chlorophenyl)-5-methoxy-2-methyloxazole (14a).

Fig. S220. IR spectra of spectra of 4-(4-chlorophenyl)-5-methoxy-2-methyloxazole (14a).

Fig. S221. 1H NMR (400 MHz, CDCl3) spectra of 4-ethyl 1-methyl 2-(4-chlorophenyl)maleate (16a).

Fig. S222. 13C NMR (126 MHz, CDCl3) spectra of 4-ethyl 1-methyl 2-(4-chlorophenyl)maleate (16a).

Fig. S223. IR spectra of spectra of 4-ethyl 1-methyl 2-(4-chlorophenyl)maleate (16a).

S12. Troubleshooting

Question: Where to buy AI-system for the reaction optimization?

Answer: No need to buy from any place the python code is available in supporting information.

Question: Where to buy the 3D printer and Arduino board?

Answer: We bought through Amazon.

Question: Any other company photo-flow reactor will work?

Answer: Yes, it’ll work but need to change the python code accordingly.

Question: Does other tube id work?

Answer: Other higher tube id (e.g., 2-5 mm) also work, but further need to optimize to get the optimal condition.

Question: Does code will work for the production 100 kg scale?

Answer: Yes, it’ll work but further auto-optimization may give you better reaction condition.

Question: Does current code will work with any other company hardware set-up?

Answer: Yes, it’ll work but slight modification needed in python code.

Question: Can we replace the In-line IR with other analysis unit?

Answer: Yes, you can replace with HPLC, In-line NMR, GC-MS, LC-MS, XAS etc but the analysis code need to rewrite as per the analysis system manual.

Question: Do we need the super computer to run the auto-reaction optimization?

Answer: We don’t need any supercomputer. Any normal PC will work for the reaction auto-optimization and on demand synthesis.

Question: Optimization will have effected by the current failure?

Answer: Yes, before starting the experiment make sure system have proper battery back-up.

Question: How many pump we can connect with one AI system?

Answer: Infinite.

S13. References:

1	Rana, A., Chauhan, R. & Singh, A. K. Thermal/photochemical micro-flow probe system for direct C–H bond functionalization of biologically active molecules. React. Chem. Eng., 9, 1313-1319, doi:10.1039/D4RE00083H (2024).

2	Jurberg, I. D. & Davies, H. M. L. Blue light-promoted photolysis of aryldiazoacetates. Chem. Sci., 9, 5112-5118, doi:10.1039/C8SC01165F (2018).

3	Tan, F. et al. Asymmetric Catalytic Insertion of α-Diazo Carbonyl Compounds into O–H Bonds of Carboxylic Acids. ACS Catal., 6, 6930-6934, doi:10.1021/acscatal.6b02184 (2016).

4	Keipour, H., Jalba, A., Delage-Laurin, L. & Ollevier, T. Copper-Catalyzed Carbenoid Insertion Reactions of α-Diazoesters and α-Diazoketones into Si–H and S–H Bonds. J. Org. Chem., 82, 3000-3010, doi:10.1021/acs.joc.6b02998 (2017).

5	Keipour, H., Jalba, A., Tanbouza, N., Carreras, V. & Ollevier, T. α-Thiocarbonyl synthesis via the FeII-catalyzed insertion reaction of α-diazocarbonyls into S–H bonds. Org. Biomol. Chem.,  17, 3098-3102, doi:10.1039/C9OB00261H (2019).

6	Pan, J. B. et al. A Spiro Phosphamide Catalyzed Enantioselective Proton Transfer of Ylides in a Free Carbene Insertion into N− H Bonds. Angew. Chem., 135, e202300691 (2023).

7	Wang, B., Howard, I. G., Pope, J. W., Conte, E. D. & Deng, Y. Bis (imino) pyridine iron complexes for catalytic carbene transfer reactions. Chem. Sci., 10, 7958-7963 (2019).

8	Wang, B., Howard, I. G., Pope, J. W., Conte, E. D. & Deng, Y. Bis(imino)pyridine iron complexes for catalytic carbene transfer reactions. Chem. Sci., 10, 7958-7963, doi:10.1039/C9SC02189B (2019).

9	Dasgupta, A. et al. Borane-catalyzed stereoselective C–H insertion, cyclopropanation, and ring-opening reactions. Chem 6, 2364-2381 (2020).

10	Mancinelli, J. P. & Wilkerson-Hill, S. M. Tris(pentafluorophenyl)borane-Catalyzed Cyclopropanation of Styrenes with Aryldiazoacetates. ACS Catal., 10, 11171-11176, doi:10.1021/acscatal.0c03218 (2020).

11	Sharland, J. C. et al. Asymmetric synthesis of pharmaceutically relevant 1-aryl-2-heteroaryl- and 1,2-diheteroarylcyclopropane-1-carboxylates. Chem. Sci., 12, 11181-11190, doi:10.1039/D1SC02474D (2021).

12	Davies, H. M. & Venkataramani, C. Dirhodium tetraprolinate-catalyzed asymmetric cyclopropanations with high turnover numbers. Org. Lett., 5, 1403-1406 (2003).

13	Singha, S., Buchsteiner, M., Bistoni, G., Goddard, R. & Fürstner, A. A New Ligand Design Based on London Dispersion Empowers Chiral Bismuth–Rhodium Paddlewheel Catalysts. J. Am.Chem. Soc., 143, 5666-5673 (2021).

14	Chen, L. et al. An Inexpensive and Recyclable Silver-Foil Catalyst for the Cyclopropanation of Alkenes with Diazoacetates under Mechanochemical Conditions. Angew. Chem. Int. Ed., 54, 11084-11087, doi:https://doi.org/10.1002/anie.201504236 (2015).

15	Hommelsheim, R., Guo, Y., Yang, Z., Empel, C. & Koenigs, R. M. Blue-Light-Induced Carbene-Transfer Reactions of Diazoalkanes. Angew. Chem.Int. Ed., 58, 1203-1207, doi:https://doi.org/10.1002/anie.201811991 (2019).

16	Li, C., Zeng, Y. & Wang, J. Au-catalyzed isomerization of cyclopropenes: a novel approach to indene derivatives. Tetrahedron Lett., 50, 2956-2959 (2009).

17	Chen, L., Leslie, D., Coleman, M. G. & Mack, J. Recyclable heterogeneous metal foil-catalyzed cyclopropenation of alkynes and diazoacetates under solvent-free mechanochemical reaction conditions. Chem. Sci., 9, 4650-4661, doi:10.1039/C8SC00443A (2018).

18	Stefkova, K., Heard, M. J., Dasgupta, A. & Melen, R. L. Borane catalysed cyclopropenation of arylacetylenes. Chem. Comm., 57, 6736-6739 (2021).

19	Muriel, B. & Waser, J. Azide radical initiated ring opening of cyclopropenes leading to alkenyl nitriles and polycyclic aromatic compounds. Angew. Chem. Int.Ed., 60, 4075-4079 (2021).

20	Chuprakov, S., Rubin, M. & Gevorgyan, V. Direct palladium-catalyzed arylation of cyclopropenes. J. Am. Chem. Soc., 127, 3714-3715 (2005).

21	Saha, A. et al. Photoinduced [3+2] Cycloaddition of Carbenes and Nitriles: A Versatile Approach to Oxazole Synthesis. Angew. Chem.Int. Ed., 62, e202308916, doi:https://doi.org/10.1002/anie.202308916 (2023).

22	Maiti, D., Saha, A., Guin, S., Maiti, D. & Sen, S. Unveiling catalyst-free electro-photochemical reactivity of aryl diazoesters and facile synthesis of oxazoles, imide-fused pyrroles and tetrahydro-epoxy-pyridines via carbene radical anions. Chem. Sci., 14, 6216-6225, doi:10.1039/D3SC00089C (2023).

23	Bai, J. et al. Visible light-induced synthesis of polysubstituted oxazoles from diazo compounds. Org. Biomol. Chem., 21, 5511-5515, doi:10.1039/D3OB00878A (2023).

24	Xiao, T., Mei, M., He, Y. & Zhou, L. Blue light-promoted cross-coupling of aryldiazoacetates and diazocarbonyl compounds. Chem. Comm., 54, 8865-8868, doi:10.1039/C8CC04609C (2018).

25	Upp, D. M. et al. Engineering Dirhodium Artificial Metalloenzymes for Diazo Coupling Cascade Reactions**. Angew. Chem. Int. Ed., 60, 23672-23677, doi:https://doi.org/10.1002/anie.202107982 (2021).