Supplementary Information

Jiaming Li, Shui Wang, Lisen Bai, Guangbing Liu, Jingzhong Tao and Wei Liu*

Jiangsu Key Lab of Environmental Engineering, Jiangsu Provincial Academy of Environmental Science, Nanjing 210036, People’s Republic of China;

*	Correspondence: jshblw@126.com; Tel.: +86 025-58527708

LC-QTOF-MS analysis and mass spectrometry data analysis

The samples were analyzed using liquid chromatography (LC; Agilent Technologies, Waldbronn, Germany) coupled with a high-resolution hybrid quadrupole time-of-flight mass spectrometer (Triple TOF 5600, AB Sciex, Foster City, CA). Reversed-phase liquid chromatographic separation was performed on a C18 column (ZORBAX Eclipse XDB-C18, 2.1 × 150 mm, 3.5 μm, Agilent, Germany) with 5% acetonitrile water (v/v) and methanol as mobile phase. Details of the gradient elution program are given in Table S1. The electrospray ionization (ESI) source was used in the mass spectrometer, and its positive and negative ion modes operate in the mass range of m/z = 50 - 1250 Da. Details regarding LC-QTOFMS analysis parameters are provided in Table S2. The default sample/blank peak intensity (= 3) was set for all chemical features to eliminate background pollutants. The instrument was automatically calibrated using a calibration solution delivered via a calibration delivery system (AB Sciex) to guarantee a mass error less than 5 ppm.

To evaluate the sensitivity of the LC-QTOF-MS instrument and screening method, we purchased 74 kinds of standards from accustandard (Table S3), and spiked into Milli-Q water, and obtained five concentrations levels: 0.1 ng/L, 1.0 ng/L, 10.0 ng/L, 100.0 ng/L, and 1000.0 ng/L. After enrichment by the above pretreatment method and screening by the above screening process, it was found that 74, 65, 60, 25 and 8 standards were detected in the samples of 1000.0 ng/L, 100.0 ng/L, 10.0 ng/L, 1.0 ng/L and 0.1 ng/L, respectively. The result proves that our screening method can identify pollutants at ng/L.

We modified the criteria used for tentative identification to help us identify more relevant compounds, by, for example, decreasing the intensity threshold from 10,000 to 1,000, and expanding the suspect list database. For the suspect screening, we also followed the approach adopted in a previous study, with suspect peaks isolated using the 5 following criteria: (1) Intensity >1000, (2) S: N >10, (3) Isotopic differences <20%, (4) Mass accuracy <5 ppm, and (5) MS/MS spectra interpretation. We compared the MS/MS spectra with the AB SCIEX commercial MS/MS spectra library and MassBank spectra library (http://www.massbank.jp/). The suspect database included the (1) AB SCIEX commercial database, which comprises 1283 chemicals (mostly pesticides and pharmaceuticals); (2) List of Hazardous Chemicals for China, which comprises 1796 chemicals, and (3) 148 chemicals that are produced and used by chemical enterprises in the CIP that are highly hazardous to the environment.

## Figure S1. Sample sites of GH River (G-1 to G-10), ZB River (ZB-1 to Zb-8), XY River (XY-1 to XY-6), and Yellow Sea coast (YS-1 to YS-10). All samples are taken from surface water or seawater.

## Table S1. Parameters of QTOF-MS.

| Parameter | Details |
| --- | --- |
| Ion Source | ESI |
| Mode | positive & negative |
| MS Scan Range | 50-1250Da |
| Product Ion Scan Range | 30-1250Da |
| Ion Source Temperature | 550℃ |
| Declustering Potential | ESI+:80V; ESI-:-80V |
| CE | ESI+:20,40,60eV; ESI-:-20,-40,-60eV |

## Table S2. Details of Gradient Elution Program for Separation.

| Time（min） | Water with 5% acetonitrile (v/v) (%) | Methanol(%) |
| --- | --- | --- |
| 0 | 100 | 0 |
| 1 | 100 | 0 |
| 9 | 75 | 25 |
| 18 | 50 | 50 |
| 28 | 25 | 75 |
| 39 | 0 | 100 |
| 48 | 0 | 100 |
| 48.10 | 100 | 0 |
| 55 | 100 | 0 |

## Table S3. Information of 74 Standards.

| Compound name | Formula | CAS | Purity | Compound name | Formula | CAS | Purity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Imidacloprid | C9H10ClN5O2 | 105827-78-9 | 99% | Metolachlor | C15H22ClNO2 | 51218-45-2 | 99% |
| Fluometuron | C10H11F3N2O | 2164-17-2 | 99% | Prometryn | C10H19N5S | 7287-19-6 | 99% |
| Buprofezin | C16H23N3OS | 69327-76-0 | 99% | Tricyclazole | C9H7N3S | 41814-78-2 | 99% |
| Carboxin | C12H13NO2S | 5234-68-4 | 99% | Amantadine | C10H17N | 768-94-5 | 99% |
| Cloquintocet-mexyl | C18H22ClNO3 | 99607-70-2 | 99% | Flutriafol | C16H13F2N3O | 76674-21-0 | 99% |
| Cyproconazole | C15H18ClN3O | 113096-99-4 | 99% | Acetamiprid | C10H11ClN4 | 135410-20-7 | 99% |
| Diazinon | C12H21N2O3PS | 333-41-5 | 99% | Clomazone | C12H14ClNO2 | 81777-89-1 | 99% |
| Difenoconazole | C19H17Cl2N3O3 | 119446-68-3 | 99% | Isoprothiolane | C12H18O4S2 | 50512-35-1 | 99% |
| Diuron | C9H10Cl2N2O | 330-54-1 | 99% | Pirimiphos-methyl | C11H20N3O3PS | 29232-93-7 | 99% |
| Linuron | C9H10Cl2N2O2 | 330-55-2 | 99% | Prosulfocarb | C14H21NOS | 52888-80-9 | 99% |
| Malathion | C10H19O6PS2 | 121-75-5 | 99% | Terbutryn | C10H19N5S | 886-50-0 | 99% |
| Metazachlor | C14H16ClN3O | 67129-08-2 | 99% | Triadimefon | C14H16ClN3O2 | 43121-43-3 | 99% |
| Praziquantel | C19H24N2O2 | 55268-74-1 | 99% | Cimetidine | C10H16N6S | 51481-61-9 | 99% |
| Prometon | C10H19N5O | 1610-18-0 | 99% | Bensulfuron methyl | C16H18N4O7S | 83055-99-6 | 99% |
| Propamocarb | C9H20N2O2 | 24579-73-5 | 99% | Isoproturon | C12H18N2O | 34123-59-6 | 99% |
| Thiabendazole | C10H7N3S | 148-79-8 | 99% | Pyrimethanil | C12H13N3 | 53112-28-0 | 99% |
| Thiamethoxam | C8H10ClN5O3S | 153719-23-4 | 99% | Lidocaine | C14H22N2O | 137-58-6 | 99% |
| Acetanilide | C8H9NO | 103-84-4 | 99% | Nicotine | C10H14N2 | 1954/11/5 | 99% |
| Tri-isopropanolamine | C9H21NO3 | 122-20-3 | 99% | Roxithromycin | C41H76N2O15 | 80214-83-1 | 99% |
| Hydroxypropazine | C9H17N5O | 7374-53-0 | 99% | Sulpiride | C15H23N3O4S | 15676-16-1 | 99% |
| Levamisole | C11H12N2S | 14769-73-4 | 99% | Telmisartan | C33H30N4O2 | 144701-48-4 | 99% |
| Ketoconazole | C26H28Cl2N4O4 | 65277-42-1 | 99% | Terbuthylazine | C9H16ClN5 | 5915-41-3 | 99% |
| Dimethomorph | C21H22ClNO4 | 110488-70-5 | 99% | Fluconazole | C13H12F2N6O | 86386-73-4 | 99% |
| Imazalil | C14H14Cl2N2O | 35554-44-0 | 99% | Irbesartan | C25H28N6O | 138402-11-6 | 99% |
| Metalaxyl | C15H21NO4 | 57837-19-1 | 99% | Ornidazole | C7H10ClN3O3 | 16773-42-5 | 99% |
| Prochloraz | C15H16Cl3N3O2 | 67747-09-5 | 99% | Melamine | C3H6N6 | 108-78-1 | 99% |
| Propiconazole | C15H17Cl2N3O2 | 60207-90-1 | 99% | Erythromycin | C37H67NO13 | 114-07-8 | 99% |
| Tebuconazole | C16H22ClN3O | 107534-96-3 | 99% | Venlafaxine | C17H27NO2 | 93413-69-5 | 99% |
| Atrazine | C8H14ClN5 | 1912-24-9 | 99% | Nicosulfuron | C15H18N6O6S | 111991-09-4 | 99% |
| Azoxystrobin | C22H17N3O5 | 131860-33-8 | 99% | 2,4-Dichlorophenol | C6H4Cl2O | 120-83-2 | 99% |
| Carbendazim | C9H9N3O2 | 10605-21-7 | 99% | Chloramphenicol | C11H12Cl2N2O5 | 56-75-7 | 99% |
| Cloquintocet | C11H8ClNO3 | 88349-88-6 | 99% | Benzimidazole | C7H6N2 | 51-17-2 | 99% |
| 4,6-Dimethoxypyrimidine | C6H8N2O2 | 5270-94-0 | 99% | N-Lauroylsarcosine | C15H29NO3 | 97-78-9 | 99% |
| Fluroxypyr | C7H5Cl2FN2O3 | 69377-81-7 | 99% | Atrazine-2-hydroxy | C8H15N5O | 2163-68-0 | 99% |
| 2,4,6-trichlorophenol | C6H3Cl3O | 1988/6/2 | 99% | Clothianidin | C6H8ClN5O2S | 210880-92-5 | 99% |
| 2-Methyl-4-chlorophenoxyacetic acid | C9H9ClO3 | 94-74-6 | 99% | 2-Amino-4,6-dimethoxypyrimidine | C6H9N3O2 | 36315-01-2 | 99% |
| 4,4′-Difluoro diphenylmethanone | C13H8F2O | 345-92-6 | 99% | 2,4-Dichlorophenoxyacetic acid | C8H6Cl2O3 | 94-75-7 | 99% |

## Table S4. Detected substances at each sampling site in GH River.

| Substance | Formula | Level | Usage | hazardous chemical | G-1 | G-2 | G-3 | G-4 | G-5 | G-6 | G-7 | G-8 | G-9 | G-10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4-Dichlorophenol | C6H4Cl2O | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| 2,4,6-Trimethylaniline | C9H13N | 2 | Pesticide, pharmaceutical or dye intermediates |  |  |  | √ | √ | √ | √ | √ | √ | √ | √ |
| 2,4,6-Trichlorophenol | C6H3Cl3O | 2 | chemicals with other uses | √ | √ |  | √ | √ | √ | √ | √ | √ | √ | √ |
| 2,4-Diaminotoluene | C7H10N2 | 2 | Dye | √ | √ |  |  | √ | √ | √ |  |  |  | √ |
| 2,4-Dinitrophenol | C6H4N2O5 | 2 | chemicals with other uses | √ |  | √ |  |  |  |  | √ | √ | √ | √ |
| 2-Ethyl anthraquinone | C16H12O2 | 2 | chemicals with other uses |  |  |  | √ | √ | √ | √ | √ |  |  |  |
| 4-Chloro-o-tolyloxyacetic acid | C9H9ClO3 | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ |  |
| 2-Nitrophenol | C6H5NO3 | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| 4-Chloro-3-methylphenol | C7H7ClO | 2 | chemicals with other uses |  | √ |  | √ | √ | √ | √ | √ |  | √ | √ |
| 2,5-Dimethoxy-4-Methylamphamin | C12H19NO2 | 2 | Pharmaceutical |  |  |  | √ | √ | √ |  | √ | √ | √ | √ |
| 9,10-Dihydroxystearic acid | C18H36O4 | 2 | chemicals with other uses |  |  |  | √ | √ | √ | √ | √ |  |  |  |
| Acetamiprid | C10H11ClN4 | 2 | Insecticide |  | √ |  | √ | √ | √ | √ | √ |  | √ |  |
| Amantadine | C10H17N | 1 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ | √ | √ | √ |  |
| Ametryn | C9H17N5S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ |  | √ |
| Amitraz metabolite | C10H14N2 | 2 | chemicals with other uses |  | √ |  | √ |  | √ | √ |  |  |  |  |
| Anileridine | C22H28N2O2 | 2 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Atrazine | C8H14ClN5 | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Azoxystrobin | C22H17N3O5 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Benomyl | C14H18N4O3 | 2 | Bactericide | √ |  |  |  |  |  |  | √ | √ | √ |  |
| Bensulfuron methyl | C16H18N4O7S | 2 | Herbicide |  | √ | √ | √ | √ | √ |  | √ | √ | √ | √ |
| Bentazone | C10H12N2O3S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ |  | √ | √ |
| Carbendazim | C9H9N3O2 | 1 | Bactericide |  | √ |  | √ | √ | √ | √ |  |  | √ | √ |
| Carbofuran | C12H15NO3 | 2 | Insecticide | √ | √ | √ | √ | √ |  | √ |  |  |  |  |
| Clomazone | C12H14ClNO2 | 1 | Herbicide |  |  |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Dibutyl phthalate | C16H22O4 | 2 | Plasticizer |  |  | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Diethyl phthalate | C12H14O4 | 2 | Plasticizer |  | √ | √ | √ | √ | √ | √ | √ |  | √ | √ |
| Xylenol | C8H10O | 2 | chemicals with other uses |  |  |  | √ | √ | √ |  | √ |  | √ |  |
| Di-n-octyl phthalate | C24H38O4 | 2 | Plasticizer |  | √ | √ | √ | √ | √ | √ | √ |  | √ | √ |
| Diuron | C9H10Cl2N2O | 1 | Herbicide |  | √ |  | √ | √ | √ |  | √ | √ | √ | √ |
| Fenuron | C9H12N2O | 2 | Herbicide |  | √ |  | √ | √ | √ | √ | √ |  |  |  |
| Hexaconazole | C14H17Cl2N3O | 2 | Bactericide |  |  | √ | √ | √ |  |  | √ | √ | √ | √ |
| Isoprothiolane | C12H18O4S2 | 2 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| 1,3-Dinitrobenzene | C6H4N2O4 | 2 | Pesticide, pharmaceutical or dye intermediates | √ |  |  |  |  |  |  |  |  | √ | √ |
| Metalaxyl | C15H21NO4 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ |  | √ |  |
| Metolachlor | C15H22ClNO2 | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Metribuzin | C8H14N4OS | 2 | Herbicide |  |  |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Naltrexone | C20H23NO4 | 2 | Pharmaceutical |  |  |  | √ |  |  |  |  |  |  |  |
| Nonylphenol | C15H24O | 2 | chemicals with other uses | √ |  | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| M-phenylenediamine | C6H8N2 | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ |  |  |  |  |  |  |  |  |  |
| 2-Phenylphenol | C12H10O | 2 | Bactericide | √ |  | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Paclobutrazol | C15H20ClN3O | 2 | Plant Growth Regulator |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Pentachlorophenol | C6HCl5O | 2 | Herbicide | √ |  |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Phenacetin | C10H13NO2 | 1 | Pharmaceutical |  |  |  | √ |  | √ | √ | √ | √ | √ | √ |
| Antipyrine | C11H12N2O | 2 | Pharmaceutical |  | √ |  | √ | √ | √ | √ | √ | √ |  |  |
| Norpseudoephedrine | C9H13NO | 2 | Pharmaceutical |  | √ |  | √ | √ | √ |  | √ | √ | √ |  |
| Prometryn | C10H19N5S | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Ranitidine | C13H22N4O3S | 2 | Pharmaceutical |  |  |  |  | √ |  | √ |  |  |  | √ |
| Rivastigmine | C14H22N2O2 | 1 | Pharmaceutical |  |  |  |  |  |  |  |  |  |  | √ |
| Sulpiride | C15H23N3O4S | 2 | Pharmaceutical |  |  |  |  |  |  | √ |  |  |  |  |
| Telmisartan | C33H30N4O2 | 1 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ | √ |  | √ | √ |
| Tetracaine | C15H24N2O2 | 2 | Pharmaceutical |  |  | √ |  |  |  | √ | √ |  | √ |  |
| Thiamethoxam | C8H10ClN5O3S | 1 | Insecticide |  |  |  | √ |  | √ | √ |  |  |  |  |
| Triazophos | C12H16N3O3PS | 2 | Insecticide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Tributylamine | C12H27N | 2 | Insecticide | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Tricyclazole | C9H7N3S | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ | √ | √ |

## Table S5. Detected substances at each sampling site in ZB River.

| Substance | Formula | Level | Usage | hazardous chemical | ZB-1 | ZB-2 | ZB-3 | ZB-4 | ZB-5 | ZB-6 | ZB-7 | ZB-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4-Dichlorophenol | C6H4Cl2O | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ |  | √ | √ | √ | √ | √ |  |
| 2,4,6-Trimethylaniline | C9H13N | 2 | Pesticide, pharmaceutical or dye intermediates |  |  |  | √ |  | √ |  |  |  |
| 2,4,6-Trichlorophenol | C6H3Cl3O | 2 | chemicals with other uses | √ |  |  | √ | √ |  |  |  |  |
| 2,4-Diaminotoluene | C7H10N2 | 2 | Dye | √ |  |  |  | √ | √ |  |  | √ |
| 2,4-Dinitrophenol | C6H4N2O5 | 2 | chemicals with other uses | √ | √ | √ | √ |  | √ | √ |  | √ |
| 2-Ethyl anthraquinone | C16H12O2 | 2 | chemicals with other uses |  | √ |  |  |  |  |  |  |  |
| 4-Chloro-o-tolyloxyacetic acid | C9H9ClO3 | 2 | Herbicide |  | √ | √ | √ |  | √ |  | √ | √ |
| 2-Nitrophenol | C6H5NO3 | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| 4-Chloro-3-methylphenol | C7H7ClO | 2 | chemicals with other uses |  | √ | √ | √ | √ | √ | √ | √ | √ |
| 2,5-Dimethoxy-4-Methylamphamin | C12H19NO2 | 2 | Pharmaceutical |  |  |  |  | √ |  |  |  |  |
| 9,10-Dihydroxystearic acid | C18H36O4 | 2 | chemicals with other uses |  |  |  |  | √ |  |  |  |  |
| Acetamiprid | C10H11ClN4 | 2 | Insecticide |  | √ |  |  | √ | √ |  | √ | √ |
| Amantadine | C10H17N | 1 | Pharmaceutical |  | √ | √ | √ |  | √ | √ | √ | √ |
| Ametryn | C9H17N5S | 2 | Herbicide |  | √ | √ | √ |  | √ | √ | √ | √ |
| Anileridine | C22H28N2O2 | 2 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Atrazine | C8H14ClN5 | 1 | Herbicide |  | √ | √ | √ |  | √ | √ | √ | √ |
| Azoxystrobin | C22H17N3O5 | 1 | Bactericide |  | √ | √ | √ |  | √ |  |  | √ |
| Barban | C11H9Cl2NO2 | 2 | Herbicide | √ |  |  |  | √ |  |  |  |  |
| Benomyl | C14H18N4O3 | 2 | Bactericide | √ |  |  |  | √ |  |  |  |  |
| Bensulfuron methyl | C16H18N4O7S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Bentazone | C10H12N2O3S | 2 | Herbicide |  | √ | √ | √ | √ | √ |  | √ | √ |
| Carbendazim | C9H9N3O2 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Carbofuran | C12H15NO3 | 2 | Insecticide | √ |  | √ |  | √ |  |  |  | √ |
| Clomazone | C12H14ClNO2 | 1 | Herbicide |  | √ | √ | √ | √ | √ |  |  |  |
| Dibutyl phthalate | C16H22O4 | 2 | Plasticizer |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Diethyl phthalate | C12H14O4 | 2 | Plasticizer |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Xylenol | C8H10O | 2 | chemicals with other uses |  |  |  |  |  |  |  | √ |  |
| Di-n-octyl phthalate | C24H38O4 | 2 | Plasticizer |  | √ | √ | √ | √ |  | √ | √ | √ |
| Fensulfothion | C11H17O4PS2 | 2 | Insecticide | √ |  |  |  |  |  |  | √ |  |
| Hexaconazole | C14H17Cl2N3O | 2 | Bactericide |  | √ | √ | √ |  | √ | √ | √ | √ |
| Isoprothiolane | C12H18O4S2 | 2 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| 1,3-Dinitrobenzene | C6H4N2O4 | 2 | Pesticide, pharmaceutical or dye intermediates | √ |  |  |  | √ |  |  |  |  |
| Metalaxyl | C15H21NO4 | 1 | Bactericide |  | √ | √ | √ |  | √ | √ | √ | √ |
| Metolachlor | C15H22ClNO2 | 1 | Herbicide |  | √ | √ |  |  | √ | √ | √ | √ |
| Metribuzin | C8H14N4OS | 2 | Herbicide |  |  |  | √ | √ |  |  |  |  |
| Naltrexone | C20H23NO4 | 2 | Pharmaceutical |  |  |  |  | √ | √ |  |  |  |
| Nonylphenol | C15H24O | 2 | chemicals with other uses | √ | √ | √ | √ | √ | √ |  |  |  |
| M-phenylenediamine | C6H8N2 | 2 | Pesticide, pharmaceutical or dye intermediates | √ |  |  |  | √ |  |  |  |  |
| 2-Phenylphenol | C12H10O | 2 | Bactericide | √ |  | √ | √ |  | √ |  | √ | √ |
| Paclobutrazol | C15H20ClN3O | 2 | Plant Growth Regulator |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Pentachlorophenol | C6HCl5O | 2 | herbicide | √ |  |  | √ |  |  |  |  |  |
| Phenacetin | C10H13NO2 | 1 | Pharmaceutical |  | √ |  | √ | √ |  |  | √ | √ |
| Antipyrine | C11H12N2O | 2 | Pharmaceutical |  | √ |  |  |  |  |  |  |  |
| Norpseudoephedrine | C9H13NO | 2 | Pharmaceutical |  | √ | √ | √ |  | √ |  | √ |  |
| Prometryn | C10H19N5S | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Prosulfocarb | C14H21NOS | 2 | Herbicide |  |  |  |  | √ |  |  |  |  |
| Ranitidine | C13H22N4O3S | 2 | Pharmaceutical |  | √ |  |  | √ |  | √ |  |  |
| Rivastigmine | C14H22N2O2 | 1 | Pharmaceutical |  |  |  |  |  | √ |  |  |  |
| Sulbactam acid | C8H11NO5S | 2 | Pharmaceutical |  |  |  |  |  | √ | √ |  |  |
| Telmisartan | C33H30N4O2 | 1 | Pharmaceutical |  |  |  | √ |  |  | √ |  |  |
| Tetracaine | C15H24N2O2 | 2 | Pharmaceutical |  |  |  |  | √ |  |  |  |  |
| Triazophos | C12H16N3O3PS | 2 | Insecticide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Tributylamine | C12H27N | 2 | Insecticide | √ | √ | √ | √ | √ | √ | √ | √ | √ |
| Tricyclazole | C9H7N3S | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ | √ | √ |
| Yohimbine | C21H26N2O3 | 2 | Pharmaceutical |  |  | √ |  |  | √ |  |  |  |

## Table S6. Detected substances at each sampling site in XY River.

| Substance | Formula | Level | Usage | hazardous chemical | XY-1 | XY-2 | XY-3 | XY-4 | XY-5 | XY-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,3-Dinitrobenzene | C6H4N2O4 | 2 | Pesticide, pharmaceutical or dye intermediates | √ |  |  | √ | √ |  |  |
| 2,4,6-Trichlorophenol | C6H3Cl3O | 2 | chemicals with other uses | √ | √ | √ | √ | √ | √ | √ |
| 2,4,6-Trimethylaniline | C9H13N | 2 | Pesticide, pharmaceutical or dye intermediates |  |  | √ | √ | √ |  |  |
| 2,4-Dichlorophenol | C6H4Cl2O | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ | √ | √ | √ | √ | √ |
| 2,4-Dinitrophenol | C6H4N2O5 | 2 | chemicals with other uses | √ | √ | √ |  |  |  | √ |
| 2,5-Dimethoxy-4-Methylamphamin | C12H19NO2 | 2 | Pharmaceutical |  |  | √ | √ |  |  |  |
| 2-Ethyl anthraquinone | C16H12O2 | 2 | chemicals with other uses |  |  | √ |  | √ | √ | √ |
| 2-Nitrophenol | C6H5NO3 | 2 | Pesticide, pharmaceutical or dye intermediates | √ | √ | √ | √ |  | √ | √ |
| 2-Phenylphenol | C12H10O | 2 | Bactericide | √ | √ | √ | √ | √ | √ | √ |
| 4-Chloro-3-methylphenol | C7H7ClO | 2 | chemicals with other uses |  | √ |  |  | √ | √ | √ |
| 4-Chloro-o-tolyloxyacetic acid | C9H9ClO3 | 2 | Herbicide |  |  |  |  | √ |  | √ |
| Acetamiprid | C10H11ClN4 | 2 | Insecticide |  | √ |  | √ | √ | √ | √ |
| Amantadine | C10H17N | 1 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ |
| Ametryn | C9H17N5S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Amitraz metabolite | C10H14N2 | 2 | chemicals with other uses |  | √ |  |  |  |  |  |
| Anileridine | C22H28N2O2 | 2 | Pharmaceutical |  |  | √ | √ | √ | √ | √ |
| Antipyrine | C11H12N2O | 2 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ |
| Atrazine | C8H14ClN5 | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Azoxystrobin | C22H17N3O5 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ |
| Barban | C11H9Cl2NO2 | 2 | Herbicide | √ |  |  | √ |  |  |  |
| Benomyl | C14H18N4O3 | 2 | Bactericide | √ | √ |  | √ | √ | √ | √ |
| Bensulfuron methyl | C16H18N4O7S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Bentazone | C10H12N2O3S | 2 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Bifenthrin | C23H22ClF3O2 | 2 | Insecticide |  |  |  |  | √ |  | √ |
| Carbendazim | C9H9N3O2 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ |
| Carbofuran | C12H15NO3 | 2 | Insecticide | √ |  | √ |  | √ |  | √ |
| Clomazone | C12H14ClNO2 | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Cyprodinil | C14H15N3 | 2 | Bactericide |  |  |  | √ |  |  | √ |
| Dibutyl phthalate | C16H22O4 | 2 | Plasticizer |  | √ | √ | √ | √ | √ | √ |
| Diethyl phthalate | C12H14O4 | 2 | Plasticizer |  | √ | √ | √ |  |  |  |
| Di-n-octyl phthalate | C24H38O4 | 2 | Plasticizer |  |  |  |  |  |  | √ |
| Diuron | C9H10Cl2N2O | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Hexaconazole | C14H17Cl2N3O | 2 | Bactericide |  | √ |  | √ | √ | √ | √ |
| Isoprothiolane | C12H18O4S2 | 2 | Bactericide |  | √ | √ | √ | √ | √ | √ |
| Metalaxyl | C15H21NO4 | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ |
| Metolachlor | C15H22ClNO2 | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Metribuzin | C8H14N4OS | 2 | Herbicide |  |  | √ | √ | √ | √ | √ |
| Nonylphenol | C15H24O | 2 | chemicals with other uses | √ | √ | √ | √ | √ | √ | √ |
| Norpseudoephedrine | C9H13NO | 2 | Pharmaceutical |  |  |  | √ | √ |  | √ |
| Paclobutrazol | C15H20ClN3O | 2 | Plant Growth Regulator |  | √ | √ | √ | √ | √ | √ |
| Pentachlorophenol | C6HCl5O | 2 | herbicide | √ |  |  |  | √ |  | √ |
| Phenacetin | C10H13NO2 | 1 | Pharmaceutical |  | √ | √ | √ | √ | √ | √ |
| Prometryn | C10H19N5S | 1 | Herbicide |  | √ | √ | √ | √ | √ | √ |
| Rivastigmine | C14H22N2O2 | 1 | Pharmaceutical |  | √ |  | √ | √ |  |  |
| Sulbactam acid | C8H11NO5S | 2 | Pharmaceutical |  |  |  |  | √ |  | √ |
| Sulpiride | C15H23N3O4S | 2 | Pharmaceutical |  |  |  |  |  | √ |  |
| Telmisartan | C33H30N4O2 | 1 | Pharmaceutical |  | √ | √ | √ |  |  | √ |
| Thiamethoxam | C8H10ClN5O3S | 1 | Insecticide |  |  |  | √ | √ |  |  |
| Triazophos | C12H16N3O3PS | 2 | Insecticide |  | √ | √ | √ | √ | √ | √ |
| Tributylamine | C12H27N | 2 | Insecticide | √ | √ | √ | √ | √ |  | √ |
| Tricyclazole | C9H7N3S | 1 | Bactericide |  | √ | √ | √ | √ | √ | √ |

## Table S7. Detected substances at each sampling site in Yellow Sea Coast.

| Substance | Formula | Level | Usage | hazardous chemical | YS-1 | YS-2 | YS-3 | YS-4 | YS-5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acifluorfen | C14H7ClF3NO5 | 2 | Pesticide, pharmaceutical or dye intermediates |  |  |  | √ |  |  |
| Albendazole | C12H16N2O4 | 2 | chemicals with other uses |  |  |  | √ | √ |  |
| Amantadine | C10H17N | 1 | Herbicide |  |  |  | √ | √ | √ |
| Ametryn | C9H17N5S | 2 | Herbicide |  | √ |  | √ | √ | √ |
| Amphetamine | C9H13N | 2 | Bactericide |  |  |  | √ |  |  |
| Atrazine | C8H14ClN5 | 1 | Pesticide, pharmaceutical or dye intermediates |  | √ | √ | √ |  | √ |
| Atrazine-2-hydroxy | C8H15N5O | 1 | Herbicide |  | √ |  |  |  |  |
| Bentazone | C10H12N2O3S | 2 | Herbicide |  | √ | √ | √ | √ | √ |
| Carbendazim | C9H9N3O2 | 1 | Bactericide |  | √ | √ |  | √ | √ |
| Clomazone | C12H14ClNO2 | 1 | Herbicide |  | √ | √ |  | √ |  |
| Diethyltoluamide | C12H17NO | 2 | Herbicide |  | √ | √ | √ |  | √ |
| Dinitro-ortho-cresol | C7H6N2O5 | 2 | Herbicide |  | √ | √ | √ | √ |  |
| Fludioxonil | C12H6F2N2O2 | 2 | Pesticide, pharmaceutical or dye intermediates |  | √ |  |  |  |  |
| Hexazinone | C12H20N4O2 | 2 | Pharmaceutical |  |  |  | √ | √ |  |
| Hydroxy-atrazine | C8H15N5O | 2 | Herbicide |  |  | √ |  | √ |  |
| Metolachlor | C15H22ClNO2 | 1 | Pharmaceutical |  |  | √ |  |  |  |
| Metribuzin | C8H14N4OS | 2 | Herbicide |  | √ | √ | √ | √ |  |
| N-propylamphetamine | C12H19N | 2 | Herbicide |  |  | √ |  |  |  |
| Phentermine | C10H15N | 2 | Pharmaceutical |  |  | √ |  |  |  |
| Pirimicarb | C11H18N4O2 | 2 | Insecticide |  |  |  | √ | √ |  |
| Prometryn | C10H19N5S | 1 | Herbicide |  | √ | √ | √ |  | √ |
| Propham | C10H13NO2 | 2 | Herbicide |  |  |  |  | √ |  |
| Simazine-2-hydroxy | C7H13N5O | 2 | Pesticide, pharmaceutical or dye intermediates |  |  |  |  |  | √ |
| Simetryn | C8H15N5S | 2 | Herbicide |  | √ | √ |  | √ | √ |
| Sulfentrazone | C11H10Cl2F2N4O3S | 2 | Pharmaceutical |  |  | √ | √ | √ |  |
| Tricyclazole | C9H7N3S | 1 | Bactericide |  | √ | √ | √ | √ | √ |