Mechanistic insights from the atomic-level quaternary structure of short-lived GPCR oligomers in live cells

Michael R. Stoneman1, Koki Yokoi1,*, Gabriel Biener1, Thomas D. Killeen1, Dhruba P. Adhikari1, Sadia Rahman1, Kaleeckal G. Harikumar2, Laurence J. Miller2, Valerică Raicu1,Ɨ

1Department of Physics, University of Wisconsin-Milwaukee, WI 53211, USA;

2Department of Molecular Pharmacology and Experimental Therapeutics, Mayo Clinic, Scottsdale, AZ, USA

Supplementary Results

Supplementary Figure 1: Eapp meta-histograms obtained from HEK-293 cells co-expressing hSecR-GFP2 and hSecR-YFP at different bin origins. The two Eapp peak values selected from each Eapp histograms of the type shown in Figure 2d, were collected to generate the meta-histograms with fixed bin size (0.015) at different bin origins: (a) 0, (b) 0.005, (c) 0.01, and (d) 0.015.

Supplementary Figure 2: Optimization of bin origin of meta-histograms obtained from HEK-293 cells co-expressing hSecR-GFP2 and hSecR-YFP: We defined peak visibility in order to optimize bin origin of the meta-histograms, as a sum of right and left amplitudes of all peaks in a meta-histogram that were selected by using peak selection criteria (see Method section). The peak visibility was calculated for each meta-histogram generated using a different bin origin (as shown in Supplementary Figure 1) and were plotted against corresponding bin origin. The optimum bin origin that corresponds to maximum value of peak visibility was found to be 0.

Supplementary Figure 3: Observation of inward movement of TM6 of hSecR in MD simulation of hSecR structure (6WZG.pdb) without ligand. The structural dynamics of the hSecR by removing ligand and G-protein is monitored by using coarse-grained molecular dynamics (CG MD) simulation. (a) The inward displacement of TM6 is plotted against the simulation time for 9 µs long simulation. The displacement is maximum (13.8 Å) at 2.89 microseconds (µs) of the simulation trajectory. (b) The hSecR structure at 2.89 µs (yellow colored) is considered as inactive structure of hSecR, which is aligned with the original active hSecR structure (cyan colored), shown are cytoplasmic and side view. The dashed red line indicates the inward displacement of TM6 compared to the original active hSecR structure.

Supplementary Figure 4: Average frequency of contact between the protomer domains of MD simulated tetramers featuring interactions between TM4/TM4 (hydrophobic) and TM5 in their initial configurations. The MD simulated FRET efficiency values from these two conjugate tetramer structures were used to achieve the best overall fit to the meta-histogram distributions obtained from cell measurements. The model incorporates two different initial configurations: one where the TM4 domains start as the (a) dimer interface and (b) tetramer interface. Contact frequency at the domain level is determined as explained in Section 3.2.1.

Supplementary Figure 5. Co-existence of two distinct and most probable tetrameric configuration (‘2’ and ‘5’) of hSecR within an octamer. Cartoon representation of hSecR octamer with the cytoplasmic view is displayed in the middle panel. A dashed line box in middle panel outlines the configuration ‘2’ (depicted in left panel) of hSecR tetramer and a solid line box outlines the configuration ‘5’ (depicted in right panel) of the hSecR tetramer.

Supplementary Figure 6: Fitting residuals determined from fitting of experimental Eapp meta-histograms obtained from HEK-293 cells co-expressing hSecR-GFP2 and hSecR-YFP. The experimental Eapp meta-histogram (see Figure 5a) obtained from cells expressing relatively low protomer concentrations was fitted using the oligomer mixture model (see Figure 4d) by using FRET efficiency values obtained from MD simulations of all TM domain conformation pairs. The fitting residual for each conformation pair is plotted against the corresponding domain pair.

Supplementary Figure 7. Determination of the best fit to each of 24 meta-histogram sub-samples using data from MD simulations of all four TM donmain conformation pairs (TM4 hydrophilic and TM5, TM4 hydrophobic and TM5, TM5 and TM6, TM4 hydrophilic and TM6). Eapp meta-histogram sub-samples (empty circle), each were fitted from MD simulations of each of four different TM domain conformation pairs. The best fit (solid red line) among four conformation pairs was obtained for each sub-sampled meta-histogram (indicated with label in legend of each plot).

Supplementary Figure 8. Frequency of occurrence for each of the four TM domain conformation pairs resulting in best fit to a subsampled metahistogram. A bootstrap method was utilized to extract Eapp peaks from the same set of Eapp peak values that was used to generate Eapp meta-histogram in Fig 4. Using this approach, 24 bootstrapped Eapp meta-histograms were generated. Each individual meta-histogram sample was then fitted with the oligomer dissociation model (see Figure 4d) using individual data from MD simulations of different TM Domain Conformation Pairs. The TM domain paring of TM4 (hydrophobic and TM5) (labeled 4/4:5/5 hydrophobic), appeared as the best fit in the majority of samples (18 out of 24 times).

Supplementary Figure 9. Best-fit to the Eapp meta-histogram (empty circles) when the dissociation model included only a mixture of monomers and dimers. In each of the simulations shown, dimers were allowed to form at only the (a) TM4 hydrophobic interface or (b) TM5 interface.

Supplementary Table 1. FRET efficiency peak locations for the single-segment FRET efficiency histograms displayed in Figure 2. Peak locations which correspond to one of the major peaks seen in the meta-histogram (0.26, 0.32, 0.43, 0.48. 0.56) are highlighted in bold.

| Histogram # | Peak 1 | Peak 2 |
| --- | --- | --- |
| 1 | 0.33 | 0.37 |
| 1’ | 0.33 | 0.26 |
| 1’’ | 0.33 | 0.26 |
| 2 | 0.26 | 0.29 |
| 2’ | 0.26 | 0.35 |
| 2’’ | 0.26 | 0.19 |
| 3 | 0.26 | 0.15 |
| 3’ | 0.26 | 0.21 |
| 3’’ | 0.27 | 0.37 |

Supplementary Table 2. List of thirty different starting tetramer structures which were constructed, inserted into membrane, and equilibrated. For each of the structures, the identifying label and TM domains corresponding to the ‘Dimer’ and ‘Tetramer’ interfaces are listed.

| Tetramer Structure | Dimer Interface | Tetramer Interface |
| --- | --- | --- |
| TM1/TM1:TM4/TM4 | TM1 | TM4 |
| TM4/TM4:TM1/TM1 | TM4 | TM1 |
| TM1/TM1:TM5/TM5 | TM1 | TM5 |
| TM5/TM5:TM1/TM1 | TM5 | TM1 |
| TM1/TM1:TM6/TM6 | TM1 | TM6 |
| TM6/TM6:TM1/TM1 | TM6 | TM1 |
| TM2/TM2:TM4/TM4 | TM2 | TM4 |
| TM4/TM4:TM2/TM2 | TM4 | TM2 |
| TM2/TM2:TM5/TM5 | TM2 | TM5 |
| TM5/TM5:TM2/TM2 | TM5 | TM2 |
| TM2/TM2:TM6/TM6 | TM2 | TM6 |
| TM6/TM6:TM2/TM2 | TM6 | TM2 |
| TM3/TM3:TM5/TM5 | TM3 | TM5 |
| TM5/TM5:TM3/TM3 | TM5 | TM3 |
| TM3/TM3:TM6/TM6 | TM3 | TM6 |
| TM6/TM6:TM3/TM3 | TM6 | TM3 |
| TM4/TM4:TM5/TM5 | TM4 | TM5 |
| TM5/TM5:TM4/TM4 | TM5 | TM4 |
| TM4/TM4:TM6/TM6 | TM4 | TM6 |
| TM6/TM6:TM4/TM4 | TM6 | TM4 |
| TM5/TM5:TM6/TM6 | TM5 | TM6 |
| TM6/TM6:TM5/TM5 | TM6 | TM5 |
| TM7/TM7:TM3/TM3 | TM7 | TM3 |
| TM3/TM3:TM7/TM7 | TM3 | TM7 |
| TM7/TM7:TM4/TM4 | TM7 | TM4 |
| TM4/TM4:TM7/TM7 | TM4 | TM7 |
| TM7/TM7:TM5/TM5 | TM7 | TM5 |
| TM5/TM5:TM7/TM7 | TM5 | TM7 |
| TM4/TM4 (hydrophobic):TM5/TM5 | TM4 hydrophobic | TM5 |
| TM5/TM5: TM4/TM4 (hydrophobic) | TM5 | TM4 hydrophobic |

Supplementary Table 3. List of eight different starting tetramer structures which MD simulations were performed on. For each of the eight tetramer structures, the identifying label, TM domains corresponding to the ‘Dimer’ and ‘Tetramer’ interfaces, which TM Domain Conformation pair the structure belonged to are listed.

| TM Domain Conformation Pair | MD-Starting Structures | Dimer Interface | Tetramer Interface |
| --- | --- | --- | --- |
| 1 | TM4/TM4:TM5/TM5 | TM4 | TM5 |
|  | TM5/TM5:TM4/TM4 | TM5 | TM4 |
| 2 | TM4/TM4 (hydrophobic):TM5/TM5 | TM4 hydrophobic | TM5 |
|  | TM5/TM5: TM4/TM4 (hydrophobic) | TM5 | TM4 hydrophobic |
| 3 | TM4/TM4:TM6/TM6 | TM4 | TM6 |
|  | TM6/TM6:TM4/TM4 | TM6 | TM4 |
| 4 | TM5/TM5:TM6/TM6 | TM5 | TM6 |
|  | TM6/TM6:TM5/TM5 | TM6 | TM5 |

Supplementary Table 4. Average dissociation probabilities for the interfaces comprising TM domain conformation pairs of hSecR oligomers which resulted in the best-fit of one or more bootstrapped FRET meta-histograms with MD simulated data. MD-simulated data of each TM domain pair configuration was used to fit separately each of the 24 Eapp meta-histogram subsamples generated by bootstrapping of the peaks of the Eapp histograms used to create the final meta-histograms. The average ± SD of dissociation probabilities for a particular TM domain conformation pair was computed exclusively from the subset of fittings wherein the respective conformation pair emerged as the optimal fit for a specific Eapp meta-histogram, with 'N' denoting the number of occurrences where the orientation best fit one of the 24 bootstrapped meta-histograms. Domain 1 and Domain 2 refers to the TM domains indicated on the left and right, respectively, of the slash in the TM Domain Conformation Pair column.

| TM Domain Conformation Pair | N | Domain 1 | Domain 2 |
| --- | --- | --- | --- |
| TM4 (hydrophobic)/TM5 | 18 | 0.56±0.06 | 0.39±0.15 |
| TM4 (hydrophilic)/TM5 | 5 | 0.40±0.20 | 0.47±0.22 |
| TM6/TM5 | 1 | 0.54 | 0.56 |

Supplementary Table 5. Relative abundances of monomers and oligomers for various TM domain conformation pairs obtained from fitting bootstrapped FRET meta-histograms with MD simulated data. MD-simulated FRET data for each TM domain conformation pair were used to fit separately each of the 24 boot-strapped Eapp meta-histograms drawn from the original Eapp meta-histogram. The average ± SD fraction of hSecR protomers existing as monomers, or participating in the formation of either dimers, trimers or tetramers was computed from the average dissociation probabilities (See Supplementary Table 2) and the formulas shown in the bottom panel of Figure 4d.

| TM Domain Conformation Pair | N | Monomers | Dimers | Trimers | Tetramers |
| --- | --- | --- | --- | --- | --- |
| TM4 (hydrophobic)/TM5 | 18 | 0.61±0.03 | 0.22±0.07 | 0.12±0.04 | 0.05±0.01 |
| TM4 (hydrophilic)/TM5 | 5 | 0.45±0.27 | 0.32±0.19 | 0.09±0.06 | 0.07±0.18 |
| TM5/TM6 | 1 | 0.18 | 0.06 | 0.02 | 0.73 |

Supplementary Table 6. Comparison of average value of FRET efficiency (Eapp) between wild type hSecR and mutant hSecR (GLY243ALA, ILE247ALA). The average Eapp values for each type of hSecR (WT and Mutant) are determined from filtered ROI segments to ensure comparable distributions of both molecular concentrations and donor-to-acceptor ratio (with the ratio centered around 1:1) between the two types. The average Eapp values were determined from single excitation (Eapp (λ2)) and two-wavelength excitations (Eapp (λ12), correcting possible direct excitation of acceptors) from average value from segments (second and third column). Also, the average Eapp values were determined from two significant peak values selected from Eapp histogram corresponding to each ROI segment [Eapp (λ2,histogram)].

| Data | Eapp (λ2) | Eapp (λ12) | Eapp (λ2,histogram) |
| --- | --- | --- | --- |
| WT-hSecR | 0.118±0.093 | 0.096±0.073 | 0.237±0.068 |
| Mutant-hSecR | 0.114±0.093 | 0.093±0.072 | 0.206±0.059 |

Supplementary Methods

SM1. Generating multiple beamlets using spatial light modulation

A lens focuses a plane wave by adding a space dependent phase to the incident beam. To focus an illumination beam using an SLM at a focal distance f from the SLM, a phase pattern, ϕ(x,y) , must be applied to the SLM mimicking that of the space dependent phase the converging lens adds to the plane wave upon traveling through it. To first order approximation, the specific form of the phase pattern for a lens focusing a plane wave can be written as a function of the position (x,y) on the SLM as:

ϕ(x,y) =πλfx-x02+y-y02 	(SM1)

where λ is the laser beam’s wavelength, and x0 and y0 are the position of the virtual lens’s center relative to the center (0,0) of the SLM. The field of the beam as it reflects from the SLM can be written as:

E(x,y) =A∙eiϕ(x,y) z ,	(SM2)

Where, z is the direction of propagation of the beam, A is the incident beam amplitude and ϕ is given in Equation (1). Extending the lens concept to multiple lenses involves superimposing the complex amplitude patterns of each lens and calculating the combined phase angle. The beam emerging from an array of M lenses can be written as an interference between beams emerging from each lens, i.e.:

ETotal=m=1MAi∙eiπλfx-x0,m2+y-y0,m2z=ATotal∙eiϕTotalz .	(SM3)

Where and x0,m, and y0,m are the center coordinates of the mth lens. The combined phase, ϕTotal, is not a simple summation of all the phases contributed by the lenses, but is determined as follows:

ϕTotal=argm=1MAm∙eiπλfx-x0,m2+y-y0,m2.	(SM4)

Here, the Am terms are independently adjusted to control the amplitude of each lens. In the ideal case the spots should have equal intensities, however in practice the spot intensities vary among spots. One reason for this variation is attributed to aberrations added at each transmission through any optical element. To tackle this inhomogeneity, we equalize the fluorescence intensity striking the camera from each individual spot by changing the amplitudes Am in an iterative procedure.

To understand how the beam intensity will look like in the lenses focal plane, we need to simulate propagation of the beam towards the focal plane using Fresnel diffraction propagation and calculate the intensity by taking resulting field multiplied by its conjugate, i.e.,

I=FrATotal∙eiϕTotal∙FrATotal∙eiϕTotal*,	(SM5)

where Fr… represent Fresnel propagation transformation and (...)* represents conjugation. The intensity at the focal plane of the lenses presented multiple spots with their central coordinates equal to the coordinates of lenses’ centers.

SM2.: Data Acquisition

Description of the 2D array of excitation voxels used for imaging

We patterned both of the excitation laser beams into a 4 x 8 array, i.e., 4 evenly spaced rows with 8 individual voxels within a row. The distance in the sample plane between adjacent excitation voxels within a row was dx=12.7 µm, and the distance between adjacent rows was dy=16.5 µm (See Supplementary Figure 10b). These distances corresponded to a distance between fluorescence spots on the EMCCD of ∆px=50 pixels along the columns and ∆py=65 pixels along the rows (Supplementary Figure 10c). The sample is scanned with two voxel arrays, each for a different excitation wavelength, so their resulting signals reach the EMCCD array concurrently. To distinguish signals from adjacent voxels excited by the two lasers, the signal from each is spatially separated on the EMCCD. The voxels from the two excitation lasers are staggered by 3.8 µm in the sample plane, corresponding to 15 pixels between the centers of the PSF spots on the EMCCD.

Supplementary Figure 10. Array of excitation voxels and corresponding fluorescence signals, generated by projecting a phase mask onto an SLM. (a) SLM-generated phase mask resulting in the excitation laser beam being patterned into an 8x4 array of beamlets. (b) Depiction of the laser array at the sample plane. The red circles indicate the center of an excitation voxel. (c) Fluorescence emission pattern of a uniformly fluorescing sample of uranine which was excited by the laser beam array generated using the phase mask depicted in A. The superimposed blue rectangle encompasses a region of pixels on the EMCCD which are used to reconstruct the micro-spectroscopic data set. A collection of snapshots such as these are reconstructed into a 3D data cube where each slice of the data cube is the 2D fluorescence emission of the field of view at a different emission wavelength. The columns of each row are summed to obtain a fluorescent intensity value to be placed at a single pixel in a single wavelength channel of the reconstructed data cube. Each of the rows within the rectangle corresponds to emission in a different wavelength band.

Raw data format

The raw data used to reconstruct the micro-spectroscopic images obtained from a single scan of the sample consisted of a series of frames obtained using the “External Trigger in Frame Transfer” mode of the iXon Ultra EMCCD. The exposure time of each individual frame (i.e., time between external triggers) was 7.5 msec and the vertical bin of the EMCCD was set to five pixels. Following the acquisition of the first frame, the galvanometer controlling the movement of excitation voxels in the 'x' direction was adjusted, moving each voxel by a distance (dx) of 253 nm in the sample plane, corresponding to a one-pixel shift of the fluorescence signal from each excitation voxel on the EMCCD. Subsequently, the galvanometers paused at this new position for the duration of the EMCCD exposure time (7.5 msec), capturing the second frame in the kinetic series (n=2). This galvo movement along the ‘x’ direction was repeated 52 times, with a 7.5 msec exposure at each step, until the signal from voxeli,j overlapped with the region on the EMCCD where the signal from voxeli+1,j struck the EMCCD in frame n=1. Then, the angle of the galvanometer controlling movement of the excitation voxels in the ‘y’ direction was adjusted such that the excitation voxel moved by a distance corresponding to dy=253 nm in the sample plane, while the position along the ‘x’ direction reverted to that of frame n=1.  Then a sweep across the sample in the ‘x’ direction was repeated. The whole process was repeated for 65 different ‘y’ galvo positions.

SM3. Image reconstruction

Calibration scan

The raw data consists of N=3380 snapshots (or frames). For each snapshot, there are M spot locations (e.g., M=32 for an 8x4 array). In order to reconstruct the raw data into 2D fluorescence images, a uniformly fluorescing sample, i.e., a 100 µM solution of fluorescein sodium salt (uranine), was first scanned using the same exact scanning acquisition parameters as was used to collect data on the cells of interest. Therefore, frames of the uranine emission were acquired at the exact same galvo positions used to scan the sample of interest.  Each frame of the calibration acquisition contains a total of M fluorescence “spots” on the EMCCD, where the fluorescence is spread along the columns of the camera due to the PSF of the emission beam and along the rows of the EMCCD due to the emission PSF and transmission grating placed before the detector. The row, rm,n, and column, cm,n, positions of the fluorescence PSF center from each image spot, m, in the 2d array of beamlets is determined and saved for each galvo position/frame, n. This set of coordinates was then used to reconstruct the raw data acquired from acquisitions of cells.

Reconstruction of a single laser excitation

The set of coordinates (rm,n, cm,n) obtained from the analysis of the uranine calibration scan is used to reconstruct the raw data into a 3D data cube where each slice of the data cube is the 2D fluorescence emission of the field of view at a different emission wavelength. For a given frame n, the fluorescence intensity value to be placed in the reconstructed fluorescence image data cube, RI, is computed for each of the M spots using the following formula:

RI(j,k,l)=x=cm,n-b/2x=cm,n+b/2Inx,y ,	(SM6)

where Inx,y is the fluorescence intensity at column x and row y=rm,n+l of frame n, and cm,n and rm,n are determined from the calibration scan. The indices j and k represent the column and row values, respectively, of the reconstructed image of spectral channel l for which the fluorescence intensity value computed from the sum is placed. The fluorescence intensity value to be placed in the reconstructed images corresponds to the summed intensity values across b columns of the raw snapshot; b is the user chosen bin size of the reconstruction (see blue rectangular region of Supplementary Figure 9c). This process of summing across b pixels is repeated for all rows y within a reconstruction window. The data along the rows of the image correspond to different wavelengths of the emission emanating from the same excitation spot. Therefore, for each snapshot, data within the reconstructing window correspond to an emission spectrum of a single excitation voxel and are reflected as such in the reconstructed dataset.  By selecting a subset of pixels from the detection PSF (as imaged by the camera), we have been able to reduce the out-of-focus signal, with consequent reduction in the thickness of the layer of sample probed (or the excitation volume).

The distance between beamlets, ∆px, across the entire array of beams does vary on the order of a pixel. Specifically, the collection of distances between beamlets forms a distribution with standard deviation equal to one. Therefore, in order to obtain a uniform sampling of the field of view, the galvanometers “oversample” the number of points between two beamlets. For example, for a set of beamlets whose average spacing is 50 pixels, the galvos acquire frames at 52 different stopping locations in order to accumulate the fluorescence data to reconstruct a single row of the output reconstructed data cube. In the calibration reconstruction scan, identical row/column pairs which originate from a different snapshot n therefore arise. However, only one of those pairs is used in the reconstructed data.

SM4. Determination of Receptor Concentration

The receptor concentration was calculated in each pixel following the protocol established by Paprocki et al. Briefly, the donor (GFP2) and acceptor (YFP) concentration in each pixel of a membrane segment are determined from the measured quantities Fexc#DA  and Fexc#AD as follows [1]:

A=1ε1AF1AD-F2ADρex,D1-ρex,Aρex,D-1,	(SM7)

D=1ε2DF2DA+(F2AD-εAρex,AA)QDQA,	(SM8)

where ρex,D=F2DAF1DA and ρex,A=F2ADF1AD are the relative emission intensities at the second excitation wavelength (800 nm) to that of the first excitation wavelength (960 nm). These two values were determined from scans of cells samples expressing either donor only or acceptor only and found to be ρex,D=10.15 and ρex,A=0.13. The molecular brightness of the donor (ε2D) and the acceptor (ε1A) were characterized using microspectroscopic measurements performed on solutions of purified GFP2 and YFP at various concentration levels. The same excitation wavelengths (800 nm for GFP2 and 960 nm for YFP), power, and exposure times specified for the cell measurements above were applied to the fluorescent protein solution measurements. Concentration-dependent fluorescence intensity plots for both donor and acceptor FPs were generated, and the slopes of these plots (psolX), representing the amount of fluorescence signal detected per molar concentration of FP, were used to calculate the molecular brightness of the respective FPs, according to the following:

εX=psolX-∞∞PSF2x,y,zdxdydz ,	(SM9)

where X stands for either the donor or acceptor species (i.e., X= D or A). PSFx,y,z represents the three-dimensional point spread function of the focused excitation beam, with z representing the laser propagation direction. To compute the volume integral in Equation (11), the 𝑃𝑆𝐹(𝑥,𝑦,𝑧) of the focused laser beam was approximated by a Gaussian-Lorentzian function, (see SM4 for details). The brightness value determined in Equation (11) is plugged into Equation (9) and Equation (10) to determine donor and acceptor concentration per image pixel. Finaly, the total concentration of the receptors in the membrane is obtained by adding the number of concentration of donors and acceptors in said pixel.

Using a simple relation between the measured fluorescence intensities and molecular brightness, as well as the relation between molecular brightness and psolX (see SM4 for details), we can calculate the average number of donors or acceptors per pixel, N. In Equation (12), we estimate the number of receptors tagged with a particular fluorophore contained within a volume corresponding to a single image pixel as:

NX=XεX-∞∞PSF2x,y,zdxdydzpsolX, 	(SM10)

Where X is the concentration of either A or D found from Equation (9) or Equation (10), respectively. The total number of receptors, NX, in a single pixel is then found by adding the number of donor-tagged and acceptor-tagged receptors found for said pixel, NTOTAL=ND+NA. The concentration range used to assemble the metahistograms (See 2.4.3 below) was 0 to 200 protomers/µm2; ; this corresponded to a NTOTAL range of 0-48 protomers/pixel.

The total fluorescence detected by the measurement system for a uniform solution (sol) of a given species X (i.e., X=D or A) can be determined by integrating the products of the fluorophore monomeric brightness, εmonX(λex), the concentration of fluorophore, CsolX(x,y,z), and the excitation profile of the laser beam over all space, as given by following []:

FsolXλex=εmonXλex.-∞∞PSF2x,y,z.CsolXx,y,zdxdydz. 	(SM11)

Here, PSF (x,y,z) represents the three-dimensional point spread function of the two-photon optical system, where z represents the direction of laser propagation. For a laser beam focused to a diffraction limited spot, PSF(x,y,z), can be approximated using a Gaussian-Lorentzian function, as follows:

PSFx,y,z=w02w021+zzR2e-2x2+y2w021+zzR2,	(SM12)

where and wo is te 1/e2 beam waist of the excitation laser at the focal point, and  zR=πw02λ, is the Rayleigh length.

It should be noted that FsolXλex corresponds to the total signal obtained for a single pixel of the micro-spectroscopic images, i.e, the signal obtained by integrating the entire emission spectrum for the given pixel. Since the solution is spatially uniform, CsolXx,y,z=CsolX, Equation (s7) can be solved for the monomeric molecular brightness:

εmonX=psolXλex-∞∞PSF2x,y,zdxdydz, 	(SM13)

where psolXλex=FsolXλexCsolX represents the amount of fluorescence intensity detected in a camera pixel per molar concentration of the fluorescent protein. The value psolXλex can be found experimentally from calibration measurements of uniform solutions of fluorescent proteins taken at various concentrations []. The molecular brightness of the donors was found using an excitation wavelength of λex2=800 nm , and the acceptors at λex1=960 nm.

Then, the average number of molecules (NX) residing in a pixel of a fluorescence image can be estimated by using following relation:

NX=FX(λex)εmonX,	(SM14)

Combing Equation (s7) with Equation (s9) results in:

NX=FX(λex)-∞∞PSF2x,y,zdxdydzpsolX.				(SM15)

SM5. Membrane ROI segmentation

Instead of taking whole cell membrane as a region of interest (ROI) or manually selecting certain portion of membrane as a ROI, we developed an algorithm to divide the cell membrane into a number of smaller segments of approximately equal areas. It involves manually outlining a cell and then applying an automatic algorithm to isolate the cell membrane, i.e., the membrane ROI and to divide the membrane ROI into smaller membrane segments of equal area.

An outer polygon was drawn manually around a cell in a fluorescence image as shown in Supplementary Figure 11a. Each vertex of the outer polygon was translated inward by given thickness of the membrane (tmem) to result an inner polygon. Only the pixels located outside of the inner polygon and inside the manually drawn outer polygon were included in the membrane ROI (see Supplementary Figure 11b). The difference of areas between the outer and the inner polygon gives total area of the membrane ROI.  Number of segments (Nseg) was obtained by dividing total area of membrane ROI by required area of segment (Aseg). Here, tmem and Aseg are two input parameters to the algorithm.

To divide the membrane ROI into the smaller segments of equal area (in pixels), coordinates of the large number of possible points along the sides of the polygons (outer and inner) were determined so that these coordinates can be distributed equally to each of the segment.

Supplementary Figure 11:  Typical example of the segmentation of a cell membrane ROI (a) Individual cells located in each of the fluorescence images are circled manually  (b) An automated segmentation algorithm is then used to create a ring-like region of interest (ROI) with a fixed width (10 pixels) which encapsulates only pixels ascribed to the plasma membrane of the  cell and (c) the ROI is divided into smaller segments comprised of approximately 200 pixels.

Specific numbers of point (np,side) that need to be considered in each side of polygon was determined by following relation:

np,side=lsidelside, maxNseg,	 (SM16)

where lside is length of a particular side of the polygon, lside, max is length of the longest side of the polygon. By defining np,side proportional to lsidelside, max, all the points generated along the polygon (in any side) could be placed equally spaced, so that dividing membrane ROI based on numbers of equally spaced points would yield segments of equal area. Also, np,side is made multiple of  Nseg, so that total numbers of points around the polygon can be divided exactly by Nseg without leaving a remainder.

Coordinates to each point (i= 1,2, 3, ……., np,side) on a particular side of the polygon are assigned, starting from known coordinates of one end of the side: (x1, y1), based on coordinates of the other end of the side: (x2, y2), by using following expressions:

x1,i+1=x1                            if x1=x2x1,i- ∆x              if x1>x2x1,i+ ∆x              if x1<x2,	(SM17)

y1,i+1=y1                            if y1=y2mx1,i+c              if y1≠y2,	(SM18)

where m is the slope and c is the y-intercept of the side of the polygon, and ∆x is small increment in x coordinates along the side of the polygon, which are determined by using following relations:

m=y2-y1x2-x1 ,	(SM19)

c=y1-mx1,	(SM20)

∆x=x2-x1np,side .	(SM21)

Coordinates to the specific number of points to each side of both polygons (outer and inner), were generated and collected, and then divided equally into Nseg numbers of subsets. Then, each subset of coordinates was collected to construct smaller ROI segments as projected in the image as shown in Supplementary Figure 11c.

SM6. Contour-guided multiple segmentation

Supplementary Figure 11: Typical example of multiple segmentation of a cell membrane ROI. (a) A typical fluorescence image of single cell. The boundary of the cell is first demarcated manually by drawing a contour around the outer edge of the cell. From this contour an automated algorithm defines a region of interest (ROI) by isolating the pixels within the plasma membrane and demarcates these pixels with the outer and inner red solid lines. The ROI was then divided into smaller segments comprised of approximately 200 pixels. (b-d) Additional segments were generated by shifting each segment in (a) three times, each time by specific number of pixels along the contour, to a new position. The original set of segments are depicted in red color, and the three new sets of segments are shown in cyan, green, and yellow color. Boundary of a typical segment is made thicker to visualize the process of shifting the segments.

Contour-guided multiple segmentation is a technique that systematically re-segments the membrane ROI. It follows the single segmentation accomplished by the membrane ROI segmentation algorithm (SM1) and produces three new set of segments by resampling the large number of points (with specific coordinates) that were generated along the outer and inner polygon of the membrane ROI. The resampling of the points to each segment is carried out three times such that each time, the new segments were generated at new positions along the contour, resulting increase in total number of segments by three times than the single segmentation.

SM7. Peaks selection criteria

The Eapp histograms for membrane segments usually display broad distribution of narrow peaks located at particular FRET efficiency values. The position of these peaks corresponds to the different configuration of oligomers. The most probable configurations are represented by the significant peaks of the histograms and the significant peaks of the histogram are obtained by applying a peak selection criterion, which follow a procedure described step by step as follows:

- Determine local maxima along the histogram curve.

- Obtain nearest left minimum, and nearest right minimum of each maximum value.

- Find left and right amplitude for each peak with following relation:

Right Amplitude=ymax-yright min,	(SM22)

Left Amplitude=ymax-yleft min,	(SM23)

where ymax is height of a maximum, yright min is height of nearest minimum right to the maximum, and  yleft min is height of nearest minimum left to the maximum.

- Determine standard deviation of each peak and their nearest left minimum and right minimum with following relations:

Peak SD=ymax ,	(SM24)

RightmaxSD=yright max ,	(SM25)

RightminSD=yright min .	(SM26)

- Segment the histogram at the horizontal position where Right or Left amplitude pass following threshold:

Right Amplitude >Peak SD+Right min SD,	(SM27)

Left Amplitude >Peak SD+Left min SD.	(SM28)

- From each segment, select a peak which has maximum amplitude over the segment.

- Selected peaks would be considered as significant peaks only if their height is greater than two counts, and both left and right amplitude is also greater than two counts.

- Ignore the peaks with height less than or equal to two.

- Merge peaks with left or right amplitude less than or equal to two counts to the nearest left or right significant peak.

- Finally, the significant peaks are obtained with their nearest right and left minimum.

SM8. Structure of GFP2 and SecR

We referred to 1EMC.pdb [2] (chain A) to prepare GFP2 [8] structure at all atomic (AA) resolution. We replaced some amino acids in the pdb files using Chimera [14] so that these proteins were consistent with the following structures used in the experiments.

GFP2: MVSKGEELFTGVVPILVELDGDVNGHKFSVSGEGEGDATYGKLTLKFICTTGKLPVPWPTLVTTLSYGVQCFSRYPDRMKQHDFFKSAMPEGYVQERTIFFKDDGNYKTRAEVKFEGDTLVNRIELKGIDFKEDGNILGHKLEYNYNSHNVYIMADKQKNGIKVNFKIRHNIEDGSVQLADHYQQNTPIGDGPVLLPDNHYLSTQSKLSKDPNEKRDHMVLLEFVTAAGITLGRTSCTRSEAR

For GFP2, the following replacements were made; Ala2 to Val, GSY66 to a sequence of Ser65-Tyr66-Gly67, His77 to Arg, Arg80 to Gln, Thr167 to Ile, Ala206 to Lys, His231 to Leu, Met233 to Arg, Asp234 to Thr, GLu235 to Ser, Leu236 to Cys, Tyr237 to Thr, Asn238 to Arg and four amino acids: “SEAR” were added at the C-terminal end.

For SecR receptor, the ligand and G-protein complex was removed from 6WZG.pdb [], and 8 missing amino acids: “AHSTGALP” were added at N-terminal end and 32 missing amino acids: “REFPLHPVASFSNSTKASHLEQSQGTCRTSII” are added at C-terminal end. The structure of missing amino acids in SecR was predicted by using alpha-fold[3]. Then SecR-GFP2 structure was constructed by joining SecR and GFP2 by a linker of two amino acids “LE”.

SM9. Determining FRET efficiency of oligomeric complex in MD simulation

To determine FRET efficiency between fluorophore pairs in an MD simulation, we computed distance (ri,j,q,f) and orientation factors (κi,j,q,f2) for all possible donor-acceptor pairs in each frame. This involved extracting relative positions of three beads in CG structures, representing the chromophore backbones (Ser65-Tyr66-Gly67). For distance calculations between fluorophores, the x, y, and z components of the distance vector (ra,b) between the center of mass of the chromophores were computed using the GROMACS command distance.  To compute the orientation factors, several quantities were extracted from each frame of the MD simulation. The difference vectors between the Gly-Ser, rGS, and Gly-Tyr, rGY, backbone beads were determined, again using the GROMACS distance command. The angle between the rGS and rGY vectors was calculated using the GROMACS gangle function.

Dipole orientations for each fluorophore were computed based on transition dipole moments reported by Ansbacher et al [4]. The family of GFP fluorophores share a common structure in the chromophore, with an imidazolidinone ring whose carbonyl group is conjugated via hydrogen bond to another ring. Ansbacher et al determined the dipole moment of GFP forms an angle 𝜔 = 74° with this imidazolidinone carbonyl bond [4]. Based on the PDB structure of GFP (1EMC.pdb ), we found the imidazolidinone carbonyl bond made an angle of 4.6° with respect to the rGY vector connecting the beads of Tyr and Gly. Assuming the transition dipole moment was in the same plane formed by rGS and rGY vectors, we determined the normalized dipole moment, μi, for each fluorophore in a specific simulation frame using a custom Python code and the quantities listed above.

The distance ra,b between fluorophore a and fluorophore b within an oligomer and their corresponding dipole moments, μa and μb, were used to calculate the arbitrary orientation factor κa,b of the fluorophore pair by using following relation:

κa,b=μa∙μb-3μa∙ra,bra,bμb∙ra,brij.	(SM29)

The FRET efficiency (En,k,q,fMD) of an oligomeric complex of size, n, and total number of donors k, having a particular oligomeric configuration, q, was determined from the distance and orientation factors between all possible donor-acceptor pairs within the oligomeric complex for a given frame, f, of an MD simulation using the following relation:

En,k,q,fMD=1ki=1kj=1n-k3κi,j,q,f2R0ri,j,q,f62+ j=1n-k3κi,j,q,f2Rconstri,j,q,f6 ,	(SM30)

where R0 = 5.64 nm is the Forster distance for the GFP2-YFP pair [5] which was found assuming an orientation factor of κ2=2/3 (i.e., assuming isotropic orientation). The sum over j is over all acceptors in the particular configuration q, while the sum over i is over all the donors in the configuration q.

References

1.	Raicu, V., Ab Initio Derivation of the FRET Equations Resolves Old Puzzles and Suggests Measurement Strategies. Biophysical Journal, 2019. 116(7): p. 1313-1327.

2.	Palm, G.J., et al., The structural basis for spectral variations in green fluorescent protein. Nature Structural Biology, 1997. 4(5): p. 361-365.

3.	Mirdita, M., et al., ColabFold: making protein folding accessible to all. Nature Methods, 2022. 19(6): p. 679-682.

4.	Ansbacher, T., et al., Calculation of transition dipole moment in fluorescent proteins—towards efficient energy transfer. Physical Chemistry Chemical Physics, 2012. 14(12): p. 4109-4117.

5.	Zimmermann, T., et al., Spectral imaging and linear un-mixing enables improved FRET efficiency with a novel GFP2-YFP FRET pair. FEBS Letters, 2002. 531(2): p. 245-249.

*Current affiliation: UCB Japan.

Ɨ Corresponding author email address: vraicu@uwm.edu