# Emerging clean technologies: policy-driven cost reductions, implications and perspectives

# Learning rates assumptions

The table below list the learning rates assumed for the different technologies along with the sources:

| Technology | Analog | Learning rate and source |
| --- | --- | --- |
| Alkaline electrolyser stack | None- Learning rate is available for Alkaline electrolysers. | 18 %±6[1] |
| PEM electrolyser stack | Alkaline electrolyser stack | 18 %±6[1] |
| Water electrolysis balance of plant & engineering,procurement and construction (BoP and PEC) | BoP and PEC costs for solar PV | Country specific[2] |
| Direct air capture- Liquid solvent | SO2 scrubbers | 13%[3] |

## Modelling cost reductions

### Water electrolysis

#### Mathematical formulation

- As described in the main paper, we use an experience curve model incorporating local and global learning to project the capital costs of water electrolysis projects.

- By the end of 2030, the capital costs of a water electrolysis project using a stack technology tech ∈Chinese PEM,Western PEM, Chinese Alkaline, Western alkaline  in  country/region reg∈{USA,EU,CHINA,ROW}  is given by:

Ctech,projectedreg= Ctech,currentglobal×xtech,projectedglobalxtech,currentglobal log⁡(1-αtechglobal)log⁡(2)+ CBoP&PEC,currentreg,tech×xprojectedregxcurrentreglog⁡(1-αBOP&PECreg)log⁡(2)

The variables used in the above equations are:

- Ctech,projectedreg are the capital costs of a water electrolysis project in country/region reg using stack technology tech on a dollar per kW basis.

- Ctech,currentglobal is the current cost of stack technology tech on a dollar per kW basis.

- CBoP&PEC,currentreg,tech is the current cost of the BoP&PEC of stack technology tech in region reg on a dollar per kW basis.

- xtech,currentglobal and xtech,projectedglobal denote the current and future cumulative deployed capacities of stack technology tech in MW.

- xcurrentregand xprojectedreg denote the current and future cumulative deployed electrolysis capacity in region reg. Note that this includes all stack technologies.

- αtechglobal is the global learning rate for stack technology tech. Note that the learning rate is the percentage by which the stack costs fall with each doubling of global cumulative capacity of the stack technology.

- αBOP&PECreg is the local BoP&PEC learning rate for region reg. The learning rate is the percentage by which the BoP&PEC costs fall with each doubling of installed electrolyser capacity in region reg.

#### Further modelling assumptions

#### Assumptions on costs:

  - The costs are projected for USA, China, Europe and the rest of the world (RoW). The costs for the BoP&PEC costs are only available for USA/EU and China. The BoP&PEC for the RoW are assumed to be the average of the cost in the west and the cost in China.

  - The BoP&PEC costs for PEM electrolysers is not available for China. We assume that the same relative cost for the BoP&PEC costs in the West holds for China as well.

  - The capital costs are based on [4].

#### Assumptions on installed electrolyser capacity:

- For Chinese-made and western-made alkaline electrolysers, the current installed capacity is taken to be x0ALK= 21.38GW. This figure is based on Bloomberg New Energy Finance (BNEF) estimates of shipments and includes alkaline electrolysers installed in the chlor-alkali industry.

- For PEM electrolysers, the current installed capacity is taken to be x0PEM= 0.4GW. This figure is based on BNEF estimate of shipments.

- The learning for BoP&PEC costs is assumed to start at x0BoP&PEC=0.5GW. This buffer assumes that learning does not start until a few projects have been built and was chosen due to lack of data on electrolyser installed capacity in each country. This buffer is reflective of multiple first-of-a-kind projects deployed by different developers before learning starts with their consequent projects.

#### Assumptions on learning rates:

- The assumed learning rate for PEM and alkaline stacks is 18%. A sensitivity analysis on the results has been made changing the learning rate from 12 to 24%.

- The assumed learning rates for the BoP&PEC are country specific and are summarized in the table below. A sensitivity analysis was conducted by applying the global learning rate 11% to all countries. It is important to note that applying a uniform BoP&PEC learning rate does not eliminate the local learning aspect of the model because the BoP&PEC cost reductions still depend on the cumulative installed capacity within a country/region.

| Country/Region | BoP&PEC learning rate(%) |
| --- | --- |
| EUROPE | 16.3 |
| USA | 13.2 |
| CHINA | 8.9 |
| ROW | 11 |

#### Assumptions on future electrolyser capacity deployments:

- To project cost reductions, an essential part is the future cumulative deployed capacity. Here we rely on the forecast of electrolyser installations from [5].

- A total of 240GW of electrolysis is forecasted in the outlook. A sensitivity analysis varying the installed capacity by -50% was carried out.

## Projections:

- The different stack technologies compete for deployment. The deployed PEM and alkaline capacities are based on the BNEF outlook, while the relative share of western versus Chinese technology variant is varied. A summary of results is shown below. The range of costs reflect sensitivity across learning rates as well as deployed capacity. The blue shaded column denotes the base case where the Chinese and Western technologies have equal share of deployment.

|  |  |  | Share of Chinese technology in global deployed capacity |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technology variant | Cost projection | 0 |  | 10 | 20 | 30 | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
| Chinese alkaline | Lowest | 131 |  | 105 | 92 | 83 | 76 |  | 71 | 67 | 64 | 61 | 59 | 57 |
|  | Base | 131 |  | 112 | 102 | 95 | 89 |  | 85 | 82 | 79 | 76 | 74 | 72 |
|  | Highest | 131 |  | 124 | 119 | 115 | 112 |  | 109 | 107 | 104 | 103 | 101 | 100 |
| Western alkaline | Lowest | 154 |  | 159 | 166 | 173 | 182 |  | 193 | 207 | 224 | 249 | 286 | 355 |
|  | Base | 195 |  | 200 | 206 | 213 | 221 |  | 230 | 242 | 256 | 276 | 305 | 355 |
|  | Highest | 270 |  | 274 | 278 | 283 | 289 |  | 295 | 303 | 312 | 322 | 336 | 355 |
| Chinese PEM | Lowest | 524 |  | 148 | 113 | 97 | 86 |  | 79 | 74 | 69 | 66 | 63 | 60 |
|  | Base | 524 |  | 213 | 176 | 157 | 144 |  | 136 | 129 | 123 | 119 | 115 | 111 |
|  | Highest | 524 |  | 328 | 291 | 271 | 257 |  | 247 | 239 | 232 | 227 | 222 | 218 |
| Western PEM | Lowest | 85 |  | 89 | 93 | 98 | 104 |  | 112 | 122 | 137 | 160 | 209 | 743 |
|  | Base | 158 |  | 163 | 168 | 175 | 183 |  | 192 | 205 | 222 | 249 | 302 | 743 |
|  | Highest | 309 |  | 315 | 321 | 329 | 339 |  | 350 | 365 | 384 | 413 | 465 | 743 |

Table 1: Projected stack costs as a function of relative deployment of Chinese and Western technologies.

- Similarly, the range of capital costs is shown below.

| Country/Region | Technology | Cost projection | Share of Chinese technology in global deployed capacity |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| USA | Chinese alkaline | Lowest | 710 | 684 | 671 | 662 | 655 | 650 | 646 | 643 | 640 | 638 | 636 |
|  |  | Base | 758 | 739 | 729 | 722 | 716 | 712 | 709 | 706 | 703 | 701 | 699 |
|  |  | Highest | 931 | 924 | 919 | 915 | 912 | 909 | 907 | 904 | 903 | 901 | 900 |
|  | Western alkaline | Lowest | 733 | 738 | 745 | 752 | 761 | 772 | 786 | 803 | 828 | 865 | 934 |
|  |  | Base | 822 | 827 | 833 | 840 | 848 | 857 | 869 | 883 | 903 | 932 | 982 |
|  |  | Highest | 1070 | 1074 | 1078 | 1083 | 1089 | 1095 | 1103 | 1112 | 1122 | 1136 | 1155 |
|  | Chinese PEM | Lowest | 1065 | 689 | 654 | 638 | 627 | 620 | 615 | 610 | 607 | 604 | 601 |
|  |  | Base | 1110 | 799 | 762 | 743 | 730 | 722 | 715 | 709 | 705 | 701 | 697 |
|  |  | Highest | 1272 | 1076 | 1039 | 1019 | 1005 | 995 | 987 | 980 | 975 | 970 | 966 |
|  | Western PEM | Lowest | 626 | 630 | 634 | 639 | 645 | 653 | 663 | 678 | 701 | 750 | 1284 |
|  |  | Base | 744 | 749 | 754 | 761 | 769 | 778 | 791 | 808 | 835 | 888 | 1329 |
|  |  | Highest | 1057 | 1063 | 1069 | 1077 | 1087 | 1098 | 1113 | 1132 | 1161 | 1213 | 1491 |
| EU | Chinese alkaline | Lowest | 504 | 478 | 465 | 456 | 449 | 444 | 440 | 437 | 434 | 432 | 430 |
|  |  | Base | 504 | 485 | 475 | 468 | 462 | 458 | 455 | 452 | 449 | 447 | 445 |
|  |  | Highest | 783 | 776 | 771 | 767 | 764 | 761 | 759 | 756 | 755 | 753 | 752 |
|  | Western alkaline | Lowest | 527 | 532 | 539 | 546 | 555 | 566 | 580 | 597 | 622 | 659 | 728 |
|  |  | Base | 568 | 573 | 579 | 586 | 594 | 603 | 615 | 629 | 649 | 678 | 728 |
|  |  | Highest | 922 | 926 | 930 | 935 | 941 | 947 | 955 | 964 | 974 | 988 | 1007 |
|  | Chinese PEM | Lowest | 873 | 497 | 462 | 446 | 435 | 428 | 423 | 418 | 415 | 412 | 409 |
|  |  | Base | 873 | 562 | 525 | 506 | 493 | 485 | 478 | 472 | 468 | 464 | 460 |
|  |  | Highest | 1134 | 938 | 901 | 881 | 867 | 857 | 849 | 842 | 837 | 832 | 828 |
|  | Western PEM | Lowest | 434 | 438 | 442 | 447 | 453 | 461 | 471 | 486 | 509 | 558 | 1092 |
|  |  | Base | 507 | 512 | 517 | 524 | 532 | 541 | 554 | 571 | 598 | 651 | 1092 |
|  |  | Highest | 919 | 925 | 931 | 939 | 949 | 960 | 975 | 994 | 1023 | 1075 | 1353 |
| CHINA | Chinese alkaline | Lowest | 244 | 218 | 205 | 196 | 189 | 184 | 180 | 177 | 174 | 172 | 170 |
|  |  | Base | 265 | 246 | 236 | 229 | 223 | 219 | 216 | 213 | 210 | 208 | 206 |
|  |  | Highest | 278 | 271 | 266 | 262 | 259 | 256 | 254 | 251 | 250 | 248 | 247 |
|  | Western alkaline | Lowest | 267 | 272 | 279 | 286 | 295 | 306 | 320 | 337 | 362 | 399 | 468 |
|  |  | Base | 329 | 334 | 340 | 347 | 355 | 364 | 376 | 390 | 410 | 439 | 489 |
|  |  | Highest | 417 | 421 | 425 | 430 | 436 | 442 | 450 | 459 | 469 | 483 | 502 |
|  | Chinese PEM | Lowest | 613 | 237 | 202 | 186 | 175 | 168 | 163 | 158 | 155 | 152 | 149 |
|  |  | Base | 629 | 318 | 281 | 262 | 249 | 241 | 234 | 228 | 224 | 220 | 216 |
|  |  | Highest | 639 | 443 | 406 | 386 | 372 | 362 | 354 | 347 | 342 | 337 | 333 |
|  | Western PEM | Lowest | 174 | 178 | 182 | 187 | 193 | 201 | 211 | 226 | 249 | 298 | 832 |
|  |  | Base | 263 | 268 | 273 | 280 | 288 | 297 | 310 | 327 | 354 | 407 | 848 |
|  |  | Highest | 424 | 430 | 436 | 444 | 454 | 465 | 480 | 499 | 528 | 580 | 858 |
| ROW | Chinese alkaline | Lowest | 477 | 451 | 438 | 429 | 422 | 417 | 413 | 410 | 407 | 405 | 403 |
|  |  | Base | 477 | 458 | 448 | 441 | 435 | 431 | 428 | 425 | 422 | 420 | 418 |
|  |  | Highest | 519 | 512 | 507 | 503 | 500 | 497 | 495 | 492 | 491 | 489 | 488 |
|  | Western alkaline | Lowest | 500 | 505 | 512 | 519 | 528 | 539 | 553 | 570 | 595 | 632 | 701 |
|  |  | Base | 541 | 546 | 552 | 559 | 567 | 576 | 588 | 602 | 622 | 651 | 701 |
|  |  | Highest | 658 | 662 | 666 | 671 | 677 | 683 | 691 | 700 | 710 | 724 | 743 |
|  | Chinese PEM | Lowest | 838 | 462 | 427 | 411 | 400 | 393 | 388 | 383 | 380 | 377 | 374 |
|  |  | Base | 838 | 527 | 490 | 471 | 458 | 450 | 443 | 437 | 433 | 429 | 425 |
|  |  | Highest | 876 | 680 | 643 | 623 | 609 | 599 | 591 | 584 | 579 | 574 | 570 |
|  | Western PEM | Lowest | 399 | 403 | 407 | 412 | 418 | 426 | 436 | 451 | 474 | 523 | 1057 |
|  |  | Base | 472 | 477 | 482 | 489 | 497 | 506 | 519 | 536 | 563 | 616 | 1057 |
|  |  | Highest | 661 | 667 | 673 | 681 | 691 | 702 | 717 | 736 | 765 | 817 | 1095 |

## Hydrogen production

- We evaluate the impact of projected cost reductions on the levelized cost of hydrogen production. The base case assumptions on the electrolysis plant are shown below.

| Name | Value |
| --- | --- |
| PEM stack lifetime (hours) | 60000 |
| Utilization rate (%) | 70 |
| Plant lifetime (years) | 10 |
| WACC | 10% |
| Capital Recovery Factor (CRF) | 0.146763315 |
| Electrolyser efficiency (%) based on the high heating value of hydrogen | 70 |
| Fixed O&M (%CAPEX) | 5 |
| Electricity costs ($/MWh) | 50 |

- The plot below depicts LCOH sensitivity to key parameters:

Figure 1: Sensitivity of the levelized cost of hydrogen to key input parameters. Values depicted for USA but similar trends hold for all regions.

- The plot below depicts the combined impact of learning-by-doing and the production tax credit 45V enacted in the Inflation Reduction Act:

Figure 2: Current and projected LCOH in the US with and without the 45V production tax credit.

## Dashboard.

An interactive dashboard accompanies this paper. The dashboard allows the user to change model inputs: learning rates, installed capacities, cost of capital, etc. and visualize both the projected capital costs as well as the projected levelized cost of hydrogen.

## Direct Air Capture

### Capital costs.

#### General assumptions

- In modelling the cost reductions of direct air capture technologies, we group them into two classes: Liquid solvent (L-DAC) and solid-sorbent (S-DAC). Each class follows an independent learning curve. The assumed learning rates and the proxies were detailed in the first section of the document.

- We use the 2030 project pipeline as an indicator of current policies’ stimulus of the industry. The announced and planned projected are retrieved from the International Energy Agency’s CCUS project database. The projected 2030 capture capacities are shown in the table below. The projects without an announced operation date were disregarded. A sensitivity analysis with respect to the project pipeline capacity was conducted by varying the projected buildout by ±50%.

| DAC technology | 2030 Project Pipeline capacity (MtCO2/year) |
| --- | --- |
| L-DAC | 3.5 |

#### Mathematical formulation

- The normalized capital costs in dollars per ton of CO2 capture capacity per year of L-DAC and S-DAC are projected to fall following the equations:

CprojectedL-DAC=CcurrentL-DAC×xprojectedL-DACxcurrentL-DAClog⁡(1-αL-DAC)log⁡(2)

The variables used in the above equations are:

- CprojectedL-DAC is the projected normalized capital costs in dollars per ton of carbon dioxide capture capacity per year of L-DAC.

- CcurrentL-DAC is the current normalized capital costs in dollars per ton of carbon dioxide capture capacity per year of L-DAC.

- xprojectedL-DAC is the projected global cumulative deployed capture capacities of L-DAC in million tons of carbon dioxide capture capacity.

- xcurrentL-DAC is the current global cumulative deployed capture capacities of L-DAC in million tons of carbon dioxide capture capacity.

- αL-DAC is the learning rate for L-DAC.

#### Further assumptions

- For L-DAC, learning is assumed to start after the first large scale project is built (xL-DACcurrent=0.5). This project is currently under construction [6].

- The initial capital costs for L-DAC are based on [6].

| DAC technology | Plant Capacity (million tons of CO2 captured per year) | Capital investment (million dollars) |
| --- | --- | --- |
| L-DAC (low capex) | 0.5 | 1300 |

### Cost of capture and net cost of removal

In this section, we model the potential reductions of the cost of capture and removal of the natural gas-powered L-DAC.

#### 2.3.2.1- General assumptions

- The following project and financial parameters were assumed:

| Parameter | Value |
| --- | --- |
| Project lifetime | 20 |
| WACC | 10% |
| Capacity factor | 90% |
| O&M (% capex) | 5 |
| Natural gas consumption (GJ/tCO2-captured) | 8.81[7] |
| Natural gas costs ($/tCO2) | 3.5 |

- The cost of removal of carbon dioxide depends on the net removal factor which is given by:

Net removal factor= CO2 removedCO2 captured= CO2 captured-CO2 emittedCO2 captured

- The cost of removal accounts for the greenhouse gases emitted in the process of capturing carbon dioxide. For the all-natural gas variant of L-DAC, the two sources of emissions are the upstream methane leakage as well as the carbon dioxide not captured from the on-site power generation (a 90% capture rate is assumed).

- We follow the methodology laid out in reference [8].

- The emissions in CO2 equivalent terms per ton of CO2 captured resulting from methane leakage is given by:

CH4 emissions (tCO2,eqtCO2captured)= NGinput GJtCO2capturedHeatofcombustionGJtCH4*leakage rate(%)*GWP of methane(tCO2,eqtCH4).

- The uncaptured emissions in the on-site power plant are given by:

CO2powergenerationtCO2tCO2captured=0.3×NGinput GJtCO2capturedHeatofcombustionGJtCH4×NGcombustionemissions(tCO2tCH4).

Where it is assumed that 30% of the natural gas used in the plant is for power generation. The heat of combustion of natural gas is taken to be  53.4GJ/tCH4, the NG combustion emissions are taken to be 2.7 tCO2/tCH4.

- The plots below show the net removal factors as well as the net removal costs.

Figure 3:Net removal factor and net removal costs for various methane leakage rates varying from 0 to 3.7%.

Figure 4: Current and projected net removal costs under GWP20 and GP100. Error bars depict sensitivity analysis with respect to projected capital costs of L-DAC facilities.

- The evolution of the cost of capture at cumulative deployment level n is calculated as follows:

COCnL-DAC=Annuityn+O&Mn+FC
= CRF1+WACC×CF× CnL-DAC+p× CnL-DAC+FC
=(CRF1+WACC×CF+p )×C0L-DAC×xnL-DACx0L-DAC-LPL-DAC +FC

where CRF is the cost recovery factor, O&M are the operations and maintenance costs, p=5% and FC are the fuel costs.

### Learning investment

In this section, we estimate the cumulative investment required to bring the costs of L-DAC capture down to a target cost net removal cost  CORn*. Multiple scenarios are investigated. First, a base learning scenario where the capital costs and the O&M costs learn at the same learning rate LRL-DAC=13%, and a fast-learning scenario where the learning rate is LRL-DAC=20%.

- We first estimate the capacity xn*L-DACat which the target cost of capture COCn*=NetRemovalFactor×CORn* is reached.

COCn*L-DAC=Annuityn*+O&MnCF+FC
                     = CRF1+WACC×CF× Cn*L-DAC+O&MnCF+FC
                     =CRF1+WACC×CF×C0L-DAC×xn*L-DACx0L-DAC-LPL-DAC +O&M0CFxn*L-DACx0L-DAC-LPL-DAC+FC

This yields:

xn*L-DAC= x0L-DACCOCn*L-DAC-FCCRF×C0L-DAC1+WACC×CF+O&M0CF-1LPL-DAC

From which we deduce the learning investment LI:

LI= x0xn*Cxdx =x0xn*C0L-DAC×xx0L-DAC-LPL-DACdx = C0L-DAC1-LPL-DAC xn*L-DAC×xn*L-DACx0L-DAC-LPL-DAC- x0L-DAC

## Electro-kerosene

In this section, we evaluate the impact of the cost reductions experienced in hydrogen production via water electrolysis and direct air capture on the production costs of e-fuels.

### General assumptions:

- The electro-fuels are produced in a Fischer-Tropsch plant. The plant’s technical and financial assumptions are displayed in the table below and are based on [9].

- The hydrogen feedstock is supplied by a PEM water electrolysis plant except in China, where an alkaline electrolyzer is assumed. The current capital costs of the PEM water electrolysis plant are discussed in the water electrolysis section of the analysis. The projected capital costs are based on our experience curve projections and are region-specific. The plant’s technical and financial assumptions were shown above. For a 70% utilization rate, one stack replacement is needed. This stack replacement is assumed to be at the costs projected in our analysis for plants built at present and at 50% the projected costs of electrolysers for plants built in 2030.

- The carbon dioxide needed for the synthesis of the FT-fuel is provided by a liquid solvent direct air capture plant. The 2022 capital investment and plant characteristics are detailed in the direct-air capture section. The 2030 capital costs are based on our experience curves projection. The cost of removal is also detailed in the direct air capture section and a CO2 transportation cost of $19.45/tCO2 is added.

### The levelized cost of fuel:

The levelized cost of the output FT-fuel is calculated as:

LCOF=FTCAPEX+IH2×LCOH+ICO2×LCOR

Where the variables are:

- FTCAPEX is the contribution of the capital cost of the FTS plant to the levelized cost of the fuel. It is computed as:

FTCAPEX= CAPEX*CRF+Fixed O&M+Variable O&MTotalYearlyOutput

- IH2is the hydrogen requirement per gallon of FT fuel produced ~1.5kg/gal.

- LCOH is the levelized cost of hydrogen production (see water electrolysis section for details).

- ICO2 is the carbon dioxide requirement per gallon of FT fuel produced ~9.5 kg/gal.

- LCOR is the levelized cost of removal of carbon dioxide from the air (see direct air capture section for details).

### Sensitivity analysis

Figure 5: Projected levelized cost of electro-kerosene by country/region and sensitivity with respect to key parameters. The DAC costs sensitivity encompasses varying the learning rate, deployed capacity as well as different levels of methane leakage as specified in the DAC section.

### Impact of e-kerosene blending on aviation fuel costs

To evaluate the potential impact of the adoption of synthetic kerosene on the economics of flights, we estimate the change in fuel costs per passenger. The fuel consumption per passenger is given by:

Fuel gal per passenger=FE gal per passenger-km×L (km)

Where:

- FE is the fuel economy of the aircraft, that is the amount of fuel consumed per passenger carried and kilometer travelled.

- L is the distance of the flight.

We assess the change in fuel costs for a flight connecting New York to London, which is one of the busiest international flight routes. We create three classes of airlines with low (27 passenger-km per liter), average (37.5 passenger-km per liter) and high (48 passenger-km per liter) fuel economy based on data from [10].

We calculate the change in fuel costs when e-kerosene is blended as follows:

change=p×Fuel×CeK+1-p×Fuel×Ck

Where p is the e-kerosene blend level, CeK is the cost of e-kerosene, Fuel is the fuel consumption of the flight per passenger as outlined above, and Ck is the cost of fossil jet fuel.

The tables below show the estimated absolute change in fuel costs of a flight connecting New York and London for different fuel economies and different jet fuel costs under a 5%, and 50% e-kerosene blend.

|  | 5% blend | Low fuel economy |  |  | Mid fuel economy |  |  | High fuel economy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fossil jet fuel cost (USD per gal) |  | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |
| e-kerosene cost (USD per gal) | 4 | 8 | 5 | 3 | 6 | 4 | 2 | 5 | 3 | 2 |
|  | 5 | 11 | 8 | 5 | 8 | 6 | 4 | 6 | 5 | 3 |
|  | 6 | 14 | 11 | 8 | 10 | 8 | 6 | 8 | 6 | 5 |
|  | 7 | 16 | 14 | 11 | 12 | 10 | 8 | 9 | 8 | 6 |
|  | 8 | 19 | 16 | 14 | 14 | 12 | 10 | 11 | 9 | 8 |
|  | 9 | 22 | 19 | 16 | 16 | 14 | 12 | 12 | 11 | 9 |
|  | 10 | 25 | 22 | 19 | 18 | 16 | 14 | 14 | 12 | 11 |
|  | 11 | 27 | 25 | 22 | 19 | 18 | 16 | 15 | 14 | 12 |
|  | 12 | 30 | 27 | 25 | 21 | 19 | 18 | 17 | 15 | 14 |
|  | 13 | 33 | 30 | 27 | 23 | 21 | 19 | 18 | 17 | 15 |
|  | 14 | 35 | 33 | 30 | 25 | 23 | 21 | 20 | 18 | 17 |

T

|  | 50% blend | Low fuel economy |  |  | Mid fuel economy |  |  | High fuel economy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fossil jet fuel cost (USD per gal) |  | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |
| e-kerosene cost ($/gal) | 4 | 82 | 55 | 27 | 58 | 39 | 19 | 46 | 31 | 15 |
|  | 5 | 109 | 82 | 55 | 78 | 58 | 39 | 61 | 46 | 31 |
|  | 6 | 136 | 109 | 82 | 97 | 78 | 58 | 77 | 61 | 46 |
|  | 7 | 164 | 136 | 109 | 117 | 97 | 78 | 92 | 77 | 61 |
|  | 8 | 191 | 164 | 136 | 136 | 117 | 97 | 107 | 92 | 77 |
|  | 9 | 218 | 191 | 164 | 156 | 136 | 117 | 123 | 107 | 92 |
|  | 10 | 246 | 218 | 191 | 175 | 156 | 136 | 138 | 123 | 107 |
|  | 11 | 273 | 246 | 218 | 195 | 175 | 156 | 153 | 138 | 123 |
|  | 12 | 300 | 273 | 246 | 214 | 195 | 175 | 168 | 153 | 138 |
|  | 13 | 328 | 300 | 273 | 234 | 214 | 195 | 184 | 168 | 153 |
|  | 14 | 355 | 328 | 300 | 253 | 234 | 214 | 199 | 184 | 168 |

### References

[1]	O. Schmidt, A. Hawkes, A. Gambhir, and I. Staffell, “The future cost of electrical energy storage based on experience rates,” Nat. Energy, vol. 2, no. 8, p. 17110, Jul. 2017, doi: 10.1038/nenergy.2017.110.

[2]	A. M. Elshurafa, S. R. Albardi, S. Bigerna, and C. A. Bollino, “Estimating the learning curve of solar PV balance–of–system for over 20 countries: Implications and policy recommendations,” J. Clean. Prod., vol. 196, pp. 122–134, Sep. 2018, doi: 10.1016/j.jclepro.2018.06.016.

[3]	K. Riahi, E. S. Rubin, M. R. Taylor, L. Schrattenholzer, and D. Hounshell, “Technological learning for carbon capture and sequestration technologies,” Energy Econ., vol. 26, no. 4, pp. 539–564, 2004, doi: https://doi.org/10.1016/j.eneco.2004.04.024.

[4]	BloombergNEF, “Electrolyzer Price Survey 2024 Rising Costs, Glitchy Tech,” New York, 2024.

[5]	BloombergNEF, “Global Electrolyzer Outlook 2030,” New York, 2022.

[6]	Oxy, “Third Quarter Earnings Conference Call.” Nov. 08, 2023.

[7]	D. W. Keith, G. Holmes, D. St. Angelo, and K. Heidel, “A Process for Capturing CO2 from the Atmosphere,” Joule, vol. 2, no. 8, pp. 1573–1594, Aug. 2018, doi: 10.1016/j.joule.2018.05.006.

[8]	N. McQueen, M. J. Desmond, R. H. Socolow, P. Psarras, and J. Wilcox, “Natural Gas vs. Electricity for Solvent-Based Direct Air Capture,” Front. Clim., vol. 2, 2021, [Online]. Available: https://www.frontiersin.org/articles/10.3389/fclim.2020.618644

[9]	F. Cheng, H. Luo, J. D. Jenkins, and E. D. Larson, “The value of low- and negative-carbon fuels in the transition to net-zero emission economies: Lifecycle greenhouse gas emissions and cost assessments across multiple fuel types,” Appl. Energy, vol. 331, p. 120388, Feb. 2023, doi: 10.1016/j.apenergy.2022.120388.

[10]	B. Graver, “Transatlantic airline fuel efficiency ranking, 2017,” 2017.

The stack’s lifetime is calculated based on the stack lifetime and the utilization rate, no stack replacements are assumed:

Plantlifetime= stacklifetime8760×utilizationrate