Biological Atrazine Removal from Low-Strength Wastewater by Moving Bed Biofilm Reactor and Upflow Fixed Bed Biofilm Reactor: Performance and Kinetic Analysis

Table S1. Operational conditions and atrazine (ATZ) concentrations in the influent of MBBR and FBBR.

| Initial Conditions |  |  |  |  | MBBR |  | FBBR |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Run | Cin Atrz | HRT (hr) | COD/N/P | CODin | CODeff | % COD Removal | CODeff | %COD Removal |
| 1 | 5 | 5 | 100:05:01 | 200 | 99.1 | 50.50 | 84.0 | 58.00 |
| 2 | 5 | 10 | 100:05:01 | 200 | 59.0 | 70.50 | 37.5 | 81.25 |
| 3 | 5 | 24 | 100:05:01 | 200 | 45.1 | 77.45 | 32.5 | 83.75 |
| 4 | 10 | 5 | 100:05:01 | 200 | 92.0 | 54.00 | 78.0 | 61.00 |
| 5 | 10 | 10 | 100:05:01 | 200 | 49.0 | 75.50 | 33.4 | 83.30 |
| 6 | 10 | 24 | 100:05:01 | 200 | 39.6 | 80.18 | 29.0 | 85.50 |
| 7 | 15 | 5 | 100:05:01 | 200 | 83.1 | 58.50 | 70.0 | 65.00 |
| 8 | 15 | 10 | 100:05:01 | 200 | 46.5 | 76.75 | 30.0 | 85.00 |
| 9 | 15 | 24 | 100:05:01 | 200 | 34.7 | 82.63 | 25.5 | 87.25 |
| 10 | 20 | 5 | 100:05:01 | 200 | 99.1 | 50.50 | 76.5 | 61.75 |
| 11 | 20 | 10 | 100:05:01 | 200 | 56.1 | 72.00 | 40.0 | 80.00 |
| 12 | 20 | 24 | 100:05:01 | 200 | 39.6 | 80.20 | 38.5 | 80.75 |
| 13 | 30 | 5 | 100:05:01 | 200 | 117.4 | 41.30 | 92.5 | 53.75 |
| 14 | 30 | 10 | 100:05:01 | 200 | 106.5 | 46.75 | 54.0 | 73.00 |
| 15 | 30 | 24 | 100:05:01 | 200 | 58.2 | 70.90 | 41.5 | 79.25 |
| 16 | 5 | 5 | 100:15:01 | 200 | 114.1 | 43.00 | 94.0 | 53.00 |
| 17 | 5 | 10 | 100:15:01 | 200 | 82.1 | 59.00 | 27.5 | 86.25 |
| 18 | 5 | 24 | 100:15:01 | 200 | 73.6 | 63.20 | 25.0 | 87.50 |
| 19 | 10 | 5 | 100:15:01 | 200 | 109.1 | 45.50 | 111.5 | 44.25 |
| 20 | 10 | 10 | 100:15:01 | 200 | 71.9 | 64.00 | 44.5 | 77.75 |
| 21 | 10 | 24 | 100:15:01 | 200 | 62.5 | 68.75 | 37.5 | 81.25 |
| 22 | 15 | 5 | 100:15:01 | 200 | 105.6 | 47.20 | 89.0 | 55.50 |
| 23 | 15 | 10 | 100:15:01 | 200 | 62.6 | 68.70 | 43.5 | 78.25 |
| 24 | 15 | 24 | 100:15:01 | 200 | 57.5 | 71.25 | 36.5 | 81.75 |
| 25 | 20 | 5 | 100:15:01 | 200 | 99.4 | 50.30 | 82.4 | 58.80 |
| 26 | 20 | 10 | 100:15:01 | 200 | 58.5 | 70.75 | 43.0 | 78.50 |
| 27 | 20 | 24 | 100:15:01 | 200 | 51.3 | 74.50 | 39.0 | 80.50 |
| 28 | 30 | 5 | 100:15:01 | 200 | 106.5 | 46.75 | 99.0 | 50.50 |
| 29 | 30 | 10 | 100:15:01 | 200 | 63.5 | 68.25 | 45.5 | 77.25 |
| 30 | 30 | 24 | 100:15:01 | 200 | 55.4 | 72.30 | 42.5 | 78.75 |