Supplementary Material 1 – MR Enterography Protocol

All MR Entrography performed on 1.5-T Siemens Avanto or 3-T Prisma

Patient Preparation

No solids for 6 hours before scan

Oral preparation agent given according to patient choice:

| Mannitol 20% w/v solution | Lactulose 3.3g/5ml oral solution |
| --- | --- |
| - 250ml mannitol 20% w/v + 750ml water to produce a 1 litre solution of mannitol 5% w/v - Single oral dose mannitol 5% w/v, volume according to weight: - 0-18kg: 375ml - 18-35kg: 500ml - 35-70kg: 1000ml - 70kg+: 1500ml | - 1-4 years: 10ml lactulose with 100ml squash or water - 5-12 years: 20ml lactulose with 100ml squash or water - 12-17 years: 50ml lactulose with 100ml squash or water |

| Step | Pulse Sequence | Coronal or Axial | Parameters |
| --- | --- | --- | --- |
| 1 | Cine-tFISP | Coronal | 30 frames/8 seconds/slice position. Slice thickness 5mm. |
| 2 | Hyoscine Butylbromide (Buscopan) 20mg/ml intravenous injection | 0.4mg/kg intravenously (0.02ml/kg). Intravenous bolus over 3-5 minutes. Maximum doses: 1 month – 4 years: 5mg (0.25ml) 4 years – 12 years: 10mg (0.5ml) >12 years: 20mg (1ml) |  |
| 3 | True FISP | Coronal | Voxel size 1 x 1 x 5mm |
| 4 | T2 HASTE fat saturated | Coronal |  |
| 5 | T2 HASTE | Axial |  |
| 6 | DWI | Axial | B=0-1000 |
| 7 | Gadolinium 0.2mls/kg intravenous |  |  |
| 8 | VIBE | Coronal | 90 second delay |