Supporting Information

Integrated ultra-wideband tunable Fourier domain mode-locked optoelectronic oscillator

Zhen Han, LiHeng Wang, Yong Zheng, Pu Zhang, YongHeng Jiang, HuiFu Xiao, XuDong Zhou, Mingrui Yuan, Mei Xian Low, Aditya Dubey, Thach Giang Nguyen, Andreas Boes, Qinfen Hao,  Guanghui Ren*, Arnan Mitchell and Yonghui Tian*

Z. Han, L.Wang, Y. Zheng, P. Zhang, Y. Jiang, H. Xiao, X. Zhou, M. Yuan, Y. Tian

School of Physical Science and Technology, Lanzhou University, Lanzhou 730000, Gansu, China

E-mail: tianyh@lzu.edu.cn

M. X. Low, A. Dubey, T. G. Nguyen, G. Ren, A. Mitchell

Integrated Photonics and Applications Centre (InPAC), School of Engineering, RMIT University, Melbourne, VIC 3001, Australia

E-mail: guanghui.ren@rmit.edu.au

A. Boes

School of Electrical and Mechancial Engineering, The University of Adelaide, Adelaide, SA 5005, Australia

Institute for Photonics Advanced Sensing, The University of Adelaide, SA 5005, Australia

Q. Hao

Institute of Computing Technology Chinese Academy of Sciences Beijing 100190, China

*Corresponding authors: guanghui.ren@rmit.edu.au, tianyh@lzu.edu.cn

Keywords: Lithium niobate on insulator (LNOI), Integrated microwave photonics (IMWP), Fourier domain mode-locked (FDML), Optoelectronic oscillator (OEO)

- Supplementary Note S1: Threshold conditions for OEO

In this section we will introduce and derive the threshold conditions of the OEO. The most basic OEO loop is shown in figure S1.

Figure S1. Basic structure of the OEO

Assuming that the laser output optical power is P0 and the voltage loaded on the modulator is Vin(t), the modulated output optical power is:

Where α and η are the insertion loss and extinction coefficient of the modulator, Vπ and VB are the half-wave voltage and bias voltage of the modulator.

The modulated optical signal is transmitted through a long optical fiber and is converted by the PD into an electrical signal, which is then selectively filtered by a band-pass filter and amplified by an electrical amplifier. The resulting signal can be expressed as:

Where ρ and R are the response coefficient and impedance of the PD, GA is the gain of the electrical amplifier.

When the input voltage of the modulator is Vin = 0, from Eq. (2), the gain of signal open loop GS is:

When the VB = 0 or Vπ, from Eq. (3), the gain of signal open loop GS is maximized:

In order to achieve self-excited oscillations, it is necessary for OEO to satisfy the initial conditions:

When the OEO achieves stable oscillation, the electrical signal is repeatedly applied to the modulator, which results in an increase in the voltage V0, and a decrease in the gain G(V0). Eventually, the system of OEO reaches an equilibrium state.

- Supplementary Note S2: Principle of Fourier domain mode-locked OEO

In this section we will introduce and derive the principle of Fourier domain mode-locked OEO.

The MBPF is adjusted by a periodic voltage signal, which alters the frequency of the generated microwave signal. The frequency-scanned oscillation should satisfy:

Where τ is the time delay of the loop, nA(t) is the noise induced by the OEO loop and  is the saturation factor of PM-IM conversion.

Eq. (6) shows that a stable solution will repeat itself after the loop round-trip delay. In the FDML OEO, the MBPF is frequency-tuned periodically, the phase change can be described as:

Where  is the target periodic phase variation and  is the phase noise.

Ideally, without any noise the stable oscillation satisfies can be described as:

According to Eq. (7) and Eq. (9),  is a solution.

We can conclude that when the round-trip time equals to n times of the period of the MBPF driving signal, the condition for achieving FDML OEO is satisfied.

Under Fourier-domain mode-locked conditions, the passband of the MBPF is also tuned to exactly the same frequency when each mode in the entire swept range of the OEO is transmitted around the loop and back to the filter again. Consequently, all modes in the swept range can exist in the FDML OEO simultaneously, breaking the limitation of mode building time in a conventional OEO.

- Supplementary Note S3: Design of modulator

In this section we will introduce the design of modulator to get more electro-optical bandwidth.

Figure S3. Simulation design of electro-optical modulator. (a) The micrograph of MZM. (b) Schematic of the cross-section. (c) Simulation of the optical mode field. (d) Simulation of the electric mode field. (e) E-O S21 results of experiments and simulations. (f) Simulation result of group refractive, microwave loss and characteristic impedance.

The design of modulator traveling wave electrodes needs to follow three main principles: (1) The refractive index matching of RF signals and optical waveguides. (2) The characteristic impedance and load resistance matching. (3) The microwave losses in the electrodes are as low as possible.

As shown in figure S3 (a), the width of ground electrode, signal electrode and the gap between the two electrodes are 100 μm, 50 μm and 6 μm, respectively. Figure S3 (b) illustrates the cross-section of the hybrid integrated SiN-LNOI platform modulator. The width of the hybrid integrated waveguide is 1 μm, corresponding to a refractive index of 2.23, and the optical simulation is shown in figure S3 (c). And the figure S3 (e) illustrates the comparison between the simulated and experimentally measured electro-optical bandwidths, which shows that the theory is in good agreement with the actual measurements. Finally, figure S3 (f) shows the simulation results of group refractive index, microwave loss and characteristic impedance considering the influence of various parameters and manufacturing process comprehensively.

- Supplementary Note S4: Relationship between OEO’s mode interval and loop length

In this section we will introduce and derive the relationship between OEO’s mode interval and loop length.

The frequency-dependent complex function F(ω) in the OEO loop can be expressed as follows:

Where F(ω) is the real part of the transfer function and the ϕ(ω) is the phase with respect to signal frequency due to dispersion of all components in the loop.

Assuming that the initial input signal is a single frequency noise:

After n loop cycles:

Where n is the number of loops and τ is the time delay in the loop.

When the gain of signal open loop is less than 1, the output signal can be expressed as:

The output power of the OEO loop can be expressed as:

OEO’s modes that can eventually stabilize the output need to satisfy the phase condition:

The modes that satisfy Eq. (17) can be a stable OEO mode whose frequency and mode interval can be expressed as:

The time delay τ in an OEO loop is primarily influenced by the loop length. Therefore, it can be concluded that the longer the OEO loop, the smaller the mode interval is.

- Supplementary Note S5: Dependence of the OEO’s phase noise on the loop length

In this section we will introduce the Dependence of the OEO’s phase noise on the loop length.

The RF spectrum of OEO can be expressed as:

Where δ is the input noise-to-signal ratio to the OEO, f is the frequency offset from the oscillation frequency, and τ is the time delay in the loop.

If the total length of single mode fiber use in OEO is 46 m, resulting in the phase noise of -93 dBc/Hz at 10 KHz. With 1000 m of single mode fiber and finer filters, the phase noise of the OEO can be calculated to be -132.66 dBc/Hz at 10 KHz [] .

- Supplementary Note S6: The insertion loss of the Fourier domain mode-locked OEO

| Components | Insertion loss (dB) |
| --- | --- |
| Grating coupler | -8.48（-4.24 dB/facet） |
| Phase modulator | -6.61 |
| MRR | -0.29 |
| Total | -15.38 |