Online Resource

Full-field, frequency-domain comparison of simulated and measured human brain deformation

Journal of Biomechanics and Modeling in Mechanobiology

Amir HG. Arani1, Ruth J. Okamoto1, Jordan D. Escarcega1, Antoine Jerusalem2, Ahmed A. Alshareef 3, Philip V. Bayly1*

1 Mechanical Engineering and Materials Science, Washington University, St. Louis, MO, USA

2 Department of Engineering Science, University of Oxford, Oxford OX1 3PJ, UK

3 Department of Mechanical Engineering, University of South Carolina, Columbia, SC, USA

* Corresponding author. E-mail: pvb@wustl.edu

S1  Gel Phantom

S1.1  Methods

Fig.  S1.1a shows an acrylic cylinder filled with Sylgard™ 527 gel, mixed in a 1:1 ratio of part A to part B (Tan et al., 2022) next to the FE model of the gel phantom. The mass of this phantom was 2.05 kg with a gel height and inner diameter of 174 and 113 mm, respectively. The image volume of MRE scans were 240×240×144 mm3 with a 3 mm voxel resolution. The cylindrical FE model of the gel phantom is based on the geometry and material properties of Sylgard 527 gel phantom. The geometry of cylinder phantom was created in ABAQUS (v. 2021, Dassault Systemes Simulia Corp.) with 15,112 hybrid quadratic tetrahedral elements of type C3D10H. The length and radius of the phantom model are 173 and 57 mm, respectively.

A linear viscoelastic model with Rayleigh damping was chosen to represent the material properties of the Sylgard 527 gel phantom. We used a mass density of ρ=990 kgm3 with a shear modulus of G=3.4 kPa for the FE model. We specified Rayleigh damping to be proportional to the stiffness with β=2.94×10-4 s (in this damping model stress σ=Gε+ βGdεdt); see Online Resource Reference for ABAQUS documentation. Both shear modulus and Rayleigh damping were estimated from inversion of the MRE data.

All surfaces of the phantom model have constraints on five degrees of freedom except the free surface in green color shown in Fig S1a. The only remaining free degree of freedom in these surfaces is the rotation around the cylinder axis (z-axis) which has a prescribed rotation of θt=5×10-5cosωt+π4 (corresponding to a complex-valued rotation coefficient of

θ0=5×10-5eiπ4). Simulated strain fields from applied harmonic motion at 20 Hz were compared to strain fields experimentally measured by MRE for a set of ten adjacent 2D axial slices (normal to the longitudinal axis) near the middle of the gel phantom.

S1.2 Comparison of Predicted and Measured Strain Fields in the Gel Phantom

Fig. S1b shows the real component of the shear strain, Re(εxy), for a gel phantom experiment involving 20 Hz harmonic motion and a phantom model simulation at the same frequency.

Fig. S1c shows the comparison of the gel phantom’s experimental strain and the strain of the simulated phantom in terms of LC and Cv. In the comparison of the experimental MRE gel phantom and the simulation phantom, highly similar strain fields generate bright regions in the LC domain. These regions together also exhibit a Cv of 0.87 for the entire comparison domain. Around the edge of the experimental phantom strain data, there is an area with very low estimated strain, which is an artifact of numerical differentiation of displacement fields near the boundary. We deliberately included this artifactual low-strain region in the comparison to demonstrate that LC is capable of capturing this disparity.

Fig. S1  Gel phantom MRE experiment and simulation. (a) a photo of gel phantom and ABAQUS model. (b) Strain components, Realεxy, for 20 Hz harmonic motion in the middle slice. (c) Local (LC) and global (Cv) strain field correlations for the gel phantom. The scale bar (white) represents 57 mm

S2  Participant Information

Participant data are included in the Brain Biomechanics Imaging Resource data repository on the Neuroimaging Tools and Resources Collaboratory (Bayly et al. 2021).

Table S1  Information about six participants in MRE experiments involving multi-frequency harmonic motion induced by a lateral actuator.

| Participant ID | U01 ID | Sex | Age | BrainVolume cm3 (including ventricles) |
| --- | --- | --- | --- | --- |
| P01 P02 P03 P04 P05 P06 | U01_WUSTL_0009_01 U01_WUSTL_0014_01 U01_WUSTL_0006_01 U01_WUSTL_0007_01 U01_WUSTL_0008_01 U01_WUSTL_0026_01 | F F M M M F | 23 25 27 31 26 20 | 1246.69 1198.75 1273.68 1531.11 1323.56 1254.49 |

Table S2  Participant-specific rigid-body motion complex-valued rotations and translations estimated from MRE images

| Frequency & Participant ID | X-rot μm/mm | Y-rot μm/mm | Z-rot μm/mm | X-trl μm | Yrot μm | Z-rot μm |
| --- | --- | --- | --- | --- | --- | --- |
| 20 Hz |  |  |  |  |  |  |
| P01 | 0.0134- 0.0367i | 0.0742- 0.2342i | 0.2145- 0.1811i | -18.9538+ 25.9197i | 2.2671- 2.1350i | 1.9588- 0.8100i |
| P02 | -0.0447+ 0.0885i | 0.2986-0.0178i | 0.2728+ 0.0942i | 4.7649+ 45.7106i | -9.8487+ 13.4011i | 1.3733+ 0.1297i |
| P03 | -0.0406+ 0.0573i | 0.1592- 0.1055i | 0.0204- 0.0728i | -15.033+ 31.5081i | -23.9983+ 19.9763i | 7.1004+ 0.4010i |
| P04 | 0.0132+ 0.0320i | 0.0378- 0.2115i | 0.0897- 0.1150i | -19.2577+ 22.0507i | -8.7700+ 1.9636i | -1.0194+ 2.2153i |
| P05 | -0.0299-0.0255i | 0.1372-0.0672i | 0.1887-0.0746i | -18.597+ 8.6616i | 9.1662+ 0.9401i | 1.6934- 0.7849i |
| P06 | 0.0713+ 0.0113i | 0.0889-0.0282i | 0.5927-0.1562i | 0.2541+ 30.5269i | -9.5843- 2.6684i | -1.6449- 1.2847i |
| 30 Hz |  |  |  |  |  |  |
| P01 | 0.0267- 0.0094i | 0.0029- 0.0364i | 0.0881- 0.0931i | -3.6687- 5.3737i | -1.6394- 6.1510i | -0.9295+ 1.1940i |
| P02 | 0.0032+ 0.0121i | 0.0533-0.0822i | 0.1453-0.0291i | -30.8422+ 4.5509i | -3.9687-2.8587i | 1.0797-0.1551i |
| P03 | -0.0247+ 0.0255i | 0.0013- 0.0413i | -0.0682- 0.0432i | -17.1282+ 3.2401i | -8.8465+ 11.0725i | -1.1891- 1.3871i |
| P04 | -0.0051+ 0.0055i | -0.0577- 0.0768i | 0.0270- 0.0780i | -20.9375- 8.2116i | -2.5345- 2.9342i | -2.3365+ 1.4631i |
| P05 | 0.0401+ 0.0082i | 0.0365-0.0406i | 0.1045-0.0227i | -19.8304+ 2.2129i | -8.4828+ 0.6414i | 4.923+ 1.5773i |
| P06 | 0.0242-0.0167i | 0.0653-0.1246i | 0.1527-0.0887i | -35.2004+ 5.0073i | 1.3068-7.4439i | 0.5779-1.3256i |
| 50 Hz |  |  |  |  |  |  |
| P01 | -0.0026+ 0.0012i | 0.0212+ 0.0139i | 0.0103+ 0.0006i | 9.0719+ 4.3164i | 1.5626+ 1.4262i | -1.7659+ 0.0999i |
| P02 | -0.0042-0.0017i | 0.0314+ 0.0124i | 0.0272-0.0168i | 10.3137+ 4.8712i | 3.2059+ 2.1828i | -0.1076+ 0.6534i |
| P03 | -0.0028- 0.0009i | 0.0100+ 0.0208i | 0.0375+ 0.0040i | 8.7532+ 0.4425i | 5.2948+ 0.3907i | -1.9793- 0.3832i |
| P04 | -0.0095+ 0.0030i | -0.0075+ 0.0202i | -0.0124+ 0.0092i | 8.7756- 5.2412i | 2.7121+ 2.7820i | -1.4899- 0.2555i |
| P05 | -0.0026-0.0003i | -0.0042+ 0.0024i | -0.0043+ 0.0071i | 2.244-1.2811i | 1.239+ 0.2444i | -1.4033+ 0.2011i |
| P06 | -0.0058-0.0024i | -0.0048+ 0.0010i | -0.0025+ 0.0088i | 8.7931-2.5938i | 3.4773+ 1.6852i | -0.5515+ 1.2045i |

S3 Material Models and Parameters

The viscoelastic behavior of both head models in ABAQUS follows a classical description of relaxation modulus defined by an N-term Prony series, as shown in the equation below (Jung et al., 2016):

| μt=μ∞+i=1Nμie-tτi, | (S1) |
| --- | --- |

where μt is the time-dependent effective relaxation shear modulus, μ∞ is the long-term shear modulus, μi is the modulus contribution for each term, τi is the time constant for each term, and N is the total number of Prony series terms, which is 2 for this study. Equation (S1) can be converted to a frequency-domain representation of viscoelastic properties by using the Laplace transform, resulting in

| μ'ω=μ∞+i=1Nμi∙τi2ω21+τi2ω2 , | (S2.a) |
| --- | --- |
| μ''(ω)=i=1Nμi∙τiω1+τiω . | (S2.b) |

Here, μ'ω and μ''ω are frequency-dependent storage and loss shear moduli, respectively.

We used the LVE material model for Model 1, but we simulated Model 2 using both hyperelastic viscoelastic (HVE) and LVE models in order to test the effect of hyperelasticity on the ABAQUS simulation analysis. The hyperelastic material model that was used for Model 2 is based on Ogden’s strain energy definition in ABAQUS, see Online Resource Reference for ABAQUS documentation,

| U=i=1N2σiαi2λ1αi+λ2αi+λ3αi-3+i=1N1Di(Jel-1)2i , | (S3) |
| --- | --- |

where σi is the i-th contribution of the initial shear modulus to each Ogden term, λ1,2,3αi are deviatoric principal stretches amplified by the Ogden parameter αi, Di determines the compressibility of the material and is inversely proportional to the bulk  modulus, and  Jel is the elastic volume change due to the strain at a given material point.

Since we used a linear perturbation step in ABAQUS for simulations, we expected the response to be linear, which can be verified using the Cv comparison metric (defined in the Methods, Section 2.6.2). We found a Cv value very close to 1, showing an exceptionally high similarity between the HVE and LVE simulations and rendering a choice between the two material models unnecessary for a linear perturbation analysis. Here, in the following tables, we provide material parameters for the two models in this paper.

Table S3  LVE and elastic material parameters in Model 1. E is the long-term elastic modulus and ν is the Poisson’s ratio. σy is the yield stress and εpis the plastic strain. References for properties: Y. Li et al. (2021) and Garcia-Gonzalez et al. (2017)

| Region | Material model | Parameters |
| --- | --- | --- |
| CSF, Ventricles Gray, White | LVE LVE | ρ= 1000 kgm3,E=1000 Pa, ν=0.499999, 50 Hz: μ'=0.3 kPa, μ''=0.628 kPa ρ= 1040 kgm3,E=4000 Pa, ν=0.4999997, 50 Hz: μ'=1.3 kPa, μ''=1.1 kPa |
| Falx Scalp Skull | Elastic Elastic Elastic Plastic | ρ= 1130 kgm3,E=31.5 MPa, ν=0.4999, ρ= 1100 kgm3,E=16.7 MPa, ν=0.42, ρ= 1412 kgm3,E=15 GPa, ν=0.22, σy=92.72 MPa at εp=0, 100 m/m |

Table S4  HVE properties for Model 2. Bulk modulus (K) is 1.46 GPa and density (ρ) is 1040 kg/m3 for HVE regions. G is the long-term shear modulus. HVE properties are based on Alshareef et al. (2021)

| HVE regions | G(Pa) | ∝ | μ1/μ∞ | μ2/μ∞ | τ1(ms) | τ2(ms) |
| --- | --- | --- | --- | --- | --- | --- |
| Deep Gray Cerebrum Gray Cerebellum Gray Corpus Callosum Cerebrum White Cerebellum White Brainstem | 2054.32 1408.37 1779.86 2286.32 1839.64 2371.69 1920.79 | 4.9243 -3.7557 2.7137 -2.3188 -3.4657 2.7137 2.6089 | 0.7515 0.7354 0.6339 0.6848 0.7641 0.6352 0.6216 | 0.0458 0.0475 0.0549 0.0522 0.0443 0.0614 0.0687 | 0.9416 1 1.4 1.2 0.9009 1.4 1.2 | 29.20 29.6 31.5 30.8 28.9 31.5 25 |

Table S5  LVE and elastic material parameters in Model 2. E is the long-term elastic modulus, ν is the Poisson’s ratio. These properties are based on Alshareef et al. (2021) except for the ventricles’ properties that depend on a modified ν based on Model 1

Region Model Parameters

SAS LVE ρ= 1133 kgm3,E=300 Pa, ν=0.49999, μ1 = 0.8,τ1= 12.5 ms Ventricles LVE ρ= 1000 kgm3,E=1260 Pa, ν=0.4999999, μ1=0.8,τ1=12.5 ms Falx/Tentorium Elastic ρ=1004kgm3, E=0.0315 GPa,ν=0.45 Foramen Elastic ρ=2070kgm3, E=0.0315 GPa, ν=0.45 Skull Elastic ρ=2070kgm3, E=3.28 GPa, ν=0.3

S4  Model 1 Registration

Fig. S2  Reconstruction and registration of the pseudo-anatomical (PA) image of Model 1 to the MNI-152 atlas. (a) Finite element Model 1 with tetrahedral elements in ABAQUS. (b) Centroids of Model 1 elements interpolated on a three-dimensional grid 240×240×180 with 1 mm voxel resolution using a custom code in MATLAB. (c) Reconstructed PA image in the first column is rigidly registered to the MNI-152 atlas image in the last column and the result is shown in the second column. ANTs affine and SyN registration is applied to the rigidly registered image in the second column to generate a fully registered model image in the third column. MNI-152 atlas image has a dimensionality of 241×286×241 with 0.8 mm voxel resolution. Scale bars (white, green, and red) represent 10 cm respectively in axial, coronal, and sagittal views in MNI-152 atlas space

S5  Supplementary Results

Here, we present all components of strain tensor as complementary results for Fig. 4.

Fig. S3  In-plane strain patterns from harmonic motion in human MRE experiment (participant P01) and from simulations of two head models. Scale bars (white, green, and red) represent 10 cm respectively in axial, coronal, and sagittal views

Fig. S4  Out-of-plane strain patterns from harmonic motion in human MRE experiment (participant P01) and from simulations of two head models. Scale bars (white, green, and red) represent 10 cm respectively in axial, coronal, and sagittal views

Fig. S5 shows the effect of changing the kernel size from 17×17×17 (the selected size in the paper) to other sizes.

Fig. S5  Local correlation (LC) results for comparison of simulated strain fields from Model 2 to strain fields measured in the MRE experiment (participant P01). Calculated LC for kernel dimensions of (a) 7, (b) 17, (c) 25, and (d) 35 voxels in each three spatial directions with 0.8 mm voxel resolution for 20 Hz harmonic motion. As the color bar indicates, the similarities of LC range from low to high on a scale of 0 to 1, with 1 being the highest similarity possible. The scale bars in white, green, and red represent 10 cm respectively in axial, coronal, and sagittal views

Fig. S6 shows that SM cumulative distributions do not change substantially before and after registration.

Fig. S6 Strain magnitude (SM) cumulative distribution based on a normal distribution fit for Model 1 data with loading from MRE experiment with participant P01. Registration only minimally affects the cumulative distribution of SM.  Cumulative distributions of a standard normal distribution are created and shown for 20 Hz, 30 Hz, and 50 Hz frequencies of harmonic motion for fully registered (“reg”) and original (“org”) strain fields using MATLAB

Online Resource Reference

ABAQUS/Standard User's Manual, v. 6.6 (Dassault Systemes Simulia Corp.).
https://classes.engineering.wustl.edu/2009/spring/mase5513/abaqus/docs/v6.6/index.html. Accessed 11 July 2024