Supplementary Information

Single-Molecule Sensing inside a Stereo- and Regiodefined Hetero-Nanopore

Wei Liu,[a]# Qiang Zhu,[a]# Chao-Nan Yang,[a] Zhong-Lin Yang,[a] Kaili Xin, Jing Ma,[a] Mathias Winterhalter[b], Yi-Lun Ying*[a][c], Yi-Tao Long[a]

[a]	W. Liu, Q. Zhu, C.-N. Yang, Z.-L. Yang, K.-L. Xin, J. Ma, Y.-L. Ying and Y. -T Long
School of Chemistry and Chemical Engineering
Nanjing University 
163 Xianlin Road, 210023 Nanjing, (P. R. China)
E-mail: yilunlun@nju.edu.cn;

[b]	M. Winterhalter

Constructor University, Campus Ring 6, D- 28759 Bremen, Germany

[c]	Y.-L Ying
Chemistry and Biomedicine Innovation Center, Nanjing University
163 Xianlin Road, 210023 Nanjing, (P. R. China)

Supplementary Methods

1.1 Materials and Reagents

Decane (anhydrous, ≥99%), KCl, ethylenediaminetetraacetic acid (EDTA), Tris(Hydroxymethyl)-aminomethane, trypsin-EDTA were purchased from Sigma-Aldrich Co. (St. Louis, MO, USA). 1,2-Diphytanoyl-sn-glycero-3-phosphocholine was obtained from Avanti Polar Lipids, Inc. (Alabaster, AL, USA). Peptides of P-L-Leu, P-D-Leu, P-L-Ile, and P-D-Ile were synthesized and purified by Sangon Biotech Co., Ltd. (Shanghai, China). NAM was purchased from Alfa Aesar Co. (Heysham, England). K238C AeL, K238G AeL, and WT AeL were purified and activated according to our previous study1. The solutions in our experiment were prepared with ultrapure water (18.2 MΩ·cm at 25 °C) from an Arium Pro (Sartorius, USA).

1.2 Nanopore measurements

The proaerolysin was treated with trypsin-EDTA to obtain the activated monomers to self-assemble into heptamer. The detailed experimental procedures for recording the ionic current of aerolysin nanopores are described in our previous study2. Briefly, the DPhPC solution was spread across a 50 μm orifice in the Delrin bilayer cup (Warner Instruments, Hamden, CT, USA). The aerolysin protein was added to the grounded cis chamber. Voltages are applied through a pair of Ag/AgCl electrodes that were immersed in the electrolyte solutions of each chamber. The data were collected by the self-developed nanopore instrument “Cube-D2”3. All data were acquired with a low-pass filter at 5 kHz and a sampling frequency of 100 kHz. Unless otherwise indicated, the reagent molecules were added into the trans chamber with a final concentration of 10 μM. The oligonucleotides/peptide was added into the cis chamber with a final concentration of 10 μM. The nanopore experiment was conducted in 3.0 M KCl and10.0 mM Tris solution at pH=7.0.

1.3 Computational Details

The wild-type aerolysin (AeL) pore structure embedded in the 1,2-dimyristoyl-sn-glycero-3-phosphocholine (DMPC) lipid bilayer of our previous work4 was applied here as the initial ones, where amino acids at the sensing regions (K238) were all replaced with Cysteine (CYS). For the purpose of this work, amino acids C238 were further mutated into N-(4-acetylpheny)-maleimide (NAM) with different numbers and positions. Here, as NAM-AeL pore is a mature heptameric one, we constructed three models. The first one is for a single mutation, the second is for double mutations with different positions where the interval number ranges from 0 to 2, and the third is for triple mutations whose interval number between adjacent mutations is 1. These constructed models were then solvated with the TIP3P water model,5 and K+ and Cl– were subsequently added so as to neutralize the whole system and set the concentration to be 3:0 M with the help of VMD.6 All molecular dynamics (MD) simulations were performed under the NAMD package suite,7 with the CHARMM36 force field.8 For the mutation NAM, the CHARMM General Force Field (CGenFF)9 parameters were applied here, and the connection between the cysteine residues (atom S) and NAM (atom C) was treated as the one between the aliphatic C for -CH2 and Sulphur. Firstly, a 1000-steps minimization using a sophisticated conjugated gradient and line search algorithm was applied to the systems assembled above.

Then, the aerolysin nanopore systems were equilibrated in the NPT ensemble for 2 ns with the C atoms of protein and atom P of lipid restrained. Next, all restraints were removed, and the equilibration simulation lasted 10 ns. Subsequently, 70 ns NPT ensemble simulations were performed with a biased voltage of 50 mV applied along the z-dimension. Throughout the whole simulation, the temperature was maintained at 295 K using the Langevin dynamic while the pressure was kept at 1 atm by the Nose-Hoover-Langevin piston10. The electrostatic calculations were handled by the particle-mesh Ewald (PME)11 with a cutoff of 12 Å. The integration time step was set to be 2 fs with the help of the SHAKE algorithm.12

1.4 Data Analysis

Data were analyzed with home-designed PyNano and OriginLab 8.0 (OriginLab Corporation, Northampton, MA, USA) (www.pynano.lab)13.

Supplementary Results and Notes

Figure S1 I-V Curve for K238C AeL in 3.0 M KCl, 10.0 mM Tris solution (pH=7.0) from - 100 mV to + 100 mV. The error bars are from 7 independent experiments.

Figure S2. Ionic current trace for the stepwise reactions between seven cysteine residues K238C AeL and seven NAM molecules. The 10 μM NAM was added from the cis side. The experiment was conducted in 3.0 M KCl and10.0 mM Tris solution at pH=7.0.

Figure S3 Control experiments of WT AeL (a) and K238G AeL(b). The 10 μM NAM was introduced into the trans chamber. The experiments were conducted in 3.0 M KCl and10.0 mM Tris solution at pH=7.0.

Figure S4 (a) In-situ stepwise generation of 1,3,5-3NAM-AeL at - 50 mV. The 10.0 μM NAM molecule was introduced into the trans side (bottom opening of the pore). (b) The sensing of single peptides with 1,3,5-3NAM-AeL at + 80 mV. After generating three successive current drops, the applied potential was changed to + 80 mV. This action prohibits the addition of the fourth NAM molecule while driving the negatively charged peptides to translocate from the cis (mouth opening of the pore) to the trans side. Then, the peptide mixture of N’-DDFFXFFDD (X is l-Leu, red circle, and l-Ile, blue circle) was added into the cis side for driving into the nanopore. The deep blockages (red star) and low blockages (blue star) are assigned to the l-Leu and l-Ile mutated peptides. The experiment was conducted in 3.0 M KCl and10.0 mM Tris solution at pH=7.0.

Figure S5 The illustration of Z’ distance along with the Z axis between the centroid of benzene and Cα of the bonded cysteine.

Figure S6 All-atom molecule dynamics simulation of 1NAM-AeL. (a) The cross-section representation of 1NAM-AeL at 238 sites (Z=0). (b) Corresponding projection of 1NAM-AeL at XY plane. (c) The distribution of Z’ inside the 1NAM-AeL.

Figure S7 The radiuses of K238C AeL (black), 1,4-2NAM-AeL (red), 1,3-2NAM-AeL (blue) and 1,2-2NAM-AeL (green).

Figure S8 (a) The number of confined ions inside K238C AeL (black), 1,4-2NAM-AeL (red), 1,3-2NAM-AeL (blue), and 1,2-2NAM-AeL (green), respectively. (b) The total ions number in the range of - 10 Å to + 20 Å.

Figure S9 In situ stepwise generation of NEM-AeL nanopore. (a) The typical reaction current trace for stepwise NEM addition in a pathway of K238C AeL → 7NEM-AeL. (b) The current histograms of I0, △I1/I0, △I2/I0, and △I3/I0 (from left to right) for the product of 1NEM-AeL, 2NEM-AeL, and 3NEM-AeL, respectively. The results were recorded from 10 independent productions. The 10.0 μM NEM was introduced into the nanopore’s mouth opening (cis side). The experiment was conducted in 3.0 M KCl and10.0 mM Tris solution at pH=7.0. Wherein, I0, I1, I2, and I3 represent the currents value of K238C AeL, 1NEM-AeL, 2NEM-AeL, and 3NEM-AeL, respectively. △I1, △I2, and △I3 ascribe for the amplitude of current drop in the reaction trajectories of K238C AeL→1NEM-AeL, 1NEM-AeL→2NAM-AeL, and 2NEM-AeL→3NEM-AeL, respectively. (b) The histogram of △I1/I0 (Left), △I2/I0 (Middle), and △I3/I0 (Right).

Supplementary Note 1 Theoretical yield calculation of 1,3,5-3NAM-AeL

Single-molecule approaches to chemical reaction analysis can provide information on the reaction trajectory14. Changes in the position of NAM tethered to the inner wall of K238C AeL affect the ionic current through the nanopore. According to Figures 1a and 1c, the experimental results show that the probability of the second NAM reacting on para positions reaches 85%. Because of the steric hindrance from the first NAM, the additional probability of the second NAM to the ortho position is only 15% due to the steric hindrance of NAM and the confinement of the 238 site. While the non-adjacent sites exhibited 85% modified preference. Moreover, the adjacent modification arouses a deeper current blockage due to the confined conformation of the phenyl groups. This result could assist us to remove the modification of the adjacent site in both 2NAM-AeL and 3NAM-AeL.

Based on these results, we assume the probability of adding the third NAM to para positions and ortho position is 85% and 15%, respectively (Figure S10a). The reaction network analysis illustrates that both 1,3-2NAM-AeL and 1,4-2NAM-AeL can produce 1,3,5-3NAM-AeL, where three NAM molecules occupy three non-adjacent positions. Therefore the theoretical probability (p1,3,5) for the production of 1,3,5-3NAM-AeL is calculated by equation (1):

p1,3,5=1×0.85×p1,3+2×0.85×p1,4                     (1)

Similarly, the production of 1,3,5-3NAM-AeL (p1,4,5), 1,4,5-3NAM-AeL (p1,3,4), 1,3,4-3NAM-AeL (p1,3,4), and 1,2,3-3NAM-AeL (p1,2,3) is calculated by equation (2)-(4):

p1,4,5=1×0.85×p1,2+2×0.15×p1,4                     (2)

p1,3,4=2×0.85×p1,2+2×0.15×p1,3+ 2×0.15×p1,4         (3)

p1,2,3=1×0.15×p1,2+1×0.15×p1,3                     (4)

Where p1,2, p1,3, and p1,4 represent the experimental yields of 1,2-2NAM-AeL, 1,3-2NAM-AeL, and 1,4-2NAM-AeL, respectively. Assuming that the p1,3=p1,4, the calculated normalized probability of those four isomers of 3NAM-AeL is 0.55, 0.13, 0.26, and 0.06 for 1,3,5-3NAM-AeL, 1,4,5-3NAM-AeL, 1,3,4-3NAM-AeL, and 1,2,3-3NAM-AeL, respectively. The 1,3,5-3NAM-AeL, the only structure in which the three phenyl groups occupy non-adjacent locations, owns the highest yields among the isomers.

Figure S10 Confirming the production of 1,3,5-3NAM-AeL. a) Analysis of the reaction network of 2NAM-AeL → 3NAM-AeL and corresponding theory probabilities (85% for the NAM bond with the non-adjacent position of the existing NAM and 15% for that in the adjacent position, data comes from Figure 1a in manuscript). b) The histogram of △I3/I0 when the 1,3-2NAM-AeL or 1,4-2NAM-AeL bond with the third NAM molecule. c) The histogram of △I3/I0 when the 1,2-2NAM-AeL bond with the third NAM molecule. d) The histogram of (I0-I3)/I0 for the 3NAM-AeL transferred from 1,3-2NAM-AeL or 1,4-2NAM-AeL. e) The histogram of (I0-I3)/I0 for the 3NAM-AeL transferred from 1,2-2NAM-AeL.

Supplementary Note 2 Confirming the production of 1,3,5-3NAM-AeL

The resulting site-selectivity of 3NAM-AeL depends on the structure of 2NAM-AeL. To accurately calculated the experimental yield calculation of 1,3,5-3NAM-AeL, we classified each drop△I3/I0 (3NAM-AeL) in Figure 1c according to the preceding drops of △I2/I0 (2NAM-AeL). After classifying with kernel density evaluation, the production of 3NAM-AeL via pathways 1 and 2 generates two current distributions (Figure S10b), while that via pathway 3 gives the dispersed current distribution (Figure S10c). To eliminate the current difference in the preceding drops (2NAM-AeL), we used the (I0-I3)/I0 for the current classification. The corresponding statistical results are shown in Figure S10d-e. For pathway 3, all the values of (I0-I3)/I0 locate at the position larger than 15%, which originates from the production of 1,4-5NAM-AeL, 1,3-4NAM-AeL, and 1,2-3NAM-AeL. For pathways 1 and 2, the values of (I0-I3)/I0 are classified into three populations. Note that two small populations are located at a position larger than 15%. These two populations could be assigned to the production of 1,4-5NAM-AeL, 1,3-4NAM-AeL, and 1,2-3NAM-AeL. Accordingly, the largest population of (I0-I3)/I0 attributes to the production of 1,3,5-3NAM-AeL. Therefore, the yield of 1,3,5-3NAM-AeL is ~66%.

Table S1 The stepwise current drops from K28C AeL to 3NAM-AeL

| Experiment No. | △I1/I0 / % | △I2/I0 / % | (I0-I2)/I0 / % | △I3/I0 / % | (I0-I3)/I0 / % |
| --- | --- | --- | --- | --- | --- |
| 1 | 2.4 | 3.2 | 5.6 | 5.4 | 11.0 |
| 2 | 3.2 | 3.3 | 6.5 | 6.5 | 13.0 |
| 3 | 3.7 | 3.4 | 7.2 | 5.4 | 12.5 |
| 4 | 3.6 | 3.5 | 7.1 | 5.8 | 12.9 |
| 5 | 3.6 | 3.5 | 7.1 | 6.4 | 13.4 |
| 6 | 3.4 | 3.6 | 7.0 | 5.3 | 12.4 |
| 7 | 2.7 | 3.7 | 6.4 | 2.6 | 9.0 |
| 8 | 2.8 | 3.7 | 6.5 | 5.1 | 11.6 |
| 9 | 4.0 | 3.9 | 7.9 | 6.7 | 14.6 |
| 10 | 3.0 | 4.1 | 7.1 | 5.8 | 12.9 |
| 11 | 3.6 | 4.2 | 7.7 | 5.6 | 13.3 |
| 12 | 3.0 | 4.2 | 7.1 | 5.3 | 12.5 |
| 13 | 3.1 | 4.2 | 7.3 | 6.7 | 13.9 |
| 14 | 3.5 | 3.9 | 7.4 | 6.4 | 13.8 |
| 15 | 3.3 | 4.2 | 7.5 | 5.0 | 12.6 |
| 16 | 4.0 | 4.4 | 8.4 | 5.6 | 14.0 |
| 17 | 3.5 | 4.3 | 7.8 | 5.7 | 13.5 |
| 18 | 3.5 | 4.3 | 7.8 | 6.4 | 14.2 |
| 19 | 3.6 | 4.4 | 7.9 | 5.5 | 13.4 |
| 20 | 5.0 | 4.4 | 9.4 | 4.7 | 14.1 |
| 21 | 3.2 | 4.5 | 7.7 | 6.6 | 14.3 |
| 22 | 3.7 | 4.5 | 8.2 | 5.3 | 13.5 |
| 23 | 3.4 | 4.7 | 8.0 | 6.2 | 14.2 |
| 24 | 3.7 | 4.7 | 8.4 | 7.2 | 15.6 |
| 25 | 4.3 | 4.8 | 9.1 | 14.9 | 23.9 |
| 26 | 3.8 | 4.9 | 8.7 | 16.4 | 25.1 |
| 27 | 3.8 | 5.4 | 9.2 | 7.2 | 16.4 |
| 28 | 4.0 | 5.5 | 9.5 | 6.7 | 16.2 |
| 29 | 3.8 | 5.6 | 9.4 | 6.7 | 16.1 |
| 30 | 4.5 | 6.1 | 10.7 | 9.3 | 20.0 |
| 31 | 4.6 | 6.5 | 11.1 | 9.0 | 20.1 |
| 32 | 5.1 | 6.8 | 11.9 | 10.5 | 22.4 |
| 33 | 3.3 | 7.6 | 10.9 | 12.7 | 23.6 |
| 34 | 3.9 | 8.2 | 12.1 | 9.7 | 21.8 |

Figure S11 I-V Curve of K238C AeL (black) and 1,3,5-3NAM-AeL (red) in 3.0 M KCl, 10.0 mM Tris solution at pH=7.0. The errors are from 7 independent experiments.

Figure S12 (a) The raw current trace of 1,3,5-3NAM-AeL under the applied potential of – 50 mV (blue) and +80 mV (red). (b) Scatter plots of the background signals from a 100 s current trace at +50 mV. The count of the background signals at + 80 mV is six times lower than that at -50 mV. The background signals barely interfere with the single-peptide/oligonucleotide events which locates at I/I0 ~ 0.4-1.0 with duration time (tD) > 3.0 ms.

Figure S13 The illustration of current and duration parameters used for single-molecule sensing with hetero-nanopore. The representing raw current trace is from 2NAM-AeL (red) and 1,3,5-3NAM-AeL (blue) sensing of poly(dA4. Wherein, the In represents the current level after the K238C AeL bonding with n NAM/NEM molecules (n=0, 1, or 2). The poly(dA)4 produces the current blockage of △In, A4 with a duration time of tD.

Table S2 The NAM-AeLs sensing of poly(dA)4‡

| K238C-AeL |  | 1NAM-AeL |  |  | 2NAM-AeL |  |  | 3NAM-AeL† |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| △I0, A4/I0 | tD / ms | (I0-I1)/I0 | △I1, A4/I0 | tD / ms | (I0-I2)/I0 | △I2, A4/I0 | tD / ms | (I0-I3)/I0 | △I3, A4/I0 | tD / ms |
| 0.43 | 5.76 | 2.7 % | 0.39 | 5.89 | 6.4 % | 0.38 | 8.53 | 9.0 % | 0.39 | 10.98 |
| 0.42 | 6.61 | 2.4 % | 0.39 | 6.92 | 5.6 % | 0.45 | 5.60 | 11.0 % | 0.46 | 11.48 |
| 0.52 | 5.51 | 3.6 % | 0.48 | 9.57 | 7.0 % | 0.49 | 7.10 | 12.2 % | 0.53 | 10.50 |
| 0.48 | 9.24 | 4.0 % | 0.45 | 5.81 | 9.5 % | 0.45 | 10.21 | 15.0 % | 0.44 | 12.18 |
| 0.45 | 7.42 | 4.3 % | 0.41 | 7.25 | 14.1 % | 0.41 | 3.87 | 23.8 % | 0.25 | 2.58 |
| 0.44 | 6.61 | 3.8 % | 0.44 | 6.92 | 8.7 % | 0.48 | 10.50 |  |  |  |
| 0.45 | 3.76 | 5.0 % | 0.42 | 4.03 | 9.4 % | 0.39 | 3.66 |  |  |  |

‡ The 10 μM poly(dA)4 was added from the cis side. The experiment was performed in 3.0 M KCl and10.0 mM Tris solution (pH=7.0) at 50 mV. The experiments are performed at 18 ºC.

† The blue box highlighted the results from 1,3,5-3NAM-AeL sensing of poly(dA)4

Table S3 The NEM-AeLs sensing of poly(dA)4‡

| K238C-AeL |  | 1NEM-AeL |  |  | 2NEM-AeL |  |  | 3NEM-AeL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| △I0, A4/I0 | tD / ms | (I0-I1)/I0 | △I1, A4/I0 | tD / ms | (I0-I2)/I0 | △I2, A4/I0 | tD / ms | (I0-I3)/I0 | △I3, A4/I0 | tD / ms |
| 0.48 | 10.72 | 2.2 % | 0.48 | 8.93 | 5.6 % | 0.46 | 9.16 | 8.7 % | 0.58 | 7.25 |
| 0.57 | 7.64 | 2.2 % | 0.54 | 8.95 | 5.1 % | 0.53 | 5.12 | 9.1 % | 0.64 | 5.03 |
| 0.46 | 6.79 | 2.1 % | 0.45 | 6.32 | 4.7 % | 0.45 | 4.46 | 7.1 % | 0.51 | 4.79 |
| 0.48 | 7.78 | 2.4 % | 0.49 | 7.43 | 5.4 % | 0.48 | 7.26 | 9.1 % | 0.57 | 5.89 |
| 0.56 | 9.16 | 2.8 % | 0.60 | 5.15 | 5.7 % | 0.60 | 5.02 | 9.0 % | 0.74 | 2.77 |
| 0.60 | 11.01 | 2.1 % | 0.64 | 8.00 | 4.8 % | 0.64 | 6.93 | 8.2 % | 0.37 | 3.17 |
| 0.52 | 8.34 | 2.4 % | 0.49 | 9.16 | 4.1 % | 0.48 | 9.36 |  |  |  |

‡ The 10 μM poly(dA)4 was added from the cis side. The experiment was performed in 3.0 M KCl and10.0 mM Tris solution (pH=7.0) at 50 mV. The experiments are performed at 25 ºC.

†The blue box highlighted the results from 1,3,5-3NEM-AeL sensing of poly(dA)4.

Figure S14 The raw data of the multiple-channel nanopore experiment on the Orbit Mini to discriminate the peptide stereoisomers by 1,3,5-3NAM-AeL. The experiments are conducted in 3.0 M KCl, 10.0 mM Tris (pH=7.0) at + 80 mV. The peptide sequence is N’-DDFFXFFDD, in which X is L-Leu, D-Leu, L-Ile, and D-Ile, respectively. The final concentration of each peptide in the cis chamber is 10 μM. This data is recorded by the Orbit Mini (Nanion, German).

Figure S15 Identifying peptide stereoisomers by unmodified K238C AeL. (a) The scatter plots of P-L-Leu (purple) and P-L-Ile (green). (b) The scatter plots of P-L-Leu (purple) and P-D-Leu (pink). c) The scatter plots of P-L-Ile (green) and P-D-Ile (orange). The 10 μM peptide mixtures were driven from the cis side under + 80 mV, respectively. The experiments were conducted in 3.0 M KCl, and 10.0 Mm Tris-HCl solution at pH 7.0.

Supplementary Note 3 Quantifying the peptide stereo-isomers

The capture frequency (f) is calculated by the number of events when 10.0 μM of individual P-L-Leu, P-D-Leu, P-L-Ile, or P-D-Ile was introduced into the cis side at + 80 mV within 10 minutes. The f could be calculated by equation 6:

fX=nX600 										(6)

Wherein, n is the number of events within 10 minutes. X refers to the P-L-Leu, P-D-Leu, P-L-Ile, or P-D-Ile.

For the peptide stereoisomers mixtures, each peptide was counted after identifying through the I/I0 and std. The number of peptide events in the mixture was corrected by fX. Then the peptide abundance in the mixture is represented by the percentage of the events after correction.

References

1	Liu, W., Yang, Z.L., Yang, C.N., Ying, Y.L. & Long, Y.T. Profiling single-molecule reaction kinetics under nanopore confinement. Chem. Sci. 13, 4109-4114 (2022).

2	Cao, C., Liao, D.F., Yu, J., Tian, H. & Long, Y.T. Construction of an aerolysin nanopore in a lipid bilayer for single-oligonucleotide analysis. Nat. Protoc. 12, 1901-1911 (2017).

3	Zhang, L.-L. et al. A two-step calibration method for evaluation high bandwidth electrochemical instrument. J. Electroanal. Chem. 915 (2022).

4	Li, M.-Y. et al. Revisiting the origin of nanopore current blockage for volume difference sensing at the atomic level. JACS Au 1, 967-976 (2021).

5	Jorgensen, W. L., Chandrasekhar, J., Madura, J. D., Impey, R. W. & Klein, M. L. Comparison of simple potential functions for simulating liquid water. J. Chem. Phy. 79, 926–935 (1983).

6	Humphrey, W., Dalke, A. & Schulten, K. VMD: visual molecular dynamics. J. Mol. Graph. 14, 33–38 (1996).

7	Phillips, J. C. et al. Scalable molecular dynamics with NAMD. J. Comput. Chem. 26, 1781–1802 (2005).

8	Best, R. B. et al. Optimization of the additive CHARMM all-atom protein force field targeting improved sampling of the backbone Φ, ψ and side-chain χ1 and χ2 dihedral angles. J. Chem. Theory. Comput. 8, 3257–3273 (2012).

9	Vanommeslaeghe, K. et al. CHARMM general force field: A force field for drug-like molecules compatible with the CHARMM all-atom additive biological force fields. J. Comput. Chem. 31, 671–690 (2010).

10	Feller, S. E., Zhang, Y., Pastor, R. W., & Brooks, B. R. Constant pressure molecular dynamics simulation: the Langevin piston method. J. Chem. Phy. 103, 4613–4621 (1995).

11	Darden, T., York, D., & Pedersen, L. The effect of long-range electrostatic interactions in simulations of macromolecular crystals–a comparison of the ewald and truncated list methods. J. Chem. Phys. 99, 10089 (1993).

12	Allen, M. P. & Tildesley, D. J. Computer simulation of liquids; Oxford university press, (2017).

13	Liu, S.C. et al. An advanced optical-electrochemical nanopore measurement system for single-molecule analysis. Rev. Sci. Instrum. 92, 121301 (2021).

14	Steffensen, M.B., Rotem, D. & Bayley, H. Single-molecule analysis of chirality in a multicomponent reaction network. Nat. Chem. 6, 603-607 (2014).

15	Xi, Z. Lu, D. Dai, J. Ye T. & Zhang, W. Determination of enantiomeric purity of binaphthol by NMR method. Univ. Chem. 2, 2109027 (2022).