Supplement of

Probabilistic Seismic Hazard Assessment of Sumatra, Indonesia

Rizki Wulandari1,2,3, Chung Han Chan1,2,3*, Jia-Cian Gao2,4, Danny H. Natawidjaja5

1 Department of Earth Sciences, National Central University, Taoyuan City, Taiwan

2 Earthquake-Disaster & Risk Evaluation and Management Center, National Central University, Taoyuan City, Taiwan

3Departement of Geophysical Engineering, Institute of Technology Sumatra, Lampung, Indonesia

4Cian-Yu Earthquake Disaster & Risk Evaluation and Management Consultants Ltd., Taoyuan City, Taiwan

5National Research and Innovation Agency (BRIN), Bandung, Indonesia

53

52 Mentawai

Figure S1. Rupture probability of Sumatran’s 53 seismogenic fault segments in the 50 years. The Poisson model was used to evaluate the black color segments, while the Brownian Passage Time (BPT) model was used to estimate the blue color segments.

Table S1. A summary of historical earthquakes along the Sumatran fault, the top and bottom are 3 and 20 km, respectively.

| ID | Structure Name |  | Slip-Rate (mm/yr) | Sense Mechanism | Dip | L (km) | M max | Historical Earthquake | Geomorphic Features | BPT in 50 years % | PoE in 50 years |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main | Segment |  |  |  |  |  |  |  |  |  |
| 1 | Andaman Spreading Ridge | North 1 | 0.75 | Normal 60S | 60 | 166 | 6.8 |  |  |  | 25.41% |
| 2 | Andaman Spreading Ridge | North 2 | 0.75 | Normal 60S | 60 | 197 | 6.9 |  |  |  | 22.39% |
| 3 | Andaman Spreading Ridge | South 1 | 0.75 | Normal 60N | 60 | 170 | 6.8 |  |  |  | 25.41% |
| 4 | Andaman Spreading Ridge | South 2 | 0.75 | Normal 60N | 60 | 212 | 7 |  |  |  | 19.69% |
| 5 | West Andaman Fault | North-West | 5 | Strike-Slip | 90 | 478 | 7.9 |  |  |  | 3.97% |
| 6 | West Andaman Fault | North-East | 5 | Strike-Slip | 90 | 669 | 8 |  |  |  | 3.24% |
| 7 | West Andaman Fault | Central | 10 | Strike-Slip | 90 | 168 | 7.3 |  |  |  | 24.51% |
| 8 | West Andaman Fault | South (Simelue) | 10 | Strike-Slip | 90 | 196 | 7.8 |  |  |  | 9.49% |
| 9 | West Andaman Backthrust | North | 10 | Reverse-Slip | 45W | 547 | 7.9 |  |  |  | 47.33% |
| 10 | West Andaman Backthrust | South | 10 | Reverse-Slip | 45W | 456 | 8.1 |  |  |  | 46.10% |
| 11 | Sumatran Fault | Oreng Fault | 1 | Strike-Slip | 90 | 36 | 6.9 |  |  |  | 6.24% |
| 12 | Sumatran Fault | Loksumawe | 1 | Strike-Slip | 90 | 36 | 6.5 |  |  |  | 13.72% |
| 13 | Sumatran Fault | Nicobar | 18 | Strike-Slip | 90 | 120 | 7.1 |  |  |  | 53.51% |
| 14 | Sumatran Fault | Seulimeum-North | 18 | Strike-Slip | 90 | 143 | 7.6 | 1964 (Ms=6.5) | Small depression on dilatational stepover | 18.61 | 23.80% |
| 15 | Sumatran Fault | Seulimeum-South | 7 | Strike-Slip | 90 | 38 | 6.9 |  |  | 51.02 | 36.29% |
| 16 | Sumatran Fault | Aceh-North | 2 | Strike-Slip | 90 | 65 | 7.2 | No record | Mountanious range, associated with thrusts |  | 6.68% |
| 17 | Sumatran Fault | Aceh-Central | 14 | Strike-Slip | 90 | 140 | 7.6 |  |  |  | 19.05% |
| 18 | Sumatran Fault | Aceh-South | 14 | Strike-Slip | 90 | 27 | 6.7 |  |  |  | 74.46% |
| 19 | Takengon Fault | Lok Tawar | 0.5 | Strike-Slip | 90 | 21 | 6.6 |  |  |  | 5.82% |
| 20 | Takengon Fault | Peusangan | 0.5 | Strike-Slip | 90 | 32 | 6.8 |  |  |  | 3.88% |
| 21 | Sumatran Fault | Tripa1-Thrust | 16 | Strike-Slip | 45W | 32 | 6.8 |  |  |  | 71.86% |
| 22 | Sumatran Fault | Tripa 2 | 7 | Strike-Slip | 90 | 103 | 7.4 | 1936 (Ms=7.2);1990 (Ms=6?) | Alas Valley | 6.82 | 14.78% |
| 23 | Sumatran Fault | Tripa 3 | 14 | Strike-Slip | 90 | 75 | 7.3 |  |  | 50.94 | 32.54% |
| 24 | Sumatran Fault | Tripa 4 | 7 | Strike-Slip | 90 | 55 | 7.1 |  |  | 34.53 | 25.76% |
| 25 | Sumatran Fault | Tripa 5 | 7 | Strike-Slip | 90 | 21 | 6.6 |  |  | 84.96 | 56.81% |
| 26 | Batee Fault | Batee-A | 6 | Strike-Slip | 90 | 44 | 7 |  |  |  | 26.96% |
| 27 | Batee Fault | Batee-B | 0.5 | Strike-Slip | 90 | 50 | 7 |  |  |  | 2.58% |
| 28 | Batee Fault | Batee-C | 0.1 | Strike-Slip | 90 | 37 | 6.9 |  |  |  | 0.64% |
| 29 | Sumatran Fault | Renun-A | 10.5 | Strike-Slip | 90 | 180 | 7.7 | 1916;1921 (Mb = 6.8); 1936 (Ms=7.2) | Tarutung Valley | 2.74 | 12.09% |
| 30 | Sumatran Fault | Renun-B | 8 | Strike-Slip | 90 | 31 | 6.8 |  |  | 74.76 | 46.95% |
| 31 | Sumatran Fault | Renun-C | 8 | Strike-Slip | 90 | 26 | 6.7 |  |  | 33.5 | 25.36% |
| 32 | Sumatran Fault | Toru | 11.5 | Strike-Slip | 90 | 95 | 7.4 | 1987 (Ms = 6.6) | Uplifted hill on the east side of the bend | 7.38 | 23.11% |
| 33 | Sumatran Fault | Angkola | 6 | Strike-Slip | 90 | 160 | 7.7 | 1892 (Ms=7.7) | Mountainous range on east side of the fault | 18.18 | 15.79% |
| 34 | Sumatran Fault | Barumun | 6.5 | Strike-Slip | 90 | 125 | 7.5 | No record | long (Sumpur) valley along the fault |  | 11.37% |
| 35 | Sumatran Fault | Sumpur | 14 | Strike-Slip | 90 | 35 | 6.9 | No record | Wide depression association with normal faults |  | 59.42% |
| 36 | Sumatran Fault | Sianok | 14 | Strike-Slip | 90 | 90 | 7.4 | 1926 (Ms ~ 7) | Lake Singkarak | 41.38 | 27.38% |
| 37 | Sumatran Fault | Sumani | 14 | Strike-Slip | 90 | 60 | 7.1 | 1943 (Ms=7.6); 1942 (Ms ~ 7) | Lake Diatas, calderas, and Talang volcano | 71.5 | 44.89% |
| 38 | Sumatran Fault | Suliti | 14 | Strike-Slip | 90 | 95 | 7.4 | 1943 (Ms=7.4) | Small depression caldera and young volcanic cone | 36.78 | 27.38% |
| 39 | Sumatran Fault | Siulak | 14 | Strike-Slip | 90 | 70 | 7.2 | 1909 (Ms=7.6); 1995(Mw=7.0) | Lake Kerinci and Kunyit Volcano | 64.95 | 38.39% |
| 40 | Sumatran Fault | Dikit | 12 | Strike-Slip | 90 | 60 | 7.1 | No record |  |  | 39.99% |
| 41 | Sumatran Fault | Ketaun | 12 | Strike-Slip | 90 | 85 | 7.3 | 1943 (Ms=7.3);1952 (Ms=6.8) | Depression valley and Kaba volcano | 36.88 | 28.64% |
| 42 | Sumatran Fault | Musi | 13.5 | Strike-Slip | 90 | 70 | 7.2 | 1979 (Ms=6.6) | Valley, depression | 45.93 | 37.31% |
| 43 | Sumatran Fault | Manna | 13.5 | Strike-Slip | 90 | 85 | 7.3 | 1893 | Mountainous range on east side of the fault | 54.64 | 31.59% |
| 44 | Sumatran Fault | Kumering-North | 12.5 | Strike-Slip | 90 | 111 | 7.5 | 1933 (Ms=7.5) | Suoh geo thermal valley | 21.9 | 20.72% |
| 45 | Sumatran Fault | Kumering-South | 12.5 | Strike-Slip | 90 | 60 | 7.1 |  |  | 67.29 | 41.25% |
| 46 | Sumatran Fault | Semangko Barat A | 8 | Strike-Slip | 90 | 90 | 7.4 | 1908 | East facing scrap | 17.45 | 16.71% |
| 47 | Sumatran Fault | Semangko Barat B | 8 | Strike-Slip | 90 | 80 | 7.3 |  |  | 26.7 | 20.14% |
| 48 | Sumatran Fault | Semangko Timur A | 5 | Strike-Slip | 90 | 12 | 6.5 |  |  | 71.7 | 43.23% |
| 49 | Sumatran Fault | Semangko Timur B | 3 | Strike-Slip | 90 | 35 | 6.9 |  |  | 19.64 | 17.57% |
| 50 | Sumatran Fault | Semangko Graben | 3 | Normal | 90 | 50 | 6.5 |  |  | 37.85 | 28.80% |
| 51 | Sumatran Fault | Ujung Kulon A | 10 | Strike-Slip | 90 | 80 | 7.3 |  |  |  | 24.51% |
| 52 | Mentawai Fault | Mentawai | 5 | Reverse-Slip | 45W | 560 | 8.2 |  |  |  | 26.17% |
| 53 | Mentawai Fault | Enggano | 5 | Reverse-Slip | 45W | 160 | 7.6 |  |  |  | 29.00% |

Table S2. A summary of historical earthquakes along the Sunda trench.

| Index | Structure Name | Segment | L (km) | W (km) | Maximum Magnitude (Mw)* | Slip rate (mm/yr) | Historical Earthquake Year (Mw) | Dip (°) | Time elapse until 2022 (years) | BPT (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | Sumatran Megathrust | Aceh-Andaman | 1300 | 200 | 9.0 | 40 | 1881 M=7.4; 1941 M=7.6; 2004 Mw = 9.2 | 20 | 18 | 0.00 |
| M2 | Sumatran Megathrust | Nias-Simelue | 400 | 200 | 8.3 | 40 | 1861 Mw=8.5; and 2005 Mw=8.7 | 20 | 17 | 1.93 |
| M3 | Sumatran Megathrust | Batu | 70 | 100 | 7.1 | 40 | 1935 Mw=7.7 | 20 | 87 | 96.7 |
| M4 | Sumatran Megathrust | Mentawai-Siberut | 200 | 200 | 7.8 | 40 | 1797 Mw = 8,7;1833 Mw = 8,9 | 20 | 189 | 69.8 |
| M5 | Sumatran Megathrust | Mentawai-Pagai | 400 | 200 | 8.3 | 40 | 2007 Mw=8.5; 2010 Mw=7.8 | 20 | 12 | 1.2 |
| M6 | Sumatran Megathrust | Enggano | 250 | 200 | 8.0 | 40 | 2000 Mw=7.0 | 20 | 22 | 20.0 |
| M7 | Sunda Strait Megathrust | Selat Sunda Banten | 280 | 200 | 8.0 | 40 | 1699 Mw=7.5; 1780 Mw=8.5 (Musson 2012, Wichman 1908) | 20 | 242 | 57.0 |

*The maximum magnitude (Mw) was estimated based on the empirical relations of Subsurface Rupture Length (Magnitude = 4.38+(1.49*log (RLD) by Wells and Coppersmith, (1994).

Table S3. Multiple-segment ruptures pair for Sumatra fault.

|  |  | No.; Rupture Scenario (ID segment); Max Length (km); Mmax; Recurrence Interval (years). 1; 51; 50; 164; 7.5; 16423. 2; 51; 50; 49; 249; 7.7; 31646. 3; 51; 50; 48; 276; 7.8; 24691. 4; 51; 49; 165; 7.5; 17883. 5; 50; 48; 197; 7.6; 43879. 6; 50; 49; 170; 7.5; 31827. 7; 50; 49; 48; 282; 7.8; 41684. 8; 49; 47; 180; 7.6; 58858. 9; 49; 48; 198; 7.6; 49407. 10; 49; 48; 47; 292; 7.8; 39841. 11; 48; 47; 207; 7.6; 42283. 12; 48; 46; 134; 7.4; 5405. 13; 48; 45; 175; 7.6; 10203. 14; 48; 47; 46; 228; 7.7; 48239. 15; 48; 47; 46; 45; 290; 7.8; 27473. 16; 48; 46; 45; 196; 7.6; 8995. 17; 47; 46; 115; 7.4; 26810. 18; 47; 45; 157; 7.5; 18713. 19; 47; 46; 45; 178; 7.6; 31299. 20; 47; 46; 45; 44; 291; 7.8; 24231. 21; 46; 45; 84; 7.2; 3574. 22; 46; 45; 44; 197; 7.6; 5776. 23; 45; 44; 176; 7.6; 6450. 24; 44; 43; 195; 7.6; 4740. 25; 43; 42; 168; 7.6; 5110. 26; 43; 42; 41; 276; 7.8; 6448. 27; 42; 41; 194; 7.6; 8114. 28; 42; 41; 40; 262; 7.7; 7023. 29; 41; 40; 175; 7.6; 8959. 30; 41; 40; 39; 253; 7.7; 7121. 31; 41; 40; 39; 38; 355; 7.9; 4684. 33; 40; 39; 145; 7.6; 4247. 34; 39; 38; 180; 7.5; 7856. 35; 39; 38; 37; 247; 7.7; 6537. 36; 38; 37; 170; 7.6; 8202. 37; 38; 37; 36; 263; 7.7; 6409. 38; 38; 37; 36; 35; 312; 7.8; 5445. 39; 37; 36; 161; 7.5; 3687. 40; 37; 36; 35; 209; 7.6; 4330. 41; 36; 35; 34; 268; 7.7; 9071. 42; 36; 35; 34; 18; 295; 7.9; 7298. 43; 35; 34; 175; 7.6; 10566. 44; 35; 33; 220; 7.7; 6021. 45; 35; 34; 33; 347; 7.9; 9430. 46; 34; 33; 298; 7.8; 8386. 47; 34; 33; 32; 405; 7.9; 8579. 48; 34; 32; 233; 7.7; 10362. 49; 34; 32; 29; 417; 8; 3879. 50; 33; 32; 278; 7.8; 5114. 51; 32; 29; 291; 7.8; 5156. 52; 29; 30; 216; 7.7; 5427. 53; 29; 31; 210; 7.7; 6108. 55; 29; 31; 23; 285; 7.8; 4185. 56; 30; 31; 57; 7.1; 1910. 57; 31; 23; 101; 7.3; 1685. 58; 23; 22; 180; 7.6; 10110. 59; 23; 22; 25; 201; 7.7; 9614. 61; 22; 25; 126; 7.5; 13656. 62; 22; 24; 159; 7.4; 11510. 63; 22; 21; 137; 7.5; 3250. 64; 22; 25; 24; 180; 7.6; 24396. 65; 22; 25; 24; 21; 213; 7.7; 10517. 66; 25; 24; 76; 7.2; 3119. 67; 25; 24; 11; 115; 7.4; 404858. 68; 25; 11; 60; 7.3; 262467. 69; 24; 21; 88; 7; 1831. 70; 24; 21; 11; 127; 7.3; 82169. 71; 24; 11; 94; 7.1; 357143. 72; 21; 18; 60; 7.1; 1934. 73; 22; 21; 18; 164; 7.5; 5698. 74; 21; 20; 66; 7.1; 18904. 75; 21; 19; 55; 7; 1598. 76; 21; 19; 20; 88; 7.3; 23305. 77; 18; 26; 71; 7.2; 31162. 78; 18; 17; 168; 7.6; 6731. 79; 18; 17; 26; 212; 7.7; 67476. 80; 17; 15; 178; 7.6; 11869. 81; 17; 16; 205; 7.6; 5338. 82; 17; 16; 15; 242; 7.7; 7132. 83; 17; 15; 14; 397; 7.9; 6150. 84; 27; 26; 94; 7.3; 105153. 85; 27; 28; 87; 7.2; 40800. 86; 27; 26; 28; 131; 7.4; 124688. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| No. | Rupture Scenario (ID segment) |  |  |  | Max Length (km) | Mmax | Recurrence Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 51 | 50 |  |  | 164 | 7.5 | 16423 |
| 2 | 51 | 50 | 49 |  | 249 | 7.7 | 31646 |
| 3 | 51 | 50 | 48 |  | 276 | 7.8 | 24691 |
| 4 | 51 | 49 |  |  | 165 | 7.5 | 17883 |
| 5 | 50 | 48 |  |  | 197 | 7.6 | 43879 |
| 6 | 50 | 49 |  |  | 170 | 7.5 | 31827 |
| 7 | 50 | 49 | 48 |  | 282 | 7.8 | 41684 |
| 8 | 49 | 47 |  |  | 180 | 7.6 | 58858 |
| 9 | 49 | 48 |  |  | 198 | 7.6 | 49407 |
| 10 | 49 | 48 | 47 |  | 292 | 7.8 | 39841 |
| 11 | 48 | 47 |  |  | 207 | 7.6 | 42283 |
| 12 | 48 | 46 |  |  | 134 | 7.4 | 5405 |
| 13 | 48 | 45 |  |  | 175 | 7.6 | 10203 |
| 14 | 48 | 47 | 46 |  | 228 | 7.7 | 48239 |
| 15 | 48 | 47 | 46 | 45 | 290 | 7.8 | 27473 |
| 16 | 48 | 46 | 45 |  | 196 | 7.6 | 8995 |
| 17 | 47 | 46 |  |  | 115 | 7.4 | 26810 |
| 18 | 47 | 45 |  |  | 157 | 7.5 | 18713 |
| 19 | 47 | 46 | 45 |  | 178 | 7.6 | 31299 |
| 20 | 47 | 46 | 45 | 44 | 291 | 7.8 | 24231 |
| 21 | 46 | 45 |  |  | 84 | 7.2 | 3574 |
| 22 | 46 | 45 | 44 |  | 197 | 7.6 | 5776 |
| 23 | 45 | 44 |  |  | 176 | 7.6 | 6450 |
| 24 | 44 | 43 |  |  | 195 | 7.6 | 4740 |
| 25 | 43 | 42 |  |  | 168 | 7.6 | 5110 |
| 26 | 43 | 42 | 41 |  | 276 | 7.8 | 6448 |
| 27 | 42 | 41 |  |  | 194 | 7.6 | 8114 |
| 28 | 42 | 41 | 40 |  | 262 | 7.7 | 7023 |
| 29 | 41 | 40 |  |  | 175 | 7.6 | 8959 |
| 30 | 41 | 40 | 39 |  | 253 | 7.7 | 7121 |
| 31 | 41 | 40 | 39 | 38 | 355 | 7.9 | 4684 |
| 33 | 40 | 39 |  |  | 145 | 7.6 | 4247 |
| 34 | 39 | 38 |  |  | 180 | 7.5 | 7856 |
| 35 | 39 | 38 | 37 |  | 247 | 7.7 | 6537 |
| 36 | 38 | 37 |  |  | 170 | 7.6 | 8202 |
| 37 | 38 | 37 | 36 |  | 263 | 7.7 | 6409 |
| 38 | 38 | 37 | 36 | 35 | 312 | 7.8 | 5445 |
| 39 | 37 | 36 |  |  | 161 | 7.5 | 3687 |
| 40 | 37 | 36 | 35 |  | 209 | 7.6 | 4330 |
| 41 | 36 | 35 | 34 |  | 268 | 7.7 | 9071 |
| 42 | 36 | 35 | 34 | 18 | 295 | 7.9 | 7298 |
| 43 | 35 | 34 |  |  | 175 | 7.6 | 10566 |
| 44 | 35 | 33 |  |  | 220 | 7.7 | 6021 |
| 45 | 35 | 34 | 33 |  | 347 | 7.9 | 9430 |
| 46 | 34 | 33 |  |  | 298 | 7.8 | 8386 |
| 47 | 34 | 33 | 32 |  | 405 | 7.9 | 8579 |
| 48 | 34 | 32 |  |  | 233 | 7.7 | 10362 |
| 49 | 34 | 32 | 29 |  | 417 | 8 | 3879 |
| 50 | 33 | 32 |  |  | 278 | 7.8 | 5114 |
| 51 | 32 | 29 |  |  | 291 | 7.8 | 5156 |
| 52 | 29 | 30 |  |  | 216 | 7.7 | 5427 |
| 53 | 29 | 31 |  |  | 210 | 7.7 | 6108 |
| 55 | 29 | 31 | 23 |  | 285 | 7.8 | 4185 |
| 56 | 30 | 31 |  |  | 57 | 7.1 | 1910 |
| 57 | 31 | 23 |  |  | 101 | 7.3 | 1685 |
| 58 | 23 | 22 |  |  | 180 | 7.6 | 10110 |
| 59 | 23 | 22 | 25 |  | 201 | 7.7 | 9614 |
| 61 | 22 | 25 |  |  | 126 | 7.5 | 13656 |
| 62 | 22 | 24 |  |  | 159 | 7.4 | 11510 |
| 63 | 22 | 21 |  |  | 137 | 7.5 | 3250 |
| 64 | 22 | 25 | 24 |  | 180 | 7.6 | 24396 |
| 65 | 22 | 25 | 24 | 21 | 213 | 7.7 | 10517 |
| 66 | 25 | 24 |  |  | 76 | 7.2 | 3119 |
| 67 | 25 | 24 | 11 |  | 115 | 7.4 | 404858 |
| 68 | 25 | 11 |  |  | 60 | 7.3 | 262467 |
| 69 | 24 | 21 |  |  | 88 | 7 | 1831 |
| 70 | 24 | 21 | 11 |  | 127 | 7.3 | 82169 |
| 71 | 24 | 11 |  |  | 94 | 7.1 | 357143 |
| 72 | 21 | 18 |  |  | 60 | 7.1 | 1934 |
| 73 | 22 | 21 | 18 |  | 164 | 7.5 | 5698 |
| 74 | 21 | 20 |  |  | 66 | 7.1 | 18904 |
| 75 | 21 | 19 |  |  | 55 | 7 | 1598 |
| 76 | 21 | 19 | 20 |  | 88 | 7.3 | 23305 |
| 77 | 18 | 26 |  |  | 71 | 7.2 | 31162 |
| 78 | 18 | 17 |  |  | 168 | 7.6 | 6731 |
| 79 | 18 | 17 | 26 |  | 212 | 7.7 | 67476 |
| 80 | 17 | 15 |  |  | 178 | 7.6 | 11869 |
| 81 | 17 | 16 |  |  | 205 | 7.6 | 5338 |
| 82 | 17 | 16 | 15 |  | 242 | 7.7 | 7132 |
| 83 | 17 | 15 | 14 |  | 397 | 7.9 | 6150 |
| 84 | 27 | 26 |  |  | 94 | 7.3 | 105153 |
| 85 | 27 | 28 |  |  | 87 | 7.2 | 40800 |
| 86 | 27 | 26 | 28 |  | 131 | 7.4 | 124688 |

Table S4. Multiple-segment ruptures pair for Sunda Trench.

| No; Ruptures Scenario; Max Length; Mmax; Recurrence Interval (years). 1; M1; M2; 1218.447; 8.9; 4070. 2; M1; M2; M3; 1555.068; 8.9; 3598. 3; M1; M2; M3; M4; 1624.065; 8.9; 4861. 4; M1; M2; M3; M4; M5; 1917.101; 9; 18155. 5; M1; M2; M3; M4; M5; M6; 2175.954; 9; 15987. 6; M1; M2; M3; M4; M5; M6; M7; 2447.889; 9.1; 15793. 7; M2; M3; 343.477; 8.3; 2260. 8; M2; M3; M4; 413.070; 8.6; 6644. 9; M2; M3; M4; M5; 711.074; 8.7; 5548. 10; M2; M3; M4; M5; M6; 975.512; 8.8; 7880. 11; M2; M3; M4; M5; M6; M7; 1254.625; 8.9; 4909. 12; M3; M4; 69.217; 8.3; 3592. 13; M3; M4; M5; 367.169; 8.5; 4218. 14; M3; M4; M5; M6; 631.777; 8.6; 5130. 15; M3; M4; M5; M6; M7; 911.368; 8.8; 7477. 16; M4; M5; 297.831; 8.5; 5531. 17; M4; M5; M6; 562.595; 8.6; 6200. 18; M4; M5; M6; M7; 842.423; 8.8; 8033. 19; M5; M6; 264.726; 8.4; 2661. 20; M5; M6; M7; 545.135; 8.7; 5266. 21; M6; M7; 280.007; 8.5; 2934. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| No | Ruptures Scenario |  |  |  |  |  |  | Max Length (km) | Mmax | Recurrence Interval (years) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | M1 | M2 |  |  |  |  |  | 1218 | 9.2 | 13173 |
| 2 | M1 | M2 | M3 |  |  |  |  | 1555 | 9.3 | 16659 |
| 3 | M1 | M2 | M3 | M4 |  |  |  | 1624 | 9.4 | 14178 |
| 4 | M1 | M2 | M3 | M4 | M5 |  |  | 1917 | 9.5 | 109529 |
| 5 | M1 | M2 | M3 | M4 | M5 | M6 |  | 2176 | 9.5 | 95511 |
| 6 | M1 | M2 | M3 | M4 | M5 | M6 | M7 | 2448 | 9.6 | 91407 |
| 7 | M2 | M3 |  |  |  |  |  | 343 | 8.4 | 6003 |
| 8 | M2 | M3 | M4 |  |  |  |  | 413 | 8.7 | 12384 |
| 9 | M2 | M3 | M4 | M5 |  |  |  | 711 | 8.9 | 9467 |
| 10 | M2 | M3 | M4 | M5 | M6 |  |  | 976 | 9.1 | 26157 |
| 11 | M2 | M3 | M4 | M5 | M6 | M7 |  | 1255 | 9.3 | 24468 |
| 12 | M3 | M4 |  |  |  |  |  | 69 | 8.3 | 6503 |
| 13 | M3 | M4 | M5 |  |  |  |  | 367 | 8.7 | 13749 |
| 14 | M3 | M4 | M5 | M6 |  |  |  | 632 | 8.9 | 10302 |
| 15 | M3 | M4 | M5 | M6 | M7 |  |  | 911 | 9.1 | 22738 |
| 16 | M4 | M5 |  |  |  |  |  | 298 | 8.6 | 11417 |
| 17 | M4 | M5 | M6 |  |  |  |  | 563 | 8.8 | 10709 |
| 18 | M4 | M5 | M6 | M7 |  |  |  | 842 | 9.1 | 24114 |
| 19 | M5 | M6 |  |  |  |  |  | 265 | 8.6 | 10473 |
| 20 | M5 | M6 | M7 |  |  |  |  | 545 | 8.9 | 8022 |
| 21 | M6 | M7 |  |  |  |  |  | 280 | 8.7 | 7504 |