Appendix

A1. Gross energy conversion

Data based on net and gross energy generation were used to construct the EPPD in order to analyze Poland's energy mix. Due to limited data availability, it was necessary to convert generated (gross) energy to used energy (energy demand). For RES, the loss originates main from distribution losses (column 3 of Table A-1), which was reported as 5.7% (Schneider Electric Polska 2015). For the case of combustion-based power plants (coal, natural gas), additional losses were reported from part of the generated energy that is used to fulfil the power plant's own needs. As the energy losses were not reported in published data, it is hence necessary to calculate the own needs coefficient (ONC).

For the power generation data for year 2022, the gross energy is given in column 1 of Table A-1 (Dusiło 2023a). The total gross energy generated by all energy sources is given as 179.0 TWh (see details in Table A-1). O the other hand, the total energy demand is given as 149.6 TWh.

Table A-1. Data for calculating ONC

|  | Gross energy [TWh] | Energy loss of the power plant (calculated) [TWh] | Distribution losses (5,7%) [TWh] | Energy demand (calculated) [TWh] |
| --- | --- | --- | --- | --- |
| Biomass | 6.2 | 0.850 | 0.305 | 5.045 |
| Biogas | 1.4 | 0.192 | 0.069 | 1.139 |
| Nuclear power | 0.0 | 0.000 | 0.000 | 0.000 |
| Hydropower | 2.0 | 0.000 | 0.114 | 1.886 |
| Wind energy, onshore | 19.4 | 0.000 | 1.106 | 18.294 |
| Wind energy, offshore | 0.0 | 0.000 | 0.000 | 0.000 |
| Photovoltaics | 8.0 | 0.000 | 0.456 | 7.544 |
| Natural gas | 11.7 | 1.604 | 0.575 | 9.521 |
| Pumped storage | 1.1 | 0.000 | 0.063 | 1.037 |
| Industrial | 2.9 | 0.398 | 0.143 | 2.360 |
| Hard coal | 79.0 | 10.830 | 3.886 | 64.284 |
| Lignite | 47.3 | 6.484 | 2.327 | 38.489 |
| In tota | 179.0 |  |  | 149.6 |

The task is to find the ONC value so that the total sum of the energy from all sources is equal to 149.6 TWh. As can be seen in Table A-1 (column 2), only the energy loss values ​​for combustion-based power plants are calculated ( = Gross energy * ONC TWh). The transmission losses in column 3 apply to all energy sources ( = 0.057*(gross energy – energy loss) TWh). It is necessary to find such an ONC value so that the sum of energy in the fifth column is 149.6 TWh. This can be done analytically or numerically using some computational toolbox (e.g. Solver add-on from MS Excel). Column 4 contains values ​​that meet the task condition. These values ​​were calculated for ONC=13.7%.

A2. Percentage of cloudy hours per daylight time in a year

In order to determine the average cloudiness in Poland and its impact on photovoltaic efficiency, it is worth calculating the percentage of cloudy hours during daylight hours in a yearly basis. This parameter is not publicly available. Hence, the following data were used for calculation:

- Average day length in subsequent months in Poland (see Table A-2)

- Number of sunny hours per year in individual regions of Poland (Pietrzak 2023).

Table A-2. Average daylight time in Poland in particular months of the year (Sołtys 2022)

| Month | Average length of daylight time [hour] |
| --- | --- |
| January | 8.5 |
| February | 9.5 |
| March | 12 |
| April | 13.5 |
| May | 15 |
| June | 16 |
| July | 15.5 |
| August | 14 |
| September | 12.5 |
| October | 10.5 |
| November | 8.5 |
| December | 8 |

The average length of day in a year calculated based on data from Table A-2 is 11.96 hours. During  year, the length of day is 4365 hours. Based on data from Pietrzak (2023), the average number of sunny hours in Poland was calculated as 1700 hours. Therefore, the number of cloudy hours during the daylight time is 2665 hours (= 4365 – 1700 h). Thus, the percentage of cloudy hours during the daylight in year is calculated as 61% (= 2665/4365*100%). The value of this parameter shows that in Poland, difficulties in generating photovoltaic energy is due to changes in the length of daylight time, changes in the angle of incidence of the sun's rays during the year, as well as a large percentage of cloudy weather.