Table 1. Isotherm, kinetic, thermodynamic and other equations used in this study.

| Model/Equation | Parameter | Ref. |
| --- | --- | --- |
| Langmuir 1qe=1qmKL1Ce+1qm RL=11+KLC0 Freundlich logqe=logKF+1nlogCe Dubinin-Radushkevich lnqe=lnqm-KDRε2 Elovich lnqeCe=ln(KEqm)-qeqm | qe (mg/g): equilibrium adsorption capacity qm (mg/g): maximum adsorption capacity KL (L/mg): Langmuir constant Ce (mg/L): equilibrium adsorbate concentration in solution Co (mg/L): initial adsorbate concentration in solution RL: separation factor KF (mg/g (L/mg)1/n): Freundlich constant n: adsorption intensity KDR (mol2/kJ2): Dubinin-Radushkevich constant ϵ: Polanyi potential, ε=RTln1+1Ce R (J/mol·K): gas constant T (K): absolute temperature E (kJ/mol): mean free energy of adsorption, E=12KDR KE (L/mg): Elovich equilibrium constant | [22-25] |
| Pseudo-first-order ln(qe-qe)=lnqe-k1t Pseudo-second-order tqt=1k2qe2+tqe Intraparticle diffusion qt=kpt1/2+C Intraparticle diffusion coefficient ln1-qtqe=ln6π2-Dπ2ta2 | k1 (h-1): pseudo-first-order rate constant k2 (g/mg·h): pseudo-second-order rate constant kP (mg/g·h1/2): intraparticle diffusion rate constant t (h): time C (mg/g): constant relevant to the thickness of boundary layer D (cm2/h): intraparticle diffusion coefficient a (cm): radius of the adsorbent | [26-28] |
| van’t Hoff eq. lnKL=-∆H0RT+∆S0R ∆G0=∆H0-T∆S0 Clausius-Clapeyron eq. d(ln Ce)dT=-∆HxRT2 | ΔH0 (kJ/mol): standard enthalpy change ΔS0 (J/mol·K): standard entropy change ΔG0 (kJ/mol): standard Gibbs free energy change ΔHx (kJ/mol): isosteric heat of adsorption | [29,30] |