## APPENDIX

In this section, the source code used for the design of a horizontal single-axis solar tracker is presented. These program codes are used both in the simulation (section 3.3) and the prototype model (section 3.4) presented in this paper. The flow chart in the result section was derived from the operation instruction of the program code given.

/*

*

*

Design of Single Axis Solar Tracker

MCU: PIC16F877A

Clcok frequency: 32.00MHz

Oscillator: 8MHz

Programmer: Sylvester Tirones

*

*

*/

// Lcd pinout settings

sbit LCD_RS at RC4_bit;

sbit LCD_EN at RC5_bit;

sbit LCD_D7 at RC3_bit;

sbit LCD_D6 at RC2_bit;

sbit LCD_D5 at RC1_bit;

sbit LCD_D4 at RC0_bit;

// Pin direction

sbit LCD_RS_Direction at TRISC4_bit;

sbit LCD_EN_Direction at TRISC5_bit;

sbit LCD_D7_Direction at TRISC3_bit;

sbit LCD_D6_Direction at TRISC2_bit;

sbit LCD_D5_Direction at TRISC1_bit;

sbit LCD_D4_Direction at TRISC0_bit;

int rotation;                       // initialize rotation as integer

int initial_Position;               // initialize initial_Position as integer

int set_Position;                   // initialize set_Position as integer

int x;                              // initializs variable x as integer

float read;                         // initialize read as floating values

unsigned char txt[14], data_, P_text[7];              // initialize txt as character

char PV_Pos [7] = "Solar:" ;

#define  LDR1  PORTD.B1

#define  LDR2  PORTD.B2

#define  LDR3  PORTD.B3

#define  LDR4  PORTD.B4

#define  LDR5  PORTD.B5

#define  LDR6  PORTD.B6

#define  LDR7  PORTD.B7

#define  r_LDR  PORTD.B0

#define ON 1

#define OFF 0

#define West PORTA.B0

#define East PORTA.B1

#define PV PORTA.B2

#define Other PORTA.B3

void direction(){

/*

if rotation = 1 ----- forward direction

if rotation = 2 ----- reverse direction

*/

switch (rotation){

case 1:                    // forward direction

Lcd_Out (1,21, "Clockwise");

East = ~ East;

West = OFF;

PORTA.B5 = 1;

PORTE.B0 = 0;

PORTE.B1 = 0;

PORTE.B2 = 0;

Delay_ms (10);

PORTA.B5 = 0;

PORTE.B0 = 1;

PORTE.B1 = 0;

PORTE.B2 = 0;

Delay_ms (10);

PORTA.B5 = 0;

PORTE.B0 = 0;

PORTE.B1 = 1;

PORTE.B2 = 0;

Delay_ms (10);

PORTA.B5 = 0;

PORTE.B0 = 0;

PORTE.B1 = 0;

PORTE.B2 = 1;

Delay_ms (10);

break;

case 2:                    // reverse direction

Lcd_Out (1,21, "Anti Clockwise");

West = ~ West;

East = OFF;

PORTA.B5 = 0;

PORTE.B0 = 0;

PORTE.B1 = 0;

PORTE.B2 = 1;

Delay_ms (10);

PORTA.B5 = 0;

PORTE.B0 = 0;

PORTE.B1 = 1;

PORTE.B2 = 0;

Delay_ms (10);

PORTA.B5 = 0;

PORTE.B0 = 1;

PORTE.B1 = 0;

PORTE.B2 = 0;

Delay_ms (10);

PORTA.B5 = 1;

PORTE.B0 = 0;

PORTE.B1 = 0;

PORTE.B2 = 0;

Delay_ms (10);

// use hexa number to decide the rotation

set_Position ++;

break;

case 3:                    // stop

Lcd_Out (1,21, "Stop.............");

break;

default:

break;

}

}

void GUI (){

IntToStr (Set_position, P_text);

UART1_Write_Text (P_text);

Delay_ms (500);

UART1_Write (13);

}

void main() {                                 // main function

TRISD = 0xFF;

TRISB = 0x00;

TRISA = 0x00;

TRISC = 0x80;

PORTD = 0x00;

PORTB = 0x00;

PORTA = 0x00;

TRISE = 0x00;

PORTE = 0x00;

ADCON1 = 0x0F;

ADCON0 = 0xF0;

UART1_Init (9600);

Lcd_Init(); // initialize LCD

Lcd_Cmd (_LCD_CURSOR_OFF);

//Welcome message

Lcd_Out (1,1, "Welcome to Smart BMS");

Lcd_Out (1,21, "Solar Tracking Sys");

Delay_ms (2000);

Lcd_Cmd (_LCD_CLEAR);

Lcd_Out (1,1, "Angle: ");

Lcd_Chr (1,10, 42);

Lcd_Out (2,1, "Position: ");

set_Position = 0;

initial_Position = 0;

START:

if (r_LDR == 0){

if (set_Position > initial_Position){

rotation = 1;

direction();

} else {

rotation = 2;

direction();

}

GUI();

} else {

rotation = 3;

initial_Position = set_Position;

switch (set_Position){

case 1:

Lcd_Out (1,7, "-30");

Lcd_Chr (2,16, 49);

LDR1 = OFF;

PORTB = 0x01;

break;

case 2:

Lcd_Out (1,7, "-60");

Lcd_Chr (2,16, 50);

LDR2 = OFF;

PORTB = 0x02;

break;

case 3:

Lcd_Out (1,7, "-90");

Lcd_Chr (2,16, 51);

LDR3 = OFF;

PORTB = 0x03;

break;

case 4:

Lcd_Out (1,7, "+60");

Lcd_Chr (2,16, 52);

LDR4 = OFF;

PORTB = 0x04;

break;

case 5:

Lcd_Out (1,7, "+30");

Lcd_Chr (2,16, 53);

LDR5 = OFF;

PORTB = 0x05;

break;

case 6:

Lcd_Out (1,7, "+90");

Lcd_Chr (2,16, 54);

LDR6 = OFF;

PORTB = 0x06;

break;

case 7:

Lcd_Out (1,7, "+120");

Lcd_Chr (2,16, 55);

LDR7 = OFF;

PORTB = 0x07;

break;

default:

break;

}

direction();

Delay_ms (1000);

}

do{

if (UART1_Data_Ready()){

data_ = UART1_Read();

switch (data_){

case 'A':

Delay_ms (20);

UART1_Write_Text (PV_Pos);

IntToStr (Set_Position, P_text);

UART1_Write_Text (P_text);

Delay_ms (500);

UART1_Write (13);

break;

default:

// do nothing

break;

}

}

if (LDR1 == ON){

set_Position = 1;

GUI();

goto START;

} else if (LDR2 == ON){

set_Position = 2;

GUI();

goto START;

} else if (LDR3 ==ON){

set_Position = 3;

GUI();

goto START;

} else if (LDR4 ==ON){

set_Position = 4;

GUI();

goto START;

} else if (LDR5 ==ON){

set_Position = 5;

GUI();

goto START;

} else if (LDR6 == ON) {

set_Position = 6;

GUI();

goto START;

} else if (LDR7 ==ON){

set_Position = 7;

GUI();

goto START;

}

} while (1);

}